3GPP TSG-RAN5 Meeting # 104	R5-244198

# Maastricht, Netherlands, 19th Aug 2024 – 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4476 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to test case 10.1.2.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Arriver Software AB |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Test case 10.1.2.3 is using NR Cell 1 instead of NGC Cell A. For specification consistency, NR Cell 1 needs to be replaced with NGC Cell A, along with the table references. |  |  |  |  |  |  |  |  |
| Summary of change: |  | NR Cell 1 is replaced with NGC Cell A under cl. 10.1.2.3.3.1. Reference to NGC Cell A from TS38.508-1 is updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect cell configuration may result in incorrect testcase implementation |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN Imapct |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

#### 10.1.2.3	Network-requested PDU session modification / new P-CSCF address / Initial IMS registration

10.1.2.3.1	Test Purpose (TP)

(1)

with { UE in 5GMM-REGISTERED.NORMAL-SERVICE state and 5GMM-CONNECTED mode and having performed IMS registration }

ensure that {

when { UE receives PDU SESSION MODIFICATION COMMAND message containing P-CSCF addresses different from the currently used P-CSCF }

then { UE sends a PDU SESSION MODIFICATION COMPLETE message and performs initial IMS registration using the first P-CSCF address received }

}

10.1.2.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.229 [44], clause U.2.2.1, U.2.2.1C, TS 24.501[22], clause 6.3.2.2. Unless otherwise stated these are Rel-15 requirements.

[TS 24.229 clause U.2.2.1]:

Prior to communication with the IM CN subsystem, the UE shall:

…

d)	acquire a P-CSCF address(es).

The methods for P-CSCF discovery are:

I.	When using IPv4, employ the DHCP RFC 2132 [20F], the DHCPv4 options for SIP servers RFC 3361 [35A], and RFC 3263 [27A] as described in subclause 9.2.1. When using IPv6, employ Dynamic Host Configuration Protocol for IPv6 (DHCPv6) RFC 3315 [40], the DHCPv6 options for SIP servers RFC 3319 [41] and DHCPv6 options for DNS RFC 3646 [56C] as described in subclause 9.2.1.

II.	Transfer P-CSCF address(es) within the 5GS PDU session establishment procedure.

The UE shall indicate the request for a P-CSCF address to the network within the extended Protocol Configuration Options information element of the PDU SESSION ESTABLISHMENT REQUEST message.

If the network provides the UE with a list of P-CSCF IPv4 or IPv6 addresses in the PDU SESSION ESTABLISHMENT ACCEPT message, the UE shall assume that the list is ordered top-down with the first P-CSCF address within the extended Protocol Configuration Options information element as the P-CSCF address having the highest preference and the last P-CSCF address within the extended Protocol Configuration Options information element as the P-CSCF address having the lowest preference.

…

The UE can freely select method I or II for P-CSCF discovery, if:

-	the UE is in the home network; or

-	the UE is roaming and the P-CSCF is to be discovered in the visited network.

…

[TS 24.229 clause U.2.2.1C]:

A UE supporting the P-CSCF restoration procedure performs one of the following procedures:

A	if the UE used method II for P-CSCF discovery and if the UE receives one or more P-CSCF address(es) in the extended Protocol Configuration Options information element of a PDU SESSION MODIFICATION COMMAND message and the one or more P-CSCF addresse(s) do not include the address of the currently used P-CSCF, then the UE shall acquire a different P-CSCF address from the one or more P-CSCF addresse(s) in the PDU SESSION MODIFICATION COMMAND message. If more than one P-CSCF address with the same container identifier (i.e. "P-CSCF IPv6 Address" or "P-CSCF IPv4 Address") are included, then the UE shall assume that the more than one P-CSCF addresses with the same container identifier are prioritised with the first P-CSCF address with the same container identifier within the Protocol Configuration Options information element as the P-CSCF address with the highest priority.

If the UE used method II for P-CSCF discovery and if the UE has previously sent the "P-CSCF Re-selection support" PCO indicator at PDU session creation and if the UE receives one or more P-CSCF address(es) in the extended Protocol Configuration Options information element of a PDU SESSION MODIFICATION COMMAND message, then the UE shall acquire a P-CSCF address from the one or more P-CSCF addresse(s) in the PDU SESSION MODIFICATION COMMAND message. If more than one P-CSCF address with the same container identifier (i.e. "P-CSCF IPv6 Address" or "P-CSCF IPv4 Address") are included, then the UE shall assume that the more than one P-CSCF addresses with the same container identifier are prioritised with the first P-CSCF address with the same container identifier within the Protocol Configuration Options information element as the P-CSCF address with the highest priority;

B	if the UE uses RFC 6223 [143] as part of P-CSCF restoration procedures, and if the P-CSCF fails to respond to a keep-alive request, then the UE shall acquire a different P-CSCF address using one of the methods I, III and IV for P-CSCF discovery described in the subclause U.2.2.1.

If the UE has an ongoing session and acquired the new P-CSCF address by using procedure A described above, the UE may wait until the UE has detected that the ongoing session has ended before performing an initial registration as specified in subclause 5.1.

…

[TS 24.501 clause 6.3.2.2]:

…

When UE has requested P-CSCF IPv6 address or P-CSCF IPv4 address and the SMF has provided P-CSCF address(es) during the PDU session establishment procedure, if the network-requested PDU session modification procedure is triggered for P-CSCF restoration, the SMF shall include the P-CSCF IP address(es) in the Extended protocol configuration options IE in the PDU SESSION MODIFICATION COMMAND message as specified in subclause 5.8.2.2 of 3GPP TS 23.380 [54].

10.1.2.3.3	Test description

10.1.2.3.3.1	Pre-test conditions

System Simulator:

-	NGC Cell A is configured as "Serving cell" according to TS 38.508-1 [4] Table 6.3.2.2-1 .

-	System information combination NR-1 as defined in TS 38.508-1 [4], subclause 4.4.3.1.2 is used.

UE:

-	None

Preamble:

-	The UE is in test state 3N-A with IMS registered as defined in TS 38.508-1 [4], subclause 4.4A.2 on NGC cell A.

10.1.2.3.2	Test procedure sequence

Table 10.1.2.3.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a PDU SESSION MODIFICATION COMMAND message containing new P-CSCF addresses. | <-- | 5GSM: PDU SESSION MODIFICATION COMMAND | - | - |
| 2 | Check: Does the UE transmit a PDU SESSION MODIFICATION COMPLETE message? | --> | 5GSM: PDU SESSION MODIFICATION COMPLETE | 1 | P |
| - | The subsequent messages are sent to the first P-CSCF address received at step 1. | - | - | - | - |
| 3-10 | Check: Does the UE correctly initiate and complete initial IMS registration using the first address from received new P-CSCF addresses? (Steps 1-8 of Annex A.2 in TS 34.229-5 [41]) | - | - | 1 | P - |
| 11 | SS transmits an RRCRelease message. | <-- | NR RRC: RRCRelease | - | - |

10.1.2.3.3.3	Specific message contents

Table 10.1.2.3.3.3-1: PDU SESSION MODIFICATION COMMAND (step 1, Table 10.1.2.3.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.7.2-9 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol configuration options |  |  |  |  |
| Container ID 1 |  | ‘0001’H |  | PCSCF_IPv6 |
| Length of container ID 1 contents |  |  | Length value determined by test implementation |  |
| Container ID 1 contents |  | IPv6 address | P-CSCF IPv6 Address different to that allocated in the PDU SESSION ESTABLISHMENT ACCEPT |  |
| Container ID 2 |  | ‘000C’H |  | PCSCF_IPv4 |
| Length of container ID 2 contents |  |  | Length value determined by test implementation |  |
| Container ID 2 contents |  | IPv4 address | P-CSCF IPv4 Address different to that allocated in the PDU SESSION ESTABLISHMENT ACCEPT |  |
| Container ID 3 |  | ‘0001’H |  | Additional_P-CSCF_IPv6 |
| Length of container ID 3 contents |  |  | Length value determined by test implementation |  |
| Container ID 3 contents |  | IPv6 address | Additional P-CSCF IPv6 Address different to that allocated in the PDU SESSION ESTABLISHMENT ACCEPT |  |
| Container ID 4 |  | ‘000C’H |  | Additional_P-CSCF_IPv4 |
| Length of container ID 4 contents |  |  | Length value determined by test implementation |  |
| Container ID 4 contents |  | IPv4 address | Additional P-CSCF IPv4 Address different to that allocated in the PDU SESSION ESTABLISHMENT ACCEPT |  |
| Condition | Explanation |  |  |  |
| P-CSCF_IPv6 | If in the PDU SESSION ESTABLISHMENT REQUEST for IMS, the IE Extended protocol configuration options contains a configuration protocol option = '0001H' (“P-CSCF IPv6 Address Request", length of contents = 0) |  |  |  |
| P-CSCF_IPv4 | If in the PDU SESSION ESTABLISHMENT REQUEST for IMS, the IE Extended protocol configuration options contains a configuration protocol option = '000CH' (“P-CSCF IPv4 Address Request", length of contents = 0) |  |  |  |
| Additional_P-CSCF_IPv6 | P-CSCF_IPv6 AND SS requires to be configured with second P-CSCF Ipv6 address |  |  |  |
| Additional_P-CSCF_IPv4 | P-CSCF_IPv4 AND SS requires to be configured with second P-CSCF Ipv4 address |  |  |  |

<End of modified section>