3GPP TSG-RAN5 Meeting # 104	R5-244206

# Maastricht, Netherlands, 19th – 23rd August 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-4 | CR | 0858 | rev |  | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of test requirement applicability rule for 8Rx tests |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_RF_FR1_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The Introduction of test requirement applicability rules for 8Rx tests are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of Introduction of test requirement applicability rule for 8Rx tests. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The WI cannot be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.2 5.1.1.3 5.1.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

5.1.1.2	Applicability of requirements for different number of RX antenna ports

The number of RX antenna ports for different RF operating bands is up to UE declaration.

The UE shall support 2, 4 or 8 RX antenna ports for different RF operating bands. The operating bands, where 4 RX antenna ports shall be the baseline, are defined in Clause 7.2 of TS 38.101-1 [3]. The UE requirements applicability for UEs with different number of RX antenna ports is defined in Table 5.1.1.2-1.

Table 5.1.1.2-1: Requirements applicability

| Supported RX antenna ports | Test type | Test list | Exceptions |
| --- | --- | --- | --- |
| UE supports only 2RX | PDSCH | All tests in Clause 5.2.2 |  |
|  | PDCCH | All tests in Clause 5.3.2 |  |
|  | PBCH | All tests in Clause 5.4.2 |  |
| UE supports only 4RX or both 2RX and 4RX | PDSCH | All tests in Clause 5.2.3 (Note 2) |  |
|  | PDCCH | All tests in Clause 5.3.3 (Note 2) |  |
|  | PBCH | All tests in Clause 5.4.2 or 5.4.3 Note 1 |  |
| UE supports 8Rx, 4Rx and 2Rx | PDSCH | All tests in Clause 5.2.3 (Note 2, 3) All tests in Clause 5.2.4 (Note 2) | If UE passes tests in Clause 5.2.4, UE can skip Test 2-1 and Test 2-2 in Clause 5.2.3 Table 5.2.3.1.1-4, Table 5.2.3.2.1-4 and Test 4-1 in Clause 5.2.3 Table 5.2.3.1.1-6, Table 5.2.3.2.1-6 |
| or | PDCCH | All tests in Clause 5.3.3 (Note 2,3) |  |
| UE supports only 8Rx and 4Rx | PBCH | All tests in Clause 5.4.3 (Note 1) |  |
| UE supports only 8Rx and 2Rx | PDSCH | All tests in Clause 5.2.2 (Note 2, 4) All tests in Clause 5.2.4 (Note 2) | If UE passes tests in Clause 5.2.4, UE can skip Test 2-1 and Test 2-2 in Clause 5.2.2 Table 5.2.2.1.1-4, Table 5.2.2.2.1-4 |
|  | PDCCH | All tests in Clause 5.3.2 (Note 2,4) |  |
|  | PBCH | All tests in Clause 5.4.2 |  |
| UE supports only 8Rx | PDSCH | All tests in Clause 5.2.4 (Note 2) |  |
|  | PDCCH | N/A |  |
|  | PBCH | N/A |  |
| Note 1: Requirements for PBCH with 4Rx is up to UE declaration. Note 2: ‘maxMIMO-Layers-r16’ is not configured during the performance requirements testing for UE supporting Release 16 per-BWP MIMO layer adaptation. Note 3: 8Rx capable UEs are tested on any of the 4Rx supported RF bands by connecting 4 out of 8 Rx with data source from system simulator, and the other 4 Rx are connected with zero input, depending on UE’s declaration and AP configuration. Requirements specified with 4Rx should be applied. Note 4: 8Rx capable UEs are tested on any of the 2Rx supported RF bands by connecting 2 out of 8 Rx with data source from system simulator, and the other 6 Rx are connected with zero input, depending on UE’s declaration and AP configuration. Requirements specified with 2Rx should be applied. Note 5: Tests defined for 8Rx capable UEs according to whether the UE is a ‘simplified 8Rx receiver’ or ‘baseline 8Rx receiver’ are specified in Table 5.1.1.3-1. |  |  |  |

5.1.1.3	Applicability of requirements for optional UE features

The performance requirements in Table 5.1.1.3-1 shall apply for UEs which support optional UE features only.

Table 5.1.1.3-1: Requirements applicability for optional UE features

| UE feature/capability [14] | Test type |  | Test list | Applicability notes |
| --- | --- | --- | --- | --- |
| SU-MIMO Interference Mitigation advanced receiver | FR1 FDD | PDSCH | Clause 5.2.2.1.1 (Test 3-1) Clause 5.2.3.1.1 (Test 5-1) |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.1 (Test 3-1) Clause 5.2.3.2.1 (Test 5-1) |  |
| Alternative additional DMRS position for co-existence with LTE CRS (additionalDMRS-DL-Alt) | FR1 FDD | PDSCH | Clause 5.2.2.1.4 (Test 1-2) Clause 5.2.3.1.4 (Test 1-2) |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.4 (Test 1-2) Clause 5.2.3.2.4 (Test 1-2) |  |
| … | … | … | … | … |
| CRS-IM in non-DSS and 30 kHz NR SCS scenario, with the assistance of network signalling on LTE channel bandwidth (crs-IM-nonDSS-NWA-30kHzSCS-r17) | FR1 TDD | PDSCH | Clause 5.2.2.2.20 (Test 1-2) Clause 5.2.3.2.19 (Test 1-2) | If the Test 2-2 in Clause 5.2.2.2.20 is passed, the test coverage can be considered fulfilled without executing Test 1-2 in clause 5.2.2.2.20. If the Test 2-2 in Clause 5.2.3.2.19 is passed, the test coverage can be considered fulfilled without executing Test 1-2 in clause 5.2.3.2.19. |
| Support for SFN scheme A for PDCCH scheduling SFN Scheme A PDSCH (sfn-SchemeA-r17) | FR1 FDD | PDSCH | Clause 5.2.2.1.20 Clause 5.2.3.1.19 |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.21 Clause 5.2.3.2.20 |  |
| Support for SFN scheme B for PDCCH scheduling SFN Scheme B PDSCH (sfn-SchemeB-r17) | FR1 FDD | PDSCH | Clause 5.2.2.1.21 Clause 5.2.3.1.20 |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.22 Clause 5.2.3.2.21 |  |
| Baseline SU-MIMO 8Rx receiver | FR1 FDD | PDSCH | Clause 5.2.4.1.1(Tests 1-1,3-1) |  |
|  | FR1 TDD | PDSCH | Clause 5.2.4.2.1(Tests 1-1,3-1) |  |
| Simplified SU-MIMO 8Rx receiver | FR1 FDD | PDSCH | Clause 5.2.4.1.1(Tests 2-1,4-1) |  |
|  | FR1 TDD | PDSCH | Clause 5.2.4.2.1(Tests 2-1,4-1) |  |

<Unchanged part skipped>

5.1.1.5.4	Applicability of different requirements for HST

The applicability rules for different HST requirements in section 5 are specified in Table 5.1.1.5.4-1.

Table 5.1.1.5.4-1: Applicability of requirements for HST

| If UE has passed |  |  | UE can skip |  |  | Applicability notes |
| --- | --- | --- | --- | --- | --- | --- |
| Test type |  | Test list | Test type |  | Test list |  |
| FR1 2Rx | PDSCH | Clause 5.2A.2.4 (Test 3), and Clause 5.2A.2.5 (Test 1-2) | FR1 2Rx | PDSCH | Clause 5.2A.2.4 (Test 2), and Clause 5.2A.2.5 (Test 1-3) |  |
| FR1 2Rx | PDSCH | Clause 5.2A.2.4 (Test 1), and Clause 5.2A.2.5 (Test 1-2) | FR1 2Rx | PDSCH | Clause 5.2A.2.4 (Test 2), and Clause 5.2A.2.5 (Test 1-1) |  |
| FR1 4Rx | PDSCH | Clause 5.2A.3.4 (Test 3), and Clause 5.2A.3.5 (Test 1-2) | FR1 4Rx | PDSCH | Clause 5.2A.3.4 (Test 2), and Clause 5.2A.3.5 (Test 1-3), |  |
| FR1 4Rx | PDSCH | Clause 5.2A.3.4 (Test 1), and Clause 5.2A.3.5 (Test 1-2) | FR1 4Rx | PDSCH | Clause 5.2A.3.4 (Test 2), and Clause 5.2A.3.5 (Test 1-1), |  |
| FR1 2Rx | PDSCH | Clause 5.2A.2.5 (Test 1-3) | FR1 2Rx | PDSCH | Clause 5.2A.2.5 (Test 1-1) |  |
| FR1 2Rx | PDSCH | Clause 5.2A.2.4 (Test 3) | FR1 2Rx | PDSCH | Clause 5.2A.2.4 (Test 1) |  |
| FR1 4Rx | PDSCH | Clause 5.2A.3.5 (Test 1-3) | FR1 4Rx | PDSCH | Clause 5.2A.3.5 (Test 1-1) |  |
| FR1 4Rx | PDSCH | Clause 5.2A.3.4 (Test 3) | FR1 4Rx | PDSCH | Clause 5.2A.3.4 (Test 1) |  |
| FR1 2Rx | PDSCH | Clause 5.2A.2.4 (Test 3) | FR1 2Rx | PDSCH | Clause 5.2.2.1.9 (Test 1-1), Clause 5.2.2.2.1 (Test 1-11), Clause 5.2.2.2.9 (Test 1-1) |  |
| FR1 2Rx | PDSCH | Clause 5.2A.2.5 (Test 1-3) | FR1 2Rx | PDSCH | Clause 5.2.2.1.10 (Test 1-1), Clause 5.2.2.2.1 (Test 1-11), Clause 5.2.2.2.10 (Test 1-1) |  |
| FR1 2Rx | PDSCH | Clause 5.2A.2.4 (Test 1) | FR1 2Rx | PDSCH | Clause 5.2.2.1.9 (Test 1-1) |  |
| FR1 2Rx | PDSCH | Clause 5.2A.2.5 (Test 1-1) | FR1 2Rx | PDSCH | Clause 5.2.2.1.10 (Test 1-1) |  |
| FR1 2Rx | PDSCH | Clause 5.2A.2.4 (Test 2) | FR1 2Rx | PDSCH | Clause 5.2.2.2.1 (Test 1-11), Clause 5.2.2.2.9 (Test 1-1) |  |
| FR1 2Rx | PDSCH | Clause 5.2A.2.5 (Test 1-2) | FR1 2Rx | PDSCH | Clause 5.2.2.2.1 (Test 1-11), Clause 5.2.2.2.10 (Test 1-1) |  |
| FR1 4Rx | PDSCH | Clause 5.2A.3.4 (Test 3) | FR1 4Rx | PDSCH | Clause 5.2.3.1.9 (Test 1-1), Clause 5.2.3.2.1 (Test 1-11), Clause 5.2.3.2.9 (Test 1-1) |  |
| FR1 4Rx | PDSCH | Clause 5.2A.3.5 (Test 1-3) | FR1 4Rx | PDSCH | Clause 5.2.3.1.10 (Test 1-1), Clause 5.2.3.2.1 (Test 1-11), Clause 5.2.3.2.10 (Test 1-1) |  |
| FR1 4Rx | PDSCH | Clause 5.2A.3.4 (Test 1) | FR1 4Rx | PDSCH | Clause 5.2.3.1.9 (Test 1-1) |  |
| FR1 4Rx | PDSCH | Clause 5.2A.3.5 (Test 1-1) | FR1 4Rx | PDSCH | Clause 5.2.3.1.10 (Test 1-1) |  |
| FR1 4Rx | PDSCH | Clause 5.2A.3.4 (Test 2) | FR1 4Rx | PDSCH | Clause 5.2.3.2.1 (Test 1-11), Clause 5.2.3.2.9 (Test 1-1) |  |
| FR1 4Rx | PDSCH | Clause 5.2A.3.5 (Test 1-2) | FR1 4Rx | PDSCH | Clause 5.2.3.2.1 (Test 1-11), Clause 5.2.3.2.10 (Test 1-1) |  |
| FR1 2Rx | PDSCH | Clause 5.2A.2.5 (Test 2-1) | FR1 2Rx | PDSCH | Clause 5.2A.2.5 (Test 1-1) |  |
| FR1 2Rx | PDSCH | Clause 5.2A.2.5 (Test 2-2) | FR1 2Rx | PDSCH | Clause 5.2A.2.5 (Test 1-2) |  |
| FR1 2Rx | PDSCH | Clause 5.2A.2.5 (Test 2-3) | FR1 2Rx | PDSCH | Clause 5.2A.2.5 (Test 1-3) |  |
| FR1 4Rx | PDSCH | Clause 5.2A.3.5 (Test 2-1) | FR1 4Rx | PDSCH | Clause 5.2A.3.5 (Test 1-1) |  |
| FR1 4Rx | PDSCH | Clause 5.2A.3.5 (Test 2-2) | FR1 4Rx | PDSCH | Clause 5.2A.3.5 (Test 1-2) |  |
| FR1 4Rx | PDSCH | Clause 5.2A.3.5 (Test 2-3) | FR1 4Rx | PDSCH | Clause 5.2A.3.5 (Test 1-3) |  |
| FR1 2Rx | PDSCH | Clause 5.2A.2.4 (Test 3), and Clause 5.2A.2.5 (Test 2-2) | FR1 2Rx | PDSCH | Clause 5.2A.2.4 (Test 2), and Clause 5.2A.2.5 (Test 2-3) |  |
| FR1 2Rx | PDSCH | Clause 5.2A.2.4 (Test 1), and Clause 5.2A.2.5 (Test 2-2) | FR1 2Rx | PDSCH | Clause 5.2A.2.4 (Test 2), and Clause 5.2A.2.5 (Test 2-1) |  |
| FR1 4Rx | PDSCH | Clause 5.2A.3.4 (Test 3), and Clause 5.2A.3.5 (Test 2-2) | FR1 4Rx | PDSCH | Clause 5.2A.3.4 (Test 2), and Clause 5.2A.3.5 (Test 2-3), |  |
| FR1 4Rx | PDSCH | Clause 5.2A.3.4 (Test 1), and Clause 5.2A.3.5 (Test 2-2) | FR1 4Rx | PDSCH | Clause 5.2A.3.4 (Test 2), and Clause 5.2A.3.5 (Test 2-1), |  |
| FR1 2Rx | PDSCH | Clause 5.2A.2.5 (Test 2-3) | FR1 2Rx | PDSCH | Clause 5.2A.2.5 (Test 2-1) |  |
| FR1 4Rx | PDSCH | Clause 5.2A.3.5 (Test 2-3) | FR1 4Rx | PDSCH | Clause 5.2A.3.5 (Test 2-1) |  |
| FR1 2Rx | PDSCH | Clause 5.2A.2.5 (Test 2-3) | FR1 2Rx | PDSCH | Clause 5.2.2.1.10 (Test 1-1), Clause 5.2.2.2.1 (Test 1-11), Clause 5.2.2.2.10 (Test 1-1), Clause 5.2.2.1.10 (Test 1-2), Clause 5.2.2.2.10 (Test 1-2) |  |
| FR1 2Rx | PDSCH | Clause 5.2A.2.5 (Test 2-1) | FR1 2Rx | PDSCH | Clause 5.2.2.1.10 (Test 1-1), Clause 5.2.2.1.10 (Test 1-2) |  |
| FR1 2Rx | PDSCH | Clause 5.2A.2.5 (Test 2-2) | FR1 2Rx | PDSCH | Clause 5.2.2.2.1 (Test 1-11), Clause 5.2.2.2.10 (Test 1-1), Clause 5.2.2.2.10 (Test 1-2) |  |
| FR1 4Rx | PDSCH | Clause 5.2A.3.5 (Test 2-3) | FR1 4Rx | PDSCH | Clause 5.2.3.1.10 (Test 1-1), Clause 5.2.3.2.1 (Test 1-11), Clause 5.2.3.2.10 (Test 1-1), Clause 5.2.3.1.10 (Test 1-2), Clause 5.2.3.2.10 (Test 1-2) |  |
| FR1 4Rx | PDSCH | Clause 5.2A.3.5 (Test 2-1) | FR1 4Rx | PDSCH | Clause 5.2.3.1.10 (Test 1-1), Clause 5.2.3.1.10 (Test 1-2) |  |
| FR1 4Rx | PDSCH | Clause 5.2A.3.5 (Test 2-2) | FR1 4Rx | PDSCH | Clause 5.2.3.2.1 (Test 1-11), Clause 5.2.3.2.10 (Test 1-1), Clause 5.2.3.2.10 (Test 1-2) |  |

5.1.1.5.5	Applicability and test rules for different CA configurations and bandwidth combination sets for 8Rx UE

The performance requirement for CA UE demodulation tests for 8Rx UE in Clause 5.2A.4 are defined independent of CA configurations and bandwidth combination sets specified in Clause 5.5A of TS 38.101-1. For 8Rx UEs supporting different CA configurations and bandwidth combination sets, the applicability and test rules are defined in Table 5.1.1.5.5-1 and Table 5.1.1.5.5-2. For simplicity, CA configuration below refers to combination of CA configuration and bandwidth combination set.

Table 5.1.1.5.5-1: Applicability and test rules for CA UE demodulation tests

| Tests | CA capability where the tests apply | CA configuration from the selected CA capability where the tests apply | CA Bandwidth combination to be tested in priority order | PCell CC configuration |
| --- | --- | --- | --- | --- |
| [Test 1 in Clause 5.2A.2.1, 5.2A.3.1 and 5.2A.4.1] | CA_C, CA_N, CA_AX | Table 5.1.1.5.5-2 | Largest aggregated CA bandwidth combination | Any of CCs |
| [Test 2 in Clause 5.2A.2.1, 5.2A.3.1and 5.2A.4.1] | CA_C, CA_N, CA_AX | Table 5.1.1.5.5-2 | Largest aggregated CA bandwidth combination | Any of CCs |
| [Test 3 in Clause 5.2A.2.1, 5.2A.3.1and 5.2A.4.1] | CA_AX | Table 5.1.1.5.5-2 | Largest aggregated CA bandwidth combination | TDD CC if supported, otherwise FDD CC |
| NOTE 1: In case CA_AX with different number of X is supported then one or two CA configurations are selected based on procedure from Table 5.1.1.5.5-2. |  |  |  |  |

Table 5.1.1.5.5-2: Selection of CA configurations

| CA capability | Step 1 | Step 2 | Step 3 | Step 4 |
| --- | --- | --- | --- | --- |
| CA_C or CA_N | If the UE supports CA configuration that for each CC, supported maximum number of Rx and maximum number of MIMO layers is 8: Select the CA configurations with the maximum number of CCs, for which the supported maximum number of Rx and MIMO layers is 8. Otherwise: Select the CA configurations with the maximum number of CCs, conditioned that at least for one CC the supported maximum number of Rx is 8 and for each CC the supported maximum number of MIMO layers is not lower than 2 | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 1. | N/A | N/A |
| CA_AX | If the UE supports CA configuration that for each CC, supported maximum number of Rx and maximum number of MIMO layers is 8: Select the CA configurations with the maximum number of CCs, for which the supported maximum number of Rx and MIMO layers is 8. Otherwise: Select the CA configurations with the maximum number of CCs, conditioned that at least for one CC the supported maximum number of Rx is 8 and for each CC the supported maximum number of MIMO layers is not lower than 2 | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 1. | If the tested UE support CA configuration that for each CC, supported maximum number of Rx and maximum number of MIMO layers is 8: Select the CA configurations with the largest number of bands and with the maximum number of CCs, for which the supported maximum number of Rx and MIMO layers is 8. Otherwise: Select the CA configurations with the largest number of bands and with the maximum number of CCs, conditioned that at least for one CC the supported maximum number of Rx is 8 and for each CC the supported maximum number of MIMO layers is not lower than 2 | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 3. |
| NOTE 1: For CA_AX capability, if CA configuration from step 2 is CA configuration with the largest number of bands then Step 3 and Step 4 are skipped. Otherwise, the two CA configurations selected from Step 2 and Step 4 are used for testing. NOTE 2: Maximum supported data rate for Step 2 and Step 4 is calculated based clause 4.1.2 of TS 38.306 [14]. NOTE 3: Tested data rate for Step 2 and Step 4 is calculated based on the equation DataRate=10-3j=1JTBSj2μj and FRCs used in the test. |  |  |  |  |

5.1.1.5.6	Applicability rule and antenna connection for CA tests with 8 RX

Within the CA configuration if any of the PCell and/or the SCells is a 4Rx supported RF band, 4 out of the 8Rx should be connected with data source from system simulator, depending on UE’s declaration and AP configuration. Requirements from Clause 5.2A.3.1 are applied.

Within the CA configuration if any of the PCell and/or the SCells is a 2Rx supported RF band, 2 out of the 8Rx should be connected with data source from system simulator, depending on UE’s declaration and AP configuration. Requirements from Clause 5.2A.2.1 are applied.

Within the CA configuration if any of the PCell and/or the SCells is a 8Rx supported RF band, all 8Rx should be connected with data source from system simulator. Requirements from Clause5.2A.4.1 are applied.

For 8Rx capable UEs, the 2Rx supported RF bands, 4Rx supported RF bands and 8Rx supported RF bands are up to UE’s declaration.

5.1.1.5.7	Applicability rule for optional UE features for 8Rx

The performance requirements in Table 5.1.1.5.7-1 shall apply for UEs which support optional UE features only.

Table 5.1.1.5.7-1: Requirements applicability for optional UE features

| UE feature/capability [14] | Test type | Test list | Applicability notes |
| --- | --- | --- | --- |
| Baseline SU-MIMO 8Rx receiver | PDSCH | Tests listed in Table 5.2A.4.1-7 |  |
| Simplified SU-MIMO 8Rx receiver | PDSCH | Tests listed in Table 5.2A.4.1-8 |  |

<End of change>