3GPP TSG-RAN5 Meeting #104	R5-244234

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | 0660 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to MAC PICS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to TS 38.306 and TS 38.331, PICS A.4.3.5-1/10 shall be Rel-15 MAC capability. Therefore, the PICS release and Mnemonic should be updated accordingly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Release and Mnemonic of A.4.3.5-1/10 were corrected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect release will be defined for this PICS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex A.4.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.523-1 CR 4500, 4508, 4509 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

A.4.3.5	MAC Implementation Capabilities

Table A.4.3.5-1: UE MAC Implementation Capabilities

| Item | UE MAC Implementation Capabilities | Ref. | Release | Mnemonic | M | If indicated "Yes" the feature shall be implemented and successfully tested for the corresponding release | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Support long DRX cycle | 38.306, 4.2.6 | Rel-15 | pc_longDRX_Cycle | Yes |  |  |
| 2 | Support short DRX cycle | 38.306, 4.2.6 | Rel-15 | pc_shortDRX_Cycle | Yes |  |  |
| 3 | Support skipping of UL transmission for an uplink grant indicated on PDCCH if no data is available for transmission | 38.306, 4.2.6 | Rel-15 | pc_skipUplinkTxDynamic | No |  |  |
| 4 | Supports the logicalChannelSR-DelayTimer | 38.306, 4.2.6 | Rel-15 | pc_logicalChannelSR_DelayTimer | No |  |  |
| 5 | Supports DRX adaptation | 38.306, 4.2.6 | Rel-16 | pc_DRX_Adaptation | No |  |  |
| 6 | Support LCH-based prioritization | 38.306, 4.2.6 | Rel-16 | pc_lch_PriorityBasedPrioritization_r16 | No |  |  |
| 7 | Supports autonomous transmission of the MAC PDU generated for a deprioritized configured uplink grant | 38.306, 4.2.6 | Rel-16 | pc_autonomousTransmission_r16 | No |  |  |
| 8 | Supports the bit rate recommendation message from the gNB to the UE as specified in TS 38.321 | 38.306, 4.2.6 | Rel-15 | pc_recommendedBitRate | No |  |  |
| 9 | Supports the bit rate recommendation query message from the UE to the gNB as specified in TS 38.321. | 38.306, 4.2.6 | Rel-15 | pc_recommendedBitRateQuery | No |  | This field is only applicable if the UE supports pc_recommendedBitRate. |
| 10 | Support PUSCH transmissions on multiple configured uplink grants | 38.306, 4.2.6 | Rel-15 | pc_multipleConfiguredGrants | No |  |  |
| 11 | Support the selection of logical channels for each UL grant based on RRC configured restriction | 38.306, 4.2.6 | Rel-15 | pc_lcp_Restriction | No |  |  |
| 12 | Support direct NR SCG SCell activation, as specified in TS 38.321, upon SCell addition and upon reconfiguration with sync of the SCG, both performed via an RRCReconfiguration message received via SRB3 or contained in an RRC(Connection)Reconfiguration message received via SRB1, as specified in TS 38.331 and TS 36.331 | 38.306, 4.2.6 | Rel-16 | pc_directSCG_SCellActivation_r16 | No |  | A UE indicating support of directSCG-SCellActivation-r16 shall indicate support of EN-DC or support of NGEN-DC as specified in TS 36.331 or support of NR-DC as specified in TS 38.331. |
| 13 | Support direct NR MCG SCell activation, as specified in TS 38.321, upon SCell addition, upon reconfiguration with sync of the MCG, as specified in TS 38.331. | 38.306, 4.2.6 | Rel-16 | pc_directMCG_SCellActivation_r16 | No |  |  |
| 14 | Support direct NR MCG SCell activation, as specified in TS 38.321, upon reception of an RRCResume message, as specified in TS 38.331. | 38.306, 4.2.6 | Rel-16 | pc_directMCG_SCellActivationResume_r16 | No |  |  |
| 15 | Support direct NR SCG SCell activation, as specified in TS 38.321: - upon reception of an RRCReconfiguration included in an RRCConnectionResume message, as specified in TS 38.331 and TS 36.331, if the UE indicates support of EN-DC or NGEN-DC, and support of resumeWithSCG-Config-r16 as specified in TS 36.331, - upon reception of an RRCReconfiguration included in an RRCResume message, as specified in TS 38.331, if the UE indicates support of NR-DC and of resumeWithSCG-Config-r16 as specified in TS 38.331. | 38.306, 4.2.6 | Rel-16 | pc_directSCG_SCellActivationResume_r16 | No |  | A UE indicating support of directSCG-SCellActivationResume-r16 shall indicate support of EN-DC or NGEN-DC and support of resumeWithSCG-Config-r16 as specified in TS 36.331 or indicate support of NR-DC and of resumeWithSCG-Config-r16 as specified in TS 38.331. |
| 16 | Support services with survival time requirement using configured grant resource and PDCP duplication, as specified in TS 38.321. | 38.306, 4.2.6 | Rel-17 | pc_survivalTime_r17 | No |  | A UE supporting this feature shall support pdcp-DuplicationMCG-orSCG-DRB or pdcp-DuplicationSplitDRB. A UE supporting this feature shall also support configuredUL-GrantType1-v1650 or configuredUL-GrantType2-v1650. |
| 17 | Support of UL LBT Failure Detection and Recovery | 38.306, 4.2.6 | Rel-16 | pc_UL_LBT_FailureDetectionRecovery_r16 | No |  | Applies to UEs supporting shared spectrum channel access (at least one of A.4.3.2-1/1 to A.4.3.2-2/5). |
| 18 | Support of disabled HARQ feedback for downlink transmission | 38.306, 4.2.6 | Rel-17 | pc_harq_FeedbackDisabled_r17 | No |  | A UE supporting this feature shall also indicate the support of nonTerrestrialNetwork-r17. |
| 19 | Support of triggering of SR when a TA report is triggered and there are no available UL-SCH resources | 38.306, 4.2.6 | Rel-17 | pc_sr_TriggeredBy_TA_Report_r17 | No |  | A UE supporting this feature shall also indicate the support of nonTerrestrialNetwork-r17. |
| 20 | Support of HARQ Mode B and the corresponding LCP restrictions for uplink transmission | 38.306, 4.2.6 | Rel-17 | pc_uplink_Harq_ModeB_r17 | No |  | A UE supporting this feature shall also indicate the support of nonTerrestrialNetwork-r17. |

[End of change]