3GPP TSG-RAN5 Meeting #104	R5-244293

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 3282 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial correction of Redcap inter-RAT CGI test case 16.6.5.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Typo in Test procedure |  |  |  |  |  |  |  |  |
| Summary of change: |  | Fixed typo in test procedure |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect test procedure |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.6.5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

16.6.5.4	NR SA FR1 Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA for 2 Rx UE

16.6.5.4.1	Test purpose

To verify the requirement for identification of a new CGI of E-UTRA cell with autonomous gaps in NR SA in TS 38.133 clause 9.4A.4.

16.6.5.4.2	Test applicability

This test applies to all types of NR RedCap UE with 2Rx from Release 17 onwards and supporting acquisition of CGI from neighbour E-UTRA cell using autonomous gap.

16.6.5.4.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.6.5.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.16.6.5.4.

16.6.5.4.4	Test description

Same as the test description given in clause 16.6.5.3.4

16.6.5.4.4.1	Initial conditions

Same as the initial conditions given in clause 16.6.5.3.4 with following exceptions:

-	Table 16.6.5.3.4.1-1 is replaced by Table 16.6.5.4.4.1-1;

-	Table 16.6.5.3.4.1-2 is replaced by Table 16.6.5.4.4.1-2;

-	Table 16.6.5.3.4.1-3 is replaced by Table 16.6.5.4.4.1-3.

Table 16.6.5.4.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.6.5.4-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE FDD |
| 16.6.5.4-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE FDD |
| 16.6.5.4-3 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode, LTE FDD |
| 16.6.5.4-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE TDD |
| 16.6.5.4-5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE TDD |
| 16.6.5.4-6 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode, LTE TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 16.6.5.4.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.6.5.4.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

Table 16.6.5.4.4.1-3: General test parameters

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | 1 NR carrier frequency is used in the test |
| LTE RF Channel Number |  | 2 | 1 LTE carrier frequency is used in the test |
| Active cell |  | Cell 1 |  |
| Neighbour cell |  | Cell 2 | Cell to be identified. |
| LTE Channel Bandwidth | MHz | 10 |  |
| LTE PDSCH/PCFICH/PDCCH/PHICH parameters |  |  | As specified in clause A.6.1.1 |
| CP length |  | Normal |  |
| Hysteresis | dB | 0 |  |
| Time To Trigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF |  |
| useAutonomousGaps |  | setup | As specified in clause 5.5.3.1 in TS 38.331[13]. |
| Time offset between cells | ms | 3 | Asynchronous cells |
| T1 | s | 5 |  |
| T2 | s | £10 |  |
| T3 | s | 5 |  |

16.6.5.4.4.2	Test procedure

Same as the test procedure given in clause 16.6.5.3.4.2 with following exceptions:

-	Step 12 in clause 16.6.5.3.4.2 is replaced by following step:

12.	If the UE

a)	transmits a MeasurementReport message containing CGI of E-UTRA Cell 2 less than 200 ms from the beginning of time period T3, and

b)	transmits ACK/NACKs no less than the minimum number of ACK/NACKs given in Table 16.6.5.4.5-5 from the start of T3 until 200 ms,

then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

16.6.5.4.4.3	Message contents

Same as the message contents given in clause 16.6.5.3.4.3.

16.6.5.4.5	Test requirements

Table 16.6.5.4.4.1-3, Table 16.6.5.4.5-1 and Table 16.6.5.4.5-2 define the primary level settings including test tolerances for the test.

Table 16.6.5.4.5-3: NR cell specific test parameters (Cell 1)

| Parameter |  | Unit | Configuration | Cell 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 |
| RF channel number |  |  | 1,2,3,4,5,6 | 1 |  |  |
| Duplex mode |  |  | 1,4 | FDD |  |  |
|  |  |  | 2,3,5,6 | TDD |  |  |
| TDD Configuration |  |  | 2,5 | TDDConf.1.1 |  |  |
|  |  |  | 3,6 | TDDConf.2.1 |  |  |
| BWchannel |  | MHz | 1,2,4,5 | 10: NRB,c = 52 |  |  |
|  |  |  | 3,6 | 20: NRB,c = 51 |  |  |
| PDSCH reference measurement channel |  |  | 1,4 | SR.1.1 FDD |  |  |
|  |  |  | 2,5 | SR.1.1 TDD |  |  |
|  |  |  | 3,6 | SR.2.1 TDD |  |  |
| CORSET reference channel |  |  | 1,4 | CR.1.1 FDD |  |  |
|  |  |  | 2,5 | CR.1.1 TDD |  |  |
|  |  |  | 3,6 | CR.2.1 TDD |  |  |
| BWP configurations | Initial DL BWP |  | 1,2,3,4,5,6 | DLBWP.0.1 |  |  |
|  | Dedicated DL BWP |  | 1,2,3,4,5,6 | DLBWP.1.1 |  |  |
|  | Initial UL BWP |  | 1,2,3,4,5,6 | ULBWP.0.1 |  |  |
|  | Dedicated UL BWP |  | 1,2,3,4,5,6 | ULBWP.1.1 |  |  |
| OCNG patternNote1 |  |  | 1,2,3,4,5,6 | OP.1 |  |  |
| SMTC configuration |  |  | 1,2,3,4,5,6 | SMTC.1 |  |  |
| SSB configuration |  |  | 1,2,4,5 | SSB.1 FR1 |  |  |
|  |  |  | 3,6 | SSB.1 RedCap FR1 |  |  |
| b2-Threshold1 |  | dBm/SCS | 1,2,4,5 | -98 |  |  |
|  |  |  | 3,6 | -95 |  |  |
| b2-Threshold2EUTRA |  | dBm/15kHz | 1,2,3,4,5,6 | -109 |  |  |
| EPRE ratio of PSS to SSS |  | dB | 1,2,3,4,5,6 | 0 |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |
| NocNote2 |  | dBm/15 KHz | 1,2,3,4,5,6 | -106 |  |  |
| NocNote2 |  | dBm/SCS | 1,2,4,5 | -106 |  |  |
|  |  |  | 3,6 | -103 |  |  |
| Ês/Noc |  | dB | 1,2,3,4,5,6 | 18 | -2 | -2 |
| Ês/IotNote3 |  | dB | 1,2,3,4,5,6 | 18 | -2 | -2 |
| SS-RSRPNote3 |  | dBm/SCS | 1,2,4,5 | -88 | -108 | -108 |
|  |  |  | 3,6 | -85 | -105 | -105 |
| SSB_RPNote3 |  | dBm/SCS | 1,2,4,5 | -88 | -108 | -108 |
|  |  |  | 3,6 | -85 | -105 | -105 |
| IoNote3 |  | dBm/9.36 MHz | 1,2,4,5 | -59.98 | -75.92 | -75.92 |
|  |  | dBm/18.36 MHz | 3,6 | -57.05 | -72.99 | -72.99 |
| Propagation condition |  |  | 1,2,3,4,5,6 | AWGN |  |  |
| Antenna Configuration and Correlation Matrix |  |  | 1,2,3,4,5,6 | 1x2 Low |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Ês/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table 16.6.5.4.5-4: E-UTRA cell specific test parameters (Cell 2)

| Parameter | Unit | Configuration | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| RF channel number |  | 1,2,3,4,5,6 | 2 |  |  |
| Duplex mode |  | 1,2,3 | FDD |  |  |
|  |  | 4,5,6 | TDD |  |  |
| TDD special subframe configurationNote1 |  | 4,5,6 | 6 |  |  |
| TDD uplink-downlink configurationNote1 |  | 4,5,6 | 1 |  |  |
| BWchannel | MHz | 1,2,3,4,5,6 | 10 MHz: NRB,c = 50 |  |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 1,2,3 | 10 MHz: R.3 FDD |  |  |
|  |  | 4,5,6 | 10 MHz: R.0 TDD |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 1,2,3 | 10 MHz: R.6 FDD |  |  |
|  |  | 4,5,6 | 10 MHz: R.6 TDD |  |  |
| OCNG PatternsNote2 |  | 1,2,3 | 10 MHz: OP.10 FDD |  |  |
|  |  | 4,5,6 | 10 MHz: OP.1 TDD |  |  |
| PBCH_RA | dB | 1,2,3,4,5,6 | 0 |  |  |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |  |
| NocNote4 | dBm/15kHz | 1,2,3,4,5,6 | -106 |  |  |
| Ês/Noc | dB | 1,2,3,4,5,6 | -Infinity | 7 | 7 |
| Ês/IotNote5 | dB | 1,2,3,4,5,6 | -Infinity | 7 | 7 |
| RSRPNote5 | dBm/15kHz | 1,2,3,4,5,6 | -Infinity | -99 | -99 |
| SCH_RPNote5 | dBm/15kHz | 1,2,3,4,5,6 | -Infinity | -99 | -99 |
| Propagation Condition |  | 1,2,3,4,5,6 | AWGN |  |  |
| Antenna Configuration and Correlation Matrix |  | 1,2,3,4,5,6 | 1x2 Low |  |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [24]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [23] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Ês/Iot, RSRP, and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

The UE shall transmit a measurement report containing the cell global identifier of Cell 2 within overall delay from the start of T3, where the overall delay is calculated as follows,

Overall delay = TRRC Processing+Tidentify_CGI_RedCap, E-UTRAN + additional margin + TTI insertion uncertainty

-	TRRC Processing = 15 ms;

-	Tidentify_CGI_RedCap, E-UTRAN = 150 ms, is the maximum allowed time for the UE to identify a new CGI of an E-UTRA cell;

-	TTI insertion uncertainty = 2 x TTIDCCH = 2ms, is the delay uncertainty when inserting the measurement report to the TTI of the uplink DCCH;

-	additional margin = 30 ms, is specified in 38.133 [6] clause 9.4A.4.2

Which gives overall delay = 15 + 150 + 30 + 2 = 197 ms, allow 200 ms.

From the start of T3 until 200 ms. The number of ACK/NACK being transmitted by the UE shall be no less than the minimum number of ACK/NACK given in Table 16.6.5.4.5-5.

The rate of correct events observed during repeated tests shall be at least 90%.

Table 16.6.5.4.5-5: Minimum number of ACK/NACKs transmitted by the UE during the overall delay

| Test configuration | Minimum number of transmitted ACK/NACKsNote 1 |
| --- | --- |
| 1,4 | 127 |
| 2,5 | 42 |
| 3,6 | 115 |
| NOTE 1: The overall ACK/NACK number is caused by two parts. Firstly, at least X ACK/NACK shall be sent during identifying the cell global identifier of cell 2, where X is defined in 38.133 [6] Table 9.4A.4.3-1. Secondly, given that continuous DL data allocation, additional 43, 14 and 34 ACK/NACK shall be sent for FDD 15 kHz, TDD 15 kHzand TDD 30 kHz, respectively, from the start of T3 until 200 ms excludes 150 ms for identifying the cell global identifier of cell 2. |  |

<<< END OF CHANGES >>>