3GPP TSG-RAN5 Meeting #104	R5-244340

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-3 | CR | 3046 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to setup for SIB31 and SIB31-NB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Mediatek India Technology Pvt. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18_Test, LTE_NBIOT_eMTC_NTN_req-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Core spec alignment of NTN setup. (TS 36.133 i60 clause A.3.28) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update NTN related setup based on core spec. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RAN5 spec will remain not aligned with core spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.8a, A.10.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< START OF CHANGES 1>>>

A.8a.1	General setup for SIB31

The general parameters for SIB31 setup is specified in Table A.8a.1-1.

Table A.8a.1-1: General setup for SIB31

| Parameter | Unit | Test 1 |
| --- | --- | --- |
| Interval between adjacent epoch time | s | LEO: 2.56 GEO, GSO: 10.24 |
| ntn-UlSyncValidityDuration | s | LEO: 5s GEO, GSO: 900s |
| k-Offset-r17 | ms | LEO: 14 GEO, GSO: 258 |
| kmac | ms | Not configured |
| ta-Common |  | 0 |
| ta-CommonDrift |  | 0 |
| ta-CommonDriftVariant |  | 0 |
| ephemerisInfo |  | According to TS 36.133 [5] Annex B.8 |
| ta-Report |  | Not configured |

A.8a.2	Satellite specific configuration for serving cell

The general parameters for SIB31 setup for serving satellite are specified in Table A.8a.2-1.

Table A.8a.2-1: SIB31 parameters setup for Serving satellite

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference configuration for serving satellite |  | SSC.1 | SSC.2 |
| Scenario |  | GSO or GEO (as per test definition) | NGSO |
| Interval between adjacent epoch time | s | 10.24 | 2.56 |
| ul-SyncValidityDuration-r17 | s | 900 | 5 |
| k-Offset-r17 | ms | 258 | 14 |
| k-Mac-r17 | ms | Not configured | Not configured |
| nta-Common-r17 |  | 0 | 0 |
| nta-CommonDrift-r17 |  | 0 | 0 |
| nta-CommonDriftVariation-r17 |  | 0 | 0 |
| ephemerisInfo |  | According to TS 36.133 [5] Annex B.8 |  |

A.8a.3	Satellite specific configuration for neighbour cell

The general parameters for SIB33 setup for neighbor cells in both serving and neighbour satellites are specified in Table Table A.8a.3-1.

Table A.8a.3-1: SIB33 parameters setup for neighbour cell

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference configuration for satellite assistance information for neighbor cell |  | NSC.1 | NSC.2 | NSC.3 | NSC.4 |
| Scenario |  | GSO or GEO (as per test definition) | NGSO | GSO or GEO (as per test definition) | NGSO |
| Interval between adjacent epoch time | s | 10.24 | 2.56 | 10.24 | 2.56 |
| neighValidityDuration-r18 | s | 900 | 5 | 900 | 5 |
| k-Mac-r18 | ms | Not configured | Not configured | Not configured | Not configured |
| nta-Common-r18 |  | 0 | 0 | 0 | 0 |
| nta-CommonDrift-r18 |  | 0 | 0 | 0 | 0 |
| nta-CommonDriftVariation-r18 |  | 0 | 0 | 0 | 0 |
| ephemerisInfo |  | Same as used for serving satellite |  | According to TS 36.133 [5] Annex B.8 NOTE1 |  |
| t-ServiceStartNeigh-r18 |  | NOTE 2 |  |  |  |
| Note 1: The ephemeris information for those neighbor cells shall be different than that used for the serving cell, i.e., these cells are associated to a different satellite. Note 2: Unless otherwise stated in the test case, t-serviceStartNeigh-r18 is not configured for the neighbor satellite. |  |  |  |  |  |

<<< END OF CHANGES 1>>>

<<< START OF CHANGES 2>>>

A.10.4.1	General setup for SIB-31-NB

The general parameters for SIB31-NB setup is specified in Table A.10.4.1-1.

Table A.10.4.1-1: General setup for SIB31-NB

| Parameter | Unit | Test 1 |
| --- | --- | --- |
| Interval between adjacent epoch time | s | LEO: 2.56 GEO, GSO: 10.24 |
| ntn-UlSyncValidityDuration | s | LEO: 5s GEO, GSO: 900s |
| k-Offset-r17 | ms | LEO: 14 GEO, GSO: 258 |
| kmac | ms | Not configured |
| ta-Common |  | 0 |
| ta-CommonDrift |  | 0 |
| ta-CommonDriftVariant |  | 0 |
| ephemerisInfo |  | According to TS 36.133 [5] Annex B.8 |
| ta-Report |  | Not configured |

A.10.4.2	Satellite specific configuration for serving cell

The general parameters for SIB31-NB setup for serving satellite is specified in Table A.10.4.2-1.

Table A.10.4.2-1: SIB31-NB parameters setup for Serving satellite

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference configuration for serving satellite |  | SSC.1 | SSC.2 |
| Scenario |  | GSO or GEO (as per test definition) | NGSO |
| Interval between adjacent epoch time | s | 10.24 | 2.56 |
| ul-SyncValidityDuration-r17 | s | 900 | 5 |
| k-Offset-r17 | ms | 258 | 14 |
| k-Mac-r17 | ms | Not configured | Not configured |
| nta-Common-r17 |  | 0 | 0 |
| nta-CommonDrift-r17 |  | 0 | 0 |
| nta-CommonDriftVariation-r17 |  | 0 | 0 |
| ephemerisInfo |  | According to TS 36.133 [5] Annex B.8 |  |

A.10.4.3	Satellite specific configuration for neighbour cell

The general parameters for SIB33-NB setup for neighbor cells in both serving and neighbour satellites are specified in Table A.10.4.3-1.

Table A.10.4.3-1: SIB33-NB parameters setup for neighbour cell

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference configuration for satellite assistance information for neighbor cell |  | NSC.1 | NSC.2 | NSC.3 | NSC.4 |
| Scenario |  | GSO or GEO (as per test definition) | NGSO | GSO or GEO (as per test definition) | NGSO |
| Interval between adjacent epoch time | s | 10.24 | 2.56 | 10.24 | 2.56 |
| neighValidityDuration-r18 | s | 900 | 5 | 900 | 5 |
| k-Mac-r18 | ms | Not configured | Not configured | Not configured | Not configured |
| nta-Common-r18 |  | 0 | 0 | 0 | 0 |
| nta-CommonDrift-r18 |  | 0 | 0 | 0 | 0 |
| nta-CommonDriftVariation-r18 |  | 0 | 0 | 0 | 0 |
| ephemerisInfo |  | Same as used for serving satellite |  | According to TS 36.133 [5] Annex B.8 NOTE1 |  |
| t-ServiceStartNeigh-r18 |  | NOTE 2 |  |  |  |
| Note 1: The ephemeris information for those neighbor cells shall be different than that used for the serving cell, i.e., these cells are associated to a different satellite. Note 2: Unless otherwise stated in the test case, t-serviceStartNeigh-r18 is not configured for the neighbor satellite. |  |  |  |  |  |

<<< END OF CHANGES 2>>>