3GPP TSG-RAN5 Meeting #104	R5-244347

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

Title:	Discussion on CQI report limitation with Table 4

Source:	MediaTek Inc

Agenda item:	5.4.20

Document for:	Discussion and endorsement

1.	Introduction

In current TS 38.521-4, the Median CQI value 1 and 15 are not taken into consideration. It’s reasonable for legacy CQI reporting cases since the SNR condition is common. However, CQI 15 is possibly reported by UE in 1024 QAM TCs.

38.521-4 4. If Median CQI is not equal to 1 or 15 and [1800] or more of the wideband CQI values are in the range (Median CQI - 1) ≤ Median CQI ≤ (Median CQI + 1) then continue with step 5, otherwise go to step 8.

This paper is aiming to refine the test procedure for 1024 QAM CQI report TCs.

2. 	Discussion

As discussed in RAN4 agreed CR R4-2408996 [1]. The SNR condition of 1024 QAM CQI reporting cases are high. UE may have a chance to report CQI 15.

For CQI reporting with 1024QAM Table 4, the corresponding CQI index is 14 and 15 as per TS 38.214 Table 5.2.2.1-5: 4-bit CQI Table 4. The SNR values are 28/29dB and 25/26dB for 2Rx and 4Rx respectively for CQI reporting definition test cases for 1024QAM under AWGN condition. As per approved WF R4-2207254 and R4-2210651, with the assumption of Tx EVM 2.5%, the selected SNR values of 28/29dB for 2Rx and 25/26dB for 4Rx correspond to CQI index 14 for 1024QAM table. But in real testing, if the Tx EVM of the TE is smaller than 2.5%, the selected SNR values can correspond to CQI index 15 for 1024QAM table, i.e. UE will report medium CQI index 15 with BLER <=0.1 during the test. As per Table A.4-5, TBS.5-1 and TBS.5-2, the corresponding MCS is the highest MCS 26 for 1024QAM table for CQI index 15. According to TS38.214 section 5.2.2.1, if the block error rate of transport block corresponding to the reported CQI index is within 0.1, the UE should be regarded as a conformant UE. The existing test metric requests the BLER corresponding to (median CQI +1) shall be greater than 0.1, if the BLER of the transport block for the reported median CQI is less than or equal to 0.1. This is inconsistent with test purpose, and this is also the reason that RAN4 usually doesn’t select SNR values for CQI test correspods to the boundary CQI index.

Observation 1: UE may report medium CQI index 15 with BLER <=0.1 during 1024 QAM tests.

In last RAN5 meeting, similar situation in DCQR test cases were discussed and agreed [2]. With proper SNR condition, if UE report CQI 15 and the PDSCH bler are lower than 0.1. It means the UE has good demodulation performance. However, this kind of UE will fail this test in current spec.

2.1.2 Unconfigurable R The other issue in DCQR cases is sometimes the NW can’t send the requested R to UE in conformance test. For example - Assume the test configuration is normal coverage, Rmax=16. - If UE report R=4 to the SS in test step 3 in 36.521-3 [1], the SS shall send NPDCCHs with repetition level 4 and 4/4 (4/8 in enhanced coverage) = 1 based on test step 4 and 6 respectively. - According to 36.213 [5], when Rmax >=8, the repetition indicator in DCI only has 2 bits, in this case, is Rmax =16, Rmax/2 =8, Rmax/4 =4 and Rmax/8 =2. This information will be used when UE does the blind detection. The problem is SS can’t send NPDCCHs with repetition 1. Test step 6 are not testable. UE report lower R basically means this UE has good demodulation performance. And in this case, this kind of good UE are not testable in step 6. Observation 3: Issue 2: A good UE may not be testable due to the limitation of R configuration IE.

Observation 2: A good UE may not be testable in 1024 QAM CQI reporting test cases.

It’s suggested to add extra judgement in test procedure. For example,

4.	If Median CQI is not equal to 1 or 15 and [1800] or more of the wideband CQI values are in the range (Median CQI - 1) ≤ Median CQI ≤ (Median CQI + 1) then continue with step 5, otherwise go to step 8.

5.	The SS shall transmit PDSCH via PDCCH DCI format [1_1] for C_RNTI to transmit the DL RMC according to the wideband median-CQI value and shall not react to the UE’s wideband CQI reports. The SS sends downlink MAC padding bits on the DL RMC. For any PDSCH transmitted by the SS, record the associated ACK, NACK and statDTX responses. The responses are then filtered as follows: for the sequence of responses for each HARQ process, discard all the statDTX responses. Continue to gather data until the number of filtered ACK+NACK responses reaches 1000.

For the filtered ACK and NACK responses if the ratio (NACK / ACK + NACK) ≤ 0.1 and the Median CQI is not equal to 15 then go to step 6, if the ratio (NACK / ACK + NACK) ≤ 0.1 and the Median CQI is equal to 15 then pass the UE for this test. Otherwise go to step 7.

Following test cases are impacted,

6.2.2.1.1.3	2Rx FDD FR1 periodic CQI reporting with Table 4 under AWGN conditions for both SA and NSA

6.2.2.2.1.4	2Rx TDD FR1 periodic CQI reporting with Table 4 under AWGN conditions for both SA and NSA

6.2.3.1.1.3	4Rx FDD FR1 periodic CQI reporting with Table 4 under AWGN conditions for both SA and NSA

6.2.3.2.1.4	4Rx TDD FR1 periodic CQI reporting with Table 4 under AWGN conditions for both SA and NSA

Proposal 1: Add extra test judgement in test steps for CQI reporting test cases with Table 4.

3. 	Conclusion

This paper is aiming to refine the test procedure for 1024 QAM CQI report TCs.

The following observations and proposals are made:

Observation 1: UE may report medium CQI index 15 with BLER <=0.1 during 1024 QAM tests.

Observation 2: A good UE may not be testable in 1024 QAM CQI reporting test cases.

Proposal 1: Add extra test judgement in test steps for CQI reporting test cases with Table 4.

The associated CRs is R5-244348.

4. 	References

- R4-2408996 [NR_DL1024QAM_FR1-Perf] Updates to CQI reporting definition test metric under AWGN for 1024QAM test, Huawei, HiSilicon, RAN4 #111

- R5-242428 Discussion on NTN DCQR cases, MediaTek Inc, RAN5 #103