3GPP TSG-RAN5 Meeting #104	R5-244437

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.123-3 | CR | 3683 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Routine maintenance for TS 34.123-3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18_Test |  |  |  |  |  | Date: |  |  | 2024-07-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In order to develop the Rel-18 test cases in TTCN-3, the current baseline of type definitions shall be upgraded. RAN5 agreed to upgrade the baseline to June 2024 to all RAN5 TTCN-3 test suites. The ASN.1 used in the postioning test suite has been upgraded to Rel-18. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The ASN.1 has been updated to include the Rel-18 updates. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The positioning test suite will not support Rel-18 UEs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex H.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## H.2.4	LCS-Definitions-Arguments ASN.1 module

LCS-Definitions-Arguments DEFINITIONS  ::=

BEGIN

IMPORTS

AddressString,

maxAddressLength,

ExtensionContainer,

PrivateExtensionList,

PrivateExtension,

maxNumOfPrivateExtensions,

PCS-Extensions,

USSD-DataCodingScheme,

USSD-String,

maxUSSD-StringLength,

ISDN-AddressString

FROM Common-Definitions-Arguments;

--*****************************************************

--Derived from SS-DataTypes in 3GPP TS 24.080

--*****************************************************

LocationNotificationArg	::= SEQUENCE {

notificationType			[0] IMPLICIT NotificationToMSUser,

locationType				[1] IMPLICIT LocationType,

lcsClientExternalID			[2] IMPLICIT LCSClientExternalID	OPTIONAL,

lcsClientName				[3] IMPLICIT LCSClientName		OPTIONAL,

...,

lcsRequestorID				[4] IMPLICIT LCSRequestorID		OPTIONAL,

lcsCodeword					[5] IMPLICIT LCSCodeword			OPTIONAL,

lcsServiceTypeID			[6] IMPLICIT LCSServiceTypeID 	OPTIONAL ,

deferredLocationExt         [7] IMPLICIT DeferredLocationExt  OPTIONAL,

rangingSlExt                [8] IMPLICIT RangingSlExt         OPTIONAL

}

-- The notificationType may only be set to notifyLocationAllowed,

-- notifyAndVerify-LocationAllowedIfNoResponse,

-- or notifyAndVerify-LocationNotAllowedIfNoResponse.

-- The locationEstimateType field of the locationType may only be set to

-- currentLocation, currentOrLastKnownLocation,

-- notificationVerificationOnly, or activateDeferredLocation.

-- The deferredLocationEventType field of the locationType may only be set to

-- enteringIntoArea, and/or leavingFromArea, and/or beingInsideArea,

-- and/or periodicLDR.

-- For LCS location notification of MT-LR for current location, the

-- locationEstimateType field of the locationType shall be set to currentLocation.

-- For LCS location notification of MT-LR for current or last known location, the

-- locationEstimateType field of the locationType shall be set to currentOrLastKnownLocation.

-- For the LCS location notification for the LDR of MS available event,

-- the locationEstimateType field of the locationType shall be set to currentLocation.

-- For LCS location notification for the LDR of change of area event,

-- the locationEstimateType field of the locationType shall be set to

-- activateDeferredLocation, and the deferredLocationEventType shall be

-- set to enteringIntoArea, and/or leavingFromArea, and/or beingInsideArea.

-- For the post positioning LCS location notification, the locationEstimateType

-- field of the locationType shall be set to notificationVerificationOnly.

-- For LCS location notification for the LDR of periodic location event,

-- the locationEstimateType field of the locationType shall be set to

-- activateDeferredLocation, and the

-- deferredLocationEventType shall be set to periodicLDR.

-- exception handling:

-- At reception of an unrecognised notificationType value the receiver shall reject the

-- operation with a return error cause of unexpected data value.

-- At reception of an unrecognised locationType value the receiver shall reject the

-- operation with a return error cause of unexpected data value.

-- At reception of an unallowed notificationType value the receiver shall either ignore the

-- received operation or reject the operation with a return error cause of unexpected

-- data value.

-- At reception of an unallowed locationType value the receiver shall either ignore the

-- received operation or reject the operation with a return error cause of unexpected

-- data value.

DeferredLocationExt ::= BIT STRING {

motionEvent	(0) } (SIZE (1..16))

-- A bit set to one indicates that the corresponding deferred location type is present.

-- A bit set to zero indicates that the corresponding deferred location type is absent.

RangingSlExt ::= BIT STRING {

rangingIndication	(0) } (SIZE (1..16))

-- A bit set to one indicates that the notification is for ranging and sidelink positioning.

LocationNotificationRes  ::= SEQUENCE {

verificationResponse	[0] IMPLICIT VerificationResponse OPTIONAL ,

locationPrivacyIndication	[1] LCS-LocationPrivacyIndication	OPTIONAL,

validTimePeriod		[2] LCS-ValidTimePeriod	OPTIONAL,

...	}

VerificationResponse::= ENUMERATED {

permissionDenied (0),

permissionGranted (1),

...

}

-- exception handling:

-- an unrecognized value shall be treated the same as value 0 (permissionDenied)

LCS-LocationPrivacyIndication ::= ENUMERATED {

locationDisallowed (0),

locationAllowed (1),

...,

rangingDisallowed (2),

rangingAllowed (3)

}

LCS-ValidTimePeriod ::= SEQUENCE {

startTime	[0] DateTime	OPTIONAL,

endTime	[1] DateTime	OPTIONAL,

... }

DateTime ::= OCTET STRING (SIZE (30))   --  @sic R5s221182 Baseline Moving sic@

-- The timestamp at which certain event occurs. Octets are coded according to RFC 3339 [18]

LCS-MOLRArg	::= SEQUENCE {

molr-Type			[0] IMPLICIT	MOLR-Type,

locationMethod		[1] IMPLICIT	LocationMethod			OPTIONAL,

lcs-QoS				[2] IMPLICIT	LCS-QoS				OPTIONAL,

lcsClientExternalID	[3] IMPLICIT 	LCSClientExternalID		OPTIONAL,

mlc-Number			[4] IMPLICIT	ISDN-AddressString		OPTIONAL,

gpsAssistanceData	[5] IMPLICIT	GPSAssistanceData		OPTIONAL,

...,

supportedGADShapes	[6] IMPLICIT	SupportedGADShapes		OPTIONAL,

lcsServiceTypeID	[7] IMPLICIT LCSServiceTypeID		OPTIONAL,

ageOfLocationInfo	[8] IMPLICIT AgeOfLocationInformation	OPTIONAL,

locationType		[9] IMPLICIT LocationType			OPTIONAL,

pseudonymIndicator	[10] IMPLICIT NULL					OPTIONAL,

h-gmlc-address 		[11] IMPLICIT GSN-Address			OPTIONAL,

locationEstimate	[12] IMPLICIT Ext-GeographicalInformation		OPTIONAL,

velocityEstimate	[13] IMPLICIT VelocityEstimate-AGPS		OPTIONAL,

referenceNumber 	[14] IMPLICIT LCS-ReferenceNumber 	OPTIONAL,

periodicLDRInfo		[15] IMPLICIT PeriodicLDRInfo		OPTIONAL,

locationUpdateRequest	[16]	IMPLICIT NULL				OPTIONAL,

sequenceNumber 		[17] IMPLICIT SequenceNumber		OPTIONAL,

terminationCause 	[18] IMPLICIT TerminationCause 		OPTIONAL,

mo-lrShortCircuit	[19] IMPLICIT NULL				OPTIONAL,

ganssAssistanceData	[20] IMPLICIT GANSSAssistanceData	OPTIONAL,

multiplePositioningProtocolPDUs	[21] IMPLICIT MultiplePositioningProtocolPDUs	OPTIONAL,

locationInfo		[22] LocationInfo			OPTIONAL,

scheduledLocTime  [23] DateTime             OPTIONAL

}

-- The parameter locationMethod shall be included if and only if the

-- molr-Type is set to value deCipheringKeys or assistanceData.

-- The parameter gpsAssistanceData shall be included if and only if the

-- molr-Type is set to value assistanceData and

-- locationMethod is set to value assistedGPS or assistedGPSandGANSS.

-- The parameter ganssAssistanceData shall be included if and only if the molr-Type is set to value

-- assistanceData and locationMethod is set to value assistedGANSS or assistedGPSandGANSS.

-- supportedGADShapes shall not be included for deferred MO-LR initiation or deferred MO-LR or MT-LR

-- responses.

-- multiplePositioningProtocolPDUs may only be included for E-UTRAN access.

-- locationMethod shall not be included for E-UTRAN access.

-- gpsAssistanceData shall not be included for E-UTRAN access.

-- h-gmlc-address shall not be included for E-UTRAN access.

-- locationEstimate shall not be included for E-UTRAN access.

-- velocityEstimate shall not be included for E-UTRAN access.

-- referenceNumber shall not be included for E-UTRAN access.

-- periodicLDRInfo shall not be included for E-UTRAN access.

-- locationUpdateRequest shall not be included for E-UTRAN access.

-- sequenceNumber shall not be included for E-UTRAN access.

-- terminationCause shall not be included for E-UTRAN access.

-- mo-lrShortCircuit shall not be included for E-UTRAN access.

-- ganssAssistanceData shall not be included for E-UTRAN access.

-- locationInfo shall be included if available for E-UTRAN access for a periodic and triggered

-- event report.

-- Only mlc-number could be used to represent the address of H-GMLC of the UE, h-gmlc-address shall not be used for this purpose.

-- if mappedQoSEps was received in LCS-PeriodicTriggeredInvokeArg to initiate deferred location event reporting from 5GS, lcs-QoS shall be set to the value of the received mappedQoSEps when a correspoding event is to be reported in EPS.

MOLR-Type ::= ENUMERATED {

locationEstimate (0),

assistanceData (1),

deCipheringKeys (2),

...,

deferredMo-lrTTTPInitiation	(3),

deferredMo-lrSelfLocationInitiation (4),

deferredMt-lrOrmo-lrTTTPLocationEstimate (5),

deferredMt-lrOrmo-lrCancellation (6) ,

periodicEvent (7),

enteringAreaEvent (8),

leavingAreaEvent (9),

beingInsideAreaEvent (10),

motionEvent (11),

maximumIntervalExpirationEvent (12)

}

-- exception handling:

-- an unrecognized value shall be rejected by the receiver with a return error cause of

-- unexpected data value.

-- For E-UTRAN access, only locationEstimate, assistanceData, periodicEvent,

-- deferredMt-lrOrmo-lrCancellation, enteringAreaEvent, leavingAreaEvent, beingInsideAreaEvent,

-- motionEvent or maximumIntervalExpirationEvent shall be included

LocationInfo ::= BIT STRING {

locationEstimate	(0) } (SIZE (1..32))

-- A bit set to one indicates that the corresponding requirement is present.

-- A bit set to zero indicates that the corresponding requirement is absent.

-- The locationEstimate is a requirement to provide the UE location for an event report to

-- the external client, but the location may be provided by either the UE or network.

LocationMethod ::= ENUMERATED {

msBasedEOTD	(0),

msAssistedEOTD (1),

assistedGPS	(2),

...,

msBasedOTDOA	(3) ,

assistedGANSS	(4),

assistedGPSandGANSS	(5)

}

-- exception handling:

-- When this parameter is received with value msBasedEOTD or msAssistedEOTD and the MS

-- is camped on an UMTS Service Area then the receiver shall reject it

-- with a return error cause of unexpected data value.

-- When this parameter is received with value msBasedOTDOA and the MS

-- is camped on a GSM Cell then the receiver shall reject it with

-- a return error cause of unexpected data value.

-- an unrecognized value shall be rejected by the receiver with

-- a return error cause of unexpected data value.

GPSAssistanceData ::= OCTET STRING (SIZE (1..38))

-- Octets 1 to 38 are coded in the same way as the octets 3 to 7+2n

-- of Requested GPS Data IE in 3GPP TS 49.031.

GANSSAssistanceData::= OCTET STRING (SIZE (1..40))

-- Octets 1 to 40 are coded in the same way as the octets 3 to 9+2n of Requested GANSS Data IE

-- in 3GPP TS 49.031 [14] .

TerminationCause ::= ENUMERATED {

subscriberTermination  (0),

uETermination  (1),

... ,

normalTermination  (2),

networkTermination  (3)

}

-- normalTermination shall be included for a normal end to periodic or triggered event

-- reporting

MultiplePositioningProtocolPDUs ::= SEQUENCE (SIZE (1..maxNumLPPMsg)) OF PositioningProtocolPDU

PositioningProtocolPDU ::= OCTET STRING

-- PositioningProtocolPDU contains a LPP message defined in 3GPP TS 36.355 [17].

maxNumLPPMsg INTEGER ::= 3

LCS-MOLRRes ::= SEQUENCE {

locationEstimate		[0] IMPLICIT Ext-GeographicalInformation	OPTIONAL,

decipheringKeys			[1] IMPLICIT DecipheringKeys			OPTIONAL,

...,

add-LocationEstimate	[2] IMPLICIT Add-GeographicalInformation 	OPTIONAL,

velocityEstimate		[3] IMPLICIT VelocityEstimate-AGPS		OPTIONAL,

referenceNumber 		[4] IMPLICIT LCS-ReferenceNumber 	OPTIONAL,

h-gmlc-address			[5]	IMPLICIT GSN-Address				OPTIONAL,

mo-lrShortCircuit		[6]	IMPLICIT NULL					OPTIONAL,

reportingPLMNList		[7]	IMPLICIT ReportingPLMNList		OPTIONAL ,

timestampOfLocationEstimate	[8]	DateTime	                OPTIONAL

}

-- Parameters locationEstimate or add-LocationEstimate (one but not both)

-- shall be included if and only if the

-- molr-Type in LocationRequestArg was set to value locationEstimate.

-- Parameter add-LocationEstimate shall not be included

-- if the supportedGADShapes parameter was not received in the LCS-MOLRArg.

-- The locationEstimate and the add-locationEstimate parameters shall not be

-- sent if the supportedGADShapes parameter has been received in LCS-MOLRArg

-- and the shape encoded in locationEstimate or add-LocationEstimate

-- is not marked as supported in supportedGADShapes.

-- In such a case LCS-MOLRArg shall be rejected with error

-- FacilityNotSupported with additional indication

-- shapeOfLocationEstimateNotSupported.

-- Parameter decipheringKeys shall be included if and only if the molr-Type

-- in LocationRequestArg was set to value deCipheringKeys.

-- Parameter velocityEstimate may only be included if the lcs-QoS in LCS-MOLRarg includes

-- velocityRequest

DecipheringKeys ::= OCTET STRING (SIZE (15))

-- Octets in DecipheringKeys are coded in the same way as the octets 3 to 17

-- of Deciphering Key IE in 3GPP TS 49.031. I.e. these octets contain

-- Current Deciphering Key, Next Deciphering Key and Ciphering Key Flag.

--********************************************************

-- Derived from MAP-Errors 3GPP 29.002

--********************************************************

SystemFailureParam ::= CHOICE {

networkResource						NetworkResource,

-- networkResource must not be used in version 3

extensibleSystemFailureParam		ExtensibleSystemFailureParam

-- extensibleSystemFailureParam must not be used in version <3

}

NetworkResource ::= ENUMERATED {

plmn (0),

hlr (1),

vlr (2),

pvlr (3),

controllingMSC (4),

vmsc (5),

eir (6),

rss (7)

}

ExtensibleSystemFailureParam ::= SEQUENCE {

networkResource				NetworkResource		OPTIONAL,

extensionContainer			ExtensionContainer	OPTIONAL,

...,

additionalNetworkResource	[0] AdditionalNetworkResource	OPTIONAL,

failureCauseParam			[1] FailureCauseParam	OPTIONAL

}

AdditionalNetworkResource ::= ENUMERATED {

sgsn (0),

ggsn (1),

gmlc (2),

gsmSCF (3),

nplr (4),

auc (5),

...,

ue (6),

mme (7)

}

-- if unknown value is received in AdditionalNetworkResource

-- it shall be ignored.

FailureCauseParam ::= ENUMERATED {

limitReachedOnNumberOfConcurrentLocationRequests (0),

... }

-- if unknown value is received in FailureCauseParam it shall be ignored

UnexpectedDataParam ::= SEQUENCE {

extensionContainer		ExtensionContainer	OPTIONAL,

...,

unexpectedSubscriber 	[0]	 NULL			OPTIONAL

}

-- the unexpectedSubscriber indication in the unexpectedDataValue error shall not be used

-- for operations that allow the unidentifiedSubscriber error.

DataMissingParam ::= SEQUENCE {

extensionContainer		ExtensionContainer	OPTIONAL,

...

}

FacilityNotSupParam ::= SEQUENCE {

extensionContainer								ExtensionContainer	OPTIONAL,

...,

shapeOfLocationEstimateNotSupported 			[0] IMPLICIT NULL	OPTIONAL,

neededLcsCapabilityNotSupportedInServingNode 	[1] IMPLICIT NULL	OPTIONAL

}

SS-SubscriptionViolationParam ::= SEQUENCE {

extensionContainer		ExtensionContainer	OPTIONAL,

...

}

PositionMethodFailure-Param ::= SEQUENCE {

positionMethodFailure-Diagnostic	[0] IMPLICIT PositionMethodFailure-Diagnostic	OPTIONAL,

extensionContainer					[1] IMPLICIT ExtensionContainer			OPTIONAL,

...

}

PositionMethodFailure-Diagnostic ::= ENUMERATED {

congestion (0),

insufficientResources (1),

insufficientMeasurementData (2),

inconsistentMeasurementData (3),

locationProcedureNotCompleted (4),

locationProcedureNotSupportedByTargetMS (5),

qoSNotAttainable (6),

positionMethodNotAvailableInNetwork (7),

positionMethodNotAvailableInLocationArea (8),

...

}

-- exception handling:

-- any unrecognized value shall be ignored

--*******************************************************

-- From 3GPP TS 29.002

--*******************************************************

NotificationToMSUser ::= ENUMERATED {

notifyLocationAllowed (0),

notifyAndVerify-LocationAllowedIfNoResponse (1),

notifyAndVerify-LocationNotAllowedIfNoResponse (2),

...,

locationNotAllowed (3) }

-- exception handling:

-- At reception of any other value than the ones listed the receiver shall ignore

-- NotificationToMSUser.

LocationType ::= SEQUENCE {

locationEstimateType		[0] IMPLICIT LocationEstimateType,

...,

deferredLocationEventType	[1] IMPLICIT DeferredLocationEventType	OPTIONAL }

LocationEstimateType ::= ENUMERATED {

currentLocation (0),

currentOrLastKnownLocation (1),

initialLocation (2),

...,

activateDeferredLocation (3),

cancelDeferredLocation (4),

notificationVerificationOnly	(5)

}

-- exception handling:

-- a ProvideSubscriberLocation-Arg containing an unrecognized LocationEstimateType

-- shall be rejected by the receiver with a return error cause of unexpected data value

DeferredLocationEventType ::= BIT STRING {

msAvailable (0),

enteringIntoArea	(1),

leavingFromArea	(2),

beingInsideArea	(3) ,

periodicLDR	(4)

} (SIZE (1..16))

-- beingInsideArea is always treated as oneTimeEvent regardless of the possible value

-- of occurrenceInfo inside areaEventInfo.

-- exception handling:

-- a ProvideSubscriberLocation-Arg containing other values than listed above in

-- DeferredLocationEventType shall be rejected by the receiver with a return error cause of

-- unexpected data value.

LCSClientExternalID ::= SEQUENCE {

externalAddress		[0] IMPLICIT ISDN-AddressString	OPTIONAL,

extensionContainer	[1] IMPLICIT ExtensionContainer	OPTIONAL,

...

}

LCSClientName ::= SEQUENCE {

dataCodingScheme	[0] IMPLICIT USSD-DataCodingScheme,

nameString			[2] IMPLICIT NameString,

...,

lcs-FormatIndicator	[3] IMPLICIT LCS-FormatIndicator	OPTIONAL

}

-- The USSD-DataCodingScheme shall indicate use of the default alphabet through the following encoding

--	bit	7 6 5 4 3 2 1 0

--			0 0 0 0 1 1 1 1

NameString ::= USSD-String (SIZE (1..maxNameStringLength))

maxNameStringLength  INTEGER ::= 63

LCSRequestorID ::= SEQUENCE {

dataCodingScheme		[0] IMPLICIT USSD-DataCodingScheme,

requestorIDString		[1] IMPLICIT RequestorIDString,

...,

lcs-FormatIndicator		[2] IMPLICIT LCS-FormatIndicator	OPTIONAL

}

LCS-FormatIndicator ::= ENUMERATED {

logicalName		(0),

e-mailAddress	(1),

msisdn			(2),

url				(3),

sipUrl			(4),

... }

RequestorIDString ::= USSD-String (SIZE (1..maxRequestorIDStringLength))

maxRequestorIDStringLength  INTEGER ::= 63

LCSCodeword ::= SEQUENCE {

dataCodingScheme		[0] IMPLICIT USSD-DataCodingScheme,

lcsCodewordString		[1] IMPLICIT LCSCodewordString,

...

}

LCSCodewordString ::= USSD-String (SIZE (1..maxLCSCodewordStringLength))

maxLCSCodewordStringLength  INTEGER ::= 20

LCSServiceTypeID ::= INTEGER (0..127)

-- the integer values 0-63 are reserved for Standard LCS service types

-- the integer values 64-127 are reserved for Non Standard LCS service types

LCS-QoS ::= SEQUENCE {

horizontal-accuracy			[0] IMPLICIT Horizontal-Accuracy	OPTIONAL,

verticalCoordinateRequest	[1] IMPLICIT NULL		OPTIONAL,

vertical-accuracy			[2] IMPLICIT Vertical-Accuracy	OPTIONAL,

responseTime				[3] IMPLICIT ResponseTime	OPTIONAL,

extensionContainer			[4] IMPLICIT ExtensionContainer	OPTIONAL,

...,

velocityRequest				[5] IMPLICIT NULL		OPTIONAL,

lcs-qos-class	            [6] LCS-QoS-Class	OPTIONAL

}

-- lcs-qos-class may only be included in MO-LR request sent by the UE to the network.

Horizontal-Accuracy ::= OCTET STRING (SIZE (1))

-- bit 8 = 0

-- bits 7-1 = 7 bit Uncertainty Code defined in 3GPP TS 23.032. The horizontal location

-- error should be less than the error indicated by the uncertainty code with 67%

-- confidence.

Vertical-Accuracy ::= OCTET STRING (SIZE (1))

-- bit 8 = 0

-- bits 7-1 = 7 bit Vertical Uncertainty Code defined in 3GPP TS 23.032.

-- The vertical location error should be less than the error indicated

-- by the uncertainty code with 67% confidence.

ResponseTime ::= SEQUENCE {

responseTimeCategory		ResponseTimeCategory,

...

}

-- note: an expandable SEQUENCE simplifies later addition of a numeric response time.

ResponseTimeCategory ::= ENUMERATED {

lowdelay (0),

delaytolerant (1),

...}

-- exception handling:

-- an unrecognized value shall be treated the same as value 1 (delaytolerant)

SupportedGADShapes ::= BIT STRING {

ellipsoidPoint (0),

ellipsoidPointWithUncertaintyCircle (1),

ellipsoidPointWithUncertaintyEllipse (2),

polygon (3),

ellipsoidPointWithAltitude (4),

ellipsoidPointWithAltitudeAndUncertaintyElipsoid (5),

ellipsoidArc (6) } (SIZE (7..16))

-- A node shall mark in the BIT STRING all Shapes defined in 3GPP TS 23.032 it supports.

-- exception handling: bits 7 to 15 shall be ignored if received.

Ext-GeographicalInformation ::= OCTET STRING (SIZE (1..maxExt-GeographicalInformation))

-- Refers to geographical Information defined in 3GPP TS 23.032.

-- This is composed of 1 or more octets with an internal structure according to

-- 3GPP TS 23.032

-- Octet 1: Type of shape, only the following shapes in 3GPP TS 23.032 are allowed:

--		(a) Ellipsoid point with uncertainty circle

--		(b) Ellipsoid point with uncertainty ellipse

--		(c) Ellipsoid point with altitude and uncertainty ellipsoid

--		(d) Ellipsoid Arc

--		(e) Ellipsoid Point

-- Any other value in octet 1 shall be treated as invalid

-- Octets 2 to 8 for case (a)  Ellipsoid point with uncertainty circle

--		Degrees of Latitude				3 octets

--		Degrees of Longitude				3 octets

--		Uncertainty code				1 octet

-- Octets 2 to 11 for case (b)  Ellipsoid point with uncertainty ellipse:

--		Degrees of Latitude				3 octets

--		Degrees of Longitude				3 octets

--		Uncertainty semi-major axis				1 octet

--		Uncertainty semi-minor axis				1 octet

--		Angle of major axis				1 octet

--		Confidence				1 octet

-- Octets 2 to 14 for case (c)  Ellipsoid point with altitude and uncertainty ellipsoid

--		Degrees of Latitude				3 octets

--		Degrees of Longitude				3 octets

--		Altitude				2 octets

--		Uncertainty semi-major axis				1 octet

--		Uncertainty semi-minor axis				1 octet

--		Angle of major axis				1 octet

--		Uncertainty altitude				1 octet

--		Confidence				1 octet

-- Octets 2 to 13 for case (d)  Ellipsoid Arc

--		Degrees of Latitude				3 octets

--		Degrees of Longitude				3 octets

--		Inner radius				2 octets

--		Uncertainty radius				1 octet

--		Offset angle				1 octet

--		Included angle				1 octet

--		Confidence				1 octet

-- Octets 2 to 7 for case (e)  Ellipsoid Point

--		Degrees of Latitude				3 octets

--		Degrees of Longitude				3 octets

--

-- An Ext-GeographicalInformation parameter comprising more than one octet and

-- containing any other shape or an incorrect number of octets or coding according

-- to 3GPP TS 23.032 shall be treated as invalid data by a receiver.

--

-- An Ext-GeographicalInformation parameter comprising one octet shall be discarded

-- by the receiver if an Add-GeographicalInformation parameter is received

-- in the same message.

--

-- An Ext-GeographicalInformation parameter comprising one octet shall be treated as

-- invalid data by the receiver if an Add-GeographicalInformation parameter is not

-- received in the same message.

maxExt-GeographicalInformation  INTEGER ::= 20

-- the maximum length allows for further shapes in 3GPP TS 23.032 to be included in later

-- versions of 3GPP TS 29.002

Add-GeographicalInformation ::= OCTET STRING (SIZE (1..maxAdd-GeographicalInformation))

-- Refers to geographical Information defined in 3GPP TS 23.032.

-- This is composed of 1 or more octets with an internal structure according to

-- 3GPP TS 23.032

-- Octet 1: Type of shape, all the shapes defined in 3GPP TS 23.032 are allowed:

-- Octets 2 to n (where n is the total number of octets necessary to encode the shape

-- according to 3GPP TS 23.032) are used to encode the shape itself in accordance with the

-- encoding defined in 3GPP TS 23.032

--

-- An Add-GeographicalInformation parameter, whether valid or invalid, received

-- together with a valid Ext-GeographicalInformation parameter in the same message

-- shall be discarded.

--

-- An Add-GeographicalInformation parameter containing any shape not defined in

-- 3GPP TS 23.032 or an incorrect number of octets or coding according to

-- 3GPP TS 23.032 shall be treated as invalid data by a receiver if not received

-- together with a valid Ext-GeographicalInformation parameter in the same message.

maxAdd-GeographicalInformation  INTEGER ::= 91

-- the maximum length allows support for all the shapes currently defined in 3GPP TS 23.032

AgeOfLocationInformation ::= INTEGER (0..32767)

-- the value represents the elapsed time in minutes since the last

-- network contact of the mobile station (i.e. the actuality of the

-- location information).

-- value "0" indicates that the MS is currently in contact with the network

-- "32767" indicates that the location information is at least 32767 minutes old

GSN-Address ::= OCTET STRING (SIZE (5..17))

-- Octets are coded according to TS 3GPP TS 23.003

LCS-ReferenceNumber ::= OCTET STRING (SIZE(1))

PeriodicLDRInfo  ::= SEQUENCE {

reportingAmount		ReportingAmount-AGPS,

reportingInterval	ReportingInterval-AGPS,

... ,

reportingOptionMilliseconds	[0]	ReportingOptionMilliseconds	OPTIONAL

}

-- reportingInterval x reportingAmount shall not exceed 8639999 (99 days, 23 hours,

-- 59 minutes and 59 seconds) for compatibility with OMA MLP and RLP

-- When reportingOptionMilliseconds is provided and supported, reportingInterval &

-- reportingAmount shall be ignored.

ReportingAmount-AGPS ::= INTEGER (1..maxReportingAmount)

maxReportingAmount INTEGER ::= 8639999

ReportingInterval-AGPS ::= INTEGER (1..maxReportingInterval)

-- ReportingInterval is in seconds

maxReportingInterval INTEGER ::= 8639999

ReportingOptionMilliseconds ::= SEQUENCE {

reportingAmountMilliseconds	ReportingAmountMilliseconds,

reportingIntervalMilliseconds	ReportingIntervalMilliseconds,

...}

-- reportingAmountMilliseconds x reportingIntervalMilliseconds shall not exceed 8639999000

-- (99 days, 23 hours, 59 minutes and 59 seconds) for compatibility with OMA MLP and RLP

ReportingAmountMilliseconds ::= INTEGER (1..maxReportingAmountMilliseconds)

maxReportingAmountMilliseconds INTEGER ::= 8639999000

ReportingIntervalMilliseconds ::= INTEGER (1..maxReportingIntervalMilliseconds)

maxReportingIntervalMilliseconds INTEGER ::= 999

VelocityEstimate-AGPS ::= OCTET STRING (SIZE (4..7))

-- Refers to Velocity description defined in 3GPP TS 23.032.

-- This is composed of 4 or more octets with an internal structure according to

-- 3GPP TS 23.032

-- Octet 1: Type of velocity, only the following types in 3GPP TS 23.032 are allowed:

--		(a) Horizontal Velocity

--		(b) Horizontal with Vertical Velocity

--		(c) Horizontal Velocity with Uncertainty

--		(d) Horizontal with Vertical Velocity and Uncertainty

-- For types Horizontal with Vertical Velocity and Horizontal with Vertical Velocity

-- and Uncertainty, the direction of the Vertical Speed is also included in Octet 1

-- Any other value in octet 1 shall be treated as invalid

-- Octets 2 to 4 for case (a) Horizontal velocity:

--		Bearing				1 octet

--		Horizontal Speed				2 octets

-- Octets 2 to 5 for case (b)  Horizontal with Vertical Velocity:

--		Bearing				1 octet

--		Horizontal Speed				2 octets

--		Vertical Speed				1 octet

-- Octets 2 to 5 for case (c)  Horizontal velocity with Uncertainty:

--		Bearing				1 octet

--		Horizontal Speed				2 octets

--		Uncertainty Speed				1 octet

-- Octets 2 to 7 for case (d)  Horizontal with Vertical Velocity and Uncertainty:

--		Bearing				1 octet

--		Horizontal Speed				2 octets

--		Vertical Speed				1 octet

--		Horizontal Uncertainty Speed			1 octet

--		Vertical Uncertainty Speed				1 octet

SequenceNumber ::= INTEGER (1..maxReportingAmount)

ReportingPLMNList::= SEQUENCE {

plmn-ListPrioritized			[0] NULL					OPTIONAL,

plmn-List 						[1] PLMNList

}

PLMNList::= SEQUENCE SIZE (1..maxNumOfReportingPLMN) OF

ReportingPLMN

maxNumOfReportingPLMN INTEGER ::= 20

ReportingPLMN::= SEQUENCE {

plmn-Id 						[0] PLMN-Id,

ran-Technology 					[1] RAN-Technology			OPTIONAL,

ran-PeriodicLocationSupport		[2] NULL					OPTIONAL,

...

}

RAN-Technology ::= ENUMERATED {

gsm			(0),

umts		(1),

... }

PLMN-Id ::= OCTET STRING (SIZE (3))

-- The internal structure is defined as follows:

-- octet 1 bits 4321	Mobile Country Code 1st digit

--         bits 8765	Mobile Country Code 2nd digit

-- octet 2 bits 4321	Mobile Country Code 3rd digit

--         bits 8765	Mobile Network Code 3rd digit

--			or filler (1111) for 2 digit MNCs

-- octet 3 bits 4321	Mobile Network Code 1st digit

--         bits 8765	Mobile Network Code 2nd digit

LCS-QoS-Class ::= ENUMERATED {

bestEffort  (0),

assured  (1),

... }

--	exception handling:

--	an unrecognized value shall be treated the same as value 0 (bestEffort)

END