3GPP TSG-RAN5 Meeting #104	R5-244538

# Maastricht, Netherlands, 19 - 23 Aug 2024

Title:							Discussion on FR2 beam correspondence for initial access

Source:			Nokia

Agenda Item:			5.3.31.9

Document for:			Discussion

# 1.	Background

RAN4 introduced FR2 beam correspondence requirement for initial access in Rel-18. RAN4 sent LS to RAN5 in [1].

In RAN5#102 meeting, Keysight [2] and Apple [3] brought contributions on this topic.

In RAN5#103 Huawei [4] brought contribution on this topic with following conclusions:

Observation 1: A beam lock function in RRC_IDLE is needed to address above testability issues.

Proposal 1: For the beam lock function in IDLE mode, the activation and deactivation could be in RRC_CONNECTED mode. UE needs to memorize the beam lock activation status when it enters RRC_IDLE mode.

Proposal 2: Beam refinement time is needed between the time point UE enters RRC_IDLE mode and UE transmits PRACH preambles.

Proposal 3: Request UE to only transmit with the optimal Tx beam during conformance testing of spherical coverage in initial access, i.e. when beam lock function is activated.

Proposal 4: Extending the beam lock function to force UE to transmit at the Pcmax level.

Observation 2: Almost all the working assumptions in RAN4 LS are feasible from RAN5 point of view.

Proposal 5: RAN5 endorses the conclusions in clause 2.5.

This document provides our view on the beam lock function in RRC_IDLE mode.

# 2	Discussion

# 2.1	Beam lock function for RRC_IDLE mode

Beam Correspondence RAN4 requirements for IDLE mode UEs will require the UE to achieve peak EIRP during the conformance testing. This will be done when the UE is transmitting the message 1 (msg1) during the random access procedure. During transmission of msg1 there are certain parameters that the UE uses such as determine the target power, the time to wait for a response from the network (ra-ResponseWindow), the maximum number of retransmissions (preambleTransMax, powerRampingStep) and preambleReceivedTargetPower which is the target power as circled in red in the current RACH Configuration (TS 38.331) copied below.

- preambleTransMax: Max number of RA preamble transmission performed before declaring a failure

- ra-ResponseWindow: Msg2 (RAR) window length in number of slots.

- powerRampingStep: Power ramping steps for PRACH

- preambleReceivedTargetPower: The target power level at the network receiver side

After sending the 1st Msg1 transmission the UE waits for a response from the network (or from System Simulator in testing). When there is no feedback (msg2 from the network) within the ra-Responsewindow, a 2nd message is transmitted with higher power. The UE increases its transmit power based on the powerRampingstep parameter. This process of sending msg1 continues till the UE either receives a response from the network (msg2) or the maximum number of transmissions is exhausted. In case of the latter, a random access failure is declared.

The BEAMLOCK function has been defined only in the CONNECTED mode currently. As described in Section 4.9.2.1 of TS 38.508-1, the System Simulator requests the UE to activate the BEAMLOCK function.  Having a similar process of BEAMLOCK activation for IDLE and INACTIVE modes is not possible as the UE is not in RRC_CONNECTED mode and the System Simulator has no way of sending this message to the UE.

Based on existing specification the system simulator cannot indicate BEAMLOCK activation command to the UE when the UE is in IDLE mode.

- Observation 2: The UE in IDLE mode reads the SIB1 which has the random access configuration information.

To counter this, we propose to introduce a new parameter in SIB1, which when enabled, shall indicate to the UE that it needs to activate the BEAMLOCK after the UE transmits Random access message 1 (msg1) ‘n’ number of times. As defined in TS 38.213, the UE transmit power of msg1 based on the following formula:

Since the UE will be tested in a controlled environment, it will be relatively simple to predict the number of power ramping steps required for the UE to achieve peak EIRP during the test.

Hence , as the intent of the UE beam correspondence test in IDLE mode is to test UE conformance requirements when it transmits the msg1 at peak EIRP, the number of power ramping steps can be pre-defined.

- Proposal 1: UE to be indicated to activate BEAMLOCK function  during beam correspondence testing for UEs in IDLE mode via SIB1 at the start of the test.

To indicate a BEAMLOCK activation via the SIB1, the SIB1 needs to be enhanced to include a new information element (IE) to reflect this This IE will have the following parameters:

- BEAM LOCK   { true, false }

- Max. msg1 transmissions. {1…n}

When included in SIB1, this will indicate to the UE to activate the BEAMLOCK after the UE transmits Random access message 1 (msg1) ‘n’ number of times (Max. msg1 transmissions). The value of ‘Max. msg1 transmissions’ parameter will be set in accordance to number of msg1 transmissions which the UE will need to transmit as a part of the power ramping procedure to achieve peak EIRP during the UE conformance testing for beam correspondence in IDLE and INACTIVE modes.

For example, if the UE will achieve peak EIRP after 3 msg 1 transmissions, then Max. msg1 transmissions will also be set to value 3.

The test procedure will then be as following:

- UE reads SIB1 which contains the BEAMLOCK indication and Max. msg1 transmissions’.

- The UE and the test environment is setup such that the UE achieves peak EIRP after 3 msg1 retransmissions. Hence n here will be set as 3 ( n=3) in the SIB1.

- UE starts the Random Access procedure by sending msg1

- The System simulator withholds the RAR

- UE sends msg1 again

- The System simulator again withholds the RAR

- Steps 2 and 3 are repeated as per the Random access power ramping procedure described in Section 2.4.

- When n=3 is reached, UE sends msg1 with BEAMLOCK activated.

- Conformance tests such as spherical coverage measurements are carried out.

Figure 1: Test call flow

- Proposal 2: SIB1 to be enhanced with a new information element for BEAMLOCK activation command to UEs in IDLE mode.

- Proposal 3: New information element for BEAMLOCK activation command for IDLE mode UEs shall have the following parameters:

- BEAM LOCK   { true, false }

- Max. msg1 transmissions. {1…n}

# 3.	Conclusion

The BEAMLOCK function has been defined only in the CONNECTED mode currently. As described in Section 4.9.2.1 of TS 38.508-1, the System Simulator requests the UE to activate the BEAMLOCK function.  Having a similar process of BEAMLOCK activation, even after BEAMLOCK is defined for IDLE and INACTIVE modes is not possible as the UE is not in RRC_CONNECTED mode and the System Simulator has no way of sending this message to the UE.

- Observation 1: Based on existing specification the system simulator cannot indicate BEAMLOCK activation command to the UE when the UE is in IDLE mode.

- Observation 2: The UE in IDLE mode reads the SIB1 which has the random access configuration information.

- Proposal 1: UE to be indicated to activate BEAMLOCK function  during beam correspondence testing for UEs in IDLE mode via SIB1 at the start of the test.

- Proposal 2: SIB1 to be enhanced with a new information element for BEAMLOCK activation command to UEs in IDLE mode.

Proposal 3: New information element for BEAMLOCK activation command for IDLE mode UEs shall have the following parameters:

- BEAM LOCK   { true, false }

- Max. msg1 transmissions. {1...n}

Because RAN2 is responsible group for SIB1 specification we propose to include RAN2 into discussion to enable BEAMLOCK activation for IDLE and INACTIVE modes.

We propose RAN5 to consider this solution and send the LS to RAN2 asking their view.

# 4.	Reference

[1]	R4-2317764, LS on demodulation of FR2 UE UL by TE

[2]	R5-241402, Initial discussions for Rel-18 beam correspondence for initial access, Keysight Technologies UK Ltd

[3] R5-240944, Time synchronization required for UE beamlock function during initial access, Apple AB Denmark

[4] R5-243823, Discussion on FR2 beam correspondence for initial access, Huawei, HiSilicon