3GPP TSG-RAN5 Meeting #104	R5-244552

Maastricht, Netherlands, Aug 19 – Aug 23, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | 0670 | rev |  | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new PC6 UE capability for Rel-18 HST FR2 test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_HST_FR2_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2024-07-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | PC6 supporting UE capability tci-StateSwitchInd-r18 has been missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | Revised table A.4.3.11-1 for Test case 7.5.8.3.2 to accommodate the changes from TS.38.306 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No PC6 supporting UE capability available for NR SA FR2 HST in Rel-18 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.3.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------- Start of change --------

## A.4.3	Baseline Implementation Capabilities

### A.4.3.11	High Speed Capabilities

Table A.4.3.11-1: High Speed Capabilities

| Item | UE High Speed Capabilities | Ref. | Release | Mnemonic | M | If indicated "Yes" the feature shall be implemented and successfully tested for the corresponding release | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Support the enhanced intra-NR and inter-RAT E-UTRAN measurement requirements to support high speed up to 500 km/h | 38.306, 4.2.19 | Rel-16 | pc_hst_meas_enh_r16 | No |  |  |
| 2 | Support the enhanced demodulation processing for HST-SFN joint transmission scheme with velocity up to 500km/h | 38.306, 4.2.19 | Rel-16 | pc_hst_demod_enh_r16 | No |  |  |
| 3 | Support the enhanced intra-NR RRM requirements to support high speed up to 500 km/h | 38.306, 4.2.19 | Rel-16 | pc_hst_intraNR_meas_enh_r16 | No |  | This PICS can only be set to true when pc_hst_meas_enh_r16 is set to false. Up to one PICS between pc_hst_intraNR_meas_enh_r16 and pc_hst_interRAT_meas_enh_r16 can be set to true |
| 4 | Supports the enhanced inter-RAT E-UTRAN RRM requirements to support high speed up to 500 km/h | 38.306, 4.2.19 | Rel-16 | pc_hst_interRAT_meas_enh_r16 | No |  | This PICS can only be set to true when pc_hst_meas_enh_r16 is set to false. Up to one PICS between pc_hst_intraNR_meas_enh_r16 and pc_hst_interRAT_meas_enh_r16 can be set to true |
| 5 | Support for enhanced inter-RAT NR measurement requirements in high-speed scenario | 36.306,4.3.33.7 | Rel-16 | pc_hst_interRAT_NR_meas_enh_r16 | No |  |  |
| 6 | Supports the enhanced RRM requirements for carrier aggregation to support high speed up to 500 km/h | 38.306, 4.2.19 | Rel-17 | pc_hst_RRM_CA_enh_r17 | No |  | FR1 only UE indicating support of this feature shall indicate support of measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16. |
| 7 | Supports the enhanced RRM requirements for inter-frequency measurements in connected mode to support high speed up to 500 km/h | 38.306, 4.2.19 | Rel-17 | pc_hst_RRM_interfreq_meas_enh_r17 | No |  | FR1 only UE indicating support of this feature shall indicate support of measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16. |
| 8 | Support the enhanced RRM requirements for inter-frequency IDLE/INACTIVE measurements to support high speed up to 500 km/h | 38.306, 5.6 | Rel-17 | pc_hst_RRM_interfreq_idle_inactive_meas_enh_r17 | No |  | FR1 only UE indicating support of this feature shall indicate support of measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16. |
| 9 | Indicates whether the UE supports one shot large UL timing adjustment. | 38.306, 4.2.7.2 | Rel-17 | pc_hst_oneStep_UL_Timing_adj_r17 | No |  | FR2 only |
| 10 | Support the enhanced intra-NR RRM requirements to support high-speed up to 350 km/h for FR2 | 38.306, 4.2.7.2 | Rel-17 | pc_hst_MeasFlagFR2_r17 | No |  | FR2 only UE indicating support of this feature shall indicate support of ue-PowerClass-v1700 set to 'pc6'. |
| NOK1 | Support the enhanced one-shot large UL transmit timing adjustment requirement to support FR2 PC6 UEs and enhanced TCI state switching delay requirements in HST FR2 | 38.306, 4.2.7.2 | Rel-18 | pc_hst_tciStateSwitchInd_r18 | No |  | FR2 only UE indicating support of this feature shall indicate support of ue-PowerClass-v1700 set to 'pc6'. |

----- End of change ------