3GPP TSG-RAN5 Meeting #104	R5-244627

# Maastricht, Netherland, Aug 19th - 23th 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 3329 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to R17 Tx switch RRM test case 6.5.7B.2 with TT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_RF_FR1_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Test description and test parameters of TC A.6.5.7B.1/A.6.5.7B.2/ A.6.5.7C.1/A.6.5.7C.2 in 38.133 were updated in R4-2408541. 6.5.7B.2 needs to be updated accordingly. - 1 port SRS is configured for Cell2 and Cell 3. As a result, the UE supporting dualSwitch actually doesn’t need to perform Tx switch to support SRS transmission (1p+1p) in test, which makes this TC meaningless. We suggest change SRS port number for Cell 2 and Cell 3 to 2. - In step 6 UE is required to continuously transmit on Cell 1, which has no normative reference in 38.133. Actually RAN4’s intention is to trigger Tx switch by SRS transmission. We suggest remove step 6. - Message contents needs to be updated in following aspects: - uplinkTxSwitchingPeriodLocation-r16 field is used to indicate the carrier on which Tx switch period locates, hence only one of involved carriers can set uplinkTxSwitchingPeriodLocation-r16 = true. We suggest locate Tx switch period on PCell for simplfy. - According to 38.306, only the carrier on which UE supports 2-layer UL-MIMO can be set to “carrier 2”. i.e. Cell 2 and Cell 3. - bandIndexUL1-r16 and bandIndexUL2-r16 indicate the band pair on which UE supports dynamic UL Tx switching. bandindexUL1/bandindexUL2 xx refers to the xxth band entry in the band combination. UE shall indicate support for 2-layer UL MIMO capabilities on one of the indicated two bands in each FeatureSet entry supporting UL 1Tx-2Tx switching, and only the band where UE supports 2-layer UL MIMO capability can work as carrier2 as defined in TS 38.101-1 [2] and TS 38.101-3 [4].. - SRS-Config: - Needs to align periodicityAndOffset-p with R4-2408541, - missing fields in SRS-ResourceSet needs to be added; - c-SRS should be 14 for Cell 1 and 25 for Cell 2/Cell 3 according to 38.133 Table A.4.4.1.1.1-3, rather 0. - As explained above, nrofSRS-Ports should be port1 and port 2 for Cell 1 and Cell 2/Cell 3 respectively. - CSI-MeasConfig: - Resource config doesn’t comply with test description in 38.133. In test 3 AP CSI-RS resources are configured on the same carrier with different time domain location. However, according to the description in 38.133 A.6.5.7.1.1, RAN4’s intention is to config one AP CSI-RS resource on Cell 1 and Cell 2/Cell 3 respectively. - powerControlOffset is not correct. In this test case AP CSI-RS is configured with 6dB power boosting, i.e. powerControlOffsetSS (power offset of NZP CSI-RS RE to SSS RE) should be 6dB. On the other hand, PDSCH and SSS have same EPRE accoding to cell-specific parameter table. Hence powerControlOffset (power offset of PDSCH RE to NZP CSI-RS RE) should be -6dB accordingly. - Report Config is configured on both Cells with reportSlotOffset = 5, which means that Cell 1/Cell 2’s L1-RSRP result will be reported on slot n+5 on Cell 1/Cell 2. However, PUSCH carrying L1-RSRP result itself can also triggering Tx switch, as a result the DL interuption caused by Tx switch may not happen before SRS transmission. To avoid unwanted Tx switch, we suggest change reportSlotOffset and report L1-RSRP of both Cell 1 and Cell 2 on PCell(Cell 1). - AperiodicTriggeringOffsetList and corresponding DCI needs to be specifed. - Message contents of all UL Tx switching TCs are substantially the same. We suggest move them to Annex H for sake of simplify and better readiability. - As explained in R4-2408547, All Rel-16/Rel-17/Rel-18 UL Tx switching RRM test cases share the same test methodology but some of them use different power level settings. We suggest align the power level setting of these test cases in RAN4 for sake of simplify and easier TE implementation. Test parameters are updated based on TT analyis. - To correct wrong test parameters. - Editorial changes. |
| Summary of change: |  | - Issue mentioned above are solved. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE may fail the test. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.7B.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.903 CR 0785 TS 38.533 CR 3332 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | RAN4 dependency RRM TT Annex F CR: R5-244630, 38.903 CR: R5-244611 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

6.5.7B.2	NR SA FR1 DL Interruptions at switching between two uplink bands in TDD-TDD CA

6.5.7B.2.1	Test purpose

The purpose of this test is to verify DL interruption requirements during UE dynamic switching between two uplink bands.

6.5.7B.2.2	Test applicability

This test applies to all types of NR UE release 17 onwards configured with uplinkTxSwitchingPeriod and supports simultaneousRxTxInterBandCA.

6.5.7B.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.5.7B.0.

The normative reference for this requirement is TS 38.133 [6] clauses A.6.5.7B.2

6.5.7B.2.4	Test description

6.5.7B.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.5.7B.2.4.1-1.

Table 6.5.7B.2.4.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 6.5.7B.2-1 | NR Cell 1: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode NR Cell 2: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode NR Cell 3: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |

Configure the test equipment and the DUT according to the parameters in Table 6.5.7B.2.4.1-2.

Table 6.5.7B.2.4.1-2: Initial conditions for DL interruptions at switching between two uplink carriers in TDD-TDD CA

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.5.7B.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.2a | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE part. - Without LTE link - Without Faders |  |  |

1. The general test parameter settings are set up according to Table 6.5.7B.2.4.1-3.

2. Message contents are defined in clause 6.5.7B.2.4.3.

3. The test scenario comprises of three NR Cells. (Cell 1, Cell 2 and Cell3). Cell 1, Cell 2 and Cell3 are configured according to Annex C.1.2 and C.1.3.

Table 6.5.7B.2.4.1-3: General test parameters for DL interruptions at switching between two uplink carriers in TDD-TDD CA

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  | Config 1 | 1, 2, 3 | Two radio channels are used for this test. |
| Active cell |  | Config 1 | Cell 1: FR1 PCell Cell 2: FR1 SCell Cell 3: FR1 SCell | Cell1: FR1 PCell on RF channel number 1 in band A Cell 2: FR1 SCell on RF channel number 2 in band B Cell 3: FR1 SCell on RF channel number 3 in band B Note: Cell 2 and Cell 3 are two contiguous aggregated carriers |
| CP length |  | Config 1 | Normal |  |
| DRX |  | Config 1 | OFF |  |
| Measurement gap pattern Id |  | Config 1 | OFF |  |
| Filter coefficient |  | Config 1 | 0 | L3 filtering is not used |
| CSI-RS configuration for L1-RSRP reporting |  | Config 1 | Cell 1: CSI-RS.2.5 TDD Cell 2: CSI-RS.2.5 TDD Cell 3: CSI-RS.2.5 TDD |  |
| T1 | s | Config 1 | 5 |  |

6.5.7B.2.4.2	Test procedure

FR1 TDD PCell (Cell 1), FR1 TDD SCell (Cell 2) and FR1 TDD SCell (Cell 3) where cell 1 in band A is with 1TX, cell2 and cell 3 in band B with 2Tx, cell2 and cell3 are two contiguous aggregated carriers. The test consists of one time period, with duration of T1. Prior to the start of the time duration T1, the UE shall be connected to Cell1, Cell2 and Cell3 and the RRC message including uplinkTxSwitching is received at the UE antenna connector.

UE is configured to transmit SRS on all the PCell and SCell on the last 2 symbols of special slot. DL interruption is expected to take place prior to the SRS symbols.

- Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Configure PCell (Cell1) and SCell (Cell2 and Cell3) according to Table 6.5.7B.2.5-1. Propagation conditions are set according to Annex C clause C.2.2.

3.	The SS shall transmit an RRCReconfiguration message to configure PCell (Cell1) and SCell (Cell2 and Cell3) as per TS 38.508-1 [7] clause 7 with the message content exceptions defined in clause 6.5.7B.2.4.3. UplinkTxSwitching is configured to the UE.

4.	The UE shall transmit RRCReconfigurationComplete message.

5.	Set the parameters according to T1 in Tables 6.5.7B.2.5-1. T1 starts.

6.	SS triggers aperiodic CSI-RS for L1-RSRP reporting with power boosting 6dB on following symbol on the 1st special slot of every radio frame on PCell (Cell 1):

-	symbol#10 if UE does not report uplinkTxSwitching-DL-Interruption-r16;

-	otherwise,

-	symbol #4 if UE capability uplinkTxSwitchingPeriod is 210us or

-	symbol #5 if UE capability uplinkTxSwitchingPeriod is 140us or

-	symbol #8 if UE capability uplinkTxSwitchingPeriod is 35us.

7.	For NR TDD Cell 2 and NR TDD Cell 3, SS triggers aperiodic CSI-RS for L1-RSRP reporting with power boosting 6dB on following symbol on the 2nd special slot of every radio frame on SCells (Cell2 and Cell3):

-	symbol#10 if UE does not report uplinkTxSwitching-DL-Interruption-r16;

-	otherwise,

-	symbol #4 if UE capability uplinkTxSwitchingPeriod is 210us or

-	symbol #5 if UE capability uplinkTxSwitchingPeriod is 140us or

-	symbol #8 if UE capability uplinkTxSwitchingPeriod is 35us.

8.	After SS transmits the DCI trigger, the UE shall send L1-RSRP report containing valid L1-RSRP report while meeting the test requirements given in Table 6.5.7A.2.5-3, if so, increase the number of passed iterations by one otherwise increase the number of failed iterations by one and switch off the UE.

9.	SS transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED TS 38.508-1 [14] clause 4.5. (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED TS 38.508-1 [14] clause 4.5),

or

-	switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10.	Repeat step 3-9 until a test verdict has been achieved.

6.5.7B.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.5.7B.2.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.11-1 with condition 1T-2T and 2SCELLS |
| Cell-specific messages and information elements contents exceptions for Cell 1 | Table H.3.11-3 with condition 1T-2T and PCELL Table H.3.11-4 with condition SwitchFrom Table H.3.11-5 with condition 1T-2T, TDD-TDD and 2SCELLS Table H.3.11-11 |
| Cell-specific messages and information elements contents exceptions for Cell 2 | Table H.3.11-3 with condition 1T-2T Table H.3.11-4 with condition SwitchTo and 2TX Table H.3.11-5 TS 38.508-1 [14] Table 7.3.1-1 with condition TDDConf.2.2 |
| Cell-specific messages and information elements contents exceptions for Cell 3 | Table H.3.11-3 with condition 1T-2T Table H.3.11-4 with condition SwitchTo and 2TX Table H.3.11-5 TS 38.508-1 [14] Table 7.3.1-1 with condition TDDConf.2.2 |

Table 6.5.7B.2.4.3-2: Physical layer parameters for DCI format 0_1 (Test procedure, step 6 and step 7)

| Derivation Path: 38.508-1[14] Table 4.3.6.1.1.2-1 |  |  |  |
| --- | --- | --- | --- |
| Parameter | Value | Value in binary | Condition |
| CSI request | First entry in CSI-AperiodicTriggerStateList | “01” | Step 6 |
|  | Second entry in CSI-AperiodicTriggerStateList | “10” | Step 7 |

Table 6.5.7B.2.4.3-1 - Table 6.5.7B.2.4.3-4: Void

6.5.7B.2.5	Test requirements

Table 6.5.7B.2.5-1 defines the primary level settings including test tolerances for DL interruptions at switching between two uplink carriers in TDD-TDD CA.

Table 6.5.7B.2.5-1: Cell specific test parameters for DL interruptions at switching between two uplink bands in TDD-TDD CA

| Parameter |  | Unit | Cell1 | Cell2 | Cell3 |
| --- | --- | --- | --- | --- | --- |
| Frequency Range |  |  | FR1 | FR1 | FR1 |
| Duplex mode | Config 1 |  | TDD | TDD | TDD |
| TDD configuration | Config 1 |  | TDDConf.2.1 except that S=’1 1DL: :2UL’; nrofDownlinkSymbols: 11 nrofUplinkSymbols: 2 | TDDConf.2.2 | TDDConf.2.2 |
| BWchannel | Config 1 |  | 40 MHz: NRB,c = 106 | 40 MHz: NRB,c = 106 | 40 MHz: NRB,c = 106 |
| Initial BWP Configuration | Config 1 |  | DLBWP.0.1 | DLBWP.0.1 | DLBWP.0.1 |
| DL dedicated BWP configuration | Config 1 |  | DLBWP.1.1 | DLBWP.1.1 | DLBWP.1.1 |
| UL dedicated BWP configuration | Config 1 |  | ULBWP.1.1 | ULBWP.1.1 | ULBWP.1.1 |
| SRS configuration | Config 1 |  | SRSConf.1 in Table A.4.4.1.1.1-3 is applied except that: resourceMappingstartPosition: 0 resourceMappingnrofSymbols: n2 periodicityAndOffset-p: sl20,5 | SRSConf.1 in Table A.4.4.1.1.1-3 is applied except that: resourceMappingstartPosition: 0 resourceMappingnrofSymbols: n2 periodicityAndOffset-p: sl20,3 nrofSRS-Ports: ports2 | SRSConf.1 in Table A.4.4.1.1.1-3 is applied except that: resourceMappingstartPosition: 0 resourceMappingnrofSymbols: n2 periodicityAndOffset-p: sl20,3 nrofSRS-Ports: ports2 |
| PDSCH Reference measurement channel | Config 1 |  | SR.2.1 TDD | SR.2.1 TDD | SR.2.1 TDD |
| RMSI CORESET parameters | Config 1 |  | CR.2.1 TDD | CR.2.1 TDD | CR.2.1 TDD |
| Dedicated CORESET parameters | Config 1 |  | CCR.2.1 TDD | CCR.2.1 TDD | CCR.2.1 TDD |
| OCNG Patterns |  |  | OP.1 | OP.1 | OP.1 |
| SMTC Configuration |  |  | SMTC.1 | SMTC.1 | SMTC.1 |
| SSB Configuration | Config 1 |  | SSB.2 FR1 | SSB.2 FR1 | SSB.2 FR1 |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low | 2x2 Low | 2x2 Low |
| EPRE ratio of PSS to SSS |  | dB | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| NocNote 2 |  | dBm/15 kHz | -104 | -104 | -104 |
| SS-RSRP Note 3 |  | dBm/SCS | -87 | -87 | -87 |
| Ês/Iot |  | dB | 14 | 14 | 14 |
| Ês/Noc |  | dB | 14 | 14 | 14 |
| NocNote 2 | Config 1 | dBm/SCS | -101 | -101 | -101 |
| IoNote3 | Config 1 | dBm/38.16 MHz | -55.78 | -55.78 | -55.78 |
| Time offset to Cell1 Note 5 |  | s | - | 0 | 0 |
| Propagation Condition |  |  | AWGN | AWGN | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Void Note 5: Receive time difference between slot boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. |  |  |  |  |  |

UE shall send L1-RSRP report while meeting the accuracy requirements defined in TS 38.133 clause 10.1.19.2.

The DL interruption lengths of X are defined in Table 6.5.7B.2.5-2.

Table 6.5.7B.2.5-2: DL interruption length on NR carrier(s) in the unit of OFDM symbols (X) for switching between two uplink carriers

|  | NR Slot length (ms) | Uplink Tx switching period Note1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | 35us | 140us | 210us |
| 0 | 1 | 2 | 3 | 4 |
| 1 | 0.5 | 3 | 6 | 7 |
| 2 | 0.25 | 4 | 10 | 14 |
| Note 1: Uplink Tx switching period depends on UE capability uplinkTxSwitchingPeriod. |  |  |  |  |

The UE shall send L1-RSRP report from the reception of DCI trigger. The L1-RSRP report shall include the results of CSI-RS#0, CSI-RS#1 and CSI-RS#2. Each L1-RSRP measurement report shall meet the corresponding absolute accuracy requirements in Table 6.5.7A.2.5-3.

The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

<End of Change 1>