3GPP TSG-RAN5 Meeting #104	R5-244654

# Maastricht, Netherland, Aug 19th - 23th 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | ???? | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of Cov_Enh2 SIG test case 7.1.1.1.22_CEonly_BWP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_cov_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - To add Cov_Enh2 SIG test case 7.1.1.1.22 - Random access procedure / Msg1 repetition / CE-only BWP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Cov_Enh2 SIG test case 7.1.1.1.22 - Random access procedure / Msg1 repetition / CE-only BWP is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work plan is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.1.22 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Upgrading of 38.523-1 to Rel-18 wil be triggered |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

7.1.1.1.22	Random access procedure / Msg1 repetition / Msg.1-based SI request

7.1.1.1.22.1	Test Purpose (TP)

(1)

with { UE is in RRC_connected state and is configured with only RACH resources for Msg.1 repetition in active BWP }

ensure that {

when { UE has data available for transmission but doesn’t receive any UL grant }

then { UE initiates a Random Access procedure with Msg.1 repetition of configured repetition number }

}

7.1.1.1.22.2	Conformance requirements

References: The conformance requirements covered in the present test case are specified in: TS 38.213 clause 8.1, 8.2, TS 38.321 clause 5.1.1b, 5.1.2, 5.1.4. Unless otherwise stated these are Rel-18 requirements.

[TS 38.213 clause 8.1]

Physical random access procedure for a UE is triggered upon request of a PRACH transmission by higher layers or by a PDCCH order or LTM Cell Switch Command MAC CE in clause 6.1.3.75 [11, TS 38.321] for a cell. A configuration by higher layers for a PRACH transmission includes the following:

-	A configuration for PRACH transmission on the cell [4, TS 38.211].

-	A preamble index, a preamble SCS, PPRACH,target, a corresponding RA-RNTI when applicable [11, TS 38.321], and a PRACH resource for the cell.

-	A number of Npreamblerep>1 preamble repetitions for the PRACH transmission if the UE would transmit the PRACH with repetitions.

A UE transmits a PRACH on a cell using the selected PRACH format with transmission power PPRACH,b,f,c(i), as described in clause 7.4, on the indicated PRACH resource or on a determined set of Npreamblerep resources using a same spatial filter in case of Npreamblerep preamble repetitions.

…

For a PRACH transmission with Npreamblerep preamble repetitions, a set consists of Npreamblerep valid PRACH occasions that are consecutive in time, use same frequency resources, and are associated with same one or more SS/PBCH block index(es), and each SS/PBCH block index is associated with same preamble indexes in all valid PRACH occasions within the set.

For a PRACH transmission with preamble repetitions, a time period, starting from frame 0, is the smallest integer number of association pattern periods such that at least one set of valid PRACH occasions for each of the NTxSSB SS/PBCH block indexes can be determined within the time period for all configured number of preamble repetitions. The set(s) of valid PRACH occasions for each configured number of preamble repetitions repeats every time period.

Within a time period, for set(s) of Npreamblerep valid PRACH occasions for a PRACH transmission with Npreamblerep preamble repetitions

-	the first valid PRACH occasion of the first set is the first valid PRACH occasion

-	the first valid PRACH occasion of subsequent sets, if any, is determined according to an ordering of valid PRACH occasions

-	first, in increasing order of frequency resource indexes for frequency multiplexed PRACH occasions

-	second, in increasing order of time resource indexes for time multiplexed PRACH occasions

where, for each frequency resource index for frequency multiplexed PRACH occasions

-	the first valid PRACH occasion of the first set is the first valid PRACH occasion

-	the first valid PRACH occasion of subsequent sets, if any,

-	is after msg1-RepetitionTimeOffsetROGroup consecutive valid PRACH occasions in time from the first valid PRACH occasion of the previous set, where each PRACH occasion is associated with same SS/PBCH block index(es) and each SS/PBCH block index is associated with same preambles, if msg1-RepetitionTimeOffsetROGroup is provided

-	is after the PRACH occasions for the previous set, if msg1-RepetitionTimeOffsetROGroup is not provided

…

[TS 38.213 clause 8.2]

In response to a PRACH transmission, a UE attempts to detect a DCI format 1_0 with CRC scrambled by a corresponding RA-RNTI during a window controlled by higher layers [11, TS 38.321] if the PRACH transmission is not triggered by a PDCCH order that includes a Cell Indicator field with non-zero value; otherwise, the UE does not attempt to detect the DCI format 1_0. The window starts at the first symbol of the earliest CORESET the UE is configured to receive PDCCH for Type1-PDCCH CSS set, as defined in clause 10.1, that is at least one symbol, after the last symbol of the last PRACH occasion corresponding to the PRACH transmission, where the symbol duration corresponds to the SCS for Type1-PDCCH CSS set as defined in clause 10.1. If NTA,adjUE or NTA,adjcommon, as defined in [4, TS 38.211], is not zero, the window starts after an additional TTA+kmac msec where TTA is defined in [4, TS 38.211] and kmac is provided by kmac or kmac=0 if kmac is not provided. The length of the window in number of slots, based on the SCS for Type1-PDCCH CSS set, is provided by ra-ResponseWindow.

…

[TS 38.321, clause 5.1.1b]

The MAC entity shall:

…

1>	else if the BWP selected for Random Access procedure is configured only with Random Access resources with msg1-Repetitions set to true:

2>	assume Msg1 repetition is applicable for the current Random Access procedure;

…

2>	else (none of rsrp-ThresholdMsg1-RepetitionNumX is configured):

3>	assume Msg1 repetition number applicable for the current Random Access procedure is the Msg1 repetition number that configured for this BWP.

[TS 38.321, clause 5.1.2]

The RA-RNTI associated with the PRACH occasion in which the Random Access Preamble is transmitted or the RA-RNTI associated with the last valid PRACH occasion in the set of PRACH occasions (as specified in TS 38.213 [6]) for Msg1 repetition, is computed as:

RA-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id

where s_id is the index of the first OFDM symbol of the PRACH occasion (0 ≤ s_id < 14), t_id is the index of the first slot of the PRACH occasion in a system frame (0 ≤ t_id < 80), where the subcarrier spacing to determine t_id is based on the value of μ specified in clause 5.3.2 in TS 38.211 [8] for μ = {0, 1, 2, 3}, and for μ = {5, 6}, t_id is the index of the 120 kHz slot in a system frame that contains the PRACH occasion (0 ≤ t_id < 80), f_id is the index of the PRACH occasion in the frequency domain (0 ≤ f_id < 8), and ul_carrier_id is the UL carrier used for Random Access Preamble transmission (0 for NUL carrier, and 1 for SUL carrier).

7.1.1.1.22.3	Test description

7.1.1.1.22.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0 except that Short_DCI condition is applied in NR Serving cell configuration.

7.1.1.1.22.3.2	Test procedure sequence

Table 7.1.1.1.22.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits an RRCReconfiguration message to reconfigure active BWP as a CE-only BWP. | <-- | RRCReconfiguration | - | - |
| 2 | The UE transmits RRCReconfigurationComplete message | --> | RRCReconfigurationComplete | - | - |
| 3 | The SS transmits a MAC PDU containing a PDCP SDU | <-- | MAC PDU | - | - |
| 4 | The SS ignores scheduling requests and does not allocate any uplink grant. | - | - | - | - |
| 5 | Check: Does the UE transmit PRACH preambles with preamble index k on 2 consecutive ROs starting from the RO #n in radio frame #m, where - 8 ≤ k < 16 for FR1, 4 ≤ k < 8 for FR2; - n = 0 for FR1 and n mod 16 = 0 for FR2; - m mod 8 = 0 for FR1 FDD and SCS15, m mod 16 = 0 for FR1 TDD, m mod 4 = 1 for FR2. | - | - | 1 | P |
| 6 | The SS transmits Random Access Response with RAPID corresponding to preamble index k and RA-RNTI associated with the RO #n+1 in radio frame #m, where k,n and m are same as step 5 | <-- | Random Access Response | - | - |

7.1.1.1.22.3.3	Specific message contents

Table 7.1.1.1.22.3.3-1: RRCReconfiguration (Table 7.1.1.1.22.3.2-1, step 1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup SEQUENCE { |  |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |  |
| spCellConfigDedicated |  | ServingCellConfig | Table 7.1.1.1.22.3.3-2 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.1.1.22.3.3-2: ServingCellConfig (Table 7.1.1.1.22.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-167 with condition DLBWP_add and ULBWP_add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| uplinkConfig SEQUENCE { |  |  |  |
| uplinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF SEQUENCE { | 1 entry |  |  |
| BWP-Uplink[1] SEQUENCE { |  | entry 1 |  |
| bwp-Common SEQUENCE { |  |  |  |
| rach-ConfigCommon CHOICE { |  |  |  |
| release | null | No legacy RACH resource is configured on BWP#1 |  |
| } |  |  |  |
| additionalRACH-ConfigList-r17 CHOICE { |  |  |  |
| setup SEQUENCE (SIZE(1..maxAdditionalRACH-r17)) OF AdditionalRACH-Config-r17 { | 1 entry |  |  |
| AdditionalRACH-Config-r17[1] SEQUENCE { |  | entry 1 |  |
| rach-ConfigCommon-r17 | RACH-ConfigCommon |  |  |
| msgA-ConfigCommon-r17 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| additionalRACH-perPCI-ToAddModList-r18 | Not present |  |  |
| additionalRACH-perPCI-ToReleaseList-r18 | Not present |  |  |
| rsrp-ThresholdMsg1-RepetitionNum2-r18 | Not present |  |  |
| rsrp-ThresholdMsg1-RepetitionNum4-r18 | Not present |  |  |
| rsrp-ThresholdMsg1-RepetitionNum8-r18 | Not present |  |  |
| preambleTransMax-Msg1-Repetition-r18 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.1.22.3.3-3: RACH-ConfigCommon (Table 7.1.1.1.22.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-128 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommon ::= SEQUENCE { |  |  |  |
| featureCombinationPreambles-r17 SEQUENCE (SIZE(1..maxFeatureCombPreamblesPerRACHResource-r17)) OF FeatureCombinationPreambles-r17{ | 1 entry |  |  |
| FeatureCombinationPreambles-r17[1] | FeatureCombinationPreambles | entry 1 Table 7.1.1.1.22.3.3-4 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.1.22.3.3-4: FeatureCombinationPreambles

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-56E with condition MSG1_REP |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FeatureCombinationPreambles-r17 ::= SEQUENCE { |  |  |  |
| startPreambleForThisPartition-r17 | 0 |  |  |
| } |  |  |  |

<End of Change 1>