3GPP TSG-RAN5 Meeting #104	R5-244668

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5330 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to IRAT redirection test case 8.1.3.7 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Authentication and security procedures were moved to earlier steps (5A-5D) with R5-240548. With these changes, UE may trigger ACTIVATE PDP CONTEXT REQUEST message immediately (for the PS call initiated at step 1) instead of waiting till step 13A. This message is currently treated as unexpected causing test case failure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added step 5E for handling ACTIVATE PDP CONTEXT REQUEST message and Void step 13A. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may unfairly fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

#### 8.1.3.7	RRC connection release / Redirection from UTRAN to E-UTRAN

8.1.3.7.1	Test Purpose (TP)

(1)

with { UE in UTRA Idle state }

ensure that {

when { UE is requested to make an outgoing PS call }

then { UE includes in the RRC CONNECTION REQUEST the IE Pre-Redirection info }

}

(2)

with { UE in UTRA CELL_DCH state }

ensure that {

when { UE receives an RRC CONNECTION RELEASE message including an IE Redirection info with E-UTRA target info E-UTRA frequency }

then { UE enters RRC_IDLE state on E-UTRAN Carrier included in IE Redirection info }

}

8.1.3.7.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 25.331, clause  8.1.3.3, 8.1.4.3 and clause 8.5.2.

[TS 25.331, clause 8.1.3.3]

The UE shall, in the transmitted RRC CONNECTION REQUEST message:

…

1>	if the UE supports E-UTRA:

2>	if the variable EUTRA_FREQUENCY_INFO_LIST contains no E-UTRA frequencies;

3>	include the IE "Pre-Redirection info";

3>	if the UE supports E-UTRA FDD:

4>	set the IE "Support of E-UTRA FDD" to TRUE.

3>	if the UE supports E-UTRA TDD:

4>	set the IE "Support of E-UTRA TDD" to TRUE.

2>	if the UE supports any of the bands that the E-UTRA frequencies included in the variable EUTRA_FREQUENCY_INFO_LIST belong to;

3>	include the IE "Pre-Redirection info";

3>	if the UE supports any of the bands that the E-UTRA FDD frequencies included in the variable EUTRA_FREQUENCY_INFO_LIST belong to:

4>	set the IE "Support of E-UTRA FDD" to TRUE.

3>	if the UE supports any of the bands that the E-UTRA TDD frequencies included in the variable EUTRA_FREQUENCY_INFO_LIST belong to:

4>	set the IE "Support of E-UTRA TDD" to TRUE.

[TS 25.331, clause 8.1.4.3]

The UE shall receive and act on an RRC CONNECTION RELEASE message in states CELL_DCH and CELL_FACH and CELL_PCH (FDD only). Furthermore this procedure can interrupt any ongoing procedures with the UE in the above listed states.

When the UE receives the first RRC CONNECTION RELEASE message; and

1>	if the message is received on the CCCH, and IE "U-RNTI" is present and has the same value as the variable U_RNTI; or

1>	if the message is received on DCCH:

the UE shall perform the RRC connection release procedure as specified below.

When the UE receives the first RRC CONNECTION RELEASE message; and

1>	if the message is received on the CCCH, the IE "UTRAN group identity" is present and there is a group identity match according to subclause 8.6.3.13:

the UE shall perform the RRC connection release procedure as specified below.

The UE shall:

1>	in state CELL_DCH:

2>	initialise the counter V308 to zero;

2>	set the IE "RRC transaction identifier" in the RRC CONNECTION RELEASE COMPLETE message to the value of "RRC transaction identifier" in the entry for the RRC CONNECTION RELEASE message in the table "Accepted transactions" in the variable TRANSACTIONS;

2>	submit an RRC CONNECTION RELEASE COMPLETE message to the lower layers for transmission using UM RLC on the DCCH to the UTRAN;

2>	if the IE "Rplmn information" is present:

3>	the UE may:

4>	store the IE on the ME together with the PLMN id for which it applies;

3>	the UE may then:

4>	utilise this information, typically indicating where a number of BCCH frequency ranges of a RAT may be expected to be found, during subsequent Rplmn selections of the indicated PLMN.

2>	if the IE "UE Mobility State Indicator" is present:

3>	consider the High-mobility state to have being detected when entering idle mode.

2>	start timer T308 when the RRC CONNECTION RELEASE COMPLETE message is sent on the radio interface.

1>	in state CELL_FACH:

2>	if the RRC CONNECTION RELEASE message was received on the DCCH:

3>	set the IE "RRC transaction identifier" in the RRC CONNECTION RELEASE COMPLETE message to	the value of "RRC transaction identifier" in the entry for the RRC CONNECTION RELEASE message in the table "Accepted transactions" in the variable TRANSACTIONS;

3>	submit an RRC CONNECTION RELEASE COMPLETE message to the lower layers for transmission using AM RLC on the DCCH to the UTRAN.

3>	when the successful transmission of the RRC CONNECTION RELEASE COMPLETE message has been confirmed by the lower layers:

4>	release all its radio resources; and

4>	indicate the release of the established signalling connections (as stored in the variable ESTABLISHED_SIGNALLING_CONNECTIONS) and established radio access bearers (as stored in the variable ESTABLISHED_RABS) to upper layers; and

4>	clear any entry for the RRC CONNECTION RELEASE message in the tables "Accepted transactions" and "Rejected transactions" in the variable TRANSACTIONS;

4>	clear the variable ESTABLISHED_SIGNALLING_CONNECTIONS;

4>	clear the variable ESTABLISHED_RABS;

4>	pass the value of the IE "Release cause" received in the RRC CONNECTION RELEASE message to upper layers;

4>	enter idle mode;

4>	perform the actions specified in subclause 8.5.2 when entering idle mode.

3>	and the procedure ends.

2>	if the RRC CONNECTION RELEASE message was received on the CCCH:

3>	release all its radio resources;

3>	indicate the release of the established signalling connections (as stored in the variable ESTABLISHED_SIGNALLING_CONNECTIONS) and established radio access bearers (as stored in the variable ESTABLISHED_RABS) to the upper layers;

3>	clear any entry for the RRC CONNECTION RELEASE message in the tables "Accepted transactions" and "Rejected transactions" in the variable TRANSACTIONS;

3>	clear the variable ESTABLISHED_SIGNALLING_CONNECTIONS;

3>	clear the variable ESTABLISHED_RABS;

3>	pass the value of the IE "Release cause" received in the RRC CONNECTION RELEASE message to upper layers;

3>	enter idle mode;

3>	perform the actions specified in subclause 8.5.2 when entering idle mode;

3>	and the procedure ends.

[TS 25.331, clause 8.5.2]

When entering idle mode from connected mode, the UE shall:

1>	clear or set variables upon leaving UTRA RRC connected mode as specified in subclause 13.4;

1>	if the RRC CONNECTION RELEASE message was received and the IE "Redirection info" was present therein:

2>	if the IE "Frequency info" is present, attempt to camp on a suitable cell on the indicated UTRA carrier included in the RRC CONNECTION RELEASE message;

2>	if the IE "GSM target cell info" is present, attempt to camp on a suitable cell of the list of cells for the indicated RAT included in the RRC CONNECTION RELEASE message. If no cells were indicated for that RAT or no suitable cell of the indicated cells for that RAT is found within 10s, attempt to camp on any suitable cell of that RAT; or

2>	if the IE "E-UTRA target info" is present, attempt to camp on any of the frequencies for the indicated RAT included in the RRC CONNECTION RELEASE message, excluding any cell indicated in the list of not allowed cells for that RAT, if present. If no suitable cell on the indicated frequencies for that RAT is found within 10s, attempt to camp on any suitable cell of that RAT, excluding any cell indicated in the list of not allowed cells for that RAT; or

2>	if no suitable cell is found on the indicated UTRA carrier or RAT camp on any suitable cell.

8.1.3.7.3	Test description

8.1.3.7.3.1	Pre-test conditions

System Simulator:

-	2 cells, one UTRA and one E-UTRA cell:

-	Cell 5 UTRA serving cell (priority 4 default)

-	Cell 1 suitable neighbour E-UTRA cell (priority 3)

-	System information combination 4 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

UTRAN Idle state

Preamble:

The UE is in state Registered, Idle mode according to section 4.5A.3A [18] on Cell 5 (serving cell).

8.1.3.7.3.2	Test procedure sequence

Table 8.1.3.7.3.2-0 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Subsequent configurations marked "T1" is applied at the point indicated in the Main behaviour description in Table 8.1.3.7.3.2-1.

Table 8.1.3.7.3.2-0: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 5 | Remark |
| --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -70 | - | The power level values are such that entering condition for event 3c is satisfied. |
|  | CPICH Ec (UTRA FDD) | dBm/3.84 MHz | - | -85 |  |
|  | PCCPCH Ec (UTRA LCR TDD) | dBm/1.28 MHz | - | -85 |  |

Table 8.1.3.7.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Make the UE initiate an outgoing PS call. | - | - | - | - |
| 2 | Check: Does the UE include the IE Pre-redirection info with Support of E-UTRA set to TRUE? | --> | RRC CONNECTION REQUEST | 1 | P |
| 3 | The SS transmits a RRC CONNECTION SETUP on SRB1 on Cell 5. | <-- | RRC CONNECTION SETUP | - | - |
| 4 | The UE transmits an RRC CONNECTION SETUP COMPLETE message | --> | RRC CONNECTION SETUP COMPLETE | - | - |
| 5 | The UE transmits the SERVICE REQUEST message for PS data call | --> | INITIAL DIRECT TRANSFER | - | - |
| 5A | The SS starts the Authentication and Ciphering Request procedure | <-- | DOWNLINK DIRECT TRANSFER | - | - |
| 5B | The UE transmits the Authentication and Ciphering Response | --> | UPLINK DIRECT TRANSFER | - | - |
| 5C | The SS transmits a SECURITY MODE COMMAND message. | <-- | SECURITY MODE COMMAND | - | - |
| 5D | The UE transmits a SECURITY MODE COMPLETE message | --> | SECURITY MODE COMPLETE | - | - |
| 5E | The UE transmits an ACTIVATE PDP CONTEXT REQUEST message NOTE: Retransmission of this message is ignored. | --> | UPLINK DIRECT TRANSFER | - | - |
| - | EXCEPTION: Steps 6a1 and 6a2 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that takes place if a capability is supported. UTRAN FDD: IF pc_UTRA_CompressedModeRequired and pc_UTRA_FeatrGrp_2 THEN Steps 6a1, 6a2 UTRAN TDD: go to step 6Aa1 | - | - | - | - |
| 6a1 | The SS transmits a PHYSICAL CHANNEL RECONFIGURATION message on Cell 5 including the DPCH compressed mode info. | <-- | PHYSICAL CHANNEL RECONFIGURATION | - | - |
| 6a2 | The UE transmits a PHYSICAL CHANNEL RECONFIGURATION COMPLETE message on Cell 5. | --> | PHYSICAL CHANNEL RECONFIGURATION COMPLETE | - | - |
| - | EXCEPTION: Steps 6Aa1 and 6Aa2 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that takes place if a capability is supported. IF pc_UTRA_FeatrGrp_2 THEN 6Aa1 to 6Aa2. | - | - | - | - |
| 6Aa1 | The SS configures the Inter-RAT measurement with E-UTRA frequency list including the DL Carrier frequency of Cell 1 | <-- | MEASUREMENT CONTROL | - | - |
| 6Aa2 | The SS changes Cell 1 and Cell 5 power level according to the row "T1" in table 8.1.3.7.3.2-0. Note: The UE starts sending MEASUREMENT REPORT messages now | - | - | - | - |
| 7-13A | Void |  |  |  |  |
| - | EXCEPTION: Step 14a describes behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that takes place if a capability is supported. | - | - | - | - |
| 14a | IF pc_UTRA_FeatrGrp_2 THEN the UE sends a MEASUREMENT REPORT message. | --> | MEASUREMENT REPORT | - | - |
| 15 | The SS transmits an RRC CONNECTION RELEASE message (IE E-UTRA target info including DL Carrier frequency of Cell 1). | <-- | RRC CONNECTION RELEASE | - | - |
| 16 | The UE transmits a RRC CONNECTION RELEASE COMPLETE message | --> | RRC CONNECTION RELEASE COMPLETE | - | - |
| 17 | The UE transmits a RRC CONNECTION RELEASE COMPLETE message | --> | RRC CONNECTION RELEASE COMPLETE | - | - |
| - | EXCEPTION : Step 18a1 to 18b6 describe behaviour that depends on whether a UE that has a PDP context activated on UTRAN | - | - | - | - |
| 18a1 | If PDP context is not active: Check: Does the test result of generic test procedure in TS 36.508 Table 6.4.2.7A-2 indicate that the UE is camped on E-UTRAN Cell 1? | - | - | 2 | - |
| 18b1 | If PDP context is active: Check: Does the test result of generic test procedure in TS 36.508 Table 6.4.2.7A-1 is performed and the UE is camped on E-UTRAN Cell 1? | - | - | 2 | - |
| 18b2 | The UE transmits a RRCConnectionRequest message on E-UTRAN Cell1 | --> | RRC: RRCConnectionRequest | - | - |
| 18b3 | SS transmit an RRCConnectionSetup message | <-- | RRC: RRCConnectionSetup | - | - |
| 18b4 | The UE transmits a RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and a EMM SERVICE REQUEST message. | --> | RRC: RRCConnectionSetupComplete NAS: SERVICE REQUEST | - | - |
| 18b5 | SS transmits a SERVICE REJECT with cause Protocol error, unspecified | <-- | SERVICE REJECT | - | - |
| 18b6 | The SS transmits an RRCConnectionRelease message to release RRC connection and move to RRC_IDLE. | <-- | RRC: RRCConnectionRelease | - | - |
| NOTE: pc_UTRA_FeatrGrp_2 is always set to TRUE when pc_UTRA_CompressedModeRequired is TRUE. |  |  |  |  |  |

8.1.3.7.3.3	Specific message or IE contents

Table 8.1.3.7.3.3-0: Conditions for specific message contents
in Table 8.1.3.7.3.3-1

| Condition | Explanation |
| --- | --- |
| Band > 64 | If E-UTRA band > 64 is selected |

Table 8.1.3.7.3.3-1: System Information Block type 19 for Cell 5 (preamble and all steps, Table 8.1.3.7.3.2-1)

| Derivation Path: 36.508 table Table 4.4.4.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType19 ::= SEQUENCE { |  |  |  |
| utra-PriorityInfoList SEQUENCE { |  |  |  |
| utra-ServingCell SEQUENCE { |  |  |  |
| priority | 4 |  |  |
| } |  |  |  |
| } |  |  |  |
| eutra-FrequencyAndPriorityInfoList SEQUENCE (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE | 1 entry |  |  |
| earfcn[1] | Downlink EARFCN of Cell 1 |  |  |
| priority[1] | 3 |  |  |
| } |  |  |  |
| v920NonCriticalExtensions SEQUENCE { |  |  | Band > 64 |
| earfcn [1] | Same downlink EARFCN as used for Cell 1 |  |  |
| priority [1] | 3 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.3.7.3.3-2: RRC CONNECTION REQUEST (step 2, Table 8.1.3.7.3.2-1)

| Derivation path: 34.108 default RRC CONNECTION REQUEST in section 9.1.1 for UTRA FDD or 9.1.2 for UTRA TDD |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Pre-redirection info |  | The presence of this IE indicates the UE support of radio access technologies that the UE could be directed to |  |
| Support of E-UTRA FDD | TRUE |  | E-UTRA-FDD |
| Support of E-UTRA TDD | TRUE |  | E-UTRA-TDD |

Table 8.1.3.7.3.3-3: MEASUREMENT CONTROL (step 6Aa1, Table 8.1.3.7.3.2-1)

| Derivation Path: 36.508, clause 4.7B.1 Table 4.7B.1-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CHOICE Measurement type | Inter-RAT measurement |  |  |
| CHOICE report criteria | Inter-RAT measurement reporting criteria |  |  |
| Parameters required for each event | 1 entry |  |  |
| Inter-RAT event identity | 3c |  |  |
| Threshold own system | Not present |  |  |
| W | Not present |  |  |
| DPCH Compressed mode status info |  |  | UTRA CM-Support |
| TGPS reconfiguration CFN | (Current CFN + (256 – TTI/10msec))mod 256 |  |  |
| Transmission gap pattern sequence | 1 entry |  |  |
| TGPSI | 1 |  |  |
| TGPS Status Flag | activate |  |  |
| TGCFN | (Current CFN + (256 – TTI/10msec))mod 256 |  |  |
| DPCH Compressed mode status info | Not Present |  | UTRA CM-Not Supported |

| Condition | Explanation |
| --- | --- |
| UTRA CM-Supported | UTRAN Compressed Mode configuration supported |
| UTRA CM-Not Supported | UTRAN Compressed Mode configuration not supported |

Table 8.1.3.7.3.3-4: RRC CONNECTION RELEASE (step 15, Table 8.1.3.7.3.2-1)

| Derivation path: 34.108 default RRC CONNECTION RELEASE in section 9.1.1 for UTRA FDD or 9.1.2 for UTRA TDD |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| N308 | 1 |  |  |
| Redirection info |  |  |  |
| Frequency info | Omitted |  |  |
| Inter-RAT info | E-UTRA |  |  |
| E-UTRA target info |  |  |  |
| E-UTRA Target Frequency Info List | 1 Entry |  |  |
| FDD |  |  | E-UTRA-FDD |
| DL Carrier frequency | The DL Carrier frequency of Cell 1 |  |  |
| Exclude-listed cells per freq list | Omitted |  |  |
| DL Carrier frequency | 65535 |  | Band > 64 |
| TDD |  |  | E-UTRA-TDD |
| DL Carrier frequency | The DL Carrier frequency of Cell 1 |  |  |
| Exclude-listed cells per freq list | Omitted |  |  |
| DL Carrier frequency | 65535 |  | Band > 64 |
| E-UTRA Target Frequency Info extension List |  |  | Band > 64 |
| EARFCN extension | EARFCN of the downlink Cell 1 carrier frequency |  |  |

Table 8.1.3.7.3.3-5: PHYSICAL CHANNEL RECONFIGURATION  (step 6a1, Table 8.1.3.7.3.2-1)

| Derivation path: 36.508, clause 4.7B.1 Table 4.7B.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| CHOICE UL/DL Mode | UL and DL, UL only or DL only (depending on the UE capability) |  |  |
| Downlink compressed mode method | SF/2(or not sent, depending on the UE capability) |  |  |
| Uplink compressed mode method | SF/2(or not sent, depending on the UE capability) |  |  |

Table 8.1.3.7.3.3-6: SERVICE REJECT (step 18b5, Table 8.1.3.7.3.2-1)

| Derivation Path: 36.508 table 4.7.2-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EMM cause | ‘01101111’B | Protocol error, unspecified |  |

<End of modified section>