3GPP TSG RAN WG5 Meeting #104	R5-244677

# Maastricht, Netherlands, 19 - 23 August 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2904 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of 6.3J for the uncompleted contents |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CAICT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ATG-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | MU/TT and message contents are still FFS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | MU/TT and message contents were updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case cannot be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3J |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Unchanged sections omitted >>

6.3J	Output power dynamics for ATG

6.3J.1	Minimum output power for ATG

6.3J.1.1	Test purpose

To verify the UE's ability to transmit with a broadband output power below the value specified in the test requirement when the power is set to a minimum value.

6.3J.1.2	Test applicability

This test case applies to all types of NR UE release 18 and forward that support NR standalone ATG.

6.3J.1.3	Minimum conformance requirements

The minimum controlled output power of the UE is defined as the power in the channel bandwidth for all transmit bandwidth configurations (resource blocks) when the power is set to a minimum value.

The minimum output power is defined as the mean power in at least one sub-frame (1 ms). The minimum output power shall not exceed the values specified in Table 6.3J.1.3-1for ATG UE with omni-directional antenna and in Table 6.3J.1.3-2 for ATG UE with antenna array.

Table 6.3J.1.3-1: Minimum output power for ATG UE with omni-directional antenna

| Channel bandwidth | (MHz) | 5,10,15,20 | 25,30,35,40,45,50 | 60,70,80,90,100 |
| --- | --- | --- | --- | --- |
| REF_SCS | (kHz) | 15 |  | 30 |
| Minimum output power | (dBm) | -15 | -15+10log10 (BWChannel /20) | -15+10log10 (BWChannel /20) |
| Measurement bandwidth | (MHz) | MBW=REF_SCS*(12*NRB+1)/1000 |  |  |
| NOTE: The minimum output power value is rounded to the nearest number down to one decimal point. |  |  |  |  |

Table 6.3J.1.3-2: Minimum output power for ATG UE with antenna array

| Channel bandwidth | (MHz) | 5,10,15,20 | 25,30,35,40,45,50 | 60,70,80,90,100 |
| --- | --- | --- | --- | --- |
| REF_SCS | (kHz) | 15 |  | 30 |
| Minimum output power | (dBm) | -19 | -19+10log10 (BWChannel /20) | -19+10log10 (BWChannel /20) |
| Measurement bandwidth | (MHz) | MBW=REF_SCS*(12*NRB+1)/1000 |  |  |
| NOTE: The minimum output power value is rounded to the nearest number down to one decimal point. |  |  |  |  |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.3J.1.

6.3J.1.4	Test description

6.3J.1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in Table 6.3J.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.3J.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A for minimum output power | Modulation | RB allocation (NOTE 1) |
| 1 | test case | DFT-s-OFDM QPSK | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.1 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.3J.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.3J.1.4.3.

6.3J.1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.3J.1.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "down" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step to ensure that the UE transmits at its minimum output power.

3.	Measure the sum of power of all antenna connectors (for ATG UE with omni-directional antennas) or of all TAB connectors (for ATG UE with antenna array) in the associated measurement channel bandwidth specified in Table 6.3J.1.5-1 or Table 6.3J.1.5-2 for the specific channel bandwidth under test. The period of measurement shall be at least the continuous duration of one active sub-frame (1ms) and in the uplink symbols. For TDD symbols with transient periods are not under test.

6.3J.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 with following exception.

Table 6.3J.1.4.3-1: PUSCH-Config

Derivation Path: TS 38.508-1 [5], Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED

6.3J.1.5	Test requirement

The minimum output power, derived in step 3 shall not exceed the values specified in Table 6.3J.1.5-1 or Table 6.3J.1.5-2.

Table 6.3J.1.5-1: Minimum output power for ATG UE with omni-directional antenna

| Channel bandwidth (MHz) | Minimum output power (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- |
| 5 | -15+TT | 4.515 |
| 10 | -15+TT | 9.375 |
| 15 | -15+TT | 14.235 |
| 20 | -15+TT | 19.095 |
| 25 | -14+TT | 23.955 |
| 30 | -13.2+TT | 28.815 |
| 35 | -12.6+TT | 33.855 |
| 40 | -12+TT | 38.895 |
| 45 | -11.5+TT | 43.575 |
| 50 | -11+TT | 48.615 |
| 60 | -10.2+TT | 58.35 |
| 70 | -9.6+TT | 68.07 |
| 80 | -9+TT | 78.15 |
| 90 | -8.5+TT | 88.23 |
| 100 | -8+TT | 98.31 |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.3J.1.5-3 |  |  |

Table 6.3.1.5-2: Minimum output power for ATG UE with antenna array

| Channel bandwidth (MHz) | Minimum output power (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- |
| 5 | -19+TT | 4.515 |
| 10 | -19+TT | 9.375 |
| 15 | -19+TT | 14.235 |
| 20 | -19+TT | 19.095 |
| 25 | -18+TT | 23.955 |
| 30 | -17.2+TT | 28.815 |
| 35 | -16.6+TT | 33.855 |
| 40 | -16+TT | 38.895 |
| 45 | -15.5+TT | 43.575 |
| 50 | -15+TT | 48.615 |
| 60 | -14.2+TT | 58.35 |
| 70 | -13.6+TT | 68.07 |
| 80 | -13+TT | 78.15 |
| 90 | -12.5+TT | 88.23 |
| 100 | -12+TT | 98.31 |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.3.1.5-3 |  |  |

Table 6.3J.1.5-3: Test Tolerance (Minimum output power for ATG)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 1.0 dB | 1.3 dB |
| 40MHz < BW ≤ 100MHz | 1.3 dB | 1.3 dB |

6.3J.2	Transmit OFF power for ATG

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- The referenced clause 6.3J.3 is FFS

6.3J.2.1	Test purpose

To verify that the UE transmit OFF power is lower than the value specified in the test requirement.

An excess Transmit OFF power potentially increases the Rise Over Thermal (RoT) and therefore reduces the cell coverage area for other UEs.

6.3J.2.2	Test applicability

The requirements of this test apply in test case 6.3J.3 Transmit ON/OFF time mask for ATG to all types of NR UE release 18 and forward that support NR standalone ATG.

6.3J.2.3	Minimum conformance requirements

Transmit OFF power is defined as the mean power in the channel bandwidth when the transmitter is OFF. The transmitter is considered OFF when the UE is not allowed to transmit on any of its ports.

The Transmit OFF power is defined as the mean power in a duration of at least one sub-frame (1ms) excluding any transient periods. The Transmit OFF power shall not exceed the values specified in Table 6.3J.2.3-1.

Table 6.3J.2.3-1: Transmit OFF power for ATG

| Channel bandwidth | (MHz) | 3,5,10,15,20,25,30,35,40,45,50 | 60,70,80,90,100 |
| --- | --- | --- | --- |
| REF_SCS | (kHz) | 15 | 30 |
| Transmit OFF power | (dBm) | -50 |  |
| Measurement bandwidth | (MHz) | MBW=REF_SCS*(12*NRB+1)/1000 |  |
| NOTE: “NRB” in the formula is the maximum transmission bandwidth configuration as defined in Table 5.3.2-1. |  |  |  |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.3J.2 and 6.3.2.

6.3J.2.4	Test description

This test is covered by clause 6.3J.3 Transmit ON/OFF time mask for ATG.

6.3J.2.5	Test requirement

The requirement for the Transmit OFF power for ATG is the sum of power of all antenna connectors (for ATG UE with omni-directional antennas) or of all TAB connectors (for ATG UE with antenna array) and shall not exceed the values specified in Table 6.3J.2.5-1.

Table 6.3J.2.5-1: Transmit OFF power for ATG

| Channel bandwidth (MHz) | Transmit OFF power (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- |
| 5 | -50+TT | 4.515 |
| 10 | -50+TT | 9.375 |
| 15 | -50+TT | 14.235 |
| 20 | -50+TT | 19.095 |
| 25 | -50+TT | 23.955 |
| 30 | -50+TT | 28.815 |
| 35 | -50+TT | 33.855 |
| 40 | -50+TT | 38.895 |
| 45 | -50+TT | 43.575 |
| 50 | -50+TT | 48.615 |
| 60 | -50+TT | 58.35 |
| 70 | -50+TT | 68.07 |
| 80 | -50+TT | 78.15 |
| 90 | -50+TT | 88.23 |
| 100 | -50+TT | 98.31 |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.3J.2.5-2 |  |  |

Table 6.3J.2.5-2: Test Tolerance (Transmit OFF power for ATG)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 1.5 dB | 1.8 dB |
| 40MHz < BW ≤ 100MHz | 1.7 dB | 1.8 dB |

6.4	Transmit signal quality

<< End of changes >>