3GPP TSG-RAN5 Meeting #104	R5-244713

# Maastricht, Netherlands, 19th Aug 2024 – 23th Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4575 | rev |  | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction and extension the scope of MBS Broadcast test case 14.1.1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_MBS_5MBS_5MBUSA-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Test coverage enhancement: searchspaceMCCH/ searchspaceMTCH set to zero is not tested in any MBS broadcast test case. The default configuration is searchspaceMCCH/ searchspaceMTCH = 1(same as searchspace for CSS). - It is not clear that when the SS start broadcasting MBSBroadcastConfiguration. - si-SchedulingInfo-v1700 is same as the default configuration in 38.508-1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add “searchspaceMCCH/ searchspaceMTCH set to zero” in title, TP and message content. For NR Cell 1, it uses SSB#1, searchspacezero is slot#1&#2, so the mcch-Window also set to slot#1&#2. 2. Clarify that SS start broadcasting MBSBroadcastConfiguration in the preamble. 3. si-SchedulingInfo-v1700 refer to TS 38.508-1 [4], Table 4.6.3-173AA 4. “UE is made not interested in receiving MBS Broadcast services” Moves to preamble. 5. update the timer to 10s in step5. Although the spec does not limit the timing for UE acquisiting MCCH information from not insterested to MBS broadcast services to insterested to MBS broadcast services, 30s is quiet long. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Insufficient test coverage. - Configuration is unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.1.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

14.1.1.2	MBS Broadcast/ MCCH Information Acquisition/ becoming interested to receive MBS broadcast services / searchspaceMCCH and searchspaceMTCH set to zero

14.1.1.2.1	Test Purpose (TP)

(1)

with { UE in NR RRC_IDLE state and camped on a cell providing SIB20 and not interested to receive MBS broadcast services and searchspaceMCCH/ searchspaceMTCH set to zero }

ensure that {

when { UE is becoming interested to receive one MBS broadcast service }

then { UE acquires the MBSBroadcastConfiguration message and starts MBS reception for this MBS broadcast service }

}

14.1.1.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.331, clauses 5.9.2.2. Unless otherwise stated these are Rel-17 requirements.

[TS 38.331, clause 5.9.2.2]

A UE shall apply the MCCH information acquisition procedure upon becoming interested to receive MBS broadcast services. A UE interested to receive MBS broadcast services shall apply the MCCH information acquisition procedure upon entering the cell providing SIB20 (e.g. upon power on, following UE mobility), upon receiving SIB20 of an SCell via dedicated signalling and upon receiving a notification that the MCCH information has changed due to the start of new MBS service(s).

14.1.1.2.3	Test description

14.1.1.2.3.1	Pre-test conditions

System Simulator:

-	System information combination NR-20 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cell 1.

-	SS starts broadcasting MBSBroadcastConfiguration in NR cell 1.

UE:

-	UE is made not interested in receiving MBS Broadcast services.-	None.

Preamble:

-	The UE is in state 1N-A on NR Cell 1 (serving cell) according to TS 38.508-1 [4] Table 4.4A.2-1 with Test Mode = on to activate UE TEST MODE C and Test Loop Function = off.

-	UE is made not interested in receiving MBS Broadcast services

14.1.1.2.3.2	Test procedure sequence

Table 14.1.1.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS updates MBSBroadcastConfiguration message to include the configuration for MBS Service ID ‘000002’H | - | - | - | - |
| 2 | The SS transmits MCCH information change notification due to the start of new MBS service. | - | - | - | - |
| 3 | Wait until SS stops transmitting MCCH information change notification. | - | - | - | - |
| 4 | UE is made interested in receiving MBS Broadcast service with MBS Service ID ‘000002’H. (Note 1) | - | - | - | - |
| 5 | Wait for 10s to ensure that UE have acquired the updated MBSBroadcastConfiguration in step 1 due to becoming interested to receiving MBS Broadcast service with MBS Service ID ‘000002’H. | - | - | - | - |
| 6-14a2 | Steps 1 to 9a2 of the generic procedures described in TS 38.508-1 [4] subclause 4.5.4.2-3 are performed with condition UE TEST LOOP MODE C. | - | - | - | - |
| - | Exception: Step 15 is repeated 5 times. | - | - | - | - |
| 15 | The SS transmits an MBS Packet on the MTCH with LCID=2 and g-RNTI = ’0002’H. | <-- | MBS Packet. | - | - |
| 16 | The SS transmits a UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST message. | <-- | NR RRC: DLInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST | - | - |
| 17 | UE responds with UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE. | --> | NR RRC: ULInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE | - | - |
| 18 | Check: Is the number of reported MBS Packets received on the MTCH in step 17 greater than zero? | - | - | 1 | P |
| Note 1: The request may be performed by MMI or AT command. |  |  |  |  |  |

14.1.1.2.3.3	Specific message contents

Table 14.1.1.2.3.3-1: SIB1 of NR Cell 1 (preamble and all steps, Table 14.1.1.2.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| servingCellConfigCommon | ServingCellConfigCommonSIB | Table 14.1.1.2.3.3-2 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| si-SchedulingInfo-v1700 | SI-SchedulingInfo-v1700 | TS 38.508-1 [4], Table 4.6.3-173AA 1 entry for SIB20 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.1.1.2.3.3-2: ServingCellConfigCommonSIB (Table 14.1.1.2.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-169 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommonSIB ::= SEQUENCE { |  |  |  |
| downlinkConfigCommon | DownlinkConfigCommonSIB | Table 14.1.1.2.3.3-3 |  |
| } |  |  |  |

Table 14.1.1.2.3.3-3: DownlinkConfigCommonSIB (Table 14.1.1.2.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-53 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DownlinkConfigCommonSIB ::= SEQUENCE { |  |  |  |
| initialDownlinkBWP | BWP-DownlinkCommon | Table 14.1.1.2.3.3-4 |  |
| } |  |  |  |

Table 14.1.1.2.3.3-4: BWP-DownlinkCommon (Table 14.1.1.2.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-10 with condition InitialBWP_SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-DownlinkCommon ::= SEQUENCE { |  |  |  |
| pdcch-ConfigCommon CHOICE { |  |  |  |
| setup | PDCCH-ConfigCommon | Table 14.1.1.2.3.3-4A |  |
| } |  |  |  |
| } |  |  |  |

Table 14.1.1.2.3.3-4A: PDCCH-ConfigCommon (Table 14.1.1.2.3.3-4)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-96 with condition MBS_Broadcast |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCCH-ConfigCommon ::= SEQUENCE { |  |  |  |
| searchSpaceMCCH-r17 | 0 |  |  |
| searchSpaceMTCH-r17 | 0 |  |  |
| } |  |  |  |

Table 14.1.1.2.3.3-4B: SIB20 of NR Cell 1 (preamble and all steps, Table 14.1.1.2.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.2-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB20-r17 ::= SEQUENCE { |  |  |  |
| mcch-Config-r17 SEQUENCE { |  |  |  |
| mcch-WindowStartSlot-r17 | 1 |  |  |
| mcch-WindowDuration-r17 | sl2 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.1.1.2.3.3-5: MBSBroadcastConfiguration (preamble and step 1, Table 14.1.1.2.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-5ABA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MBSBroadcastConfiguration-r17 := SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| mbsBroadcastConfiguration-r17 SEQUENCE { |  |  |  |
| mbs-SessionInfoList-r17 | MBS-SessionInfoList-Service1 | Table 14.1.1.2.3.3-6 | Preamble |
|  | MBS-SessionInfoList-Service1and2 | Table 14.1.1.2.3.3-7 | Step 1 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.1.1.2.3.3-6: MBS-SessionInfoList-Service1 (Table 14.1.1.2.3.3-5)

| Derivation Path: TS 38.508-1 [4], Table 4.6.7-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MBS-SessionInfoList-r17 ::= SEQUENCE (SIZE (1..maxNrofMBS-Session-r17)) OF MBS-SessionInfo-r17 { | 1 entry |  |  |
| MBS-SessionInfo-r17[1] SEQUENCE { |  | entry 1 |  |
| mbs-SessionId-r17 | TMGI-r17 with condition Service1 | Table 14.1.1.2.3.3-8 |  |
| g-RNTI-r17 | ’0001’H |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.1.1.2.3.3-7: MBS-SessionInfoList-Service1and2 (Table 14.1.1.2.3.3-5)

| Derivation Path: TS 38.508-1 [4], Table 4.6.7-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MBS-SessionInfoList-r17 ::= SEQUENCE (SIZE (1..maxNrofMBS-Session-r17)) OF MBS-SessionInfo-r17 { | 2 entries |  |  |
| MBS-SessionInfo-r17[1] SEQUENCE { |  | entry 1 |  |
| mbs-SessionId-r17 | TMGI-r17 with condition Service1 | Table 14.1.1.2.3.3-8 |  |
| g-RNTI-r17 | ’0001’H |  |  |
| mrb-ListBroadcast-r17 SEQUENCE (SIZE (1..maxNrofMRB-Broadcast-r17)) OF MRB-InfoBroadcast-r17 { | 1 entry |  |  |
| MRB-InfoBroadcast-r17[1] SEQUENCE { |  | entry 1 |  |
| pdcp-Config-r17 SEQUENCE { |  |  |  |
| pdcp-SN-SizeDL-r17 | Not present |  |  |
| headerCompression-r17 CHOICE { |  |  |  |
| notUsed | NULL |  |  |
| } |  |  |  |
| t-Reordering-r17 | Not present |  |  |
| } |  |  |  |
| rlc-Config-r17 SEQUENCE { |  |  |  |
| logicalChannelIdentity-r17 | 1 |  |  |
| sn-FieldLength-r17 | Not present |  |  |
| t-Reassembly-r17 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| mtch-SchedulingInfo-r17 | Not present |  |  |
| mtch-NeighbourCell-r17 | Not present |  |  |
| pdsch-ConfigIndex-r17 | Not present |  |  |
| mtch-SSB-MappingWindowIndex-r17 | Not present |  |  |
| } |  |  |  |
| MBS-SessionInfo-r17[2] SEQUENCE { |  | entry 2 |  |
| mbs-SessionId-r17 | TMGI-r17 with condition Service2 | Table 14.1.1.2.3.3-8 |  |
| g-RNTI-r17 | ’0002’H |  |  |
| mrb-ListBroadcast-r17 SEQUENCE (SIZE (1..maxNrofMRB-Broadcast-r17)) OF MRB-InfoBroadcast-r17 { | 1 entry |  |  |
| MRB-InfoBroadcast-r17[1] SEQUENCE { |  | entry 1 |  |
| pdcp-Config-r17 SEQUENCE { |  |  |  |
| pdcp-SN-SizeDL-r17 | Not present |  |  |
| headerCompression-r17 CHOICE { |  |  |  |
| notUsed | NULL |  |  |
| } |  |  |  |
| t-Reordering-r17 | Not present |  |  |
| } |  |  |  |
| rlc-Config-r17 SEQUENCE { |  |  |  |
| logicalChannelIdentity-r17 | 2 |  |  |
| sn-FieldLength-r17 | Not present |  |  |
| t-Reassembly-r17 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| mtch-SchedulingInfo-r17 | Not present |  |  |
| mtch-NeighbourCell-r17 | Not present |  |  |
| pdsch-ConfigIndex-r17 | Not present |  |  |
| mtch-SSB-MappingWindowIndex-r17 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.1.1.2.3.3-8: TMGI-r17 (Table 14.1.1.2.3.3-6, Table 14.1.1.2.3.3-7)

| Derivation Path: TS 38.508-1 [4], Table 4.6.7-9 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| TMGI-r17 ::= SEQUENCE { |  |  |  |  |
| plmn-Id-r17 CHOICE { |  |  |  |  |
| plmn-Index-r17 |  | 1 |  |  |
| } |  |  |  |  |
| serviceId-r17 |  | ‘000002’H | OCTET STRING (SIZE (3)) | Service2 |
|  |  | ‘000001’H | OCTET STRING (SIZE (3)) | Service1 |
| } |  |  |  |  |
| Condition | Explanation |  |  |  |
| Service1 | Broadcast MBS Service with MBS service id ‘000001’H |  |  |  |
| Service2 | Broadcast MBS Service with MBS service id ‘000002’H |  |  |  |

Table 14.1.1.2.3.3-9: ACTIVATE TEST MODE (preamble, Table 14.1.1.2.3.2-1)

Derivation Path: TS 36.508 [6], Table 4.7A-1, condition UE TEST LOOP MODE C

Table 14.1.1.2.3.3-10: CLOSE UE TEST LOOP (step 14a1, Table 14.1.1.2.3.2-1)

| Derivation Path: TS 38.508-1 [4], 4.7A.2, condition UE TEST LOOP MODE C and Broadcast MRB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UE test loop mode C LB setup |  | MRB ID |  |
| MRB ID | ‘0 0 0 0 0 0 0 1 0 0 0 0 0 0 0 0 1 0 0 0 0 0 0 0’B | Bit1 of Octet1 = 1: Broadcast MRB. Bit4 – bit1 of Octet2 = 0 0 0 0 and bit8 of Octet3 = 1: Identity of the logical channel of broadcast MTCH is 2. |  |

Table 14.1.1.2.3.3-11: UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST (step 16, Table 14.1.1.2.3.2-1)

Derivation Path: TS 36.508 [6], Table 4.7A-9

Table 14.1.1.2.3.3-12: Physical layer parameters for DCI format 4_0 (all steps, Table 14.1.1.2.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.3.6.1.5.1-1, condition MCCH-RNTI |  |  |  |
| --- | --- | --- | --- |
| Parameter | Value | Value in binary | Condition |
| MCCH change notification | MSB indicates no new MBS service(s) start. LSB indicates no modification of MCCH information other than the change caused by start of new MBS service(s). | “00” | NOT Step 2 |
|  | MSB indicates a new MBS service start. LSB indicates no modification of MCCH information other than the change caused by start of new MBS service(s). | “10” | Step 2 |

<End of modified section 1>