3GPP TSG-RAN5 Meeting #104	R5-244809

# Maastricht, Netherlands, 19th Aug. – 23rd Aug. 2024

Title:							Testability analysis on UL power setting in FR2 Random Access

Source:			Anritsu

Agenda Item:			5.4.20

Document for:			Endorsement

# 1.	Introduction

Uplink power setting in FR2 Random Access test cases is known as an issue of FR2 RRM test cases. However, this known issue has not been discussed because it depends on relative power tolerance which was under discussion in FR2 TRx due to testability issue. This paper provides testability analysis on the uplink power setting to update the status of the known issue.

# 2.	Discussion

In TS 38.533 [1], FR2 Random Access test cases 5.3.2.2.1 and 5.3.2.2.2 are incomplete for sub-tests 2, 3, 7, and sub-tests 3, 4, respectively.

5.3.2.2.1	EN-DC FR2 contention based random access

Editor's note: This test case is incomplete for Test 2, 3 and 7. The following aspects are either missing or not yet determined:

-	The settable window for first preamble uplink power and the uplink calibration process are FFS.

-	The test requirement for absolute uplink power is FFS.

-	The test requirement for relative uplink power is FFS.

-	The uncertainty value and test requirement for PRACH timing are in [ ]

-	The results of the TT analysis are provisional until the corresponding MU values are agreed

5.3.2.2.2	EN-DC FR2 non-contention based random access

Editor's note: This test case is incomplete for Test 3 and 4. The following aspects are either missing or not yet determined:

-	The settable window for first preamble uplink power and the uplink calibration process are FFS.

-	The test requirement for absolute uplink power is FFS.

-	The test requirement for relative uplink power is FFS.

-	The uncertainty value and test requirement for PRACH timing are in [ ]

-	The results of the TT analysis are provisional until the corresponding MU values are agreed

According to test procedures and test requirements, following testability analyses are required for the incomplete sub-tests.

- Analysis of test requirement for absolute power tolerance when the first preamble is set to 0.6 dBm.

- Analysis of test requirement for relative power tolerance when the preambles are ramped up by 2 dB from the first preamble.

Observation 1: Required testability analyses are test requirements for absolute power tolerance of the first preamble and relative power tolerance of the preambles after the first preamble.

# 2.1.	Analysis on absolute power tolerance of the first preamble

Table 6.3.4.2.3-1: Absolute power tolerance

| Power Range | Tolerance |
| --- | --- |
| Pint ≥ P ≥ Pmin | ± 14.0 dB |
| Pmax ≥ P > Pint | ± 12.0 dB |

According to Table 6.3.4.2.3-1 [2], the minimum requirement of absolute power tolerance is defined for two power ranges Pint ≥ P ≥ Pmin and Pmax ≥ P > Pint. Pmin is the minimum output power and is defined as -13 dBm in Table 6.3.1.3.2-1 [2]. Pmax is defined as the minimum peak EIRP, and Table 6.2.1.1.3.3-1 [2] defines them as 22.4 dBm, 20.6 dBm, 18.7 dBm, 16.0 dBm for FR2a, FR2b, FR2c, FR2d, respectively. Pint is defined as Pmax – 12.0 dB in Table 6.3.4.2.3-2 [2] and is calculated as 10.4 dBm, 8.6 dBm, 6.7 dBm, 4.0 dBm for each frequency sub-range. From the above values of Pmin, Pint, Pmax, the power range Pint ≥ P ≥ Pmin is applicable for all frequency sub-ranges when the first preamble is set to 0.6 dBm. Therefore, the minimum requirement of absolute power tolerance is ±14.0 dB, and it means that possible UL level is -13.4 dBm (= 0.6 dBm – 14.0 dB) to +14.6 dBm (= 0.6 dBm + 14.0 dB). In addition to the minimum requirement, tolerance due to MU is needed to define test requirement.

Observation 2: The minimum requirement of absolute power tolerance is ±14.0 dB when the first preamble is set to 0.6 dBm.

Now, there is a testability issue for the possible UL level. For 100 MHz bandwidth used in the test case, agreed testable UL level in Table 6.3.1.5-2 [2] is -10.6 dBm, -5.5 dBm, [-4.5] dBm for FR2a, FR2b, FR2c, respectively. Therefore, the minimum possible UL level of -13.4 dBm cannot be tested due to low UL level.

Observation 3: The minimum possible UL level of the first preamble cannot be tested due to low UL level.

# 2.2.	Analysis on relative power tolerance of the preambles after the first preamble

Table 6.3.4.3.3-1: Relative power tolerance, Pint ≥ P ≥ Pmin

| Power step ∆P (Up or down) (dB) | All combinations of PUSCH and PUCCH, PUSCH/PUCCH and SRS transitions between sub-frames, PRACH (dB) |
| --- | --- |
| ΔP < 2 | ±5.0 |
| 2 ≤ ΔP < 3 | ±6.0 |
| 3 ≤ ΔP < 4 | ±7.0 |
| 4 ≤ ΔP < 10 | ±8.0 |
| 10 ≤ ΔP < 15 | ±10.0 |
| 15 ≤ ΔP | ±11.0 |
| NOTE: The requirements apply with ue-BeamLockFunction enabled. |  |

Table 6.3.4.3.3-2: Relative power tolerance, PUMAX ≥ P > Pint

| Power step ∆P (Up or down) (dB) | All combinations of PUSCH and PUCCH, PUSCH/PUCCH and SRS transitions between sub-frames, PRACH (dB) |
| --- | --- |
| ΔP < 2 | ±3.0 |
| 2 ≤ ΔP < 3 | ±4.0 |
| 3 ≤ ΔP < 4 | ±5.0 |
| 4 ≤ ΔP < 10 | ±6.0 |
| 10 ≤ ΔP < 15 | ±8.0 |
| 15 ≤ ΔP | ±9.0 |
| NOTE 1: The requirements apply with ue-BeamLockFunction enabled. NOTE 2: For PUSCH to PUSCH transitions with the allocated resource blocks fixed in frequency and no transmission gaps other than those generated by downlink subframes, guard periods: for a power step ΔP = 1 dB, the relative power tolerance for transmission is ± 1.0 dB. |  |

According to clause 6.3.4.3.3 [2], the minimum requirement of relative power tolerance is also defined for two power ranges Pint ≥ P ≥ Pmin and PUMAX ≥ P > Pint. Pmin and Pint are the same as defined for absolute power tolerance. PUMAX is the measured configured maximum UL level, and the possible minimum PUMAX is calculated as below.

Table 1 Calculation process of possible minimum PUMAX in FR2 Random Access test cases

|  | FR2a | FR2b | FR2c | FR2d | Reference, expression |
| --- | --- | --- | --- | --- | --- |
| Minimum peak EIRP | 22.4 dBm | 20.6 dBm | 18.7 dBm | 16.0 dBm | Table 6.2.1.1.3.3-1 [2] |
| MPRnarrow | 0 dB |  |  |  | Clause 6.2.2.3.3 [2] Note 1 |
| MPRWT | 2 dB |  |  |  | Table 6.2.2.3.3-1 [2] Note 1, 2 |
| MPR | 2 dB |  |  |  | max(MPRnarrow, MPRWT) |
| T(MPR) | 1.5 dB |  |  |  | Table 6.2.4.3-1 [2] |
| MBR | 0.75 dB | 0.75 dB | 0.5 dB | 0.7 dB | Table 6.2.1.1.3.3-4 and Table 6.2.1.1.3.3-5 [2] |
| PUMAX | 18.15 dBm | 16.35 dBm | 14.7 dBm | 11.8 dBm | Minimum peak EIRP – MPR – T(MPR) – MBR |
| Note 1 Based on the message contents, PRACH for this test case is assumed as equivalent to RB allocation of 12@0. Note 2 According to 6.2.2.0 [2], PRACH is assumed as DFT-s-OFDM QPSK for MPR calculation. |  |  |  |  |  |

Now, there are two issues for the minimum requirement of relative power tolerance. Based on the clause 6.3.4.3.3 [2], the minimum requirement of relative power tolerance is applicable when the UL levels before and after the ramping up are within the power range of Pmin of -13 dBm to PUMAX of 18.15 dBm, 16.35 dBm, 14.7 dBm, 11.8 dBm for each frequency sub-ranges. The first issue is for the lower boundary. The minimum possible UL level before the ramping up is -13.4 dBm (= 0.6 dBm – 14.0 dB) which is lower than Pmin of -13 dBm. The second issue is for the upper boundary. The maximum possible UL level after the ramping up is +16.6 dBm (= 0.6 dBm + 14.0 dB + 2 dB) which is higher than PUMAX for FR2b to FR2d. Due to the above issues, there is a concern that the minimum requirement of relative power tolerance cannot be applied to the ramping up.

Observation 4: The minimum requirement of relative power tolerance may not be applicable when the preambles are ramped up by 2 dB from the first preamble.

# 3.	Conclusion

Testability analysis for the incomplete sub-tests in FR2 Random Access test cases is summarized as below.

Observation 1: Required testability analyses are test requirements for absolute power tolerance of the first preamble and relative power tolerance of the preambles after the first preamble.

Observation 2: The minimum requirement of absolute power tolerance is ±14.0 dB when the first preamble is set to 0.6 dBm.

Observation 3: The minimum possible UL level of the first preamble cannot be tested due to low UL level.

Observation 4: The minimum requirement of relative power tolerance may not be applicable when the preambles are ramped up by 2 dB from the first preamble.

Considering Observation 3 and Observation 4, it is assumed that the incomplete sub-tests are not testable. We recommend to not test these sub-tests.

Proposal 1: It is not recommended testing of sub-tests 2, 3, and 7 of RRM test case 5.3.2.2.1 EN-DC FR2 contention based random access.

Proposal 2: It is not recommended testing of sub-tests 3 and 4 of RRM test case 5.3.2.2.2 EN-DC FR2 non-contention based random access.

# 4.	References

[1] TS 38.533, “User Equipment (UE) conformance specification; Radio Resource Management (RRM)”, V18.3.0 (2024-06)

[2] TS 38.521-2, “User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone”, V18.3.0 (2024-06)