3GPP TSG-RAN5 Meeting #104	R5-244874

# Maastricht, Netherlands, Aug 19-23, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2930 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating Time mask for switching across three uplink bands test case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MC_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Test case 6.3C.3.6 is designed for testing the uplink switching time mask for configurations such as CA_nXA_nYA-nZA, which is a three-band configuration and each band configured with one uplink carrier. Thus the clause title is updated to be accurate. And the test configuration table, test procedure, message contents and test requirements need to be updated to correct some errors. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Updating minimum requirement to align with core specification - Updating title of clause 6.3C.3.6 - Test configuration table format is updated: the uplink configurations are listed in the order of bands. The specific modulation and RB allocation are not changed. - Updating test procedure for subtest 1 and subtest 2. Considering UL MIMO is not supported on all SUL bands, in this test case SUL band won't be scheduled to transmit UL MIMO - Updating message contents and test requirements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case 6.3C.3.6 is incorrect |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3C.3.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### <Start of Changes>

##### 6.3C.3.0.6	Time mask for switching across up to four uplink bands

The switching time mask requirements specified in this sub-clause are applicable for an NR SUL band configuration with inter-band CA when the capability supportedBandPairListNR-r18 is present, and are only applicable for uplink switching mechanisms specified in clause [6.1.6] of TS 38.214 [10].

In the NR SUL band configuration with inter-band CA, the number of uplink bands with different carrier frequencies is up to four.  NR UL carrier(s) in each of the up to four uplink bands are capable of one or two transmit antenna connector(s), according to the UE capability FeatureSetUplinkPerCC.

The switching time masks in Figure 6.3C.3.0.6-1 and Figure 6.3C.3.0.6-2 are applicable to each of the uplink band pairs in the SUL band configuration with inter-band CA, and are applicable to uplink transmissions when configured with switchedUL or dualUL by the parameter switchingOptionConfigForBandPair. To simplify the figures, the two bands in different band pairs are denoted as NR band X and band Y. The uplink transmission on either band X or band Y is with one or two transmit antenna connector(s),

–	if NR UL carriers in both bands in one band pair are capable of one transmit antenna connector, 1Tx-1Tx switching is supported for the band pair;

–	if NR UL carrier(s) in one band of one band pair is capable of one transmit antenna connector, and NR UL carrier(s) in the other band of the band pair is capable of two transmit antenna connectors, 1Tx-2Tx switching is supported for the band pair;

–	if NR UL carriers in both bands of one band pair are capable of two transmit antenna connectors, 2Tx-2Tx switching is supported for the band pair.

For each band pair, the switching periods described in Figure 6.3C.3.0.6-1 and Figure 6.3C.3.0.6-2 are located in either NR band X or band Y as indicated in RRC signalling uplinkTxSwitchingBandList [7]. For each band pair, the length of uplink switching period X is indicated by RRC signalling switchingPeriodConfigForBandPair. UE shall be capable to transmit until the beginning of the switching period and after the end of switching period with the exception of transient periods.

Figure 6.3C.3.0.6-1: Time mask for switching between band X and band Y, where the switching period is located in band X

Figure 6.3C.3.0.6-2: Time mask for switching between band X and band Y, where the switching period is located in band Y

The following applies for the uplink switching cases specified in Figure 6.3C.3.0.6-1 and 6.3C.3.0.6-2 in a band pair with switchingOptionConfigForBandPair set to either switchedUL or dualUL:

-	if an uplink switching is triggered for an uplink transmission starting at T0 based on higher layer configuration(s) or DCI(s) received before T0 − Toffset as specified in [10] and the UE is not configured or scheduled with uplink transmissions for a duration of at least the length of uplink switching period of X µs indicated by RRC signalling switchingPeriodConfigForBandPair on any of the carriers band X and band Y before T0,

-	the configuration of the location of the switching period by [uplinkTxSwitchingBandList-r18] is ignored by the UE;

-	transient periods of 10 s are located at the end of the last symbol(s) configured or scheduled on the carrier(s) before T0 and at the start of the first symbol(s) configured or scheduled at T0 on the switch-to carrier(s).

In addition to the requirements in Figure 6.3C.3.0.6-1 and Figure 6.3C.3.0.6-2, the requirements in Figure 6.3C.3.0.6-3 to 6.3C.3.0.6-5 are applicable when dualUL are supported on certain band pair(s) in the SUL band configuration with inter-band CA.

The switching time masks in Figure 6.3C.3.0.6-3 and Figure 6.3C.3.0.6-4 are applicable when dualUL is supported for at least two uplink band pairs in the band configuration. The two band pairs supporting dualUL are denoted as band pairs of {band X and band Z} and {band Y and band Z}. When one transmitter is switched between band X and band Y,

–	As baseline UE behaviour, the UE is not required to transmit on any of the three bands during time period T1 located on band X and band Z, where T1 is the length of switching period for the band pair of band X and band Y, as shown in Figure 6.3C.3.0.6-4.

–	As optional UE behaviours, when the UE indicates band Z in the capability bandIndexUnaffected,

–	if the UE indicates maintainedUL-Trans for band pair of {band X and band Y}, UE shall be capable of uplink transmission on band Z during the switching period that is located on band X, and UE is not required to transmit on band X and Y during time period T1 located on band X, where T1 is the length of switching period for the band pair of band X and band Y, as shown in Figure 6.3C.3.0.6-3;

–	otherwise, theUE is not required to transmit on any of the three bands during the switching period indicated by UE capability periodOnULBands [located on band X and band Z, as shown in Figure 6.3C.3.0.6-4.

In Figure 6.3C.3.0.6-3 and Figure 6.3C.3.0.6-4, the uplink transmission on band X, band Y and band Z are all with one transmit antenna connector and one antenna port.

Figure 6.3C.3.0.6-3: Time mask for one transmitter switching between band X and band Y, and UE is capable of uplink transmission on band Z during the switching period

The following applies for the uplink switching cases specified in Figure 6.3C.3.0.6-3 in a band pair with switchingOptionConfigForBandPair set to dualUL:

-	if an uplink switching is triggered for an uplink transmission starting at T0 based on higher layer configuration(s) or DCI(s) received before T0 − Toffset as specified in [10] and the UE is not configured or scheduled with uplink transmissions for a duration of at least the length of uplink switching periods indicated by UE capability uplinkTxSwitchingPeriodForBandPair on any of the carriers in band X and band Y before T0,

-	the configuration of the location of the switching period by uplinkTxSwitchingBandList is ignored by the UE;

-	transient periods of 10 s are located at the end of the last symbol(s) configured or scheduled on the carrier(s) before T0 and at the start of the first symbol(s) configured or scheduled at T0 on the switch-to carrier(s).

Figure 6.3C.3.0.6-4: Time mask for one transmitter switching between band X and band Y, and UE is not capable of uplink transmission on band Z during the switching period

The switching time mask in Figure 6.3C.3.0.6-5 is applicable when dualUL is supported for at least one uplink band pair including band X and band Y, and two transmit antenna connectors are supported on at least one uplink band of band Z. When one transmitter is switched between band X and band Z, and across the same time, the other transmitter is switched between Y and band Z, the switching time mask in Figure 6.3C.3.0.6-5 is applicable.

–	As baseline UE behaviour, UE is not required to transmit on any of the three bands during time period with the larger one of swithching period T2 and T3, where T2 is the length of switching period for the band pair of band X and band Z, and T3 is the length of switching period for the band pair of band Y and band Z.

–	As optional UE behaviour when UE additionally reports band pair of {band X and band Y} and band Z in the capability uplinkTxSwitchingAdditionalPeriodDualUL-List, UE is not required to transmit on any of the three bands during time period indicated by UE capability switchingAdditionalPeriodDualUL.

In Figure 6.3C.3.0.6-5, the uplink transmission on band X and band Y is with one transmit antenna connector and one antenna port, and the uplink transmission on band Z is with two transmit antenna connectors and two antenna ports. The switching period location is configured according to [7], and band Z is with the highest priority according to the RRC configuration uplinkTxSwitchingBandList.

Figure 6.3C.3.0.6-5: Time mask for one transmitter switching between band X and band Z, and one transmitter switching between band Y and band Z

The following applies for the uplink switching case specified in Figure 6.3C.3.0.6-5 and with uplinkTxSwitchingOptionForBandPair set to dualUL for at least one band pair

-	if uplink switching on a band pair is triggered for an uplink transmission starting at T0 based on higher layer configuration(s) or DCI(s) received before T0 − Toffset as specified in [10] and the UE is not configured or scheduled with uplink transmissions for a duration of at least the maximum of the lengths of uplink switching periods indicated by RRC signalling switchingPeriodConfigForBandPair on any of the carriers in band X, band Y and band Z before T0 on any switched-to carrier

-	the configuration of the location of the switching period and the priority of bands in the uplinkTxSwitchingBandList are ignored by the UE

-	transient periods of 10 s are located at the end of the last symbol(s) configured or scheduled on the switched-from carrier(s) before T0 on any switched-to and at the start of the first symbol(s) configured or scheduled at T0 on the switch-to carrier(s)

The requirements in this sub-clause apply for the case of synchronized network deployment for the uplink bands.

### <Unchanged Skipped>

#### 6.3C.3.6	Time mask for switching across three uplink bands (3CC)

6.3C.3.6.1	Test purpose

To verify that the time mask for switching across three uplink bands for an NR SUL band configuration with inter-band CA meets the requirements given in 6.3C.3.0.6 and 6.3C.3.0.6a.

The time mask for switching across three uplink bands defines the transient period(s) allowed when the capability supportedBandPairListNR-r18 is present.

6.3C.3.6.2	Test purpose

6.3C.3.6.2	Test applicability

This test case applies to all types of NR UE release 18 and forward that support 3 bands SUL band configuration with inter-band CA and dynamic UL Tx switching.

6.3C.3.6.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.3C.3.0.6 and 6.3C.3.0.6a.

6.3C.3.6.4	Test description

6.3C.3.6.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configuration specified in 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration, and are shown in table 6.3C.3.6.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A.2 and A.3. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.3C.3.6.4.1-1: Test Configuration Table for switching across three uplink bands (3CC)

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  | Normal |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Mid range for SUL carrier Mid range for Non-SUL carriers |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Highest for SUL carrier Highest for Non-SUL carriers |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | 15kHz for SUL carrier, lowest supported SCS for Non-SUL carrier |  |  |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration for NUL band 1 |  |  | Uplink Configuration for NUL band 2 |  | SUL Configuration |  |
|  |  | Modulation | RB allocation (Note 3) |  | Modulation | RB allocation (Note 3) | Modulation | RB allocation (Note 3) |
| 1 | NA | CP-OFDM QPSK | Inner Full |  | CP-OFDM QPSK | Inner Full | CP-OFDM QPSK | Inner Full |
| NOTE 1: For an inter-band CA with SUL configuration CA_nA_nB-nC, the two NUL bands are noted as Band_NUL1 and Band_NUL2, and the SUL band is noted as Band_SUL. NOTE 2: Test Channel Bandwidths are checked separately for each SUL band combination, the applicable channel bandwidths are specified in Table 5.5C-1. NOTE 3: The specific configuration of each RB allocation is defined in Table 6.1-1 |  |  |  |  |  |  |  |  |

-	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.8 for TE diagram and section A.3.2.5.5 for UE diagram.

-	The parameter setting for the cell are set up according to the TS 38.508-1 [5] subclause 4.4.3.

-	Downlink signals are initially setup according to Annex C.0, C.1, C.2 and uplink signals according to Annex G.0, G.1, G.2, and G.3.0 with consideration of supplementary uplink physical channels.

6.3C.3.6.4.2	Test procedure

In this test case, for the inter-band CA with SUL configuration CA_nA_nB-nC, the two NUL bands are noted as Band_NUL1 and Band_NUL2, and the SUL band is noted as Band_SUL. One uplink carrier is configured on each band.

Sub test 1 is to verify the time mask for switching between Band_NUL1 and Band_SUL with the switching period located on Band_NUL1, and switching between Band_SUL and Band_NUL2 with the switching period located on Band_NUL2 (Figure 6.3C.3.0.6-1 and 6.3C.3.0.6-2). UE transmits uplink on one band of the band pair in the band combination before and after uplink switching. The uplink transmission on either Band_NUL1 or Band_NUL2 is with two transmit antenna connectors, and the uplink transmission on Band_SUL is with one transmit antenna connector.

Sub test 2 is to verify the time mask for switching with one transmitter switching between Band_NUL1 and Band_SUL, and one transmitter switching between Band_NUL2 and Band_SUL. The switching period is located in Band_NUL1 and Band_NUL2 (Figure 6.3C.3.0.6-5). The test is performed only when UE is capable of simultaneous uplink transmission on the Band_NUL1 and Band_NUL2 with dualUL declared for the band pair. The uplink transmission on Band_NUL1 and Band_NUL2 is with one transmit antenna connector and one antenna port, and the uplink transmission on Band_SUL is with two transmit antenna connectors and two antenna ports.

- Sub test 1:

1.1 Configure SCC according to Annex C.0, C.1, C.2 for all downlink physical channels.

1.2 The SS shall configure SCC as per TS 38.508-1 [5] clause 5.5.1. Message contents are defined in clause 6.3C.3.6.4.2.

1.3 SS activates SCC by sending the activation MAC CE (Refer TS 38.321 [18], clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133 [19], clause 9.3).

1.4 SS send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

1.5	SS sends uplink scheduling information via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.3C.3.6.4.1-1 on Band_NUL1 on slot n-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with the condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2.

1.6	The SS sends uplink scheduling information via DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.3C.3.6.4.1-1 on Band_SUL starting on slot n and slot m, with slot m+1 being uplink slots for Band_NUL2 and m ≥ n+20 when SCS=15kHz (m ≥ n+40 when SCS=30 kHz, m ≥ n+80 when SCS=60 kHz). Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

1.7	Measure the sum of output power of UE PUSCH transmission for Band_NUL1 over all antenna connectors during slot n-1 excluding a transient period of 10 µs and a Switching period X µs in the end of slot n-1. The length of uplink switching period X is indicated by RRC signalling switchingPeriodConfigForBandPair.

1.8	Measure the sum of output power of UE PUSCH transmission on Band_SUL over all antenna connectors during slot n and slot m excluding a transient period of 10 µs in the beginning of slot n and excluding a transient period of 10 µs in the end of slot m. The length of uplink switching period X is indicated by RRC signalling switchingPeriodConfigForBandPair.

1.9	SS sends uplink scheduling information via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.3C.3.5.4.1-1 on Band_NUL2 on slot m+1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with the condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2.

1.10	Measure the sum of output power of UE PUSCH transmission for Band_NUL2 over all antenna connectors during slot m+1 excluding a transient period of 10 µs and a Switching period X µs in the beginning of slot m+1.

- Sub test 2:

2.1 Configure SCC according to Annex C.0, C.1, C.2 for all downlink physical channels.

2.2 The SS shall configure SCC as per TS 38.508-1 [5] clause 5.5.1. Message contents are defined in clause 6.3C.3.6.4.2 with switching period location is configured on Band_NUL1 and Band_NUL2.

2.3 SS activates SCC by sending the activation MAC CE (Refer TS 38.321 [18], clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133 [19], clause 9.3).

2.4 SS send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

2.5	SS sends uplink scheduling information via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.3C.3.6.4.1-1 on both Band_NUL1 and Band_NUL2 on slot n-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.6	The SS sends uplink scheduling information via DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.3C.3.6.4.1-1 on Band_SUL starting on slot n and slot m, with slot m+1 being uplink slots for Band_NUL1 and Band_NUL2 and m ≥ n+20 when SCS=15kHz (m ≥ n+40 when SCS=30 kHz, m ≥ n+80 when SCS=60 kHz). Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with the condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2.

2.7	Measure the sum of output power of UE PUSCH transmission for Band_NUL1 and Band_NUL2 over all antenna connectors during slot n-1 excluding a transient period of 10 µs and a Switching period T µs in the end of slot n-1. The length of uplink switching period T is the larger one of swithching period T2 and T3, where T2 is the length of switching period for the band pair of Band_NUL1 and Band_SUL, and T3 is the length of switching period for the band pair of Band_NUL2 and Band_SUL indicated by RRC signalling switchingPeriodConfigForBandPair. If UE additionally reports band pair of {Band_NUL1 and Band_NUL2} and Band_SUL in the capability uplinkTxSwitchingAdditionalPeriodDualUL-List, the length of T is indicated by UE capability switchingAdditionalPeriodDualUL

2.8	Measure the sum of output power of UE PUSCH transmission on Band_SUL over all antenna connectors during slot n and slot m excluding a transient period of 10 µs in the beginning of slot n and in the end of slot m.

2.9	SS sends uplink scheduling information via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.3C.3.6.4.1-1 on both Band_NUL1 and Band_NUL2 on slot m+1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.10	Measure the sum of output power of UE PUSCH transmission on Band_NUL1 and Band_NUL2 over all antenna connectors during slot m+1 excluding a transient period of 10 µs and a Switching period T µs in the beginning of slot m+1. The length of uplink switching period T is the larger one of swithching period T2 and T3, where T2 is the length of switching period for the band pair of Band_NUL1 band and Band_SUL, and T3 is the length of switching period for the band pair of Band_NUL2 band and Band_SUL indicated by RRC signalling switchingPeriodConfigForBandPair. If UE additionally reports band pair of {Band_NUL1 and Band_NUL2} and Band_SUL in the capability uplinkTxSwitchingAdditionalPeriodDualUL-List, the length of T is indicated by UE capability switchingAdditionalPeriodDualUL.

6.3C.3.6.4.3		Message contents

Message contents in initial conditions are according to TS 38.508-1 [5] subclause 4.3.6.1.1.2 ensuring UL/SUL indicator in Table 4.3.6.1.1.2-1 with condition SUL, subclause 4.6 ensuring Table 4.6.1-28 with condition SUL AND (RF OR RRM), Tables 4.6.3-14 with condition SUL_SUL for SUL carrier and following exceptions:

Table 6.3C.3.6.4.3-1: RRCReconfiguration

| Derivation Path: TS 38.508-1, Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) Table 6.3C.3.6.4.3-2 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 6.3C.3.6.4.3-2: CellGroupConfig

| Derivation Path: 38.508-1[5], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| spCellConfigDedicated | ServingCellConfig | Table 6.3C.3.6.4.3-3 |  |
| } |  |  |  |
| reportUplinkTxDirectCurrentMoreCarrier-r17 | Not present |  |  |
| prioSCellPRACH-OverSP-PeriodicSRS-r17 | Not present |  |  |
| ncr-FwdConfig-r18 | Not present |  |  |
| autonomousDenialParameters-r18 | Not present |  |  |
| nonCollocatedTypeMRDC-r18 | Not present |  |  |
| nonCollocatedTypeNR-CA-r18 | Not present |  |  |
| uplinkTxSwitchingMoreBands-r18 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| uplinkTxSwitchingBandList-r18 SEQUENCE (SIZE (1..maxSimultaneousBands)) OF FreqBandIndicatorNR { | 3 entries |  |  |
| FreqBandIndicatorNR[1] | FreqBandIndicatorNR of Band_SUL | entry 1 |  |
| FreqBandIndicatorNR[2] | FreqBandIndicatorNR of Band_NUL1 | entry 2 |  |
| FreqBandIndicatorNR[3] | FreqBandIndicatorNR of Band_NUL2 | entry 3 |  |
| } |  |  |  |
| uplinkTxSwitchingBandPairList-r18 SEQUENCE (SIZE (1.. maxULTxSwitchingBandPairs)) OF UplinkTxSwitchingBandPairConfig-r18 { | 3 entries |  |  |
| UplinkTxSwitchingBandPairConfig-r18[1] SEQUENCE { |  | entry 1 |  |
| bandInfoUL1-r18 | 2 |  |  |
| bandInfoUL1-r18 | 3 |  |  |
| switchingOptionConfigForBandPair-r18 | switchedUL |  | Subtest1 |
|  | dualUL |  | Subtest2 |
| switching2T-Mode-r18 | Not present |  |  |
| switchingPeriodConfigForBandPair-r18 | Not present | 210 us | pc_switchingPeriod _210us_r18 |
|  | n35us | 35 us | pc_switchingPeriod _35us_r18 |
|  | n140us | 140 us | pc_switchingPeriod _140us_r18 |
| } |  |  |  |
| UplinkTxSwitchingBandPairConfig-r18[2] SEQUENCE { |  | entry 2 |  |
| bandInfoUL1-r18 | 1 |  |  |
| bandInfoUL1-r18 | 3 |  |  |
| switchingOptionConfigForBandPair-r18 | switchedUL |  |  |
| switching2T-Mode-r18 | Not present |  |  |
| switchingPeriodConfigForBandPair-r18 | Not present | 210 us | pc_switchingPeriod _210us_r18 |
|  | n35us | 35 us | pc_switchingPeriod _35us_r18 |
|  | n140us | 140 us | pc_switchingPeriod _140us_r18 |
| } |  |  |  |
| UplinkTxSwitchingBandPairConfig-r18[3] SEQUENCE { |  | entry 3 |  |
| bandInfoUL1-r18 | 1 |  |  |
| bandInfoUL1-r18 | 2 |  |  |
| switchingOptionConfigForBandPair-r18 | SwitchedUL |  |  |
| switching2T-Mode-r18 | Not present |  |  |
| switchingPeriodConfigForBandPair-r18 | Not present | 210 us | pc_switchingPeriod _210us_r18 |
|  | n35us | 35 us | pc_switchingPeriod _35us_r18 |
|  | n140us | 140 us | pc_switchingPeriod _140us_r18 |
| } |  |  |  |
| } |  |  |  |
| UplinkTxSwitchingAssociatedBandDualUL-List-r18 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Subtest1 | When UE performing subtest 1 in this test case |
| Subtest2 | When UE performing subtest 2 in this test case |

Table 6.3C.3.6.4.3-3: ServingCellConfig

| Derivation Path: TS 38.508-1 Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| supplementaryUplink SEQUENCE { |  |  |  |
| initialUplinkBWP | BWP-UplinkDedicated | Table 6.3C.3.6.4.3-4 |  |
| } |  |  |  |
| } |  |  |  |

Table 6.3C.3.6.4.3-4: BWP-UplinkDedicated

| Derivation Path: TS 38.508-1, Table 4.6.3-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkDedicated ::= SEQUENCE { |  |  |  |
| pucch-Config | Not present |  |  |
| pusch-Config CHOICE { |  |  |  |
| setup | PUSCH-Config |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.3C.3.6.4.3-5: FrequencyInfoUL

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-61 FrequencyInfoUL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 23 |  | Subtest1 |
|  | 20 |  | Subtest2 |

Table 6.3C.3.6.4.3-6: FrequencyInfoUL-SIB

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 23 |  | Subtest1 |
|  | 20 |  | Subtest2 |

6.3C.3.6.5	Test requirement

The requirement for the power measured in step 1.7, 1.8, 1.10, 2.7, 2.8, 2.10 of the test procedures shall not exceed the values specified in table 6.3C.3.6.5-1.

Table 6.3C.3.6.5-1: Time mask for switching across three bands for SUL with inter-band CA (On power)

|  | Measured output power |  |
| --- | --- | --- |
| Transmit ON power | Same as Test ID 1 of Table 6.2D.2.5-1 | Band_NUL1, Band_NUL2 in Test step 1.7 and 1.10 |
|  | Same as Test ID 19 in appropriate tables in subclause 6.2C.4.5 | Band_SUL in Test step 1.8 |
|  | Same as test ID 1 in table 6.2A.2.1.5-1 | Sum of power on Band_NUL1 and Band_NUL2 in Test step 2.7 and 2.10 |
|  | Same as Test ID 1 of Tables 6.2D.2_1.5-1 and 6.2D.2_1.5-1a | Band_SUL in Test step 2.8 |
| Note 1: TT is same as in clause 6.2D.2.5 and 6.2C.4.5 for NUL bands and SUL band in Subtest 1 Note 2: TT is same as in clause 6.2A.2.1.5 and 6.2D.2_1.5 for NUL bands and SUL band in Subtest 2 |  |  |

### <End of Changes>