3GPP TSG-RAN5 Meeting #104	R5-244875

# Maastricht, Netherlands, Aug 19-23, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2931 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding new uplink switching time mask test case 6.3C.3.6_1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MC_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For the uplink switching time mask requirements across three uplink bands: a new test case is needed for configurations such as CA_nXC_nYA-nZA or CA_nXA_nYC-nZA , which is a three-band configuration and at least one band configured with more than one uplink carrier. Since a maximum of two uplink CCs can be simultaneously transmitted, in this case the uplink carriers on two NUL bands cannot be scheduled at same time. Thus only the requirements of Fig 6.3C.3.0.6-1 and 6.3C.3.0.6-1 can be tested in this test case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introducing new test case 6.3C.3.6_1 Time mask for switching across three uplink bands (more than 3CC) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | uplink switching time mask requirements across three uplink bands can’t be tested for configurations such as CA_nXC_nYA-nZA or CA_nXA_nYC-nZA |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3C.3.6_1 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TR 38.905 CR 0933 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TP in R5-244878 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### <Start of Changes>

6.3C.3.6.5	Test requirement

The requirement for the power measured in step 1.7, 1.8, 1.9, 2.6, 2.7, 2.9 of the test procedures shall not exceed the values specified in table 6.3C.3.6.5-1.

Table 6.3C.3.6.5-1: Time mask for switching across three bands for SUL with inter-band CA (On power)

|  | Measured output power |  |
| --- | --- | --- |
| Transmit ON power | Same as Test ID 1 of Table 6.2D.2.5-1 as appropriate | Carrier 1, Carrier 2 in Test step 1,7, 1.10 |
|  | Same as Test ID 1 of Table 6.2D.2_1.5-1 and 6.2D.2_1.5-1a as appropriate | Carrier 3 in Test step 1,8, 2.8 |
|  | Same as Test ID 23 of Table 6.2A.2.1.5-1 | Sum of power on Carrier1 and Carrier2 in Test step 2.7, 2.10 |
| NOTE1: TT is FFS |  |  |

#### 6.3C.3.6_1	Time mask for switching across three uplink bands (more than 3CC)

6.3C.3.6_1.1	Test purpose

To verify that the time mask for switching across three uplink bands for an NR SUL band configuration with inter-band CA meets the requirements given in 6.3C.3.0.6 and 6.3C.3.0.6a.

The time mask for switching across three uplink bands defines the transient period(s) allowed when the capability supportedBandPairListNR-r18 is present.

6.3C.3.6_1.21	Test purpose

6.3C.3.6_1.2	Test applicability

This test case applies to all types of NR UE release 18 and forward that support 3 bands SUL band configuration with inter-band CA (more than 3CC) and dynamic UL Tx switching.

6.3C.3.6.3_1	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.3C.3.0.6 and 6.3C.3.0.6a.

6.3C.3.6_1.4	Test description

6.3C.3.6_1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configuration specified in 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration, and are shown in table 6.3C.3.6.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A.2 and A.3. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.3C.3.6_1.4.1-1: Test Configuration Table for switching across three uplink bands (more than 3CC)

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  | Normal |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Mid range for SUL carrier Mid range for Non-SUL carriers |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Highest for SUL carrier Highest for Non-SUL carriers (Highest NRB_agg for Non-SUL aggregate carriers) |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | 15kHz for SUL carrier, lowest supported SCS for Non-SUL carrier |  |  |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration for NUL band 1 |  |  | Uplink Configuration for NUL band 2 |  | SUL Configuration |  |
|  |  | Modulation | RB allocation (Note 3) |  | Modulation | RB allocation (Note 3) | Modulation | RB allocation (Note 4) |
| 1 | NA | CP-OFDM QPSK | Inner Full |  | CP-OFDM QPSK | Inner Full | CP-OFDM QPSK | Inner Full |
| NOTE 1: For an inter-band CA with SUL configuration CA_nA_nB-nC, the two NUL bands are noted as Band_NUL1 and Band_NUL2, and the SUL band is noted as Band_SUL. NOTE 2: Test Channel Bandwidths are checked separately for each SUL band combination, the applicable channel bandwidths are specified in Table 5.5C-1. NOTE 3: For the NR band configured with one single carrier, the specific configuration of each RB allocation is defined in Table 6.1-1. For the NR band configured with two contiguous carriers, the specific configuration of each RB allocation is defined in Table 6.1A-1a. NOTE 4: The specific configuration of each RB allocation is defined in Table 6.1-1 |  |  |  |  |  |  |  |  |

Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.8 for TE diagram and section A.3.2.5.5 for UE diagram.

-	The parameter setting for the cell are set up according to the TS 38.508-1 [5] subclause 4.4.3.

-	Downlink signals are initially setup according to Annex C.0, C.1, C.2 and uplink signals according to Annex G.0, G.1, G.2, and G.3.0 with consideration of supplementary uplink physical channels.

6.3C.3.6_1.4.2	Test procedure

In this test case, for the inter-band CA with SUL configuration CA_nA_nB-nC, the two NUL bands are noted as Band_NUL1 and Band_NUL2, and the SUL band is noted as Band_SUL. One uplink carrier is configured on the SUL band. At least one NUL band is configured with two contiguous uplink carriers depending on the specific band configuration.

This test case is to verify the time mask for switching between Band_NUL1 and Band_SUL with the switching period located on Band_NUL1, and switching between Band_SUL and Band_NUL2 with the switching period located on Band_NUL2 (Figure 6.3C.3.0.6-1 and 6.3C.3.0.6-2). UE transmits uplink on one band of the band pair in the band combination before and after uplink switching. The uplink transmission on either Band_NUL1 or Band_NUL2 is with two transmit antenna connectors, and the uplink transmission on Band_SUL is with one transmit antenna connector.

1 Configure SCC according to Annex C.0, C.1, C.2 for all downlink physical channels.

2 The SS shall configure SCC as per TS 38.508-1 [5] clause 5.5.1. Message contents are defined in clause 6.3C.3.6_1.4.2.

3 SS activates SCC by sending the activation MAC CE (Refer TS 38.321 [18], clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133 [19], clause 9.3).

4 SS send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

5	SS sends uplink scheduling information via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.3C.3.6_1.4.1-1 on Band_NUL1 on slot n-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with the condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2.

6	The SS sends uplink scheduling information via DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.3C.3.6_1.4.1-1 on Band_SUL starting on slot n and slot m, with slot m+1 being uplink slots for Band_NUL2 and m ≥ n+20 when SCS=15kHz (m ≥ n+40 when SCS=30 kHz, m ≥ n+80 when SCS=60 kHz). Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

7	Measure the sum of output power of UE PUSCH transmission for Band_NUL1 over all antenna connectors during slot n-1 excluding a transient period of 10 µs and a Switching period X µs in the end of slot n-1. The length of uplink switching period X is indicated by RRC signalling switchingPeriodConfigForBandPair.

8	Measure the sum of output power of UE PUSCH transmission on Band_SUL over all antenna connectors during slot n and slot m excluding a transient period of 10 µs in the beginning of slot n and excluding a transient period of 10 µs and a Switching period X µs in the end of slot m. The length of uplink switching period X is indicated by RRC signalling switchingPeriodConfigForBandPair.

9	SS sends uplink scheduling information via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.3C.3.6_1.4.1-1 on Band_NUL2 on slot m+1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with the condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2.

10	Measure the sum of output power of UE PUSCH transmission for Band_NUL2 over all antenna connectors during slot m+1 excluding a transient period of 10 µs in the beginning of slot m+1.

6.3C.3.6_1.4.3		Message contents

Message contents in initial conditions are according to TS 38.508-1 [5] subclause 4.3.6.1.1.2 ensuring UL/SUL indicator in Table 4.3.6.1.1.2-1 with condition SUL, subclause 4.6 ensuring Table 4.6.1-28 with condition SUL AND (RF OR RRM), Tables 4.6.3-14 with condition SUL_SUL for SUL carrier and following exceptions:

Table 6.3C.3.6_1.4.3-1: RRCReconfiguration

| Derivation Path: TS 38.508-1, Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) Table 6.3C.3.6_1.4.3-2 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 6.3C.3.6_1.4.3-2: CellGroupConfig

| Derivation Path: 38.508-1[5], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| spCellConfigDedicated | ServingCellConfig | Table 6.3C.3.6_1.4.3-3 |  |
| } |  |  |  |
| reportUplinkTxDirectCurrentMoreCarrier-r17 | Not present |  |  |
| prioSCellPRACH-OverSP-PeriodicSRS-r17 | Not present |  |  |
| ncr-FwdConfig-r18 | Not present |  |  |
| autonomousDenialParameters-r18 | Not present |  |  |
| nonCollocatedTypeMRDC-r18 | Not present |  |  |
| nonCollocatedTypeNR-CA-r18 | Not present |  |  |
| uplinkTxSwitchingMoreBands-r18 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| uplinkTxSwitchingBandList-r18 SEQUENCE (SIZE (1..maxSimultaneousBands)) OF FreqBandIndicatorNR { | 3 entries |  |  |
| FreqBandIndicatorNR[1] | FreqBandIndicatorNR of Band_NUL1 | entry 1 |  |
| FreqBandIndicatorNR[2] | FreqBandIndicatorNR of Band_NUL2 | entry 2 |  |
| FreqBandIndicatorNR[3] | FreqBandIndicatorNR of Band_SUL | entry 3 |  |
| } |  |  |  |
| uplinkTxSwitchingBandPairList-r18 SEQUENCE (SIZE (1.. maxULTxSwitchingBandPairs)) OF UplinkTxSwitchingBandPairConfig-r18 { | 3 entries |  |  |
| UplinkTxSwitchingBandPairConfig-r18[1] SEQUENCE { |  | entry 1 |  |
| bandInfoUL1-r18 | 1 |  |  |
| bandInfoUL1-r18 | 2 |  |  |
| switchingOptionConfigForBandPair-r18 | switchedUL |  |  |
| switching2T-Mode-r18 | Not present |  |  |
| switchingPeriodConfigForBandPair-r18 | Not present | 210 us | pc_switchingPeriod _210us_r18 |
|  | n35us | 35 us | pc_switchingPeriod _35us_r18 |
|  | n140us | 140 us | pc_switchingPeriod _140us_r18 |
| } |  |  |  |
| UplinkTxSwitchingBandPairConfig-r18[2] SEQUENCE { |  | entry 2 |  |
| bandInfoUL1-r18 | 1 |  |  |
| bandInfoUL1-r18 | 3 |  |  |
| switchingOptionConfigForBandPair-r18 | switchedUL |  |  |
| switching2T-Mode-r18 | Not present |  |  |
| switchingPeriodConfigForBandPair-r18 | Not present | 210 us | pc_switchingPeriod _210us_r18 |
|  | n35us | 35 us | pc_switchingPeriod _35us_r18 |
|  | n140us | 140 us | pc_switchingPeriod _140us_r18 |
| } |  |  |  |
| UplinkTxSwitchingBandPairConfig-r18[3] SEQUENCE { |  | entry 3 |  |
| bandInfoUL1-r18 | 2 |  |  |
| bandInfoUL1-r18 | 3 |  |  |
| switchingOptionConfigForBandPair-r18 | SwitchedUL |  |  |
| switching2T-Mode-r18 | Not present |  |  |
| switchingPeriodConfigForBandPair-r18 | Not present | 210 us | pc_switchingPeriod _210us_r18 |
|  | n35us | 35 us | pc_switchingPeriod _35us_r18 |
|  | n140us | 140 us | pc_switchingPeriod _140us_r18 |
| } |  |  |  |
| } |  |  |  |
| UplinkTxSwitchingAssociatedBandDualUL-List-r18 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.3C.3.6_1.4.3-3: ServingCellConfig

| Derivation Path: TS 38.508-1 Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| supplementaryUplink SEQUENCE { |  |  |  |
| initialUplinkBWP | BWP-UplinkDedicated | Table 6.3C.3.6_1.4.3-4 |  |
| } |  |  |  |
| } |  |  |  |

Table 6.3C.3.6_1.4.3-4: BWP-UplinkDedicated

| Derivation Path: TS 38.508-1, Table 4.6.3-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkDedicated ::= SEQUENCE { |  |  |  |
| pucch-Config | Not present |  |  |
| pusch-Config CHOICE { |  |  |  |
| setup | PUSCH-Config |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.3C.3.6_1.4.3-5: FrequencyInfoUL

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-61 FrequencyInfoUL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 23 |  | Subtest1 |
|  | 20 |  | Subtest2 |

Table 6.3C.3.6_1.4.3-6: FrequencyInfoUL-SIB

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 23 |  | Subtest1 |
|  | 20 |  | Subtest2 |

6.3C.3.6_1.5	Test requirement

The requirement for the power measured in step 7, 8, 10 of the test procedures shall not exceed the values specified in table 6.3C.3.6_1.5-1.

Table 6.3C.3.6_1.5-1: Time mask for switching across three bands for SUL with inter-band CA (On power)

|  | Measured output power |  |
| --- | --- | --- |
| Transmit ON power | Same as Test ID 1 of Table 6.2D.2.5-1 | Band_NUL1 in step 7 with single uplink carrier configured; Band_NUL2 in step 10 with single uplink carrier configured |
|  | Same as test ID 1 in table 6.2H.1.2.5-1 | Band_NUL1 in step 7 with contiguous 2UL CA configured; Band_NUL2 in step 10 with contiguous 2UL CA configured |
|  | Same as Test ID 19 in appropriate tables in subclause 6.2C.4.5 | Band_SUL in Test step 8 |
| Note 1： TT is same as in 6.2D.2 for NUL band with single UL carrier or 6.2H.1.2 for NUL band with intra-band UL CA carriers. Same TT as 6.2C.4 for SUL carrier. |  |  |

### <End of Changes>