3GPP TSG-RAN5 Meeting #104	R5-244950

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-2 | CR | 0384 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of Test Case 5.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NIST |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCPTT-ConTest |  |  |  |  |  | Date: |  |  | 2024-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The conformance requirements needed to be updated from Rel-13 to Rel-14 for this test case |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated the conformance requirements from Rel-13 to Rel-14 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A tester may falsely pass a UE or falsely fail a UE when comparing performance to old conformance requirements |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<START OF CHANGES>

## 5.2	Configuration / Group Creation / Group Regroup Creation / Group Regroup Teardown

5.2.1	Test Purpose (TP)

(1)

with { UE (MCPTT Client) attached to EPS services }

ensure that {

when { the MCPTT user requests formation of a new MCPTT group }

then { the UE (MCPTT client) initiates creation of the group by sending a HTTP PUT request }

}

(2)

with { UE (MCPTT Client) having access to at least two MCPTT groups }

ensure that {

when { the MCPTT user requests the groups to be combined }

then { the UE (MCPTT client) initiates creation of the temporary group by sending a HTTP POST request }

}

(3)

with { UE (MCPTT Client) having access to a temporary group }

ensure that {

when { the MCPTT user requests temporary group tear down }

then { the UE (MCPTT client) initiates tear down of the temporary group by sending a HTTP DELETE request }

}

5.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.481 clauses 6.3.2.2.1, 6.3.2.2.2, 6.3.14.1, 6.3.14.2, 6.3.15.1 and 6.3.15.2; TS 33.180 clauses 7.3.2. Unless otherwise stated these are Rel-14 requirements.

[TS 24.481 clause 6.3.2.2.1]

In order to create a group document, a GC shall create an XML document of the application usage specified in subclause 7.2.1 and shall send the XML document to the network according to procedures specified in IETF RFC 4825 [22] "Create or Replace a Document". The GC shall set the Request-URI of the HTTP PUT request to an XCAP URI in users' tree where the XUI is set to a group creation XUI configuration parameter.

[TS 24.481 clause 6.3.2.2.2]

In order to create a group document, a GMC shall perform the procedures in subclause 6.3.2.2.1 specified for GC.

[TS 24.481 clause 6.3.14.1]

This procedure enables a GMC to initiate creation of a temporary MCS group by combining MCS groups.

NOTE:	The temporary MCS group formation procedure does not ensure that the MCSs of the temporary MCS group are the same as MCSs of each constituent MCS group of the temporary MCS group.

[TS 24.481 clause 6.3.14.2]

In order to form a temporary MCS group, a GMC shall send a HTTP POST request according to procedures specified in IETF RFC 2616 [21] and subclause 6.2.3. In the HTTP POST request, the GMC:

a)	shall set the Request-URI to an XCAP URI:

1)	in users tree where the XUI is set to a group creation XUI configuration parameter; and

2)	with the document selector identifying the temporary MCS group to be created; and

b)	shall include an application/vnd.3gpp.GMOP+xml MIME body containing a GMOP document requesting group regroup creation specified in subclause 7.3.4.3, with a <group> element containing a group document for an MCS group. In the group document, the GMC shall include the <on-network-temporary> element according to subclause 7.2. In the <on-network-temporary> element, the GMC shall include <constituent-MCPTT-group-IDs> element according to subclause 7.2. In the <constituent-MCPTT-group-IDs> element, the GMC shall include one <constituent-MCPTT-group-ID> element according to subclause 7.2 for each MCS group to be combined.

Upon reception of an HTTP 2xx response to the sent HTTP POST request, the GMC shall consider the temporary MCS group formation as successful.

Upon reception of an HTTP 409 (Conflict) response with at least one <alt-value> element in the <uniqueness-failure> error element, the GMC may repeat procedures of the present subclause and identify the temporary MCS group being formed with an MCS Group ID indicated in an <alt-value> element.

[TS 24.481 clause 6.3.15.1]

This procedure enables a GMC to initiate tear down of a temporary MCS group.

[TS 24.481 clause 6.3.15.2]

In order to tear down a temporary MCS group, the GMC shall send an HTTP DELETE request with Request-URI with an XCAP URI identifying a group document of the temporary MCS group according to procedures specified in IETF RFC 4825 [22] "Delete an Element".

[TS 33.180 clause 7.3.2]

The group creation procedure is described in clause 10.2.3 of 3GPP TS 23.280 [36] and applies to the MCPTT scenario of normal group creation by an MC administrator and user regrouping operations by an authorized user/dispatcher.  To establish the security context for the group, the GMS follows the procedures in clause 5.7 to create a new GMK and GMK-ID.

The encapsulated GMK and GUK-ID is sent to group members by the GMS within a notification message (step 4 in clause 10.2.3 of 3GPP TS 23.280 [36]). The procedure is equivalent to that described in clause 5.7 of this specification.

[TS 33.180 clause 7.3.3.2]

Group Regroup procedures for the MC system are described in clause 10.2.4.1 of 3GPP TS 23.280 [36]. To create the security context for the temporary group, the GMS follows the procedures in clause 5.7, creating a new GMK and GMK-ID for the temporary group.

An encapsulated GMK and GUK-ID is sent to the temporary group members by the GMS within a notification message (step 5 in clause 10.2.4.1 of 3GPP TS 23.280 [36]). The procedure is equivalent to that described in clause 5.7.

5.2.3	Test description

5.2.3.1	Pre-test conditions

Test configuration:

-	Single cell configuration according to TS 36.579-1 [2] clause 5.2.2.2.1.

Preamble:

-	The UE has performed procedure 'Initial registration' as described in TS 36.579-1 [2] clause 5.4.2.

-	UE States at the end of the preamble:

-	The UE is in RRC_IDLE state.

-	The client is registered for MCPTT.

5.2.3.2	Test procedure sequence

Table 5.2.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Void | - | - | - | - |
| - | EXCEPTION: Procedure 'MCX CO communication' as described in TS 36.579-1 [2] clause 5.4.3 is started to establish an RRC connection. | - | - | - | - |
| 2 | Make the UE (MCPTT client) request creation of a new group (group B). (NOTE 1) | - | - | - | - |
| 3 | Check: Does the UE (MCPTT client) correctly perform procedure 'MCX CO Group Creation' as described in TS 36.579-1 [2] Table 5.3.26.3-1 to create group B? | - | - | 1 | P |
| 4-12 | Void | - | - | - | - |
| 13 | Make the UE (MCPTT client) request creation of a temporary group T combining groups A and B. (NOTE 1) | - | - | - | - |
| 14 | Check: Does the UE (MCPTT client) correctly perform procedure 'MCX CO Group Creation' as described in TS 36.579-1 [2] Table 5.3.27.3-1 to create temporary group T? | - | - | 2 | P |
| 14A | Check: Does the UE (MCPTT client) correctly perform procedure 'NW initiated notifications regarding temporary group creation' as described in TS 36.579-1 [2] Table 5.3.22.3-1 to update the group document of group A and retrieve the GMK for the temporary group | - | - | 2 | P |
| 15-20 | Void | - | - | - | - |
| 21 | Make the UE (MCPTT client) request tear down of temporary group T. (NOTE 1) | - | - | - | - |
| 22 | Check: Does the UE (MCPTT client) correctly perform procedure 'MCX CO Group Tear Down' as described in TS 36.579-1 [2] Table 5.3.28.3-1 to delete temporary group T? | - | - | 3 | P |
| 22A | Check: Does the UE (MCPTT client) correctly perform procedure 'NW initiated notifications regarding temporary group tear down' as described in TS 36.579-1 [2] Table 5.3.22.3-1 to update the group document and the GKTP document of group A? | - | - | 3 | P |
| 23-26 | Void | - | - | - | - |
| NOTE 1: This is expected to be done via a suitable implementation dependent mechanism and may be manually or automatically initiated. |  |  |  |  |  |

5.2.3.3	Specific message contents

Table 5.2.3.3-1..16: Void

Table 5.2.3.3-17: HTTP PUT (step 3, Table 5.2.3.2-1;
step 1, TS 36.579-1 [2], Table 5.3.26.3-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.4.4-1, condition GROUPCREATE

Table 5.2.3.3-18: HTTP POST (step 14, Table 5.2.3.2-1;
step 1, TS 36.579-1 [2], Table 5.3.27.3-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.4.3-1, condition TEMPGROUP

Table 5.2.3.3-19: HTTP 200 (OK) (step 14, Table 5.2.3.2-1;
step 2, TS 36.579-1 [2], Table 5.3.27.3-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.4.6-1, condition TEMPGROUP

Table 5.2.3.3-20: HTTP DELETE (step 22, Table 5.2.3.2-1;
step 1, TS 36.579-1 [2], Table 5.3.28.3-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.4.5-1, condition TEMPGROUP

<END OF CHANGES>