3GPP TSG-RAN5 Meeting #104	R5-244965

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-7 | CR | 0065 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of New Test Case 6.7.1 IP Connectivity CO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NIST |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCProtoc16_enh2MCPTT_eMCData2-ConTest |  |  |  |  |  | Date: |  |  | 2024-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There was no IP Connectivity CO test case for MCData |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added IP Connectivity CO test case 6.7.1 and clause 6.7 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | IP Connectivity CO functionality would not be tested for MCData |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7, 6.7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<START OF CHANGES>

## 6.7	IP Connectivity

### 6.7.1	On-network / IP Connectivity / Client Originated (CO)

6.7.1.1	Test Purpose (TP)

(1)

with { UE (MCDATA Client) registered and authorised for MCDATA Service }

ensure that {

when { the MCDATA User requests to initiate a IP Connectivity session }

then { UE (MCDATA Client) sends a request to establish a IP Connectivity session via a SIP INVITE message and then responds to the SIP 200 (OK) message with a SIP ACK message and sends a payload using Generic Routing Encapsulation (GRE) }

}

(2)

with { UE (MCDATA Client) having established a IP Connectivity session }

ensure that {

when { the MCDATA User requests to release the IP Connectivity session }

then { UE (MCDATA Client) sends a SIP BYE message }

}

6.7.1.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.282, clause 20.2.1, TS 24.582, clause 13.1.2. Unless otherwise stated, these are Rel-16 requirements.

[TS 24.282, clause 20.2.1]

When a MCData client receives the request by a user or user application to establish a IP Connectivity session with another MCData client the MCData client shall generate a SIP INVITE request in accordance with 3GPP TS 24.229 [5] with the clarifications given below. The MCData ID of the target MCData client may be explicitly included in the request from the user or user application. If the target MCData ID is not included in the request, the MCData client may implicitly determine the target MCData ID by using the target IP Information included in the request to find a match in the One-to-One communication list of the MCData user profile document as specified in 3GPP TS 24.484 [12]. If the MCData ID of the target MCData client is determined implicitly by the target IP Information included in the request, the client searches in leaves below /<x>/<x>/Common/OnetoOne/UserList/<x>/Entry/IPInformation/<x>Entry/ for a match in the IP Information. The MCData ID is given by matching the user entry.

The MCData client:

1)	shall include the g.3gpp.mcdata.ipconn  media feature tag and the g.3gpp.icsi-ref media feature tag with the value of "urn:urn-7:3gpp-service.ims.icsi.mcdata.ipconn " in the Contact header field of the SIP INVITE request according to IETF RFC 3840 [16];

2)	shall include an Accept-Contact header field containing the g.3gpp.mcdata.ipconn media feature tag along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [8];

3)	shall include an Accept-Contact header field with the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcdata.ipconn" along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [8];

4)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcdata.ipconn" (coded as specified in 3GPP TS 24.229 [5]), in a P-Preferred-Service header field according to IETF RFC 6050 [7] in the SIP INVITE request;

5)	should include the "timer" option tag in the Supported header field;

6)	should include the Session-Expires header field according to IETF RFC 4028 [38]. It is recommended that the "refresher" header field parameter is omitted. If included, the "refresher" header field parameter shall be set to "uac";

7)	shall insert in the SIP INVITE request a MIME resource-lists body with the MCData ID of the invited MCData user, according to rules and procedures of IETF RFC 5366 [18];

8)	shall contain an application/vnd.3gpp.mcdata-info+xml MIME body with the <mcdatainfo> element containing the <mcdata-Params> element with:

a)	the <request-type> element set to a value of "one-to-one-ipconn"; and

b)	if the MCData client is aware of active functional aliases and if an active functional alias is to be included in the SIP INVITE request, the <functional-alias-URI> element set to the URI of the used functional alias;

9)	shall set the Request-URI of the SIP INVITE request to the public service identity identifying the participating MCData function serving the MCData user;

NOTE 1:	The MCData client is configured with public service identity identifying the participating MCData function serving the MCData user.

10)	may include a P-Preferred-Identity header field in the SIP INVITE request containing a public user identity as specified in 3GPP TS 24.229 [5];

11)	shall include an SDP offer according to 3GPP TS 24.229 [5] with the clarifications given in subclause 20.1.1; and

12)	shall send the SIP INVITE request towards the MCData server according to 3GPP TS 24.229 [5].

On receipt of a SIP 2xx response to the SIP INVITE request, the MCData client:

1)	shall send a SIP ACK request as specified in 3GPP TS 24.229 [5];

2)	shall start the SIP Session timer according to rules and procedures of IETF RFC 4028 [38]; and

3)	shall interact with MC Data user or user application.

On receipt of a SIP 4xx response, a SIP 5xx response or a SIP 6xx response to the SIP INVITE request, the MCData client:

1)	shall indicate to the MCData user or user application that the IP Connectivity session could not be established; and

2)	shall send a SIP ACK request as specified in 3GPP TS 24.229 [5].

On receipt of an indication from the media plane indicating that the IP Connectivity session could not be established, the MCData client:

1)	shall generate a SIP BYE request according to 3GPP TS 24.229 [5] with:

a)	Reason code set to "FAILURE_CAUSE";

b)	cause set to "1"; and

c)	text set to "Media bearer or QoS lost";

2)	shall set the Request-URI to the MCData session identity to release; and

3)	shall send a SIP BYE request towards MCData server according to 3GPP TS 24.229 [5].

[TS 24.582, clause 13.1.2]

Upon receiving a request by an MCData user, or an IP packet from an IP application, the MCData client shall follow the procedure in 20.2.1 in 3GPP TS 24.282 [8]. The IP address received in the 200 OK response in this procedure shall be used to establish an IP tunnel. The IP tunnel shall be based on Generic Routing Encapsulation (GRE) as specified in RFC 2784 [19], and as explained in subclause 13.4. The Key field value of each GRE packet header uniquely identifies the IP connectivity session that the GRE packet payload is associated with.

NOTE:	How the client sets the session identifier is implementation specific.

The MCData client shall act as an IP relay for IP traffic between the IP application and the IP tunnel to the far endpoint. Once the IP tunnel is established, the IP applications can exchange IP data. The client that receives the IP packets from the IP application shall perform encapsulation to the tunnelling protocol, while the client that receives IP packets from the IP tunnel shall perform de-encapsulation from the tunnelling protocol before passing the IP data to the IP application.

6.7.1.3	Test description

6.7.1.3.1	Pre-test conditions

Test configuration:

-	Single cell configuration according to TS 36.579-1 [2] clause 5.2.2.2.1.

Preamble:

-	The UE has performed procedure 'Initial registration' as described in TS 36.579-1 [2] clause 5.4.2.

-	UE States at the end of the preamble:

-	The UE is in RRC_IDLE state.

-	The client is registered for MCData.

6.7.1.3.2	Test procedure sequence

Table 6.7.1.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Make the UE (MCData client) establish a IP Connectivity session with another MCData client to send a message. (NOTE 1) | - | - | - | - |
| 2-6 | Check: Does the UE (MCData client) correctly perform steps 1 through 5 of procedure 'CO MCData Call Establishment' as described in TS 36.579-1 [2] Table 5.3C.2.3-1 to establish an IP Connectivity session with another MCData client? | - | - | 1 | P |
| - | EXCEPTION: The communication described in in steps XXX are transmitted over a IP connection that has been established based on Generic Routing Encapsulation (GRE). | - | - | - | - |
| 7 | Check: Does the UE (MCData client) correctly sends a payload packet via Generic Routing Encapsulation (GRE)? | - | - | 1 | P |
| 8 | Make the UE (MCData client) release the IP Connectivity session. (NOTE 1) | - | - | - | - |
| 9 | Check: Does the UE (MCData client) correctly perform procedure 'CO MCData call release' as described in TS 36.579-1 [2] Table 5.3C.6.3-1? (NOTE 2) | - | - | 2 | P |
| 10 | The procedure 'MCX communication release' as described in TS 36.579-1 [2] clause 5.4.14 is performed to release the RRC connection. | - | - | - | - |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. NOTE 2: The procedure does not release the RRC connection. |  |  |  |  |  |

6.7.1.3.3	Specific message contents

Table 6.7.1.3.3-1: SIP INVITE from the UE (step 3, Table 6.7.1.3.2-1;
step 2, TS 36.579-1 [2] Table 5.3C.2.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.5.1-1, condition MCD_1to1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Contact |  |  |  |  |
| addr-spec | SIP URI |  |  |  |
| user-info and host | IP address or FQDN |  |  |  |
| port | protected server port of UE | as assigned during registration |  |  |
| feature-param | "+g.3gpp.mcdata.ipconn" |  |  |  |
| feature-param | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcdata.ipconn" |  |  |  |
| Accept-Contact |  |  |  |  |
| ac-value[1] |  |  |  |  |
| feature-param | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcdata.ipconn" |  |  |  |
| req-param | "require" |  |  |  |
| explicit-param | "explicit" |  |  |  |
| ac-value[2] |  |  |  |  |
| feature-param | "+g.3gpp.mcdata.ipconn" |  |  |  |
| req-param | "require" |  |  |  |
| explicit-param | "explicit" |  |  |  |
| P-Preferred-Service |  |  |  |  |
| Service-ID | "urn:urn-7:3gpp-service.ims.icsi.mcdata.ipconn" |  |  |  |
| Message-body |  |  |  |  |
| MIME body part |  | SDP message |  |  |
| MIME-part-body | As described in Table 6.7.1.3.3-2 |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | MCData-Info as described in Table 6.7.1.3.3-3 |  |  |  |

Table 6.7.1.3.3-2: SDP for SIP INVITE (Table 6.7.1.3.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.1.1-3, condition MCDATA_SDS, SDP_OFFER, SDS_SESSION, MCD_1to1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Media description[1] |  | Media description for data |  |  |
| media description |  | m= line media = message |  |  |
| port | "0" | The client shall add a zero port number value to the media descriptions of the SDP offer, in order to inform network entities that media resources are not requested for the session |  |  |

Table 6.7.1.3.3-3: MCData-Info (Table 6.7.1.3.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.1-3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdata-info |  |  |  |  |
| mcdata-Params |  |  |  |  |
| request-type | "one-to-one-ipconn" |  |  |  |

Table 6.7.1.3.3-4: SIP 200 (OK) from the SS (step 5, Table 6.7.1.3.2-1;
step 4, TS 36.579-1 [2] Table 5.3C.2.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.17.1.2-1, condition INVITE-RSP |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Contact |  |  |  |  |  |  |  |  |  |
| addr-spec |  |  |  |  |  |  |  |  |  |
| user-info and host |  | tsc_MCData_SessionId |  |  |  |  |  |  |  |
| port |  | not present |  |  |  |  |  |  |  |
| feature-param |  | "+g.3gpp.mcdata.ipconn" |  |  |  |  |  |  |  |
| feature-param |  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcdata.ipconn" |  |  |  |  |  |  |  |
| feature-param |  | "isfocus" |  |  |  |  |  |  |  |
| Message-body |  |  |  |  |
| SDP message | As described in Table 6.1.5.9.3-5 |  |  |  |

Table 6.7.1.3.3-5: SDP for SIP 200 (OK) (Table 6.7.1.3.3-4)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.1.2-3, condition MCDATA_SDS, SDP_ANSWER, SDS_SESSION |  |  |  |  |
| --- | --- | --- | --- | --- |
| Media description[1] |  | Media description for data |  |  |
| media description |  | m= line media = message |  |  |
| port | "0" |  |  |  |

Table 6.7.1.3.3-6: GRE encapsulated user data packet (step 7, Table 6.7.1.3.2-1)

| Derivation Path: 24.582 [32], Table 13.4-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| GRE header |  |  |  |
| C | 0 |  |  |
| Reserved | 0 |  |  |
| K | 1 |  |  |
| S | 0 |  |  |
| Reserved | 0000 |  |  |
| Reserved | 00000 |  |  |
| Ver | 000 |  |  |
| Protocol Type | 00 |  |  |
| Key | Any allowed value |  |  |
| Payload packet |  |  |  |
| Payload packet | any allowed value | The payload packet Example: “abcdEFGH” |  |

<END OF CHANGES>