3GPP TSG-RAN5 Meeting #104	R5-244970

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-7 | CR | 0070 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of New Test Case 6.8.4 Message Store Function Synchronization |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NIST |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCProtoc16_enh2MCPTT_eMCData2-ConTest |  |  |  |  |  | Date: |  |  | 2024-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There was no Message Store test cases for MCData |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added Message Store test case 6.8.4 for Message Store Function Synchronization |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Message Store Function Synchronization would not be tested for MCData |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.8.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<START OF CHANGES>

### 6.8.4	On-network / MCData Message Store / Synchronization / Synchronization notifications / Search-based synchronization

6.8.4.1	Test Purpose (TP)

(1)

with { UE (MCData Client)registered and authorized for MCData service }

ensure that {

when { UE (MCData Client) receives a notification about changes in the message store using using the Message Store Function via an HTTP POST message }

then { UE (MCData Client) responds with a HTTP 204 (No Content) message }

}

(2)

with { UE (MCData Client)registered and authorized for MCData service }

ensure that {

when { the MCDATA User requests to search for changes in the message store using the Message Store Function }

then { UE (MCData Client) requests to search for changes in the message store using the Message Store Function via an HTTP POST message }

}

6.8.4.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.282, clauses 21.2.16.2, 21.2.17.1. Unless otherwise stated, these are Rel-16 requirements.

[TS 24.282 clause 21.2.16.2]

Upon receipt of the HTTP POST request from the MCData message store, as per clause 21.2.16.1, the message store client acting as an HTTP server:

1)	shall process the HTTP POST request by following the procedures described in clause 6.22.5 of OMA-TS-REST_NetAPI_NMS-V1_0-20190528-C [66]; and

a)	either use the notification content and the reported "restartToken" and "index" as specified in clause 5.1.5.1 of OMA-TS-REST_NetAPI_NMS-V1_0-20190528-C [66] to have the client’s local message store updated accordingly; or

b)	use the notification as a trigger to subsequently search the MCData message store for the list of changes as specified in clause 21.2.11.1; and

2)	shall generate and send an HTTP response towards the message store function indicating the result of the operation as per clause 6.22.2 of OMA-TS-REST_NetAPI_NMS-V1_0-20190528-C [66].

NOTE:	The notifications about changes in the MCData message store can be used by the message store client to synchronize its local message store with the MCData message store in two distinguished ways which are listed in sub-bullets "a" and "b" above.

[TS 24.282 clause 21.2.17.1]

To search for changes (e.g. newly created objects, recently deleted objects, etc) in the MCData message store using the message store function, the message store client, acting as an HTTP client shall follow the procedure described in clause 21.2.2.1 with following clarification:

1)	shall use the search criterion of "CreatedObjects", "VanishedObjects" or "Flag" in the HTTP POST request as specified in clause 5.1.5.2 and 5.4.2.2  of OMA-TS-REST_NetAPI_NMS-V1_0-20190528-C [66] in order to retrieve from the message store the list of the newly created object, recently deleted object and/or changes to flags respectively.

6.8.4.3	Test description

6.8.4.3.1	Pre-test conditions

Test configuration:

-	Single cell configuration according to TS 36.879-1 [2] clause 5.2.2.2.1.

Preamble:

-	The UE has performed procedure 'Initial registration' as described in TS 36.879-1 [2] clause 5.4.2.

-	UE States at the end of the preamble:

-	The UE is in RRC_IDLE state.

-	The client is registered for MCData.

6.8.4.3.2	Test procedure sequence

Table 6.8.4.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Make the UE (MCData client) create a subscription to notifications about changes in the message store function. (NOTE 1) | - | - | - | - |
| 2 | The UE (MCData client) correctly performs procedure 'Message Store Function Object Upload using HTTP' as described in TS 36.579-1 [2] Table 5.3C.14.2-1 to create a subscription to notifications about changes in the message store function? | - | - | - | - |
| 3 | Check: Does the UE (MCData client) correctly perform procedure 'Message Store Function Post Notification using HTTP' as described in TS 36.579-1 [2] Table 5.3C.19.2-1 for notifications of changes in the message store using the message store function? | - | - | 1 | P |
| 4 | Make the UE (MCData client) search for changes in the message store function. (NOTE 1) | - | - | - | - |
| 5 | Check: Does the UE (MCData client) correctly perform procedure 'Message Store Function Post Request using HTTP' as described in TS 36.579-1 [2] Table 5.3C.17.2-1 to search for changes in the message store function? | - | - | 2 | P |
| 6 | Make the UE (MCData client) delete a subscription to notifications about changes in the message store function. (NOTE 1) | - | - | - | - |
| 7 | The UE (MCData client) correctly perform procedure 'Message Store Function Delete using HTTP' as described in TS 36.579-1 [2] Table 5.3C.15.2-1 to delete a subscription to notifications about changes in the message store function? | - | - | - | - |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. |  |  |  |  |  |

6.8.4.3.3	Specific message contents

Table 6.8.4.3.3-1: HTTP POST from the UE (Step 2, Table 6.8.4.3.2-1; step 2, TS 36.579-1 [2] Table 5.3C.14.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.4.3-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Status-Line |  |  |  |  |
| Method |  |  |  |  |
| Request-URI |  |  |  |  |
| uri | tsc_MCData_MsgSF_URI | The absolute URI identifying the resource on a media storage function |  |  |
| Authorization |  |  |  |  |
| authentication-scheme | “Bearer” |  |  |  |
| b64token | Access token as assigned to the UE by Token Response |  |  |  |
| Host |  |  |  |  |
| host | tsc_MCData_MsgSF_Hostname | hostname identifying the message store function | TS 24.282 [87], clause 21.2.12A.1 |  |
| port | not present |  |  |  |
| Content-Type |  |  |  |  |
| media-type | "application/xml" |  |  |  |
| Subscription |  |  | RESTful Network API for NMS [40], clauses 6.20 |  |
| callbackReference |  |  |  |  |
| notifyURL | px_IPv4_Address3_UE | The client-side notification URL |  |  |
| callbackData | "abcd" |  |  |  |
| duration | "7200" |  |  |  |
| clientCorrelator | "12345" |  |  |  |

Table 6.8.4.3.3-2: HTTP 201 Created from the SS (step 2, Table 6.8.4.3.2-1; step 3, TS 36.579-1 [2] Table 5.3C.14.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.4.7-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Content-Type |  |  |  |  |
| media-type | "application/xml" |  |  |  |
| Subscription |  |  | RESTful Network API for NMS [40], clauses 6.20 |  |
| callbackReference |  |  |  |  |
| notifyURL | px_IPv4_Address3_UE | The client-side notification URL |  |  |
| callbackData | "abcd" |  |  |  |
| duration | "7200" |  |  |  |
| clientCorrelator | "12345" |  |  |  |
| resourceURL | tsc_MCData_MsgSF_URI & "/subscriptions/sub001" |  |  |  |
| index | "1" |  |  |  |
| restartToken | "abc123" |  |  |  |

Table 6.8.4.3.3-3: HTTP POST from the SS (Step 3, Table 6.8.4.3.2-1; step 2, TS 36.579-1 [2] Table 5.3C.19.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.4.3-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Status-Line |  |  |  |  |
| Method |  |  |  |  |
| Request-URI |  |  |  |  |
| uri | tsc_MCData_MsgSF_URI | The absolute URI identifying the resource on a media storage function |  |  |
| Authorization |  |  |  |  |
| authentication-scheme | “Bearer” |  |  |  |
| b64token | Access token as assigned to the UE by Token Response |  |  |  |
| Host |  |  |  |  |
| host | tsc_MCData_MsgSF_Hostname | hostname identifying the message store function | TS 24.282 [87], clause 21.2.12A.1 |  |
| port | not present |  |  |  |
| Content-Type |  |  |  |  |
| media-type | "application/xml" |  |  |  |
| EventList |  |  | RESTful Network API for NMS [40], clauses 6.22 |  |
| callBackData |  |  |  |  |  |  |  |  |  |
| index |  |  |  |  |  |  |  |  |  |
| restartToken |  |  |  |  |

Table 6.8.4.3.3-4: HTTP POST from the UE (Step 5, Table 6.8.4.3.2-1; step 2, TS 36.579-1 [2] Table 5.3C.17.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.4.3-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Status-Line |  |  |  |  |
| Method |  |  |  |  |
| Request-URI |  |  |  |  |
| uri | tsc_MCData_MsgSF_URI | The absolute URI identifying the resource on a media storage function |  |  |
| Authorization |  |  |  |  |
| authentication-scheme | “Bearer” |  |  |  |
| b64token | Access token as assigned to the UE by Token Response |  |  |  |
| Host |  |  |  |  |
| host | tsc_MCData_MsgSF_Hostname | hostname identifying the message store function | TS 24.282 [87], clause 21.2.14A.1 |  |
| port | not present |  |  |  |
| Content-Type |  |  |  |  |
| media-type | "application/xml" |  |  |  |
| Subscription |  |  | RESTful Network API for NMS [40], clauses 6.21 |  |
| duration | "10800" |  |  |  |

Table 6.8.4.3.3-5: HTTP 200 (OK) Created from the SS (step 5, Table 6.8.4.3.2-1; step 3, TS 36.579-1 [2] Table 5.3C.17.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.4.6-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| ETag |  |  |  |  |
| entity-tag | Any value as selected by the SS |  |  |  |
| Content-Type |  |  |  |  |
| media-type | "application/xml" |  |  |  |
| Subscription |  |  | RESTful Network API for NMS [40], clauses 6.21 |  |
| callbackReference |  |  |  |  |
| notifyURL | px_IPv4_Address3_UE | The client-side notification URL |  |  |
| callbackData | "abcd" |  |  |  |
| duration | "10800" |  |  |  |
| clientCorrelator | "12345" |  |  |  |
| resourceURL | tsc_MCData_MsgSF_URI & "/subscriptions/sub001" |  |  |  |
| index | "2" |  |  |  |
| restartToken | "abc123" |  |  |  |

Table 6.8.4.3.3-5: HTTP DELETE from the UE (Step 7, Table 6.8.4.3.2-1; step 2, TS 36.579-1 [2] Table 5.3C.15.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.4.5-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  |  |  |
| Method | "DELETE" |  |  |  |
| Request-URI |  |  |  |  |
| uri | tsc_MCData_MsgSF_URI & "/subscriptions/sub001" | The subscription to be deleted |  |  |
| Authorization |  |  |  |  |
| authentication-scheme | “Bearer” |  |  |  |
| b64token | Access token as assigned to the UE by Token Response |  |  |  |
| Host |  |  |  |  |
| host | tsc_MCData_MsgSF_Hostname | hostname identifying the message store function | TS 24.282 [87], clause 21.2.13A.1 |  |
| port | not present |  |  |  |

<END OF CHANGES>