3GPP TSG-RAN5 Meeting #104	R5-245014

# Maastricht, Netherlands, 19 Aug – 23 Aug, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2966 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of 6.4E.2.3 NR V2X Carrier leakage |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The NR V2X Carrier leakage test cases are still missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Completing the test cases. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test cases remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4E.2.3.1(new), 6.4E.2.3.1D(new), 6.4E.2.3.2(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TR 38.905 CR 0941 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

6.4E.2.2.1D.4.2	Test procedure

1.	The UE starts to perform the V2X sidelink communication according to [SL-V2X-Preconfiguration]. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the V2X RMC.

2.	Configure the UE to transmit at PUMAX level.

3.	Measure the EVM and  using Global In-Channel Tx-Test (Annex E) for each of transmit antenna. The measurement period is [15] subframes. When V2X transmissions are shortened due to transmission gap of 1 symbol at the end of the subframe, the EVM measurement interval is reduced by one symbol, accordingly.

4.	Modify SL-V2X-Preconfiguration to ensure the UE to transmit at a relative low power, according to Table [TBD].

5.	Ensure the UE is in State [TBD] in Transmit Mode according to TS 38.508-1 [5] clause [TBD] using the new UL power control setting.

6.	The UE starts to perform the V2X sidelink communication according to [SL-V2X-Preconfiguration]. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the V2X RMC.

7.	Measure EVM and  using Global In-Channel Tx-Test (Annex E) for each of transmit antenna. The measurement period is [15] subframes. When V2X transmissions are shortened due to transmission gap of 1 symbol at the end of the subframe, the EVM measurement interval is reduced by one symbol, accordingly.

6.4E.2.2.1D.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 and 5.4 with the following exceptions:

FFS

6.4E.2.2.1D.5	Test requirement

FFS

6.4E.2.3	Carrier leakage for V2X

6.4E.2.3.1	Carrier leakage for V2X / non-concurrent operation

6.4E.2.3.1.1	Test purpose

Carrier leakage expresses itself as unmodulated sine wave with the carrier frequency or centre frequency of aggregated transmission bandwidth configuration. It is an interference of approximately constant amplitude and independent of the amplitude of the wanted signal.

The purpose of this test is to exercise the UE transmitter to verify its modulation quality in terms of carrier leakage.

6.4E.2.3.1.2	Test applicability

This test case applies to all types of NR UE release 16 and forward that support NR V2X sidelink communication.

6.4E.2.3.1.3	Minimum conformance requirements

Carrier leakage of NR V2X sidelink transmission, the requirements for NR PUSCH in Table 6.4.2.2.3-1 shall be applied.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.4E.2.3.

6.4E.2.3.1.4	Test description

6.4E.2.3.1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.2E.1-1 and Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.4E.2.3.1.4.1-1. The details of the V2X reference measurement channels (RMCs) are specified in Annex A.7.5 and the GNSS configuration in TS 38.508-1 [5] subclause 4.11.

Table 6.4E.2.3.1.4.1-1: Test Configuration Table for PSSCH and PSCCH

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1.8 |  |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified TS 38.508-1 [5] subclause 4.3.1 |  |  | Mid |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | V2X Configuration to Transmit |  |  |
|  |  | Modulation |  | PSCCH and PSSCH RB allocation (Note 1) |
| 1 | Default | CP-OFDM QPSK |  | Outer_10RB_Left |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1E-1. NOTE 2: When the reported DC carrier position is within Outer_10RB_Left, use Outer_10RB_Right for UL RB allocation. |  |  |  |  |

1.	Connect the SS and GNSS simulator to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.9.1 for TE diagram and section A.3.2.7 for UE diagram.

2.	The parameter settings for the V2X sidelink transmission over PC5 are pre-configured according to TS 38.508-1 [5] subclause 4.10. Message content exceptions are defined in clause 6.4E.2.3.1.4.3.

3.	The V2X Reference Measurement Channel is set according to Table 6.4E.2.3.1.4.1-1.

4.	The GNSS simulator is configured for Scenario #1: static in Geographical area #1, as defined in TS38.508-1 [5] Table 4.11.2-2. Geographical area #1 is also pre-configured in the UE.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state Out_of_Coverage with generic procedure parameters Sidelink On, Test Loop Function On with UE test loop mode E closed for Transmit Mode according to TS 38.508-1 [5] clause 4.5.

6.4E.2.3.1.4.2	Test procedure

1.	The UE starts to perform the V2X sidelink communication according to SL-PreconfigurationNR with condition of test point 1 in Table 6.4E.2.3.1.4.3-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the NR Sidelink RMC.

2.	Configure the UE to transmit at PUMAX level, according to test point 1 in Table 6.4E.2.3.1.4.3-1.

3.	Measure the carrier leakage using Global In-Channel Tx-Test (Annex E). When sidelink transmissions are shortened due to transmission gap of 1 symbol at the end of the subframe, the measurement interval is reduced by one symbol, accordingly.

4.	Modify SL-V2X-Preconfiguration to ensure the UE to transmit at a relative low power, according to test point 2 in Table 6.4E.2.3.1.4.3-1.

5.	Ensure the UE is in state Out_of_Coverage with generic procedure parameters Sidelink On, Test Loop Function On with UE test loop mode E closed for Transmit Mode according to TS 38.508-1 [5] clause 4.5.

6.	The UE starts to perform the V2X sidelink communication according to SL-PreconfigurationNR. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the NR Sidelink RMC.

7.	Measure the carrier leakage using Global In-Channel Tx-Test (Annex E). When sidelink transmissions are shortened due to transmission gap of 1 symbol at the end of the subframe, the measurement interval is reduced by one symbol, accordingly.

8.	Modify SL-V2X-Preconfiguration to ensure the UE to transmit at a relative low power, according to test point 3 in Table 6.4E.2.3.1.4.3-1.

9.	Ensure the UE is in state Out_of_Coverage with generic procedure parameters Sidelink On, Test Loop Function On with UE test loop mode E closed for Transmit Mode according to TS 38.508-1 [5] clause 4.5.

10.	The UE starts to perform the V2X sidelink communication according to SL-PreconfigurationNR. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the NR Sidelink RMC.

11.	Measure the carrier leakage using Global In-Channel Tx-Test (Annex E). When sidelink transmissions are shortened due to transmission gap of 1 symbol at the end of the subframe, the measurement interval is reduced by one symbol, accordingly.

6.4E.2.3.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.10 with the following exceptions:

Table 6.4E.2.3.1.4.3-1: SL-ResourcePool for PSCCH/PSSCH Testing

| Derivation Path: TS 38.508-1 [5], Table 4.6.6-25 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-ResourcePool-r16 ::= SEQUENCE { |  |  |  |
| sl-PSCCH-Config-r16 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| sl-TimeResourcePSCCH-r16 | As defined in Table 6.1E-2 |  |  |
| sl-FreqResourcePSCCH-r16 | As defined in Table 6.1E-2 |  |  |
| } |  |  |  |
| } |  |  |  |
| sl-SubchannelSize-r16 | As defined in Table 6.1E-2 |  |  |
| sl-StartRB-Subchannel-r16 | As defined in Table 6.1E-2 |  |  |
| sl-NumSubchannel-r16 | As defined in Table 6.1E-2 |  |  |
| sl-PowerControl-r16 SEQUENCE { |  |  |  |
| sl-MaxTransPower-r16 | 10 |  | Test point 1 |
|  | 5 |  | Test point 2 |
|  | -15 |  | Test point 3 |
| sl-Alpha-PSSCH-PSCCH-r16 | Not present |  |  |
| dl-Alpha-PSSCH-PSCCH-r16 | Not present |  |  |
| sl-P0-PSSCH-PSCCH-r16 | Not present |  |  |
| dl-P0-PSSCH-PSCCH-r16 | Not present |  |  |
| dl-Alpha-PSFCH-r16 | Not present |  |  |
| dl-P0-PSFCH-r16 | Not present |  |  |
| dl-P0-PSSCH-PSCCH-r17 | Not present |  |  |
| sl-P0-PSSCH-PSCCH-r17 | Not present |  |  |
| dl-P0-PSFCH-r17 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

6.4E.2.3.1.5	Test requirement

Each of the n carrier leakage results, derived in Annex E.3.1, shall not exceed the values in table 6.4E.2.3.1.5-1. Allocated RBs are not under test.

Table 6.4E.2.3.1.5-1: Test requirements for Relative Carrier Leakage Power

| Parameters UE output power | Relative limit (dBc) |
| --- | --- |
| Output power > 10 dBm | -28 + TT |
| 0 dBm ≤ Output power ≤ 10 dBm | -25 + TT |
| -30 dBm ≤ Output power < 0 dBm | -20 + TT |
| NOTE 1: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. NOTE 2: Test tolerance TT = 0.8 dB. |  |

6.4E.2.3.1D	Carrier leakage for V2X / non-concurrent operation / SL-MIMO

6.4E.2.3.1D.1	Test purpose

Same test purpose as 6.4E.2.3.1.1.

6.4E.2.3.1D.2	Test applicability

This test case applies to all types of NR UE release 16 and forward that support NR V2X sidelink communication and SL-MIMO.

6.4E.2.3.1D.3	Minimum conformance requirements

For NR V2X UE supporting SL MIMO, the transmit modulation quality requirements for single carrier shall apply to  each transmit antenna connector.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.4E.2.1.

6.4E.2.3.1D.4	Test description

Same test description as in 6.4E.2.3.1.4 for PSSCH/PSCCH subtest with following exceptions:

For initial conditions:

1.	Connect the SS and GNSS simulator to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.9.2 for TE diagram and section A.3.2.7 for UE diagram.

6.	Ensure the UE is in state 4-A as defined in TS 38.508-1 [4], subclause 4.4A using generic procedure parameter Sidelink (On), Cast Type (Unicast), GNSS Sync (On) and Transmit Mode with SL-MIMO. The UE is synchronized to GNSS,

Steps 3, 7 and 11 in test procedures are replaced by:

3.	Measure the carrier leakage using Global In-Channel Tx-Test (Annex E) on each transmit antenna connector. When sidelink transmissions are shortened due to transmission gap of 1 symbol at the end of the subframe, the measurement interval is reduced by one symbol, accordingly.

7.	Measure the carrier leakage using Global In-Channel Tx-Test (Annex E) on each transmit antenna connector. When sidelink transmissions are shortened due to transmission gap of 1 symbol at the end of the subframe, the measurement interval is reduced by one symbol, accordingly.

11.	Measure the carrier leakage using Global In-Channel Tx-Test (Annex E) on each transmit antenna connector. When sidelink transmissions are shortened due to transmission gap of 1 symbol at the end of the subframe, the measurement interval is reduced by one symbol, accordingly.

6.4E.2.3.1D.5	Test requirement

The n carrier leakage results on each transmit antenna connector, derived in Annex E.3.1, shall not exceed the values in table 6.4E.2.3.1D.5-1. Allocated RBs are not under test.

Table 6.4E.2.3.1D.5-1: Test requirements for Relative Carrier Leakage Power

| Parameters UE output power | Relative limit (dBc) |
| --- | --- |
| Output power > 10 dBm | -28 + TT |
| 0 dBm ≤ Output power ≤ 10 dBm | -25 + TT |
| -30 dBm ≤ Output power < 0 dBm | -20 + TT |
| NOTE 1: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. NOTE 2: Test tolerance TT = 0.8 dB. |  |

6.4E.2.3.2	Carrier leakage for V2X / con-current operation

6.4E.2.3.2.1	Test purpose

Same test purpose as in 6.4E.2.3.1 for concurrent operation.

6.4E.2.3.2.2	Test applicability

No test case details are specified. The carrier leakage requirements for concurrent operation apply and are tested in clauses 6.4.2.2 for NR Uu operation and 6.4E.2.3.1 for NR sidelink option.

6.4E.2.4	In-band emissions for V2X

6.4E.2.4.1	In-band emissions for V2X / non-concurrent operation

Editor’s Note:

- The test case is not completed due to the following aspects are not yet determined:

- Test configuration is TBD

- Target power level is TBD

6.4E.2.4.1.1	Test purpose

< End of Changes >