3GPP TSG-RAN5 Meeting #104	R5-245048

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4615 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New testcase 7.1.1.3.20 Correct handling of MAC control information / Delay Status / Delay status reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_XR_enh_plus_CT1-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Delay status reporting functionality has been added to core specification and needs to be tested. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding new testcase including test purpose, conformance requirements and test description including Pre-test condition and Test procedure sequence. Specific message contents left to TBD state. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Delay status reporting functionality will not be verified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR 38.523-2 CR 0505 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------------------------------------------- Skipped chapters ---------------------------------------------------------------

------------------------------------------------------- Start of first change -----------------------------------------------------------

##### 7.1.1.3.20	Correct handling of MAC control information / Delay Status / Delay status reporting

7.1.1.3.20.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state }

ensure that {

when { smallest remaining value of the PDCP discardTimers among all the data buffered for the LCG becomes below remainingTimeThreshold of the LCG }

then { UE reports a DSR including LCGi=1 and Remaining Timer and Buffer Size for the LCGi  }

}

7.1.1.3.20.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.321, clauses 5.4.9, 6.1.3.72 and 6.1.3.1. Unless otherwise stated these are Rel-18 requirements.

[TS 38.321, clause 5.4.9]

The Delay Status Reporting (DSR) procedure is used to provide the serving gNB with delay status of LCGs. This delay status for an LCG includes remaining time, which is the smallest remaining value of the running PDCP discardTimers among SDUs that are buffered for the LCG but have not been transmitted in any MAC PDU as specified in clause 7.3 in TS 38.323 [4], and the total amount of delay-critical UL data for the LCG according to the data volume calculation procedure specified in clause 5.5 in TS 38.322 [3] and clause 5.6 in TS 38.323 [4] for the associated RLC and PDCP entities, respectively.

RRC controls the DSR procedure by configuring the following parameter:

-	remainingTimeThreshold: the threshold on remaining time for triggering a DSR for an LCG.

If an LCG is configured for delay status reporting, the MAC entity shall:

1>	if the smallest remaining value of the running PDCP discardTimers among all the SDUs buffered for the LCG that has not been transmitted in any MAC PDU and has not been reported as data volume in a DSR MAC CE becomes below remainingTimeThreshold of the LCG; and

1>	if there is no DSR pending for the LCG:

2>	trigger a DSR for the LCG.

If there is at least one DSR pending, the MAC entity shall:

1>	if UL-SCH resources are available for a new transmission and the UL-SCH resources can accommodate the DSR MAC CE plus its subheader as a result of logical channel prioritization:

2>	instruct the Multiplexing and Assembly procedure to generate the DSR MAC CE as specified in clause 6.1.3.72.

…

NOTE:	The availability of UL-SCH resources for the transmission of the DSR MAC CE follows the same critieria specified in clause 5.4.5.

An SDU is considered to be associated with a DSR if it has not been transmitted in any MAC PDU and it is associated with the LCG which triggered the DSR and the remaining value of its PDCP discardTimer is below remainingTimeThreshold.

A MAC PDU shall contain at most one DSR MAC CE. The MAC entity shall not include a DSR MAC CE in a MAC PDU if the MAC PDU can accommodate the SDUs associated with all the pending DSRs.

After a DSR is triggered, it is considered as pending until it is cancelled. The MAC entity shall cancel a pending DSR, either when all the SDUs associated with the DSR have been discarded, or when a MAC PDU is transmitted and this MAC PDU includes a DSR MAC CE that contains the delay information of all the SDUs associated with the DSR (as described in the clause 6.1.3.72). The MAC entity may cancel a pending DSR when a MAC PDU is transmitted and this MAC PDU includes all the SDUs associated with the DSR but is not sufficient to include the DSR MAC CE and its subheader.

[TS 38.321, clause 6.1.3.72]

The Delay Status Report (DSR) MAC CE is identified by MAC subheader with an eLCID as specified in Table 6.2.1-2b.

The fields in the DSR MAC CE are defined as follows:

-	LCGi: This field indicates the presence of delay information (i.e. the Remaining Time and Buffer Size fields) for the LCG i. The LCGi field set to 1 indicates that the delay information for the LCG i is reported. The LCGi field set to 0 indicates that the delay information for the LCG i is not reported;

-	Remaining Time: This field indicates the shortest remaining value of running PDCP discardTimer (described in clause 7.3 in TS 38.323 [4]) among all PDCP SDUs that are buffered for an LCG but have not been transmitted in any MAC PDU, at the time of the first symbol of the first PUSCH transmission that includes this DSR MAC CE. The length of this field is 6 bits. This field is present only if the buffer size indicated by the corresponding Buffer Size field is not zero; otherwise, this field is reserved and set to 0. If present, the value r in this field indicates a remaining time within the range of (r, r + 1] msec;

-	BT: This field is present only if the corresponding LCG is configured with additionalBS-TableAllowed and the buffer size indicated by the corresponding Buffer Size field is not zero; otherwise, this field is reserved and set to 0. If present, the BT field set to 1 indicates that the buffer sizes specified in Table 6.1.3.1-3 are used to set the value of the Buffer Size field, while the BT field set to 0 indicates that the buffer sizes specified in Table 6.1.3.1-2 are used instead;

-	Buffer Size: The Buffer Size field indicates the total amount of delay-critical UL data for an LCG according to the data volume calculation procedure specified in clause 5.5 in TS 38.322 [3] and clause 5.6 in TS 38.323 [4] for the associated RLC and PDCP entities, respectively, after the MAC PDU has been built. If the corresponding LCG is configured with additionalBS-TableAllowed and the amount of delay-critical UL data for an LCG is within the buffer sizes specified in Table 6.1.3.1-3, the MAC entity shall use the buffer sizes specified in Table 6.1.3.1-3 to set the value of this field; otherwise, the MAC entity shall use Table 6.1.3.1-2 instead. This field is indicated in number of bytes. The length of this field is 8 bits.

The DSR MAC CE shall include delay information of all LCGs which have pending DSRs when the MAC PDU containing this DSR MAC CE is to be built. The Remaining Time, the BT, and the Buffer Size fields for an LCG shall be reported in two consecutive octets. These three fields for different LCGs shall be included in a DSR MAC CE in ascending order based on the LCGi.

Figure 6.1.3.72-1: DSR MAC CE

[TS 38.321, clause 6.1.3.1]

Table 6.1.3.1-2: Buffer size levels (in bytes) for 8-bit Buffer Size field

| Index | BS value | Index | BS value | Index | BS value | Index | BS value |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 64 | ≤ 560 | 128 | ≤ 31342 | 192 | ≤ 1754595 |
| 1 | ≤ 10 | 65 | ≤ 597 | 129 | ≤ 33376 | 193 | ≤ 1868488 |
| 2 | ≤ 11 | 66 | ≤ 635 | 130 | ≤ 35543 | 194 | ≤ 1989774 |
| 3 | ≤ 12 | 67 | ≤ 677 | 131 | ≤ 37850 | 195 | ≤ 2118933 |
| 4 | ≤ 13 | 68 | ≤ 720 | 132 | ≤ 40307 | 196 | ≤ 2256475 |
| 5 | ≤ 14 | 69 | ≤ 767 | 133 | ≤ 42923 | 197 | ≤ 2402946 |
| 6 | ≤ 15 | 70 | ≤ 817 | 134 | ≤ 45709 | 198 | ≤ 2558924 |
| 7 | ≤ 16 | 71 | ≤ 870 | 135 | ≤ 48676 | 199 | ≤ 2725027 |
| 8 | ≤ 17 | 72 | ≤ 926 | 136 | ≤ 51836 | 200 | ≤ 2901912 |
| 9 | ≤ 18 | 73 | ≤ 987 | 137 | ≤ 55200 | 201 | ≤ 3090279 |
| 10 | ≤ 19 | 74 | ≤ 1051 | 138 | ≤ 58784 | 202 | ≤ 3290873 |
| 11 | ≤ 20 | 75 | ≤ 1119 | 139 | ≤ 62599 | 203 | ≤ 3504487 |
| 12 | ≤ 22 | 76 | ≤ 1191 | 140 | ≤ 66663 | 204 | ≤ 3731968 |
| 13 | ≤ 23 | 77 | ≤ 1269 | 141 | ≤ 70990 | 205 | ≤ 3974215 |
| 14 | ≤ 25 | 78 | ≤ 1351 | 142 | ≤ 75598 | 206 | ≤ 4232186 |
| 15 | ≤ 26 | 79 | ≤ 1439 | 143 | ≤ 80505 | 207 | ≤ 4506902 |
| 16 | ≤ 28 | 80 | ≤ 1532 | 144 | ≤ 85730 | 208 | ≤ 4799451 |
| 17 | ≤ 30 | 81 | ≤ 1631 | 145 | ≤ 91295 | 209 | ≤ 5110989 |
| 18 | ≤ 32 | 82 | ≤ 1737 | 146 | ≤ 97221 | 210 | ≤ 5442750 |
| 19 | ≤ 34 | 83 | ≤ 1850 | 147 | ≤ 103532 | 211 | ≤ 5796046 |
| 20 | ≤ 36 | 84 | ≤ 1970 | 148 | ≤ 110252 | 212 | ≤ 6172275 |
| 21 | ≤ 38 | 85 | ≤ 2098 | 149 | ≤ 117409 | 213 | ≤ 6572925 |
| 22 | ≤ 40 | 86 | ≤ 2234 | 150 | ≤ 125030 | 214 | ≤ 6999582 |
| 23 | ≤ 43 | 87 | ≤ 2379 | 151 | ≤ 133146 | 215 | ≤ 7453933 |
| 24 | ≤ 46 | 88 | ≤ 2533 | 152 | ≤ 141789 | 216 | ≤ 7937777 |
| 25 | ≤ 49 | 89 | ≤ 2698 | 153 | ≤ 150992 | 217 | ≤ 8453028 |
| 26 | ≤ 52 | 90 | ≤ 2873 | 154 | ≤ 160793 | 218 | ≤ 9001725 |
| 27 | ≤ 55 | 91 | ≤ 3059 | 155 | ≤ 171231 | 219 | ≤ 9586039 |
| 28 | ≤ 59 | 92 | ≤ 3258 | 156 | ≤ 182345 | 220 | ≤ 10208280 |
| 29 | ≤ 62 | 93 | ≤ 3469 | 157 | ≤ 194182 | 221 | ≤ 10870913 |
| 30 | ≤ 66 | 94 | ≤ 3694 | 158 | ≤ 206786 | 222 | ≤ 11576557 |
| 31 | ≤ 71 | 95 | ≤ 3934 | 159 | ≤ 220209 | 223 | ≤ 12328006 |
| 32 | ≤ 75 | 96 | ≤ 4189 | 160 | ≤ 234503 | 224 | ≤ 13128233 |
| 33 | ≤ 80 | 97 | ≤ 4461 | 161 | ≤ 249725 | 225 | ≤ 13980403 |
| 34 | ≤ 85 | 98 | ≤ 4751 | 162 | ≤ 265935 | 226 | ≤ 14887889 |
| 35 | ≤ 91 | 99 | ≤ 5059 | 163 | ≤ 283197 | 227 | ≤ 15854280 |
| 36 | ≤ 97 | 100 | ≤ 5387 | 164 | ≤ 301579 | 228 | ≤ 16883401 |
| 37 | ≤ 103 | 101 | ≤ 5737 | 165 | ≤ 321155 | 229 | ≤ 17979324 |
| 38 | ≤ 110 | 102 | ≤ 6109 | 166 | ≤ 342002 | 230 | ≤ 19146385 |
| 39 | ≤ 117 | 103 | ≤ 6506 | 167 | ≤ 364202 | 231 | ≤ 20389201 |
| 40 | ≤ 124 | 104 | ≤ 6928 | 168 | ≤ 387842 | 232 | ≤ 21712690 |
| 41 | ≤ 132 | 105 | ≤ 7378 | 169 | ≤ 413018 | 233 | ≤ 23122088 |
| 42 | ≤ 141 | 106 | ≤ 7857 | 170 | ≤ 439827 | 234 | ≤ 24622972 |
| 43 | ≤ 150 | 107 | ≤ 8367 | 171 | ≤ 468377 | 235 | ≤ 26221280 |
| 44 | ≤ 160 | 108 | ≤ 8910 | 172 | ≤ 498780 | 236 | ≤ 27923336 |
| 45 | ≤ 170 | 109 | ≤ 9488 | 173 | ≤ 531156 | 237 | ≤ 29735875 |
| 46 | ≤ 181 | 110 | ≤ 10104 | 174 | ≤ 565634 | 238 | ≤ 31666069 |
| 47 | ≤ 193 | 111 | ≤ 10760 | 175 | ≤ 602350 | 239 | ≤ 33721553 |
| 48 | ≤ 205 | 112 | ≤ 11458 | 176 | ≤ 641449 | 240 | ≤ 35910462 |
| 49 | ≤ 218 | 113 | ≤ 12202 | 177 | ≤ 683087 | 241 | ≤ 38241455 |
| 50 | ≤ 233 | 114 | ≤ 12994 | 178 | ≤ 727427 | 242 | ≤ 40723756 |
| 51 | ≤ 248 | 115 | ≤ 13838 | 179 | ≤ 774645 | 243 | ≤ 43367187 |
| 52 | ≤ 264 | 116 | ≤ 14736 | 180 | ≤ 824928 | 244 | ≤ 46182206 |
| 53 | ≤ 281 | 117 | ≤ 15692 | 181 | ≤ 878475 | 245 | ≤ 49179951 |
| 54 | ≤ 299 | 118 | ≤ 16711 | 182 | ≤ 935498 | 246 | ≤ 52372284 |
| 55 | ≤ 318 | 119 | ≤ 17795 | 183 | ≤ 996222 | 247 | ≤ 55771835 |
| 56 | ≤ 339 | 120 | ≤ 18951 | 184 | ≤ 1060888 | 248 | ≤ 59392055 |
| 57 | ≤ 361 | 121 | ≤ 20181 | 185 | ≤ 1129752 | 249 | ≤ 63247269 |
| 58 | ≤ 384 | 122 | ≤ 21491 | 186 | ≤ 1203085 | 250 | ≤ 67352729 |
| 59 | ≤ 409 | 123 | ≤ 22885 | 187 | ≤ 1281179 | 251 | ≤ 71724679 |
| 60 | ≤ 436 | 124 | ≤ 24371 | 188 | ≤ 1364342 | 252 | ≤ 76380419 |
| 61 | ≤ 464 | 125 | ≤ 25953 | 189 | ≤ 1452903 | 253 | ≤ 81338368 |
| 62 | ≤ 494 | 126 | ≤ 27638 | 190 | ≤ 1547213 | 254 | > 81338368 |
| 63 | ≤ 526 | 127 | ≤ 29431 | 191 | ≤ 1647644 | 255 | Reserved |

Table 6.1.3.1-3. Refined buffer size levels (in bytes) for 8-bit Buffer Size field

| Index | BS value | Index | BS value | Index | BS Value | Index | BS Value |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | > 4751 and ≤ 5000 | 64 | ≤ 17584 | 128 | ≤ 61841 | 192 | ≤ 217489 |
| 1 | ≤ 5099 | 65 | ≤ 17933 | 129 | ≤ 63069 | 193 | ≤ 221805 |
| 2 | ≤ 5200 | 66 | ≤ 18289 | 130 | ≤ 64320 | 194 | ≤ 226207 |
| 3 | ≤ 5303 | 67 | ≤ 18652 | 131 | ≤ 65596 | 195 | ≤ 230695 |
| 4 | ≤ 5408 | 68 | ≤ 19022 | 132 | ≤ 66898 | 196 | ≤ 235273 |
| 5 | ≤ 5516 | 69 | ≤ 19399 | 133 | ≤ 68226 | 197 | ≤ 239942 |
| 6 | ≤ 5625 | 70 | ≤ 19784 | 134 | ≤ 69580 | 198 | ≤ 244703 |
| 7 | ≤ 5737 | 71 | ≤ 20177 | 135 | ≤ 70960 | 199 | ≤ 249559 |
| 8 | ≤ 5851 | 72 | ≤ 20577 | 136 | ≤ 72368 | 200 | ≤ 254511 |
| 9 | ≤ 5967 | 73 | ≤ 20985 | 137 | ≤ 73804 | 201 | ≤ 259562 |
| 10 | ≤ 6085 | 74 | ≤ 21402 | 138 | ≤ 75269 | 202 | ≤ 264713 |
| 11 | ≤ 6206 | 75 | ≤ 21827 | 139 | ≤ 76763 | 203 | ≤ 269966 |
| 12 | ≤ 6329 | 76 | ≤ 22260 | 140 | ≤ 78286 | 204 | ≤ 275323 |
| 13 | ≤ 6455 | 77 | ≤ 22702 | 141 | ≤ 79839 | 205 | ≤ 280786 |
| 14 | ≤ 6583 | 78 | ≤ 23152 | 142 | ≤ 81424 | 206 | ≤ 286358 |
| 15 | ≤ 6713 | 79 | ≤ 23611 | 143 | ≤ 83040 | 207 | ≤ 292041 |
| 16 | ≤ 6847 | 80 | ≤ 24080 | 144 | ≤ 84687 | 208 | ≤ 297836 |
| 17 | ≤ 6983 | 81 | ≤ 24558 | 145 | ≤ 86368 | 209 | ≤ 303746 |
| 18 | ≤ 7121 | 82 | ≤ 25045 | 146 | ≤ 88082 | 210 | ≤ 309774 |
| 19 | ≤ 7262 | 83 | ≤ 25542 | 147 | ≤ 89830 | 211 | ≤ 315921 |
| 20 | ≤ 7407 | 84 | ≤ 26049 | 148 | ≤ 91612 | 212 | ≤ 322190 |
| 21 | ≤ 7554 | 85 | ≤ 26566 | 149 | ≤ 93430 | 213 | ≤ 328583 |
| 22 | ≤ 7703 | 86 | ≤ 27093 | 150 | ≤ 95284 | 214 | ≤ 335104 |
| 23 | ≤ 7856 | 87 | ≤ 27631 | 151 | ≤ 97175 | 215 | ≤ 341754 |
| 24 | ≤ 8012 | 88 | ≤ 28179 | 152 | ≤ 99103 | 216 | ≤ 348535 |
| 25 | ≤ 8171 | 89 | ≤ 28738 | 153 | ≤ 101070 | 217 | ≤ 355452 |
| 26 | ≤ 8333 | 90 | ≤ 29309 | 154 | ≤ 103076 | 218 | ≤ 362505 |
| 27 | ≤ 8499 | 91 | ≤ 29890 | 155 | ≤ 105121 | 219 | ≤ 369699 |
| 28 | ≤ 8667 | 92 | ≤ 30483 | 156 | ≤ 107207 | 220 | ≤ 377035 |
| 29 | ≤ 8839 | 93 | ≤ 31088 | 157 | ≤ 109335 | 221 | ≤ 384517 |
| 30 | ≤ 9015 | 94 | ≤ 31705 | 158 | ≤ 111504 | 222 | ≤ 392147 |
| 31 | ≤ 9194 | 95 | ≤ 32334 | 159 | ≤ 113717 | 223 | ≤ 399929 |
| 32 | ≤ 9376 | 96 | ≤ 32976 | 160 | ≤ 115973 | 224 | ≤ 407865 |
| 33 | ≤ 9562 | 97 | ≤ 33630 | 161 | ≤ 118275 | 225 | ≤ 415959 |
| 34 | ≤ 9752 | 98 | ≤ 34298 | 162 | ≤ 120622 | 226 | ≤ 424213 |
| 35 | ≤ 9946 | 99 | ≤ 34978 | 163 | ≤ 123016 | 227 | ≤ 432631 |
| 36 | ≤ 10143 | 100 | ≤ 35672 | 164 | ≤ 125457 | 228 | ≤ 441216 |
| 37 | ≤ 10344 | 101 | ≤ 36380 | 165 | ≤ 127946 | 229 | ≤ 449971 |
| 38 | ≤ 10549 | 102 | ≤ 37102 | 166 | ≤ 130485 | 230 | ≤ 458900 |
| 39 | ≤ 10759 | 103 | ≤ 37839 | 167 | ≤ 133074 | 231 | ≤ 468007 |
| 40 | ≤ 10972 | 104 | ≤ 38589 | 168 | ≤ 135715 | 232 | ≤ 477294 |
| 41 | ≤ 11190 | 105 | ≤ 39355 | 169 | ≤ 138408 | 233 | ≤ 486765 |
| 42 | ≤ 11412 | 106 | ≤ 40136 | 170 | ≤ 141155 | 234 | ≤ 496425 |
| 43 | ≤ 11639 | 107 | ≤ 40933 | 171 | ≤ 143956 | 235 | ≤ 506276 |
| 44 | ≤ 11870 | 108 | ≤ 41745 | 172 | ≤ 146813 | 236 | ≤ 516322 |
| 45 | ≤ 12105 | 109 | ≤ 42573 | 173 | ≤ 149726 | 237 | ≤ 526568 |
| 46 | ≤ 12345 | 110 | ≤ 43418 | 174 | ≤ 152697 | 238 | ≤ 537017 |
| 47 | ≤ 12590 | 111 | ≤ 44280 | 175 | ≤ 155727 | 239 | ≤ 547674 |
| 48 | ≤ 12840 | 112 | ≤ 45158 | 176 | ≤ 158817 | 240 | ≤ 558542 |
| 49 | ≤ 13095 | 113 | ≤ 46055 | 177 | ≤ 161969 | 241 | ≤ 569625 |
| 50 | ≤ 13355 | 114 | ≤ 46968 | 178 | ≤ 165183 | 242 | ≤ 580929 |
| 51 | ≤ 13620 | 115 | ≤ 47900 | 179 | ≤ 168461 | 243 | ≤ 592457 |
| 52 | ≤ 13890 | 116 | ≤ 48851 | 180 | ≤ 171804 | 244 | ≤ 604213 |
| 53 | ≤ 14166 | 117 | ≤ 49820 | 181 | ≤ 175213 | 245 | ≤ 616203 |
| 54 | ≤ 14447 | 118 | ≤ 50809 | 182 | ≤ 178690 | 246 | ≤ 628431 |
| 55 | ≤ 14734 | 119 | ≤ 51817 | 183 | ≤ 182236 | 247 | ≤ 640902 |
| 56 | ≤ 15026 | 120 | ≤ 52846 | 184 | ≤ 185852 | 248 | ≤ 653620 |
| 57 | ≤ 15324 | 121 | ≤ 53894 | 185 | ≤ 189540 | 249 | ≤ 666590 |
| 58 | ≤ 15628 | 122 | ≤ 54964 | 186 | ≤ 193302 | 250 | ≤ 679818 |
| 59 | ≤ 15938 | 123 | ≤ 56054 | 187 | ≤ 197137 | 251 | ≤ 693308 |
| 60 | ≤ 16255 | 124 | ≤ 57167 | 188 | ≤ 201049 | 252 | ≤ 707066 |
| 61 | ≤ 16577 | 125 | ≤ 58301 | 189 | ≤ 205039 | 253 | ≤ 721097 |
| 62 | ≤ 16906 | 126 | ≤ 59458 | 190 | ≤ 209108 | 254 | ≤ 735406 |
| 63 | ≤ 17242 | 127 | ≤ 60638 | 191 | ≤ 213257 | 255 | ≤ 750000 |

7.1.1.3.20.3	Test description

7.1.1.3.20.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0 with the exception of 2 AM DRBs on NR cell configured according to Table 7.1.1.3.20.3.1-1.

Table 7.1.1.3.20.3.1-1: Logical Channel Configuration Settings

| Parameter | DRB1 | DRB2 |
| --- | --- | --- |
| LogicalChannelIdentity | LCH4(DRB-Identity +3) | LCH5(DRB-Identity +3) |
| Priority | 7 | 6 |
| prioritizedBitRate | 0kbs | 0kbs |
| logicalChannelGroup | 2 (LCG ID#2) | 1 (LCG ID#1) |
| discardTimer | 500ms | 500ms |
| remainingTimeThreshold | 400ms | 300ms |

7.1.1.3.20.3.2	Test procedure sequence

Table 7.1.1.3.20.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS ignores scheduling requests and does not allocate any uplink grant. |  |  |  |  |
| 2 | The SS sends a MAC PDU including an RLC PDU of size 12 bytes on LCH5. | <-- | MAC PDU (RLC SDU on LCH5) | - | - |
| 3 | The SS sends a MAC PDU including an RLC PDU of size 20 bytes on LCH4. | <-- | MAC PDU (RLC SDU on LCH4) | - | - |
| 4 | Wait for discardTimer to be less than remainingTimeThreshold value for LCH4. | - | - | - | - |
| 5 | The SS allocates an UL Grant of TBD bits. | <-- | (UL grant) |  |  |
| 6 | Check: Does the UE transmit MAC PDU containing an DSR for indicating remaining time and pending data (‘Buffer size’ field > ‘0’) for the LCGi = [1,2]? | --> | MAC PDU (DSR) | 1 | P |
| 7 | The SS allocates an UL Grant of TBD bits. | <-- | (UL grant) | - | - |

7.1.1.3.20.3.3	Specific message contents

TBD

-------------------------------------------------------- End of first change -----------------------------------------------------------