3GPP TSG RAN WG5#104	R5-245107

# Maastricht, Netherlands, August 19 – August 23 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-4 | CR | 0870 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Core spec alignment, applicability of requirements for eRedCap |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_enh_plus_CT1-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Applicablity of requirements for eRedCap is missing in 38.521-4. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Applicablity of requirements for eRedCap, as per 38.101-4 v.18.4.0 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Applicablity of requirements for eRedCap will remain missing in 38.521-4. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### << Start of changes >>

Table 5.1.1.11-1: Requirements applicability for RedCap

| UE capability | Test type |  | Test list | Applicability notes |
| --- | --- | --- | --- | --- |
| RedCap with 1RX | FR1 FDD and HD-FDD (Note 1) | PDSCH | All tests in Clause 5.2.1.1.1 |  |
|  |  | PDCCH | All tests in Clause 5.3.1.1.1 |  |
|  |  | PBCH | All tests in Clause 5.4.1.1 |  |
|  |  | SDR | Clause 5.5. 3 |  |
|  | FR1 TDD | PDSCH | All tests in Clause 5.2.1.2.1 |  |
|  |  | PDCCH | All tests in Clause 5.3.1.2.1 |  |
|  |  | PBCH | All tests in Clause 5.4.1.2 |  |
|  |  | SDR | Clause 5.5.3 |  |
| RedCap with 2RX | FR1 FDD and HD-FDD (Note 1) | PDSCH | All tests in Clause 5.2.2.1.17 |  |
|  |  | PDCCH | All tests in Clause 5.3.2.1.4 |  |
|  |  | PBCH | Clause 5.4.2.1 (Table 5.4.2.1-2 Test 1) Clause 5.4.2.1 (Table 5.4.2.1-3 Test 1) |  |
|  |  | SDR | Clause 5.5.3 |  |
|  | FR1 TDD | PDSCH | All tests in Clause 5.2.2.2.18 |  |
|  |  | PDCCH | All tests in Clause 5.3.2.2.4 |  |
|  |  | PBCH | Clause 5.4.2.2 (Table 5.4.2.2-4 Test 1) Clause 5.4.2.2 (Table 5.4.2.2-5 Test 1) |  |
|  |  | SDR | Clause 5.5.3 |  |
| Note 1: If UE support only HD-FDD in a FDD band, this UE is tested with HD-FDD mode otherwise UE is tested with full-duplex FDD mode |  |  |  |  |

#### 5.1.1.14	Applicability of requirements for eRedCap

The performance requirements in Table 5.1.1.14-1 shall apply for UEs which support optional feature supportOfERedCap-r18.

Other performance requirements mandatory for UE supporting NR operation defined in Section 5 but not included in table 5.1.1.14-1 should not be considered applicable to eRedCap UEs.

Table 5.1.1.14-1: Requirements applicability for eRedCap UEs

| UE capability | Test type |  | Test list | Applicability notes |
| --- | --- | --- | --- | --- |
| eRedCap with 1RX | FR1 FDD and HD-FDD (Note 1) | PDSCH | Clause 5.2.1.1.x |  |
|  |  | PDCCH | All tests in Clause 5.3.1.1.1 |  |
|  |  | PBCH | All tests in Clause 5.4.1.1 |  |
|  |  | SDR | Clause 5.5.1 |  |
|  | FR1 TDD | PDSCH | Clause 5.2.1.2.x |  |
|  |  | PDCCH | All tests in Clause 5.3.1.2.1 |  |
|  |  | PBCH | All tests in Clause 5.4.1.2 |  |
|  |  | SDR | Clause 5.5.1 |  |
| eRedCap with 2RX | FR1 FDD and HD-FDD (Note 1) | PDSCH | Clause 5.2.2.1.x |  |
|  |  | PDCCH | All tests in Clause 5.3.2.1.4 |  |
|  |  | PBCH | Clause 5.4.2.1 (Table 5.4.2.1-2 Test 1) Clause 5.4.2.1 (Table 5.4.2.1-3 Test 1) |  |
|  |  | SDR | Clause 5.5.1 |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.x |  |
|  |  | PDCCH | All tests in Clause 5.3.2.2.4 |  |
|  |  | PBCH | Clause 5.4.2.2 (Table 5.4.2.2-4 Test 1) Clause 5.4.2.2 (Table 5.4.2.2-5 Test 1) |  |
|  |  | SDR | Clause 5.5.1 |  |
| Note 1: If UE support only HD-FDD in a FDD band, this UE is tested with HD-FDD mode otherwise UE is tested with full-duplex FDD mode |  |  |  |  |

#### << End of changes >>