3GPP TSG-RAN5 Meeting #104	R5-245147

# Maastricht, Netherlands, August 19-23, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2974 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of inter-band CA 4Rx reference sensitivity for handheld UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Rel-18 RAN4 WI enabled several FDD bands to support 4Rx operation for handheld UE and defined the corresponding reference sensitivity requirements. RAN5 should added the 4Rx test requirements for inter-band CA configurations involving those FDD bands. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the minimum requirements and test requirements for inter-band CA configurations to support 4Rx operation for handheld UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The 4Rx operation for mentioned inter-band CA configurations can’t be tested. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3A.1_1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

7.3A.1_1.5	Test requirement

For inter-band carrier aggregation the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.2.2 with parameters specified in Table 7.3A.1_1.5-1 for PC3 2UL CA configuration and CA configuration with single uplink carrier indicating power class 3 in TS 38.508-2 table A.4.3.2A.4.1-3, and in Table 7.3A.1_1.5-1a for PC2 2UL CA configuration and CA configuration with single uplink carrier indicating power class 2 in TS 38.508-2 table A.4.3.2A.4.1-3. The test requirement of configurations for CA operating band including Band n41 also apply for the corresponding CA operating bands with Band n90 replacing Band n41.

Table 7.3A.1_1.5-1: Reference sensitivity requirement for inter band CA with PC3 single UL carrier or PC3 2UL CA

| CA configuration | Test ID |  | NR band | CBW | REFSENS test requirement of NR band (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | (MHz) | 2Rx | 4Rx |
| CA_n1A-n3A | 1 |  | n1 | 5 | -100+23+TT | -100 +23+TT |
|  |  |  | n3 | 5 | -97+TT | --97-2.7+TT |
|  | 2 |  | n1 | 5 | -100+TT | -100-2.7+TT |
|  |  |  | n3 | 5 | -97+3+TT | --97+3+TT |
|  | 3 |  | n1 | 5 | -89.6+TT | -89.6-2.7+TT |
|  |  |  | n3 | 5 | -97+19.7+TT | -97+19.7+TT |
| CA_n1A-n8A | 1 |  | n1 | 5 | -100+6+TT | -100+6+TT |
|  |  |  | n8 | 5 | -97+TT | -97-2.76+TT -97-2.47+TT |
| CA_n1A-n28A | 1 |  | n1 | 5 | -100+10.2+TT | -100+10.2+TT |
|  |  |  | n28 | 5 | -98.5+TT | -98.5-2.76+TT -98.5-2.47+TT |
|  | 2 |  | n1 | 50 | -89.6+1.1+TT | -89.6+1.1+TT |
|  |  |  | n28 | 5 | -98.5+TT | -98.5-2.76+TT -98.5-2.47+TT |
| CA_n1A-n41A | 1 |  | n1 | 50 | -93.8+TT | -93.8-2.7+TT |
|  |  |  | n41 | 10 | -94.8+6.1+TT | -94.8+6.1+TT |
|  | 2 |  | n1 | 20 | -93.8+TT | -93.8-2.7+TT |
|  |  |  | n41 | 100 | -94.8+0.7+TT | -94.8+0.7+TT |
|  | 3 |  | n1 | 5 | -93.8+18.1+TT | -93.8+18.1+TT |
|  |  |  | n41 | 100 | -94.8+TT | -94.8-2.7+TT |
| CA_n1A-n77A | 1 |  | n1 | 5 | -93.8+TT | -93.8-2.7+TT |
|  |  |  | n77 | 10 | -95.34+27.1+TT | -95.34+27.1+TT |
|  | 2 |  | n1 | 20 | -93.8+TT | -93.8-2.7+TT |
|  |  |  | n77 | 100 | -85.14+13.8+TT | -85.14+13.8+TT |
|  | 3 |  | n1 | 5 | -93.8+TT | -93.8-2.7+TT |
|  |  |  | n77 | 10 | -95.34+1.9+TT | -95.34+1.9+TT |
| CA_n1A-n78A | 1 |  | n1 | 5 | -100 +8+TT | -100 +8+TT |
|  |  |  | n78 | 10 | -95.8 +TT | -95.8 -2.2 +TT |
| CA_n2A-n48A | 1 |  | n2 | 5 | -98.0 +TT | -100.7+TT |
|  |  |  | n48 | 10 | -95.8+1.9+TT | -95.8+1.9+TT |
|  | 2 |  | n2 | 5 | -98.0 +12+TT | -98.0 +12+TT |
|  |  |  | n48 | 20 | -92.7 +TT | -94.9 +TT |
| CA_n2A-n66A | 1 |  | n2 | 5 | -98.0 +20+TT | -98.0+20+TT |
|  |  |  | n66 | 5 | -99.5+TT | -102.2+TT |
|  | 2 |  | n2 | 5 | -98.0 +TT | -100.7 +TT |
|  |  |  | n66 | 5 | -99.5 +4+TT | -99.5+4+TT |
| CA_n2A-n77A | 1 |  | n2 | 5 | -98+TT | -98-2.7+TT |
|  |  |  | n774 | 10 | -95.3+23.9+TT | -95.3+23.9+TT |
|  | 2 |  | n2 | 10 | -94.8+TT | -94.8-2.7+TT |
|  |  |  | n774 | 100 | -89.4+13.8+TT | -89.4+13.8+TT |
|  | 3 |  | n2 | 5 | -98+TT | -98-2.7+TT |
|  |  |  | n774 | 10 | -95.3+1.1+TT | -95.3+1.1+TT |
|  | 4 |  | n2 | 5 | -98+6.7+TT | -98+6.7+TT |
|  |  |  | n774 | 10 | -95.3+TT | -95.3-2.2+TT |
|  | 5 |  | n2 | 20 | -91.8+3,7+TT | -91.8+3,7+TT |
|  |  |  | n774 | 20 | -92.2+TT | -92.2-2.2+TT |
|  | 6 |  | n2 | 5 | -98.0 +26+TT | -98.0+28.7+TT |
|  |  |  | n774 | 10 | -95.3+TT | -97.5+TT |
|  | 7 |  | n2 | 5 | -98.0 +8+TT | -98.0 +8+TT |
|  |  |  | n774 | 10 | -95.3+TT | -97.5+TT |
|  | 8 |  | n2 | 5 | -98.0 +5+TT | -98.0 +5+TT |
|  |  |  | n774 | 10 | -95.3+TT | -97.5+TT |
| CA_n3A-n8A | 1 |  | n3 | 10 | -93.8+TT | -93.8-2.7+TT |
|  |  |  | n8 | 5 | -97.0+8+TT | -97.0-+8+TT |
|  | 2 |  | n3 | 10 | -93.8+6.4+TT | -93.8+6.4+TT |
|  |  |  | n8 | 5 | -97.0+TT | -97.0-2.76+TT -97.0-2.47+TT |
| CA_n3A-n41A |  | 1 | n3 | 5 | -97.0+8.2+TT | -97.0+8.2+TT |
|  |  |  | n41 | 10 | -94.9 +TT | -94.9 -2.7 +TT |
|  |  | 2 | n3 | 5 | -97.0+0.6+TT | -97.0+0.6+TT |
|  |  |  | n41 | 100 | -84.6+TT | -84.6 -2.7+TT |
| CA_n3A-n77A | 1 |  | n3 | 5 | -97+TT | -97-2.7+TT |
|  |  |  | n77 | 10 | -95.3+23.9+TT | -95.3+23.9+TT |
|  | 2 |  | n3 | 10 | -93.8+TT | -93.8-2.7+TT |
|  |  |  | n77 | 100 | -85+13.8+TT | -85+13.8+TT |
|  | 3 |  | n3 | 5 | -97+TT | -97-2.7+TT |
|  |  |  | n77 | 10 | -95.3+1.1+TT | -95.3+1.1+TT |
|  | 4 |  | n3 | 5 | -97.0 +26.0 +TT | -97.0 +26 +TT |
|  |  |  | n77 | 10 | -95.3 +TT | -95.3 -2.2+TT |
|  | 5 |  | n3 | 5 | -97.0 +8.0 +TT | -97.0+8 +TT |
|  |  |  | n77 | 10 | -95.3 +TT | -95.3 -2.2+TT |
| CA_n3A-n78A | 1 |  | n3 | 5 | -97+TT | -97-2.7+TT |
|  |  |  | n78 | 10 | -95.8+23.9+TT | -95.8+23.9+TT |
|  | 2 |  | n3 | 10 | -93.8+TT | -93.8-2.7+TT |
|  |  |  | n78 | 100 | -85.5+13.8+TT | -85.5+13.8+TT |
|  | 3 |  | n3 | 5 | -97+TT | -97-2.7+TT |
|  |  |  | n78 | 10 | -95.8+1.1+TT | -95.8+1.1+TT |
|  | 4 |  | n3 | 5 | -97+26+TT | -97+26+TT |
|  |  |  | n78 | 10 | -95.8+TT | -95.8-2.2+TT |
|  | 5 |  | n3 | 5 | -97+8+TT | -97+8+TT |
|  |  |  | n78 | 10 | -95.8+TT | -95.8-2.2+TT |
| CA_n5A-n66A | 1 |  | n5 | 5 | -98.0+30+TT | -98.0+30+TT |
|  |  |  | n66 | 5 | -99.5+TT | -102.2+TT |
| CA_n5A-n77A | 1 |  | n5 | 5 | -98+TT | -98-2.47+TT |
|  |  |  | n77 | 10 | -95.3+10.5+TT | -95.3+10.5+TT |
|  | 2 |  | n5 | 5 | -98+TT | -98-2.47+TT |
|  |  |  | n77 | 100 | -85+1.4+TT | -85+1.4+TT |
|  | 3 |  | n5 | 5 | -98+TT | -98-2.47+TT |
|  |  |  | n77 | 10 | -95.3+10.4+TT | -95.3+10.4+TT |
|  | 4 |  | n5 | 5 | -98+TT | -98-2.47+TT |
|  |  |  | n77 | 100 | -85+0.7+TT | -85+0.7+TT |
|  | 5 |  | n5 | 20 | -90.8 +2.7+TT | -90.8+2.7+TT |
|  |  |  | n77 | 20 | -92.2+TT | -94.4+TT |
|  | 6 |  | n5 | 5 | -98.0 +5.7+TT | -98.0+5.7+TT |
|  |  |  | n77 | 10 | -95.3+TT | -97.5+TT |
|  | 7 |  | n5 | 5 | -98.0 +8.3+TT | -98.0+8.3+TT |
|  |  |  | n77 | 10 | -95.3+TT | -97.5+TT |
|  | 8 |  | n5 | 5 | -98.0 +5.5+TT | -98.0+5.5+TT |
|  |  |  | n77 | 10 | -95.3+TT | -97.5+TT |
| CA_n5A-n78A | 1 |  | n5 | 5 | -98+TT | -98-2.47+TT |
|  |  |  | n78 | 10 | -95.8+10.5+TT | -95.8+10.5+TT |
|  | 2 |  | n5 | 5 | -98+TT | -98-2.47+TT |
|  |  |  | n78 | 100 | -85.5+1.4+TT | -85.5+1.4+TT |
| CA_n7A-n78A | 1 |  | n7 | 5 | -98+4.5+TT | -98+4.5+TT |
|  |  |  | n78 | 100 | -85.5+TT | -85.5-2.2+TT |
| CA_n8A-n78A | 1 |  | n8 | 5 | -97+TT | -97-2.76+TT -97-2.47+TT |
|  |  |  | n78 | 10 | -95.8+10.8+TT | -95.8+10.8+TT |
|  | 2 |  | n8 | 5 | -97+TT | -97-2.76+TT -97-2.47+TT |
|  |  |  | n78 | 100 | -85.5+1.4+TT | -85.5+1.4+TT |
|  | 3 |  | n8 | 5 | -97+8.3+TT | -97+8.3+TT |
|  |  |  | n78 | 10 | -95.8+TT | -95.8+TT |
| CA_n14A-n77A | 1 |  | n14 | 5 | -97.0 +TT | - |
|  |  |  | n77 | 10 | -95.3 +10.4 +TT | -95.3+10.4 +TT |
|  | 2 |  | n14 | 5 | -97.0 +TT | - |
|  |  |  | n77 | 100 | -85.1 +0.7+TT | -85.1 +0.7+TT |
|  | 3 |  | n14 | 5 | -97.0 + 5.5 +TT | - |
|  |  |  | n77 | 10 | -95.3 +TT | -95.3 -2.2 +TT |
|  | 4 |  | n14 | 5 | -97.0 + 31 +TT | - |
|  |  |  | n77 | 10 | -95.3 +TT | -95.3 -2.2 +TT |
|  | 5 |  | n14 | 10 | -93.8 + 28 +TT | - |
|  |  |  | n77 | 10 | -95.3 +TT | -95.3 -2.2 +TT |
| CA_n20A-n78A | 1 |  | n20 | 5 | -97.0+TT | -97.0-2.47+TT |
|  |  |  | n78 | 10 | -95.8+10.8+TT | -95.8+10.8+TT |
|  | 2 |  | n20 | 5 | -97.0+TT | -97.0-2.47+TT |
|  |  |  | n78 | 100 | -85.5+1.4+TT | -85.5+1.4+TT |
|  | 3 |  | n20 | 5 | -97.0 + 11 +TT | -97.0+11+TT |
|  |  |  | n78 | 10 | -95.8 +TT | -95.8 -2.2 +TT |
| CA_n25A-n66A | 1 |  | n66 | 5 | -99.5+TT | -99.5-2.7+TT |
|  |  |  | n25 | 5 | -96.5+20+TT | - |
|  | 2 |  | n66 | 5 | -99.5+23+TT | -99.5+23+TT |
|  |  |  | n25 | 5 | -96.5+TT | - |
|  | 3 |  | n66 | 5 | -99.5+4+TT | -99.5+4+TT |
|  |  |  | n25 | 5 | -96.5+TT | - |
| CA_n25A-n77A | 1 |  | n25 | 5 | -96.5+TT | - |
|  |  |  | n77 | 10 | -95.3+23.9+TT | -95.3+23.9+TT |
|  | 2 |  | n25 | 10 | -93.3+TT | - |
|  |  |  | n77 | 100 | -85.1+13.8+TT | -85.1+13.8+TT |
|  | 3 |  | n25 | 5 | -96.5+TT | - |
|  |  |  | n77 | 10 | -95.3+1.1+TT | -95.3+1.1+TT |
| CA_n25A-n78A | 1 |  | n25 | 5 | -96.5+TT | - |
|  |  |  | n78 | 10 | -95.8+23.9+TT | -95.3+23.9+TT |
|  | 2 |  | n25 | 10 | -93.3+TT | - |
|  |  |  | n78 | 100 | -85.5+13.8+TT | -85.5+13.8+TT |
|  | 3 |  | n25 | 5 | -96.5+TT | - |
|  |  |  | n78 | 10 | -95.8+1.1+TT | -95.3+1.1+TT |
| CA_n26A-n66A | 1 |  | n26 | 5 | -97.55 + 30 +TT | -97.6+30+TT |
|  |  |  | n66 | 5 | -99.5 +TT | -99.5-2.7 +TT |
| CA_n26A-n70A |  | 1 | n26 | 5 | -97.55 + 30 +TT | -97.5+30+TT |
|  |  |  | n70 | 5 | -92.7 + TT | -92.7 - 2.7 +TT |
| CA_n28A-n40A | 1 |  | n28 | 5 | -98.5+37.8+TT | -98.5+37.8+TT |
|  |  |  | n40 | 5 | -100.0+TT | -100.0-2.7+TT |
|  | 2 |  | n28 | 20 | -90.8+30.3+TT | -90.8+30.3+TT |
|  |  |  | n40 | 20 | -100.0+TT | -100.0-2.7+TT |
| CA_n28A-n77A | 1 |  | n28 | 5 | -98.5 +TT | -98.5-2.76+TT -98.5-2.47+TT |
|  |  |  | n77 | 10 | -95.3 +10.4 +TT | -95.3+10.4 +TT |
|  | 2 |  | n28 | 5 | -98.5 +TT | -98.5 -2.76+TT -98.5-2.47+TT |
|  |  |  | n77 | 100 | -85.4 +0.7 +TT | -85.4+0.7 +TT |
|  | 3 |  | n28 | 5 | -85.4 +0.7 +TT | -85.4+0.7 +TT |
|  |  |  | n77 | 10 | -95.3 +TT | -95.3-2.2 +TT |
| CA_n28A-n78A | 1 |  | n28 | 5 | -98.5 +TT | -98.5 -2.76 +TT -98.5-2.47+TT |
|  |  |  | n78 | 10 | -95.8 +10.4 +TT | -95.8+10.4 +TT |
|  | 2 |  | n28 | 5 | -98.5 +TT | -98.5-2.76 +TT -98.5-2.47+TT |
|  |  |  | n78 | 100 | -85.5 +0.7 +TT | -85.5 +0.7 +TT |
|  | 3 |  | n28 | 5 | -98.5 + 31 +TT | -98.5 + 31 +TT |
|  |  |  | n78 | 10 | -95.8 +TT | -92.7 -2.2 +TT |
| CA_n29A-n71A | 1 |  | n29 | 5 | -97.0 + 17.5 + TT | - |
|  |  |  | n71 | 20 | -86.0 + TT | -86.0-2.76 + TT -86.0-2.47 + TT |
| CA_n30A-n66A | 1 |  | n30 | 10 | -95.8 + TT | -95.8 -2.7 + TT |
|  |  |  | n66 | 15 | -94.5 + 8.3 + TT | -94.5 + 8.3 + TT |
| CA_n30A-n77A | 1 |  | n30 | 5 | -99 + 10.4 + TT | -99 + 10.4 + TT |
|  |  |  | n77 | 10 | -95.3 + TT | -95.3 -2.2 + TT |
|  | 2 |  | n30 | 10 | -95.8 + 8 + TT | -95.8 + 8 + TT |
|  |  |  | n77 | 10 | -95.3 + TT | -95.3 -2.2 + TT |
|  | 3 |  | n30 | 5 | -99 + 8 + TT | -99 + 8 + TT |
|  |  |  | n77 | 10 | -95.3 + TT | -95.3 -2.2 + TT |
| CA_n39A-n41A | 1 |  | n39 | 5 | -100 +TT | -100 -2.7 +TT |
|  |  |  | n41 | 5 | -94.8 +8.1 +TT | -94.8 +8.1 +TT |
|  | 2 |  | n39 | 5 | -100 +TT | -100 -2.7 +TT |
|  |  |  | n41 | 100 | -94.8 +3.3 +TT | -94.8 +3.3 +TT |
|  | 3 |  | n39 | 10 | -100 +4.3 +TT | -100 +4.3 +TT |
|  |  |  | n41 | 5 | -94.8+TT | -94.8-2.7+TT |
|  | 4 |  | n39 | 10 | -100 +0.8 +TT | -100 +0.8 +TT |
|  |  |  | n41 | 40 | -94.8+TT | -94.8-2.7+TT |
|  | 5 |  | n39 | 40 | -90.6 +TT | -90.6 -2.7 +TT |
|  |  |  | n41 | 10 | -94.8 +3.3 +TT | -94.8 +3.3 +TT |
|  | 6 |  | n39 | 100 | -100 +1.6 +TT | -100 +1.6 +TT |
|  |  |  | n41 | 10 | -84.5 +TT | -84.5 -2.7+TT |
| CA_n41A-n66A | 1 |  | n41 | 100 | -84.6+TT | -84.6-2.7+TT |
|  |  |  | n66 | 5 | -99.5+10.5+TT | -99.5+10.5+TT |
| CA_n41A-n71A | 1 |  | n41 | 10 | -94,8 + 10.8 + TT | -94,8+ 10.8 + TT |
|  |  |  | n71 | 5 | -97.2 + TT | -97.2-2.76 + TT -97.2-2.47 + TT |
|  | 2 |  | n41 | 100 | -84.6 + 1.4 + TT | -84.6+ 1.4 + TT |
|  |  |  | n71 | 5 | -97.2 +TT | -97.2 -2.76+ TT -97.2-2.47 + TT |
| CA_n41A-n77A | 1 |  | n41 | 10 | -94.8+10.4+TT | -94.8+10.4+TT |
|  |  |  | n77 | 20 | -95.3+TT | -95.3-2.2+TT |
|  | 2 |  | n41 | 100 | -94.8+6.3+TT | -94.8+6.3+TT |
|  |  |  | n77 | 20 | -95.3+TT | -95.3-2.2+TT |
|  | 3 |  | n41 | 60 | -94.8+TT | -94.8-2.7+TT |
|  |  |  | n77 | 10 | -95.3+2.7+TT | -95.3+2.7+TT |
|  | 4 |  | n41 | 100 | -94.8+TT | -94.8-2.7+TT |
|  |  |  | n77 | 10 | -95.3+2.7+TT | -95.3+2.7+TT |
|  | 5 |  | n41 | 100 | -94.8+TT | -94.8-2.7+TT |
|  |  |  | n77 | 10 | -95.3+8.3+TT | -95.3+8.3+TT |
|  | 6 |  | n41 | 10 | -94.8+4.5+TT | -94.8+4.5+TT |
|  |  |  | n77 | 100 | -95.3+TT | -95.3-2.2+TT |
|  | 7 |  | n41 | 100 | -94.8+4.5+TT | -94.8+4.5+TT |
|  |  |  | n77 | 100 | -95.3+TT | -95.3-2.2+TT |
| CA_n48A-n66A | 1 |  | n48 | 5 | -99.0 + TT | -99.0 -2.2+ TT |
|  |  |  | n66 | 5 | -99.5 + 5.0 + TT | -99.5 +5.0+TT |
|  | 2 |  | n48 | 5 | -99.0+27.1 + TT | -99.0+ 27.1+TT |
|  |  |  | n66 | 5 | -99.5 +TT | -99.5 -2.7 +TT |
|  | 3 |  | n48 | 100 | -85.5 + 13.8 +TT | -85.5-2.2 + 13.8 +TT |
|  |  |  | n66 | 40 | -90.1 +TT | -90.1 -2.7 +TT |
|  | 4 |  | n48 | 10 | -95.8 + 1.9+ TT | -95.8+ 1.9 + TT |
|  |  |  | n66 | 5 | -99.5 +TT | -99.5 -2.7 +TT |
| CA_n48A-n70A | 1 |  | n48 | 10 | -95.8 + TT | -95.8-2.2 + TT |
|  |  |  | n70 | 15 MHz UL /25 MHz DL | -92.7 +26 +TT | -92.7 +26 +TT |
| CA_n66A-n71A | 1 |  | n66 | 5 | -99.5 +5 +TT | -99.5+5 +TT |
|  |  |  | n71 | 5 | -97.2 +TT | -97.2-2.76 +TT -97.2-2.47 + TT |
| CA_n66A-n77A | 1 |  | n66 | 5 | -99.5+TT | -102.2+TT |
|  |  |  | n77 | 10 | -95.3 +23.9+TT | -95.3 +23.9+TT |
|  | 2 |  | n66 | 20 | -93.3+TT | -96.0+TT |
|  |  |  | n77 | 100 | -85.1 +13.8+TT | -85.1+13.8+TT |
|  | 3 |  | n66 | 5 | -99.5+TT | -102.2+TT |
|  |  |  | n77 | 10 | -95.3 +1.1+TT | -95.3+1.1+TT |
|  | 4 |  | n66 | 5 | -99.5 +31 +TT | -99.5+31 +TT |
|  |  |  | n77 | 10 | -95.3 +TT | -97.5+TT |
|  | 5 |  | n66 | 5 | -99.5 +5 +TT | -99.5+5 +TT |
|  |  |  | n77 | 10 | -95.3 +TT | -97.5+TT |
| CA_n66A-n78A | 1 |  | n66 | 5 | -99.5+TT | -99.5-2.7+TT |
|  |  |  | n78 | 10 | -95.8+23.9+TT | -95.8+23.9+TT |
|  | 2 |  | n66 | 20 | -93.3+TT | -93.3-2.7+TT |
|  |  |  | n78 | 100 | -85.5+13.8+TT | -85.5+13.8+TT |
|  | 3 |  | n66 | 5 | -99.5+TT | -99.5-2.7+TT |
|  |  |  | n78 | 10 | -95.8+1.1+TT | -95.8+1.1+TT |
| CA_n70A-n71A | 1 |  | n70 | 25 | -92.7 +4.1 +TT | -92.7+4.1 +TT |
|  |  |  | n71 | 5 | -94.0 +TT | -94.0-2.76 +TT -97.2-2.47 + TT |
|  | 2 |  | n70 | 5 | -100.0 +9.9 +TT | -100.0+9.9 +TT |
|  |  |  | n71 | 5 | -97.2 +TT | -97.2-2.76 +TT -97.2-2.47 + TT |
|  | 3 |  | n70 | 5 | -100.0 +5 +TT | -100.0+5 +TT |
|  |  |  | n71 | 5 | -97.2 +TT | -97.2-2.76 +TT -97.2-2.47 + TT |
| CA_n71A-n77A | 1 |  | n71 | 5 | -97.2 +5.5 +TT | -97.2+5.5 +TT |
|  |  |  | n77 | 10 | -95.3 +TT | -95.3 -2.2 +TT |
| CA_n71A-n78A | 1 |  | n71 | 5 | -97.2 +TT | -99.9 +TT -97.2-2.47 + TT |
|  |  |  | n78 | 10 | -95.8 +10.4 +TT | -95.8 +10.4 +TT |
|  | 2 |  | n71 | 5 | -97.2 +5.5 +TT | -97.2 +5.5 +TT |
|  |  |  | n78 | 10 | -95.8 +TT | -95.8 -2.2 +TT |
| CA_n78A-n79A | 1 |  | n78 | 100 | -85.5+TT | -85.5-2.2+TT |
|  |  |  | n79 | 10 | -96.1 +2+TT | -96.1 +2+TT |
|  | 2 |  | n78 | 100 | -85.5+TT | -85.5-2.2+TT |
|  |  |  | n79 | 40 | -89.6 +2+TT | -89.6 +2+TT |
|  | 3 |  | n78 | 10 | -96.1 +2.6+TT | -96.1+2.6+TT |
|  |  |  | n79 | 100 | -85.5+TT | -85.5-2.2+TT |
|  | 4 |  | n78 | 100 | -85.5+2.6+TT | -85.5+2.6+TT |
|  |  |  | n79 | 100 | -85.5+TT | -85.5-2.2+TT |
| Note 1: The transmitter shall be set to maximum output power level (Table 7.3A.3.5-2) Note 2: The reference measurement channel is specified in Annexe A2.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2. Note 3: TT for each frequency and channel bandwidth is specified in Table 7.3.2.5-3. Note 4: The requirement is modified by -0.5 dB when the assigned UE channel bandwidth is confined within 3300 - 3800 MHz. Note 5: Values are modified by -0.5dB when carrier channel BW is between 865MHz and 894MHz. Note 6: When 4 Rx operation is supported by FWA form factor. Note 7: When 4Rx operation is supported by handheld UE. |  |  |  |  |  |  |

Table 7.3A.1_1.5-1a: Reference sensitivity requirement for inter band CA with PC2 single UL carrier or PC2 2UL CA

| CA configuration | Test ID | NR band | CBW | REFSENS test requirement of NR band (dBm) |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | (MHz) | 2Rx | 4Rx |
| CA_n1A-n78A | 1 | n1 | 5 | -100 +17.8+TT | -100 +17.8+TT |
|  |  | n78 | 10 | -95.8 +TT | -95.8 -2.2 +TT |
| CA_n2A-n77A | 4 | n2 | 5 | -98+9.1+TT | -98+9.1+TT |
|  |  | n774 | 10 | -95.3+TT | -95.3-2.2+TT |
|  | 5 | n2 | 5 | -91.8+6.7+TT | -91.8+6.7+TT |
|  |  | n774 | 10 | -92.2+TT | -92.2-2.2+TT |
|  | 6 | n2 | 5 | -98.0+32.1+TT | -98.0 +32.1+TT |
|  |  | n774 | 10 | -95.3+TT | -95.3+TT |
|  | 7 | n2 | 5 | -98.0 +19.1+TT | -98.0 +19.1+TT |
|  |  | n774 | 10 | -95.3+TT | -97.5+TT |
|  | 9 | n2 | 5 | -98.0 +1.0 +TT | -98.0+1.0 +TT |
|  |  | n774 | 100 | -85.1 +TT | -87.3 +TT |
| CA_n3A-n41A | 1 | n3 | 5 | -97.0+18.4+TT | -97.0 +18.4+TT |
|  |  | n41 | 10 | -94.9 +TT | -94.9 -2.7 +TT |
|  | 2 | n3 | 5 | -97.0+2.3+TT | -97.0+2.3+TT |
|  |  | n41 | 100 | -84.6+TT | -84.6 -2.7+TT |
| CA_n3A-n78A | 6 | n3 | 5 | -97.0 + 8.1 +TT | -97.0 +8.1 +TT |
|  |  | n78 | 5 | -97.0 +TT | -97.0 -2.7 +TT |
|  | 7 | n3 | 40 | -89.4 + 1.0 +TT | -89.4 + 1.0+TT |
|  |  | n78 | 40 | -89.4 +TT | -89.4 -2.7 +TT |
| CA_n5A-n77A | 5 | n5 | 20 | -90.8 +4.3+TT | -90.8+4.3+TT |
|  |  | n77 | 20 | -92.2+TT | -94.4+TT |
|  | 6 | n5 | 10 | -98.0 +8.1+TT | -98.0+8.1+TT |
|  |  | n77 | 10 | -95.3+TT | -97.5+TT |
|  | 7 | n5 | 5 | -98.0 +18.6+TT | -98.0+18.6+TT |
|  |  | n77 | 10 | -95.3+TT | -97.5+TT |
| CA_n14A_n77A | 4 | n14 | 5 | -97.0 + 34 +TT | - |
|  |  | n77 | 10 | -95.3+TT | -97.5+TT |
|  | 5 | n14 | 10 | -93.8 + 31 +TT | - |
|  |  | n77 | 10 | -95.3+TT | -97.5+TT |
| CA_n41A-n79A | 1 | n41 | 100 | -84.6+TT | -84.6 -2.7 +TT |
|  |  | n79 | 10 | -98.9+3.1+TT | -98.9+3.1+TT |
|  | 2 | n41 | 10 | -94.8+3.5+TT | -94.8+3.5+TT |
|  |  | n79 | 100 | -85.6+TT | -85.6 -2.2+TT |
| CA_n66A-n77A | 4 | n66 | 5 | -99.5 +34.33 +TT | -99.5 +34.33 +TT |
|  |  | n77 | 10 | -95.3 +TT | -97.5+TT |
|  | 5 | n66 | 5 | -99.5 +11.27 +TT | -99.5+11.27 +TT |
|  |  | n77 | 10 | -95.3 +TT | -97.5+TT |
| CA_n78A-n79A | 1 | n78 | 100 | -85.5+TT | -85.5-2.2+TT |
|  |  | n79 | 10 | -96.1+5+TT | -96.1+5+TT |
|  | 2 | n78 | 100 | -85.5+TT | -85.5-2.2+TT |
|  |  | n79 | 40 | -89.6 +5+TT | -89.6+5+TT |
|  | 3 | n78 | 10 | -96.1+5.6+TT | -96.1+5.6+TT |
|  |  | n79 | 100 | -85.5+TT | -85.5-2.2+TT |
|  | 4 | n78 | 100 | -85.5+5.6+TT | -85.5+5.6+TT |
|  |  | n79 | 100 | -85.5+TT | -85.5-2.2+TT |
|  | 5 | n78 | 100 | -85.5+TT | -85.5-2.2+TT |
|  |  | n79 | 100 | -85.5+5+TT | -85.5+5+TT |
| Note 1: The transmitter shall be set to maximum output power level (Table 7.3A.3.5-2) Note 2: The reference measurement channel is specified in Annexe A2.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2. Note 3: TT for each frequency and channel bandwidth is specified in Table 7.3.2.5-3. Note 4: The requirement is modified by -0.5 dB when the assigned UE channel bandwidth is confined within 3300 - 3800 MHz. Note 5: Values are modified by -0.5dB when carrier channel BW is between 865MHz and 894MHz. Note 6: When 4 Rx operation is supported by FWA form factor. |  |  |  |  |  |

<End of Changes>