3GPP TSG-RAN5 Meeting #104 	R5-245215

# Maastricht, Netherlands, 19th Aug. 2024 - 23rd Aug. 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3319 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of test frequency for n8 and n71 CBW 35 MHz |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_lic_bands_BW_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The availabe bandwidth of NR band n8 and 71 is 35 MHz, so the test frequency of Low/Mid/High are the same when CBW = 35 MHz. It should be clarified in 4.3.1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added notes to Table 4.3.1.1.1.8-x and Table 4.3.1.1.1.71-x. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It will make a confusion when testing n8 and n71 with CBW = 25 MHz. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.1.1.8, 4.3.1.1.1.71 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

4.3.1.1.1.8	Reference test frequencies for NR operating band n8

Table 4.3.1.1.1.8-1: Test frequencies for NR operating band n8 and SCS 15 kHz

| UL/DL Bandwidth combination [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5/5 | 25 | Downlink | Low | 927.5 | 185500 | 925.25 | 185050 | 0 | 15 | 2318 | 185530 | 4 | 1 | 1 (2) | 3 |
|  |  |  | Mid | 942.5 | 188500 | 921.89 | 184378 | 102 |  | 2354 | 188410 | 0 | 0 | 0 (0) | 102 |
|  |  |  | High | 957.5 | 191500 | 864.53 | 172906 | 504 |  | 2393 | 191530 | 4 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 882.5 | 176500 | 880.25 | 176050 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 804.53 | 160906 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 912.5 | 182500 | 909.17 | 181834 | 6 |  | - | - | - | - | - | - |
| 10/10 | 52 | Downlink | Low | 930 | 186000 | 925.32 | 185064 | 0 | 15 | 2319 | 185550 | 6 | 1 | 1 (2) | 3 |
|  |  |  | Mid | 942.5 | 188500 | 919.46 | 183892 | 102 |  | 2348 | 187930 | 2 | 0 | 0 (0) | 102 |
|  |  |  | High | 955 | 191000 | 859.6 | 171920 | 504 |  | 2383 | 190610 | 2 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 885 | 177000 | 880.32 | 176064 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 802.1 | 160420 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 910 | 182000 | 904.24 | 180848 | 6 |  | - | - | - | - | - | - |
| 10/25 | 133 | Downlink | Low | 937.5 | 187500 | 925.53 | 185106 | 0 | 15 | 2344 | 185550 | 4 | 0 | 1 (2) | 2 |
|  |  |  | Mid | 942.5 | 188500 | 912.17 | 182434 | 102 |  | 2344 | 186490 | 8 | 0 | 0 (0) | 102 |
|  |  |  | High | 947.5 | 189500 | 844.81 | 168962 | 504 |  | 2344 | 187490 | 8 | 0 | 0 (0) | 504 |
|  | 52 | Uplink | Low | 892.5 | 178500 | 887.82 | 177564 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 802.1 | 160420 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 902.5 | 180500 | 896.74 | 179348 | 6 |  | - | - | - | - | - | - |
| 10/35 | 188 | Downlink | Low | 942.5 | 188500 | 925.58 | 185116 | 0 | 15 | 2318 | 185530 | 6 | 1 | 0 (0) | 1 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 52 | Uplink | Low | 897.5 | 179500 | 892.82 | 178564 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| 15/15 | 79 | Downlink | Low | 932.5 | 186500 | 925.39 | 185078 | 0 | 15 | 2320 | 185570 | 8 | 1 | 1 (2) | 3 |
|  |  |  | Mid | 942.5 | 188500 | 917.03 | 183406 | 102 |  | 2342 | 187450 | 4 | 0 | 0 (0) | 102 |
|  |  |  | High | 952.5 | 190500 | 854.67 | 170934 | 504 |  | 2370 | 189630 | 4 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 887.5 | 177500 | 880.39 | 176078 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 799.67 | 159934 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 907.5 | 181500 | 899.31 | 179862 | 6 |  | - | - | - | - | - | - |
| 15/25 | 133 | Downlink | Low | 937.5 | 187500 | 925.53 | 185106 | 0 | 15 | 2319 | 185550 | 4 | 0 | 1 (2) | 2 |
|  |  |  | Mid | 942.5 | 188500 | 912.17 | 182434 | 102 |  | 2330 | 186490 | 8 | 0 | 0 (0) | 102 |
|  |  |  | High | 947.5 | 189500 | 844.81 | 168962 | 504 |  | 2344 | 187490 | 8 | 0 | 0 (0) | 504 |
|  | 79 | Uplink | Low | 892.5 | 178500 | 885.39 | 177078 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 799.67 | 159934 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 902.5 | 180500 | 894.31 | 178862 | 6 |  | - | - | - | - | - | - |
| 15/35 | 188 | Downlink | Low | 942.5 | 188500 | 925.58 | 185116 | 0 | 15 | 2318 | 185530 | 6 | 1 | 0 (0) | 1 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 79 | Uplink | Low | 897.58 | 179500 | 890.398 | 178078 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| 20/20 | 106 | Downlink | Low | 935 | 187000 | 925.46 | 185092 | 0 | 15 | 2318 | 185530 | 2 | 0 | 1 (2) | 2 |
|  |  |  | Mid | 942.5 | 188500 | 914.6 | 182920 | 102 |  | 2336 | 186970 | 6 | 0 | 0 (0) | 102 |
|  |  |  | High | 950 | 190000 | 849.74 | 169948 | 504 |  | 2357 | 188650 | 6 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 890 | 178000 | 880.46 | 176092 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 797.24 | 159448 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 905 | 181000 | 894.38 | 178876 | 6 |  | - | - | - | - | - | - |
| 20/25 | 133 | Downlink | Low | 937.5 | 187500 | 925.53 | 185106 | 0 | 15 | 2319 | 185550 | 4 | 0 | 1 (2) | 2 |
|  |  |  | Mid | 942.5 | 188500 | 912.17 | 182434 | 102 |  | 2330 | 186490 | 8 | 0 | 0 (0) | 102 |
|  |  |  | High | 947.5 | 189500 | 844.81 | 168962 | 504 |  | 2344 | 187490 | 8 | 0 | 0 (0) | 504 |
|  | 106 | Uplink | Low | 892.5 | 178500 | 882.96 | 176592 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 797.24 | 159448 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 902.5 | 180500 | 891.88 | 178376 | 6 |  | - | - | - | - | - | - |
| 20/35 | 188 | Downlink | Low | 942.5 | 188500 | 925.58 | 185116 | 0 | 15 | 2318 | 185530 | 6 | 1 | 0 (0) | 1 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 106 | Uplink | Low | 897.5 | 179500 | 887.96 | 177592 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: 10/35 MHz, 15/35 MHz and 20/35 MHz UL/DL Bandwidth combination is tested with Low range test frequency only. Low range test frequency shall be used instead of Mid range and High range test frequencies. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.8-2: Test frequencies for NR operating band n8 and SCS 30 kHz

| UL/DL Bandwidth combination [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10/10 | 24 | Downlink | Low | 930 | 186000 | 925.68 | 185136 | 0 | 15 | 2325 | 186030 | 10 | 0 | 2 (7) | 14 |
|  |  |  | Mid | 942.5 | 188500 | 901.46 | 180292 | 102 |  | 2354 | 188410 | 18 | 0 | 0 (5) | 214 |
|  |  |  | High | 955 | 191000 | 769.24 | 153848 | 504 |  | 2389 | 191090 | 6 | 0 | 3 (8) | 1024 |
|  |  | Uplink | Low | 885 | 177000 | 880.68 | 176136 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 711.74 | 142348 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 910 | 182000 | 903.52 | 180704 | 6 |  | - | - | - | - | - | - |
| 10/25 | 65 | Downlink | Low | 937.5 | 187500 | 925.8 | 185160 | 0 | 15 | 2325 | 186030 | 2 | 0 | 2 (7) | 14 |
|  |  |  | Mid | 942.5 | 188500 | 894.08 | 178816 | 102 |  | 2336 | 186970 | 6 | 0 | 1 (6) | 216 |
|  |  |  | High | 947.5 | 189500 | 754.36 | 150872 | 504 |  | 2350 | 187970 | 6 | 0 | 1 (6) | 1020 |
|  | 24 | Uplink | Low | 892.5 | 178500 | 888.18 | 177636 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 711.74 | 142348 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 902.5 | 180500 | 896.02 | 179204 | 6 |  | - | - | - | - | - | - |
| 10/35 | 92 | Downlink | Low | 942.5 | 188500 | 925.94 | 185188 | 0 | 15 | 2324 | 186010 | 10 | 0 | 1 (6) | 12 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 24 | Uplink | Low | 897.56 | 179500 | 893.18 | 178636 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| 15/15 | 38 | Downlink | Low | 932.5 | 186500 | 925.66 | 185132 | 0 | 15 | 2326 | 186050 | 18 | 0 | 2 (7) | 14 |
|  |  |  | Mid | 942.5 | 188500 | 898.94 | 179788 | 102 |  | 2348 | 187930 | 2 | 0 | 1 (6) | 216 |
|  |  |  | High | 952.5 | 190500 | 764.22 | 152844 | 504 |  | 2373 | 189870 | 6 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low | 887.5 | 177500 | 880.66 | 176132 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 709.22 | 141844 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 907.5 | 181500 | 898.5 | 179700 | 6 |  | - | - | - | - | - | - |
| 15/25 | 65 | Downlink | Low | 937.5 | 187500 | 925.8 | 185160 | 0 | 15 | 2325 | 186030 | 2 | 0 | 2 (7) | 14 |
|  |  |  | Mid | 942.5 | 188500 | 894.08 | 178816 | 102 |  | 2336 | 186970 | 6 | 0 | 1 (6) | 216 |
|  |  |  | High | 947.5 | 189500 | 754.36 | 150872 | 504 |  | 2350 | 187970 | 6 | 0 | 1 (6) | 1020 |
|  | 38 | Uplink | Low | 892.5 | 178500 | 885.66 | 177132 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 709.22 | 141844 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 902.5 | 180500 | 893.5 | 178700 | 6 |  | - | - | - | - | - | - |
| 15/35 | 92 | Downlink | Low | 942.5 | 188500 | 925.94 | 185188 | 0 | 15 | 2324 | 186010 | 10 | 0 | 1 (6) | 12 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 38 | Uplink | Low | 897.5 | 179500 | 890.66 | 178132 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| 20/20 | 51 | Downlink | Low | 935 | 187000 | 925.82 | 185164 | 0 | 15 | 2324 | 186010 | 18 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 942.5 | 188500 | 896.6 | 179320 | 102 |  | 2342 | 187450 | 22 | 0 | 0 (5) | 214 |
|  |  |  | High | 950 | 190000 | 759.38 | 151876 | 504 |  | 2360 | 188890 | 2 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low | 890 | 178000 | 880.82 | 176164 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 706.88 | 141376 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 905 | 181000 | 893.66 | 178732 | 6 |  | - | - | - | - | - | - |
| 20/25 | 65 | Downlink | Low | 937.5 | 187500 | 925.8 | 185160 | 0 | 15 | 2325 | 186030 | 2 | 0 | 2 (7) | 14 |
|  |  |  | Mid | 942.5 | 188500 | 894.08 | 178816 | 102 |  | 2336 | 186970 | 6 | 0 | 1 (6) | 216 |
|  |  |  | High | 947.5 | 189500 | 754.36 | 150872 | 504 |  | 2350 | 187970 | 6 | 0 | 1 (6) | 1020 |
|  | 51 | Uplink | Low | 892.5 | 178500 | 883.32 | 176664 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 706.88 | 141376 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 902.5 | 180500 | 891.16 | 178232 | 6 |  | - | - | - | - | - | - |
| 20/35 | 92 | Downlink | Low | 942.5 | 188500 | 925.94 | 185188 | 0 | 15 | 2324 | 186010 | 10 | 0 | 1 (6) | 12 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 51 | Uplink | Low | 897.5 | 179500 | 888.32 | 177664 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: 10/35 MHz, 15/35 MHz and 20/35 MHz UL/DL Bandwidth combination is tested with Low range test frequency only. Low range test frequency shall be used instead of Mid range and High range test frequencies. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

4.3.1.1.1.71	Reference test frequencies for NR operating band n71

Table 4.3.1.1.1.71-1: Test frequencies for NR operating band n71 and SCS 15 kHz

| UL/DL Bandwidth combination [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5/5 | 25 | Downlink | Low | 619.5 | 123900 | 617.25 | 123450 | 0 | 15 | 1548 | 123870 | 8 | 1 | 0 (0) | 1 |
|  |  |  | Mid | 634.5 | 126900 | 613.89 | 122778 | 102 |  | 1587 | 126990 | 0 | 1 | 2 (4) | 107 |
|  |  |  | High | 649.5 | 129900 | 556.53 | 111306 | 504 |  | 1623 | 129870 | 8 | 1 | 0 (0) | 505 |
|  |  | Uplink | Low | 665.5 | 133100 | 663.25 | 132650 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 680.5 | 136100 | 587.53 | 117506 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 695.5 | 139100 | 692.17 | 138434 | 6 |  | - | - | - | - | - | - |
| 5/10 | 52 | Downlink | Low | 622 | 124400 | 617.32 | 123464 | 0 | 15 | 1549 | 123890 | 10 | 1 | 0 (0) | 1 |
|  |  |  | Mid | 634.5 | 126900 | 611.46 | 122292 | 102 |  | 1581 | 126510 | 2 | 1 | 2 (4) | 107 |
|  |  |  | High | 647 | 129400 | 551.6 | 110320 | 504 |  | 1610 | 128890 | 10 | 1 | 0 (0) | 505 |
|  | 25 | Uplink | Low | 665.5 | 133100 | 663.25 | 132650 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 678 | 135600 | 585.03 | 117006 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 690.5 | 138100 | 687.17 | 137434 | 6 |  | - | - | - | - | - | - |
| 10/10 | 52 | Downlink | Low | 622 | 124400 | 617.32 | 123464 | 0 | 15 | 1549 | 123890 | 10 | 1 | 0 (0) | 1 |
|  |  |  | Mid | 634.5 | 126900 | 611.46 | 122292 | 102 |  | 1581 | 126510 | 2 | 1 | 2 (4) | 107 |
|  |  |  | High | 647 | 129400 | 551.6 | 110320 | 504 |  | 1610 | 128890 | 10 | 1 | 0 (0) | 505 |
|  |  | Uplink | Low | 668 | 133600 | 663.32 | 132664 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 680.5 | 136100 | 585.1 | 117020 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 693 | 138600 | 687.24 | 137448 | 6 |  | - | - | - | - | - | - |
| 10/15 | 79 | Downlink | Low | 624.5 | 124900 | 617.39 | 123478 | 0 | 15 | 1547 | 123850 | 4 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 634.5 | 126900 | 609.03 | 121806 | 102 |  | 1575 | 126030 | 4 | 1 | 2 (4) | 107 |
|  |  |  | High | 644.5 | 128900 | 546.67 | 109334 | 504 |  | 1600 | 127970 | 8 | 1 | 1 (2) | 507 |
|  | 52 | Uplink | Low | 668 | 133600 | 663.32 | 132664 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 678 | 135600 | 582.6 | 116520 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 688 | 137600 | 682.24 | 136448 | 6 |  | - | - | - | - | - | - |
| 15/15 | 79 | Downlink | Low | 624.5 | 124900 | 617.39 | 123478 | 0 | 15 | 1547 | 123850 | 4 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 634.5 | 126900 | 609.03 | 121806 | 102 |  | 1575 | 126030 | 4 | 1 | 2 (4) | 107 |
|  |  |  | High | 644.5 | 128900 | 546.67 | 109334 | 504 |  | 1600 | 127970 | 8 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 670.5 | 134100 | 663.39 | 132678 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 680.5 | 136100 | 582.67 | 116534 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 690.5 | 138100 | 682.31 | 136462 | 6 |  | - | - | - | - | - | - |
| 15/20 | 106 | Downlink | Low | 627 | 125400 | 617.46 | 123492 | 0 | 15 | 1548 | 123870 | 6 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 634.5 | 126900 | 606.6 | 121320 | 102 |  | 1569 | 125550 | 6 | 1 | 2 (4) | 107 |
|  |  |  | High | 642 | 128400 | 541.74 | 108348 | 504 |  | 1587 | 126990 | 10 | 1 | 1 (2) | 507 |
|  | 79 | Uplink | Low | 670.5 | 134100 | 663.39 | 132678 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 678 | 135600 | 580.17 | 116034 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 685.5 | 137100 | 677.31 | 135462 | 6 |  | - | - | - | - | - | - |
| 20/20 | 106 | Downlink | Low | 627 | 125400 | 617.46 | 123492 | 0 | 15 | 1548 | 123870 | 6 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 634.5 | 126900 | 606.6 | 121320 | 102 |  | 1569 | 125550 | 6 | 1 | 2 (4) | 107 |
|  |  |  | High | 642 | 128400 | 541.74 | 108348 | 504 |  | 1587 | 126990 | 10 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 673 | 134600 | 663.46 | 132692 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 680.5 | 136100 | 580.24 | 116048 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 688 | 137600 | 677.38 | 135476 | 6 |  | - | - | - | - | - | - |
| 20/25 | 133 | Downlink | Low | 629.5 | 125900 | 617.53 | 123506 | 0 | 15 | 1549 | 123890 | 8 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 634.5 | 126900 | 604.17 | 120834 | 102 |  | 1563 | 125070 | 8 | 1 | 2 (4) | 107 |
|  |  |  | High | 639.5 | 127900 | 536.81 | 107362 | 504 |  | 1574 | 126010 | 0 | 0 | 2 (4) | 508 |
|  | 106 | Uplink | Low | 675.5 | 135100 | 665.96 | 133192 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 680.5 | 136100 | 580.24 | 116048 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 685.5 | 137100 | 674.88 | 134976 | 6 |  | - | - | - | - | - | - |
| 20/30 | 160 | Downlink | Low | 632 | 126400 | 617.6 | 123520 | 0 | 15 | 1550 | 124090 | 10 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 634.5 | 126900 | 601.74 | 120348 | 102 |  | 1557 | 124590 | 10 | 1 | 2 (4) | 107 |
|  |  |  | High | 637 | 127400 | 531.88 | 106376 | 504 |  | 1564 | 125090 | 10 | 1 | 2 (4) | 509 |
|  | 106 | Uplink | Low | 678 | 135600 | 668.46 | 133692 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 680.5 | 136100 | 580.24 | 116048 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 683 | 136600 | 672.38 | 134476 | 6 |  | - | - | - | - | - | - |
| 20/35 | 188 | Downlink | Low | 634.5 | 126900 | 617.58 | 123516 | 0 | 15 | 1551 | 124110 | 6 | 2 | 2 (4) | 6 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 106 | Uplink | Low | 680.5 | 136100 | 670.96 | 134192 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: 20/35 MHz UL/DL Bandwidth combination is tested with Low range test frequency only. Low range test frequency shall be used instead of Mid range and High range test frequencies. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.71-2: Test frequencies for NR operating band n71 and SCS 30 kHz

| UL/DL Bandwidth combination [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10/10 | 24 | Downlink | Low | 622 | 124400 | 617.68 | 123536 | 0 | 15 | 1555 | 124370 | 14 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 634.5 | 126900 | 593.46 | 118692 | 102 |  | 1587 | 126990 | 6 | 0 | 3 (8) | 220 |
|  |  |  | High | 647 | 129400 | 461.24 | 92248 | 504 |  | 1616 | 129370 | 14 | 0 | 1 (6) | 1020 |
|  |  | Uplink | Low | 668 | 133600 | 663.68 | 132736 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 680.5 | 136100 | 494.74 | 98948 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 693 | 138600 | 686.52 | 137304 | 6 |  | - | - | - | - | - | - |
| 10/15 | 38 | Downlink | Low | 624.5 | 124900 | 617.66 | 123532 | 0 | 15 | 1553 | 124330 | 2 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 634.5 | 126900 | 590.94 | 118188 | 102 |  | 1578 | 126270 | 6 | 0 | 0 (5) | 214 |
|  |  |  | High | 644.5 | 128900 | 456.22 | 91244 | 504 |  | 1606 | 128450 | 18 | 0 | 2 (7) | 1022 |
|  | 24 | Uplink | Low | 668 | 133600 | 663.68 | 132736 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 678 | 135600 | 492.24 | 98448 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 688 | 137600 | 681.52 | 136304 | 6 |  | - | - | - | - | - | - |
| 15/15 | 38 | Downlink | Low | 624.5 | 124900 | 617.66 | 123532 | 0 | 15 | 1553 | 124330 | 2 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 634.5 | 126900 | 590.94 | 118188 | 102 |  | 1578 | 126270 | 6 | 0 | 0 (5) | 214 |
|  |  |  | High | 644.5 | 128900 | 456.22 | 91244 | 504 |  | 1606 | 128450 | 18 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 670.5 | 134100 | 663.66 | 132732 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 680.5 | 136100 | 492.22 | 98444 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 690.5 | 138100 | 681.5 | 136300 | 6 |  | - | - | - | - | - | - |
| 15/20 | 51 | Downlink | Low | 627 | 125400 | 617.82 | 123564 | 0 | 15 | 1554 | 124350 | 22 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 634.5 | 126900 | 588.6 | 117720 | 102 |  | 1572 | 125790 | 2 | 0 | 0 (5) | 214 |
|  |  |  | High | 642 | 128400 | 451.38 | 90276 | 504 |  | 1593 | 127470 | 14 | 0 | 2 (7) | 1022 |
|  | 38 | Uplink | Low | 670.5 | 134100 | 663.66 | 132732 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 678 | 135600 | 489.72 | 97944 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 685.5 | 137100 | 676.5 | 135300 | 6 |  | - | - | - | - | - | - |
| 20/20 | 51 | Downlink | Low | 627 | 125400 | 617.82 | 123564 | 0 | 15 | 1554 | 124350 | 22 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 634.5 | 126900 | 588.6 | 117720 | 102 |  | 1572 | 125790 | 2 | 0 | 0 (5) | 214 |
|  |  |  | High | 642 | 128400 | 451.38 | 90276 | 504 |  | 1593 | 127470 | 14 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 673 | 134600 | 663.82 | 132764 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 680.5 | 136100 | 489.88 | 97976 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 688 | 137600 | 676.66 | 135332 | 6 |  | - | - | - | - | - | - |
| 20/25 | 65 | Downlink | Low | 629.5 | 125900 | 617.8 | 123560 | 0 | 15 | 1555 | 124370 | 6 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 634.5 | 126900 | 586.08 | 117216 | 102 |  | 1566 | 125310 | 10 | 0 | 0 (5) | 214 |
|  |  |  | High | 639.5 | 127900 | 446.36 | 89272 | 504 |  | 1580 | 126490 | 22 | 0 | 2 (7) | 1022 |
|  | 51 | Uplink | Low | 675.5 | 135100 | 666.32 | 133264 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 680.5 | 136100 | 489.88 | 97976 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 685.5 | 137100 | 674.16 | 134832 | 6 |  | - | - | - | - | - | - |
| 20/30 | 78 | Downlink | Low | 632 | 126400 | 617.96 | 123592 | 0 | 15 | 1553 | 124330 | 6 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 634.5 | 126900 | 583.74 | 116748 | 102 |  | 1560 | 124830 | 6 | 0 | 0 (5) | 214 |
|  |  |  | High | 637 | 127400 | 441.52 | 88304 | 504 |  | 1567 | 125330 | 6 | 0 | 0 (5) | 1018 |
|  | 51 | Uplink | Low | 678 | 135600 | 668.82 | 133764 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 680.5 | 136100 | 489.88 | 97976 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 683 | 136600 | 671.66 | 134332 | 6 |  | - | - | - | - | - | - |
| 20/35 | 92 | Downlink | Low | 634.5 | 126900 | 617.94 | 123588 | 0 | 15 | 1554 | 124350 | 14 | 0 | 0 (5) | 10 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 51 | Uplink | Low | 680.5 | 136100 | 671.32 | 134264 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid |  |  |  |  |  |  | - | - | - | - | - | - |
|  |  |  |  |  |  |  |  |  |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: 20/35 MHz UL/DL Bandwidth combination is tested with Low range test frequency only. Low range test frequency shall be used instead of Mid range and High range test frequencies. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<<End of change>>