3GPP TSG-RAN5 Meeting #104	R5-245254

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4647 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to NR NTN test case 6.1.2.18b |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As specified in TS 38.304, 5.2.4.2, UE shall perform measurements before t-service expiry regardless of the distance between UE and the serving cell reference location. So, test case can be further enhanced by adding reference location and distance threshold parameters in SIB 19. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added distance related parameters in Table 6.1.2.18b.3.3-4. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Reduced test coverage. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.2.18b |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

#### 6.1.2.18b	NR NTN / Cell reselection, Sintrasearch, Snonintrasearch / time based measurements

6.1.2.18b.1	Test Purpose (TP)

(1)

with { UE supporting time-based measurements in NR RRC_IDLE state on a cell which provides access by NR NTN }

ensure that {

when { t-Service of the serving cell is present in SIB19 and Sintrasearch is non-zero in system information }

then { UE performs intra-frequency measurements and reselects the highest ranked cell up on Srxlev > SIntraSearchP }

}

(2)

with { UE supporting time-based measurements in NR RRC_IDLE state on a cell which provides access by NR NTN }

ensure that {

when { t-Service of the serving cell is present in SIB19 and Snonintrasearch is non-zero in system information }

then { UE performs inter-frequency measurements and reselects the cell which belong to the lower priority frequency upon Srxlev > Snonintrasearch }

}

6.1.2.18b.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.304, clauses 5.2.4.2 and 5.2.4.7.0. Unless otherwise stated these are Rel-17 requirements.

[TS 38.304, clause 5.2.4.2]

Following rules are used by the UE to limit needed measurements:

…

If the t-Service of the serving cell is present in SIB19, and if UE supports time-based measurement initiation, the UE shall perform intra-frequency, inter-frequency or inter-RAT measurements before the t-Service, regardless of the distance between UE and the serving cell reference location or whether the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ, or Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, The exact time to start measurement before t-Service is up to UE implementation. UE shall perform measurements of higher priority NR inter-frequency or inter-RAT frequencies according to TS 38.133 [8] regardless of the remaining service time of the serving cell (i.e. time remaining until t-Service).

[TS 38.304, clause 5.2.4.7.0]

Cell reselection parameters are broadcast in system information and are read from the serving cell as follows:

…

cellReselectionPriority

This specifies the absolute priority for NR frequency or E-UTRAN frequency.

…

Qqualmin

This specifies the minimum required quality level in the cell in dB.

Qrxlevmin

This specifies the minimum required Rx level in the cell in dBm.

Qrxlevminoffsetcell

This specifies the cell specific Rx level offset in dB to Qrxlevmin.

Qqualminoffsetcell

This specifies the cell specific quality level offset in dB to Qqualmin.

…

SIntraSearchP

This specifies the Srxlev threshold (in dB) for intra-frequency measurements.

SIntraSearchQ

This specifies the Squal threshold (in dB) for intra-frequency measurements.

SnonIntraSearchP

This specifies the Srxlev threshold (in dB) for NR inter-frequency and inter-RAT measurements.

SnonIntraSearchQ

This specifies the Squal threshold (in dB) for NR inter-frequency and inter-RAT measurements.

…

TreselectionRAT

This specifies the cell reselection timer value. For each target NR frequency and for each RAT other than NR, a specific value for the cell reselection timer is defined, which is applicable when evaluating reselection within NR or towards other RAT (i.e. TreselectionRAT for NR is TreselectionNR, for E-UTRAN TreselectionEUTRA).

NOTE:	TreselectionRAT is not broadcast in system information but used in reselection rules by the UE for each RAT.

TreselectionNR

This specifies the cell reselection timer value TreselectionRAT for NR. The parameter can be set per NR frequency as specified in TS 38.331 [3].

TreselectionEUTRA

This specifies the cell reselection timer value TreselectionRAT for E-UTRAN.

ThreshX, HighP

This specifies the Srxlev threshold (in dB) used by the UE when reselecting towards a higher priority RAT/ frequency than the current serving frequency. Each frequency of NR and E-UTRAN might have a specific threshold.

ThreshX, HighQ

This specifies the Squal threshold (in dB) used by the UE when reselecting towards a higher priority RAT/ frequency than the current serving frequency. Each frequency of NR and E-UTRAN might have a specific threshold.

ThreshX, LowP

This specifies the Srxlev threshold (in dB) used by the UE when reselecting towards a lower priority RAT/ frequency than the current serving frequency. Each frequency of NR and E-UTRAN might have a specific threshold.

ThreshX, LowQ

This specifies the Squal threshold (in dB) used by the UE when reselecting towards a lower priority RAT/ frequency than the current serving frequency. Each frequency of NR and E-UTRAN might have a specific threshold.

ThreshServing, LowP

This specifies the Srxlev threshold (in dB) used by the UE on the serving cell when reselecting towards a lower priority RAT/ frequency.

ThreshServing, LowQ

This specifies the Squal threshold (in dB) used by the UE on the serving cell when reselecting towards a lower priority RAT/ frequency.

…

6.1.2.18b.3	Test description

6.1.2.18b.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1, NR Cell 11 and NR Cell 12 as defined in TS 38.508-1 [4] Table 4.4.2-2 is used.

-	NR Cell 1 uses System information combination NR-29 as defined in TS 38.508-1 [4] Table 4.4.3.1.2-1 is used in NR cells.

-	NR Cell 11 and NR Cell 12 uses System information combination NR-32 as defined in TS 38.508-1 [4] Table 4.4.3.1.2-1 is used in NR cells.

UE:

-	The pre-configured UE location is defined in TS 38.508-1 [4] Clause 4.5C.

Preamble:

-	The UE is in state 3N-A on NR Cell 1 (serving cell) according to TS 38.508-1 [4] Table 4.4A.2-3.

6.1.2.18b.3.2	Test procedure sequence

Table 6.1.2.18b.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the NR cells at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause. Configurations marked "T1" and "T2" are applied at the points indicated in the Main behaviour description in Table 6.1.2.18b.3.2-2.

Table 6.1.2.18b.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | NR Cell 1 | NR Cell 11 | NR Cell 12 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -88 | "Off" | "Off" | The power level values are assigned to ensure the UE registered on NR Cell 1 and Srxlev of NR Cell 1 is greater than Sintrasearch. |
|  | SIntraSearch | dB | 10 | - | - |  |
|  | Srxlev | dB | 22 | - | - |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -94 | -88 | "Off" | NR Cell 11 becomes the highest ranked cell. |
|  | SIntraSearch | dB | 10 | 10 | - |  |
|  | Srxlev | dB | 16 | 22 | - |  |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | "Off" | -94 | -88 | NR Cell 12 becomes the highest ranked cell. |
|  | SnonIntraSearch | dB | - | 10 | 10 |  |
|  | Srxlev | dB | - | 16 | 22 |  |
| Note: Power level “Off” is defined in TS 38.508-1 [4] Table 6.2.2.1-3. |  |  |  |  |  |  |

Table 6.1.2.18b.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a CONFIGURATION UPDATE COMMAND message including Local time zone, Universal time and local time zone, Network daylight saving time information. (Note 1) | <-- | CONFIGURATION UPDATE COMMAND | - | - |
| 2 | The SS transmits an RRCRelease message. | <-- | NR RRC: RRCRelease | - | - |
| 3 | The SS re-adjusts the SS/PBCH EPRE levels according to row "T1" in table 6.1.2.18a.3.2-1. | - | - | - | - |
| 4 | Check: Does the test result of generic test procedure in TS 38.508-1 [4] Table 4.9.5.2.2-1 indicate that the UE is camped on NR Cell 11 within 6mins after step 1? NOTE: The UE performs a " REGISTRATION REQUEST procedure with type "mobility registration updating” with PLMN1 and PLMN2 listed as an Equivalent PLMN in the REGISTRATION ACCEPT message; the RRC connection is released. | - | - | 1 | - |
| 5 | The SS re-adjusts the SS/PBCH EPRE levels according to row “T2” in table 6.1.2.18a.3.2-1. | - | - | - | - |
| 6 | Check: Does the test result of generic test procedure in TS 38.508-1 [4] Table 4.9.5.2.2-1 indicate that the UE is camped on NR Cell 12 within 9mins after step 1? NOTE: The UE performs a " REGISTRATION REQUEST procedure with type "mobility registration updating” with PLMN1 and PLMN2 listed as an Equivalent PLMN in the REGISTRATION ACCEPT message; the RRC connection is released. | - | - | 2 | - |
| Note 1: SS derives Local time zone, Universal time and local time zone, Network daylight saving time based on the time of test execution. |  |  |  |  |  |

6.1.2.18b.3.3	Specific message contents

Table 6.1.2.18b.3.3-1: Conditions for specific message contents in Tables below

| Condition | Explanation |
| --- | --- |
| NR Cell 1 | This condition applies to system information transmitted on NR Cell 1. |
| NR Cell 11 | This condition applies to system information transmitted on NR Cell 11. |
| NR Cell 12 | This condition applies to system information transmitted on NR Cell 12. |

Table 6.1.2.18b.3.3-2: SIB2 of NR Cell 1, NR Cell 11 and NR Cell 12 (preamble and all steps, Table 6.1.2.18b.3.2-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB2 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| s-NonIntraSearch | 5 | 10 dB |  |
| threshServingLowP | 10 | 20 dB | NR Cell 11 |
| cellReselectionPriority | 5 |  | NR Cell 1 and NR Cell 11 |
|  | 1 |  | NR Cell 12 |
| } |  |  |  |
| intraFreqCellReselectionInfo SEQUENCE { |  |  |  |
| s-IntraSearchP | 5 | 10 dB |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.18b.3.3-3: SIB4 of NR Cell 11 and NR Cell 12 (preamble and all steps, Table 6.1.2.18b.3.2-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB4 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo { | 1 entry |  |  |
| InterFreqCarrierFreqInfo[1] SEQUENCE { |  | entry 1 |  |
| dl-CarrierFreq | Same downlink ARFCN as used for NR Cell 12 |  | NR Cell 11 |
|  | Same downlink ARFCN as used for NR Cell 1 |  | NR Cell 12 |
| cellReselectionPriority | 5 |  | NR Cell 12 |
|  | 1 |  | NR Cell 11 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.18b.3.3-4: SIB19 of NR Cell 1 and NR Cell 11 (preamble and all steps, Table 6.1.2.18b.3.2-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.2-18C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| t-service-r17 | Time as applied in Universal time and local time zone IE at step 1 of Table 6.1.2.18b.3.2-2 + 6mins |  | NR Cell 1 |
|  | Time as applied in Universal time and local time zone IE at step 1 of Table 6.1.2.18b.3.2-2 + 9mins |  | NR Cell 11 |
| referenceLocation-r17 | '2273BA566355'H |  |  |
| distanceThresh-r17 | 20000 |  |  |
| } |  |  |  |
| Note 1: Value is calculated in multiples of 10 ms after 00:00:00 on Gregorian calendar date 1 January 1900. |  |  |  |

Table 6.1.2.18b.3.3-5: REGISTRATION ACCEPT (step 4 and step 6, Table 6.1.2.18b.3.2-2)

| Derivation path: TS 38.508-1 Table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Equivalent PLMNs | 002-11 | PLMN 2 | Step 4 |
|  | 001-01 | PLMN 1 | Step 6 |

<End of modified section>