3GPP TSG-RAN5 Meeting #104	R5-245291

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

Agenda item:	5.5.1.13

Source: 	Qualcomm Inc

Title: 	Simulation results to derive min test time for nb-ntn demod test cases

Document for:	Discussion and Endorsement

## 1.	Introduction

TS 36.521-4 [3] introduced NTN-TDL channel models and legacy TDL models for NPDSCH and NPDCCH Demodulation test cases with satellite access based on the requirements defined in [4]. This paper discusses the simulation results to derive the minimum test time for these NB-NTN test cases.

## 2.	Discussion

As per [5] it was agreed to follow similar approach as NR for determining the minimum test time. The procedure is outlined in clause G.3.5

Pasted below for reference.

- Run the PDSCH simulations for an extended period of time at the SNR of interest for multiple seeds.

- The minimum test time is determined by the time it took the throughput curve to settle within +/- 2% of the target throughput value across all seeds.

- If the throughput does not settle within the +/- 2% throughput limit within a reasonable amount of time for any seed, expand the threshold to +/- 3.5% of the target throughput value.

- To account for the additional 1.5% throughput variation, determine from the Throughput vs SNR curve, what is the SNR delta corresponding to this 1.5 % throughput increase.

- That SNR value is the added SNR uncertainty due to finite test time.

The NPDSCH simulations for below mentioned 2 sets of NB-NTN NPDSCH demod test cases in [3] were run.

Table 8.3.1.1.1.3-2: Minimum performance for NPDSCH under Standalone with 1 NRS port

| Test number | Bandwidth | Carrier Type | Reference Channel | Repetition number | Propagation condition | Number of NRS ports | Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 200kHz | Anchor | R.NB.1 FDD | 32 | NTN-TDLC5-200 | 1 | 1x1 | 70% | -4.7 | NB1, NB2 |
| 2 | 200kHz | Non-anchor | R.NB.2 FDD | 128 | NTN-TDLA100-10 | 1 | 1x1 | 70% | -10.6 | NB1, NB2 |

Table 8.3.1.1.2.3-2: Minimum performance under standalone

| Test number | Bandwidth | Carrier Type | Reference Channel | Repetition number | Propagation condition | Number of NRS ports | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 200kHz | Anchor | R.NB.6 FDD (Note 2) | 32 | EPA5 | 1 | 70% | -3.4 | NB1, NB2 |
| 2 (Note 1) | 200kHz | Non-anchor | R.NB.6-1 FDD (Note 2) | 256 | ETU1 | 1 | 70% | -10.2 | NB1, NB2 |
| Note 1: Applicable to UE supporting Non-Anchor mode of operation. Note 2: R.NB.6 FDD has the same parameters with R.NB.1 FDD in Table A.3.12.1.1-1. R.NB.6-1 FDD has the same parameters with R.NB.2 FDD in Table A.3.12.1.1-1. |  |  |  |  |  |  |  |  |  |

Figure1: NPDSCH Throughput Simulations for Test1 of TC 8.3.1.1.1.3-2 (NTN-TDLC5-200) at 70% target throughput SNR, 30 seeds

Figure2: NPDSCH Throughput Simulations for Test2 of TC 8.3.1.1.1.3-2 (NTN-TDLA100-10) at 70% target throughput SNR, 30 seeds

Figure3: NPDSCH Throughput Simulations for Test1 of TC 8.3.1.1.2.3-2 (EPA5) at 70% target throughput SNR, 30 seeds

Figure4: NPDSCH Throughput Simulations for Test2 of TC 8.3.1.1.2.3-2 (ETU1) at 70% target throughput SNR, 30 seeds

The simulation for the 4 scenarios took into account the minimum scheduling gap needed between consecutive NPDCCH/NPDSCH. What the simulation did not factor is the round trip delay time. The RTT time becomes a significant contributing factor for GSO where K-offset is specified as 258 slots.

In our opinion, for a conducted NB-NTN test setup there is no propagation delay between eNB emulating SAN and the UE. Hence the eNB can schedule NPDCCH/NPDSCH as soon as possible taking into account single HARQ process, NPDCCH/NPDSCH repetition number, NPDCCH/NPDSCH scheduling gap overhead due to subframes with NPBCH/NPSS/NSSS/NSib and Rmax*G value to align with NPDCCH search space.

Hence we propose TE to schedule NPDCCH/NPDSCH as soon possible without factoring the RTT delay in the NPDSCH and NPDCCH Demodulation test case.

Proposal 1: TE to schedule NPDCCH/NPDSCH without factoring the RTT delay.

Looking at the simulation results for 30 seeds for each scenario, summarizing the convergence time in table 2-1.

Table 2-1: Convergence time

| Scenario | Convergence time for +/- 3.5% of the peak Tput (seconds) |
| --- | --- |
| 8.3.1.1.1.3-2_Test1_NTN-TDLC5-200 | 460 |
| 8.3.1.1.1.3-2_Test2_NTN-TDLA100-10 | 400 |
| 8.3.1.1.2.3-2 _Test1_EPA5 | 300 |
| 8.3.1.1.2.3-2 _Test2_ETU1 | 620 |

With the justification that RTT does not need to be factored in the scheduling of NPDSCH/NPDCCH, the minimum test time derived as per our simulation results does not need to be scaled and is the true test time for the given scenario.

Proposal 2: Minimum test time in Annex G table to be updated based on table 2-1

## 3.	Conclusion

Proposal 1: TE to schedule NPDCCH/NPDSCH without factoring the RTT delay.

Proposal 2: Minimum test time in Annex G table to be updated based on table 2-1

## 4.		References

[1]	R5-191250, Discussion paper on statistical tput calculation for demod test cases, Qualcomm

[2]	R5-194687, Discussion on minimum test time for PDSCH and PDCCH with more simulation results, Qualcomm

[3]	TS 36.521-4 v18.2.0

[4]	TS 36.102 v18.6.0

[5]	R5-242427, Discussion on minimum test time simulation methods for NB-IoT NTN, Mediatek