# Title:	Test Tolerances for PDSCH CRS interference mitigation under NR-LTE coexistence scenario

Source:	Qualcomm

# 1.	Introduction

The purpose of this document is to present the methodology and principles used for the development of Test Tolerances for PDSCH CRS interference mitigation under NR-LTE coexistence scenario.

The test case in question, from 38.521-4, is:

- 5.2.2.1.18,		2Rx FDD FR1 for PDSCH CRS interference mitigation under NR-LTE coexistence scenario

This document describes the process to derive the Test Tolerances. The calculations are provided in the accompanying spreadsheet.

# 2. Test case in TS 38.101-4

The test conditions are defined in the following extracts from TS 38.101-4 v18.4.0.

##### 5.2.2.1.18	Minimum requirements for PDSCH CRS interference mitigation under NR-LTE coexistence scenario

The performance requirements are specified in Table 5.2.2.1.18.0-4, with the addition of test parameters in Tables 5.2.2.1.18.0-2 and 5.2.2.1.18.0-3 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.2.1.18.0-1.

Table 5.2.2.1.18.0-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify PDSCH CRS interference mitigation performance under 2 receive antenna conditions with CRS rate matching configured for the serving cell | 1-1 |

Table 5.2.2.1.18.0-2: Test parameters for the serving cell

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD |
| Active DL BWP index |  |  | 1 |
| NR UL transmission with a 7.5 kHz shift to the LTE raster |  |  | true |
| PDCCH configuration | Symbols with PDCCH |  | Symbol# 2 |  |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 3 |
|  | Length (L) |  | 9 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Position of the first DM-RS for downlink |  | 3 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| CRS for rate matching (Note 1) | LTE carrier centre subcarrier location |  | Same as NR carrier centre subcarrier location |
|  | LTE carrier BW | MHz | 10 |
|  | Number of antenna ports |  | 2 |
|  | v-shift |  | 0 |
| Number of HARQ Processes |  |  | 4 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2 |
| Note 1: No MBSFN is configured on LTE carrier. Note 2: Network-based CRS interference mitigation is disabled on LTE carrier. |  |  |  |

Table 5.2.2.1.18.0-3: Test parameters for the LTE interference cells

| Parameter |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- |
| Propagation conditions and MIMO configuration (Note 1) |  |  | TDLA30-10 ULA Low | TDLA30-10 ULA Low |
| INR (Note 2) |  | dB | 10.45 | 4.6 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | Antenna ports 0,1 |
| Carrier centre subcarrier location |  |  | Same as the serving carrier centre subcarrier location | Same as the serving carrier centre subcarrier location |
| BWChannel |  | MHz | 10 | 10 |
| Cyclic Prefix |  |  | Normal | Normal |
| Physical cell ID |  |  | 1 | 2 |
| Number of control OFDM symbols |  |  | 2 | 2 |
| PDSCH transmission mode |  |  | 4 | 4 |
| Interference model |  |  | As specified in clause B.7 | As specified in clause B.7 |
| Probability of occurrence of PDSCH data |  | % | 20 | 20 |
| Probability of occurrence of transmission rank | Rank 1 | % | 80 | 80 |
|  | Rank 2 | % | 20 | 20 |
| Downlink power allocation |  | dB | -3 | -3 |
|  |  | dB | -3 | -3 |
|  |  | dB | 0 | 0 |
| Precoding granularity |  | PRB | 6 | 6 |
| Time offset to the serving cell |  | us | 3 | -1 |
| Frequency offset to the serving cell |  | Hz | 300 | -100 |
| MBSFN |  |  | Not configured | Not configured |
| Network-based CRS interference mitigation |  |  | Disabled | Disabled |
| Note 1: The channel for the LTE interference cells and the serving cell are independent. Note 2: Defined in B.6.1. |  |  |  |  |

Table 5.2.2.1.18.0-4: Minimum performance for Rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-7.3 FDD | 10 / 15 | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 11.9 |

# 3. Discussion

The test consists of a single subtest:

- Subtest 1-1: Checks the PDSCH CRS interference mitigation performance on the NR serving cell in the presence of two LTE interfering cells with CRS rate matching configured on the NR serving cell.

From a Test Tolerance analysis perspective, subtest 1-1 is analyzed in the accompanying spreadsheet.

The Test system measures PDSCH performance, and the minimum performance requirement is expressed as the SNR required to achieve 70% of the maximum throughput obtainable from the specified Reference Measurement Channel. We note that the SNR is specified as  of the NR Cell in the spreadsheet.

To analyze the test case, it is helpful to visualize these as pie-charts showing the distribution of power, as below. Separate charts are given for all power within the channel bandwidth, and for the power within Noc’ (noise and interference only).

# 4. Calculation of Test Tolerances

General approach

The general approach is given in the steps below:

- Copy the originally specified key parameters from the core requirements

- Where relevant, calculate derived parameters from the core requirements

- Define uncertainties for a minimum set of parameters

- Define controlled parameters (critical to the test verdict), calculate sensitivity factors and uncertainty

- Determine which original or derived parameters to offset (apply Test Tolerances to) and by how much

- Recalculate original or derived parameters including Test Tolerances

- Check that the controlled parameters meet requirements to get the correct test verdict

Each step is explained below, and the calculations are given in the accompanying spreadsheet.

a) Original specified key parameters

The key parameters need to be copied from Tables 5.2.2.1.18-3 and 5.2.2.1.18-4 in TS 38.101-4, which are INR of the LTE interfering cells Cell 1 and Cell 2, SNR of the target NR cell. Since we need to compute the power contributions of each cell in linear domain, we need to factor Noc for this case. For ease of understanding, we use the term Es/Noc for each cell’s SNR. All the other parameters such as fractions of power and Es/Noc’ (SINR) are derived.

The key parameters appear in section a) of the accompanying spreadsheet.

b) Derived parameters

A number of derived parameters are calculated, using the base information in a). The reason for deriving each additional parameter is given in the “Comment” column of section b) in the accompanying spreadsheet.

Note that the original specified Es/Noc for each Cell is a parameter settable by the Test system, and therefore subject to uncertainty. By contrast, NR Cell Es/Noc’ is not directly settable by the test system, because it depends on several cell powers. Although Es/Noc’ is not specified in the test case, this SINR is critical for the test case.

c) Uncertainties

The SS provides 3 cells on the same frequency, and AWGN. We propose to control the following parameters:

- AWGN absolute power, Noc ±1.5 dB

- Ratio of NR cell signal / Fading, Ês1 / Noc ±0.8 dB

- Ratio of LTE Cell 1 signal / Fading, Ês2 / Noc ±0.8 dB

- Ratio of LTE Cell 2 signal / Fading, Ês3 / Noc ±0.8 dB

This choice forms a minimum set, so the superposition principle can be applied if necessary.

The absolute level of Noc is specified as ±1.5 dB as for other demodulation tests, and is not critical.

All Cells are faded, so the uncertainty for Ês/Noc needs to be widened from the usual default value of ±0.3dB. The approach taken here aligns with the values used for demodulation test cases in Annex F.1 of TS 38.521-4:

Overall system uncertainty for fading conditions comprises two quantities:

1. Signal-to-noise ratio uncertainty ±0.3dB

2. Fading profile power uncertainty ±0.7dB for 2Tx

Items 1 and 2 are assumed to be uncorrelated so can be root sum squared:

Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty2 + Fading profile power uncertainty2) = SQRT (0.32 + 0.72) = 0.76dB, round up to 0.8dB.

The effect of AWGN flatness and signal flatness is not included, because for wanted signal demodulation one of the critical parameters is Es/Noc’, which is SINR. As the largest proportion of the overall interference is the interfering cells, the demodulation is mainly determined by SIR. The signal and the interference generated by the test equipment are subject to the same flatness effect, so in the ratio signal-to-interference the flatness variation will cancel.

d) Controlled parameters critical to verdict

In this type of multi-cell demodulation test case, there is not a simple one-to-one relationship between the parameters that can be set by the test equipment, and their effect on parameters determining the test verdict.

It is therefore essential to identify those parameters determining the test verdict. In this test case the following parameters are critical:

- NR Cell Es/Noc'

- LTE Cell 1 Es/Noc (the ratio of Interferer to Noc)

- LTE Cell 2 Es/Noc (the ratio of Interferer to Noc)

The stimulus set by the Test system should be within these constraints for the UE measurement. The reason for each parameter being critical to the test verdict is given briefly in the “Comment” column of section d) in the accompanying spreadsheet. Information about the value to be achieved is given later in the “Comment” column of section g) in the spreadsheet.

Having identified the parameters critical to the test verdict which need to be controlled, we now need to consider how they are affected by the parameters which can be set by the test equipment. This is done by working out “sensitivity factors”. A sensitivity factor is just the ratio (effect on a critical parameter y / a test equipment uncertainty x) and is usually in dB/dB. Often it can be derived by inspection as one or zero. For example, an error of 1dB in the NR Cell Es/Noc would cause 1dB error in the NR Cell Es/Noc’, so the sensitivity factor is 1.000. However, an error of 1dB in the absolute AWGN level Noc would cause no change to NR Cell Es/Noc’ because this ratio is independent of the absolute AWGN value, so the sensitivity factor is zero.

In some cases, the sensitivity factor is an intermediate value. For example, the LTE Cell 1 Es/Noc has an effect on NR Cell Es/Noc’ which depends on ratios of the powers making up the total. In such cases a sensitivity factor value between 0 and 1 results. It is important to calculate these correctly to obtain the overall uncertainty.

For example:

- The effect of LTE Cell 1 Es/Noc uncertainty on NR Cell Es/Noc’ is x0.741

LTE Cell 1 forms (74.1%/(6.7%+74.1%+19.3%) of the total Noc’. A change in the power of LTE Cell 1 Es/Noc alone is diluted in the overall interference. These factors can also be derived intuitively, by looking at the Power in Noc’ pie chart in section 3.

Having filled in the matrix of sensitivity factors, the accompanying spreadsheet calculates the overall uncertainty for each controlled parameter, taking into account the uncertainties and sensitivity factors for each parameter that can be set by the test equipment. This process follows the superposition principle. More details and explanation can be found in section 4 of TR 38.903.

For the test system uncertainties, the normal procedure of combining uncorrelated uncertainties root-sum-square is followed.

e) Determine parameters to offset

We observe the test equipment uncertainties could take the UE outside the specified LTE Cell 1 Es/Noc and LTE Cell 2 Es/Noc values. An LTE Cell 1 Es/Noc or LTE Cell 2 Es/Noc value lower than that specified in the test makes the wanted signal harder to demodulate (lower Es/Noc means that the proportion of AWGN, which is not cancelled, becomes higher). LTE Cell 1 Es/Noc and LTE Cell 2 Es/Noc are therefore increased by an amount sufficient to achieve a minimum Es/Noc of at least the requirement value specified in section g).

We observe that increasing the LTE Cell 1 Es/Noc and LTE Cell 2 Es/Noc values has decreased the SINR (NR Cell Es/Noc’), and that the test equipment uncertainties could reduce it further. NR Cell Es/Noc is therefore increased by an amount sufficient to ensure that the minimum NR Cell Es/Noc’ value meets the original requirement. This change does not affect the LTE Cell 1 Es/Noc and LTE Cell 2 Es/Noc values, so if the steps are taken in the order given this part of the process is not iterative.

f) Parameters modified by Test Tolerances

Based on the process in e), the set of parameters in a) and b) is reproduced in section f) of the accompanying spreadsheet, but this time modified by the Test Tolerances (applied offsets).

LTE Cell 1 Es/Noc is increased by an amount sufficient to achieve the original LTE Cell 1 Es/Noc value. This offset is identical to the Es/Noc test system uncertainty 0.8dB.

LTE Cell 2 Es/Noc is increased by an amount sufficient to achieve the original LTE Cell 2 Es/Noc value. This offset is identical to the Es/Noc test system uncertainty 0.8dB.

NR Cell Es/Noc is increased by an amount sufficient to achieve the original NR Cell Es/Noc’ (SINR) derived in b). The actual offset is found empirically by observing the minimum/maximum parameter values in step g).

Re-derived parameters are calculated using the same methods as were used in step b).

g) Check controlled parameters Min/Max

Using a format similar to that in step d), the nominal value of each controlled parameter is recalculated, as at least some will have changed from the original due to the application of the Test Tolerances in step f).

The minimum and maximum values, due to variability from uncertainties, of controlled parameters is then calculated and compared against the requirements (NR Cell Es/Noc’, LTE Cell 1 Es/Noc and LTE Cell 2 Es/Noc). The critical requirement for each parameter is given briefly in the “Comment” column of section g) in the accompanying spreadsheet. The cases closest to limit (in these test cases, all limits are one-sided) are identified by turquoise cells in the spreadsheet. If all the stimulus requirements are met, then the chosen stimulus offsets are acceptable.

Given the uncertainty values and Test Tolerances proposed above, the stimulus requirements are met.

# 5. Treatment of other similar test cases

Similar test tolerances have been computed for other PDSCH tests with CRS interference mitigation under NR-LTE coexistence in the attached spreadsheet. These are 5.2.2.2.19, 5.2.3.1.17 and 5.2.3.2.18.