# Title:	Test Tolerances for NR SA FR2 Configured Grant based Small Data Transmissions (CG-SDT)

Source:	Huawei, HiSilicon, Qualcomm

# 1.	Introduction

The purpose of this document is to present the methodology and principles used for the development of Test Tolerances for 2Rx NR SA FR2 Configured Grant based Small Data Transmissions (CG-SDT).

The test cases in question, from 38.533, are:

- 7.2.1.1,		NR SA FR2 TA Validation for CG-SDT

- 17.2.1.1,		NR SA FR2 TA validation for CG-SDT for 2 Rx

This document describes the process to derive the Test Tolerances. The calculations are provided in the accompanying spreadsheet.

# 2. Prototype test case in Annex A of TS 38.133

The test conditions are defined in the following extracts from TS 38.133 v17.15.0 Annex A:

#### A.7.2.1.1	TA validation for CG-SDT in FR2

##### A.7.2.1.1.1	Test Purpose and Environment

The purpose of this test is to partly verify that the UE properly perform TA validation for CG-SDT transmission in clause 5.5.3. The test includes two sub-tests, Sub-test#1 for testing valid TA where UE can initiate CG-SDT transmission, and Sub-test#2 for testing invalid TA where UE does not initiate CG-SDT transmission. Subtest#2 is only tested if Sub-test#1 is passed. For each sub-test, UE is configured with CG-SDT configurations when entering RRC Inactive state. Sub-test#1 consists of four successive time periods, with time duration of T1, T2, T3 and T4 respectively. Sub-test#2 consists of two successive time periods, with time duration of T5 and T6 respectively. There is one cell, which is the active NR cell in FR2. Figure A.7.2.1.1.1-1 shows the variation of the RSRP over the duration of Sub-test#1, and Figure A.7.2.1.1.1-2 shows the variation of the RSRP over the duration of Sub-test#2.

In Sub-test#1:

-	Prior to the time point TA, the UE shall be fully synchronized to PCell (Cell 1), be registered to the cell and have entered RRC connected mode.

-	Before starting the test at time point TA, test equipment configures RSRP to P0. At time point TB, RSRP is changed from P0 to P1.

-	At time point TC which is W1 after time point TB, UE expect to receive RRC release with CG SDT configuration and RRC status is changed to INACTIVE status.

-	At time point TD, RSRP is changed from P1 to P0.

-	At time point TE, RSRP is changed from P0 to P1. TE must be W2 before TF.

-	Test equipment triggers UL data arrival at UE lower layer at time point TF. After time point TF, test equipment observes whether UE transmits with CG-SDT no later than TG which is W3 after TF.

-	After time point TG, RRC status is changed from RRC INACTIVE to RRC CONNECTED.

In Sub-test#2:

-	Prior to the time point TA, the UE shall pass Sub-test#1 and have entered RRC connected mode. Otherwise, Sub-test#2 shall not be executed.

-	From time point TA to time point TD, RSRP is set to P1.

-	At time point TC, which is W1 after time point TB, UE expect to receive RRC release with CG SDT configuration and RRC status is changed to INACTIVE status.

-	At time point TD, RSRP is changed from P1 to P2.

-	Test equipment triggers UL data arrival at UE lower layer at time point TF. TF is 3360ms after TD. After time point TF, test equipment observes whether UE transmits with CG-SDT no later than TG which is W3 after TF.

W1 equals to 480ms and W2 equals to 480ms based on requirements in clause 5.5.3. W3 is 1060ms.

Supported test configurations are shown in table A.7.2.1.1.1-1. The test parameters are given in Tables A.7.2.1.1.1-2 and A.7.2.1.1.1-3.

Table A.7.2.1.1.1-1: Supported test configurations for FR2 PCell

| Configuration | Description |
| --- | --- |
| 1 | TDD, SSB SCS 120 KHz, data SCS 120KHz, BW 100 MHz |

Table A.7.2.1.1.1-2: General test parameters for TA validation for CG-SDT in FR2

| Parameter |  | Unit | Value | Value |
| --- | --- | --- | --- | --- |
|  |  |  | SubTest 1 | Subtest 2 |
| Active PCell |  |  | Cell 1 | Cell 1 |
| RF Channel Number |  |  | 1 | 1 |
| Duplex mode | Config 1 |  | TDD | TDD |
| BWchannel | Config 1 | MHz | 100: NRB,c = 66 | 100: NRB,c = 66 |
| Data RBs allocated | Config 1 |  | 6 | 6 |
| DL initial BWP configuration | Config 1 |  | DLBWP.0.1 | DLBWP.0.1 |
| UL initial BWP configuration | Config 1 |  | ULBWP.0.1 | ULBWP.0.1 |
| TDD Configuration | Config 1 |  | TDDConf.3.1 | TDDConf.3.1 |
| RMSI CORESET Reference Channel | Config 1 |  | CR.3.1 DD | CR.3.1 DD |
| SSB Configuration | Config 1 |  | SSB.3 FR2 | SSB.3 FR2 |
| SMTC Configuration | Config 1 |  | SMTC.1 | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing | Config 1 | kHz | 120 | 120 |
| PRACH Configuration | Config 1 |  | Table A.3.8.3.4 | Table A.3.8.3.4 |
| OCNG parameters |  |  | OP.5 | OP.5 |
| CP length |  |  | Normal | Normal |
| Correlation Matrix and Antenna Configuration |  |  | 2x2 Low | 2x2 Low |
| DRX |  | s | 0.64 | 0.64 |
| cg-SDT-RSRP-ThresholdSSB |  | dBm | -113 | -113 |
| cg-SDT-RSRP-ChangeThreshold |  | dB | 14 | 4 |
| cg-SDT-TimeAlignmentTime |  |  | infinity | infinity |
| CG-SDT resource period |  | ms | 320 | 320 |
| T1 |  | s | 0.8 | NA |
| T2 |  | s | 0.96 | NA |
| T3 |  | s | 3.04 | NA |
| T4 |  | s | 1.54 | NA |
| T5 |  | s | NA | 1.76 |
| T6 |  | s | NA | 4.58 |

Table A.7.2.1.1.1-3: Cell specific test parameters TA validation for CG-SDT in FR2

| Parameter |  | Unit | Test 1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 | T6 |
| AoA setup |  |  | Setup 1 defined in A.3.15 |  |  |  |  |  |
| Assumption for UE beams Note 4 |  |  | Rough |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 4 |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB | 0 |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB | 0 |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |  |
|  | Config 1 | dBm/15kHz | -104 | -104 | -104 | -104 | -107 | -107 |
|  | Config 1 | dBm/SCS | -95 | -95 | -95 | -95 | -98 | -98 |
|  | Config 1 | dB | 0 | 14 | 0 | 14 | 18.25 | 0.25 |
|  | Config 1 | dB | 0 | 14 | 0 | 14 | 18.25 | 0.25 |
| SS-RSRP | Config 1 | dBm/SCS | -95 | -81 | -95 | -81 | -79.75 | -97.75 |
| Io | Config 1 | dBm/95.04 MHz | -68.16 | -57.00 | -68.16 | -57.00 | -55.85 | -71.03 |
| Propagation condition |  |  | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in B.2.1.3 and does not limit UE implementation or test system implementation. |  |  |  |  |  |  |  |  |

Figure A.7.2.1.1.1-1: RSRP variation for TA validation for CG-SDT Sub-test#1

Figure A.7.2.1.1.1-2: RSRP variation for TA validation for CG-SDT Sub-test#2

##### A.7.2.1.1.2	Test Requirements

The UE behaviour in each test during time durations shall be as follows:

During Sub-test#1, UE shall transmit UL data with CG-SDT within 1060ms after time point TF.

During Sub-test#2, after passing Sub-test#1, UE shall not transmit UL data with CG-SDT.

# The rate of correct events observed during repeated tests shall be at least 90%.3. Discussion

The test consists in two subtests, the first one comprising of 4 time periods (T1, T2, T3 and T4), and the second one comprising of 2 time periods (T5, and T6). Each of the two subtests performs a different check:

- Subtest 1: checks that the UE correctly initiates CG-SDT operation when the SS-RSRP is above threshold (positive test)

- Subtest 2: checks that the UE correctly does not initiate CG-SDT operation when the SS-RSRP is below threshold (negative test)

The execution of subtest 2 is conditioned to the verdict on subtest 1: subtest 2 is only performed if subtest 1 passed.

From a Test Tolerance analysis perspective, each subtest can be analyzed independently – when the UE is performing one of the two checks described above, only the test conditions and parameters for that particular subtest apply. It is helpful to show the SS-RSRP variation, the effect of the measurement uncertainty (MU) as well as the criteria the UE needs to satisfy –or not satisfy, respectively– on each of the subtests:

Figure 3-1: SS-RSRP variation for subtest 1 considering the effect of MU

Figure 3-2: SS-RSRP variation for subtest 2 considering the effect of MU

# 4. Calculation of Test Tolerances

General approach

The general approach is given in the steps below:

- Copy the originally specified key parameters from the core requirements

- Where relevant, calculate derived parameters from the core requirements

- Define uncertainties for a minimum set of parameters

- Define controlled parameters (critical to the test verdict), calculate sensitivity factors and uncertainty

- Determine which original or derived parameters to offset (apply Test Tolerances to) and by how much

- Recalculate original or derived parameters including Test Tolerances

- Check that the controlled parameters meet requirements to get the correct test verdict

Each step is explained below, and the calculations are given in the accompanying spreadsheet.

a) Original specified key parameters

The key parameters need to be copied from Tables A.7.2.1.1.2-1, A.7.2.1.1.2-2 and A.7.2.1.1.2-3 in TS 38.133. The key parameters are selected as the minimum set to define the cell power levels, and which are subject to a test system uncertainty which may affect the verdict of the test. The signalled parameters cg-SDT-RSRP-ChangeThreshold and cg-SDT-RSRP-ThresholdSSB are also copied.

b) Derived parameters

A number of derived parameters are calculated, using the base information in a). The reason for deriving each additional parameter is given in the “Comment” column of section b) in the accompanying spreadsheet.

In subtest 1, during T4, the UE shall measure the SS-RSRP level of SSB#0 and compare it with threshold cg-SDT-RSRP-ThresholdSSB. Furthermore, the UE shall measure the SS-RSRP level of SSB#0 and compare it with the stored SS-RSRP level of SSB#0 during T2, when the CG-SDT configuration was received, and with the signaled thresholds. One of the criteria to be satisfied is that the SS-RSRP variation between these two time instances is less than cg-SDT-RSRP-ChangeThreshold. Therefore, the following derived parameter is calculated:

- |RSRP(T4) – RSRP(T2)|

Which needs to be less than cg-SDT-RSRP-ChangeThreshold in order to be satisfied. As depicted in section 3, Figure 1, the combined effect of measurement uncertainties will affect the ability of the UE to perform accurate RSRP measurements, which, when compared against this static threshold need to still meet the requirements.

Similarly, in subtest 2, during T6, the UE shall measure the SS-RSRP level of SSB#0 and compare it with threshold cg-SDT-RSRP-ThresholdSSB, furthermore, the UE shall measure the SS-RSRP level of SSB#0 and compare it with the stored SS-RSRP level of SSB#0 during T5, when the CG-SDT configuration was received, and with the signaled thresholds. One of the criteria to be satisfied is that the SS-RSRP variation between these two time instances is greater than cg-SDT-RSRP-ChangeThreshold. Therefore, the following derived parameter is calculated:

- |RSRP(T6) – RSRP(T5)|

Which needs to be greater than cg-SDT-RSRP-ChangeThreshold in order to be not satisfied. As depicted in section 3, Figure 2, the combined effect of measurement uncertainties will affect the ability of the UE to perform accurate RSRP measurements, which, when compared against this static threshold need to still meet the requirements.

c) Uncertainties

The SS provides 1 NR Cell without fading to the UE. We propose to control the following parameters:

- Frequency 1 AWGN Absolute power, Noc ±5.65 dB averaged over BWConfig

- Frequency 1 AWGN Absolute power, Noc ±5.65 dB for PRBs #RBJ-RBJ+19

- Ratio of NR Cell 1 signal / noise, Ês1 / Noc ±0.3 dB averaged over BWConfig

- Ratio of NR Cell 1 signal / noise, Ês1 / Noc ±0.3 dB for PRBs #RBJ-RBJ+19

In this test the UE measures the SS-RSRP over specific Physical Resource Block (PRB) numbers # RBJ to # RBJ+19. In addition, this test has separate constraints on the overall power in the configured bandwidth Io.

This choice forms a minimum set, so the superposition principle can be applied if necessary.

In addition, the UE SS-RSRP measurement accuracy need to be considered. Since UE SS-RSRP measurements are compared with a signaled threshold, any potential inaccuracy could affect the test verdict. The comparasion consists of two aspects:

- First, the UE shall compare the RSRP in T4 & T6 with an absolute RSRP threshlold cg-SDT-RSRP-ThresholdSSB. So intra-frequency RSRP absolute accuracy is needed to be considered;

- Second, the UE shall compare the relative RSRP |RSRP(T4) - RSRP(T2)| and |RSRP(T6) - RSRP(T5)| with differential RSRP threshold. So intra-frequency RSRP relative accuracy is needed to be considered.

However, TS 38.133 does not specify any absolute or relative accuracy requirements for SDT triggering. It has been agreed at RAN5#103 to use the L1-RSRP measurement accuracy requirements as a valid proxy for UE’s measurement accuracy uncertainty. The rationale is that L1-RSRP measurements are unfiltered and are performed on a per SSB basis. Therefore:

- When comparing RSRP with cg-SDT-RSRP-ThresholdSSB in T4 and T6, a measurement uncertainty of 8.5dB (which is the L1-RSRP absolute accuracy for FR2 when Io value is larger than -70dBm/Ch BW) shall be used.

- When comparing differential RSRP with cg-SDT-RSRP-ChangeThreshold, a measurement uncertainty of 6.5dB (which is the L1-RSRP relative accuracy for FR2) shall be used.

- In addition, the side conditions for L1-RSRP accuracy requirements need to be satisfied.

##### 10.1.20.1.1	Absolute Accuracy

The accuracy requirements in Table 10.1.20.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for L1-RSRP measurements are fulfilled according to Annex B.2.4.1 for a corresponding Band for each relevant SSB.

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

Table 10.1.20.1.1-1: SSB based L1-RSRP absolute accuracy in FR2

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |
|  |  |  | Minimum Io |  |  | Maximum Io |
| dB | dB | dB | dBm / SCSSSB Note 2 |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  | SCSSSB = 120kHz | SCSSSB = 240kHz |  |  |
| ±6.5 | ±9.5 | ≥-3 | Same value as SSB_RP in Table B.2.4.1-2, according to UE Power class, operating band and angle of arrival |  | N/A | -70 |
| 8.5 | 11.5 | ≥-3 | N/A |  | -70 | -50 |
| NOTE 1: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. NOTE 2: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. NOTE 3: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |  |

##### 10.1.20.1.2	Relative Accuracy

The relative accuracy of SSB based L1-RSRP is defined as the L1-RSRP measured from one SSB compared to the largest measured value of L1-RSRP among all SSBs of the cell (serving cell or cell with different PCI from serving cell) on which UE performs L1-RSRP measurements.

The accuracy requirements in Table 10.1.20.1.2-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for L1-RSRP measurements are fulfilled according to Annex B.2.4.1 for a corresponding Band for each relevant SSB.

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

Table 10.1.20.1.2-1: SSB based L1-RSRP relative accuracy in FR2

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |
|  |  |  | Minimum Io |  | Maximum Io |
| dB | dB | dB | dBm / SCSSSB Note 3 |  | dBm/BWChannel |
|  |  |  | SCSSSB = 120kHz | SCSSSB = 240kHz |  |
| 6.5 | 9.5 | ≥-3 | Same value as SSB_RP in Table B.2.4.1-2, according to UE Power class, operating band and angle of arrival |  | -50 |
| NOTE 1: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter SSB Ês/Iot is the minimum SSB Ês/Iot of the pair of SSBs to which the requirement applies. NOTE 3: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. NOTE 4: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |

To apply accuracy requirements, the conditions in TS 38.133 clause B.2.4.1 need to be satisfied:

### B.2.4.1	Conditions for SSB based L1-RSRP reporting

This clause defines the following conditions for NR L1-RSRP measurement reporting and corresponding procedures performed based on SSBs: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.4.1-1 for FR1 NR cells.

The conditions are defined in Table B.2.4.1-2 for FR2 NR cells.

Table B.2.4.1-2: Conditions for SSB based L1-RSRP measurements in FR2

| Parameter | Angle of arrival | NR operating bands |  |  | Minimum SSB_RP Note 2, Note 3 |  |  |  |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | dBm / SCSSSB |  |  |  |  | dB |
|  |  |  | SCSSSB = 120 kHz |  |  |  |  |  | SCSSSB = 240 kHz |  |
|  |  |  | UE Power class |  |  |  |  |  | UE Power class |  |
|  |  |  | 1 | 2 | 3 | 4 | 5 | 6 | 1, 2, 3, 4, 5, 6 |  |
| Conditions | Rx Beam Peak | n257 | -125.3+Y1 | -110.8 | -109.1 | -124.8+Y4 | -120.4+Y5 | -120.4+ Y6 | (Value for SCSSSB = 120 kHz) +3dB | ≥-3 |
|  |  | n258 | -125.3+Y1 | -110.8 | -109.1 | -124.8+Y4 | -120.6+Y5 | -120.6+ Y6 |  |  |
|  |  | n259 |  |  | -105.5 |  | -117.5+Y5 |  |  |  |
|  |  | n260 | -122.3+Y1 |  | -106.5 | -122.8+Y4 |  |  |  |  |
|  |  | n261 | -125.3+Y1 | -110.8 | -109.1 | -124.8+Y4 |  | -120.4+ Y6 |  |  |
|  |  | n262 | -120.3+Y1 | -105.6 | -103.6 | -118.8+Y4 |  |  |  |  |
|  | Spherical coverage Note 1 | n257 | -117.3+Z1 | -99.8 | -98.2 | -115.8+Z4 | -112.4+Z5 | -112.4+Z6 | (Value for SCSSSB = 120 kHz) +3dB | ≥-3 |
|  |  | n258 | -117.3+Z1 | -99.8 | -98.2 | -115.8+Z4 | -112.6+Z5 | -112.6+Z6 |  |  |
|  |  | n259 |  |  | -92.7 |  |  |  |  |  |
|  |  | n260 | -114.3+Z1 |  | -93.9 | -110.8+Z4 |  |  |  |  |
|  |  | n261 | -117.3+Z1 | -99.8 | -98.2 | -115.8+Z4 |  | -112.4+Z6 |  |  |
|  |  | n262 | -112.3+Z1 | -93.7 | -90.5 | -106.7+Z4 |  |  |  |  |
| NOTE 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. NOTE 2: Values specified at the Reference point to give minimum SSB Ês/Iot, with no applied noise. NOTE 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ∆MBP,n and Spherical coverage values are increased by ∆MBS,n, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19]. |  |  |  |  |  |  |  |  |  |  |

For band with frequency < 40.6GHz, band n259 has the tightest SSB_RP side conditions.

In summary, the side conditions to be satisfied are:

- SSB Es/Iot ≥ -3dB

- SSB_RP ≥ -105.5dBm/120kHz

- Io ≥ 105.5dBm/120kHz and Io ≤ -50dBm/Ch BW

And UE gain variation is defined in 38.133 Annex B.2.1.5:

#### B.2.1.5.1	Gain to SS-RSRP measurement point for Rx Beam Peak angle of arrival

In clause 5.1.1 of TS 38.215 [4] SS-RSRP is defined to be measured based on the combined signal from antenna elements corresponding to a given receiver branch. The reference point for requirement parameters from the UE perspective is the input of the UE antenna array. The gain “G” relates the combined signal from antenna elements corresponding to a given receiver branch to the reference point for requirement parameters.

The gain “G” affects absolute signal level values reported by the UE.

Figure B.2.1.5.1-1: Gain and Reference point for requirement parameters

The gain range for each power class is specified in Table B.2.1.5.1-1.

Table B.2.1.5.1-1: UE gain G, Rx beam peak direction

|  | UE Power class |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 |
| Minimum, dBi | FFS | FFS | -10 | FFS |
| Maximum, dBi | FFS | FFS | +20 | FFS |

Gain range in spherical coverage directions may be lower than in Rx beam peak direction, according to the difference between the EIS spherical coverage value specified in TS 38.101-2 [19] clause 7.3.4 and the Reference sensitivity level specified in TS 38.101-2 [19] clause 7.3.2.

The UE gain variation can be expressed as (maximum UE gain - minimum UE gain)/2 according to 38.903 Annex A. For UE PC3, it's 15dB.

The spherical coverage gain variation is expressed as (UE REFSENS - UE Spherical coverage)/2 according to 38.903 Annex A. where UE REFSENS is specifed in 38.101-2 Table 7.3.2.3-1. For UE PC3 with 100MHz CBW, spherical coverage gain variation is 6.4dB. In this test case Rx peak AoA is selected so spherical coverage gain variation doesn’t need to be considered as specified in 38.903 Annex A. This value has been greyed out in spreadsheet for better readability.

d) Controlled parameters critical to verdict

The controlled parameters listed in the accompanying spreadsheet have been derived by study of the test case and the relevant clauses in TS 38.133.

Having identified the parameters which need to be controlled, we now need to consider how they are affected by the parameters which can be set by the test equipment. This is done by working out “sensitivity factors”. A sensitivity factor is just the ratio (effect on a critical parameter y / a test equipment uncertainty x), and is usually in dB/dB. Often it can be derived by inspection as one or zero. For example, an error of 1dB in the absolute AWGN level Noc would cause 1dB error in the SS-RSRP, so the sensitivity factor is 1.000 during all time intervals. Similarly, an error of 1dB in the Es/Noc would cause 1dB error in the SS-RSRP, so the sensitive factor is also 1.000 during all time intervals.

Having filled in the matrix of sensitivity factors, the accompanying spreadsheet calculates the overall uncertainty for each controlled parameter, considering the uncertainties and sensitivity factors for each parameter that can be set by the test equipment, and the UE measurement accuracies where relevant. This process follows the superposition principle. Uncertainties are calculated separately for each subtest and for each time interval.

For the test system uncertainties, the normal procedure of combining uncorrelated uncertainties root-sum-square is followed.

When the test system uncertainties and the UE reporting accuracies are combined, the (root-sum square of test system uncertainties) is added arithmetically to the UE reporting accuracies. If all the uncertainties were combined root-sum square, the resulting smaller test limits could case a conformant test system to fail a conformant UE, which would be unacceptable.

e) Determine parameters to offset

When taking all measurement uncertainties into consideration, and as depicted in Figure 3-1 and Figure 3-2, for subtest 1 and 2 correspondingly, the following analysis applies:

Subtest 1:

Following inequality can be derived:

RSRPT4-MU-(RSRPT2+MU)≤Threshold

Which can be reduced to:

RSRPT4-RSRPT2-2*MU≤Threshold

With numerical values:

-81-(-81)-2*6.92≤14dB

13.84dB≤14dB

Therefore the inequality can be satisfied with the nominal values and no additional offsets are required.

Subtest 2:

Following inequality can be derived:

RSRPT6+MU-(RSRPT5-MU)≥Threshold

Which can be reduced to:

RSRPT6-RSRPT5-2*MU≥Threshold

With numerical values:

-79.75-(-97.75)-2*6.92≥4dB

4.16dB≥4dB

Therefore the inequality can be satisfied with the nominal values and no additional offsets are required.

As verified in section g) of the analysis spreadsheet, with the parameters supplied in 38.133 there is no need to add any additional offset to meet the requirements, when considering the test uncertainties and measurement accuracies described in the previous sections.

f) Parameters modified by Test Tolerances

As stated in section e), the set of parameters in a) and b) is reproduced in section f) of the accompanying spreadsheet, and no offsets are required to meet the requirements, when considering the test uncertainties and measurement accuracies described in the previous sections. This has been empirically verified in section g).

g) Check controlled parameters Min/Max

It can be seen that even when considering the uncertainty values, all the requirements are met:

- SSB#0 Es/Iot minimum ≥ -3dB

- SSB#0 SSB_RP minimum ≥ -105.5dBm/SCS

- -50dBm/CBW ≥ Io of q0 minimum ≥ -81.70dBm/CBW = -105.5dBm/SCS

- UE measured SSB#0 SS-RSRP minimum ≥ -113dBm = cg-SDT-RSRP-ThresholdSSB

- UE measured |RSRP(T4) – RSRP(T2)| maximum < 14dB = cg-SDT-RSRP-ChangeThreshold

- UE measured |RSRP(T6) – RSRP(T5)| minimum > 4dB = cg-SDT-RSRP-ChangeThreshold

# 5. Treatment of similar test cases

There is also an equivalent RedCap FR2 SDT test case in 38.533, 17.2.1.1. Considering the test parameters, power levels, and test requirements in these two test cases are exactly the same, the test tolerlance analysis presented in this document for 7.2.1.1 can also be extended to 17.2.1.1.