3GPP TSG-RAN5 Meeting #104	R5-245436

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4653 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrrection to NR5GC CAG test case 6.5.2.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NG_RAN_PRN_Vertical_LAN-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The support of CAG is indicated by the UE during the registration procedure which is currently being checked in TTCN. This is not captured in the specific message contents as it is not as per the default values specified in TS 38.508-1, Table 4.7.1-6 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added a specific message content Table 6.5.2.6.3.3-1A to specify it |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification remains incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | No TTCN impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 6.5.2.6	CAG / cell reservation

6.5.2.6.1	Test Purpose (TP)

(1)

with { UE in Automatic network selection mode and provisioned with "CAG information list" }

ensure that {

when { UE is switched on or returns to coverage, and a cell which is not a CAG cell with cell status “true” for other use becomes available }

then { UE does not attempt to reselect to the higher ranked cell }

}

(2)

with { UE camped normally in state NR RRC_IDLE on a CAG cell and UE’s Allowed CAG list is not empty }

ensure that {

when { a higher ranked CAG cell is found with cell status as “not barred” and “not reserved” for operator use and “true” for other use, and not “true” for future use }

then { UE does reselect to the higher ranked cell }

}

6.5.2.6.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.304: clauses 5.2.4.4, 5.2.4.6 and 5.3.1. Unless otherwise stated these are Rel-16 requirements.

[TS 38.304, clause 5.2.4.4]

For the highest ranked cell (including serving cell) according to cell reselection criteria specified in clause 5.2.4.6, for the best cell according to absolute priority reselection criteria specified in clause 5.2.4.5, the UE shall check if the access is restricted according to the rules in clause 5.3.1.

If that cell and other cells have to be excluded from the candidate list, as stated in clause 5.3.1, the UE shall not consider these as candidates for cell reselection. This limitation shall be removed when the highest ranked cell changes.

If the highest ranked cell or best cell according to absolute priority reselection rules is an intra-frequency or inter-frequency cell which is not suitable due to one or more of the following reasons:

-	this cell belongs to a PLMN which is not indicated as being equivalent to the registered PLMN, or

-	this cell is a CAG cell that belongs to a PLMN which is equivalent to the registered PLMN but with no CAG-ID that is present in the UE's allowed CAG list being broadcasted, or

-	this cell is not a CAG cell and the CAG-only indication in the UE is set, or

-	this cell does not belong to a SNPN that is equal to the registered or selected SNPN of the UE in SNPN access mode,

the UE shall not consider this cell and, for operation in licensed spectrum, other cells on the same frequency as candidates for reselection for a maximum of 300 seconds.

For operation with shared spectrum channel access, when the highest ranked cell or best cell is not a candidate for reselection per the previous paragraph, the UE should continue to consider other cells on the same frequency for cell reselection, however if the second highest ranked cell on this frequency is also not suitable due to one or more of the above reasons, the UE may consider this frequency to be the lowest priority for a maximum of 300 seconds.

If the highest ranked cell or best cell according to absolute priority reselection rules is an intra-frequency or inter-frequency cell which is not suitable due to being part of the "list of 5GS forbidden TAs for roaming", the UE shall not consider this cell and other cells on the same frequency as candidates for reselection for a maximum of 300 seconds.

If the highest ranked cell or best cell according to absolute priority reselection rules is an inter-RAT cell which is not suitable due to being part of the "list of forbidden TAs for roaming" or belonging to a PLMN which is not indicated as being equivalent to the registered PLMN, the UE shall not consider this cell and other cells on the same frequency, as candidates for reselection for a maximum of 300 seconds.

If the UE enters into state any cell selection, any limitation shall be removed. If the UE is redirected under NR control to a frequency for which the timer is running, the limitation(s) on that frequency shall be removed.

[TS 38.304, clause 5.2.4.6]

The cell-ranking criterion Rs for serving cell and Rn for neighbouring cells is defined by:

Rs = Qmeas,s +Qhyst - Qoffsettemp Rn = Qmeas,n -Qoffset - Qoffsettemp

where:

| Qmeas | RSRP measurement quantity used in cell reselections. |
| --- | --- |
| Qoffset | For intra-frequency: Equals to Qoffsets,n, if Qoffsets,n is valid, otherwise this equals to zero. For inter-frequency: Equals to Qoffsets,n plus Qoffsetfrequency, if Qoffsets,n is valid, otherwise this equals to Qoffsetfrequency. |
| Qoffsettemp | Offset temporarily applied to a cell as specified in TS 38.331 [3]. |

The UE shall perform ranking of all cells that fulfil the cell selection criterion S, which is defined in 5.2.3.2.

The cells shall be ranked according to the R criteria specified above by deriving Qmeas,n and Qmeas,s and calculating the R values using averaged RSRP results.

If rangeToBestCell is not configured, the UE shall perform cell reselection to the highest ranked cell. If this cell is found to be not-suitable, the UE shall behave according to subclause 5.2.4.4.

If rangeToBestCell is configured, then the UE shall perform cell reselection to the cell with the highest number of beams above the threshold (i.e. absThreshSS-BlocksConsolidation) among the cells whose R value is within rangeToBestCell of the R value of the highest ranked cell. If there are multiple such cells, the UE shall perform cell reselection to the highest ranked cell among them. If this cell is found to be not-suitable, the UE shall behave according to subclause 5.2.4.4.

In all cases, the UE shall reselect the new cell, only if the following conditions are met:

-	the	new cell is better than the serving cell according to the cell reselection criteria specified above during a time interval TreselectionRAT;

-	more than 1 second has elapsed since the UE camped on the current serving cell.

[TS 38.304, clause 5.3.1]

Cell status and cell reservations are indicated in the MIB or SIB1 message as specified in TS 38.331 [3] by means of following fields:

-	cellBarred (IE type: "barred" or "not barred") 
Indicated in MIB message. In case of multiple PLMNs or NPNs indicated in SIB1, this field is common for all PLMNs and NPNs

-	cellReservedForOperatorUse (IE type: "reserved" or "not reserved") 
Indicated in SIB1 message. In case of multiple PLMNs or NPNs indicated in SIB1, this field is specified per PLMN or per SNPN.

-	cellReservedForOtherUse (IE type: "true") 
Indicated in SIB1 message. In case of multiple PLMNs indicated in SIB1, this field is common for all PLMNs.

-	cellReservedForFutureUse (IE type: "true") 
Indicated in SIB1 message. In case of multiple PLMNs or NPNs indicated in SIB1, this field is common for all PLMNs and NPNs.

NOTE 0:	IAB-MT ignores the cellBarred, cellReservedForOperatorUse, cellReservedForFutureUse and intraFreqReselection (i.e. treats intraFreqReselection as if it was set to allowed) as defined in TS 38.331 [3]. IAB-MT also ignores cellReservedForOtherUse for cell barring determination (i.e. NPN capable IAB-MT considers cellReservedForOtherUse for determination of an NPN-only cell) as defined in TS 38.331 [3].

-	iab-Support (IE type: "true")
Indicated in SIB1 message. In case of multiple PLMNs or NPNs indicated in SIB1, this field is specified per PLMN or per SNPN.

When cell status is indicated as "not barred" and "not reserved" for operator use and not "true" for other use and not "true" for future use,

-	All UEs shall treat this cell as candidate during the cell selection and cell reselection procedures.

When cell broadcasts any CAG-IDs or NIDs and the cell status is indicated as "not barred" and "not reserved" for operator use and "true" for other use, and not "true" for future use:

-	All NPN-capable UEs shall treat this cell as candidate during the cell selection and cell reselection procedures, other UEs shall treat this cell as if cell status is "barred".

When cell status is indicated as "true" for other use, and either cell does not broadcast any CAG-IDs or NIDs or does not broadcast any CAG-IDs and the UE is not operating in SNPN Access Mode,

-	The UE shall treat this cell as if cell status is "barred".

When cell status is indicated as "true" for future use,

-	The UE shall treat this cell as if cell status is "barred".

When cell status is indicated as "not barred" and "reserved" for operator use for any PLMN/SNPN and not "true" for other use and not "true" for future use,

-	UEs assigned to Access Identity 11 or 15 operating in their HPLMN/EHPLMN shall treat this cell as candidate during the cell selection and reselection procedures if the field cellReservedForOperatorUse for that PLMN set to "reserved".

-	UEs assigned to Access Identity 11 or 15 shall treat this cell as candidate during the cell selection and reselection procedures if the field cellReservedForOperatorUse for selected/registered SNPN is set to "reserved".

-	UEs assigned to an Access Identity 0, 1, 2 and 12 to 14 shall behave as if the cell status is "barred" in case the cell is "reserved for operator use" for the registered PLMN/SNPN or the selected PLMN/SNPN.

NOTE 1:	Access Identities 11, 15 are only valid for use in the HPLMN/ EHPLMN; Access Identities 12, 13, 14 are only valid for use in the home country as specified in TS 22.261 [12].

When cell status "barred" is indicated or to be treated as if the cell status is "barred",

-	The UE is not permitted to select/reselect this cell, not even for emergency calls.

-	The UE shall select another cell according to the following rule:

-	If the cell is to be treated as if the cell status is "barred" due to being unable to acquire the MIB:

-	the UE may exclude the barred cell as a candidate for cell selection/reselection for up to 300 seconds.

-	the UE may select another cell on the same frequency if the selection criteria are fulfilled.

-	else:

-	If the field intraFreqReselection in MIB message is set to "allowed":

-	the UE may select another cell on the same frequency if re-selection criteria are fulfilled;

-	If the cell is to be treated as if the cell status is "barred" due to being unable to acquire the SIB1:

-	the UE may exclude the barred cell as a candidate for cell selection/reselection for up to 300 seconds;

-	else:

-	the UE shall exclude the barred cell as a candidate for cell selection/reselection for 300 seconds.

-	If the field intraFreqReselection in MIB message is set to "not allowed":

-	If the cell is to be treated as if the cell status is "barred" due to being unable to acquire the SIB1:

-	the UE may exclude the barred cell as a candidate for cell selection/reselection for up to 300 seconds;

-	If the cell operates in licensed spectrum:

-	the UE shall not re-select to another cell on the same frequency as the barred cell and exclude such cell(s) as candidate(s) for cell selection/reselection for 300 seconds;

-	else:

-	the UE may select to another cell on the same frequency if the reselection criteria are fulfilled.

-	else:

-	If the cell operates in licensed spectrum, or if this cell belongs to a PLMN which is indicated as being equivalent to the registered PLMN or the selected PLMN of the UE, or if this cell belongs to the registered SNPN or the selected SNPN of the UE:

-	the UE shall not re-select to another cell on the same frequency as the barred cell and exclude such cell(s) as candidate(s) for cell selection/reselection for 300 seconds;

-	else:

-	the UE may select to another cell on the same frequency if the reselection criteria are fulfilled.

-	the UE shall exclude the barred cell as a candidate for cell selection/reselection for 300 seconds.

The cell selection of another cell may also include a change of RAT.

NOTE 2:	If barring of a cell is triggered by the condition of trackingAreaCode not being provided, as specified in TS 38.331 [3], the barring only applies to this PLMN and the UE can re-evaluate the barring condition again due to selection of another PLMN.

6.5.2.6.3	Test description

6.5.2.6.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1, NR Cell 3 and NR Cell 6.

-	NR Cell 3 and NR Cell 6 are CAG cells.

-	System information combination NR-4 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used for NR Cell 1, NR Cell 3 and NR Cell 6.

-	The PLMNs are identified in the test by the identifiers in Table 6.5.2.6.3.1-1 and the PLMN settings are defined in TS 36.523-1 [13] table 6.0.1-1.

-	NR Cell 6 TAC is set to TAC=2.

Table 6.5.2.6.3.1–1: PLMN and CAG identifiers

| NR Cell | PLMN name | CAG IDs |
| --- | --- | --- |
| 1 | PLMN 1 | - |
| 3 | PLMN 2 | 1 |
| 6 | PLMN 2 | 2 |

UE:

-	The UE is in Automatic PLMN selection mode.

Preamble:

-	The UE is registered on NR Cell 1 using the procedure described in TS 38.508-1 [4] Table 4.5.2.2-2, except that the REGISTRATION ACCEPT message includes CAG information list with CAG-ID 1 and CAG-ID 2 before it is in state Switched OFF (State 0N-B).

6.5.2.6.3.2	Test procedure sequence

Table 6.5.2.6.3.2-1/2 shows the cell configurations used during the test. The configuration T0 indicates the initial conditions for preamble. Subsequent configurations marked “T1”, “T2” and “T3” are applied at the points indicated in the Main behaviour description in Table 6.5.2.6.3.2-3. Cell powers are chosen for a serving cell and a non-suitable “Off” cell as defined in TS 38.508-1 [4] Table 6.2.2.1-3 for FR1 and Table 6.2.2.2-2 for FR2.

Table 6.5.2.6.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 3 | NR Cell 6 | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -88 | "Off" | "Off" | The power level values are assigned to ensure the UE registered on NR Cell 1. |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -88 | -88 | "Off" |  |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | "Off" | -88 | -78 | The power level values are assigned to satisfy RNR Cell 3 < RNR Cell 6. |
| T3 | SS/PBCH SSS EPRE | dBm/SCS | "Off" | -88 | "Off" |  |

Table 6.5.2.6.3.2-2: Time instances of cell power level and parameter changes for FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 3 | NR Cell 6 | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -91 | "Off" | "Off" | The power level values are assigned to ensure the UE registered on NR Cell 1. |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -91 | -91 | "Off" |  |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | "Off" | -91 | -82 | The power level values are assigned to satisfy RNR Cell 3 < RNR Cell 6. |
| T3 | SS/PBCH SSS EPRE | dBm/SCS | "Off" | -91 | "Off" |  |

Table 6.5.2.6.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SIB1 for NR cell 1 is updated according to Table 6.5.2.6.3.3-2. | - | - | - | - |
| 1A | Wait for 2.1* modification period second for the SS to transmit modified system information. (Note 1) | - | - | - | - |
| 2 | The UE is switched on. | - | - | - | - |
| 3 | Check: Does the UE transmit a RRCSetupRequest message on NR Cell 1 within 60s ? | --> | NR RRC: RRCSetupRequest | 1 | F |
| 4 | The SS changes cell power levels according to row "T1" in Table 6.5.2.6.3.2-1 for FR1 and Table 6.5.2.6.3.2-2 for FR2. | - | - | - | - |
| 5-22a1 | Steps 2-20a1 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed on NR Cell 3. | - | - | - | - |
| 23 | SS adjusts cell levels according to row T2 of table 6.5.2.6.3.2-1/2. | - | - | - | - |
| 24 | Check: Does the UE transmit a RRCSetupRequest message on NR Cell 6. | --> | NR RRC: RRCSetupRequest | 2 | P |
| 25-29 | Steps 2-6a1 of Table 4.9.5.2.2-1 of the generic procedure in TS 38.508-1 [4] are performed on NR Cell 6. | - | - | - | - |
| 30 | Void | - | - | - | - |
| 31 | SS adjusts cell levels according to row T3 of table 6.5.2.6.3.2-1/2. | - | - | - | - |
| 32-36 | Steps 1 to 5 of the generic test procedure in TS 38.508-1 [4] Table 4.9.5.2.2-1 are performed on NR Cell 3. | - | - | - | - |
| Note 1: The modification period, expressed in number of radio frames = modificationPeriodCoeff * defaultPagingCycle. |  |  |  |  |  |

6.5.2.6.3.3	Specific message contents

Table 6.5.2.6.3.3-1: REGISTRATION REQUEST (Preamble)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| 5GS registration type |  |  |  |
| 5GS registration type value | '001'B | Initial registration |  |
| 5GMM capability |  |  |  |
| CAG | ‘1’B | CAG supported |  |

Table 6.5.2.6.3.3-1: REGISTRATION ACCEPT for NR Cell 1 (preamble)

| Derivation path: TS 38.508-1 [4], table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| CAG information list |  | MCC : 002; MNC : 11; CAG only : 0;CAG-ID 1 : 1, CAG-ID 2 : 2 |  |
| Length of CAG information list contents | ‘0000 0000 0000 1101’B |  |  |
| Entry 1 |  |  |  |
| Length of entry contents | ‘0000 1100’B |  |  |
| MCC | MCC of NR Cell 3 | See Table 6.5.2.6.3.1–1 |  |
| MNC | MNC of NR Cell 3 | See Table 6.5.2.6.3.1–1 |  |
| CAG only | ‘0’B |  |  |
| CAG-ID 1 | 1 | CAG ID is coded as a 32 bit BITSTRING |  |
| CAG-ID 2 | 2 | CAG ID is coded as a 32 bit BITSTRING |  |
| Equivalent PLMNs | PLMN2 |  |  |

Table 6.5.2.6.3.3-2: SIB1 for NR Cell 1 (Step 1, Table 6.5.2.6.3.2-3)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-28 with condition CAG |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| cellReservedForOtherUse | true |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.5.2.6.3.3-3: Void

Table 6.5.2.6.3.3-4: SIB1 for NR Cell 6 (preamble and all steps, Table 6.5.2.6.3.2-3)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-28 with condition CAG |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| cellReservedForOtherUse | true |  |  |
| npn-IdentityInfoList-r16 SEQUENCE (SIZE (1..maxPLMN)) OF NPN-IdentityInfo-r16 { | 1 entry |  |  |
| NPN-IdentityInfo-r16[1] SEQUENCE { |  | entry 1 |  |
| npn-IdentityList-r16 SEQUENCE (SIZE (1..maxPLMN)) OF NPN-Identity-r16 { | 1 entry |  |  |
| NPN-Identity-r16[1] CHOICE { |  | entry 1 |  |
| pni-npn-r16 SEQUENCE { |  |  |  |
| plmn-Identity-r16 SEQUENCE { |  |  |  |
| mcc | 002 | PLMN2 MCC |  |
| mnc | 11 | PLMN2 MNC |  |
| } |  |  |  |
| cag-IdentityList-r16 SEQUENCE (SIZE (1..maxNPN-r16)) OF CAG-IdentityInfo-r16 { | 1 entry |  |  |
| CAG-IdentityInfo-r16[1] SEQUENCE { |  | entry 1 |  |
| cag-identity-r16 | 2 | CAG ID is coded as a 32 bit BITSTRING |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| trackingAreaCode-r16 | 000000000000000000000010 |  |  |
| cellIdentity-r16 | Cell Identity of NR Cell 6 |  |  |
| cellReservedForOperatorUse-r16 | notReserved |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.5.2.6.3.3-5: SIB1 for NR Cell 3 (preamble and  all steps, Table 6.5.2.6.3.2-3)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-28 with condition CAG |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| cellReservedForOtherUse | true |  |  |
| npn-IdentityInfoList-r16 SEQUENCE (SIZE (1..maxPLMN)) OF NPN-IdentityInfo-r16 { | 1 entry |  |  |
| NPN-IdentityInfo-r16[1] SEQUENCE { |  | entry 1 |  |
| npn-IdentityList-r16 SEQUENCE (SIZE (1..maxPLMN)) OF NPN-Identity-r16 { | 1 entry |  |  |
| NPN-Identity-r16[1] CHOICE { |  | entry 1 |  |
| pni-npn-r16 SEQUENCE { |  |  |  |
| plmn-Identity-r16 SEQUENCE { |  |  |  |
| mcc | 002 | PLMN2 MCC |  |
| mnc | 11 | PLMN2 MNC |  |
| } |  |  |  |
| cag-IdentityList-r16 SEQUENCE (SIZE (1..maxNPN-r16)) OF CAG-IdentityInfo-r16 { | 1 entry |  |  |
| CAG-IdentityInfo-r16[1] SEQUENCE { |  | entry 1 |  |
| cag-identity-r16 | 1 | CAG ID is coded as a 32 bit BITSTRING |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| trackingAreaCode-r16 | TAC of NR Cell 3 |  |  |
| cellIdentity-r16 | Cell Identity of NR Cell 3 |  |  |
| cellReservedForOperatorUse-r16 | notReserved |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.5.2.6.3.3-6: REGISTRATION ACCEPT (Step 17, Table 6.5.2.6.3.2-3)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| CAG information list |  | MCC : 002; MNC : 11; CAG only : 1; CAG-ID 1 : 2, CAG ID 2: 1 |  |
| Length of CAG information list contents | ‘0000 0000 0000 1101’B |  |  |
| Entry 1 |  |  |  |
| Length of entry contents | ‘0000 1100’B |  |  |
| MCC | MCC of NR Cell 6 | See Table 6.5.2.6.3.1-1 |  |
| MNC | MNC of NR Cell 6 | See Table 6.5.2.6.3.1-1 |  |
| CAG only | ‘1’B |  |  |
| CAG-ID 1 | 2 | CAG ID is coded as a 32 bit BITSTRING |  |
| CAG-ID 2 | 1 | CAG ID is coded as a 32 bit BITSTRING |  |

Table 6.5.2.6.3.3-7: REGISTRATION ACCEPT (Step 35, Table 6.5.2.6.3.2-3)

| Derivation path: TS 38.508-1 [4], table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| CAG information list |  |  |  |
| Length of CAG information list contents | ‘0000 0000 0000 0000’B |  |  |
| Entry 1 | Not present |  |  |