3GPP TSG-RAN5 Meeting #104	R5-245600

# Maastricht, Netherlands, 19th Aug 2024 – 23th Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4587 | rev | 1 | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add new XR test case 7.1.1.6.7 - multi-PUSCH CG Type2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_XR_enh_plus_CT1-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The test case configure one CG, the CG period is 40ms (default configuration in 38.508-1 Table 4.6.3-26:ConfiguredGrantConfig). DCI 01 is used to activate CG Type2, and there are two valid CG PUSCH occasions in a CG period. The test case is added to test that UE used the correct CG PUSCH occasions and correct harq process id to transmit UL MAC PDU based on multi-PUSCH CG Type2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new XR test case 7.1.1.6.7 -Multiple CG PUSCH occasions per CG period and CG Type2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR_XR_enh_plus_CT1-UEConTest is incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.6.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.523-2 CR 0498 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from: R5-244728r1. |  |  |  |  |  |  |  |  |

<Start of modified section 1>

7.1.1.6.7	Correct handling of UL grant / Multiple CG PUSCH occasions per CG period / Configured grant Type 2

7.1.1.6.7.1	Test Purpose (TP)

(1)

with { UE in RRC_Connected state with DRB established and configured uplink grant is enabled with multi-PUSCH Configured grant type 2 }

ensure that {

when { UE receives an UL configured grant type 2 addressed to its stored CS-RNTI with NDI set as 0 and PDCCH content indicates single UL grant Type 2 scheduling activation }

then { UE transmits UL MAC PDU using the first transmission opportunity of PUSCH where the configured uplink grant was (re-)initialised}

}

(2)

with { UE in RRC_Connected state with DRB established and multi-PUSCH Configured grant type 2 is activated}

ensure that {

when { The symbol in which equation [(SFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot)+ (slot number in the frame × numberOfSymbolsPerSlot) + symbol number in the slot] =[(SFNstart time × numberOfSlotsPerFrame × numberOfSymbolsPerSlot+ slotstart time × numberOfSymbolsPerSlot + symbolstart time) + N × periodicity] modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot)is satisfied }

then { UE transmits UL MAC PDU using the first configured uplink grant in a multi-PUSCH configured grant }

}

(3)

with { UE in RRC_Connected state with DRB established and multi-PUSCH Configured grant type 2 is activated }

ensure that {

when { UE has transmitted UL MAC PDU using the first configured uplink grant in a multi-PUSCH configured grant or first transmission opportunity of PUSCH where the configured uplink grant was (re-)initialised, and UL Data is still available }

then { UE transmits UL MAC PDU in the Mth (1 < M ≤ nrofSlotsInCG-Period) configured uplink grant within a periodicity occurs (M-1) × numberOfSymbolsPerSlot symbols after the symbol in which the first configured uplink grant in that periodicity occurs }

}

(4)

with { UE in RRC_Connected state with DRB established and multi-PUSCH Configured grant type 2 is activated }

ensure that {

when { UE receives a UL grant addressed to its CS-RNTI with NDI set as 1 for retransmission }

then { UE re-transmits MAC PDU as per the new grant }

}

7.1.1.6.7.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 38.321 clauses 5.4.1 and 5.8.2, 3GPP TS 38.300 clause 16.15.4.1. Unless otherwise stated these are Rel-18 requirements.

[TS 38.321, clause 5.4.1]

For a multi-PUSCH configured grant (as specified in clause 5.8.2) configured with neither harq-ProcID-Offset2 nor cg-RetransmissionTimer, the HARQ Process ID associated with the first symbol of a UL transmission is derived from the following equation:

HARQ Process ID = [nrofSlotsInCG-Period× floor (CURRENT_symbol / periodicity) + ID_OFFSET] modulo nrofHARQ-Processes

For a multi-PUSCH configured grant configured with harq-ProcID-Offset2, the HARQ Process ID associated with the first symbol of a UL transmission is derived from the following equation:

HARQ Process ID = [nrofSlotsInCG-Period × floor (CURRENT_symbol / periodicity) + ID_OFFSET] modulo nrofHARQ-Processes + harq-ProcID-Offset2

where, if cg-SDT-PeriodicityExt (as defined in TS 38.331 [5]) is not configured, CURRENT_symbol = (SFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot + slot number in the frame × numberOfSymbolsPerSlot + symbol number in the slot), and numberOfSlotsPerFrame and numberOfSymbolsPerSlot refer to the number of consecutive slots per frame and the number of consecutive symbols per slot, respectively as specified in TS 38.211 [8]; alternatively, if cg-SDT-PeriodicityExt (as defined in TS 38.331 [5]) is configured, CURRENT_symbol = ((H-SFN × numberOfSFNperH-SFN + SFN) × numberOfSlotsPerFrame × numberOfSymbolsPerSlot + slot number in the frame × numberOfSymbolsPerSlot + symbol number in the slot), and numberOfSFNperH-SFN, numberOfSlotsPerFrame and numberOfSymbolsPerSlot refer to the number of consecutive frames per H-SFN, the number of consecutive slots per frame and the number of consecutive symbols per slot, respectively as specified in TS 38.211 [8]. For a multi-PUSCH configured grant, ID_OFFSET equals 0 for the first configured uplink grant within a periodicity of the configuration and K for the Kth (1 ≤ K < nrofSlotsInCG-Period) valid configured uplink grant after the first configured uplink grant within the same periodicity. A configured uplink grant in a multi-PUSCH configured grant is considered valid if it satisfies the conditions specified in clause 6.1 in TS 38.214 [7].

[TS 38.321, clause 5.8.2]

There are two types of transmission without dynamic grant:

-	configured grant Type 1 where an uplink grant is provided by RRC, and stored as configured uplink grant;

-	configured grant Type 2 where an uplink grant is provided by PDCCH, and stored or cleared as configured uplink grant based on L1 signalling indicating configured uplink grant activation or deactivation.

Type 1 and Type 2 are configured by RRC for a Serving Cell per BWP. Multiple configurations can be active simultaneously in the same BWP. For Type 2, activation and deactivation are independent among the Serving Cells. For the same BWP, the MAC entity can be configured with both Type 1 and Type 2.

A multi-PUSCH configured grant has multiple consecutive configured uplink grants within a periodicity. Both Type 1 and Type 2 can be configured for a multi-PUSCH configured grant by RRC.

…

RRC configures the following parameters when the configured grant Type 2 is configured:

-	cs-RNTI: CS-RNTI for activation, deactivation, and retransmission;

-	periodicity: periodicity of the configured grant Type 2;

-	nrofHARQ-Processes: the number of HARQ processes for configured grant;

-	harq-ProcID-Offset: offset of HARQ process for configured grant configured with cg-RetransmissionTimer for operation with shared spectrum channel access;

-	harq-ProcID-Offset2: offset of HARQ process for configured grant not configured with cg-RetransmissionTimer.

…

RRC configures the following parameter when a multi-PUSCH configured grant is configured:

-	nrofSlotsInCG-Period: the number of configured uplink grants in a periodicity of a multi-PUSCH configured grant.

….

After an uplink grant is configured for a configured grant Type 2, the MAC entity shall consider sequentially that the configured uplink grant, or the first configured uplink grant in a multi-PUSCH configured grant, in the Nth (N ≥ 0) periodicity occurs in the symbol for which:

[(SFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot)
	+ (slot number in the frame × numberOfSymbolsPerSlot) + symbol number in the slot] =
	[(SFNstart time × numberOfSlotsPerFrame × numberOfSymbolsPerSlot
	+ slotstart time × numberOfSymbolsPerSlot + symbolstart time) + N × periodicity]
	modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot)

where SFNstart time, slotstart time, and symbolstart time are the SFN, slot, and symbol, respectively, of the first transmission opportunity of PUSCH where the configured uplink grant was (re-)initialised.

For a multi-PUSCH configured grant Type 2, the Mth (1 < M ≤ nrofSlotsInCG-Period) configured uplink grant within the same periodicity occurs (M-1) × numberOfSymbolsPerSlot symbols after the symbol in which the first configured uplink grant in that periodicity occurs.

…

When the configured uplink grant is released by upper layers, all the corresponding configurations shall be released and all corresponding uplink grants shall be cleared.

The MAC entity shall:

1>	if at least one configured uplink grant confirmation has been triggered and not cancelled; and

1>	if the MAC entity has UL resources allocated for new transmission:

2>	if, in this MAC entity, at least one configured uplink grant is configured by configuredGrantConfigToAddModList:

3>	instruct the Multiplexing and Assembly procedure to generate a Multiple Entry Configured Grant Confirmation MAC CE as defined in clause 6.1.3.31.

2>	else:

3>	instruct the Multiplexing and Assembly procedure to generate a Configured Grant Confirmation MAC CE as defined in clause 6.1.3.7.

2>	cancel all triggered configured uplink grant confirmation(s).

For a configured grant Type 2, the MAC entity shall clear the configured uplink grant(s) immediately after first transmission of Configured Grant Confirmation MAC CE or Multiple Entry Configured Grant Confirmation MAC CE which confirms the configured uplink grant deactivation.

Retransmissions use:

-	repetition of configured uplink grants; or

-	received uplink grants addressed to CS-RNTI; or

-	configured uplink grants with cg-RetransmissionTimer, cg-RRC-RetransmissionTimer or cg-SDT-RetransmissionTimer configured.

[TS 38.321, clause 16.15.4.1]

The following enhancements for configured grant-based PUSCH transmission are introduced:

-	Support of multiple CG PUSCH transmission occasions within a single period of a CG configuration;

-	Indication of unused CG PUSCH occasion(s) of a CG configuration with Uplink Control Information multiplexed in CG PUSCH transmission of the CG configuration.

[TS 38.213, clause 10.2]

If a UE is provided a single configuration for UL grant Type 2 PUSCH or for SPS PDSCH, validation of the DCI format is achieved if all fields for the DCI format are set according to Table 10.2-1 or Table 10.2-2.

Table 10.2-1: Special fields for single DL SPS or single UL grant Type 2 scheduling activation PDCCH validation when a UE is provided a single SPS PDSCH or UL grant Type 2 configuration in the active DL/UL BWP of the scheduled cell

|  | DCI format 0_0/0_2 | DCI format 0_1 | DCI format 1_0/1_2/4_1 | DCI format 1_1/4_2 |
| --- | --- | --- | --- | --- |
| HARQ process number (if present) | set to all '0's | set to all '0's | set to all '0's | set to all '0's |
| Redundancy version (if present) | set to all '0's | For the enabled transport block: set to all '0's | set to all '0's | For the enabled transport block: set to all '0's |

7.1.1.6.7.3	Test description

7.1.1.6.7.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0 and UM DRB should be established on NR Cell 1.The loop back size is set to accommodate one RLC SDU in UL of same size as one RLC SDU in DL and 1 byte MAC subheader for Configured Grant Confirmation MAC CE.

7.1.1.6.7.3.2	Test procedure sequence

Table 7.1.1.6.7.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits NR RRCReconfiguration message to configure UL configured grant type 2. | <-- | (NR RRC: RRCReconfiguration) | - | - |
| 2 | The UE transmits NR RRCReconfigurationComplete message. | --> | (NR RRC: RRCReconfigurationComplete) | - | - |
| 3 | SS transmits a DL MAC PDU containing 4 RLC SDUs (RLC SDU #1, RLC SDU #2, RLC SDU #3 and RLC SDU #4) on UM DRB. The size of each RLC SDU is 99 bytes | <-- | MAC PDU (four RLC SDUs) | - | - |
| 4 | The UE transmits a Scheduling Request, indicating that loop back SDUs are ready for transmission in UL RLC. | --> | (SR) | - | - |
| 5 | SS transmits a UL configured grant type 2 addressed to UE’s stored CS-RNTI in Slot ‘n’, SFN ‘m’ of PDCCH, NDI=0, allowing the UE to transmit one loop back SDU and 1 byte MAC subheader for Configured Grant Confirmation MAC CE. (Note 1) | <-- | (UL configured Grant type 2) | - | - |
| 6 | Check: Does the UE transmit a MAC PDU containing RLC SDU #1 received in step 3 and and a Configured Grant Confirmation MAC CE in Slot ’n+K2’, SFN ‘m’? (Note 2) | --> | MAC PDU (RLC SDU #1 and Configured Grant Confirmation MAC CE) | 1 | P |
| 7 | Check: Does the UE transmit a MAC PDU containing RLC SDU #2 received in step 3 in the next UL slot after slot ‘n+K2’, SFN ‘m’? (Note 2) | --> | MAC PDU (RLC SDU #2) | 3 | P |
| 8 | Check: Does the UE transmit a MAC PDU containing RLC SDU #3 received in step 3 in slot ‘n+K2’, SFN ‘m+4’? (Note 2) | --> | MAC PDU (RLC SDU #3) | 2 | P |
| 9 | Check: Does the UE transmit a MAC PDU containing RLC SDU #4 received in step 3 in the next UL slot after slot ‘n+K2’, SFN ‘m+4’? (Note 2) | --> | MAC PDU (RLC SDU #4) | 3 | P |
| 10 | SS transmits a UL grant addressed to UE’s stored CS-RNTI with NDI set as 1 and HARQ process id set to be the same as step 9 in SFN ‘m+4’ , allowing the UE to retransmit one loop back SDU.(Note 3) | <-- | (UL Grant) | - | - |
| 11 | Check: Does the UE retransmit the MAC PDU containing RLC SDU #4 as in step 9. | --> | MAC PDU (RLC SDU #4) | 4 | P |
| 12 | The SS transmits NR RRCReconfiguration message to release UL configured grant type 2. | <-- | (NR RRC: RRCReconfiguration | - | - |
| 13 | The UE transmits NR RRCReconfigurationComplete message. | --> | (NR RRC: RRCReconfigurationComplete | - | - |
| Note 1: TB size for configured grant type 2 is 848 bits, which is enough to allow the UE to transmit one PDU at a time (99 bytes RLC SDU + 1 byte UM RLC Header + 2 bytes MAC Sub PDU header + 2 bytes for short BSR or padding + 1 byte MAC subheader for Configured Grant Confirmation MAC CE). Note 2: For (TDD AND SCS15) OR SCS120, n=0 and K2=4, the valid CG PUSCH occasion is Slot 4 and Slot 9; For other condtion, n=4 and K2=4, the valid CG PUSCH occasion is Slot 8 and Slot 9. Note 3: HARQ process id is calculated based on the formula “HARQ Process ID = [nrofSlotsInCG-Period× floor (CURRENT_symbol / periodicity) + ID_OFFSET] modulo nrofHARQ-Processes” specified in TS 38.321 5.4.1. |  |  |  |  |  |

7.1.1.6.7.3.3	Specific message contents

Table 7.1.1.6.7.3.3-1: RRCReconfiguration (step 1, Table 7.1.1.6.7.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| radioBearerConfig | Not present |  |  |
| secondaryCellGroup | Not Present |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| masterCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) |  |
| dedicatedNAS-MessageList | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.6.7.3.3-2: CellGroupConfig (Table 7.1.1.6.7.3.3-1)

| Derivation path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlc-BearerToAddModList | Not present |  |  |
| mac-CellGroupConfig | Not present |  |  |
| physicalCellGroupConfig SEQUENCE { |  |  |  |
| cs-RNTI CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| RNTI-Value | ‘FFE0’H |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| reconfigurationWithSync | Not present |  |  |
| spCellConfigDedicated SEQUENCE { |  |  |  |
| uplinkConfig SEQUENCE { |  |  |  |
| initialUplinkBWP SEQUENCE { |  |  |  |
| configuredGrantConfig | ConfiguredGrantConfig | Table 7.1.1.6.7.3.3-3 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.6.7.3.3-3: ConfiguredGrantConfig (Table 7.1.1.6.7.3.3-2)

| Derivation path: TS 38.508-1 [4], Table 4.6.3-26 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ConfiguredGrantConfig ::= SEQUENCE { |  |  |  |
| uci-OnPUSCH CHOICE { |  |  |  |
| setup CHOICE { |  |  |  |
| semiStatic | BetaOffsets | TS 38.508-1[4], Table 4.6.3-6A |  |
| } |  |  |  |
| } |  |  |  |
| cg-RetransmissionTimer-r16 | Not present |  |  |
| cg-minDFI-Delay-r16 | Not present |  |  |
| cg-nrofPUSCH-InSlot-r16 | Not present |  |  |
| cg-nrofSlots-r16 | Not present |  |  |
| cg-UCI-Multiplexing-r16 | Not present |  |  |
| cg-COT-SharingOffset-r16 | Not present |  |  |
| betaOffsetCG-UCI-r16 | Not present |  |  |
| cg-COT-SharingList-r16 | Not present |  |  |
| harq-ProcID-Offset-r16 | Not present |  |  |
| harq-ProcID-Offset2-r16 | Not present |  |  |
| configuredGrantConfigIndex-r16 | Not present |  |  |
| configuredGrantConfigIndexMAC-r16 | Not present |  |  |
| periodicityExt-r16 | Not present |  |  |
| startingFromRV0-r16 | Not present |  |  |
| phy-PriorityIndex-r16 | Not present |  |  |
| autonomousTx-r16 | Not present |  |  |
| cg-betaOffsetsCrossPri0-r17 | Not present |  |  |
| cg-betaOffsetsCrossPri1-r17 | Not present |  |  |
| mappingPattern-r17 | Not present |  |  |
| sequenceOffsetForRV-r17 | Not present |  |  |
| p0-PUSCH-Alpha2-r17 | Not present |  |  |
| powerControlLoopToUse2-r17 | Not present |  |  |
| cg-COT-SharingList-r17 | Not present |  |  |
| periodicityExt-r17 | Not present |  |  |
| repK-v1710 | Not present |  |  |
| nrofHARQ-Processes-v1700 | Not present |  |  |
| harq-ProcID-Offset2-v1700 | Not present |  |  |
| configuredGrantTimer-v1700 | Not present |  |  |
| cg-minDFI-Delay-v1710 | Not present |  |  |
| harq-ProcID-Offset-v1730 | Not present |  |  |
| cg-nrofSlots-r17 | Not present |  |  |
| disableCG-RetransmissionMonitoring-r18 | Not present |  |  |
| nrofSlotsInCG-Period-r18 | 6 |  | (TDD AND SCS15) OR SCS120 |
|  | 2 |  |  |
| uto-UCI-Config-r18 | Not present |  |  |
| } |  |  |  |

Table 7.1.1.6.7.3.3-4: RRCReconfiguration (step 12, Table 7.1.1.6.7.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| radioBearerConfig | Not present |  |  |
| secondaryCellGroup | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) |  |
| dedicatedNAS-MessageList | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.6.7.3.3-5: CellGroupConfig (Table 7.1.1.6.7.3.3-4)

| Derivation path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| spCellConfigDedicated SEQUENCE { |  |  |  |
| uplinkConfig SEQUENCE { |  |  |  |
| initialUplinkBWP SEQUENCE { |  |  |  |
| configuredGrantConfig CHOICE { |  |  |  |
| release | Null |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

<End of modified section 1>