3GPP TSG-RAN5 Meeting #104	R5-245630

# Maastricht, Netherlands, August 19-23, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3327 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of Cell DTX and DRX related IEs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | Netw_Energy_NR-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The affected CRs added new test cases for cell DTX/DRX operation. This document is to add the related IEs into TS 38.508-1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding Cell DTX and DRX related IEs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Cell DTX and DRX test case can’t be tested properly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.523-1 CR 4626 - 4629 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R5-245489r1 |  |  |  |  |  |  |  |  |

<Start of Changes>

–	CellDTX-DRX-Config

Table 4.6.3-18A: CellDTX-DRX-Config

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellDTX-DRX-Config-r18 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

<Unchanged Text Skipped>

PDCCH-Config

Table 4.6.3-95: PDCCH-Config

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCCH-Config ::= SEQUENCE { |  |  |  |
| controlResourceSetToAddModList | Not present |  | EN-DC, MSS |
| controlResourceSetToAddModList SEQUENCE(SIZE (1..3)) OF ControlResourceSet { | 1 entry |  |  |
| ControlResourceSet[1] | ControlResourceSet | entry 1 |  |
| } |  |  |  |
| controlResourceSetToReleaseList | Not present |  |  |
| searchSpacesToAddModList SEQUENCE(SIZE (1..10)) OF SearchSpace { | 1 entry |  |  |
| SearchSpace[1] | SearchSpace with condition USS | entry 1 |  |
|  | SearchSpace with condition MSS | entry 1 | MSS |
|  | SearchSpace with condition SLSS | entry 1 | SLSS |
|  | SearchSpace with condition DCI_2_6 | entry 1 | DCI_2_6 |
|  | SearchSpace with condition DCI_2_9 | entry 1 | DCI_2_9 |
| } |  |  |  |
| searchSpacesToReleaseList | Not present |  |  |
| downlinkPreemption | Not present |  |  |
| tpc-PUSCH | Not present |  |  |
| tpc-PUCCH | Not present |  |  |
| tpc-SRS | Not present |  |  |
| controlResourceSetToAddModListSizeExt-v1610 | Not present |  |  |
| controlResourceSetToReleaseListSizeExt-r16 | Not present |  |  |
| searchSpacesToAddModListExt-r16 | Not present |  |  |
| searchSpacesToAddModListExt-r16 SEQUENCE(SIZE (1..10)) OF SearchSpaceExt-r16 { | 1 entry |  | DCI_2_6 |
| SearchSpaceExt-r16[1] SEQUENCE{ |  | entry 1 |  |
| controlResourceSetId-r16 | ControlResourceSetId |  |  |
| searchSpaceType-r16 SEQUENCE { |  |  |  |
| common-r16 SEQUENCE { |  |  |  |
| dci-Format2-4-r16 | Not present |  |  |
| dci-Format2-5-r16 | Not present |  |  |
| dci-Format2-6-r16 SEQUENCE { |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| searchSpaceGroupIdList-r16 | Not present |  |  |
| freqMonitorLocations-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| uplinkCancellation-r16 | Not present |  |  |
| monitoringCapabilityConfig-r16 | Not present |  |  |
| searchSpaceSwitchConfig-r16 | Not present |  |  |
| searchSpacesToAddModListExt-v1700 | Not present |  |  |
| searchSpacesToAddModListExt-v1700 SEQUENCE(SIZE (1..10)) OF SearchSpaceExt-v1700 { | 1 entry |  | MSS |
| SearchSpaceExt-v1700[1] SEQUENCE { |  | entry 1 |  |
| monitoringSlotPeriodicityAndOffset-v1710 | Not present |  |  |
| monitoringSlotsWithinSlotGroup-r17 | Not present |  |  |
| duration-r17 | Not present |  |  |
| searchSpaceType-r17 SEQUENCE { |  |  |  |
| common-r17 SEQUENCE { |  |  |  |
| dci-Format4-0-r17 | Not present |  |  |
| dci-Format4-1-r17 | Not present |  |  |
| dci-Format4-2-r17 | Not present |  |  |
| dci-Format4-1-AndFormat4-2-r17 SEQUENCE { |  |  |  |
| } |  |  |  |
| dci-Format2-7-r17 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| searchSpaceGroupIdList-r17 | Not present |  |  |
| searchSpaceLinkingId-r17 | Not present |  |  |
| } |  |  |  |
| monitoringCapabilityConfig-v1710 | Not present |  |  |
| searchSpaceSwitchConfig-r17 | Not present |  |  |
| pdcch-SkippingDurationList-r17 | Not present |  |  |
| searchSpacesToAddModListExt-v1800 | Not present |  |  |
| searchSpacesToAddModListExt-v1800 SEQUENCE(SIZE (1..10)) OF SearchSpaceExt-v1800 { | 1 entry |  | DCI_2_9 |
| SearchSpaceExt-v1800[1] SEQUENCE { |  | entry 1 |  |
| searchSpaceType-r18 SEQUENCE { |  |  |  |
| common-r18 SEQUENCE { |  |  |  |
| dci-Format2-9-r18 SEQUENCE { |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| MSS | SearchSpace for MBS Multicast receptioin |
| SLSS | SearchSpace for SL mode 1 transmission |
| DCI_2_6 | Transmission of DCI_2_6 is required |
| DCI_2_9 | Transmission of DCI_2_9 is required |

<Unchanged Text Skipped>

SearchSpace

Table 4.6.3-162: SearchSpace

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| searchSpaceId | SearchSpaceId with condition CSS |  | CSS |
|  | SearchSpaceId with condition USS |  | USS |
|  | SearchSpaceId with condition SISS |  | SISS |
|  | SearchSpaceId with condition MSS |  | MSS |
|  | SearchSpaceId with condition SLSS |  | SLSS |
|  | SearchSpaceId with condition DCI_2_6 |  | DCI_2_6 |
|  | SearchSpaceId with condition DCI_2_9 |  | DCI_2_9 |
|  | SearchSpaceId with condition PEI |  | PEI |
| controlResourceSetId | ControlResourceSetId |  |  |
|  | ControlResourceSetId with condition Common0 |  | CSS, SISS, PEI |
| monitoringSlotPeriodicityAndOffset CHOICE { |  |  |  |
| sl1 | NULL |  |  |
| sl5 | 0 |  | (CSS OR USS) AND HD_FDD |
| sl10 | 5 |  | SISS |
| } |  |  |  |
| duration | Not present | 1 slot per default |  |
|  | 2 |  | SISS |
|  | 4 |  | (CSS OR USS) AND HD_FDD |
| monitoringSymbolsWithinSlot | 10000000000000 |  |  |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel1 | n0 |  |  |
| aggregationLevel2 | n4 |  | FR1 |
|  | n3 |  | FR2 |
| aggregationLevel4 | n2 |  |  |
| aggregationLevel8 | n1 |  |  |
| aggregationLevel16 | n0 |  |  |
| } |  |  |  |
| searchSpaceType | Not present |  | MSS |
| searchSpaceType CHOICE { |  |  |  |
| common SEQUENCE { |  |  | CSS, SISS, DCI_2_6, PEI |
| dci-Format0-0-AndFormat1-0 SEQUENCE { |  |  |  |
| } |  |  |  |
| dci-Format2-0 | Not present |  |  |
| dci-Format2-1 | Not present |  |  |
| dci-Format2-2 | Not present |  |  |
| dci-Format2-3 | Not present |  |  |
| } |  |  |  |
| ue-Specific SEQUENCE { |  |  | USS, SLSS |
| dci-Formats | formats0-0-And-1-0 |  | Short_DCI |
| dci-Formats | formats0-1-And-1-1 |  |  |
| dci-Formats-MT-r16 | Not present |  |  |
| dci-FormatsSL-r16 | formats3-0 |  | SLSS |
| dci-FormatsExt-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| CSS | Common SearchSpace |
| USS | UE-Specific SearchSpace |
| Short_DCI | Used in test scenarios requiring DCI formats 0-0 and 1-0 on USS |
| SISS | SearchSpace for SI |
| MSS | SearchSpace for MBS Multicast reception |
| PEI | Paging Early Indication is configured in the cell. |
| SLSS | SearchSpace for SL mode 1 transmission |
| DCI_2_6 | Transmission of DCI_2_6 is required |
| DCI_2_9 | Transmission of DCI_2_9 is required |
| HD_FDD | SIG AND pc_halfDuplexFDD_TypeA_RedCap_r17 (i.e HD_FDD UE are performing signalling test on FDD band) |

–	SearchSpaceId

Table 4.6.3-163: SearchSpaceId

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpaceID | 1 |  | CSS |
|  | 2 |  | USS |
|  | 3 |  | SISS |
|  | 4 |  | PEI |
|  | 5 |  | MSS |
|  | 6 |  | SLSS |
|  | 7 |  | DCI_2_6 |
|  | 9 |  | DCI_2_9 |

| Condition | Explanation |
| --- | --- |
| CSS | Common SearchSpace |
| USS | UE-Specific SearchSpace |
| SISS | SearchSpace for SI |
| PEI | Paging Early Indication is configured in the cell. |
| MSS | SearchSpace for MBS Multicast reception. |
| SLSS | SearchSpace for SL mode 1 transmission |
| DCI_2_6 | Transmission of DCI_2_6 is required |
| DCI_2_9 | Transmission of DCI_2_9 is required |

<End of Changes>