3GPP TSG-RAN5 Meeting #104	R5-245637

# Maastricht, Netherlands, 19th Aug 2024 – 23th Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3288 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of PUSCH-Config for DCI format 0_3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MC_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | DCI format 0-3 configuration is need to be added in PUSCH-config.It is used in singleDCI test case 7.1.1.7.3.x and 7.1.1.7.5.x. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add pusch-ConfigDCI-0-3-r18 in PUSCH-config. When the PUSCH is scheduled by DCI 0-3, pusch-ConfigDCI-0-3-r18 will be used. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR_MC_enh-UEConTest WI is incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from: R5-244697r1. |  |  |  |  |  |  |  |  |

<Start of modified section 1>

–	PUSCH-Config

Table 4.6.3-118: PUSCH-Config

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-Config ::= SEQUENCE { |  |  |  |
| dataScramblingIdentityPUSCH | Not present |  |  |
| txConfig | Not Present |  | Short_DCI |
|  | codebook |  |  |
| dmrs-UplinkForPUSCH-MappingTypeA CHOICE { |  |  |  |
| setup | DMRS-UplinkConfig |  |  |
| } |  |  |  |
| dmrs-UplinkForPUSCH-MappingTypeB | Not present |  |  |
| pusch-PowerControl | PUSCH-PowerControl |  |  |
| frequencyHopping | Not present |  |  |
| frequencyHoppingOffsetLists | Not present |  |  |
| resourceAllocation | resourceAllocationType1 |  |  |
| pusch-TimeDomainAllocationList | Not present |  |  |
| pusch-AggregationFactor | Not present |  |  |
| mcs-Table | Not present |  |  |
| mcs-TableTransformPrecoder | Not present |  |  |
| transformPrecoder | enabled |  | TRANSFORM_PRECODER_ENABLED |
|  | Not present | TRANSFORM_PRECODER_DISABLED |  |
| codebookSubset | Not present |  | Short_DCI |
|  | nonCoherent |  |  |
| maxRank | Not present |  | Short_DCI |
|  | 1 |  |  |
|  | 2 |  | 2TX_UL_MIMO |
| rbg-Size | Not present |  |  |
| uci-OnPUSCH CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| betaOffsets CHOICE { |  |  |  |
| semiStatic | BetaOffsets |  |  |
| } |  |  |  |
| scaling | f1 |  |  |
| } |  |  |  |
| } |  |  |  |
| tp-pi2BPSK | Not present |  |  |
| maxRank-v1810 | Not present |  |  |
| sTx-2Panel-r18 | Not present |  |  |
| multipanelSchemeSDM-r18 | Not present |  |  |
| multipanelSchemeSFN-r18 | Not present |  |  |
| codebookTypeUL-r18 | Not present |  |  |
| applyIndicatedTCI-State-r18 | Not present |  |  |
| dynamicTransformPrecoderFieldPresenceDCI-0-1-r18 | Not present |  |  |
| dynamicTransformPrecoderFieldPresenceDCI-0-2-r18 | Not present |  |  |
| pusch-ConfigDCI-0-3-r18 | Not present |  |  |
| pusch-ConfigDCI-0-3-r18 CHOICE { |  |  | DCI_0_3 |
| setup SEQUENCE { |  |  |  |
| resourceAllocationDCI-0-3-r18 | resourceAllocationType1 |  |  |
| rbg-SizeDCI-0-3-r18 | Not present |  |  |
| resourceAllocationType1GranularityDCI-0-3-r18 | Not present | granularity is 1 PRB |  |
| numberOfBitsForRV-DCI-0-3-r18 | 2 |  |  |
| harq-ProcessNumberSizeDCI-0-3-r18 | 4 |  |  |
| uci-OnPUSCH-ListDCI-0-3-r18 CHOICE { |  |  |  |
| setup SEQUENCE (SIZE (1..2)) OF UCI-OnPUSCH { |  |  |  |
| UCI-OnPUSCH[1] SEQUENCE { |  |  |  |
| betaOffsets CHOICE { |  |  |  |
| semiStatic | BetaOffsets |  |  |
| } |  |  |  |
| scaling | f1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| TRANSFORM_PRECODER_ENABLED | Transform precoding is enabled (DFT-s-OFDM UL waveform is configured) |
| 2TX_UL_MIMO | UL-MIMO test cases with 2 Tx antenna ports |
| Short_DCI | Used in test scenarios requiring DCI formats 0-0 and 1-0 on USS |
| DCI_0_3 | Transmission of DCI_0_3 is required |

<End of modified section 1>