3GPP TSG-RAN5 Meeting #104	R5-245730

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 1087 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for adding missing information for PC6 in RF Tx part |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_HST_FR2-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The PC6 is missing in the test case with inner full uplink configuration |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding PC6 in section 6.2.5, 6.3.3.2, 6.4.2.6, 6.5.2.1_1, 6.5.3.2, 6.5.3.2_1,6.5.3.3, 6.5.3.3_1 Adding missing PC6 in section 6.4D.2.2, 6.4D.2.3, 6.4.2.3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | WI cannot complete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.5, 6.3.3.2, 6.4.2.3, 6.4.2.6, 6.5.2.1_1, 6.5.3.2, 6.5.3.2_1,6.5.3.3, 6.5.3.3_1, 6.4D.2.2, 6.4D.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R5-245140r2 |  |  |  |  |  |  |  |  |

<Start of the changes 1>

6.2.5	UE Maximum Output Power – EIRP with UL Gaps

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

-	The test case is incomplete for band n259.

-	Initial conditions are pending analysis for PC1, PC5 and PC7.

-	MU and TT are pending for PC1, PC5, PC6 and PC7.

- 	MU and TT is not finalized for PC3 extreme testing conditions-

<Skip unchanged part>

6.2.5.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in Table 6.2.2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2.5.4.1-1: Test Configuration Table for power class 2,3, 4 and 6

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  |  | Normal, TL, TH |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Low range, Mid Range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Lowest, 100 MHz, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | 120 kHz |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | ChBw (NOTE 2) | SCS | Downlink Configuration | Uplink Configuration |  |  |
| 1 | 100 | Default | - | Modulation |  | RB allocation (NOTE 1) |
|  |  |  |  | DFT-s-OFDM QPSK |  | Inner_Full for PC2, PC3, |
|  |  |  |  |  |  | PC4 and PC6 |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1 for PC2, PC3, PC4, PC6 and PC7 or Table 6.1-2 for PC1. NOTE 2: The 200MHz and 400MHz bandwidths are not applicable to PC7 RedCap UEs |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [10] Annex A, Figure A.3.1.2.1 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [10] subclause 4.4.3.

3.	Downlink signals are initially set up according to TS 38.521-1 [2] Annex C.0, C.1, C.2, and uplink signals according to TS 38.521-1 [2] Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Annex A.2.3-1 with the uplink duty cycle Z set to 20%.

5.	Propagation conditions are set according to TS 38.521-1 [2] Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED  with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [10] clause 4.5. Message contents are defined in clause 6.2.2.4.3.

<End of Change 1>

<Skip unchanged part>

<Start of Change 2>

6.3.3.2	General ON/OFF time mask

Editor’s Note: The following aspects are either missing or not yet determined:

- Measurement Uncertainty and Test Tolerances are FFS for power class 1, 2, 4 and 6.

- Measurement Uncertainty and Test Tolerances are FFS for band n259.

<Skip unchanged part>

6.3.3.2.4	Test description

6.3.3.2.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.2-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in Table 6.3.3.2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.3.3.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  | Normal, TL, TH |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1. |  | Highest |  |
| Test Parameters |  |  |  |
|  | Downlink Configuration | Uplink Configuration |  |
| Test ID | - | Modulation | RB allocation (NOTE 1) |
| 1 |  | DFT-s-OFDM QPSK | Inner_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 for PC2, PC3, PC4, PC6 and PC7 or Table 6.1-2 for PC1. |  |  |  |

1.	Connection between SS and UE is shown in TS 38.508-1 [10] Annex A Figure A.3.3.1.1 for TE diagram and Figure A.3.4.1.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [10] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C, and uplink signals according to Annex G.

4.	The UL Reference Measurement Channels are set according to Table 6.3.3.2.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [10] clause 4.5. Message contents are defined in clause 6.3.3.2.4.3.

<Skip unchanged part>

Table 6.3.3.2.5-5: Relaxation required for OFF power for PC1 and PC3 UEs

| Operating band | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
| n257, n258, n261 | EIRP - 1 dB | EIRP + 2 dB | EIRP + 5 dB | EIRP + 8 dB |
| n260 | EIRP + 2 dB | EIRP + 5 dB | EIRP + 8 dB | EIRP + 11 dB |
| NOTE 1: EIRP is measured value in the ON power sub test, and the unit is dBm. |  |  |  |  |

Table 6.3.3.2.5-6: Relaxation required for OFF power for PC6 UEs

FSS

<End of Change 2>

<Skip unchanged part>

6.4	Transmit signal quality

<Start of Change 3>

6.4.2.6	Phase continuity requirements for DMRS bundling

<Skip unchanged part>

6.4.2.6.4	Test description

6.4.2.6.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of channel bandwidth and sub-carrier spacing, are shown in Table 6.4.2.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.4.2.6.4.1-1: Test Configuration Table for PUSCH

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest |  |
| Test Parameters |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | - | Modulation | RB allocation (NOTE 1) |
| 1 |  | DFT-s-OFDM PI/2 BPSK | Inner_Full for PC2, PC3, PC4 and PC6 Inner_Full_Region1 for PC1 |
| 2 (NOTE 4) |  | DFT-s-OFDM QPSK | Inner_Full for PC2, PC3, PC4 and PC6 Inner_Full_Region1 for PC1 |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 for PC2, PC3, PC4, PC6 and PC7 or Table 6.1-2 for PC1. NOTE 2: Test Channel Bandwidths are checked separately for each NR band, which applicable channel bandwidths are specified in Table 5.3.5-1. NOTE 3: The following test points are not testable for PC3 devices: FR2a channel bandwidth 200MHz: test points 8, 13 and 14 FR2a channel bandwidth 400MHz: test points 7, 8, 11, 12, 13 and 14 FR2b channel bandwidth 50MHz: test points 13 and 14 FR2b channel bandwidth 100MHz: test points 7, 8, 13 and 14 FR2b channel bandwidth 200MHz: test points 7, 8, 13 and 14 FR2b channel bandwidth 400MHz: test points 5, 6, 7, 8, 11, 12, 13 and 14 NOTE 4: This test point shall be skipped if device supports mpr-PowerBoost-FR2-r16 UE capability. |  |  |  |

<End of Change 3>

<Skip unchanged part>

<Start of Change 4>

6.4.2.3	In-band emissions

<Skip unchanged part>

6.4.2.3.5	Test requirement

<Skip unchanged part>

Table 6.4.2.3.5-4: Test requirements for in-band emissions for power class 4

<Skip unchanged part>

Table 6.4.2.3.5-5: FFS

Table 6.4.2.3.5-6: Test requirements for in-band emissions for power class 6

| Parameter description | Unit | Limit (NOTE 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB | max-25 -10.log10NRBLCRB, 20.log10EVM- 5.¦∆RB-1LCRB, -55.1dBm-PRB+TT |  | Any non-allocated (NOTE 2) |
| IQ Image | dB | -25+TT | Output power > 17 dBm | Image frequencies (NOTES 2, 3) |
|  |  | -20+TT | Output power ≤ 17 dBm |  |
| Carrier leakage | dBc | -25+TT | Output power > 7 dBm | Carrier frequency (NOTES 4, 5) |
|  |  | -20+TT | -6 dBm ≤ Output power ≤ 7 dBm |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of (- 25 dB) and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. is defined in NOTE 10. NOTE 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs. For Pi/2 BPSK with Spectrum Shaping, the limit is expressed as a ratio of measured power in one non-allocated RB to the measured power in the allocated RB with highest PSD NOTE 3: The applicable frequencies for this limit are those that are enclosed in the reflection of the allocated bandwidth, based on symmetry with respect to the carrier frequency, but excluding any allocated RBs. NOTE 4: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. NOTE 5: The applicable frequencies for this limit depend on the parameter txDirectCurrentLocation in UplinkTxDirectCurrent IE, and are those that are enclosed in the RBs containing the DC frequency but excluding any allocated RB. NOTE 6: LCRB is the Transmission Bandwidth (see Clause 5.3). NOTE 7: NRB is the Transmission Bandwidth Configuration (see Clause 5.3). NOTE 8: EVM s the limit for the modulation format used in the allocated RBs. NOTE 9: RB is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. RB = 1 or RB = -1 for the first adjacent RB outside of the allocated bandwidth). NOTE 10: is an average of the transmitted power over 10 sub-frames normalized by the number of allocated RBs, measured in dBm. NOTE 11: All powers are EIRP in beam peak direction. NOTE 12: In case the parameter 3300 or 3301 is reported from UE via txDirectCurrentLocation IE, IQ Image and Carrier leakage limit do not apply and General limit applies for all non-allocated frequencies. |  |  |  |  |

<End of Change 4>

<Start of Change 5>

6.4D.2.2	Carrier leakage for UL MIMO

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- OTA test procedure for UL MIMO is still under investigation.

- Test config table is FFS.

- TP analysis is FFS.

- Measurement Uncertainty and Test Tolerances are FFS.

6.4D.2.2.1	Test purpose

The purpose of this test is to exercise the UE transmitter to verify its modulation quality in terms of carrier leakage.

6.4D.2.2.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that support UL MIMO.

6.4D.2.2.3	Minimum conformance requirements

For a UE supporting UL MIMO, the Carrier leakage is an additive sinusoid waveform. The carrier leakage requirement is defined for each component carrier. The measurement interval is one slot in the time domain. The relative carrier leakage power is a power ratio of the additive sinusoid waveform to the power in the modulated waveform.

The requirement is verified with the test metric of Carrier Leakage (Link=TX beam peak direction, Meas=Link angle).

When carrier leakage is contained inside the spectrum confined within the configured UL and DL CCs, the relative carrier leakage power shall not exceed the values specified in Table 6.4D.2.2.3-1 for power class 1 UEs.

Table 6.4D.2.2.3-1: Minimum requirements for relative carrier leakage power for power class 1

| Parameters | Relative Limit (dBc) |
| --- | --- |
| EIRP > 17 dBm | -25 |
| 4 dBm ≤ EIRP ≤ 17 dBm | -20 |

When carrier leakage is contained inside the spectrum occupied by the configured UL CCs and DL CCs, the relative carrier leakage power shall not exceed the values specified in Table 6.4D.2.2.3-2 for power class 2.

Table 6.4D.2.2.3-2: Minimum requirements for relative carrier leakage power for power class 2

| Parameters | Relative Limit (dBc) |
| --- | --- |
| EIRP > 6 dBm | -25 |
| -13 dBm ≤ EIRP ≤ 6 dBm | -20 |

When carrier leakage is contained inside the spectrum occupied by the configured UL CCs and DL CCs, the relative carrier leakage power shall not exceed the values specified in Table 6.4D.2.2.3-3 for power class 3 UEs.

Table 6.4D.2.2.3-3: Minimum requirements for relative carrier leakage power for power class 3

| Parameters | Relative Limit (dBc) |
| --- | --- |
| EIRP > 0 dBm | -25 |
| -13 dBm ≤ EIRP ≤ 0 dBm | -20 |

When carrier leakage is contained inside the spectrum occupied by the configured UL CCs and DL CCs, the relative carrier leakage power shall not exceed the values specified in Table 6.4D.2.2.3-4 for power class 4.

Table 6.4D.2.2.3-4: Minimum requirements for relative carrier leakage power for power class 4

| Parameters | Relative Limit (dBc) |
| --- | --- |
| EIRP > 11 dBm | -25 |
| -13 dBm ≤ EIRP ≤11 dBm | -20 |

The normative reference for this requirement is TS 38.101-2[3] clause 6.4D.2.

Table 6.4D.2.2.3-5:

FFS

When carrier leakage is contained inside the spectrum occupied by the configured UL CCs and DL CCs, the relative carrier leakage power shall not exceed the values specified in Table 6.4D.2.2.3-6 for power class 6.

Table 6.4D.2.2.3-6: Minimum requirements for relative carrier leakage power for power class 6

| Parameters | Relative Limit (dBc) |
| --- | --- |
| EIRP > 7 dBm | -25 |
| -6 dBm ≤ EIRP ≤ 7 dBm | -20 |

The normative reference for this requirement is TS 38.101-2[3] clause 6.4D.2.

<Skip unchanged part>

6.4D.2.2.4.2	Test procedure

1.	Retrieve the LO position from the parameter txDirectCurrentLocation in UplinkTxDirectCurrent IE.

2.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.4D.2.2.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

3.	Set the UE in the Inband Tx beam peak direction found with a 3D EIRP scan as performed in Annex K.1.1. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for the UE Tx beam selection to complete.

4.	Send uplink power control commands to the UE using 1dB power step size to ensure that the UE EIRPTotal = EIRPθ  + EIRPφ measured by the test system is within the Uplink power control window, defined as +MU to +(MU + Uplink power control window size) dB of the target power level Preq, where:

-	Preq is the power level specified in Table 6.4D.2.2.4.2-1 according to the power class.

-	MU is the test system uplink absolute power measurement uncertainty and is specified in Table F.1.2-1 under carrier leakage sub-clause for the carrier frequency f and the channel bandwidth BW.

-	Uplink power control window size = 1dB (UE power step size) + 5 dB (UE power step tolerance) + (Test system uplink relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 38.101-1 [2], Table 6.3.4.3-1 and is 5dB for 1dB power step size, and the Test system uplink relative power measurement uncertainty is specified in Table F.1.2-1.

Allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for the UE Tx beam selection to complete.

5.	SS activates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.2 using condition TxRx.

6.	Measure carrier leakage using Global In-Channel Tx-Test (Annex E) for the - and -polarization at the LO position obtained in step 1. For TDD, only slots consisting of only UL symbols are under test. Calculate CarrLeak = min(CarrLeakθ , CarrLeakφ).

7.	SS deactivates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.3.

NOTE 1:	The BEAM_SELECT_WAIT_TIME default value is defined in Annex K.

NOTE 2:	The purpose of the Uplink power control window is to ensure that the actual UE output power is no less than the target power level, and as close as possible to the target power level. The relationship between the Uplink power control window, the target power level and the corresponding possible actual UE Uplink power window is illustrated in Annex F.4.2.

Table 6.4.2.2.4.2-1: UE EIRP Preq (dBm) for carrier leakage

| Power Class | Preq (dBm) for step 3 |
| --- | --- |
| Power Class 1 | 17 |
| Power Class 2 | 6 |
| Power Class 3 | 0 |
| Power Class 4 | 11 |
| Power Class 6 | 7 |

Table 6.4.2.2.4.2-2: Void

6.4D.2.2.4.3	Message contents

Message contents are according to TS 38.508-1 [10] subclause 4.6 ensuring Table 4.6.3-182 with condition 2TX_UL_MIMO and with TRANSFORM_PRECODER_ENABLED condition in Table 4.6.3-118 PUSCH-Config.

6.4D.2.2.5	Test requirement

For each of the n carrier leakage results derived in Annex E.3.1 for θ- and φ-polarization the minimum is calculated according to

CarrLeak = min(CarrLeakθ , CarrLeakφ), where

.

Each of the n carrier leakage results CarrLeak shall not exceed the values in Table 6.4D.2.2.5-1 to Table 6.4D.2.2.5-4. Allocated RBs are not under test.

Table 6.4D.2.2.5-1a: Test requirements for relative carrier leakage power for power class 1

| Parameter | Relative limit (dBc) |
| --- | --- |
| 17 dBm + MU < EIRP ≤ 17 dBm + MU + Uplink power control window size | -25 + TT |

Table 6.4D.2.2.5-1b: Test Tolerance (carrier leakage for power class 1)

| Test Metric | FR2a | FR2b |
| --- | --- | --- |
| Max device size ≤ 30 cm | TBD | TBD |

Table 6.4D.2.2.5-2a: Test requirements for relative carrier leakage power for power class 2

| Parameter | Relative limit (dBc) |
| --- | --- |
| 6 dBm + MU < EIRP ≤ 6 dBm + MU + Uplink power control window size | -25 + TT |

Table 6.4D.2.2.5-2b: Test Tolerance (carrier leakage for power class 2)

| Test Metric | FR2a | FR2b |
| --- | --- | --- |
| Max device size ≤ 30 cm | TBD | TBD |

Table 6.4D.2.2.5-3a: Test requirements for relative carrier leakage power for power class 3

| Parameter | Relative limit (dBc) |
| --- | --- |
| 0 dBm + MU < EIRP ≤ 0 dBm + MU + Uplink power control window size | -25 + TT |

Table 6.4D.2.2.5-3b: Test Tolerance (carrier leakage for power class 3)

| Test Metric | FR2a | FR2b |
| --- | --- | --- |
| Max device size ≤ 30 cm | TBD | TBD |

Table 6.4D.2.2.5-4a: Test requirements for relative carrier Leakage Power for power class 4

| Parameter | Relative limit (dBc) |
| --- | --- |
| 11 dBm + MU < EIRP ≤ 11 dBm + MU + Uplink power control window size | -25 + TT |

Table 6.4D.2.2.5-4b: Test Tolerance (carrier leakage for power class 4)

| Test Metric | FR2a | FR2b |
| --- | --- | --- |
| Max device size ≤ 30 cm | TBD | TBD |

Table 6.4D.2.2.5-5a

FFS

Table 6.4D.2.2.5-5b

FFS

Table 6.4D.2.2.5-6a: Test requirements for relative carrier Leakage Power for power class 6

| Parameter | Relative limit (dBc) |
| --- | --- |
| 11 dBm + MU < EIRP ≤ 11 dBm + MU + Uplink power control window size | -25 + TT |

Table 6.4D.2.2.5-6b: Test Tolerance (carrier leakage for power class 6)

| Test Metric | FR2a | FR2b |
| --- | --- | --- |
| Max device size ≤ 30 cm | TBD | TBD |

<End of Change 5>

<Start of Change 6>

6.4D.2.3	In-band emissions for UL MIMO

<Skip unchanged part>

6.4D.2.3.3	Minimum conformance requirements

For a UE supporting UL MIMO, the in-band emission is defined as the average across 12 sub-carriers and as a function of the RB offset from the edge of the allocated UL transmission bandwidth. The in-band emission is measured as the ratio of the UE output power in a non–allocated RB to the UE output power in an allocated RB. The IBE requirement does not apply if UE declares support for mpr-PowerBoost-FR2-r16, UL transmission excluding Pi/2 BPSK is such that MPRf,c = 0 and when NS_200 applies, and the network configures the UE to operate with mpr-PowerBoost-FR2-r16.

The basic in-band emissions measurement interval is identical to that of the EVM test.

The requirement is verified with the test metric of In-band emission (Link=TX beam peak direction, Meas=Link angle).

The relative in-band emission shall not exceed the values specified in Table 6.4D.2.3.3-1 for power class 1 UEs.

The average of the in-band emission measurement over 10 sub-frames shall not exceed the values specified in Table 6.4D.2.3.3-1 for power class 1, Table 6.4D.2.3.3-2 for power class 2, Table 6.4D.2.3.3-3 for power class 3 and Table 6.4D.2.3.3-4 for power class 4 UEs.

<Skip unchanged part>

Table 6.4D.2.3.3-5: FFS

FFS

Table 6.4D.2.3.3-6: Requirements for in-band emissions for power class 6

| Parameter description | Unit | Limit (NOTE 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB | max-25 -10.log10NRBLCRB, 20.log10EVM- 5.¦∆RB-1LCRB, -55.1dBm-PRB |  | Any non-allocated (NOTE 2) |
| IQ Image | dB | -25 | Output power > 17 dBm | Image frequencies (NOTES 2, 3) |
|  |  | -20 | Output power ≤ 17 dBm |  |
| Carrier leakage | dBc | -25 | Output power > 7 dBm | Carrier frequency (NOTES 4, 5) |
|  |  | -20 | -6 dBm ≤ Output power ≤ 7 dBm |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of (- 25 dB) and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. is defined in NOTE 10. NOTE 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs. For Pi/2 BPSK with Spectrum Shaping, the limit is expressed as a ratio of measured power in one non-allocated RB to the measured power in the allocated RB with highest PSD NOTE 3: The applicable frequencies for this limit are those that are enclosed in the reflection of the allocated bandwidth, based on symmetry with respect to the carrier frequency, but excluding any allocated RBs. NOTE 4: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. NOTE 5: The applicable frequencies for this limit depend on the parameter txDirectCurrentLocation in UplinkTxDirectCurrent IE, and are those that are enclosed in the RBs containing the DC frequency but excluding any allocated RB. NOTE 6: LCRB is the Transmission Bandwidth (see Clause 5.3). NOTE 7: NRB is the Transmission Bandwidth Configuration (see Clause 5.3). NOTE 8: EVM s the limit for the modulation format used in the allocated RBs. NOTE 9: RB is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. RB = 1 or RB = -1 for the first adjacent RB outside of the allocated bandwidth). NOTE 10: is an average of the transmitted power over 10 sub-frames normalized by the number of allocated RBs, measured in dBm. NOTE 11: All powers are EIRP in beam peak direction. |  |  |  |  |

The normative reference for this requirement is TS 38.101-2 [3] clause 6.4.2.3.

The average of the in-band emission measurement over 10 sub-frames shall not exceed the values specified in Table 6.4D.2.3.3-6 for power class 6 UEs.

<Skip unchanged part>

6.4D.2.3.4.2	Test procedure

Test procedure for PUSCH:

1.1	Retrieve the LO position from the parameter txDirectCurrentLocation in UplinkTxDirectCurrent IE.

1.2	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.4D.2.3.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

1.3	Set the UE in the Inband Tx beam peak direction found with a 3D EIRP scan as performed in Annex K.1.1. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 2) for the UE Tx beam selection to complete.

1.4	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE output power is Preq + PW ± PW, where Preq is the power level specified in Tables 6.4D.2.3.4.2-1 according to the power class with power ID = 1. PW is the power window according to Table 6.4D.2.3.4.2-2 for the carrier frequency f and the channel bandwidth BW. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 2) for the UE Tx beam selection to complete.

1.5	SS activates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.2 using condition TxRx.

1.6	Measure In-band emission IEθ, IEφ using Global In-Channel Tx-Test (Annex E) for the θ- and φ-polarizations, respectively. For TDD, only slots consisting of only UL symbols are under test. Calculate IE = IEθ + IEφ, where the calculation is based on linear power ratios.

1.7	SS deactivates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.3.

1.8	Repeat steps 1.3 through 1.6 until In-band emissions have been measured for all power IDs in Table 6.4D.2.3.4.2-1.

NOTE 1:	When switching to DFT-s-OFDM waveform, as specified in Table 6.4D.2.3.4.1-1, send an NR RRCReconfiguration message according to TS 38.508-1 [10] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

NOTE 2:	The BEAM_SELECT_WAIT_TIME default value is defined in Annex K.

Table 6.4D.2.3.4.2-1: Parameters for In-band emissions

| Power ID | Unit | Level for power class 1 | Level for power class 2 | Level for power class 3 | Level for power class 4 | Level for power class 6 |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | dBm | 27 | 16 | 10 | 21 | 17 |
| 2 | dBm | 17 | 6 | 0 | 11 | 7 |

Table 6.4D.2.3.4.2-2: Power Window (dB) for In-band emissions PUSCH and PUCCH

TBD

Test procedure for PUCCH:

2.1	Retrieve the LO position from the parameter txDirectCurrentLocation in UplinkTxDirectCurrent IE.

2.2	PUCCH is set according to Table 6.4D.2.3.4.1-2. SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 6.4D.2.3.4.1-2. The SS sends downlink MAC padding bits on the DL RMC. The transmission of PDSCH will make the UE send uplink ACK/NACK using PUCCH.

2.3	Set the UE in the Inband Tx beam peak direction found with a 3D EIRP scan as performed in Annex K.1.1. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for the UE Tx beam selection to complete.

2.4	Send the appropriate TPC commands in the uplink scheduling information for PUCCH to the UE until UE output power is Preq + PW ± PW, where Preq is the power level specified in Tables 6.4D.2.3.4.2-1 according to the power class with power ID = 1. PW is the power window according to Table 6.4D.2.3.4.2-2 for the carrier frequency f and the channel bandwidth BW. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for the UE Tx beam selection to complete.

2.5	SS activates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.2 using condition TxRx.

2.6	Measure In-band emission IEθ, IEφ using Global In-Channel Tx-Test (Annex E) for the θ- and φ-polarizations, respectively. Calculate IE = IEθ + IEφ, where the calculation is based on linear power ratios.

2.7	SS deactivates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.3.

2.8	Repeat steps 2.3 through 2.6 until In-band emissions have been measured for all power IDs in Table 6.4D.2.3.4.2-1.

NOTE 1:	The BEAM_SELECT_WAIT_TIME default value is defined in Annex K.

NOTE 2:	When switching to DFT-s-OFDM waveform, as specified in Table 6.4D.2.3.4.1-1, send an NR RRCReconfiguration message according to TS 38.508-1 [10] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.4D.2.3.4.3	Message contents

Message contents are according to TS 38.508-1 [10] subclause 4.6 ensuring Table 4.6.3-182 with condition 2TX_UL_MIMO.

6.4D.2.3.5	Test requirement

For power ID1 and ID2, the averaged in-band emissions result, derived in Annex E.4.3 shall not exceed the corresponding values for IQ Image and Carrier Leakage in Table 6.4D.2.3.5-1 for power class 1 UEs.

<Skip unchanged part>

Table 6.4D.2.3.3-5 FFS

Table 6.4D.2.3.5-6: Requirements for in-band emissions for power class 6

| Parameter description | Unit | Limit (NOTE 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB | max-25 -10.log10NRBLCRB, 20.log10EVM- 5.¦∆RB-1LCRB, -55.1dBm-PRB+TT |  | Any non-allocated (NOTE 2) |
| IQ Image | dB | -25 + TT | Output power > 17 dBm | Image frequencies (NOTES 2, 3) |
|  |  | -20 + TT | Output power ≤ 17 dBm |  |
| Carrier leakage | dBc | -25 + TT | Output power > 7 dBm | Carrier frequency (NOTES 4, 5) |
|  |  | -20 + TT | -6 dBm ≤ Output power ≤ 7 dBm |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of (- 25 dB) and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. is defined in NOTE 10. NOTE 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs. For Pi/2 BPSK with Spectrum Shaping, the limit is expressed as a ratio of measured power in one non-allocated RB to the measured power in the allocated RB with highest PSD NOTE 3: The applicable frequencies for this limit are those that are enclosed in the reflection of the allocated bandwidth, based on symmetry with respect to the carrier frequency, but excluding any allocated RBs. NOTE 4: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. NOTE 5: The applicable frequencies for this limit depend on the parameter txDirectCurrentLocation in UplinkTxDirectCurrent IE, and are those that are enclosed in the RBs containing the DC frequency but excluding any allocated RB. NOTE 6: LCRB is the Transmission Bandwidth (see Clause 5.3). NOTE 7: NRB is the Transmission Bandwidth Configuration (see Clause 5.3). NOTE 8: EVM s the limit for the modulation format used in the allocated RBs. NOTE 9: RB is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. RB = 1 or RB = -1 for the first adjacent RB outside of the allocated bandwidth). NOTE 10: is an average of the transmitted power over 10 sub-frames normalized by the number of allocated RBs, measured in dBm. NOTE 11: All powers are EIRP in beam peak direction. NOTE 12: In case the parameter 3300 or 3301 is reported from UE via txDirectCurrentLocation IE, IQ Image and Carrier leakage limit do not apply and General limit applies for all non-allocated frequencies. |  |  |  |  |

The average of the in-band emission measurement over 10 sub-frames shall not exceed the values specified in Table 6.4D.2.3.5-6 for power class 6 UEs.

<End of Change 6>

<skip unchanged part>

<Start of Change 7>

6.5.2.1_1	Spectrum Emission Mask with Power Boost

Editor’s note: The following aspects are either missing or not yet determined:

- Measurement Uncertainties and Test Tolerances are FFS for power class 1, 2, 4 and 6.

6.5.2.1_1.1	Test purpose

Same as clause 6.5.2.1.1.

6.5.2.1_1.2	Test applicability

This test case applies to all types of NR UE release 16 and forward supporting mpr-PowerBoost-FR2-r16 UE capability.

6.5.2.1_1.3	Minimum conformance requirements

Same as clause 6.5.2.1.3.

6.5.2.1_1.4	Test description

6.5.2.1_1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on NR operating bands specified in Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth and subcarrier spacing, are shown in Table 6.5.2.1_1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5.2.1_1.4.1-1: Test Configuration Table

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Mid Range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | Highest |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |  |
|  |  | Default | - | Modulation |  | RB allocation (NOTE 1) |
| 1 | 50 |  |  | DFT-s-OFDM QPSK |  | Inner_Full for PC2, PC3, PC4 |
| 2 | 100 |  |  |  |  | and PC6 |
| 3 | 200 |  |  |  |  | Inner_Full_Region1 for |
| 4 | 400 |  |  |  |  | PC1 |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1 for PC2, PC3. PC4 and PC6 or Table 6.1-2 for PC1. |  |  |  |  |  |  |

1.	Connection between SS and UE is shown in TS 38.508-1 [10] Annex A, Figure A.3.3.1.1 for TE diagram and clause A.3.4.1.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [10] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C, and uplink signals according to Annex G.

4.	The UL Reference Measurement channels are set according to Table 6.5.2.1_1.4.1-1.

5.	Propagation conditions are set according to Annex B.0

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [10] clause 4.5. Message contents are defined in clause 6.5.2.1_1.4.3

<End of Change 7>

<skip unchanged part>

<Start of Change 8>

6.5.3	Spurious emissions

<skip unchanged part>

6.5.3.1	Transmitter Spurious emissions

<skip unchanged part>

6.5.3.1.4	Test description

6.5.3.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on NR operating bands specified in Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth and subcarrier spacing, are shown in Table 6.5.3.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5.3.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  | Low range, High range (NOTE 2) |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  | Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | 120kHz |  |
| Test Parameters |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | - | Modulation | RB allocation (NOTE 1) |
| 1 (NOTE 4) |  | DFT-s -OFDM QPSK | Inner_Full for PC2, PC3, PC4, PC5 and PC6 Inner_Full_Region1 for PC1 |
| 2 |  | DFT-s -OFDM QPSK | Inner_1RB for PC2, PC3, PC4, PC5 and PC6 Inner_Partial for PC1 (NOTE 3) |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 for PC2, PC3, PC4, PC5 and PC7 or Table 6.1-2 for PC1. NOTE 2: When testing Low range test only in Frequency Range lower than (FUL_low – ΔfOOB) and when testing High range test only in Frequency Range higher than (FUL_high + ΔfOOB). NOTE 3: When testing Low range configure uplink RB to Inner_1RB_Left for PC2, PC3, PC4 and PC5 or Inner_Partial_Left_Region1 for PC1 and when testing High range configure uplink RB to Inner_1RB_Right for PC2, PC3, PC4 and PC5 or Inner_Partial_Right_Region1 for PC1. NOTE 4: This test point shall be skipped if device supports mpr-PowerBoost-FR2-r16 UE capability. |  |  |  |

1.	Connection between SS and UE is shown in TS 38.508-1 [10] Annex A, Figure A.3.3.1.3 for TE diagram and Figure A.3.4.1.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [10] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C, and uplink signals according to Annex G.

4.	The UL Reference Measurement channels are set according to Table 6.5.3.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [10] clause 4.5. Message contents are defined in clause 6.5.3.1.4.3.

<End of Change 8>

<Skip unchanged part>

<Start of Change 9>

6.5.3.1_1	Transmitter Spurious emissions with Power Boost

<Skip unchanged part>

6.5.3.1_1.4	Test description

6.5.3.1_1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on NR operating bands specified in Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth and subcarrier spacing, are shown in Table 6.5.3.1_1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5.3.1_1.4.1-1: Test Configuration Table

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Low Range, High Range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | 120kHz |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |  |
|  |  | Default | - | Modulation |  | RB allocation (NOTE 1) |
| 1 | 50 |  |  | DFT-s-OFDM QPSK |  | Inner_Full for PC2, PC3, PC4 |
| 2 | 100 |  |  |  |  | and PC6 |
| 3 | 200 |  |  |  |  | Inner_Full_Region1 for |
| 4 | 400 |  |  |  |  | PC1 |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1 for PC2, PC3, PC4 and PC6 or Table 6.1-2 for PC1. NOTE 2: When testing Low range test only in Frequency Range lower than (FUL_low – ΔfOOB) and when testing High range test only in Frequency Range higher than (FUL_high + ΔfOOB). |  |  |  |  |  |  |

1.	Connection between SS and UE is shown in TS 38.508-1 [10] Annex A, Figure A.3.3.1.3 for TE diagram and Figure A.3.4.1.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [10] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C, and uplink signals according to Annex G.

4.	The UL Reference Measurement channels are set according to Table 6.5.3.1_1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [10] clause 4.5. Message contents are defined in clause 6.5.3.1_1.4.3.

<Skip unchanged part>

<End of Change 9>

<Start of Change 10>

6.5.3.2	Spurious emission band UE co-existence

<Skip unchanged part>

6.5.3.2.4	Test description

6.5.3.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on NR operating bands specified in Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth and subcarrier spacing, are shown in Table 6.5.3.2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5.3.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  | Low range, High range (NOTE 2) |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  | Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | 120kHz |  |
| Test Parameters |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | - | Modulation | RB allocation (NOTE 1) |
| 1 (NOTE 4) |  | DFT-s-OFDM QPSK | Inner_Full for PC2, PC3, PC4, PC5, PC6 and PC7 Inner_Full_Region1 for PC1 |
| 2 |  | DFT-s-OFDM QPSK | Inner_1RB for PC2, PC3, PC4, PC5, PC6 and PC7 Inner_Partial for PC1 (NOTE 3) |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 for PC2, PC3, PC4, PC5, PC6 and PC7 or Table 6.1-2 for PC1. NOTE 2: When testing Low range test only in Frequency Range lower than (FUL_low – ΔfOOB) and when testing High range test only in Frequency Range higher than (FUL_high + ΔfOOB). NOTE 3: When testing Low range configure uplink RB to Inner_1RB_Left for PC2, PC3, PC4, PC5 and PC6 or Inner_Partial_Left_Region1 for PC1 and when testing High range configure uplink RB to Inner_1RB_Right for PC2, PC3, PC4, PC5 and PC6 or Inner_Partial_Right_Region1 for PC1. NOTE 4: This test point shall be skipped if device supports mpr-PowerBoost-FR2-r16 UE capability. |  |  |  |

1.	Connection between SS and UE is shown in TS 38.508-1 [10] Annex A, Figure A.3.3.1.3 for TE diagram and Figure A.3.4.1.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [10] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C, and uplink signals according to Annex G.

4.	The UL Reference Measurement channels are set according to Table 6.5.3.2.4.1-1

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [10] clause 4.5. Message contents are defined in clause 6.5.3.2.4.3.

<Skip unchanged part>

<End of Change 10>

<Start of Change 11>

6.5.3.2_1	Spurious emission band UE co-existence with Power Boost

<Skip unchanged part>

6.5.3.2_1.4	Test description

6.5.3.2_1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on NR operating bands specified in Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth and subcarrier spacing, are shown in Table 6.5.3.2.1_1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5.3.2.1_1.4.1-1: Test Configuration Table

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Low Range, High Range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | 120kHz |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |  |
|  |  | Default | - | Modulation |  | RB allocation (NOTE 1) |
| 1 | 50 |  |  | DFT-s-OFDM QPSK |  | Inner_Full for PC2, PC3, PC4 |
| 2 | 100 |  |  |  |  | and PC6 |
| 3 | 200 |  |  |  |  | Inner_Full_Region1 for |
| 4 | 400 |  |  |  |  | PC1 |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1 for PC2, PC3, PC4 and PC6 or Table 6.1-2 for PC1. NOTE 2: When testing Low range test only in Frequency Range lower than (FUL_low – ΔfOOB) and when testing High range test only in Frequency Range higher than (FUL_high + ΔfOOB). |  |  |  |  |  |  |

1.	Connection between SS and UE is shown in TS 38.508-1 [10] Annex A, Figure A.3.3.1.3 for TE diagram and Figure A.3.4.1.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [10] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C, and uplink signals according to Annex G.

4.	The UL Reference Measurement channels are set according to Table 6.5.3.2.1_1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [10] clause 4.5. Message contents are defined in clause 6.5.3.2.1_1.4.3.

<Skip unchanged part>

<End of Change 11>

<Start of Change 12>

6.5.3.3	Additional spurious emissions

<Skip unchanged part>

6.5.3.3.4	Test description

6.5.3.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on NR operating bands specified in Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth and subcarrier spacing, are shown in Table 6.5.3.3.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5.3.3.4.1-1: Test Configuration Table for NS_202

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  | Low range, High range (NOTE 2) |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  | Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | 120kHz |  |
| Test Parameters |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | - | Modulation | RB allocation (NOTE 1) |
| 1 (NOTE 5, 6) |  | DFT-s-OFDM QPSK | Inner_Full |
| 2 |  | DFT-s-OFDM QPSK | Inner_1RB_Left for PC2, PC3, PC4, PC5 and PC6 Inner_Partial for PC1 (NOTE 3) |
| 3 (NOTE 4) |  | DFT-s-OFDM 64QAM | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 for PC2, PC3, PC4, PC5 and PC6 or Table 6.1-2 for PC1. NOTE 2: When testing Low range test only in Frequency Range lower than (FUL_low – ΔfOOB) and when testing High range test only in Frequency Range higher than (FUL_high + ΔfOOB). NOTE 3: When testing Low range configure uplink RB to Inner_1RB_Left for PC2, PC3, PC4, PC5 and PC6 or Inner_Partial_Left_Region1 for PC1 and when testing High range configure uplink RB to Inner_1RB_Right for PC2, PC3, PC4, PC5 and PC6 or Inner_Partial_Right_Region1 for PC1. NOTE 4: Test ID only applicable to PC1 NOTE 5: Test ID not applicable to PC1. NOTE 6: This test point shall be skipped if device supports mpr-PowerBoost-FR2-r16 UE capability. |  |  |  |

Table 6.5.3.3.4.1-2: Test Configuration Table for NS_203

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  | Low range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  | Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | 120kHz |  |
| Test Parameters |  |  |  |  |  |
| Test ID | Frequency | Channel Bandwidth | Downlink Configuration | Uplink Configuration |  |
|  |  |  | - | Modulation | RB allocation (NOTE 1) |
| 1 | Default | Default |  | DFT-s-OFDM QPSK | Inner_Full |
| 2 | Default | Default |  | DFT-s-OFDM QPSK | Inner_1RB_Left for PC2, PC3, PC4 and PC6 Inner_Partial_Left_Region1 for PC1 |
| 3 (NOTE 2) | Low range + Channel Bandwidth (NOTE 3) | Default |  | DFT-s-OFDM QPSK | Inner_Partial_Left_Region1 |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 for PC2, PC3, PC4 and PC6 or Table 6.1-2 for PC1. NOTE 2: Test ID only applicable to PC1. NOTE 3: Test frequency for test ID 3 is sepecified in Table 6.2.3.4.1-4. |  |  |  |  |  |

<Skip unchanged part>

<End of Change 12>

<Start of Change 13>

6.5.3.3_1	Additional spurious emissions with Power Boost

<Skip unchanged part>

6.5.3.3_1.4	Test description

6.5.3.3_1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on NR operating bands specified in Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth and subcarrier spacing, are shown in Table 6.5.3.3_1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5.3.3_1.4.1-1: Test Configuration Table

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Low Range, High Range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | 120kHz |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |  |
|  |  | Default | - | Modulation |  | RB allocation (NOTE 1) |
| 1 | 50 |  |  | DFT-s-OFDM QPSK |  | Inner_Full for PC2, PC3, PC4 |
| 2 | 100 |  |  |  |  | and PC6 |
| 3 | 200 |  |  |  |  | Inner_Full_Region1 for |
| 4 | 400 |  |  |  |  | PC1 |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1 for PC2, PC3, PC4 and PC6 or Table 6.1-2 for PC1. NOTE 2: When testing Low range test only in Frequency Range lower than (FUL_low – ΔfOOB) and when testing High range test only in Frequency Range higher than (FUL_high + ΔfOOB). |  |  |  |  |  |  |

<Skip unchanged part>

<End of Change 13>