3GPP TSG-RAN5 Meeting #104	R5-245752

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4538 | rev | 2 | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 9.1.6.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | After Step 1, some UEs may try to release any existing PDU sessions before transmitting the De-registration REQ. This needs to be accommodated. According to current Prose, at Step 13a4 the UE is expected to initiate a De-registration REQ upon USIM removal. A UE may also release any PDU sessions prior to transmitting De-registration upon USIM removal. Current Prose does not accommodate this behaviour. This needs to be accommodated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduced new Step 1Aa1 to handle the PDU Session Release REQ. Also added an EXCEPTION before this indicating that this is optional behaviour. Voided Step 13a4 and introduced new Steps 13a5Aa1 – 13a5Ab1 to accommodate the PDU session release and De-registration behaviour. Also update the Table 9.1.6.1.1.3.3-1 to be applicable to Steps 13a5Aa1a3 and 13a5Ab1. Introduced new Steps 1Aa1 and 8Aa1 to handle the IMS deregistration, and new Steps 1Aa1a1 and 8Aa1a1 to handle the PDU Session Release REQ. Also added an EXCEPTION before Steps 1Aa1 and 8Aa1 indicating that this is optional behaviour for a UE that has previously registered for IMS. Voided Step 13a4 and introduced new Steps 13a5a1 – 13a6 to accommodate the PDU session release and De-registration behaviour. Added an EXCEPTION before Step 13a5a1 indicating that this is optional behaviour for a UE that has previously registered for IMS Also corrected some font and formatting issues. Updated step number in Table 9.1.6.1.1.3.3-1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.6.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Associated with R5s240425 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This document is final version of R5-245664 |  |  |  |  |  |  |  |  |

##### 9.1.6.1.1	UE-initiated de-registration / Switch off / Abnormal / De-registration and 5GMM common procedure collision

9.1.6.1.1.1	Test Purpose (TP)

(1)

with { the UE in 5GMM-REGISTERED state }

ensure that {

when { the UE is switched off }

then { the UE shall send DEREGISTRATION REQUEST message with De-registration type IE indicated to "Switch off" }

}

(2)

with { the UE in 5GMM-DEREGISTERED-INITIATED state }

ensure that {

when { the UE receives a DEREGISTRATION REQUEST message before the UE-initiated de-registration procedure has been completed }

then { the UE ignores the message and shall continue de-registration procedure }

}

(3)

with { the UE in 5GMM-DEREGISTERED-INITIATED state }

ensure that {

when { the UE receives a 5GMM common procedure before the UE-initiated de-registration procedure has been completed }

then { the UE ignores the message and shall continue de-registration procedure }

}

(4)

with { the UE supports remove USIM without power down and in 5GMM-REGISTERED state }

ensure that {

when { the USIM is removed from the UE }

then { the UE shall send DEREGISTRATION REQUEST message with De-registration type IE indicated to "Switch off" }

}

9.1.6.1.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.501, clauses 5.5.2.1, 5.5.2.2.1 and 5.5.2.2.6. Unless otherwise stated these are Rel-15 requirements.

[TS 24.501, clause 5.5.2.1]

The de-registration procedure is used:

a)	by the UE to de-register for 5GS services over 3GPP access when the UE is registered over 3GPP access;;

b)	by the UE to de-register for 5GS services over 3GPP access, non-3GPP access, or both when the UE is registered in the same PLMN over both accesses;

c)	by the network to inform the UE that it is deregistered for 5GS services over 3GPP access when the UE is registered over 3GPP access;

d)	by the network to inform the UE that it is deregistered for 5GS services over 3GPP access, non-3GPP access, or both when the UE is registered in the same PLMN over both accesses; and

e)	by the network to inform the UE to re-register to the network.

The de-registration procedure with appropriate de-registration type shall be invoked by the UE:

a)	if the UE is switched off; and

b)	as part of the eCall inactivity procedure defined in subclause 5.5.3.

The de-registration procedure with appropriate de-registration type shall be invoked by the network:

a)	if the network informs whether the UE should re-register to the network.

The de-registration procedure with appropriate access type shall be invoked by the UE:

a)	if the UE wants to de-register for 5GS services over 3GPP access when the UE is registered over 3GPP access; or

b)	the UE wants to de-register for 5GS services over 3GPP access, non-3GPP access, or both when the UE is registered in the same PLMN over both accesses.

If the de-registration procedure is triggered due to USIM removal, the UE shall indicate "switch off" in the de-registration type IE.

If the de-registration procedure is requested by the UDM for a UE that has an emergency PDU session, the AMF shall not send a DEREGISTRATION REQUEST message to the UE.

If the de-registration procedure for 5GS services is performed, the PDU sessions, if any, for this particular UE are released locally without peer-to-peer signalling between the UE and the network.

The UE is allowed to initiate the de-registration procedure even if the timer T3346 is running.

NOTE:	When the UE has no PDU sessions over non-3GPP access, or the UE moves all the PDU sessions over a non-3GPP access to a 3GPP access, the UE and the AMF need not initiate de-registration over the non-3GPP access.

The AMF shall provide the UE with a non-3GPP de-registration timer.

[TS 24.501, clause 5.5.2.2.1]

The de-registration procedure is initiated by the UE by sending a DEREGISTRATION REQUEST message (see example in figure 5.5.2.2.1). The De-registration type IE included in the message indicates whether the de-registration procedure is due to a "switch off" or not. The access type included in the message indicates whether the de-registration procedure is:

a)	for 5GS services over 3GPP access when the UE is registered over 3GPP access only;

b)	for 5GS services over non-3GPP access when the UE is registered over non-3GPP access only; or

c)	for 5GS services over 3GPP access, non-3GPP access or both 3GPP access and non-3GPP access when the UE is registered in the same PLMN over both accesses.

If the UE has a valid 5G-GUTI, the UE shall populate the 5GS mobile identity IE with the valid 5G-GUTI. If the UE does not have a valid 5G-GUTI, the UE shall populate the 5GS mobile identity IE with its SUCI.

If the UE does not have a valid 5G-GUTI and it does not have a valid SUCI, then the UE shall populate the5GSmobile identity IE with its PEI.

If the de-registration request is not due to switch off and the UE is in the state 5GMM-REGISTERED or 5GMM-REGISTERED-INITIATED, timer T3521 shall be started in the UE after the DEREGISTRATION REQUEST message has been sent. The UE shall enter the state 5GMM-DEREGISTERED-INITIATED.

If the UE is to be switched off, the UE shall try for a period of 5 seconds to send the DEREGISTRATION REQUEST message. During this period, the UE may be switched off as soon as the DEREGISTRATION REQUEST message has been sent.

[TS 24.501, clause 5.5.2.2.6]

…

d)	De-registration procedure collision.

De-registration containing de-registration type "switch off":

-	If the UE receives a DEREGISTRATION REQUEST message before the UE-initiated de-registration procedure has been completed, this message shall be ignored and the UE-initiated de-registration procedure shall continue.

Otherwise:

-	If the UE receives a DEREGISTRATION REQUEST message before the UE-initiated de-registration procedure has been completed, it shall treat the message as specified in subclause 5.5.2.3.2 with the following modification:

-	If the DEREGISTRATION REQUEST message received by the UE contains de-registration type "re-registration required", and the UE-initiated de-registration procedure is with de-registration type "normal de-registration", the UE need not initiate the registration procedure for initial registration.

e)	De-registration and 5GMM common procedure collision.

De-registration containing de-registration type "switch off":

-	If the UE receives a message used in a 5GMM common procedure before the de-registration procedure has been completed, this message shall be ignored and the de-registration procedure shall continue.

Otherwise:

-	If the UE receives a message used in a 5GMM common procedure before the de-registration procedure has been completed, both the 5GMM common procedure and the de-registration procedure shall continue.

9.1.6.1.1.3	Test description

9.1.6.1.1.3.1	Pre-test conditions

System Simulator:

-	NGC Cell A.

UE:

-	None.

Preamble:

-	The UE is in state 3N-A on NGC Cell A according to TS 38.508-1 [4].

9.1.6.1.1.3.2	Test procedure sequence

Table 9.1.6.1.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0 | SS stops sending RLC acknowledgments. | - | - | - | - |
| 1 | Cause switch off | - | - | - | - |
| - | EXCEPTION: Step 1Aa1 below specifies optional behaviour if the UE has previously performed IMS registration | - | - | - | - |
| 1Aa1 | The UE may perform the generic test procedure described in TS 34.229-5 [47] Annex A.11 | - | - | - | - |
| - | EXCEPTION: Step 1Aa1a1 may be performed depending on UE implementation |  |  |  |  |
| 1Aa1a1 | The UE transmits a PDU SESSION RELEASE REQUEST message. | --> | PDU SESSION RELEASE REQUEST | - | - |
| 2 | Check: Does the UE transmit a DEREGISTRATION REQUEST with the De-registration type IE indicating "switch off"? | --> | DEREGISTRATION REQUEST | 1 | P |
| 3 | The SS transmits a DEREGISTRATION REQUEST message. | <-- | DEREGISTRATION REQUEST | - | - |
| 3A | SS resumes sending RLC acknowledgments | - | - | - | - |
| 4 | Check: Does the UE transmit a DEREGISTRATION ACCEPT message within 6 seconds (T3522)? | --> | DEREGISTRATION ACCEPT | 2 | F |
| 5 | The SS releases the RRC connection. | - | - | - | - |
| 6 | Switch on the UE | - | - | - | - |
| 7 | The UE performs Registration procedure as specified in TS 38.508-1 [4] subclause 4.5.2 with 'connected without release'. | - | - | - | - |
| 7A | SS stops sending RLC acknowledgments. | - | - | - | - |
| 8 | Cause switch off. | - | - | - | - |
| - | EXCEPTION: Step 8Aa1 below specifies optional behaviour if the UE has previously performed IMS registration | - | - | - | - |
| 8Aa1 | The UE may perform the generic test procedure described in TS 34.229-5 [47] Annex A.11 | - | - | - | - |
| - | EXCEPTION: Step 8Aa1a1 may be performed depending on UE implementation |  |  |  |  |
| 8Aa1a1 | The UE transmits a PDU SESSION RELEASE REQUEST message. | --> | PDU SESSION RELEASE REQUEST | - | - |
| 9 | The UE transmits a DEREGISTRATION REQUEST with the De-registration type IE indicating "switch off". | --> | DEREGISTRATION REQUEST | - | - |
| 10 | The SS transmits an IDENTITY REQUEST message. | <-- | IDENTITY REQUEST | - | - |
| 10A | SS resumes sending RLC acknowledgments |  |  |  |  |
| 11 | Check: Does the UE transmit an IDENTITY RESPONSE message within 6 seconds (T3570)? | --> | IDENTITY RESPONSE | 3 | F |
| 12 | The SS releases the RRC connection. | - | - | - | - |
| - | EXCEPTION: Steps 13a1 to 13a4 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that take place if the UE supports remove USIM without power down: pc_USIM_Removal = TRUE [29] | - | - | - | - |
| 13a1 | Switch on the UE | - | - | - | - |
| 13a2 | The UE performs Registration procedure as specified in TS 38.508-1 [4] subclause 4.5.2 with 'connected without release'. | - | - | - | - |
| 13a3 | Cause removal of USIM from the UE without powering down. | - | - | - | - |
| 13a4 | Void |  |  |  |  |
| - | EXCEPTION: Step 13a5a1 below specifies optional behaviour if the UE has previously performed IMS registration | - | - | - | - |
| 13a5a1 | The UE may perform the generic test procedure described in TS 34.229-5 [47] Annex A.11 | - | - | - | - |
| - | EXCEPTION: Steps 13a5a1a1 – 13a5a1a2 may be performed depending on UE implementation and repeated for all active PDU Sessions | - | - | - | - |
| 13a5a1a1 | The UE transmits a PDU SESSION RELEASE REQUEST message. | --> | PDU SESSION RELEASE REQUEST | - | - |
| 13a5a1a2 | PDU session release procedure defined in clause 4.9.21 of TS 38.508-1 [4] is performed | - | - | - | - |
| 13a6 | Check: Does the UE transmit a DEREGISTRATION REQUEST with the De-registration type IE indicating "switch off"? | --> | DEREGISTRATION REQUEST | 4 | P |

9.1.6.1.1.3.3	Specific message contents

Table 9.1.6.1.1.3.3-1: DEREGISTRATION REQUEST (Steps 2, 9 and 13a6, Table 9.1.6.1.1.3.2-1)

| Derivation path: TS 38.508-1 [4], table 4.7.1-12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| De-registration type |  |  |  |
| Switch off | ‘1’B |  |  |