3GPP TSG-RAN5 Meeting #104                                                 	R5-245800

# Maastricht, August 19 - 23 2024

Agenda Item:

Source: 	Ericsson

Title: 	Discussion regarding handling of missing test frequencies for CA combinations

Document for:	Discussion and Endorsement

# Introduction

It has been decided that test frequencies for all possible CA CBW combinations, FR1 and FR2, can not be added in some test frequency tables in 38.508-1 (see discussion paper R5-241364). It is still to be solved how to handle the case when a CA CBW combination that is not present in the test frequency tables needs to be verified.

# Discussion

It is described in Ref [1] how to determine the test frequencies for Intra-band non-contiguous CA. The determination of the test frequencies includes:

- Defining the configuration and related input parameters for the specific band and CA combination.

- Determination of test frequencies for the sub-block combination. See Ref [1].

- Determination of SSB, coreset and signalling parameters for both carriers as a possible PCell. This step might adjust the frequencies determined in step 2 slightly. See Ref [2].

This is a quite complex process with many steps/calculations. Even though the process is described in Ref [1] and Ref [2], there is a risk of that different tools/implementations will come up with different test frequencies as a result.

How will the test frequencies, to be used for testing a CA CBW combination, be generated to ensure that all TE venders are using the same test frequencies for a specific CA combination?

Proposal 1:

A RAN5 “tool” will be developed and provided to be used to calculate test frequencies for testing in case of CA combination is not available in the table for test frequencies.

Proposal 1A:

A pilot tool for calculating test frequencies, supporting at least one CA configuration for intra band non-contiguous CA, including UL CA, ( CA_n77(2A) ), shall be implemented in python and presented for the RAN5 RF group.

Proposal 1B:

The tool will be developed in Excel.

Proposal 2:

In case of needed CBW combination is not available in the test frequency table, the table should be updated to include the needed CBW combination.

Proposal 3:

In case of needed CBW combination is not available in the initial test frequency table, a reference to Annex C will refer to how to calculate test frequencies according to Ref [1] and Ref [2], for TE vendors to use for defining test frequencies for any CBW combination.

# Additional information/discussion related to proposal 1A

## Development and maintenance

### Test cases

There would be a need for developing some test cases to verify that calculated values are the same after code updates, for (at least) one already supported CA combination.

### Storage and version handling

There are two GIT servers that can be used for version handling and distribution of the tool.

3GPP Forge, for 3GPP SW-based standards (OpenAPI, …), https://forge.3gpp.org/

ETSI Labs, for SW tools used to generate 3GPP SW-based standards: https://labs.etsi.org/

The “3GPP Forge” seems the suitable server to use.

### Handling updates

How would any updates of the code be handled? There is a need to define how updates of the tool would be reviewed. Should updates be announced/contributed via CRs or should it be done offline? Offline seems as the better option but it needs to be secured that updates are properly reviewed and agreed on.

One option is to keep RAN5 updated by filing a CR/discussion paper describing updates that is done since last meeting.

## Functionality of the tool

To keep the development and maintenance of the tool to a minimum, at least for the first versions of the tool, the functionality should be limited. Some thinking/ideas for limitations listed below:

- Only test frequencies for CA configurations that is not available in 38.508-1 and 38.508-2 will be supported. Since there is only one configuration in 38.508-1 that has a table for test frequencies without having all combinations in the table ( CA_n77(2A) including UL CA ), the first version of the tool would only support this particular CA combination. The work would be more than tenfold to make a tool that would support any CA configuration, or even intra non-contigeuous CA, including UL, for any band.

- A number of parameters, needed for the calculations, would have to be “hard coded” in the tool for a specific CA combination for a specific band. The hard coded parameters would be manually derived from Ran4/Ran5 specifications and added to the code.

- The tool would typically need the following inputs from user:

  - CA configuration

  - SCS for carrier

  - BW combination

- Some tables that contains parameters needed for calculations that are not band specific, CA configurarion specific,  or expected to be updated often are implemented in the code.

Regarding input parameters listed in bullet three, the tool will not necessarily check the input parameters, like whether the carrier BW combination is valid for that specific CA combination and SCS carrier.

# Conclusion

This document describes proposals on to handle the case of test frequencies for intra-band non-contiguous CA, including UL, FR1 and FR2, is not listed in 38.508-1.

Proposal 1: RAN5 to provide a “tool” for test frequency calculations for CA combinations.

Proposal 1A: A pilot tool for CA combinations to be developed in Python and presented for the RAN5 RF                group.

Proposal 1B: The tool will be developed in Excel

Proposal 2: Test frequency tables to be updated “on demand”.

Proposal 3: Test frequencies to be calculated according to Ref [1] and Ref [2] by TE vendors.

# References

[1]		38.508-1 User Equipment (UE) conformance specification; clause C.2.4.3.

[2]		38.508-1 User Equipment (UE) conformance specification; clause C.3.2.