Title:	Discussion on test point selection for Frequency Error test case in FR2

Source:		Rohde & Schwarz

# Introduction

At RAN#78, it was decided that for each NR test case a discussion of test point selection regarding frequency, channel BW, SCS, RB allocation and waveform is required and that the analysis is documented in TR 38.905.

In this contribution the selection of frequency, channel BW, SCS, RB allocation and waveform for the Frequency Error in FR2 is analyzed for SISO and UL MIMO.

# Discussion

First, the test points for the SISO TC 6.4.1 are analyzed. Next, the differences for the UL-MIMO TC 6.4D.1 are considered.

The Frequency Error is a measure for the deviation between the DL frequency provided by the SS and the transmitted frequency by the UE and is defined in section 6.4.1 in TS 38.101-2 [1].

The minimum requirement is specified as follows:

6.4.1	Frequency Error

The UE modulated carrier frequency shall be accurate to within ±0.1 PPM observed over a period of 1 msec compared to the carrier frequency received from the NR gNB.

The frequency error is defined as a directional requirement. The requirement is verified in beam locked mode with the test metric of Frequency (Link=TX beam peak direction, Meas=Link angle).

## Test Environment

For FR2 the testing of extreme conditions is FFS. Therefore, the frequency error in FR2 is tested with normal conditions and an editor’s note is added that extreme condition testing is FFS.

Proposal 1: Define Test Environment as normal. Add an editor’s note that testing of extreme conditions for Frequency Error in FR2 is FFS.

## Test Frequencies

The main factor of Carrier Frequency Error is the frequency difference of the oscillating circuit or PLL in the transmitter. The frequency conversion to make the RF frequency from the reference frequency has small impact to the Carrier Frequency Error.

From the above, testing only 1 test frequency seems to be good to balance the test accuracy and time. From the perspective of test frequency, the most critical situation cannot be defined because the performance of the oscillating circuit has small impact to the conversion of RF frequency. So, it is preferable to test for Mid frequency range.

Proposal 2: Select Mid range as test frequency for NR Frequency Error for FR2.

## Test Channel Bandwidth

From the perspective of carrier frequency error, the test channel bandwidth seems to have no impact. The channel bandwidth is for the modulated signal, so it does not impact to the RF carrier frequency.

The above analysis indicates that testing only 1 test channel bandwidth seems to be enough. Considering the most critical situation, the highest channel bandwidth might be impact to Carrier Frequency Error because it treats the widest frequency.

Proposal 3: Select the highest channel bandwidth to measure Frequency Error.

## Test Subcarrier Spacing

The situation of the test subcarrier spacing is the same of that of the test channel bandwidth. From the perspective of Carrier Frequency Error, the test subcarrier spacing also seems to be no impact to it and then testing only 1 test subcarrier spacing seems to be enough. Considering the most critical situation as it results in the highest spectrum efficiency.

Proposal 4: Select the lowest Subcarrier Spacing to test.

## DL and UL configuration

The frequency error shall be tested with the DL at REFSENS level [1]. In order to avoid tighter test requirements as in the reference sensitivity test case the same UL and DL RB allocation and waveform shall be applied. TS 38.101-1 requires that REFSENS is tested with DFT-s-OFDM as UL waveform.

Proposal 5: Apply DFT-s-OFDM as UL waveform for Frequency Error single layer TC 6.4.1.

Proposal 6: Follow the UL and DL RB allocation of REFSENS TC for Frequency Error in FR2.

## Exceptions for UL MIMO

The minimum requirement is specified as follows:

6.4D.1	Frequency error for UL MIMO

For a UE supporting UL MIMO, the UE basic measurement interval of modulated carrier frequency is 1 UL slot. The mean value of basic measurements of UE modulated carrier frequency at each layer shall be accurate to within ± 0.1 PPM observed over a period of 1ms of cumulated measurement intevals compared to the carrier frequency received from the NR Node B.

CP-OFDM is mandatory for UL-MIMO transmission with two layer data, therefore DFT-s-OFDM is replaced by CP-OFDM for UL-MIMO measurement.

Proposal 7: Apply CP-OFDM as UL waveform for Frequency Error for UL MIMO TC 6.4D.1.

# Conclusion

To sum up, the following updates for the selection of test points is proposed for frequency error in FR2:

Proposal 1: Define Test Environment as normal. Add an editor’s note that testing of extreme conditions for Frequency Error in FR2 is FFS.

Proposal 2: Select Mid range as test frequency for NR Frequency Error for FR2.

Proposal 3: Select the highest channel bandwidth to measure Frequency Error.

Proposal 4: Select the lowest Subcarrier Spacing to test.

Proposal 5: Apply DFT-s-OFDM as UL waveform for Frequency Error single layer TC 6.4.1.

Proposal 6: Follow the UL and DL RB allocation of REFSENS TC for Frequency Error in FR2.

Proposal 7: Apply CP-OFDM as UL waveform for Frequency Error for UL MIMO TC 6.4D.1.

# Number of test points for Frequency Error

| Environmental conditions | Maximum Number of Frequencies | Maximum Number of ChBW | Number SCS | Number of steps (mod and RB) | Maximum Number of Test Steps |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 1 | 1 | 1 |

# References

- TS 38.101-2, V15.2.0, 3GPP TSG RAN4, June 2018

- R5-181929, Comparison of NR UL Signal CCDFs, Intel Deutschland GmbH, 3GPP TSG RAN5, RAN5 5G-NR Adhoc #2, Taoyuan, Taiwan, April 2018