3GPP TSG-RAN5 Meeting #104	R5-245864

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-5 | CR | 0077 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | A-MPR requirements updates related to NTN band n254 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple Benelux B.V. - Belgium |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_lic_bands_BW_R18-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Core requirements related to NTN band n254 have been added for the A-MPR test in RAN4 NTN specifications |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update A-MPR test case related to RAN5 NTN test specifications added for n254 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing test requirements for n254 for NTN A-MPR test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1 adds missing AMPR tables and updates Editor’s notes. Some content already in spec has change mark removed R5-245456r1->R5-245864 |  |  |  |  |  |  |  |  |

### <Start of Changes>

### 6.2.3	UE additional maximum output power reduction

Editor's Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Initial condition and call setup procedure to support NR satellite access are to be updated

- Message exceptions specific to satellite access is to be updated

- Annex F MU/TT is to be updated

#### 6.2.3.1	Test purpose

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR satellite band number of the applicable operating band, the IE field freqBandIndicatorNR and an associated value of additionalSpectrumEmission in the relevant RRC information elements [8].

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2.1.3-1. Unless stated otherwise, the total reduction to UE maximum output power is max(MPR, A-MPR) where MPR is defined in clause 6.2.2. Outer and inner allocation notation used in clause 6.2.3 is defined in clause 6.2.2. In absence of modulation and waveform types the A-MPR applies to all modulation and waveform types.

#### 6.2.3.2	Test applicability

[to be updated]

#### 6.2.3.3	Minimum conformance requirements

##### 6.2.3.3.1	General

Table 6.2.3.3.1-1 specifies the additional requirements with their associated network signalling values and the allowed A-MPR and applicable operating band(s) for each NS value. The mapping of NR satellite band numbers and values of the additionalSpectrumEmission to network signalling labels is specified in Table 6.2.3.3.1-2.

Table 6.2.3.3.1-1: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (clause) | NR satellite Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_01 |  | Table 5.2.2-1 | 5, 10, 15, 20 | Table 5.3.2-1 | N/A |
| NS_24 | 6.5.3.3.13 in 3GPP TS 38.101-1 [5] | n256 | 5, 10, 15, 20 | Table 6.2.3.15-1 in 3GPP TS 38.101-1 [5] | Clause 6.2.3.15 in 3GPP TS 38.101-1 [5]2 |
| NS_02N | 6.5.3.3.3.1 | n255 | 5, 10, 15, 20 |  | N/A |
| NS_100 | 6.5.2.4.2 in 3GPP TS 38.101-1 [5] | n2561 |  |  | Table 6.2.3.1-2 in 3GPP TS 38.101-1 [5] |
| NS_03N | 6.5.3.3.3 | n254 | 5, 10, 15 |  | Clause 6.2.3.2 |
| NS_04N | 6.5.3.3.4 | n254 | 5 |  | Clause 6.2.3.3 |
| NS_05N | 6.5.3.3.4 | n254 | 5, 10, 15 |  | Clause 6.2.3.4 |
| NOTE 1: This NS can be signalled for NR bands that have UTRA services deployed. NOTE 2: A-MPR for the upper 5 MHz of the band is not specified, and therefore shall be used as a guard band. |  |  |  |  |  |

Table 6.2.3.3.1-2: Mapping of network signalling label

| NR satellite band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n256 | NS_01 | NS_24 | NS_100 |  |  |  |  |  |
| n255 | NS_01 | NS_02N |  |  |  |  |  |  |
| n254 | NS_01 | NS_03N | NS_04N | NS_05N |  |  |  |  |
| NOTE: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of 3GPP TS 38.331 [8]. |  |  |  |  |  |  |  |  |

#### 6.2.3.3.2	A-MPR for NS_03N

Table 6.2.3.3.2-1: A-MPR regions for NS_03N

| Channel BW | Carrier Center Frequency | RB_start*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 5MHz | 1612.5 <= fc < 1613.9 | <= 0.36 | <= 0.36 | A1 |
|  |  |  | >= 2.88 | A2 |
|  | 1613.9 <= fc < 1615.7 |  | >= 3.24 | A3 |
| 10MHz | 1615 <= fc < 1620.1 | <= 1.8 | <= 5.04 | A4 |
|  |  | <= 1.8 | > 5.04 | A5 |
|  |  | > 7.2 | > 0 | A6 |
|  |  | > 1.8 | >= 2.88 | A2 |
|  | 1620.1 <= fc < 1621.5 |  | <= 6.48 | A6 |
|  |  | <= 0.36 | <= 0.36 | A1 |
|  | fc = 1621.5 |  | >= 7.2 | A1 |
| 15MHz | all | <= 3.6 | <= 5.04 | A4 |
|  |  | <= 3.6 | > 5.04 | A5 |
|  |  | > 10.44 |  | A6 |
|  |  | > 3.6 | >= 4.32 | A2 |

Table 6.2.3.3.2-2: A-MPR for NS_03N

|  | Modulation | A1 | A2 | A3 | A4 | A5 | A6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK | 2.5 | 3.0 | 1.0 | 4.0 | 6.5 | 1.5 |
|  | QPSK | 2.5 | 4.0 | 2.5 | 6.0 | 7.0 | 2.0 |
|  | 16QAM | 3.0 | 4.5 | 3.0 | 6.5 | 7.5 | 2.5 |
|  | 64QAM | 3.5 | 5 | 3.5 | 7 | 8 | 3 |
|  | 256QAM | 4.5 | 6 | 4.5 | 8 | 9 | 4 |
| CP-OFDM | QPSK | 3.5 | 6.0 | 4.0 | 8.0 | 10.0 | 4.0 |
|  | 16QAM | 3.5 | 6.0 | 4.0 | 8.0 | 10.0 | 4.0 |
|  | 64QAM | 3.5 | 6.0 | 4.0 | 8.0 | 10.0 | 4.0 |
|  | 256QAM | 3.5 | 6.0 | 4.0 | 8.0 | 10.0 | 4.0 |

#### 6.2.3.3.3	A-MPR for NS_04N

Table 6.2.3.3.3-1: A-MPR regions for NS_04N

| Channel BW | Carrier Center Frequency | RB_start*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 5MHz | 1612.5 <= fc < 1613.9 | <= 0.36 | <= 0.36 | A1 |
|  |  |  | >= 2.88 | A2 |
|  | 1613.9 <= fc < 1615.7 |  | >= 3.24 | A3 |

Table 6.2.3.3.3-2: A-MPR for NS_04N

|  | Modulation | A1 | A2 | A3 |
| --- | --- | --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK | 2.5 | 3.0 | 1.0 |
|  | QPSK | 2.5 | 4.0 | 2.5 |
|  | 16QAM | 3.0 | 4.5 | 3.0 |
|  | 64QAM | 3.5 | 5 | 3.5 |
|  | 256QAM | 4.5 | 6 | 4.5 |
| CP-OFDM | QPSK | 3.5 | 6.0 | 4.0 |
|  | 16QAM | 3.5 | 6.0 | 4.0 |
|  | 64QAM | 3.5 | 6.0 | 4.0 |
|  | 256QAM | 3.5 | 6.0 | 4.0 |

#### 6.2.3.3.4	A-MPR for NS_05N

Table 6.2.3.3.4-1: A-MPR regions for NS_05N

| Channel BW | Carrier Center Frequency | RB_start*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 5MHz | 1622.4 < fc <= 1624 | <= 3.6 | > 0.36 | A3 |
|  |  |  | >= 2.88 | A1 |
| 10MHz | 1615 <= fc < 1620.1 | <= 1.8 | <= 5.04 | A4 |
|  |  | <= 1.8 | > 5.04 | A5 |
|  |  | > 7.2 | > 0 | A6 |
|  |  | > 1.8 | >= 2.88 | A2 |
|  | 1620.1 <= fc <= 1621.5 |  | <= 7.2 | A6 |
|  |  | <= 0.36 | <= 0.36 | A1 |
|  |  | > 7.2 | > 0 | A6 |
| 15MHz | all | <= 3.6 | <= 5.04 | A4 |
|  |  | <= 3.6 | > 5.04 | A5 |
|  |  | > 10.44 |  | A6 |
|  |  | > 3.6 | >= 4.32 | A2 |

Table 6.2.3.3.4-2: A-MPR for NS_05N

|  | Modulation | A1 | A2 | A3 | A4 | A5 | A6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK | 1.5 | 5.0 | 1.5 | 6.5 | 6.5 | 2.0 |
|  | QPSK | 1.5 | 5.0 | 1.5 | 6.5 | 7.0 | 2.5 |
|  | 16QAM | 2.0 | 5.5 | 2.0 | 7.0 | 7.5 | 3.0 |
|  | 64QAM | 2.5 | 6 | 2.5 | 7.5 | 8 | 3.5 |
|  | 256QAM | 3.5 | 7 | 3.5 | 8.5 | 9 | 4.5 |
| CP-OFDM | QPSK | 3.0 | 6.5 |  | 8.0 | 10.0 | 4.5 |
|  | 16QAM | 3.0 | 6.5 |  | 8.0 | 10.0 | 4.5 |
|  | 64QAM | 3.0 | 6.5 |  | 8.0 | 10.0 | 4.5 |
|  | 256QAM | 3.0 | 6.5 |  | 8.0 | 10.0 | 4.5 |

### <End of Changes>