3GPP TSG-RAN5 Meeting #104	R5-245867

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-3 | CR | 3050 | rev | 1 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of NGSO configuration for NTN in TC 13.1.1.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Mediatek India Technology Pvt. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_enh_plus_CT1-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As described in IoT NTN enh WP, update the NGSO configuration to TC 13.1.1.3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update NGSO configuration. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | WI will not be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 13.1.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This is the outcome of R5-244344 with 1 revision. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-244344r1: Update detailed description in test applicability. Add editor’s note for rel-18 test condition. |  |  |  |  |  |  |  |  |

<<< START OF CHANGES 1>>>

13.1.1.3	HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage with UE specific DRX

Editor’s Note: The test configuration for rel-18 and onward UE is incomplete. The following aspects are not yet determined:

- Ephemeris in SIB33 for GSO/NGSO is missing in 36.508

13.1.1.3.1	Test purpose

To verify that when the current and target cell operates on the same carrier frequency the Cat-NB1 UE is able to search and measure cells to meet the intra-frequency cell re-selection requirements.

13.1.1.3.2	Test applicability

This test applies to all types of NB-IoT HD-FDD category NB1 UE supporting UE specific DRX and satellite access from release 17 and forward.

Release 17 UEs should be tested under GEO scenario.

Release 18 and onward UEs should be tested under GSO/NGSO scenario based on UE capability.

<<< skip unchanged part>>>

Table 13.1.1.3.4-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GEO/GSO, HD-FDD duplex mode |
| 2 | NGSO, HD-FDD duplex mode |
| Note 1: If UE supports both NGSO and GSO, the test case Config 1 can be skipped if the UE passes test case Config 2. GEO configuration only applies for Rel-17 UEs. GSO configuration is applicable for Rel-18 and onward UEs, when SIB33 is provided to the UE. Note 2: Config 2 is applicable when SIB33 is provided to the UE. |  |

Table 13.1.1.3.4-2: General test parameters for HD-FDD intra frequency cell reselection test case for Cat-NB1 UE in normal coverage

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NB-IOT operational mode |  |  | Standalone |  |
| Satellite information | Config 1 |  | SSC.1 for nCell1 NSC.1 for nCell2 | GSO |
|  | Config 2 |  | SSC.2 for nCell1 NSC.2 for nCell2 | NGSO |
| Initial condition | Active cell |  | nCell1 |  |
|  | Neighbour cells |  | nCell2 |  |
| T2 end condition | Active cell |  | nCell2 |  |
|  | Neighbour cells |  | nCell1 |  |
| Final condition | Visited cell |  | nCell1 |  |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| s-IntraSearchP-v1360 |  |  | 63 (126 dB) | to trigger intra-frequency measurement in this test |
| DRX cycle length |  | s | 1.28 | The value shall be used for all cells in the test. |
| T1 |  | s | >7 | During T1, nCell2 shall be powered off, and during the off time the physical cell identity shall be changed. The intention is to ensure that nCell2 has not been detected by the UE prior to the start of period T2 |
| T2 |  | s | 60 | T2 is defined so that cell re-selection time is taken into account. Once the UE has reselected to nCell2 (within T2) T3 starts |
| T3 |  | s | 15 | T3 is defined so that cell re-selection time is taken into account. |

<<< END OF CHANGES 1>>>