3GPP TSG-RAN WG5 Meeting #104	R5-245938

Maastricht, Netherlands, 19th – 23rd August 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-1 | CR | 0546 | rev | 1 | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update TC 17.2.3 with TT analysis results |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2024-07-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The test tolerance for TC 17.2.3 is not added in and analysis is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Change the bandwidth from 10MHz to 20MHz and 40MHz to 50MHz - Updated the TT analysis related parameters based on the analysis |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The positioning test case 17.2.3 will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 17.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

17.2.3	PRS-RSRPP measurement reporting delay test case for single positioning frequency layer without measurement gap in FR1 SA

17.2.3.1	Test purpose

The purpose of the test is to verify that the PRS-RSRPP measurement meets the reporting delay requirements without measurement gaps specified in TS 38.133 [50] clause 9.9.6.6 in an environment with AWGN propagation conditions.

17.2.3.2	Test applicability

This test applies to all types of NR UE release 17 onwards that supports DL-AoD positioning, maxDL-PRS-FirstPathRSRP-MeasPerTRP, supportedDL-PRS-ProcessingSamples and prs-Processing Window.

17.2.3.3	Minimum conformance requirements

Same as clause 17.2.1.3.

17.2.3.4	Test description

17.2.3.4.1	Initial conditions

The supported test configurations are listed in Table 17.2.3.4-1.

Table 17.2.3.4-1: Test Configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Test Environment: Normal, as defined in TS 38.508-1 [45] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [45] clause 4.3.1.

Channel bandwidth to be tested: are specified in Table 17.2.3.4-1.

1.	Connect the SS and AWGN noise sources to the UE antenna connector or antenna connectors as shown in Annex A, Figure A.13.

2.	The general test parameter settings are set up according to Table 17.2.3.5-1 and Table 17.2.3.5-2.

3.	Propagation conditions are set according to clause 4.16.2.

4.	Message contents are defined in clause 17.2.3.4.3.

5.	In the test there are two synchronous cells: Cell 1 and Cell 2. Cell 1 is the reference as well as the PCell. Cell 2 is the neighbour cell. All cells are on the same RF channel distributed in single positioning frequency layer.

17.2.3.4.2	Test procedure

The test consists of two sub-tests; Sub-test 1 is for Nsample = 4 and sub-test 2 is for Nsample = 1. Sub-test 2 is only conducted when UE also supporting supportedDL-PRS-ProcessingSamples-RRC-CONNECTED.The test consists of two consecutive time intervals, with duration of T1 and T2. During time duration T1, the UE shall not have any timing information of Cell 2. All two cells transmit PRS during T2. The beginning of the time interval T2 shall be aligned with the beginning of the first PRS processing window instance containing the PRS resources.

The NR-DL-AoD-ProvideAssistanceData and NR-DL-AoD-RequestLocationInformation as defined in TS 37.355 [49] clause 6.5.11 shall be provided to the UE during T1. The last TTI containing the two messages shall be provided to the UE T ms before the start of T2, where T = 50 ms is the maximum processing time of the DL-AoD assistance data and location information request.

- Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, according to TS 38.508-1 [45] clause 4.5.

2.	The SS shall send a RESET UE POSITIONING STORED INFORMATION message.

3.	Set the parameters according to Table 17.2.3.5-1 and Table 17.2.3.5-2. Propagation conditions are set according to clause 4.16.2.

4.	T1 starts.

5.	The SS shall transmit an LPP REQUEST CAPABILITIES message.

6.	The UE shall transmit an LPP PROVIDE CAPABILITIES message indicating the DL-AoD capabilities supported by the UE in the NR-DL-AoD-ProvideCapabilities IE.

7.	The SS shall transmit an RRCReconfiguration message with the PPW configuration.

8.	The UE shall transmit RRCReconfigurationComplete message.

9.	The SS shall send a LPP PROVIDE ASSISTANCE DATA message, including the NR-DL-AoD-ProvideAssistanceData IE. The position of neighbour Cell 2 is described in 3GPP TS 37.571-5 [20]. If the UE message at step 6 includes the ackRequested IE set to TRUE, then the SS shall send an acknowledgment in the LPP PROVIDE ASSISTANCE DATA message.

10.	The SS shall send a LPP REQUEST LOCATION INFORMATION message, including the NR-DL-AoD-RequestLocationInformation IE such that the UE receives the message T ms before the start of T2, where T = 50 ms.

11.	The SS sends a Positioning Measurement Gap Activation command containing an ID to activate the associated measurement gap

12.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 17.2.3.5-2.

13.	The UE shall transmit a LPP PROVIDE LOCATION INFORMATION message including the NR-DL-AoD-ProvideLocationInformation IE within the response time (see clause 4.16.3). The UE shall perform and report the PRS-RSRPP measurements for Cell 2 with respect to the reference cell in the DL-TDOA assistance data, Cell 1. If the UE transmits an NR-DL-AoD-ProvideLocationInformation IE including the nr-DL-PRS-FirstPathRSRP-Result-r17 field for Cell 2 within the response time then the number of successful tests is increased by one. If the UE fails to report the NR-DL-AoD-ProvideLocationInformation IE with both the nr-DL-PRS-RSRP-Result-r16 fields included within the response time then the number of failure tests is increased by one.

14.	If the UE message at step 13 includes the ackRequested IE set to TRUE, the SS shall send a LPP acknowledgement message.

15.	Repeat steps 2-14 in Table 17.2.3.4-1 until the confidence level according to Annex D is achieved.

If all (applicable) sub-tests pass, the whole test passes. If one (applicable) sub-test fails, the whole test fails.

17.2.3.4.3	Message contents

Table 17.2.3.4.3-1: RESET UE POSITIONING STORED INFORMATION

| Derivation Path: 38.509 [44] clause 6.6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UE Positioning Technology | 0 0 0 1 0 0 0 | DL-AoD |  |

Table 17.2.3.4.3-2: LPP Request Capabilities

| Information Element | Value/remark |
| --- | --- |
| nr-DL-AoD-RequestCapabilities-r16 | TRUE |

Table 17.2.3.4.3-3: RRCReconfiguration

| Derivation Path: TS 38.508-1 [45], table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | OCTET STRING (CONTAINING CellGroupConfig) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 17.2.3.4.3-4: CellGroupConfig (RRCReconfiguration, Table 17.2.3.4.3-3)

| Derivation Path: 38.508-1 [45], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| spCellConfigDedicated SEQUENCE { |  |  |  |
| initialDownlinkBWP SEQUENCE { |  |  |  |
| dl-PPW-PreConfigToAddModList-r17 SEQUENCE (SIZE (1..maxNrofPPW-Config-r17)) OF DL-PPW-PreConfig-r17 { | 1 entry |  |  |
| DL-PPW-PreConfig-r17[1] SEQUENCE { |  | entry 1 |  |
| dl-PPW-ID-r17 | 0 |  |  |
| dl-PPW-PeriodicityAndStartSlot-r17 CHOICE { | The periodicity is 160ms and the starting slot offset is 11 ms for any SCS configuration |  |  |
| scs120 CHOICE { |  |  |  |
| n1280 | 4 |  |  |
| } |  |  |  |
| } |  |  |  |
| length-r17 | 80 |  |  |
| type-r17 | type1A |  |  |
| priority-r17 | st1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 17.2.3.4.3-5: LPP ProvideAssistanceData

| Derivation Path: 37.355 clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LPP-Message ::= SEQUENCE { |  |  |  |
| transactionID SEQUENCE { |  |  |  |
| initiator | locationServer |  |  |
| transactionNumber | (0..255) |  |  |
| } |  |  |  |
| endTransaction | TRUE |  |  |
| sequenceNumber | Not present |  |  |
| acknowledgement | Not present |  |  |
| lpp-MessageBody CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| provideAssistanceData SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| provideAssistanceData-r9 SEQUENCE { |  |  |  |
| commonIEsProvideAssistanceData | Not present |  |  |
| a-gnss-ProvideAssistanceData | Not present |  |  |
| otdoa-ProvideAssistanceData | Not present |  |  |
| epdu-Provide-AssistanceData | Not present |  |  |
| sensor-ProvideAssistanceData-r15 | Not present |  |  |
| tbs-ProvideAssistanceData-r15 | Not present |  |  |
| wlan-ProvideAssistanceData-r15 | Not present |  |  |
| nr-Multi-RTT-ProvideAssistanceData-r16 | Not present |  |  |
| nr-DL-AoD-ProvideAssistanceData-r16 SEQUENCE { |  |  |  |
| nr-DL-PRS-AssistanceData-r16 | As defined in Table 17.2.3.4.3-6 |  |  |
| nr-SelectedDL-PRS-IndexList-r16 | Not present |  |  |
| nr-PositionCalculationAssistance-r16 SEQUENCE { |  |  | Depending on UE capabilities, i.e. support for UE-based DL-AoD |
| nr-TRP-LocationInfo-r16 | As defined in TS 37.571-5 [20] |  |  |
| nr-DL-PRS-BeamInfo-r16 | Not present |  |  |
| nr-RTD-Info-r16 | Not present |  |  |
| nr-TRP-BeamAntennaInfo-r17 | Not present |  |  |
| nr-DL-PRS-Expected-LOS-NLOS-Assistance-r17 | Not present |  |  |
| nr-DL-PRS-TRP-TEG-Info-r17 | Not present |  |  |
| } |  |  |  |
| nr-DL-AoD-Error-r16 | Not present |  |  |
| nr-DL-PRS-BeamInfo-r17 | Not present |  |  |
| nr-On-Demand-DL-PRS-Configurations-r17 | Not present |  |  |
| } |  |  |  |
| nr-DL-TDOA-ProvideAssistanceData-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 17.2.3.4.3-6: NR-DL-PRS-AssistanceData

| Derivation Path: 37.355 clause 6.4.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NR-DL-PRS-AssistanceData-r16 ::= SEQUENCE { |  |  |  |
| nr-DL-PRS-ReferenceInfo-r16 SEQUENCE { |  |  |  |
| dl-PRS-ID-r16 | 0 |  |  |
| nr-DL-PRS-ResourceID-List-r16 | Not present |  |  |
| nr-DL-PRS-ResourceSetID-r16 | Not present |  |  |
| } |  |  |  |
| nr-DL-PRS-AssistanceDataList-r16 SEQUENCE (SIZE (1..nrMaxFreqLayers-r16)) OF NR-DL-PRS-AssistanceDataPerFreq-r16 { | 1 entry |  |  |
| NR-DL-PRS-AssistanceDataPerFreq-r16[1] SEQUENCE { |  | entry 1 |  |
| nr-DL-PRS-PositioningFrequencyLayer-r16 SEQUENCE { |  |  |  |
| dl-PRS-SubcarrierSpacing-r16 | kHz15 |  |  |
|  | kHz30 |  | Test Configuration 3 |
| dl-PRS-ResourceBandwidth-r16 | 21 | 104 PRBs | Sub-test 1 Config 1 and Config 2 |
|  | 28 | 132 PRBs | Sub-test 1 Config 3 |
|  | 7 | 48 PRBs | Sub-test 2 |
| dl-PRS-StartPRB-r16 | 0 |  |  |
| dl-PRS-PointA-r16 | absoluteFrequencyPointA as defined for the DL frequency of the Cell 1 |  |  |
| dl-PRS-CombSizeN-r16 | n4 |  |  |
| dl-PRS-CyclicPrefix-r16 | normal |  |  |
| } |  |  |  |
| nr-DL-PRS-AssistanceDataPerFreq-r16 SEQUENCE (SIZE (1..nrMaxTRPsPerFreq-r16)) OF NR-DL-PRS-AssistanceDataPerTRP-r16{ | 2 entries |  |  |
| NR-DL-PRS-AssistanceDataPerTRP-r16[1] SEQUENCE { |  | entry 1 | Cell 1 |
| dl-PRS-ID-r16 | 0 |  |  |
| nr-PhysCellID-r16 | Cell 1 |  |  |
| nr-CellGlobalID-r16 | Cell 1 |  |  |
| nr-ARFCN-r16 | Cell 1 |  |  |
| nr-DL-PRS-SFN0-Offset-r16 SEQUENCE { |  |  |  |
| sfn-Offset-r16 | 0 |  |  |
| integerSubframeOffset-r16 | 0 |  |  |
| } |  |  |  |
| nr-DL-PRS-ExpectedRSTD-r16 | 0 |  |  |
| nr-DL-PRS-ExpectedRSTD-Uncertainty-r16 | 0 |  |  |
| nr-DL-PRS-Info-r16 | As specified in Table 17.2.3.4.3-7 |  |  |
| } |  |  |  |
| NR-DL-PRS-AssistanceDataPerTRP-r16[2] SEQUENCE { |  | entry 2 | Cell 2 |
| dl-PRS-ID-r16 | 1 |  |  |
| nr-PhysCellID-r16 | Cell 2 |  |  |
| nr-CellGlobalID-r16 | Cell 2 |  |  |
| nr-ARFCN-r16 | Cell 2 |  |  |
| nr-DL-PRS-SFN0-Offset-r16 SEQUENCE { |  |  |  |
| sfn-Offset-r16 | 0 |  |  |
| integerSubframeOffset-r16 | 0 |  |  |
| } |  |  |  |
| nr-DL-PRS-ExpectedRSTD-r16 | 0 | About 0 s |  |
| nr-DL-PRS-ExpectedRSTD-Uncertainty-r16 | 17 | About 0.58 s |  |
| nr-DL-PRS-Info-r16 | As specified in Table 17.2.3.4.3-7 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 17.2.3.4.3-7: NR-DL-PRS-Info

| Derivation Path: TS 37.355 [49] clause 6.4.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NR-DL-PRS-Info-r16 ::= SEQUENCE { |  |  |  |
| nr-DL-PRS-ResourceSetList-r16 SEQUENCE (SIZE (1..nrMaxSetsPerTrp-r16)) OF NR-DL-PRS-ResourceSet-r16 { | 1 entry |  |  |
| NR-DL-PRS-ResourceSet-r16[1] SEQUENCE { |  | entry 1 |  |
| nr-DL-PRS-ResourceSetID-r16 | 0 |  |  |
| dl-PRS-Periodicity-and-ResourceSetSlotOffset-r16 CHOICE { |  |  |  |
| scs15-r16 CHOICE { |  |  | Config 1 and Config 2 |
| n160-r16 | 10 |  |  |
| } |  |  |  |
| scs30-r16 CHOICE { |  |  | Config 3 |
| n320-r16 | 20 |  |  |
| } |  |  |  |
| dl-PRS-ResourceRepetitionFactor-r16 | Not present |  |  |
| dl-PRS-ResourceTimeGap-r16 | Not present |  |  |
| dl-PRS-NumSymbols-r16 | n4 |  |  |
| dl-PRS-MutingOption1-r16 SEQUENCE { |  |  |  |
| dl-prs-MutingBitRepetitionFactor-r16 | Not present |  |  |
| nr-option1-muting-r16 CHOICE { |  |  |  |
| po2-r16 | 10 |  | Cell 1 |
|  | 01 |  | Cell 2 |
| } |  |  |  |
| } |  |  |  |
| dl-PRS-MutingOption2-r16 | Not present |  |  |
| dl-PRS-ResourcePower-r16 | 27 |  |  |
| dl-PRS-ResourceList-r16 SEQUENCE (SIZE (1..nrMaxResourcesPerSet-r16)) OF NR-DL-PRS-Resource-r16 { | 2 entries |  |  |
| NR-DL-PRS-Resource-r16[1] SEQUENCE { |  | entry 1 |  |
| nr-DL-PRS-ResourceID-r16 | 0 |  |  |
| dl-PRS-SequenceID-r16 | 0 |  |  |
| dl-PRS-CombSizeN-AndReOffset-r16 CHOICE { |  |  |  |
| n4-r16 | 0 |  |  |
| } |  |  |  |
| dl-PRS-ResourceSlotOffset-r16 | 4 |  |  |
| dl-PRS-ResourceSymbolOffset-r16 | 0 |  |  |
| dl-PRS-QCL-Info-r16 | Not present |  |  |
| } |  |  |  |
| NR-DL-PRS-Resource-r16[2] SEQUENCE { |  | entry 2 |  |
| nr-DL-PRS-ResourceID-r16 | 1 |  |  |
| dl-PRS-SequenceID-r16 | 0 |  |  |
| dl-PRS-CombSizeN-AndReOffset-r16 CHOICE { |  |  |  |
| n4-r16 | 0 |  |  |
| dl-PRS-ResourceSlotOffset-r16 | 4 |  |  |
| dl-PRS-ResourceSymbolOffset-r16 | 0 |  |  |
| dl-PRS-QCL-Info-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 17.2.3.4.3-8: LPP RequestLocationInformation

| Derivation Path: 37.355 clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LPP-Message ::= SEQUENCE { |  |  |  |
| transactionID SEQUENCE { |  |  |  |
| initiator | locationServer |  |  |
| transactionNumber | 1 |  |  |
| } |  |  |  |
| endTransaction | FALSE |  |  |
| sequenceNumber | Not present |  |  |
| acknowledgement | Not present |  |  |
| lpp-MessageBody CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| requestLocationInformation SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| requestLocationInformation-r9 SEQUENCE { |  |  |  |
| commonIEsRequestLocationInformation SEQUENCE { |  |  |  |
| locationInformationType | locationMeasurementsRequired |  |  |
| triggeredReporting | Not present |  |  |
| periodicalReporting | Not present |  |  |
| additionalInformation | onlyReturnInformationRequested |  |  |
| qos SEQUENCE { |  |  |  |
| horizontalAccuracy | Not present |  |  |
| verticalCoordinateRequest | FALSE |  |  |
| verticalAccuracy | Not present |  |  |
| responseTime SEQUENCE { |  |  |  |
| time | See 17.2.3.5 | Result of the response time calculation rounded up to the next second if response time <= 128s. Result of the response time calculation rounded up to the next multiple of ten seconds if response time > 128s. |  |
| responseTimeEarlyFix-r12 | Not present |  | Rel-12 onwards |
| unit-r15 | Not present |  |  |
|  | ten-seconds |  | Calculated response time >128s |
| } |  |  |  |
| velocityRequest | FALSE |  |  |
| } |  |  |  |
| environment | Not present |  |  |
| locationCoordinateTypes | Not present |  |  |
| velocityTypes | Not present |  |  |
| } |  |  |  |
| a-gnss-RequestLocationInformation | Not present |  |  |
| otdoa-RequestLocationInformation | Not present |  |  |
| ecid-RequestLocationInformation | Not present |  |  |
| epdu-RequestLocationInformation | Not Present |  |  |
| sensor-RequestLocationInformation-r13 | Not present |  |  |
| tbs-RequestLocationInformation-r13 | Not present |  |  |
| wlan-RequestLocationInformation-r13 | Not present |  |  |
| bt-RequestLocationInformation-r13 | Not present |  |  |
| nr-ECID-RequestLocationInformation-r16 | Not present |  |  |
| nr-Multi-RTT-RequestLocationInformation-r16 | Not present |  |  |
| nr-DL-AoD-RequestLocationInformation-r16 SEQUENCE { |  |  |  |
| nr-AssistanceAvailability-r16 | FALSE |  |  |
| nr-DL-AoD-ReportConfig-r16 SEQUENCE { |  |  |  |
| maxDL-PRS-RSRP-MeasurementsPerTRP-r16 | Not present |  |  |
| maxDL-PRS-RSRP-MeasurementsPerTRP-r17 | Not present |  |  |
| maxDL-PRS-RSRPP-MeasurementsPerTRP-r17 | Present. Set depending on UE capabilities. |  |  |
| nr-los-nlos-IndicatorRequest-r17 | Not present |  |  |
| reducedDL-PRS-ProcessingSamples-r17 | requested |  | Sub-test 2 |
|  | Not present |  | Sub-test 1 |
| lowerRxBeamSweepingFactor-FR2-r17 | Not present |  |  |
| } |  |  |  |
| multiMeasInSameReport-r17 |  |  |  |
| } |  |  |  |
| nr-DL-TDOA-RequestLocationInformation-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 17.2.3.4.3-9: LPP ProvideLocation Information

| Derivation Path: 37.355 clause 6.5.12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LPP-Message ::= SEQUENCE { |  |  |  |
| transactionID SEQUENCE { |  |  |  |
| initiator | locationServer |  |  |
| transactionNumber | 1 |  |  |
| } |  |  |  |
| endTransaction | TRUE |  |  |
| sequenceNumber | (0..255) |  |  |
| acknowledgement |  |  |  |
| lpp-MessageBody CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| provideLocationInformation SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| provideLocationInformation-r9 SEQUENCE { |  |  |  |
| commonIEsProvideLocationInformation | Not present. |  |  |
| a-gnss-ProvideLocationInformation | Not present |  |  |
| otdoa-ProvideLocationInformation | Not present |  |  |
| ecid-ProvideLocationInformation | Not present |  |  |
| epdu-ProvideLocationInformation | Not present |  |  |
| sensor-ProvideLocationInformation-r13 | Not present |  |  |
| tbs-ProvideLocationInformation-r13 | Not present |  |  |
| wlan-ProvideLocationInformation-r13 | Not present |  |  |
| bt-ProvideLocationInformation-r13 | Not present |  |  |
| nr-ECID-ProvideLocationInformation-r16 | Not present |  |  |
| nr-Multi-RTT-ProvideLocationInformation-r16 | Not present |  |  |
| nr-DL-AoD-ProvideLocationInformation-r16 SEQUENCE { |  |  |  |
| nr-DL-AoD-SignalMeasurementInformation-r16 SEQUENCE { |  |  |  |
| nr-DL-AoD-MeasList-r16 SEQUENCE (SIZE(1..nrMaxTRPs-r16)) OF NR-DL-AoD-MeasElement-r16 { |  |  |  |
| NR-DL-AoD-MeasElement-r16[1] SEQUENCE { |  | entry 1 |  |
| dl-PRS-ID-r16 | INTEGER (0..255) |  |  |
| nr-PhysCellID-r16 | Cell 1 |  |  |
| nr-CellGlobalID-r16 |  |  |  |
| nr-ARFCN-r16 |  |  |  |
| nr-DL-PRS-ResourceID-r16 |  |  |  |
| nr-DL-PRS-ResourceSetID-r16 |  |  |  |
| nr-TimeStamp-r16 |  |  |  |
| nr-DL-PRS-RSRP-Result-r16 | Present. Any value acceptable. |  |  |
| nr-DL-PRS-RxBeamIndex-r16 |  |  |  |
| nr-DL-AoD-AdditionalMeasurements-r16 |  |  |  |
| nr-DL-PRS-FirstPathRSRP-Result-r17 | Present. Any value acceptable. |  |  |
| nr-los-nlos-Indicator-r17 |  |  |  |
| nr-DL-AoD-AdditionalMeasurementsExt-r17 |  |  |  |
| } |  |  |  |
| NR-DL-AoD-MeasElement-r16[2] SEQUENCE { |  | entry 2 |  |
| dl-PRS-ID-r16 | INTEGER (0..255) |  |  |
| nr-PhysCellID-r16 | Cell 2 |  |  |
| nr-CellGlobalID-r16 |  |  |  |
| nr-ARFCN-r16 |  |  |  |
| nr-DL-PRS-ResourceID-r16 |  |  |  |
| nr-DL-PRS-ResourceSetID-r16 |  |  |  |
| nr-TimeStamp-r16 |  |  |  |
| nr-DL-PRS-RSRP-Result-r16 | Present. Any value acceptable. |  |  |
| nr-DL-PRS-RxBeamIndex-r16 |  |  |  |
| nr-DL-AoD-AdditionalMeasurements-r16 |  |  |  |
| nr-DL-PRS-FirstPathRSRP-Result-r17 | Present. Any value acceptable. |  |  |
| nr-los-nlos-Indicator-r17 |  |  |  |
| nr-DL-AoD-AdditionalMeasurementsExt-r17 |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| nr-dl-AoD-LocationInformation-r16 |  |  |  |
| nr-DL-AoD-Error-r16 | Not present |  |  |
| nr-DL-AoD-SignalMeasurementInstances-r17 | Not present |  |  |
| nr-DL-AoD-LocationInformationInstances-r17 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

17.2.3.5	Test requirement

Table 17.2.3.5-1 and Table 17.2.3.5-2 define the primary level settings including the test tolerances for the test.

Table 17.2.3.5-1: General test parameters

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Reference cell |  | 1, 2, 3 | Cell 1 | Cell 1 is the PCell and the DL-AoD reference cell in the positioning assistance data. |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell 2 is a neighbour cell in the positioning assistance data. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |
| BWchannel | MHz | 1 | 20: NRB,c = 106 |  |
|  |  | 2 | 20: NRB,c = 106 |  |
|  |  | 3 | 50: NRB,c = 133 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| PPW configuration |  | 1,2,3 | PPW.1 | As defined in TS 38.133 [50] A.3.33 |
| CP length |  | 1, 2, 3 | Normal |  |
| DRX |  | 1, 2, 3 | NA | OFF |
| Time offset between serving and neighbour cells | s | 1, 2, 3 | - set to the UE reported capability for receive time difference threshold if the UE reported value is < 3µs - 3µs otherwise | Synchronous cells |
| Expected RSTD | s | 1, 2, 3 | 0 |  |
| Expected RSTD uncertainty | s | 1, 2, 3 | Same as time offset |  |
| T1 | s | 1, 2, 3 | 2 |  |
| T2 | ms | 1, 2, 3 | 10 | In this test UE is configured to measure single PFL within the configured PPW. |

Table 17.2.3.5-2: Cell specific test parameters T2

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Sub-test 1 | Sub-test 2 | Sub-test 1 | Sub-test 2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | N/A |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  | N/A |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | N/A |  |
| PRS configuration |  | 1 | PRS.1.4 FR1 |  | PRS.1.4 FR1 |  |
|  |  | 2 | PRS.1.4 FR1 |  | PRS.1.4 FR1 |  |
|  |  | 3 | PRS.2.4 FR1 |  | PRS.2.4 FR1 |  |
| PRS muting info |  | 1, 2, 3 | ‘10’ |  | ‘01’ |  |
| Note 2 | dBm/SCS | 1 | -98 | -102.6 | -98 | -102.6 |
|  |  | 2 | -98 | -102.6 | -98 | -102.6 |
|  |  | 3 | -95 | -99.6 | -95 | -99.6 |
| Note 2 | dBm/15 kHz | 1 | -98 | -102.6 | -98 | -102.6 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -2.41 | 0.38 | -12.12 | -5.62 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -2 | 2.98 | -10 | -0.87 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS-RSRP Note 3 | dBm/SCS kHz | 1 | -100 | -99.62 | -108 | -103.47 |
|  |  | 2 | -100 | -99.62 | -108 | -103.47 |
|  |  | 3 | -97 | -96.62 | -105 | -100.47 |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -100 | -100 | -104 | -104 |
|  |  | 2 | -100 | -100 | -104 | -104 |
|  |  | 3 | -97 | -97 | -101 | -101 |
| Io | dBm/19.08 MHz | 1 | -64.57 | -65.75 | -64.57 | -65.75 |
|  | dBm/19.08 MHz | 2 | -64.57 | -65.75 | -64.57 | -65.75 |
|  | dBm/47.88 MHz | 3 | -60.59 | -61.77 | -60.59 | -61.77 |
| Propagation Condition |  | 1, 2, 3 | Two-tap channel defined in 38.101-4 Annex B.2.4, a = 1, τd=0.45 µs and fD=5 Hz |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP/PRS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

The PRS-RSRPP measurement time fulfils the requirements specified in clause 17.2.3.3.

The UE shall perform and report the PRS RSRPP measurements for Cell 2 with respect to the reference cell in the DL-AoD assistance data, Cell 1, within the time duration specified in TS 38.133 [50] clause 9.9.6.6 starting from the beginning of time interval T2

The rate of the correct events for the neighbour cell observed during repeated tests shall be at least 90%, where the reported PRS RSRPP measurement for each correct event shall be within the PRS RSRPP reporting range specified in TS 38.133 Clause 10.1.38, i.e., between PRS RSRPP_0 and PRS RSRPP_126..

The response time is equal to the LPP time IE value plus the test tolerance. The LPP time IE value is derived from the RSTD measurement period plus T, where T = 50ms. The LPP response time calculation procedure could be divided into two situations based on whether the UE support ppw-durationOfPRS-Processing1-r17 or ppw-durationOfPRS-Processing2-r17.The PRS-RSRPP measurement period follows the equation:

TPRS-RSRP, total=i=1LTPRS-RSRP,i+L-1*maxTeffect,i if any of the positioning frequency layers are in Case 1, or

TPRS-RSRP, total=maxTPRS-RSRP,i+Tuncertainty,i, if all the positioning frequency layers are in Case 2,

TPRS-RSRP,i=NRxBeam,i*NPRS,islotN'Lavailable_PRS,iN*Nsample-1*Teffect,i+Tlast

Where NRxBeam,i = 1 NPRS,islot=2, Lavailable_PRS,i=285,Nsample=4 for sub-test 1, Nsample=1 for sub-test 2. N is the parameter ppw-durationOfPRS-ProcessingSymbolsN from TS 37.355 [49], N’ is the parameter ppw-maxNumOfDL-PRS-ResProcessedPerSlot from TS 37.355 [49], Tlast,i = Ti +PPWL and Teffect,i = TiTavailable_PRS,i*Tavailable_PRS,i.

Tavailable_PRS,i= LCMTPRS,i,PPWRPi; where Tprs = 160 ms, and the repetition periodicity PPWRPi=160ms. Therefore, Tavailable_PRS,i=160

Ti depends on the UE parameter ppw-durationOfPRS-ProcessingSymbolsT from TS 37.355 [49]

Finally, it results in the following equation:

1*2N'0.285N*4-1*Teffect+Tlast

Where the remaining parameters depend on the UE capabilities. The LPP time IE ranges between 0.641s and 30.88s. The value of the LPP time IE is rounded up to the next second (if the value is >128s, it should be rounded up to the next multiple of ten seconds). The result is transmitted in the response time IE in the LPP-RequestLocationInformation. The LPP time IE ranges between 1s and 31s.

The test tolerance for the response time is 300ms. Therefore, the response time ranges between 1.3s and 31.3s.

The values of N’, N and Ti and the effect in the response time equation are defined in Table 17.2.3.5-3, Table 17.2.3.5-4 and Table 17.2.3.5-5 for reference.

Table 17.2.3.5-3: value of N’ based on ppw-maxNumOfDL-PRS-ResProcessedPerSlot

| ppw-maxNumOfDL-PRS-ResProcessedPerSlot | 2N' |
| --- | --- |
| n1 | 2 |
| >=n2 | 1 |

Table 17.2.3.5-4: value of N based on ppw-durationOfPRS-ProcessingSymbolsN

| ppw-durationOfPRS-ProcessingSymbolsN | 0.035N |
| --- | --- |
| msDot125 | 3 |
| msDot25 | 2 |
| >=msDot5 | 1 |

Table 17.2.3.5-5: value of Teffect and Tlast based on ppw-durationOfPRS-ProcessingSymbolsT-r17

| ppw-durationOfPRS-ProcessingSymbolsT-r17 | Teffect | Tlast |
| --- | --- | --- |
| ms1 | 160 | 161 |
| ms2 | 160 | 162 |
| ms4 | 160 | 164 |
| ms8 | 160 | 168 |
| ms16 | 160 | 176 |
| ms20 | 160 | 180 |
| ms30 | 160 | 190 |
| ms40 | 160 | 200 |
| ms80 | 160 | 240 |
| ms160 | 160 | 320 |
| ms320 | 320 | 480 |
| ms640 | 640 | 800 |
| ms1280 | 1280 | 1440 |

The test tolerances are defined in clauses C.1.6 and C.2.5.

The rate of successful tests during repeated tests shall be at least 90% with a confidence level of 95%.