3GPP TSG-RAN5 Meeting #104	R5-245951

# Maastricht, Netherlands, 19th – 23rd August 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.522 | CR | 0465 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of applicability of FR2 CA and MIMO test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The status of the following test cases needs to be aligned: - FR2 frequency error for UL MIMO - FR2 NSA additional spurious emissions for UL MIMO The additional information for FR2 SA occupied bandwidth for UL MIMO missing the execution recommendation. The applicability for FR2 NSA occupied bandwidth for UL MIMO is FFS. The following test cases are missing: - 6.2B.3.4_1.1 UE Additional Maximum Output power reduction for inter-band EN-DC including FR2 (2 NR CCs) - 6.2B.3.4_1.2 UE Additional Maximum Output power reduction for inter-band EN-DC including FR2 (3 NR CCs) - 6.2B.3.4_1.3 UE Additional Maximum Output power reduction for inter-band EN-DC including FR2 (4 NR CCs) Clause 6.2B.3.4D is mispositioned. There is a typo in clause of test case UE maximum output power with additional requirements for CA (4UL CA). The title of test case 6.2B.2.4D does not indicate that the test is for UL MIMO. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The applicability for above listed TCs has been updated or added. Clause 6.2B.3.4D has been moved. Typo has been corrected. The title of test case 6.2B.2.4D has been clarified. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test cases will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.2, 4.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.521-2 CR 1079 TS/TR 38.521-2 CR 1080 TS/TR 38.521-3 CR 1815 TS/TR 38.521-3 CR 1817 TS/TR 38.521-3 CR 1818 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | SA test cases update in CR R5-245948 and R5-245984, NSA test cases update in CR R5-245950, R5-244906, and R5-244909 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the final outcome of one revision of R5-244899. |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

### 4.1.2	FR2 standalone conformance test cases

Table 4.1.2-1: Applicability of RF SA FR2 conformance test cases, ref. TS 38.521-2 [2]

| Clause | TC Title | Release | Applicability |  | Tested Bands/CA-Configurations Selection | Branch | Additional Information |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Condition | Comment |  |  |  |
| 6 | Transmitter Characteristics |  |  |  |  |  |  |
| 6.2.1.1 | UE maximum output power - EIRP and TRP | Rel-15 | C006j | Release 15 UEs supporting 5GS FR2 and Release 16 and forward UEs supporting 5GS FR2 and not supporting either CSI-RS or SSB based enhanced Beam Correspondence | D005 | PC1 PC2 (NOTE 1) PC3 PC4 (NOTE 1) | Skip TC 6.2.1.1 if UE supports NSA and TS 38.521-3 TC 6.2B.1.4.1 has been executed. |
| 6.2.1.2 | UE maximum output power - Spherical coverage | Rel-15 | C006j | Release 15 UEs supporting 5GS FR2 and Release 16 and forward UEs supporting 5GS FR2 and not supporting either CSI-RS or SSB based enhanced Beam Correspondence | D005 | PC1 PC2 (NOTE 1) PC3 PC4 (NOTE 1) | Skip TC 6.2.1.2 if UE supports NSA and TS 38.521-3 TC 6.2B.1.4.2 has been executed. |
| 6.2.1.1_1 | UE maximum output power - EIRP and TRP (Rel16 and forward) | Rel-16 | C006k | Release 16 and forward UEs supporting 5GS FR2 and supporting either SSB-based or CSI-RS based enhanced beam correspondence | D005 | PC3 | Skip TC 6.2.1.1_1 if UE supports NSA and TS 38.521-3 TC 6.2B.1.4.1 has been executed. |
| 6.2.1.2_1 | UE maximum output power - Spherical coverage (Rel16 and forward) | Rel-16 | C006k | Release 16 and forward UEs supporting 5GS FR2 and supporting either SSB-based or CSI-RS based enhanced beam correspondence | D005 | PC3 | Skip TC 6.2.1.2_1 if UE supports NSA and TS 38.521-3 TC 6.2B.1.4.2 has been executed. |
| 6.2.2 | UE maximum output power reduction | Rel-15 | C006 | UEs supporting 5GS FR2 | D005 | PC1 PC2 (NOTE 1) PC3 PC4 (NOTE 1) | Skip TC 6.2.2 if UE supports NSA and TS 38.521-3 TC 6.2B.2.4 has been executed. Skip TC 6.2.2 for PC3 UE of Rel-15 and Rel-16 supporting modifiedMPRbehaviour bit 0 or PC3 UE of Rel-17, if TC 6.2.2_1 has been executed. |
| 6.2.2_1 | UE maximum output power reduction enhancements | Rel-15 Rel-16 | C006o | UEs supporting 5GS FR2 and supporting modifiedMPRbehaviour bit 0. | D005 | PC3 | Skip TC 6.2.2_1 if UE supports NSA and TS 38.521-3 TC 6.2B.2.4a has been executed. |
|  |  | Rel-17 | C006 | UEs supporting 5GS FR2 |  |  |  |
| 6.2.3 | UE maximum output power with additional requirements | Rel-15 | C006 | UEs supporting 5GS FR2 | D005 | PC1 (NOTE 1) PC2 (NOTE 1) PC3 PC4 (NOTE 1) PC6 (NOTE 1) | Skip TC 6.2.3 if UE supports NSA and TS 38.521-3 TC 6.2B.3.4 has been executed. |
| 6.2.4_1 | Configured transmitted power with Power Boost | Rel-16 | C006w | UEs supporting 5GS FR2 and mpr-PowerBoost-FR2-r16 | D005 | PC1 (NOTE 1) PC2 (NOTE 1) PC3 PC4 (NOTE 1) | Skip TC 6.2.4_1 if UE supports NSA and TS 38.521-3 TC 6.2B.4.1.4_1 has been executed. |
| 6.2A.1.1.1 | UE maximum output power - EIRP and TRP for CA (2UL CA) | Rel-15 | C053 | UEs supporting 5GS FR2 and CA (2UL CA) | E020 | PC1 (NOTE 1) PC2 (NOTE 1) PC3 PC4 (NOTE 1) | Skip TC 6.2A.1.1.1 if UE supports NSA and TS 38.521-3 TC 6.2B.1.4_1.1.1 has been executed. |
| 6.2A.1.1.2 | UE maximum output power - EIRP and TRP for CA (3UL CA) | Rel-15 | C054 | UEs supporting 5GS FR2 and CA (3UL CA) | E021 | PC1 (NOTE 1) PC2 (NOTE 1) PC3 PC4 (NOTE 1) | Skip TC 6.2A.1.1.2 if UE supports NSA and TS 38.521-3 TC 6.2B.1.4_1.2.1 has been executed. |
| 6.2A.1.1.3 | UE maximum output power - EIRP and TRP for CA (4UL CA) | Rel-15 | C055 | UEs supporting 5GS FR2 and CA (4UL CA) | E022 | PC1 (NOTE 1) PC2 (NOTE 1) PC3 PC4 (NOTE 1) | Skip TC 6.2A.1.1.3 if UE supports NSA and TS 38.521-3 TC 6.2B.1.4_1.3.1 has been executed. |
| 6.2A.1.2.1 | UE maximum output power - Spherical coverage for CA (2UL CA) | Rel-15 | C053 | UEs supporting 5GS FR2 and CA (2UL CA) | E020 | PC1 (NOTE 1) PC2 (NOTE 1) PC3 PC4 (NOTE 1) | Skip TC 6.2A.1.2.1 if UE supports NSA and TS 38.521-3 TC 6.2B.1.4_1.1.2 has been executed. |
| 6.2A.1.2.2 | UE maximum output power - Spherical coverage for CA (3UL CA) | Rel-15 | C054 | UEs supporting 5GS FR2 and CA (3UL CA) | E021 | PC1 (NOTE 1) PC2 (NOTE 1) PC3 PC4 (NOTE 1) | Skip TC 6.2A.1.2.2 if UE supports NSA and TS 38.521-3 TC 6.2B.1.4_1.2.2 has been executed. |
| 6.2A.1.2.3 | UE maximum output power - Spherical coverage for CA (4UL CA) | Rel-15 | C055 | UEs supporting 5GS FR2 and CA (4UL CA) | E022 | PC1 (NOTE 1) PC2 (NOTE 1) PC3 PC4 (NOTE 1) | Skip TC 6.2A.1.2.3 if UE supports NSA and TS 38.521-3 TC 6.2B.1.4_1.3.2 has been executed. |
| 6.2A.2.1 | UE maximum output power reduction for CA (2UL CA) | Rel-15 | C053 | UEs supporting 5GS FR2 and CA (2UL CA) | E020 | PC1 (NOTE 1) PC2 (NOTE 1) PC3 PC4 (NOTE 1) | Skip TC 6.2A.2.1 if UE supports NSA and TS 38.521-3 TC 6.2B.2.4_1.1 has been executed. |
| 6.2A.3.1 | UE maximum output power with additional requirements for CA (2UL CA) | Rel-15 | C053 | UEs supporting 5GS FR2 and CA (2UL CA) | E020 | PC1 PC2 (NOTE 1) PC3 PC4 (NOTE 1) | Skip TC 6.2A.3.1 if UE supports NSA and TS 38.521-3 TC 6.2B.3.4_1.1 has been executed. |
| 6.2A.3.2 | UE maximum output power with additional requirements for CA (3UL CA) | Rel-15 | C054 | UEs supporting 5GS FR2 and CA (3UL CA) | E021 | PC1 PC2 (NOTE 1) PC3 PC4 (NOTE 1) | Skip TC 6.2A.3.2 if UE supports NSA and TS 38.521-3 TC 6.2B.3.4_1.2 has been executed. |
| 6.2A.3.3 | UE maximum output power with additional requirements for CA (4UL CA) | Rel-15 | C055 | UEs supporting 5GS FR2 and CA (4UL CA) | E022 | PC1 PC2 (NOTE 1) PC3 PC4 (NOTE 1) | Skip TC 6.2A.3.3 if UE supports NSA and TS 38.521-3 TC 6.2B.3.4_1.3 has been executed. |
| 6.2D.1.1 | UE maximum output power - EIRP and TRP for UL MIMO | Rel-15 | C060 | UEs supporting 5GS FR2 and UL-MIMO | D023 | PC1 PC2 (NOTE 1) PC3 PC4 (NOTE 1) PC5 PC6 (NOTE 1) | Skip TC 6.2D.1.1 if UE supports NSA and TS 38.521-3 TC 6.2B.1.4D.1 has been executed. |
| 6.2D.1.2 | UE maximum output power - Spherical coverage for UL MIMO | Rel-15 | C060 | UEs supporting 5GS FR2 and UL-MIMO | D023 | PC1 PC2 (NOTE 1) PC3 PC4 (NOTE 1) PC5 (NOTE 1) | Skip TC 6.2D.1.2 if UE supports NSA and TS 38.521-3 TC 6.2B.1.4D.2 has been executed. |
| 6.2D.2 | UE maximum output power reduction for UL MIMO | Rel-15 | C060 | UEs supporting 5GS FR2 and UL-MIMO | D023 | PC1 (NOTE 1) PC2 (NOTE 1) PC3 PC4 (NOTE 1) PC5 (NOTE 1) | Skip TC 6.2D.2 if UE supports NSA and TS 38.521-3 TC 6.2B.2.4D has been executed. |
| 6.2D.3 | UE maximum output power with additional requirements for UL MIMO | Rel-15 | C060 | UEs supporting 5GS FR2 and UL-MIMO | D023 | PC1 (NOTE 1) PC2 (NOTE 1) PC3 PC4 (NOTE 1) PC5 (NOTE 1) | Skip TC 6.2D.3 if UE supports NSA and TS 38.521-3 TC 6.2B.3.4D has been executed. |
| 6.2.5 | UE Maximum Output Power – EIRP with UL Gaps | Rel-17 | FFS | FFS | FFS |  | NOTE 1 |
| 6.3.1 | Minimum output power | Rel-15 | C006 | UEs supporting 5GS FR2 | D005 | PC1 PC2 (NOTE 1) PC3 PC4 (NOTE 1) PC6 (NOTE 1) | Skip TC 6.3.1 if UE supports NSA and TS 38.521-3 TC 6.3B.1.4 has been executed. |
| 6.3.2 | Transmit OFF power | Rel-15 | C006 | UEs supporting 5GS FR2 | D005 |  |  |
| 6.3.3.2 | General ON/OFF time mask | Rel-15 | C006 | UEs supporting 5GS FR2 | D005 |  | Skip TC 6.3.3.2 if UE supports NSA and TS 38.521-3 TC 6.3B.3.4 has been executed. |
| 6.3.3.4 | PRACH time mask | Rel-15 | C006 | UEs supporting 5GS FR2 | D005 |  | NOTE 1 |
| 6.3.4.2 | Absolute power tolerance | Rel-15 | C006 | UEs supporting 5GS FR2 | D005 |  | Skip TC 6.3.4.2 if UE supports NSA and TS 38.521-3 TC 6.3B.8.1.4 has been executed. |
| 6.3.4.3 | Relative power tolerance | Rel-15 | C006 | UEs supporting 5GS FR2 | D005 |  | NOTE 1 |
| 6.3.4.4 | Aggregate power tolerance | Rel-15 | C006 | UEs supporting 5GS FR2 | D005 |  | Skip TC 6.3.4.2 if UE supports NSA and TS 38.521-3 TC 6.3B.8.3.4 has been executed. |
| 6.3A.1.1 | Minimum output power for CA (2UL CA) | Rel-15 | C053 | UEs supporting 5GS FR2 and CA (2UL CA) | E020 | PC1 (NOTE 1) PC2 (NOTE 1) PC3 PC4 (NOTE 1) | Skip TC 6.3A.1.1 if UE supports NSA and TS 38.521-3 TC 6.3B.1.4_1.1 has been executed. |
| 6.3A.1.2 | Minimum output power for CA (3UL CA) | Rel-15 | C054 | UEs supporting 5GS FR2 and CA (3UL CA) | E021 | PC1 (NOTE 1) PC2 (NOTE 1) PC3 PC4 (NOTE 1) | Skip TC 6.3A.1.2 if UE supports NSA and TS 38.521-3 TC 6.3B.1.4_1.2 has been executed. |
| 6.3A.1.3 | Minimum output power for CA (4UL CA) | Rel-15 | C055 | UEs supporting 5GS FR2 and CA (4UL CA) | E022 | PC1 (NOTE 1) PC2 (NOTE 1) PC3 PC4 (NOTE 1) | Skip TC 6.3A.1.3 if UE supports NSA and TS 38.521-3 TC 6.3B.1.4_1.3 has been executed. |
| 6.3A.1.4 | Minimum output power for CA (5UL CA) | Rel-15 | C056 | UEs supporting 5GS FR2 and CA (5UL CA) | E023 | PC1 (NOTE 1) PC2 (NOTE 1) PC3 PC4 (NOTE 1) | Skip TC 6.3A.1.4 if UE supports NSA and TS 38.521-3 TC 6.3B.1.4_1.4 has been executed. |
| 6.3A.1.5 | Minimum output power for CA (6UL CA) | Rel-15 | C057 | UEs supporting 5GS FR2 and CA (6UL CA) | E024 | PC1 (NOTE 1) PC2 (NOTE 1) PC3 PC4 (NOTE 1) | Skip TC 6.3A.1.5 if UE supports NSA and TS 38.521-3 TC 6.3B.1.4_1.5 has been executed. |
| 6.3A.1.6 | Minimum output power for CA (7UL CA) | Rel-15 | C058 | UEs supporting 5GS FR2 and CA (7UL CA) | E025 | PC1 (NOTE 1) PC2 (NOTE 1) PC3 PC4 (NOTE 1) | Skip TC 6.3A.1.6 if UE supports NSA and TS 38.521-3 TC 6.3B.1.4_1.6 has been executed. |
| 6.3A.1.7 | Minimum output power for CA (8UL CA) | Rel-15 | C059 | UEs supporting 5GS FR2 and CA (8UL CA) | E026 | PC1 (NOTE 1) PC2 (NOTE 1) PC3 PC4 (NOTE 1) | Skip TC 6.3A.1.7 if UE supports NSA and TS 38.521-3 TC 6.3B.1.4_1.7 has been executed. |
| 6.3A.2.1 | Void |  |  |  |  |  |  |
| 6.3A.2.2 | Void |  |  |  |  |  |  |
| 6.3A.2.3 | Void |  |  |  |  |  |  |
| 6.3A.3.1.1 | General ON/OFF time mask for CA (2UL CA) | Rel-15 | C053 | UEs supporting 5GS FR2 and CA (2UL CA) | E020 |  | NOTE 1 |
| 6.3A.4.2.1 | Absolute power tolerance for CA (2UL CA) | Rel-15 | C053 | UEs supporting 5GS FR2 and CA (2UL CA) | E020 | PC1 (NOTE 1) PC2 (NOTE 1) PC3 PC4 (NOTE 1) | Skip TC 6.3A.4.2.1 if UE supports NSA and TS 38.521-3 TC 6.3B.8.1.4_1.1 has been executed. |
| 6.3A.4.2.2 | Absolute power tolerance for CA (3UL CA) | Rel-15 | C054 | UEs supporting 5GS FR2 and CA (3UL CA) | E021 | PC1 (NOTE 1) PC2 (NOTE 1) PC3 PC4 (NOTE 1) | Skip TC 6.3A.4.2.2 if UE supports NSA and TS 38.521-3 TC 6.3B.8.1.4_1.2 has been executed. |
| 6.3A.4.2.3 | Absolute power tolerance for CA (4UL CA) | Rel-15 | C055 | UEs supporting 5GS FR2 and CA (4UL CA) | E022 | PC1 (NOTE 1) PC2 (NOTE 1) PC3 PC4 (NOTE 1) | Skip TC 6.3A.4.2.3 if UE supports NSA and TS 38.521-3 TC 6.3B.8.1.4_1.3 has been executed. |
| 6.3A.4.2.4 | Absolute power tolerance for CA (5UL CA) | Rel-15 | C056 | UEs supporting 5GS FR2 and CA (5UL CA) | E023 |  | NOTE 1 |
| 6.3A.4.2.5 | Absolute power tolerance for CA (6UL CA) | Rel-15 | C057 | UEs supporting 5GS FR2 and CA (6UL CA) | E024 |  | NOTE 1 |
| 6.3A.4.2.6 | Absolute power tolerance for CA (7UL CA) | Rel-15 | C058 | UEs supporting 5GS FR2 and CA (7UL CA) | E025 |  | NOTE 1 |
| 6.3A.4.2.7 | Absolute power tolerance for CA (8UL CA) | Rel-15 | C059 | UEs supporting 5GS FR2 and CA (8UL CA) | E026 |  | NOTE 1 |
| 6.3A.4.4.1 | Aggregate power tolerance for CA (2UL CA) | Rel-15 | C053 | UEs supporting 5GS FR2 and CA (2UL CA) | E020 | PC1 (NOTE 1) PC2 (NOTE 1) PC3 PC4 (NOTE 1) | Skip TC 6.3A.4.4.1 if UE supports NSA and TS 38.521-3 TC 6.3B.8.3.4_1.1 has been executed. |
| 6.3A.4.4.2 | Aggregate power tolerance for CA (3UL CA) | Rel-15 | C054 | UEs supporting 5GS FR2 and CA (3UL CA) | E021 | PC1 (NOTE 1) PC2 (NOTE 1) PC3 PC4 (NOTE 1) | Skip TC 6.3A.4.4.2 if UE supports NSA and TS 38.521-3 TC 6.3B.8.3.4_1.2 has been executed. |
| 6.3A.4.4.3 | Aggregate power tolerance for CA (4UL CA) | Rel-15 | C055 | UEs supporting 5GS FR2 and CA (4UL CA) | E022 | PC1 (NOTE 1) PC2 (NOTE 1) PC3 PC4 (NOTE 1) | Skip TC 6.3A.4.4.3 if UE supports NSA and TS 38.521-3 TC 6.3B.8.3.4_1.3 has been executed. |
| 6.3A.4.4.4 | Aggregate power tolerance for CA (5UL CA) | Rel-15 | C056 | UEs supporting 5GS FR2 and CA (5UL CA) | E023 |  | NOTE 1 |
| 6.3A.4.4.5 | Aggregate power tolerance for CA (6UL CA) | Rel-15 | C057 | UEs supporting 5GS FR2 and CA (6UL CA) | E024 |  | NOTE 1 |
| 6.3A.4.4.6 | Aggregate power tolerance for CA (7UL CA) | Rel-15 | C058 | UEs supporting 5GS FR2 and CA (7UL CA) | E025 |  | NOTE 1 |
| 6.3A.4.4.7 | Aggregate power tolerance for CA (8UL CA) | Rel-15 | C059 | UEs supporting 5GS FR2 and CA (8UL CA) | E026 |  | NOTE 1 |
| 6.3D.1 | Minimum output power for UL MIMO | Rel-15 | C060 | UEs supporting 5GS FR2 and UL MIMO | D023 | PC1 (NOTE 1) PC2 (NOTE 1) PC3 PC4 (NOTE 1) PC5 | Skip TC 6.3D.1 if UE supports NSA and TS 38.521-3 TC 6.3B.1.4D has been executed. |
| 6.3D.2 | Transmit OFF power for UL MIMO | Rel-15 | C060 | UEs supporting 5GS FR2 and UL MIMO | D023 | PC1 PC2 (NOTE 1) PC3 PC4 (NOTE 1) PC5 (NOTE 1) | Skip TC 6.3D.2 if UE supports NSA and TS 38.521-3 TC 6.3B.2.4D has been executed. |
| 6.3D.3.1 | General ON/OFF time mask for UL MIMO | Rel-15 | C060 | UEs supporting 5GS FR2 and UL MIMO | D023 |  | NOTE 1 |
| 6.3D.3.4 | Void |  |  |  |  |  |  |
| 6.4.1 | Frequency error | Rel-15 | C006 | UEs supporting 5GS FR2 | D005 |  | Skip TC 6.4.1 if UE supports NSA and TS 38.521-3 TC 6.4B.1.4 has been executed. |
| 6.4.2.1 | Error vector magnitude | Rel-15 | C006 | UEs supporting 5GS FR2 | D005 | PC1 PC2 PC3 PC4 | NOTE 1 Skip TC 6.4.2.1 if UE supports NSA and TS 38.521-3 TC 6.4B.2.4.1 has been executed. |
| 6.4.2.1_1 | Error vector magnitude with Power Boost | Rel-16 | C006w | UEs supporting 5GS FR2 and supporting mpr-PowerBoost-FR2-r16 | D005 | PC1 PC2 PC3 PC4 | NOTE 1 Skip TC 6.4.2.1_1 if UE supports NSA and TS 38.521-3 TC 6.4B.2.4.1a has been executed. |
| 6.4.2.2 | Carrier leakage | Rel-15 | C006 | UEs supporting 5GS FR2 | D005 | PC1 (NOTE 1) PC2 (NOTE 1) PC3 PC4 (NOTE 1) PC6 (NOTE 1) | Skip TC 6.4.2.2 if UE supports NSA and TS 38.521-3 TC 6.4B.2.4.2 has been executed. |
| 6.4.2.3 | In-band emissions | Rel-15 | C006 | UEs supporting 5GS FR2 | D005 | PC1 PC2 PC3 PC4 PC6 | NOTE 1 Skip TC 6.4.2.3 if UE supports NSA and TS 38.521-3 TC 6.4B.2.4.3 has been executed. |
| 6.4.2.4 | EVM equalizer spectrum flatness | Rel-15 | C006 | UEs supporting 5GS FR2 | D005 |  | NOTE 1 Skip TC 6.4.2.4 if UE supports NSA and TS 38.521-3 TC 6.4B.2.4.4 has been executed. |
| 6.4.2.5 | EVM spectral flatness for pi/2 BPSK modulation | Rel-15 | C006b | UEs supporting 5GS FR2 and pi/2 BPSK modulation | D005 |  | NOTE 1 Skip TC 6.4.2.5 if UE supports NSA and TS 38.521-3 TC 6.4B.2.4.5 has been executed. |
| 6.4.2.6 | Phase continuity requirements for DMRS bundling | Rel-17 | C314 | Release 17 and forward UEs supporting 5GS FR2, dmrs-BundlingPUCCH-Rep-r17 and either dmrs-BundlingPUSCH-multiSlot-r17 or dmrs-BundlingPUSCH-RepTypeA-r17 or dmrs-BundlingPUSCH-RepTypeB-r17 | D005 |  | NOTE 1 |
| 6.4A.1.1 | Frequency error for CA (2UL CA) | Rel-15 | C053 | UEs supporting 5GS FR2 and CA (2UL CA) | E020 |  | NOTE 1 |
| 6.4A.1.2 | Frequency error for CA (3UL CA) | Rel-15 | C054 | UEs supporting 5GS FR2 and CA (3UL CA) | E021 |  | NOTE 1 |
| 6.4A.1.3 | Frequency error for CA (4UL CA) | Rel-15 | C055 | UEs supporting 5GS FR2 and CA (4UL CA) | E022 |  | NOTE 1 |
| 6.4A.2.1.1 | Error vector magnitude for CA (2UL CA) | Rel-15 | C053 | UEs supporting 5GS FR2 and CA (2UL CA) | E020 | PC1 PC2 PC3 PC4 | NOTE 1 |
| 6.4A.2.1.2 | Error vector magnitude for CA (3UL CA) | Rel-15 | C054 | UEs supporting 5GS FR2 and CA (3UL CA) | E021 | PC1 PC2 PC3 PC4 | NOTE 1 |
| 6.4A.2.1.3 | Error vector magnitude for CA (4UL CA) | Rel-15 | C055 | UEs supporting 5GS FR2 and CA (4UL CA) | E022 | PC1 PC2 PC3 PC4 | NOTE 1 |
| 6.4A.2.1.4 | Error vector magnitude for CA (5UL CA) | Rel-15 | C056 | UEs supporting 5GS FR2 and CA (5UL CA) | E023 | PC1 PC2 PC3 PC4 | NOTE 1 |
| 6.4A.2.1.5 | Error vector magnitude for CA (6UL CA) | Rel-15 | C057 | UEs supporting 5GS FR2 and CA (6UL CA) | E024 | PC1 PC2 PC3 PC4 | NOTE 1 |
| 6.4A.2.1.6 | Error vector magnitude for CA (7UL CA) | Rel-15 | C058 | UEs supporting 5GS FR2 and CA (7UL CA) | E025 | PC1 PC2 PC3 PC4 | NOTE 1 |
| 6.4A.2.1.7 | Error vector magnitude for CA (8UL CA) | Rel-15 | C059 | UEs supporting 5GS FR2 and CA (8UL CA) | E026 | PC1 PC2 PC3 PC4 | NOTE 1 |
| 6.4A.2.2.1 | Carrier leakage for CA (2UL CA) | Rel-15 | C053 | UEs supporting 5GS FR2 and CA (2UL CA) | E020 | PC1 PC2 PC3 PC4 | NOTE 1 |
| 6.4A.2.2.2 | Carrier leakage for CA (3UL CA) | Rel-15 | C054 | UEs supporting 5GS FR2 and CA (3UL CA) | E021 | PC1 PC2 PC3 PC4 | NOTE 1 |
| 6.4A.2.2.3 | Carrier leakage for CA (4UL CA) | Rel-15 | C055 | UEs supporting 5GS FR2 and CA (4UL CA) | E022 | PC1 PC2 PC3 PC4 | NOTE 1 |
| 6.4A.2.2.4 | Carrier leakage for CA (5UL CA) | Rel-15 | C056 | UEs supporting 5GS FR2 and CA (5UL CA) | E023 | PC1 PC2 PC3 PC4 | NOTE 1 |
| 6.4A.2.2.5 | Carrier leakage for CA (6UL CA) | Rel-15 | C057 | UEs supporting 5GS FR2 and CA (6UL CA) | E024 | PC1 PC2 PC3 PC4 | NOTE 1 |
| 6.4A.2.2.6 | Carrier leakage for CA (7UL CA) | Rel-15 | C058 | UEs supporting 5GS FR2 and CA (7UL CA) | E025 | PC1 PC2 PC3 PC4 | NOTE 1 |
| 6.4A.2.2.7 | Carrier leakage for CA (8UL CA) | Rel-15 | C059 | UEs supporting 5GS FR2 and CA (8UL CA) | E026 | PC1 PC2 PC3 PC4 | NOTE 1 |
| 6.4A.2.3.1 | In-band emissions for CA (2UL CA) | Rel-15 | C053 | UEs supporting 5GS FR2 CA (2UL CA) | E020 | PC1 PC2 PC3 PC4 | NOTE 1 |
| 6.4A.2.3.2 | In-band emissions for CA (3UL CA) | Rel-15 | C054 | UEs supporting 5GS FR2 and CA (3UL CA) | E021 | PC1 PC2 PC3 PC4 | NOTE 1 |
| 6.4A.2.3.3 | In-band emissions for CA (4UL CA) | Rel-15 | C055 | UEs supporting 5GS FR2 and CA (4UL CA) | E022 | PC1 PC2 PC3 PC4 | NOTE 1 |
| 6.4A.2.3.4 | In-band emissions for CA (5UL CA) | Rel-15 | C056 | UEs supporting 5GS FR2 and CA (5UL CA) | E023 | PC1 PC2 PC3 PC4 | NOTE 1 |
| 6.4A.2.3.5 | In-band emissions for CA (6UL CA) | Rel-15 | C057 | UEs supporting 5GS FR2 and CA (6UL CA) | E024 | PC1 PC2 PC3 PC4 | NOTE 1 |
| 6.4A.2.3.6 | In-band emissions for CA (7UL CA) | Rel-15 | C058 | UEs supporting 5GS FR2 and CA (7UL CA) | E025 | PC1 PC2 PC3 PC4 | NOTE 1 |
| 6.4A.2.3.7 | In-band emissions for CA (8UL CA) | Rel-15 | C059 | UEs supporting 5GS FR2 and CA (8UL CA) | E026 | PC1 PC2 PC3 PC4 | NOTE 1 |
| 6.4D.1 | Frequency error for UL MIMO | Rel-15 | C060 | UEs supporting 5GS FR2 and UL MIMO | D023 |  | Skip TC 6.4D.1 if UE supports NSA and TS 38.521-3 6.4B.1.4D has been executed. |
| 6.4D.3 | Time alignment error for UL MIMO | Rel-15 | C060 | UEs supporting 5GS FR2 and UL MIMO | D023 |  | NOTE 1 |
| 6.5.1 | Occupied bandwidth | Rel-15 | C006 | UEs supporting 5GS FR2 | D005 |  | Skip TC 6.5.1 if UE supports NSA and TS 38.521-3 TC 6.5B.1.4 has been executed. |
| 6.5.2.1 | Spectrum Emission Mask | Rel-15 | C006 | UEs supporting 5GS FR2 | D005 |  | Skip TC 6.5.2.1 if UE supports NSA and TS 38.521-3 TC 6.5B.2.4.1 has been executed. |
| 6.5.2.1_1 | Spectrum Emission Mask with Power Boost | Rel-16 | C006w | UEs supporting 5GS FR2 and supporting mpr-PowerBoost-FR2-r16 | D005 | PC1 (NOTE 1) PC2 (NOTE 1) PC3 PC4 (NOTE 1) | Skip TC 6.5.2.1_1 if UE supports NSA and TS 38.521-3 TC 6.5B.2.4.1a has been executed. |
| 6.5.2.3 | Adjacent channel leakage ratio | Rel-15 | C006 | UEs supporting 5GS FR2 | D005 |  | Skip TC 6.5.2.3 if UE supports NSA and TS 38.521-3 TC 6.5B.2.4.3 has been executed. |
| 6.5.3.1 | Transmitter Spurious emissions | Rel-15 | C006 | UEs supporting 5GS FR2 | D005 | PC1 PC2 (NOTE 1) PC3 PC4 (NOTE 1) | Skip TC 6.5.3.1 if UE supports NSA and TS 38.521-3 TC 6.5B.3.4.1 has been executed. |
| 6.5.3.1_1 | Transmitter Spurious emissions with Power Boost | Rel-16 | C006w | UEs supporting 5GS FR2 and supporting mpr-PowerBoost-FR2-r16 | D005 | PC1 (NOTE 1) PC2 (NOTE 1) PC3 PC4 (NOTE 1) | Skip TC 6.5.3.1_1 if UE supports NSA and TS 38.521-3 TC 6.5B.3.4.1a has been executed. |
| 6.5.3.2 | Spurious emission band UE co-existence | Rel-15 | C006 | UEs supporting 5GS FR2 | D005 | PC1 PC2 (NOTE 1) PC3 PC4 (NOTE 1) | Skip TC 6.5.3.2 if UE supports NSA and TS 38.521-3 TC 6.5B.3.4.2 has been executed. |
| 6.5.3.2_1 | Spurious emission band UE co-existence with Power Boost | Rel-16 | C006w | UEs supporting 5GS FR2 and supporting mpr-PowerBoost-FR2-r16 | D005 | PC1 (NOTE 1) PC2 (NOTE 1) PC3 PC4 (NOTE 1) | Skip TC 6.5.3.2_1 if UE supports NSA and TS 38.521-3 TC 6.5B.3.4.2a has been executed. |
| 6.5.3.3 | Additional spurious emissions | Rel-15 | C006 | UEs supporting 5GS FR2 | D005 | PC1 (NOTE 1) PC2 (NOTE 1) PC3 PC4 (NOTE 1) | Skip TC 6.5.3.3 if UE supports NSA and TS 38.521-3 TC 6.5B.4.4 has been executed. |
| 6.5.3.3_1 | Additional spurious emissions with Power Boost | Rel-16 | C006w | UEs supporting 5GS FR2 and supporting mpr-PowerBoost-FR2-r16 | D005 | PC1 (NOTE 1) PC2 (NOTE 1) PC3 PC4 (NOTE 1) | Skip TC 6.5.3.3_1 if UE supports NSA and TS 38.521-3 TC 6.5B.4.4a has been executed. |
| 6.5A.1.1 | Occupied bandwidth for CA (2UL CA) | Rel-15 | C053 | UEs supporting 5GS FR2 and CA (2UL CA) | E020 |  | NOTE 1 Skip TC 6.5A.1.1 if UE supports NSA and TS 38.521-3 TC 6.5B.1.4_1.1 has been executed. |
| 6.5A.1.2 | Occupied bandwidth for CA (3UL CA) | Rel-15 | C054 | UEs supporting 5GS FR2 and CA (3UL CA) | E021 |  | NOTE 1 Skip TC 6.5A.1.2 if UE supports NSA and TS 38.521-3 TC 6.5B.1.4_1.2 has been executed. |
| 6.5A.1.3 | Occupied bandwidth for CA (4UL CA) | Rel-15 | C055 | UEs supporting 5GS FR2 and CA (4UL CA) | E022 |  | NOTE 1 Skip TC 6.5A.1.3 if UE supports NSA and TS 38.521-3 TC 6.5B.1.4_1.3 has been executed. |
| 6.5A.1.4 | Occupied bandwidth for CA (5UL CA) | Rel-15 | C056 | UEs supporting 5GS FR2 and CA (5UL CA) | E023 |  | NOTE 1 |
| 6.5A.1.5 | Occupied bandwidth for CA (6UL CA) | Rel-15 | C057 | UEs supporting 5GS FR2 and CA (6UL CA) | E024 |  | NOTE 1 |
| 6.5A.1.6 | Occupied bandwidth for CA (7UL CA) | Rel-15 | C058 | UEs supporting 5GS FR2 and CA (7UL CA) | E025 |  | NOTE 1 |
| 6.5A.1.7 | Occupied bandwidth for CA (8UL CA) | Rel-15 | C059 | UEs supporting 5GS FR2 and CA (8UL CA) | E026 |  | NOTE 1 |
| 6.5A.2.1.1 | Spectrum Emission Mask for CA (2UL CA) | Rel-15 | C053 | UEs supporting 5GS FR2 and CA (2UL CA) | E020 |  | Skip TC 6.5A.2.1.1 if UE supports NSA and TS 38.521-3 TC 6.5B.2.4.1_1.1 has been executed. |
| 6.5A.2.1.2 | Spectrum Emission Mask for CA (3UL CA) | Rel-15 | C054 | UEs supporting 5GS FR2 and CA (3UL CA) | E021 |  | NOTE 1 Skip TC 6.5A.2.1.2 if UE supports NSA and TS 38.521-3 TC 6.5B.2.4.1_1.2 has been executed. |
| 6.5A.2.1.3 | Spectrum Emission Mask for CA (4UL CA) | Rel-15 | C055 | UEs supporting 5GS FR2 and CA (4UL CA) | E022 |  | NOTE 1 Skip TC 6.5A.2.1.3 if UE supports NSA and TS 38.521-3 TC 6.5B.2.4.1_1.3 has been executed. |
| 6.5A.2.1.4 | Spectrum Emission Mask for CA (5UL CA) | Rel-15 | C056 | UEs supporting 5GS FR2 and CA (5UL CA) | E023 |  | NOTE 1 |
| 6.5A.2.1.5 | Spectrum Emission Mask for CA (6UL CA) | Rel-15 | C057 | UEs supporting 5GS FR2 and CA (6UL CA) | E024 |  | NOTE 1 |
| 6.5A.2.1.6 | Spectrum Emission Mask for CA (7UL CA) | Rel-15 | C058 | UEs supporting 5GS FR2 and CA (7UL CA) | E025 |  | NOTE 1 |
| 6.5A.2.1.7 | Spectrum Emission Mask for CA (8UL CA) | Rel-15 | C059 | UEs supporting 5GS FR2 and CA (8UL CA) | E026 |  | NOTE 1 |
| 6.5A.2.2.1 | Adjacent channel leakage ratio for CA (2UL CA) | Rel-15 | C053 | UEs supporting 5GS FR2 and CA (2UL CA) | E020 |  | Skip TC 6.5A.2.2.1 if UE supports NSA and TS 38.521-3 TC 6.5B.2.4.3_1.1 has been executed. |
| 6.5A.2.2.2 | Adjacent channel leakage ratio for CA (3UL CA) | Rel-15 | C054 | UEs supporting 5GS FR2 and CA (3UL CA) | E021 |  | Skip TC 6.5A.2.2.2 if UE supports NSA and TS 38.521-3 TC 6.5B.2.4.3_1.2 has been executed. |
| 6.5A.2.2.3 | Adjacent channel leakage ratio for CA (4UL CA) | Rel-15 | C055 | UEs supporting 5GS FR2 and CA (4UL CA) | E022 |  | Skip TC 6.5A.2.2.3 if UE supports NSA and TS 38.521-3 TC 6.5B.2.4.3_1.3 has been executed. |
| 6.5A.2.2.4 | Adjacent channel leakage ratio for CA (5UL CA) | Rel-15 | C056 | UEs supporting 5GS FR2 and CA (5UL CA) | E023 |  | NOTE 1 |
| 6.5A.2.2.5 | Adjacent channel leakage ratio for CA (6UL CA) | Rel-15 | C057 | UEs supporting 5GS FR2 and CA (6UL CA) | E024 |  | NOTE 1 |
| 6.5A.2.2.6 | Adjacent channel leakage ratio for CA (7UL CA) | Rel-15 | C058 | UEs supporting 5GS FR2 and CA (7UL CA) | E025 |  | NOTE 1 |
| 6.5A.2.2.7 | Adjacent channel leakage ratio for CA (8UL CA) | Rel-15 | C059 | UEs supporting 5GS FR2 and CA (8UL CA) | E026 |  | NOTE 1 |
| 6.5A.3.1.1 | General spurious emissions for CA (2UL CA) | Rel-15 | C053 | UEs supporting 5GS FR2 and CA (2UL CA) | E020 | PC1 (NOTE 1) PC2 (NOTE 1) PC3 PC4 (NOTE 1) | Skip TC 6.5A.3.1.1 if UE supports NSA and TS 38.521-3 TC 6.5B.3.4.1_1.1 has been executed. |
| 6.5A.3.1.2 | General spurious emissions for CA (3UL CA) | Rel-15 | C054 | UEs supporting 5GS FR2 and CA (3UL CA) | E021 | PC1 (NOTE 1) PC2 (NOTE 1) PC3 PC4 (NOTE 1) | Skip TC 6.5A.3.1.2 if UE supports NSA and TS 38.521-3 TC 6.5B.3.4.1_1.2 has been executed. |
| 6.5A.3.1.3 | General spurious emissions for CA (4UL CA) | Rel-15 | C055 | UEs supporting 5GS FR2 and CA (4UL CA) | E022 | PC1 (NOTE 1) PC2 (NOTE 1) PC3 PC4 (NOTE 1) | Skip TC 6.5A.3.1.3 if UE supports NSA and TS 38.521-3 TC 6.5B.3.4.1_1.3 has been executed. |
| 6.5A.3.1.4 | General spurious emissions for CA (5UL CA) | Rel-15 | C056 | UEs supporting 5GS FR2 and CA (5UL CA) | E023 | PC1 (NOTE 1) PC2 (NOTE 1) PC3 PC4 (NOTE 1) | Skip TC 6.5A.3.1.4 if UE supports NSA and TS 38.521-3 TC 6.5B.3.4.1_1.4 has been executed. |
| 6.5A.3.1.5 | General spurious emissions for CA (6UL CA) | Rel-15 | C057 | UEs supporting 5GS FR2 and CA (6UL CA) | E024 | PC1 (NOTE 1) PC2 (NOTE 1) PC3 PC4 (NOTE 1) | Skip TC 6.5A.3.1.5 if UE supports NSA and TS 38.521-3 TC 6.5B.3.4.1_1.5 has been executed. |
| 6.5A.3.1.6 | General spurious emissions for CA (7UL CA) | Rel-15 | C058 | UEs supporting 5GS FR2 and CA (7UL CA) | E025 | PC1 (NOTE 1) PC2 (NOTE 1) PC3 PC4 (NOTE 1) | Skip TC 6.5A.3.1.6 if UE supports NSA and TS 38.521-3 TC 6.5B.3.4.1_1.6 has been executed. |
| 6.5A.3.1.7 | General spurious emissions for CA (8UL CA) | Rel-15 | C059 | UEs supporting 5GS FR2 and CA (8UL CA) | E026 | PC1 (NOTE 1) PC2 (NOTE 1) PC3 PC4 (NOTE 1) | Skip TC 6.5A.3.1.7 if UE supports NSA and TS 38.521-3 TC 6.5B.3.4.1_1.7 has been executed. |
| 6.5A.3.2.1 | Spurious emission band UE co-existence for CA (2UL CA) | Rel-15 | C053 | UEs supporting 5GS FR2 CA (2UL CA) | E020 | PC1 (NOTE 1) PC2 (NOTE 1) PC3 PC4 (NOTE 1) | Skip TC 6.5A.3.2.1 if UE supports NSA and TS 38.521-3 TC 6.5B.3.4.2_1.1 has been executed. |
| 6.5A.3.2.2 | Spurious emission band UE co-existence for CA (3UL CA) | Rel-15 | C054 | UEs supporting 5GS FR2 and CA (3UL CA) | E021 | PC1 (NOTE 1) PC2 (NOTE 1) PC3 PC4 (NOTE 1) | Skip TC 6.5A.3.2.2 if UE supports NSA and TS 38.521-3 TC 6.5B.3.4.2_1.2 has been executed. |
| 6.5A.3.2.3 | Spurious emission band UE co-existence for CA (4UL CA) | Rel-15 | C055 | UEs supporting 5GS FR2 and CA (4UL CA) | E022 | PC1 (NOTE 1) PC2 (NOTE 1) PC3 PC4 (NOTE 1) | Skip TC 6.5A.3.2.3 if UE supports NSA and TS 38.521-3 TC 6.5B.3.4.2_1.3 has been executed. |
| 6.5A.3.2.4 | Spurious emission band UE co-existence for CA (5UL CA) | Rel-15 | C056 | UEs supporting 5GS FR2 and CA (5UL CA) | E023 | PC1 (NOTE 1) PC2 (NOTE 1) PC3 PC4 (NOTE 1) | Skip TC 6.5A.3.2.4 if UE supports NSA and TS 38.521-3 TC 6.5B.3.4.2_1.4 has been executed. |
| 6.5A.3.2.5 | Spurious emission band UE co-existence for CA (6UL CA) | Rel-15 | C057 | UEs supporting 5GS FR2 and CA (6UL CA) | E024 | PC1 (NOTE 1) PC2 (NOTE 1) PC3 PC4 (NOTE 1) | Skip TC 6.5A.3.2.5 if UE supports NSA and TS 38.521-3 TC 6.5B.3.4.2_1.5 has been executed. |
| 6.5A.3.2.6 | Spurious emission band UE co-existence for CA (7UL CA) | Rel-15 | C058 | UEs supporting 5GS FR2 and CA (7UL CA) | E025 | PC1 (NOTE 1) PC2 (NOTE 1) PC3 PC4 (NOTE 1) | Skip TC 6.5A.3.2.6 if UE supports NSA and TS 38.521-3 TC 6.5B.3.4.2_1.6 has been executed. |
| 6.5A.3.2.7 | Spurious emission band UE co-existence for CA (8UL CA) | Rel-15 | C059 | UEs supporting 5GS FR2 and CA (8UL CA) | E026 | PC1 (NOTE 1) PC2 (NOTE 1) PC3 PC4 (NOTE 1) | Skip TC 6.5A.3.2.7 if UE supports NSA and TS 38.521-3 TC 6.5B.3.4.2_1.7 has been executed. |
| 6.5A.3.3.1 | Additional spurious emissions for CA (2UL CA) | Rel-15 | C053 | UEs supporting 5GS FR2 CA (2UL CA) | E020 | PC1 | Skip TC 6.5A.3.3.1 if UE supports NSA and TS 38.521-3 TC 6.5B.4.4.1_1.1 has been executed. |
| 6.5A.3.3.2 | Additional spurious emissions for CA (3UL CA) | Rel-15 | C054 | UEs supporting 5GS FR2 and CA (3UL CA) | E021 | PC1 | Skip TC 6.5A.3.3.2 if UE supports NSA and TS 38.521-3 TC 6.5B.4.4.1_1.2 has been executed. |
| 6.5A.3.3.3 | Additional spurious emissions for CA (4UL CA) | Rel-15 | C055 | UEs supporting 5GS FR2 and CA (4UL CA) | E022 | PC1 | Skip TC 6.5A.3.3.3 if UE supports NSA and TS 38.521-3 TC 6.5B.4.4.3_1.3 has been executed. |
| 6.5A.3.3.4 | Additional spurious emissions for CA (5UL CA) | Rel-15 | C056 | UEs supporting 5GS FR2 and CA (5UL CA) | E023 |  | NOTE 1 |
| 6.5A.3.3.5 | Additional spurious emissions for CA (6UL CA) | Rel-15 | C057 | UEs supporting 5GS FR2 and CA (6UL CA) | E024 |  | NOTE 1 |
| 6.5A.3.3.6 | Additional spurious emissions for CA (7UL CA) | Rel-15 | C058 | UEs supporting 5GS FR2 and CA (7UL CA) | E025 |  | NOTE 1 |
| 6.5A.3.3.7 | Additional spurious emissions for CA (8UL CA) | Rel-15 | C059 | UEs supporting 5GS FR2 and CA (8UL CA) | E026 |  | NOTE 1 |
| 6.5D.1 | Occupied bandwidth for UL MIMO | Rel-15 | C060 | UEs supporting 5GS FR2 and UL MIMO | D023 |  | NOTE 1 Skip TC 6.5D.1 if UE supports NSA and TS 38.521-3 TC 6.5B.1.4D has been executed. |
| 6.5D.2.1 | Spectrum Emission Mask for UL MIMO | Rel-15 | C060 | UEs supporting 5GS FR2 and UL MIMO | D023 | PC1 PC2 (NOTE 1) PC3 PC4 (NOTE 1) PC5 | Skip TC 6.5D.2.1 if UE supports NSA and TS 38.521-3 TC 6.5B.2.4.1D has been executed. |
| 6.5D.2.2 | Adjacent channel leakage ratio for UL MIMO | Rel-15 | C060 | UEs supporting 5GS FR2 and UL MIMO | D023 | PC1 PC2 (NOTE 1) PC3 PC4 (NOTE 1) PC5 | Skip TC 6.5D.2.2 if UE supports NSA and TS 38.521-3 TC 6.5B.2.4D.3 has been executed. |
| 6.5D.3.1 | Transmitter Spurious emissions for UL MIMO | Rel-15 | C060 | UEs supporting 5GS FR2 and UL MIMO | D023 | PC1 PC2 (NOTE 1) PC3 PC4 (NOTE 1) | Skip TC 6.5D.3.1 if UE supports NSA and TS 38.521-3 TC 6.5B.3.4.1D has been executed. |
| 6.5D.3.2 | Spurious emission band UE co-existence for UL MIMO | Rel-15 | C060 | UEs supporting 5GS FR2 and UL MIMO | D023 | PC1 PC2 (NOTE 1) PC3 PC4 (NOTE 1) | Skip TC 6.5D.3.2 if UE supports NSA and TS 38.521-3 TC 6.5B.3.4.2D has been executed. |
| 6.5D.3.3 | Additional spurious emissions for UL MIMO | Rel-15 | C060 | UEs supporting 5GS FR2 and UL MIMO | D023 | PC1 (NOTE 1) PC2 (NOTE 1) PC3 PC4 (NOTE 1) | Skip TC 6.5D.3.3 if UE supports NSA and TS 38.521-3 TC 6.5B.4.4D has been executed. |
| 6.6.1 | Beam correspondence - EIRP | Rel-15 | C008 | Release 15 UEs supporting 5GS FR2 and not beam correspondence without UL beam sweeping and release 16 and forward UEs that do not support SSB-based or CSI-RS based enhanced beam correspondence and do not support enhanced beam correspondence without UL beam sweeping | D005 | PC3 | Skip TC 6.6.1 if UE supports NSA and TS 38.521-3 TC 6.6B.4 has been executed. |
| 6.6.2 | Enhanced Beam correspondence - EIRP | Rel-16 | C008a | UEs supporting 5GS FR2 and support either CSI-RS or SSB based beam correspondence and do not support beam correspondence without UL beam sweeping | D005 | PC3 | Skip TC 6.6.2 if UE supports NSA and TS 38.521-3 TC 6.6B.5 has been executed. |
| 7 | Receiver Characteristics |  |  |  |  |  |  |
| 7.3.2 | Reference sensitivity power level | Rel-15 | C006 | UEs supporting 5GS FR2 | D005 | PC1 PC2 (NOTE 1) PC3 PC4 (NOTE 1) | Skip TC 7.3.2 if UE supports NSA and TS 38.521-3 TC 7.3B.2.4 has been executed. |
| 7.3A.2.1 | Reference sensitivity power level for CA (2DL CA) | Rel-15 | C006c | UEs supporting 5GS FR2 and CA (2DL CA) | E032 | PC1 (NOTE 1) PC2 (NOTE 1) PC3 PC4 (NOTE 1) | Skip TC 7.3A.2.1 if UE supports NSA and TS 38.521-3 TC 7.3B.2.4_1.1 has been executed |
| 7.3A.2.2 | Reference sensitivity power level for CA (3DL CA) | Rel-15 | C006d | UEs supporting 5GS FR2 and CA (3DL CA) | E033 | PC1 (NOTE 1) PC2 (NOTE 1) PC3 PC4 (NOTE 1) | Skip TC 7.3A.2.2 if UE supports NSA and TS 38.521-3 TC 7.3B.2.4_1.2 has been executed |
| 7.3A.2.3 | Reference sensitivity power level for CA (4DL CA) | Rel-15 | C006e | UEs supporting 5GS FR2 and CA (4DL CA) | E034 | PC1 (NOTE 1) PC2 (NOTE 1) PC3 PC4 (NOTE 1) | Skip TC 7.3A.2.3 if UE supports NSA and TS 38.521-3 TC 7.3B.2.4_1.3 has been executed |
| 7.3A.2.4 | Reference sensitivity power level for CA (5DL CA) | Rel-15 | C006f | UEs supporting 5GS FR2 and CA (5DL CA) | E035 | PC1 PC2 PC3 PC4 | NOTE 1 Skip TC 7.3A.2.4 if UE supports NSA and TS 38.521-3 TC 7.3B.2.4_1.4 has been executed |
| 7.3A.2.5 | Reference sensitivity power level for CA (6DL CA) | Rel-15 | C006g | UEs supporting 5GS FR2 and CA (6DL CA) | E036 | PC1 PC2 PC3 PC4 | NOTE 1 Skip TC 7.3A.2.5 if UE supports NSA and TS 38.521-3 TC 7.3B.2.4_1.5 has been executed |
| 7.3A.2.6 | Reference sensitivity power level for CA (7DL CA) | Rel-15 | C006h | UEs supporting 5GS FR2 and CA (7DL CA) | E037 | PC1 PC2 PC3 PC4 | NOTE 1 Skip TC 7.3A.2.6 if UE supports NSA and TS 38.521-3 TC 7.3B.2.4_1.6 has been executed |
| 7.3A.2.7 | Reference sensitivity power level for CA (8DL CA) | Rel-15 | C006i | UEs supporting 5GS FR2 and CA (8DL CA) | E038 | PC1 PC2 PC3 PC4 | NOTE 1 Skip TC 7.3A.2.7 if UE supports NSA and TS 38.521-3 TC 7.3B.2.4_1.7 has been executed |
| 7.3A.3.1 | EIS spherical coverage for Inter-band CA (2DL CA) | Rel-16 | C006c | UEs supporting 5GS FR2 and CA (2DL CA) | E032 | PC1 PC2 PC3 PC4 | NOTE 1 |
| 7.3A.3.2 | EIS spherical coverage for Inter-band CA (3DL CA) | Rel-16 | C006d | UEs supporting 5GS FR2 and CA (3DL CA) | E033 | PC1 PC2 PC3 PC4 | NOTE 1 |
| 7.3A.3.3 | EIS spherical coverage for Inter-band CA (4DL CA) | Rel-16 | C006e | UEs supporting 5GS FR2 and CA (4DL CA) | E034 | PC1 PC2 PC3 PC4 | NOTE 1 |
| 7.3A.3.4 | EIS spherical coverage for CA (5DL CA) | Rel-16 | C006f | UEs supporting 5GS FR2 and CA (5DL CA) | E035 | PC1 PC2 PC3 PC4 | NOTE 1 |
| 7.3A.3.5 | EIS spherical coverage for CA (6DL CA) | Rel-16 | C006g | UEs supporting 5GS FR2 and CA (6DL CA) | E036 | PC1 PC2 PC3 PC4 | NOTE 1 |
| 7.3A.3.6 | EIS spherical coverage for CA (7DL CA) | Rel-16 | C006h | UEs supporting 5GS FR2 and CA (7DL CA) | E037 | PC1 PC2 PC3 PC4 | NOTE 1 |
| 7.3A.3.7 | EIS spherical coverage for CA (8DL CA) | Rel-16 | C006i | UEs supporting 5GS FR2 and CA (8DL CA) | E038 | PC1 PC2 PC3 PC4 | NOTE 1 |
| 7.3.4 | EIS spherical coverage | Rel-15 | C006 | UEs supporting 5GS FR2 | D005 | PC1 PC2 PC3 PC4 | Skip TC 7.3.4 if UE supports NSA and TS 38.521-3 TC 7.3B.4 has been executed. |
| 7.4 | Maximum input level | Rel-15 | N/A | not recommended due to testability issues | N/A |  | NOTE 1 |
| 7.4A.1 | Maximum input level for CA (2DL CA) | Rel-15 | N/A | not recommended due to testability issues | N/A |  | NOTE 1 |
| 7.4A.2 | Maximum input level for CA (3DL CA) | Rel-15 | N/A | not recommended due to testability issues | N/A |  | NOTE 1 |
| 7.4A.3 | Maximum input level for CA (4DL CA) | Rel-15 | N/A | not recommended due to testability issues | N/A |  | NOTE 1 |
| 7.4A.4 | Maximum input level for CA (5DL CA) | Rel-15 | N/A | not recommended due to testability issues | N/A |  | NOTE 1 |
| 7.4A.5 | Maximum input level for CA (6DL CA) | Rel-15 | N/A | not recommended due to testability issues | N/A |  | NOTE 1 |
| 7.4A.6 | Maximum input level for CA (7DL CA) | Rel-15 | N/A | not recommended due to testability issues | N/A |  | NOTE 1 |
| 7.4A.7 | Maximum input level for CA (8DL CA) | Rel-15 | N/A | not recommended due to testability issues | N/A |  | NOTE 1 |
| 7.5 | Adjacent channel selectivity | Rel-15 | C006 | UEs supporting 5GS FR2 | D005 |  | Skip TC 7.5 if UE supports NSA and TS 38.521-3 TC 7.5B.4 has been executed. |
| 7.6.2 | In-band blocking | Rel-15 | C006 | UEs supporting 5GS FR2 | D005 |  | Skip TC 7.6.2 if UE supports NSA and TS 38.521-3 TC 7.6B.2.4 has been executed. |
| 7.9 | Spurious emissions | Rel-15 | C006 | UEs supporting 5GS FR2 | D005 |  | Skip TC 7.9 if UE supports NSA and TS 38.521-3 TC 7.9B.4 has been executed. |
| NOTE 1: The test case/branch is incomplete for any Band/CA/DC Configuration but has basic test configurations already. NOTE 1 can be removed only when the test case/branch is complete for at least one Band/CA/DC Configuration for at least one feature included in the test case/branch. Detailed completion status can be found in the corresponding test case section in 38.521-2. NOTE 2: Void. NOTE 3: Void. NOTE 4: Void. |  |  |  |  |  |  |  |

Table 4.1.2-1a: Void

Table 4.1.2-1b: Void

Table 4.1.2-1c: Void

### 4.1.3	NR interworking between NR FR1 and NR FR2 and between NR and LTE conformance test cases

Table 4.1.3-1: Applicability of RF EN-DC FR1 and FR2 conformance test cases, ref. TS 38.521-3 [3]

| Clause | TC Title | Release | Applicability |  | Tested Bands/CA/DC-Configurations Selection | Branch | Additional Information |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Condition | Comment |  |  |  |
| 6 | Transmitter characteristics |  |  |  |  |  |  |
| 6.2B | Transmitter power for DC |  |  |  |  |  |  |
| 6.2B.1.1 | UE Maximum Output Power for Intra-Band Contiguous EN-DC | Rel-15 | C009 | UEs supporting Intra-Band Contiguous EN-DC (2UL CCs) | E003 |  |  |
| 6.2B.1.2 | UE Maximum Output Power for Intra-Band Non-Contiguous EN-DC | Rel-15 | C010 | UEs supporting Intra-Band non-contiguous EN-DC (2UL CCs) | E004 |  |  |
| 6.2B.1.3 | UE Maximum Output Power for Inter-Band EN-DC within FR1 (1 E-UTRA CC, 1 NR CC) | Rel-15 | C011 | UEs supporting Inter-Band EN-DC within FR1 (2UL CCs) | E005 E005d | PC3 PC2 |  |
| 6.2B.1.3_1 | UE Maximum Output Power for Inter-Band EN-DC within FR1 (2 E-UTRA CCs, 1 NR CC) | Rel-16 | C011d | UEs supporting Inter-Band EN-DC within FR1 (2UL E-UTRA CCs, 1UL NR CC) | E005z | PC3 |  |
| 6.2B.1.4.1 | UE Maximum Output Power for Inter-Band EN-DC including FR2 (1 NR CC) - EIRP and TRP | Rel-15 | C012 | UEs supporting Inter-Band EN-DC including FR2 with 1 NR UL CC | E010 | PC1 PC2 (NOTE 1) PC3 PC4 (NOTE 1) | NOTE 5 Skip TC 6.2B.1.4.1 if UE supports SA and TSC 38.521-2 TC 6.2.1.1 has been executed. |
| 6.2B.1.4.2 | UE Maximum Output Power for Inter-Band EN-DC including FR2 (1 NR CC) - Spherical Coverage | Rel-15 | C012 | UEs supporting Inter-Band EN-DC including FR2 with 1 NR UL CC | E010 | PC1 PC2 (NOTE 1) PC3 PC4 (NOTE 1) | NOTE 5 Skip TC 6.2B.1.4.2 if UE supports SA and TSC 38.521-2 TC 6.2.1.2 has been executed. |
| 6.2B.1.4_1 | UE Maximum Output Power for Inter-Band EN-DC including FR2 (>1 NR CC) |  |  |  |  |  |  |
| 6.2B.1.4_1.1.1 | UE Maximum Output Power for Inter-Band EN-DC including FR2 (2 NR CCs) - EIRP and TRP | Rel-15 | C012b | UEs supporting Inter-Band EN-DC including FR2 with 2 NR UL CCs | E011 | PC1 (NOTE 1) PC2 (NOTE 1) PC3 PC4 (NOTE 1) | NOTE 5 Skip TC 6.2B.1.4_1.1.1 if UE supports SA and TS 38.521-2 TC 6.2A.1.1.1 has been executed. |
| 6.2B.1.4_1.1.2 | UE Maximum Output Power for Inter-Band EN-DC including FR2 (2 NR CCs) - Spherical Coverage | Rel-15 | C012b | UEs supporting Inter-Band EN-DC including FR2 with 2 NR UL CCs | E011 | PC1 (NOTE 1) PC2 (NOTE 1) PC3 PC4 (NOTE 1) | NOTE 5 Skip TC 6.2B.1.4_1.1.2 if UE supports SA and TS 38.521-2 TC 6.2A.1.2.1 has been executed. |
| 6.2B.1.4_1.2.1 | UE Maximum Output Power for Inter-Band EN-DC including FR2 (3 NR CCs) - EIRP and TRP | Rel-15 | C012c | UEs supporting Inter-Band EN-DC including FR2 with 3 NR UL CCs | E012 | PC1 (NOTE 1) PC2 (NOTE 1) PC3 PC4 (NOTE 1) | NOTE 5 Skip TC 6.2B.1.4_1.2.1 if UE supports SA and TS 38.521-2 TC 6.2A.1.1.2 has been executed. |
| 6.2B.1.4_1.2.2 | UE Maximum Output Power for Inter-Band EN-DC including FR2 (3 NR CCs) - Spherical Coverage | Rel-15 | C012c | UEs supporting Inter-Band EN-DC including FR2 with 3 NR UL CCs | E012 | PC1 (NOTE 1) PC2 (NOTE 1) PC3 PC4 (NOTE 1) | NOTE 5 Skip TC 6.2B.1.4_1.2.2 if UE supports SA and TS 38.521-2 TC 6.2A.1.2.2 has been executed. |
| 6.2B.1.4_1.3.1 | UE Maximum Output Power for Inter-Band EN-DC including FR2 (4 NR CCs) - EIRP and TRP | Rel-15 | C012d | UEs supporting Inter-Band EN-DC including FR2 with 4 NR UL CCs | E013 | PC1 (NOTE 1) PC2 (NOTE 1) PC3 PC4 (NOTE 1) | NOTE 5 Skip TC 6.2B.1.4_1.3.1 if UE supports SA and TS 38.521-2 TC 6.2A.1.1.3 has been executed. |
| 6.2B.1.4_1.3.2 | UE Maximum Output Power for Inter-Band EN-DC including FR2 (4 NR CCs) - Spherical Coverage | Rel-15 | C012d | UEs supporting Inter-Band EN-DC including FR2 with 4 NR UL CCs | E013 | PC1 (NOTE 1) PC2 (NOTE 1) PC3 PC4 (NOTE 1) | NOTE 5 Skip TC 6.2B.1.4_1.3.2 if UE supports SA and TS 38.521-2 TC 6.2A.1.2.3 has been executed. |
| 6.2B.1.4D.1 | UE Maximum Output Power for Inter-Band EN-DC including FR2 for UL MIMO - EIRP and TRP | Rel-15 | C012p | UEs supporting Inter-Band EN-DC including FR2 and UL MIMO | E043 | PC1 PC2 (NOTE 1) PC3 PC4 (NOTE 1) PC5 | NOTE 5 Skip TC 6.2B.1.4D.1 if UE supports SA and TS 38.521-2 TC 6.2D.1.1 has been executed. |
| 6.2B.1.4D.2 | UE Maximum Output Power for Inter-Band EN-DC including FR2 for UL MIMO - Spherical Coverage | Rel-15 | C012p | UEs supporting Inter-Band EN-DC including FR2 and UL MIMO | E043 | PC1 PC2 (NOTE 1) PC3 PC4 (NOTE 1) PC5 (NOTE 1) | NOTE 5 Skip TC 6.2B.1.4D.2 if UE supports SA and TS 38.521-2 TC 6.2D.1.2 has been executed. |
| 6.2B.1.6 | UE Maximum Output Power for Inter-Band EN-DC including FR2 (1 NR CC) - EIRP with UL Gaps | Rel-15 | C012b | UEs supporting Inter-Band EN-DC including FR2 and Uplink Gaps | E011 | PC1 PC2 PC3 PC4 | NOTE 1 NOTE 5 Skip TC 6.2B.1.6 if UE supports SA and TS 38.521-2 TC 6.2.5 has been executed. |
| 6.2B.2.1 | UE Maximum Output Power reduction for Intra-Band Contiguous EN-DC | Rel-15 | C009 | UEs supporting Intra-Band Contiguous EN-DC (2UL CCs) | E003 |  | Test execution is not necessary if TS 38.521-3 TC 6.5B.2.1.3 is executed. |
| 6.2B.2.2 | UE Maximum Output Power reduction for Intra-Band Non-Contiguous EN-DC | Rel-15 | C010 | UEs supporting Intra-Band non-contiguous EN-DC (2UL CCs) | E004 |  | Test execution is not necessary if TS 38.521-3 TC 6.5B.2.2.3 has been executed. |
| 6.2B.2.3 | UE Maximum Output Power reduction for Inter-Band EN-DC within FR1 (1 NR CC) | Rel-15 | C011 | UEs supporting Inter-Band EN-DC within FR1 with 1 NR UL CC | E005b | PC3 PC2 | NOTE 5 Test execution is not necessary if TS 38.521-3 TC 6.5B.2.3.3.1 is executed. Skip TC 6.2B.2.3 if UE supports SA and TS 38.521-1 TC 6.2.2 or 6.5.2.4.1 has been executed. |
| 6.2B.2.4 | UE Maximum Output Power reduction for Inter-Band EN-DC including FR2 (1 NR CC) | Rel-15 | C012z | UEs supporting Inter-Band EN-DC including FR2 with 1 NR UL CC and modified MPR behaviour | E010 |  | NOTE 1 NOTE 5 Skip TC 6.2B.2.4 if UE supports SA and TS 38.521-2 TC 6.2.2 has been executed. |
| 6.2B.2.4a | UE maximum output power reduction enhancements for Inter-Band EN-DC including FR2 (1 NR CC) | Rel-16 | C012q | UEs supporting Inter-Band EN-DC including FR2 with 1 NR UL CC and modified MPR behaviour bit 0. | E010 | PC3 |  |
| 6.2B.2.4_1 | UE Maximum Output Power reduction for Inter-Band EN-DC including FR2 (>1 NR CC) |  |  |  |  |  |  |
| 6.2B.2.4_1.1 | UE Maximum Output Power reduction for Inter-Band EN-DC including FR2 (2 NR CCs) | Rel-15 | C012b | UEs supporting Inter-Band EN-DC including FR2 with 2 NR UL CCs | E011 |  | NOTE 1 NOTE 5 Skip TC 6.2B.2.4_1.1 if UE supports SA and TS 38.521-2 TC 6.2A.2.1 has been executed. |
| 6.2B.2.4_1.2 | UE Maximum Output Power reduction for Inter-Band EN-DC including FR2 (3 NR CCs) | Rel-15 | C012c | UEs supporting Inter-Band EN-DC including FR2 with 3 NR UL CCs | E012 |  | NOTE 1 NOTE 5 Skip TC 6.2B.2.4_1.1 if UE supports SA and TS 38.521-2 TC 6.2A.2.2 has been executed. |
| 6.2B.2.4_1.3 | UE Maximum Output Power reduction for Inter-Band EN-DC including FR2 (4 NR CCs) | Rel-15 | C012d | UEs supporting Inter-Band EN-DC including FR2 with 4 NR UL CCs | E013 |  | NOTE 1 NOTE 5 Skip TC 6.2B.2.4_1.1 if UE supports SA and TS 38.521-2 TC 6.2A.2.3 has been executed. |
| 6.2B.2.4D | UE Maximum Output Power reduction for Inter-Band EN-DC including FR2 for UL MIMO (1 NR CC) | Rel-15 | C012p | UEs supporting Inter-Band EN-DC including FR2 and UL MIMO | E043 | PC1 (NOTE 1) PC2 (NOTE 1) PC3 PC4 (NOTE 1) PC5 (NOTE 1) | NOTE 5 Skip TC 6.2B.2.4D if UE supports SA and TS 38.521-2 TC 6.2D.2 has been executed. |
| 6.2B.3.1 | UE Additional Maximum Output Power reduction for Intra-band contiguous EN-DC | Rel-15 | C009z | UEs supporting Intra-Band Contiguous EN-DC (2UL CCs) and modified MPR behaviour | E003 |  |  |
| 6.2B.3.2 | UE Additional Maximum Output Power reduction for Intra-Band Non-Contiguous EN-DC | Rel-15 | FFS | FFS | FFS |  | NOTE 1 |
| 6.2B.3.3 | UE Additional Maximum Output power reduction for inter-band EN-DC within FR1 (1 NR CC) | Rel-15 | C011z | UEs supporting Inter-Band EN-DC within FR1 with 1 NR UL CC and modified MPR behaviour | E005b | PC3 PC2 | NOTE 5 Test execution is not necessary if TS 38.521-3 TCs 6.5B.2.3.2, 6.5B.2.3.3.2 and 6.5B.4.3 are executed. Skip TC 6.2B.3.3 if UE supports SA and TS 38.521-1 TC 6.2.3 has been executed, or TS 38.521-1 TCs 6.5.2.3, 6,5,2,4,2 and 6.5.3.3 have been executed. |
| 6.2B.3.4 | UE Additional Maximum Output Power reduction for Inter-Band EN-DC including FR2 (1 NR CC) | Rel-15 | C012z | UEs supporting Inter-Band EN-DC including FR2 with 1 NR UL CC and modified MPR behaviour | E010 | PC3 PC2 | NOTE 5 Skip TC 6.2B.3.4 if UE supports SA and TS 38.521-2 TC 6.2.3 has been executed. |
| 6.2B.3.4_1 | UE Additional Maximum Output power reduction for inter-band EN-DC including FR2 (> 1 NR CC) |  |  |  |  |  |  |
| 6.2B.3.4_1.1 | UE Additional Maximum Output power reduction for inter-band EN-DC including FR2 (2 NR CCs) | Rel-15 | C012b | UEs supporting Inter-Band EN-DC including FR2 with 2 NR UL CCs | E011 | PC1 PC2 (NOTE 1) PC3 PC4 (NOTE 1) | NOTE 5 Skip TC 6.2B.3.4_1.1 if UE supports SA and TS 38.521-2 TC 6.2A.3.1 has been executed. |
| 6.2B.3.4_1.2 | UE Additional Maximum Output power reduction for inter-band EN-DC including FR2 (3 NR CCs) | Rel-15 | C012c | UEs supporting Inter-Band EN-DC including FR2 with 3 NR UL CCs | E012 | PC1 PC2 (NOTE 1) PC3 PC4 (NOTE 1) | NOTE 5 Skip TC 6.2B.3.4_1.2 if UE supports SA and TS 38.521-2 TC 6.2A.3.2 has been executed. |
| 6.2B.3.4_1.3 | UE Additional Maximum Output power reduction for inter-band EN-DC including FR2 (4 NR CCs) | Rel-15 | C012d | UEs supporting Inter-Band EN-DC including FR2 with 4 NR UL CCs | E013 | PC1 PC2 (NOTE 1) PC3 PC4 (NOTE 1) | NOTE 5 Skip TC 6.2B.3.4_1.3 if UE supports SA and TS 38.521-2 TC 6.2A.3.3 has been executed. |
| 6.2B.3.4D | UE Additional Maximum Output Power reduction for Inter-Band EN-DC including FR2 (1 NR CC) for UL MIMO | Rel-15 | C012p | UEs supporting Inter-Band EN-DC including FR2 and UL MIMO | E043 | PC1 (NOTE 1) PC2 (NOTE 1) PC3 PC4 (NOTE 1) PC5 (NOTE 1) | NOTE 5 Skip TC 6.2B.3.4D if UE supports SA and TS 38.521-2 TC 6.2D.3 has been executed. |
| 6.2B.4.1.1 | Configured Output Power Level for Intra-Band Contiguous EN-DC | Rel-15 | C009 | UEs supporting Intra-Band Contiguous EN-DC (2UL CCs) | E003 |  |  |
| 6.2B.4.1.2 | Configured Output Power for Intra-Band Non-Contiguous EN-DC | Rel-15 | C010 | UEs supporting Intra-Band Non-Contiguous EN-DC (2UL CCs) | E004 |  |  |
| 6.2B.4.1.3 | Configured Output Power for Inter-Band EN-DC within FR1 (1 E-UTRA CC, 1 NR CC) | Rel-15 | C011 | UEs supporting Inter-Band EN-DC within FR1 (2UL CCs) | E005 |  |  |
| 6.2B.4.1.3_1 | Configured Output Power for Inter-Band EN-DC within FR1 (2 E-UTRA CCs, 1 NR CC) | Rel-16 | C011d | UEs supporting Inter-Band EN-DC within FR1 (2UL E-UTRA CCs, 1UL NR CC) | E005z | PC3 |  |
| 6.2B.4.1.4 | Configured Output Power for Inter-Band EN-DC including FR2 (1 NR CC) | Rel-15 | C012 | UEs supporting inter-band EN-DC including FR2 with 1 NR UL CC | E010 |  |  |
| 6.2B.4.1.4_1 | Configured Output Power with Power Boost for Inter-Band EN-DC including FR2 (1 NR CC) | Rel-16 | C012w | UEs supporting inter-band EN-DC including FR2 with 1 NR UL CC and mpr-PowerBoost-FR2-r16 | E010 | PC1 (NOTE 1) PC2 (NOTE 1) PC3 PC4 (NOTE 1) | NOTE 5 Skip TC 6.2B.4.1.4_1 if UE supports NSA and TS 38.521-2 TC 6.2.4_1 has been executed. |
| 6.2H | Transmitter power for DC with UL MIMO |  |  |  |  |  |  |
| 6.2H.1.3 | UE maximum output power for inter-band EN-DC with UL MIMO within FR1 | Rel-17 | C330 | UEs supporting inter-band EN-DC (2UL CCs) with UL MIMO | E045 | PC3 PC2 |  |
| 6.2L | Transmitter power for DC with Tx Diversity |  |  |  |  |  |  |
| 6.2L.1.3 | UE maximum output power for inter-band EN-DC with Tx Diversity within FR1 | Rel-17 | C332 | UEs supporting inter-band EN-DC (2UL CCs) with Tx Diversity | E046 | PC3 PC2 |  |
| 6.3B | Output power dynamics for DC |  |  |  |  |  |  |
| 6.3B.1.1 | Minimum Output power for intra-band contiguous EN-DC | Rel-15 | C009 | UEs supporting intra-band contiguous EN-DC (2UL CCs) | E003 |  |  |
| 6.3B.1.2 | Minimum output power for intra-band non-contiguous EN-DC | Rel-15 | C010 | UEs supporting intra-band non-contiguous EN-DC (2UL CCs) | E004 |  | NOTE 5 Skip TC 6.3B.1.2 if UE supports SA and TS 38.521-1 TC 6.3.1 has been executed. |
| 6.3B.1.3 | Minimum output power for inter-band EN-DC within FR1 (1 NR CC) | Rel-15 | C011b | UEs supporting inter-band EN-DC within FR1 with 1 NR UL CC | E005b |  | NOTE 5 Skip TC 6.3B.1.3 if UE supports SA and TS 38.521-1 TC 6.3.1 has been executed. |
| 6.3B.1.4 | Minimum Output Power for EN-DC Interband including FR2 (1 NR CC) | Rel-15 | C012 | UEs supporting inter-band EN-DC including FR2 with 1 NR UL CC | E010 |  | NOTE 5 Skip TC 6.3B.1.4 if UE supports SA and TS 38.521-2 TC 6.3.1 has been executed. |
| 6.3B.1.4_1 | Minimum output power for inter-band EN-DC including FR2 (>1 NR CC) |  |  |  |  |  |  |
| 6.3B.1.4_1.1 | Minimum output power for inter-band EN-DC including FR2 (2 NR CCs) | Rel-15 | C012b | UEs supporting Inter-Band EN-DC including FR2 with 2 NR UL CCs | E011 |  | NOTE 5 Skip TC 6.3B.1.4_1.1 if UE supports SA and TS 38.521-2 TC 6.3A.1.1 has been executed. |
| 6.3B.1.4_1.2 | Minimum output power for inter-band EN-DC including FR2 (3 NR CCs) | Rel-15 | C012c | UEs supporting Inter-Band EN-DC including FR2 with 3 NR UL CCs | E012 |  | NOTE 5 Skip TC 6.3B.1.4_1.2 if UE supports SA and TS 38.521-2 TC 6.3A.1.2 has been executed. |
| 6.3B.1.4_1.3 | Minimum output power for inter-band EN-DC including FR2 (4 NR CCs) | Rel-15 | C012d | UEs supporting Inter-Band EN-DC including FR2 with 4 NR UL CCs | E013 |  | NOTE 5 Skip TC 6.3B.1.4_1.3 if UE supports SA and TS 38.521-2 TC 6.3A.1.3 has been executed. |
| 6.3B.1.4D | Minimum output power for inter-band EN-DC including FR2 for UL MIMO | Rel-15 | C012p | UEs supporting inter-band EN-DC including FR2 and UL MIMO | E043 | PC1 (NOTE 1) PC2 (NOTE 1) PC3 PC4 (NOTE 1) PC5 | NOTE 5 Skip TC 6.3B.1.4D if UE supports SA and TS 38.521-2 TC 6.3D.1 has been executed. |
| 6.3B.2.4 | Transmit OFF Power for inter-band EN-DC including FR2 (1 NR CC) | Rel-15 | C012 | UEs supporting inter-band EN-DC including FR2 with 1 NR UL CC | E005 |  |  |
| 6.3B.2.4_1 | Void |  |  |  |  |  |  |
| 6.3B.2.4D | Transmit OFF Power for inter-band EN-DC including FR2 for UL-MIMO | Rel-15 | C012p | UEs supporting inter-band EN-DC including FR2 and UL MIMO | E043 | PC1 PC2 (NOTE 1) PC3 PC4 (NOTE 1) PC5 (NOTE 1) | NOTE 5 Skip TC 6.3B.2.4D if UE supports SA and TS 38.521-2 TC 6.3D.2 has been executed. |
| 6.3B.3.1 | Transmit ON/OFF time mask for intra-band contiguous EN-DC | Rel-15 | C009 | UEs supporting intra-band contiguous EN-DC (2UL CCs) | E003 |  | NOTE 5 Skip TC 6.3B.3.1 if UE supports SA and TS 38.521-1 TC 6.3.3.2 has been executed. |
| 6.3B.3.2 | Transmit ON/OFF time mask for intra-band non-contiguous EN-DC | Rel-15 | C010 | UEs supporting intra-band non-contiguous EN-DC (2UL CCs) | E004 |  | NOTE 5 Skip TC 6.3B.3.2 if UE supports SA and TS 38.521-1 TC 6.3.3.2 has been executed. |
| 6.3B.3.3 | Transmit ON/OFF time mask for inter-band EN-DC within FR1 (1 NR CC) | Rel-15 | C011b | UEs supporting inter-band EN-DC within FR1 with 1 NR UL CC | E005b |  | NOTE 5 Skip TC 6.3B.3.3 if UE supports SA and TS 38.521-1 TC 6.3.3.2 has been executed. |
| 6.3B.3.4 | Transmit ON/OFF time mask for inter-band EN-DC including FR2 (1 NR CC) | Rel-15 | C012 | UEs supporting inter-band EN-DC including FR2 with 1 NR UL CC | E010 |  | NOTE 5 Skip TC 6.3B.3.4 if UE supports SA and TS 38.521-2 TC 6.3.3.2 has been executed. |
| 6.3B.3.4_1 | Transmit ON/OFF time mask for inter-band EN-DC including FR2 (>1 NR CC) |  |  |  |  |  |  |
| 6.3B.3.4_1.1 | Transmit ON/OFF time mask for inter-band EN-DC including FR2 (2 NR CCs) | Rel-15 | C012b | UEs supporting inter-band EN-DC including FR2 with 2 NR UL CCs | E011 |  | NOTE 5 Skip TC 6.3B.3.4_1.1 if UE supports SA and TS 38.521-2 TC 6.3A.3.1.1 has been executed. |
| 6.3B.3_1.1 | E-UTRA and NR switching time mask for switching between two uplink carriers for inter-band EN-DC | Rel-16 | C126a | UEs supporting inter-band EN-DC within FR1 with 1 NR UL CC and dynamic UL Tx switching | E031b |  | NOTE 1 |
| 6.3B.4.1 | PRACH time mask for intra-band contiguous EN-DC | Rel-15 | C009 | UEs supporting intra-band contiguous EN-DC (2UL CCs) | E003 |  | NOTE 5 Skip TC 6.3B.4.1 if UE supports SA and TS 38.521-1 TC 6.3.3.4 has been executed. |
| 6.3B.4.2 | PRACH Time Mask for intra-band non-contiguous EN-DC | Rel-15 | C010 | UEs supporting intra-band non-contiguous EN-DC (2UL CCs) | E004 |  | NOTE 5 Skip TC 6.3B.4.2 if UE supports SA and TS 38.521-1 TC 6.3.3.4 has been executed. |
| 6.3B.4.3 | PRACH Time Mask for inter-band EN-DC within FR1 (1 NR CC) | Rel-15 | C011b | UEs supporting inter-band EN-DC within FR1 with 1 NR UL CC | E005b |  | NOTE 5 Skip TC 6.3B.4.3 if UE supports SA and TS 38.521-1 TC 6.3.3.4 has been executed. |
| 6.3B.4.4 | PRACH Time Mask for inter-band EN-DC including FR2 (1 NR CC) | Rel-15 | C012 | UEs supporting Inter-Band EN-DC including FR2 with 1 NR UL CC | E010 |  | NOTE 1 NOTE 5 Skip TC 6.3B.4.4 if UE supports SA and TS 38.521-2 TC 6.3.3.4 has been executed. |
| 6.3B.8.1.1 | Absolute Power Tolerance for intra-band contiguous EN-DC | Rel-15 | C009 | UEs supporting intra-band contiguous EN-DC (2 UL CCs) | E003 |  | NOTE 5 Skip TC 6.3B.8.1.1 if UE supports SA and TS 38.521-1 TC 6.3.4.2 has been executed. |
| 6.3B.8.1.2 | Absolute Power Tolerance for intra-band non-contiguous EN-DC | Rel-15 | C010 | UEs supporting intra-band non-contiguous EN-DC (2 UL CCs) | E004 |  | NOTE 5 Skip TC 6.3B.8.1.2 if UE supports SA and TS 38.521-1 TC 6.3.4.2 has been executed. |
| 6.3B.8.1.3 | Absolute Power Tolerance for inter-band EN-DC within FR1 (1 NR CC) | Rel-15 | C011b | UEs supporting inter-band EN-DC within FR1 with 1 NR UL CC | E005b |  | NOTE 5 Skip TC 6.3B.8.1.3 if UE supports SA and TS 38.521-1 TC 6.3.4.2 has been executed. |
| 6.3B.8.1.4 | Absolute Power Tolerance for inter-band EN-DC including FR2 (1 NR CC) | Rel-15 | C011b | UEs supporting inter-band EN-DC within FR1 with 1 NR UL CC | E005b |  | NOTE 5 Skip TC 6.3B.8.1.4 if UE supports SA and TS 38.521-2 TC 6.3.4.2 has been executed. |
| 6.3B.8.1.4_1 | Absolute power tolerance for inter-band EN-DC including FR2 (>1 NR CC) |  |  |  |  |  |  |
| 6.3B.8.1.4_1.1 | Absolute power tolerance for inter-band EN-DC including FR2 (2 NR CC) | Rel-16 | C012b | UEs supporting Inter-Band EN-DC including FR2 with 2 NR UL CCs | E011 | PC1 (NOTE 1) PC2 (NOTE 1) PC3 PC4 (NOTE 1) PC5 (NOTE 1) | NOTE 5 Skip TC 6.3B.8.1.4_1.1 if UE supports SA and TS 38.521-2 TC 6.3A.4.2.1 has been executed. |
| 6.3B.8.1.4_1.2 | Absolute power tolerance for inter-band EN-DC including FR2 (3 NR CC) | Rel-16 | C012c | UEs supporting Inter-Band EN-DC including FR2 with 3 NR UL CCs | E012 | PC1 (NOTE 1) PC2 (NOTE 1) PC3 PC4 (NOTE 1) PC5 (NOTE 1) | NOTE 5 Skip TC 6.3B.8.1.4_1.2 if UE supports SA and TS 38.521-2 TC 6.3A.4.2.2 has been executed. |
| 6.3B.8.1.4_1.3 | Absolute power tolerance for inter-band EN-DC including FR2 (4 NR CC) | Rel-16 | C012d | UEs supporting Inter-Band EN-DC including FR2 with 4 NR UL CCs | E013 | PC1 (NOTE 1) PC2 (NOTE 1) PC3 PC4 (NOTE 1) PC5 (NOTE 1) | NOTE 5 Skip TC 6.3B.8.1.4_1.3 if UE supports SA and TS 38.521-2 TC 6.3A.4.2.3 has been executed. |
| 6.3B.8.2.1 | Relative Power Tolerance for intra-band contiguous EN-DC | Rel-15 | C009 | UEs supporting intra-band contiguous EN-DC (2 UL CCs) | E003 |  | NOTE 5 Skip TC 6.3B.8.2.1 if UE supports SA and TS 38.521-1 TC 6.3.4.3 has been executed. |
| 6.3B.8.2.2 | Relative Power Tolerance for intra-band non-contiguous EN-DC | Rel-15 | C010 | UEs supporting intra-band non-contiguous EN-DC (2 UL CCs) | E004 |  | NOTE 5 Skip TC 6.3B.8.2.2 if UE supports SA and TS 38.521-1 TC 6.3.4.3 has been executed. |
| 6.3B.8.2.3 | Relative Power Tolerance for inter-band EN-DC within FR1 (1 NR CC) | Rel-15 | C011b | UEs supporting inter-band EN-DC within FR1 with 1 NR UL CC | E005b |  | NOTE 5 Skip TC 6.3B.8.2.3 if UE supports SA and TS 38.521-1 TC 6.3.4.3 has been executed. |
| 6.3B.8.2.4 | Relative Power Tolerance for inter-band EN-DC including FR2 (1 NR CC) | Rel-15 | FFS | FFS | FFS |  | NOTE 1 |
| 6.3B.8.3.1 | Aggregate Power Tolerance for intra-band contiguous EN-DC | Rel-15 | C009 | UEs supporting intra-band contiguous EN-DC (2 UL CCs) | E003 |  | NOTE 5 Skip TC 6.3B.8.3.1 if UE supports SA and TS 38.521-1 TC 6.3.4.4 has been executed. |
| 6.3B.8.3.2 | Aggregate Power Tolerance for intra-band non-contiguous EN-DC | Rel-15 | C010 | UEs supporting intra-band non-contiguous EN-DC (2 UL CCs) | E004 |  | NOTE 5 Skip TC 6.3B.8.3.2 if UE supports SA and TS 38.521-1 TC 6.3.4.4 has been executed. |
| 6.3B.8.3.3 | Aggregate Power Tolerance for inter-band EN-DC within FR1 (1 NR CC) | Rel-15 | C011b | UEs supporting inter-band EN-DC within FR1 with 1 NR UL CC | E005b |  | NOTE 5 Skip TC 6.3B.8.3.3 if UE supports SA and TS 38.521-1 TC 6.3.4.4 has been executed. |
| 6.3B.8.3.4 | Aggregate Power Tolerance for inter-band EN-DC including FR2 (1 NR CC) | Rel-15 | C011b | UEs supporting inter-band EN-DC within FR1 with 1 NR UL CC | E005b |  | NOTE 5 Skip TC 6.3B.8.3.4 if UE supports SA and TS 38.521-2 TC 6.3.4.4 has been executed. |
| 6.3B.8.3.4_1 | Aggregate power tolerance for inter-band EN-DC including FR2 (>1 NR CC) |  |  |  |  |  |  |
| 6.3B.8.3.4_1.1 | Aggregate power tolerance for inter-band EN-DC including FR2 (2 NR CC) | Rel-16 | C012b | UEs supporting Inter-Band EN-DC including FR2 with 2 NR UL CCs | E011 | PC1 (NOTE 1) PC2 (NOTE 1) PC3 PC4 (NOTE 1) PC5 (NOTE 1) | NOTE 5 Skip TC 6.3B.8.3.4_1.1 if UE supports SA and TS 38.521-2 TC 6.3A.4.4.1 has been executed. |
| 6.3B.8.3.4_1.2 | Aggregate power tolerance for inter-band EN-DC including FR2 (3 NR CC) | Rel-16 | C012c | UEs supporting Inter-Band EN-DC including FR2 with 3 NR UL CCs | E012 | PC1 (NOTE 1) PC2 (NOTE 1) PC3 PC4 (NOTE 1) PC5 (NOTE 1) | NOTE 5 Skip TC 6.3B.8.3.4_1.2 if UE supports SA and TS 38.521-2 TC 6.3A.4.4.2 has been executed. |
| 6.3B.8.3.4_1.3 | Aggregate power tolerance for inter-band EN-DC including FR2 (4 NR CC) | Rel-16 | C012d | UEs supporting Inter-Band EN-DC including FR2 with 4 NR UL CCs | E013 | PC1 (NOTE 1) PC2 (NOTE 1) PC3 PC4 (NOTE 1) PC5 (NOTE 1) | NOTE 5 Skip TC 6.3B.8.3.4_1.3 if UE supports SA and TS 38.521-2 TC 6.3A.4.4.3 has been executed. |
| 6.4B | Transmit Signal Quality for DC |  |  |  |  |  |  |
| 6.4B.1.1 | Frequency Error for intra-band contiguous EN-DC | Rel-15 | C009 | UEs supporting Intra-Band Contiguous EN-DC (2UL CCs) | E003 |  | NOTE 5 Skip TC 6.4B.1.1 if UE supports SA and TS 38.521-1 TC 6.4.1 has been executed. |
| 6.4B.1.2 | Frequency Error for intra-band non-contiguous EN-DC | Rel-15 | C010 | UEs supporting intra-band non-contiguous EN-DC (2UL CCs) | E004 |  | NOTE 5 Skip TC 6.4B.1.2 if UE supports SA and TS 38.521-1 TC 6.4.1 has been executed. |
| 6.4B.1.3 | Frequency error for Inter-band EN-DC within FR1 (1 NR CC) | Rel-15 | C011b | UEs supporting inter-band EN-DC within FR1 with 1 NR UL CC | E005b |  | NOTE 5 Skip TC 6.4B.1.3 if UE supports SA and TS 38.521-1 TC 6.4.1 has been executed. |
| 6.4B.1.4 | Frequency Error for inter-band EN-DC including FR2 (1 NR CC) | Rel-15 | C012 | UEs supporting Inter-Band EN-DC including FR2 with 1 NR UL CC | E010 |  | NOTE 5 Skip TC 6.4B.1.4 if UE supports SA and TS 38.521-2 TC 6.4.1 has been executed. |
| 6.4B.1.4_1 | Frequency Error for Inter-band EN-DC including FR2 (>1 NR CC) |  |  |  |  |  |  |
| 6.4B.1.4_1.1 | Frequency Error for inter-band EN-DC including FR2 (2 NR CCs) | Rel-15 | C012b | UEs supporting Inter-Band EN-DC including FR2 with 2 NR UL CCs | E011 |  | NOTE 5 Skip TC 6.4B.1.4_1.1 if UE supports SA and TS 38.521-2 TC 6.4A.1.1 has been executed. |
| 6.4B.1.4_1.2 | Frequency Error for inter-band EN-DC including FR2 (3 NR CCs) | Rel-15 | C012c | UEs supporting Inter-Band EN-DC including FR2 with 3 NR UL CCs | E012 |  | NOTE 5 Skip TC 6.4B.1.4_1.2 if UE supports SA and TS 38.521-2 TC 6.4A.1.2 has been executed. |
| 6.4B.1.4_1.3 | Frequency Error for inter-band EN-DC including FR2 (4 NR CCs) | Rel-15 | C012d | UEs supporting Inter-Band EN-DC including FR2 with 4 NR UL CCs | E013 |  | NOTE 5 Skip TC 6.4B.1.4_1.3 if UE supports SA and TS 38.521-2 TC 6.4A.1.3 has been executed. |
| 6.4B.1.4D | Frequency error for inter-band EN-DC including FR2 for UL-MIMO | Rel-15 | C012p | UEs supporting Inter-Band EN-DC including FR2 and UL MIMO | E043 |  | NOTE 5 Skip TC 6.4B.1.4D if UE supports SA and TS 38.521-2 TC 6.4D.1 has been executed. |
| 6.4B.2.1.1 | Error Vector Magnitude for intra-band contiguous EN-DC | Rel-15 | C009 | UEs supporting intra-band contiguous EN-DC (2UL CCs) | E003 |  | NOTE 5 Skip TC 6.4B.2.1.1 if UE supports SA and TS 38.521-1 TC 6.4.2.1 has been executed. |
| 6.4B.2.1.2 | Carrier Leakage for intra-band contiguous EN-DC | Rel-15 | C009 | UEs supporting intra-band contiguous EN-DC (2UL CCs) | E003 |  | NOTE 5 Skip TC 6.4B.2.1.2 if UE supports SA and TS 38.521-1 TC 6.4.2.2 has been executed. |
| 6.4B.2.1.3 | In-band Emissions for intra-band contiguous EN-DC | Rel-15 | C009 | UEs supporting intra-band contiguous EN-DC (2UL CCs) | E003 |  |  |
| 6.4B.2.1.4 | EVM Equalizer Flatness for intra-band contiguous EN-DC | Rel-15 | C009 | UEs supporting intra-band contiguous EN-DC (2UL CCs) | E003 |  | NOTE 5 Skip TC 6.4B.2.1.4 if UE supports SA and TS 38.521-1 TC 6.4.2.4 has been executed. |
| 6.4B.2.2.1 | Error Vector Magnitude for intra-band non-contiguous EN-DC | Rel-15 | C010 | UEs supporting intra-band non-contiguous EN-DC (2UL CCs) | E004 |  | NOTE 5 Skip TC 6.4B.2.2.1 if UE supports SA and TS 38.521-1 TC 6.4.2.1 has been executed. |
| 6.4B.2.2.2 | Carrier Leakage for intra-band non-contiguous EN-DC | Rel-15 | C010 | UEs supporting intra-band non-contiguous EN-DC (2UL CCs) | E004 |  | NOTE 5 Skip TC 6.4B.2.2.2 if UE supports SA and TS 38.521-1 TC 6.4.2.2has been executed. |
| 6.4B.2.2.3 | In-band Emissions for intra-band non-contiguous EN-DC | Rel-15 | C010 | UEs supporting intra-band non-contiguous EN-DC (2UL CCs) | E004 |  | NOTE 5 Skip TC 6.4B.2.2.3 if UE supports SA and TS 38.521-1 TC 6.4.2.3 has been executed. |
| 6.4B.2.2.4 | EVM Equalizer Flatness for intra-band non-contiguous EN-DC | Rel-15 | C010 | UEs supporting intra-band non-contiguous EN-DC (2UL CCs) | E004 |  | NOTE 5 Skip TC 6.4B.2.2.4 if UE supports SA and TS 38.521-1 TC 6.4.2.4 has been executed. |
| 6.4B.2.3.1 | Error Vector Magnitude for inter-band EN-DC within FR1 (1 NR CC) | Rel-15 | C011b | UEs supporting inter-band EN-DC within FR1 with 1 NR UL CC | E005b |  | NOTE 5 Skip TC 6.4B.2.3.1 if UE supports SA and TS 38.521-1 TC 6.4.2.1 has been executed. |
| 6.4B.2.3.2 | Carrier Leakage for inter-band EN-DC within FR1 (1 NR CC) | Rel-15 | C011b | UEs supporting inter-band EN-DC within FR1 with 1 NR UL CC | E005b |  | NOTE 5 Skip TC 6.4B.2.3.2 if UE supports SA and TS 38.521-1 TC 6.4.2.2 has been executed. |
| 6.4B.2.3.3 | In-band Emissions for inter-band EN-DC within FR1 (1 NR CC) | Rel-15 | C011b | UEs supporting inter-band EN-DC within FR1 with 1 NR UL CC | E005b |  | NOTE 5 Skip TC 6.4B.2.3.3 if UE supports SA and TS 38.521-1 TC 6.4.2.3 has been executed. |
| 6.4B.2.3.4 | EVM Equalizer Flatness for inter-band EN-DC within FR1 (1 NR CC) | Rel-15 | C011b | UEs supporting inter-band EN-DC within FR1 with 1 NR UL CC | E005b |  | NOTE 5 Skip TC 6.4B.2.3.4 if UE supports SA and TS 38.521-1 TC 6.4.2.4 has been executed. |
| 6.4B.2.4.1 | Error Vector Magnitude for inter-band EN-DC including FR2 (1 NR CC) | Rel-15 | C012 | UEs supporting Inter-band including FR2 with 1 NR UL CC | E010 |  | NOTE 1 NOTE 5 Skip TC 6.4B.2.4.1 if UE supports SA and TS 38.521-2 TC 6.4.2.1 has been executed. |
| 6.4B.2.4.1a | Error Vector Magnitude with Power Boost for inter-band EN-DC including FR2 (1 NR CC) | Rel-16 | C012w | UEs supporting inter-band EN-DC including FR2 with 1 NR UL CC and mpr-PowerBoost-FR2-r16 | E010 | PC1 PC2 PC3 PC4 | NOTE 1 NOTE 5 Skip TC 6.4B.2.4.1a if UE supports SA and TS 38.521-2 TC 6.4.2.1_1 has been executed. |
| 6.4B.2.4.1_1 | Error Vector Magnitude for inter-band EN-DC including FR2 (>1 NR CC) |  |  |  |  |  |  |
| 6.4B.2.4.1_1.1 | Error Vector Magnitude for inter-band EN-DC including FR2 (2 NR CCs) | Rel-15 | FFS | FFS | FFS |  | NOTE 1 NOTE 5 Skip TC 6.4B.2.4.1_1.1 if UE supports SA and TS 38.521-2 TC 6.4A.2.1.1 has been executed. |
| 6.4B.2.4.1_1.2 | Error Vector Magnitude for inter-band EN-DC including FR2 (3 NR CCs) | Rel-15 | FFS | FFS | FFS |  | NOTE 1 NOTE 5 Skip TC 6.4B.2.4.1_1.2 if UE supports SA and TS 38.521-2 TC 6.4A.2.1.2 has been executed. |
| 6.4B.2.4.1_1.3 | Error Vector Magnitude for inter-band EN-DC including FR2 (4 NR CCs) | Rel-15 | FFS | FFS | FFS |  | NOTE 1 NOTE 5 Skip TC 6.4B.2.4.1_1.3 if UE supports SA and TS 38.521-2 TC 6.4A.2.1.3 has been executed. |
| 6.4B.2.4.1D | Error Vector Magnitude for inter-band EN-DC including FR2 for UL MIMO | FFS | FFS | FFS | FFS |  | NOTE 1 |
| 6.4B.2.4.2 | Carrier Leakage for inter-band EN-DC including FR2 (1 NR CC) | Rel-15 | C012 | UEs supporting Inter-band including FR2 with 1 NR UL CC | E010 |  | NOTE 5 Skip TC 6.4B.2.4.2 if UE supports SA and TS 38.521-2 TC 6.4.2.2 has been executed. |
| 6.4B.2.4.2_1 | Carrier Leakage for inter-band EN-DC including FR2 (>1 NR CC) |  |  |  |  |  |  |
| 6.4B.2.4.2_1.1 | Carrier Leakage for inter-band EN-DC including FR2 (2 NR CCs) | Rel-15 | FFS | FFS | FFS |  | NOTE 1 NOTE 5 Skip TC 6.4B.2.4.2_1.1 if UE supports SA and TS 38.521-2 TC 6.4A.2.2.1 has been executed. |
| 6.4B.2.4.2_1.2 | Carrier Leakage for inter-band EN-DC including FR2 (3 NR CCs) | Rel-15 | FFS | FFS | FFS |  | NOTE 1 NOTE 5 Skip TC 6.4B.2.4.2_1.2 if UE supports SA and TS 38.521-2 TC 6.4A.2.2.2 has been executed. |
| 6.4B.2.4.2_1.3 | Carrier Leakage for inter-band EN-DC including FR2 (4 NR CCs) | Rel-15 | FFS | FFS | FFS |  | NOTE 1 NOTE 5 Skip TC 6.4B.2.4.2_1.3 if UE supports SA and TS 38.521-2 TC 6.4A.2.2.3 has been executed. |
| 6.4B.2.4.2D | Carrier Leakage for inter-band EN-DC including FR2 for UL MIMO | FFS | FFS | FFS | FFS |  | NOTE 1 |
| 6.4B.2.4.3 | In-band Emissions for inter-band EN-DC including FR2 (1 NR CC) | Rel-15 | C012 | UEs supporting Inter-band including FR2 with 1 NR UL CC | E010 |  | NOTE 1 NOTE 5 Skip TC 6.4B.2.4.3 if UE supports SA and TS 38.521-2 TC 6.4.2.3 has been executed. |
| 6.4B.2.4.3D | In-band Emissions for inter-band EN-DC including FR2 for UL MIMO | FFS | FFS | FFS | FFS |  | NOTE 1 |
| 6.4B.2.4.3_1 | In-band Emissions for inter-band EN-DC including FR2 (>1 NR CC) |  |  |  |  |  |  |
| 6.4B.2.4.3_1.1 | In-band Emissions for inter-band EN-DC including FR2 (2 NR CCs) | FFS | FFS | FFS | FFS |  | NOTE 1 |
| 6.4B.2.4.3_1.2 | In-band Emissions for inter-band EN-DC including FR2 (3 NR CCs) | FFS | FFS | FFS | FFS |  | NOTE 1 |
| 6.4B.2.4.3_1.3 | In-band Emissions for inter-band EN-DC including FR2 (4 NR CCs) | FFS | FFS | FFS | FFS |  | NOTE 1 |
| 6.4B.2.4.4 | EVM Equalizer Flatness for inter-band EN-DC including FR2 (1 NR CC) | Rel-15 | C012 | UEs supporting Inter-band including FR2 with 1 NR UL CC | E010 |  | NOTE 1 NOTE 5 Skip TC 6.4B.2.4.4 if UE supports SA and TS 38.521-2 TC 6.4.2.4 has been executed. |
| 6.4B.2.4.4D | EVM Equalizer Flatness for inter-band EN-DC including FR2 for UL MIMO | FFS | FFS | FFS | FFS |  | NOTE 1 |
| 6.4B.2.4.5 | EVM spectral flatness for pi/2 BPSK modulation for inter-band EN-DC including FR2 (1 NR CC) | Rel-15 | C012f | UEs supporting Inter-band including FR2 with 1 NR UL CC and pi/2 BPSK modulation | E010 |  | NOTE 1 NOTE 5 Skip TC 6.4B.2.4.5 if UE supports SA and TS 38.521-2 TC 6.4.2.5 has been executed. |
| 6.5B | Output RF spectrum emissions for DC |  |  |  |  |  |  |
| 6.5B.1.1 | Occupied bandwidth for Intra-Band Contiguous EN-DC | Rel-15 | C009 | UEs supporting intra-band contiguous EN-DC (2UL CCs) | E003 |  | NOTE 1 |
| 6.5B.1.2 | Occupied bandwidth for Intra-Band Non-Contiguous EN-DC | Rel-15 | C010 | UEs supporting intra-band non-contiguous EN-DC (2UL CCs) | E004 |  | NOTE 1 NOTE 5 Skip TC 6.5B.1.2 if UE supports SA and TS 38.521-1 TC 6.5.1 has been executed. |
| 6.5B.1.3 | Occupied bandwidth for Inter-Band EN-DC within FR1 (1 NR CC) | Rel-15 | C011b | UEs supporting inter-band EN-DC within FR1 with 1 NR UL CC | E005b |  | NOTE 5 Skip TC 6.5B.1.3 if UE supports SA and TS 38.521-1 TC 6.5.1 has been executed. |
| 6.5B.1.4 | Occupied bandwidth for Inter-Band EN-DC including FR2 (1 NR CC) | Rel-15 | C012 | UEs supporting Inter-band including FR2 with 1 NR UL CC | E010 |  | NOTE 5 Skip TC 6.5B.1.4 if UE supports SA and TS 38.521-2 TC 6.5.1 has been executed. |
| 6.5B.1.4_1 | Occupied bandwidth for Inter-band EN-DC including FR2 (>1 NR CC) |  |  |  |  |  |  |
| 6.5B.1.4_1.1 | Occupied bandwidth for inter-band EN-DC including FR2 (2 NR CCs) | Rel-15 | FFS | FFS | FFS |  | NOTE 1 NOTE 5 Skip TC 6.5B.1.4_1.1 if UE supports SA and TS 38.521-2 TC 6.5A.1.1 has been executed. |
| 6.5B.1.4_1.2 | Occupied bandwidth for inter-band EN-DC including FR2 (3 NR CCs) | Rel-15 | FFS | FFS | FFS |  | NOTE 1 NOTE 5 Skip TC 6.5B.1.4_1.2 if UE supports SA and TS 38.521-2 TC 6.5A.1.2 has been executed. |
| 6.5B.1.4_1.3 | Occupied bandwidth for inter-band EN-DC including FR2 (4 NR CCs) | Rel-15 | FFS | FFS | FFS |  | NOTE 1 NOTE 5 Skip TC 6.5B.1.4_1.3 if UE supports SA and TS 38.521-2 TC 6.5A.1.3 has been executed. |
| 6.5B.1.4D | Occupied bandwidth for inter-band EN-DC including FR2 for UL MIMO | Rel-15 | C012p | UEs supporting Inter-Band EN-DC including FR2 and UL MIMO | E043 |  | NOTE 1 NOTE 5 Skip TC 6.5B.1.4D if UE supports SA and TS 38.521-2 TC 6.5D.1 has been executed. |
| 6.5B.2.1.1 | Spectrum emissions mask for intra-band contiguous EN-DC | Rel-15 | C009 | UEs supporting intra-band contiguous EN-DC (2UL CCs) | E003 |  |  |
| 6.5B.2.1.2 | Additional spectrum emissions mask for intra-band contiguous EN-DC | Rel-15 | C009 | UEs supporting intra-band contiguous EN-DC (2UL CCs) | E003 |  |  |
| 6.5B.2.1.3 | Adjacent channel leakage ratio for intra-band contiguous EN-DC | Rel-15 | C009 | UEs supporting intra-band contiguous EN-DC (2UL CCs) | E003 |  |  |
| 6.5B.2.2.1 | Spectrum emissions mask for intra-band non-contiguous EN-DC | Rel-15 | C010 | UEs supporting intra-band non-contiguous EN-DC (2UL CCs) | E004 |  |  |
| 6.5B.2.2.2 | Additional Spectrum emissions mask for intra-band non-contiguous EN-DC | FFS | FFS | FFS | FFS |  | NOTE 1 |
| 6.5B.2.2.3 | Adjacent channel leakage ratio for intra-band non-contiguous EN-DC | Rel-15 | C010 | UEs supporting intra-band non-contiguous EN-DC (2UL CCs) | E004 |  |  |
| 6.5B.2.3.1 | Spectrum emissions mask for Inter-band EN-DC within FR1 (1 NR CC) | Rel-15 | C011b | UEs supporting Inter-band EN-DC within FR1 with 1 NR UL CC | E005b |  | NOTE 5 Skip TC 6.5B.2.3.1 if UE supports SA and TS 38.521-1 TC 6.5.2.2 has been executed. |
| 6.5B.2.3.2 | Additional Spectrum emissions mask for Inter-band EN-DC within FR1 (1 NR CC) | Rel-15 | C011b | UEs supporting Inter-band EN-DC within FR1 with 1 NR UL CC | E005b |  | NOTE 5 Skip TC 6.5B.2.3.2 if UE supports SA and TS 38.521-1 TC 6.5.2.3 has been executed. |
| 6.5B.2.3.3.1 | NR - Adjacent channel leakage ratio for inter-band EN-DC within FR1 (1 NR CC) | Rel-15 | C011b | UEs supporting Inter-band EN-DC within FR1 with 1 NR UL CC | E005b |  | NOTE 5 Skip TC 6.5B.2.3.3.1 if UE supports SA and TS 38.521-1 TC 6.5.2.4.1 has been executed. |
| 6.5B.2.3.3.2 | UTRA - Adjacent channel leakage ratio for inter-band EN-DC within FR1 (1 NR CC) | Rel-15 | C011b | UEs supporting Inter-band EN-DC within FR1 with 1 NR UL CC | E005b |  | NOTE 5 Skip TC 6.5B.2.3.3.2 if UE supports SA and TS 38.521-1 TC 6.5.2.4.2 has been executed. |
| 6.5B.2.4.1 | Spectrum emissions mask for Inter-band EN-DC including FR2 (1 NR CC) | Rel-15 | C012 | UEs supporting Inter-band including FR2 with 1 NR UL CC | E010 |  | NOTE 5 Skip TC 6.5B.2.4.1 if UE supports SA and TS 38.521-2 TC 6.5.2.1 has been executed. |
| 6.5B.2.4.1a | Spectrum emissions mask with Power Boost for Inter-band EN-DC including FR2 (1 NR CC) | Rel-16 | C012w | UEs supporting inter-band EN-DC including FR2 with 1 NR UL CC and mpr-PowerBoost-FR2-r16 | E010 | PC1 (NOTE 1) PC2 (NOTE 1) PC3 PC4 (NOTE 1) | NOTE 5 Skip TC 6.5B.2.4.1a if UE supports SA and TS 38.521-2 TC 6.5.2.1_1 has been executed. |
| 6.5B.2.4.1_1 | Spectrum emissions mask for Inter-band EN-DC including FR2 (>1 NR CC) |  |  |  |  |  |  |
| 6.5B.2.4.1_1.1 | Spectrum emissions mask for Inter-band EN-DC including FR2 (2 NR CCs) | Rel-15 | C012b | UEs supporting Inter-Band EN-DC including FR2 with 2 NR UL CCs | E011 |  | NOTE 5 Skip TC 6.5B.2.4.1_1.1 if UE supports SA and TS 38.521-2 TC 6.5A.2.1.1 has been executed. |
| 6.5B.2.4.1_1.2 | Spectrum emissions mask for Inter-band EN-DC including FR2 (3 NR CCs) | Rel-15 | C012c | UEs supporting Inter-Band EN-DC including FR2 with 3 NR UL CCs | E012 |  | NOTE 5 Skip TC 6.5B.2.4.1_1.2 if UE supports SA and TS 38.521-2 TC 6.5A.2.1.2 has been executed. |
| 6.5B.2.4.1_1.3 | Spectrum emissions mask for Inter-band EN-DC including FR2 (4 NR CCs) | Rel-15 | C012d | UEs supporting Inter-Band EN-DC including FR2 with 4 NR UL CCs | E013 |  | NOTE 5 Skip TC 6.5B.2.4.1_1.3 if UE supports SA and TS 38.521-2 TC 6.5A.2.1.3 has been executed. |
| 6.5B.2.4.1D | Spectrum emissions mask for inter-band EN-DC including FR2 for UL MIMO | Rel-15 | C012p | UEs supporting Inter-Band EN-DC including FR2 and UL MIMO | E043 | PC1 PC2 (NOTE 1) PC3 PC4 (NOTE 1) PC5 | NOTE 5 Skip TC 6.5B.2.4.1D if UE supports SA and TS 38.521-2 TC 6.5D.2.1 has been executed. |
| 6.5B.2.4.3 | Adjacent channel leakage ratio for Inter-band EN-DC including FR2 (1 NR CC) | Rel-15 | C012 | UEs supporting Inter-band including FR2 with 1 NR UL CC | E010 |  | NOTE 5 Skip TC 6.5B.2.4.3 if UE supports SA and TS 38.521-2 TC 6.5.2.3 has been executed. |
| 6.5B.2.4.3_1 | Adjacent channel leakage ratio for Inter-band EN-DC including FR2 (>1 NR CC) |  |  |  |  |  |  |
| 6.5B.2.4.3_1.1 | Adjacent channel leakage ratio for Inter-band EN-DC including FR2 (2 NR CCs) | Rel-15 | C012b | UEs supporting Inter-Band EN-DC including FR2 with 2 NR UL CCs | E011 |  | NOTE 5 Skip TC 6.5B.2.4.3_1.1 if UE supports SA and TS 38.521-2 TC 6.5A.2.2.1 has been executed. |
| 6.5B.2.4.3_1.2 | Adjacent channel leakage ratio for Inter-band EN-DC including FR2 (3 NR CCs) | Rel-15 | C012c | UEs supporting Inter-Band EN-DC including FR2 with 3 NR UL CCs | E012 |  | NOTE 5 Skip TC 6.5B.2.4.3_1.2 if UE supports SA and TS 38.521-2 TC 6.5A.2.2.2 has been executed. |
| 6.5B.2.4.3_1.3 | Adjacent channel leakage ratio for Inter-band EN-DC including FR2 (4 NR CCs) | Rel-15 | C012d | UEs supporting Inter-Band EN-DC including FR2 with 4 NR UL CCs | E013 |  | NOTE 5 Skip TC 6.5B.2.4.3_1.3 if UE supports SA and TS 38.521-2 TC 6.5A.2.2.3 has been executed. |
| 6.5B.2.4.3_1.4 | Adjacent channel leakage ratio for Inter-band EN-DC including FR2 (5 NR CCs) | Rel-15 | C012i | UEs supporting Inter-Band EN-DC including FR2 with 5 NR UL CCs | E014a |  | NOTE 1 NOTE 5 Skip TC 6.5B.2.4.3_1.4 if UE supports SA and TS 38.521-2 TC 6.5A.2.2.4 has been executed. |
| 6.5B.2.4.3_1.5 | Adjacent channel leakage ratio for Inter-band EN-DC including FR2 (6 NR CCs) | Rel-15 | C012j | UEs supporting Inter-Band EN-DC including FR2 with 6 NR UL CCs | E039 |  | NOTE 1 NOTE 5 Skip TC 6.5B.2.4.3_1.5 if UE supports SA and TS 38.521-2 TC 6.5A.2.2.5 has been executed. |
| 6.5B.2.4.3_1.6 | Adjacent channel leakage ratio for Inter-band EN-DC including FR2 (7 NR CCs) | Rel-15 | C012k | UEs supporting Inter-Band EN-DC including FR2 with 7 NR UL CCs | E040 |  | NOTE 1 NOTE 5 Skip TC 6.5B.2.4.3_1.6 if UE supports SA and TS 38.521-2 TC 6.5A.2.2.6 has been executed. |
| 6.5B.2.4.3_1.7 | Adjacent channel leakage ratio for Inter-band EN-DC including FR2 (8 NR CCs) | Rel-15 | C012l | UEs supporting Inter-Band EN-DC including FR2 with 8 NR UL CCs | E041 |  | NOTE 1 NOTE 5 Skip TC 6.5B.2.4.3_1.7 if UE supports SA and TS 38.521-2 TC 6.5A.2.2.7 has been |
| 6.5B.2.4D.3 | Adjacent channel leakage ratio for inter-band EN-DC including FR2 for UL MIMO | Rel-15 | C012p | UEs supporting Inter-Band EN-DC including FR2 and UL MIMO | E043 | PC1 PC2 (NOTE 1) PC3 PC4 (NOTE 1) PC5 | NOTE 5 Skip TC 6.5B.2.4D.3 if UE supports SA and TS 38.521-2 TC 6.5D.2.2 has been executed. |
| 6.5B.3.1.1 | General spurious emissions for intra-band contiguous EN-DC | Rel-15 | C009 | UEs supporting intra-band contiguous EN-DC (2UL CCs) | E003 |  | NOTE 5 Skip TC 6.5B.3.1.1 if UE supports SA and TS 38.521-1 TC 6.5.3.1 has been executed. |
| 6.5B.3.1.2 | Spurious emission band UE co-existence for intra-band contiguous EN-DC | Rel-15 | C009 | UEs supporting intra-band contiguous EN-DC (2UL CCs) | E003 |  |  |
| 6.5B.3.2.1 | General spurious emissions for intra-band non-contiguous EN-DC | Rel-15 | C010 | UEs supporting intra-band non-contiguous EN-DC (2UL CCs) | E004 |  | NOTE 5 Skip TC 6.5B.3.2.1 if UE supports SA and TS 38.521-1 TC 6.5.3.1 has been executed. |
| 6.5B.3.2.2 | Spurious emission band UE co-existence for intra-band non-contiguous EN-DC | Rel-15 | C010 | UEs supporting intra-band non-contiguous EN-DC (2UL CCs) | E004 |  |  |
| 6.5B.3.3.1 | General spurious emissions for Inter-band EN-DC within FR1 | Rel-15 | C011 | UEs supporting Inter-band EN-DC within FR1 (2UL CCs) | E005 |  | Test only one EN-DC combination per 5G NR band. Skip LTE anchor agnostic approach testing in TC 6.5B.3.3.1 if UE supports SA and TS 38.521-1 TC 6.5.3.1 has been executed. |
| 6.5B.3.3.2 | Spurious emission band UE co-existence for Inter-band within FR1 | Rel-15 | C011 | UEs supporting Inter-band EN-DC within FR1 (2UL CCs) | E005 |  | For LTE anchor agnostic approach testing in TC 6.5B.3.3.2: 1. NOTE 5 applied. 2. Skip the testing if UE supports SA and TS 38.521-1 TC 6.5.3.2 has been executed. |
| 6.5B.3.4.1 | General Spurious Emissions for Inter-band including FR2 (1 NR CC) | Rel-15 | C012 | UEs supporting Inter-band including FR2 with 1 NR UL CC | E010 |  | NOTE 5 Skip TC 6.5B.3.4.1 if UE supports SA and TS 38.521-2 TC 6.5.3.1 has been executed. |
| 6.5B.3.4.1a | General Spurious Emissions with Power Boost for Inter-band including FR2 (1 NR CC) | Rel-16 | C012w | UEs supporting inter-band EN-DC including FR2 with 1 NR UL CC and mpr-PowerBoost-FR2-r16 | E010 | PC1 (NOTE 1) PC2 (NOTE 1) PC3 PC4 (NOTE 1) | NOTE 5 Skip TC 6.5B.3.4.1a if UE supports SA and TS 38.521-2 TC 6.5.3.1_1 has been executed. |
| 6.5B.3.4.1_1 | General Spurious emissions for Inter-band EN-DC including FR2 (>1 NR CC) |  |  |  |  |  |  |
| 6.5B.3.4.1_1.1 | General Spurious Emissions for Inter-band EN-DC including FR2 (2 NR CCs) | Rel-15 | C012b | UEs supporting Inter-Band EN-DC including FR2 with 2 NR UL CCs | E011 |  | NOTE 5 Skip TC 6.5B.3.4.1_1.1 if UE supports SA and TS 38.521-2 TC 6.5A.3.1.1 has been executed. |
| 6.5B.3.4.1_1.2 | General Spurious Emissions for Inter-band EN-DC including FR2 (3 NR CCs) | Rel-15 | C012c | UEs supporting Inter-Band EN-DC including FR2 with 3 NR UL CCs | E012 |  | NOTE 5 Skip TC 6.5B.3.4.1_1.2 if UE supports SA and TS 38.521-2 TC 6.5A.3.1.2 has been executed. |
| 6.5B.3.4.1_1.3 | General Spurious Emissions for Inter-band EN-DC including FR2 (4 NR CCs) | Rel-15 | C012d | UEs supporting Inter-Band EN-DC including FR2 with 4 NR UL CCs | E013 |  | NOTE 5 Skip TC 6.5B.3.4.1_1.3 if UE supports SA and TS 38.521-2 TC 6.5A.3.1.3 has been executed. |
| 6.5B.3.4.1_1.4 | General Spurious Emissions for Inter-band EN-DC including FR2 (5 NR CCs) | Rel-15 | C012i | UEs supporting Inter-Band EN-DC including FR2 with 5 NR UL CCs | E014a |  | NOTE 5 Skip TC 6.5B.3.4.1_1.4 if UE supports SA and TS 38.521-2 TC 6.5A.3.1.4 has been executed. |
| 6.5B.3.4.1_1.5 | General Spurious Emissions for Inter-band EN-DC including FR2 (6 NR CCs) | Rel-15 | C012j | UEs supporting Inter-Band EN-DC including FR2 with 6 NR UL CCs | E039 |  | NOTE 5 Skip TC 6.5B.3.4.1_1.5 if UE supports SA and TS 38.521-2 TC 6.5A.3.1.5 has been executed. |
| 6.5B.3.4.1_1.6 | General Spurious Emissions for Inter-band EN-DC including FR2 (7 NR CCs) | Rel-15 | C012k | UEs supporting Inter-Band EN-DC including FR2 with 7 NR UL CCs | E040 |  | NOTE 5 Skip TC 6.5B.3.4.1_1.6 if UE supports SA and TS 38.521-2 TC 6.5A.3.1.6 has been executed. |
| 6.5B.3.4.1_1.7 | General Spurious Emissions for Inter-band EN-DC including FR2 (8 NR CCs) | Rel-15 | C012l | UEs supporting Inter-Band EN-DC including FR2 with 8 NR UL CCs | E041 |  | NOTE 5 Skip TC 6.5B.3.4.1_1.7 if UE supports SA and TS 38.521-2 TC 6.5A.3.1.7 has been executed. |
| 6.5B.3.4.1D | General Spurious Emissions for inter-band EN-DC including FR2 for UL MIMO | Rel-15 | C012p | UEs supporting Inter-Band EN-DC including FR2 and UL MIMO | E043 | PC1 PC2 (NOTE 1) PC3 PC4 (NOTE 1) PC5 (NOTE 1) | NOTE 5 Skip TC 6.5B.3.4.1D if UE supports SA and TS 38.521-2 TC 6.5D.3.1 has been executed. |
| 6.5B.3.4.2 | Spurious emission band UE co-existence for Inter-band including FR2 (1 NR CC) | Rel-15 | C012 | UEs supporting Inter-band including FR2 with 1 NR UL CC | E010 | PC1 PC2 (NOTE 1) PC3 PC4 (NOTE 1) PC5 (NOTE 1) | NOTE 5 Skip TC 6.5B.3.4.2 if UE supports SA and TS 38.521-2 TC 6.5.3.2 has been executed. |
| 6.5B.3.4.2a | Spurious emission band UE co-existence with Power Boost for Inter-band including FR2 (1 NR CC) | Rel-16 | C012w | UEs supporting inter-band EN-DC including FR2 with 1 NR UL CC and mpr-PowerBoost-FR2-r16 | E010 | PC1 (NOTE 1) PC2 (NOTE 1) PC3 PC4 (NOTE 1) | NOTE 5 Skip TC 6.5B.3.4.2a if UE supports SA and TS 38.521-2 TC 6.5.3.2_1 has been executed. |
| 6.5B.3.4.2_1 | Spurious emission band UE co-existence for Inter-band including FR2 (>1 NR CC) |  |  |  |  |  |  |
| 6.5B.3.4.2_1.1 | Spurious emission band UE co-existence for Inter-band EN-DC including FR2 (2NR CCs) | Rel-15 | C012b | UEs supporting Inter-Band EN-DC including FR2 with 2 NR UL CCs | E011 |  | NOTE 5 Skip TC 6.5B.3.4.2_1.1 if UE supports SA and TS 38.521-2 TC 6.5A.3.2.1 has been executed. |
| 6.5B.3.4.2_1.2 | Spurious emission band UE co-existence for Inter-band EN-DC including FR2 (3NR CCs) | Rel-15 | C012c | UEs supporting Inter-Band EN-DC including FR2 with 3 NR UL CCs | E012 |  | NOTE 5 Skip TC 6.5B.3.4.2_1.2 if UE supports SA and TS 38.521-2 TC 6.5A.3.2.2 has been executed. |
| 6.5B.3.4.2_1.3 | Spurious emission band UE co-existence for Inter-band EN-DC including FR2 (4NR CCs) | Rel-15 | C012d | UEs supporting Inter-Band EN-DC including FR2 with 4 NR UL CCs | E013 |  | NOTE 5 Skip TC 6.5B.3.4.2_1.3 if UE supports SA and TS 38.521-2 TC 6.5A.3.2.3 has been executed. |
| 6.5B.3.4.2_1.4 | Spurious emission band UE co-existence for Inter-band EN-DC including FR2 (5 NR CCs) | Rel-15 | C012i | UEs supporting Inter-Band EN-DC including FR2 with 5 NR UL CCs | E014a |  | NOTE 5 Skip TC 6.5B.3.4.2_1.4 if UE supports SA and TS 38.521-2 TC 6.5A.3.2.4 has been executed. |
| 6.5B.3.4.2_1.5 | Spurious emission band UE co-existence for Inter-band EN-DC including FR2 (6 NR CCs) | Rel-15 | C012j | UEs supporting Inter-Band EN-DC including FR2 with 6 NR UL CCs | E039 |  | NOTE 5 Skip TC 6.5B.3.4.2_1.5 if UE supports SA and TS 38.521-2 TC 6.5A.3.2.5 has been executed. |
| 6.5B.3.4.2_1.6 | Spurious emission band UE co-existence for Inter-band EN-DC including FR2 (7 NR CCs) | Rel-15 | C012k | UEs supporting Inter-Band EN-DC including FR2 with 7 NR UL CCs | E040 |  | NOTE 5 Skip TC 6.5B.3.4.2_1.6 if UE supports SA and TS 38.521-2 TC 6.5A.3.2.6 has been executed. |
| 6.5B.3.4.2_1.7 | Spurious emission band UE co-existence for Inter-band EN-DC including FR2 (8 NR CCs) | Rel-15 | C012l | UEs supporting Inter-Band EN-DC including FR2 with 8 NR UL CCs | E041 |  | NOTE 5 Skip TC 6.5B.3.4.2_1.7 if UE supports SA and TS 38.521-2 TC 6.5A.3.2.7 has been executed. |
| 6.5B.3.4.2D | Spurious emission band UE co-existence for inter-band EN-DC including FR2 for UL MIMO | Rel-15 | C012p | UEs supporting Inter-Band EN-DC including FR2 and UL MIMO | E043 | PC1 PC2 (NOTE 1) PC3 PC4 (NOTE 1) PC5 (NOTE 1) | NOTE 5 Skip TC 6.5B.3.4.2D if UE supports SA and TS 38.521-2 TC 6.5D.3.2 has been executed. |
| 6.5B.4.1 | Additional Spurious Emissions for Intra-band contiguous EN-DC | Rel-15 | C009 | UEs supporting intra-band contiguous EN-DC (2UL CCs) | E003 |  |  |
| 6.5B.4.2 | Additional Spurious Emissions for Intra-band non-contiguous EN-DC | Rel-15 | C010 | UEs supporting intra-band non-contiguous EN-DC (2UL CCs) | E004 |  | NOTE 1 |
| 6.5B.4.3 | Additional Spurious Emissions for Inter-band EN-DC within FR1 (1 NR CC) | Rel-15 | C011b | UEs supporting inter-band EN-DC within FR1 with 1 NR UL CC | E005b |  | NOTE 5 Skip TC 6.5B.4.3 if UE supports SA and TS 38.521-1 TC 6.5.3.3 has been executed. |
| 6.5B.4.4 | Additional Spurious Emissions for Inter-band including FR2 (1 NR CC) | Rel-15 | C012 | UEs supporting Inter-band including FR2 with 1 NR UL CC | E010 |  | NOTE 5 Skip TC 6.5B.4.4 if UE supports SA and TS 38.521-2 TC 6.5.3.3 has been executed. |
| 6.5B.4.4a | Additional Spurious Emissions with Power Boost for Inter-band including FR2 (1 NR CC) | Rel-16 | C012w | UEs supporting inter-band EN-DC including FR2 with 1 NR UL CC and mpr-PowerBoost-FR2-r16 | E010 | PC1 (NOTE 1) PC2 (NOTE 1) PC3 PC4 (NOTE 1) | NOTE 5 Skip TC 6.5B.4.4a if UE supports SA and TS 38.521-2 TC 6.5.3.3_1 has been executed. |
| 6.5B.4.4_1 | Additional Spurious Emissions for Inter-band including FR2 (>1 NR CC) |  |  |  |  |  |  |
| 6.5B.4.4_1.1 | Additional Spurious Emissions for Inter-band including FR2 (2 NR CC) | Rel-15 | C012b | UEs supporting Inter-Band EN-DC including FR2 with 2 NR UL CCs | E011 | PC1 | NOTE 5 Skip TC 6.5B.4.4_1.1 if UE supports SA and TS 38.521-2 TC 6.5A.3.3.1 has been executed. |
| 6.5B.4.4_1.2 | Additional Spurious Emissions for Inter-band including FR2 (3 NR CC) | Rel-15 | C012c | UEs supporting Inter-Band EN-DC including FR2 with 3 NR UL CCs | E012 | PC1 | NOTE 5 Skip TC 6.5B.4.4.1_1.2 if UE supports SA and TS 38.521-2 TC 6.5A.3.3.2 has been executed. |
| 6.5B.4.4_1.3 | Additional Spurious Emissions for Inter-band including FR2 (4 NR CC) | Rel-15 | C012d | UEs supporting Inter-Band EN-DC including FR2 with 4 NR UL CCs | E013 | PC1 | NOTE 5 Skip TC 6.5B.4.4.1_1.3 if UE supports SA and TS 38.521-2 TC 6.5A.3.3.3 has been executed. |
| 6.5B.4.4D | Additional Spurious Emissions for Inter-band including FR2 for UL MIMO | Rel-15 | C012p | UEs supporting Inter-Band EN-DC including FR2 and UL MIMO | E043 | PC1 (NOTE 1) PC2 (NOTE 1) PC3 PC4 (NOTE 1) | NOTE 5 Skip TC 6.5B.4.4D if UE supports SA and TS 38.521-2 TC 6.5D.3.3 has been executed. |
| 6.5B.5.3 | Transmit Intermodulation for Inter-band EN-DC within FR1 (1 NR CC) | Rel-15 | C011b | UEs supporting Inter-band EN-DC within FR1 with 1 NR UL CC | E005b |  | NOTE 5 Skip TC 6.5B.5.3 if UE supports SA and TS 38.521-1 TC 6.5.4 has been executed. |
| 6.6B.4 | Beam Correspondence for inter-band EN-DC including FR2 (1 NR CC) - EIRP | Rel-15 | C011b | UEs supporting Inter-band EN-DC within FR2 with 1 NR UL CC and not beam correspondence without UL beam sweeping and release 16 and forward UEs that do not support SSB-based or CSI-RS based enhanced beam correspondence and do not support enhanced beam correspondence without UL beam sweeping | E005b |  | NOTE 1 NOTE 5 Skip TC 6.6B.4 if UE supports SA and TS 38.521-2 TC 6.6.1 has been executed. |
| 6.6B.5 | Enhanced Beam correspondence for inter-band EN-DC including FR2 (1 NR CC) - EIRP | Rel-16 | C011b | UEs supporting Inter-band EN-DC within FR2 with 1 NR UL CC and support either CSI-RS or SSB based beam correspondence and do not support beam correspondence without UL beam sweeping | E005b |  | NOTE 1 NOTE 5 Skip TC 6.6B.5 if UE supports SA and TS 38.521-2 TC 6.6.2 has been executed. |
| 7 | Receiver Characteristics |  |  |  |  |  |  |
| 7.3B | Reference sensitivity level for DC |  |  |  |  |  |  |
| 7.3B.2.1 | Reference sensitivity for intra-band contiguous EN-DC (2 CCs) | Rel-15 | C009a | UEs supporting intra-band contiguous EN-DC (2DL CCs) | E003a |  |  |
| 7.3B.2.2 | Reference sensitivity for Intra-band non-contiguous EN-DC (2 CCs) | Rel-15 | C010a | UEs supporting intra-band non-contiguous EN-DC (2DL CCs) | E004a |  | NOTE 5 Skip TC 7.3B.2.2 if UE supports SA and TS 38.521-1 TC 7.3.2 has been executed. |
| 7.3B.2.3 | Reference sensitivity for Inter-band EN-DC within FR1 (2 CCs) | Rel-15 | C011a | UEs supporting inter-band EN-DC within FR1 (2DL CCs) | E005a E005d | PC2 PC3 | For LTE anchor agnostic approach testing in TC 7.3B.2.3: 1. NOTE 5 applied. 2. Skip the testing if UE supports SA and TS 38.521-1 TC 7.3.2 has been executed. |
| 7.3B.2.3_1 | Reference sensitivity for EN-DC within FR1 (>2 CCs) |  |  |  |  |  |  |
| 7.3B.2.3_1.1 | Reference sensitivity for EN-DC within FR1 (3 CCs) | Rel-15 | C045 | UEs supporting EN-DC within FR1 (3DL CCs) | E006 |  |  |
| 7.3B.2.3_1.2 | Reference sensitivity for EN-DC within FR1 (4 CCs) | Rel-15 | C046 | UEs supporting EN-DC within FR1 (4DL CCs) | E007 |  |  |
| 7.3B.2.3_1.3 | Reference sensitivity for EN-DC within FR1 (5 CCs) | Rel-15 | C047 | UEs supporting EN-DC within FR1 (5DL CCs) | E008 |  |  |
| 7.3B.2.4 | Reference sensitivity for Inter-band EN-DC including FR2 (1 NR CC) | Rel-15 | C012a | UEs supporting inter-band EN-DC including FR2 with 1 NR DL CC | E010a |  | NOTE 5 Skip TC 7.3B.2.4 if UE supports SA and TS 38.521-2 TC 7.3.2 has been executed. |
| 7.3B.2.4_1 | Reference sensitivity for Inter-band EN-DC including FR2 (>1 NR CC) |  |  |  |  |  |  |
| 7.3B.2.4_1.1 | Reference sensitivity for Inter-band EN-DC including FR2 (2 NR CCs) | Rel-15 | C012e | UEs supporting inter-band EN-DC including FR2 with 2 NR DL CCs | E011a |  | NOTE 5 Skip TC 7.3B.2.4_1.1 if UE supports SA and TS 38.521-2 TC 7.3A.2.1 has been executed. |
| 7.3B.2.4_1.2 | Reference sensitivity for Inter-band EN-DC including FR2 (3 NR CCs) | Rel-15 | C012g | UEs supporting inter-band EN-DC including FR2 with 3 NR DL CCs | E012a |  | NOTE 5 Skip TC 7.3B.2.4_1.2 if UE supports SA and TS 38.521-2 TC 7.3A.2.2 has been executed. |
| 7.3B.2.4_1.3 | Reference sensitivity for Inter-band EN-DC including FR2 (4 NR CCs) | Rel-15 | C012h | UEs supporting inter-band EN-DC including FR2 with 4 NR DL CCs | E013a |  | NOTE 5 Skip TC 7.3B.2.4_1.3 if UE supports SA and TS 38.521-2 TC 7.3A.2.3 has been executed. |
| 7.3B.2.4_1.4 | Reference sensitivity for Inter-band EN-DC including FR2 (5 NR CCs) | Rel-15 | FFS | UEs supporting inter-band EN-DC including FR2 with 5 NR DL CCs | FFS |  | NOTE 1 NOTE 5 Skip TC 7.3B.2.4_1.4 if UE supports SA and TS 38.521-2 TC 7.3A.2.4 has been executed. |
| 7.3B.2.4_1.5 | Reference sensitivity for Inter-band EN-DC including FR2 (6 NR CCs) | Rel-15 | FFS | UEs supporting inter-band EN-DC including FR2 with 6 NR DL CCs | FFS |  | NOTE 1 NOTE 5 Skip TC 7.3B.2.4_1.5 if UE supports SA and TS 38.521-2 TC 7.3A.2.5 has been executed. |
| 7.3B.2.4_1.6 | Reference sensitivity for Inter-band EN-DC including FR2 (7 NR CCs) | Rel-15 | FFS | UEs supporting inter-band EN-DC including FR2 with 7 NR DL CCs | FFS |  | NOTE 1 NOTE 5 Skip TC 7.3B.2.4_1.6 if UE supports SA and TS 38.521-2 TC 7.3A.2.6 has been executed. |
| 7.3B.2.4_1.7 | Reference sensitivity for Inter-band EN-DC including FR2 (8 NR CCs) | Rel-15 | FFS | UEs supporting inter-band EN-DC including FR2 with 8 NR DL CCs | FFS |  | NOTE 1 NOTE 5 Skip TC 7.3B.2.4_1.7 if UE supports SA and TS 38.521-2 TC 7.3A.2.7 has been executed. |
| 7.3B.2.4D | Reference sensitivity for inter-band EN-DC including FR2 for UL MIMO | FFS | FFS | FFS | FFS |  | NOTE 1 |
| 7.3B.4 | EIS Spherical Coverage for Inter-band EN-DC including FR2 (1 NR CC) | Rel-15 | C012a | UEs supporting inter-band EN-DC including FR2 with 1 NR DL CC | E010a |  | NOTE 5 Skip TC 7.3B.4 if UE supports SA and TS 38.521-2 TC 7.3.4 has been executed. |
| 7.4B | Maximum Input Level for DC |  |  |  |  |  |  |
| 7.4B.1 | Maximum Input Level for Intra-Band Contiguous EN-DC (2 CCs) | Rel-15 | C009a | UEs supporting Intra-Band Contiguous EN-DC (2DL CCs) | E003a |  |  |
| 7.4B.2 | Maximum Input Level for Intra-Band Non-Contiguous EN-DC (2 CCs) | Rel-15 | C010a | UEs supporting Intra-Band Non-Contiguous EN-DC (2DL CCs) | E004a |  | NOTE 5 Skip TC7.4B.2 if UE supports SA and TS 38.521-1 TC 7.4 has been executed |
| 7.4B.3 | Maximum Input Level for Inter-band EN-DC within FR1 (1 NR CC) | Rel-15 | C011c | UEs supporting Inter-band EN-DC within FR1 with 1 NR DL CC | E005c |  | NOTE 5 Skip TC 7.4B.3 if UE supports SA and TS 38.521-1 TC 7.4 has been executed. |
| 7.4B.3_1 | Maximum Input Level for EN-DC within FR1 (>2 CCs) |  |  |  |  |  |  |
| 7.4B.3_1.1 | Maximum Input Level for EN-DC within FR1 (3 CCs) | Rel-15 | C045 | UEs supporting EN-DC within FR1 (3DL CCs) | E006 |  |  |
| 7.4B.3_1.2 | Maximum Input Level for EN-DC within FR1 (4 CCs) | Rel-15 | C046 | UEs supporting EN-DC within FR1 (4DL CCs) | E007 |  |  |
| 7.4B.3_1.3 | Maximum Input Level for EN-DC within FR1 (5 CCs) | Rel-15 | C047 | UEs supporting EN-DC within FR1 (5DL CCs) | E008 |  |  |
| 7.4B.3_1.4 | Maximum Input Level for EN-DC within FR1 (6 CCs) | Rel-15 | FFS | FFS | FFS |  | NOTE 1 |
| 7.4B.4 | Maximum Input Level for inter-band EN-DC including FR2 (1 NR CC) | Rel-15 | C012a | UEs supporting Inter-band including FR2 with 1 NR DL CC | E010a |  | NOTE 1 NOTE 5 Skip TC 7.4B.4 if UE supports SA and TS 38.521-2 TC 7.4 has been executed. |
| 7.4B.4_1 | Maximum Input Level for inter-band EN-DC including FR2 (>1 NR CC) |  |  |  |  |  |  |
| 7.4B.4_1.1 | Maximum Input Level for inter-band EN-DC including FR2 (2 NR CCs) | FFS | FFS | FFS | FFS |  | NOTE 1 |
| 7.4B.4_1.2 | Maximum Input Level for inter-band EN-DC including FR2 (3 NR CCs) | FFS | FFS | FFS | FFS |  | NOTE 1 |
| 7.4B.4_1.3 | Maximum Input Level for inter-band EN-DC including FR2 (4 NR CCs) | FFS | FFS | FFS | FFS |  | NOTE 1 |
| 7.4B.4_1.4 | Maximum Input Level for inter-band EN-DC including FR2 (5 NR CCs) | FFS | FFS | FFS | FFS |  | NOTE 1 |
| 7.4B.4D | Maximum Input Level for inter-band EN-DC including FR2 for UL MIMO | FFS | FFS | FFS | FFS |  | NOTE 1 |
| 7.5B | Adjacent channel selectivity for DC |  |  |  |  |  |  |
| 7.5B.1 | Adjacent Channel Selectivity for intra-band contiguous EN-DC (2 CCs) | Rel-15 | C009a | UEs supporting intra-band contiguous EN-DC (2DL CCs) | E003a |  | NOTE 1 |
| 7.5B.2 | Adjacent Channel Selectivity for intra-band non-contiguous EN-DC (2 CCs) | Rel-15 | C010a | UEs supporting intra-band non-contiguous EN-DC (2DL CCs) | E004a |  | NOTE 1 NOTE 5 Skip TC 7.5B.2 if UE supports SA and TS 38.521-1 TC 7.5 has been executed. |
| 7.5B.3 | Adjacent Channel Selectivity for inter-band EN-DC within FR1 (1 NR CC) | Rel-15 | C011c | UEs supporting inter-band EN-DC within FR1 with 1 NR DL CCs and one or more LTE DL CC(s) | E005c |  | NOTE 5 Skip TC 7.5B.3 if UE supports SA and TS 38.521-1 TC 7.5 has been executed. |
| 7.5B.3_1 | Adjacent Channel Selectivity for EN-DC within FR1 (>2 CCs) |  |  |  |  |  |  |
| 7.5B.3_1.1 | Adjacent Channel Selectivity for EN-DC within FR1 (2 NR CCs) | Rel-15 | C063 | UEs supporting inter-band or intra-band non-contiguous EN-DC within FR1 with 2 NR DL CCs | E027 E029 |  | NOTE 5 Skip TC 7.5B.3_1.1 if UE supports SA and TS 38.521-1 TC 7.5A.1 has been executed. |
| 7.5B.3_1.2 | Adjacent Channel Selectivity for EN-DC within FR1 (3 NR CCs) | Rel-15 | C064 | UEs supporting inter-band or intra-band non-contiguous EN-DC within FR1 with 3 NR DL CCs | E028 E030 |  | NOTE 5 Skip TC 7.5B.3_1.2 if UE supports SA and TS 38.521-1 TC 7.5A.2 has been executed. |
| 7.5B.3_1.3 | Adjacent Channel Selectivity for EN-DC within FR1 (4 NR CCs) | Rel-15 | C064a | UEs supporting intra-band non-contiguous EN-DC within FR1 with 4 NR DL CCs | E028a E030a |  | NOTE 5 Skip TC 7.5B.3_1.3 if UE supports SA and TS 38.521-1 TC 7.5A.3 has been executed. |
| 7.5B.3_1.4 | Adjacent Channel Selectivity for EN-DC within FR1 (5 NR CCs) | Rel-15 | C064b | UEs supporting intra-band non-contiguous EN-DC within FR1 with 5 NR DL CCs | E028b E030b |  | NOTE 1 NOTE 5 Skip TC 7.5B.3_1.4 if UE supports SA and TS 38.521-1 TC 7.5A.4 has been executed. |
| 7.5B.4 | Adjacent Channel Selectivity for inter-band EN-DC including FR2 (1 NR CC) | Rel-15 | FFS | FFS | FFS |  | NOTE 1 NOTE 5 Skip TC 7.5B.4 if UE supports SA and TS 38.521-2 TC 7.5 has been executed. |
| 7.5B.4_1 | Adjacent Channel Selectivity for inter-band EN-DC including FR2 (>1 NR CC) |  |  |  |  |  |  |
| 7.5B.4_1.1 | Adjacent Channel Selectivity for inter-band EN-DC including FR2 (2 NR CCs) | Rel-15 | FFS | FFS | FFS |  | NOTE 1 NOTE 5 Skip TC 7.5B.4_1.1 if UE supports SA and TS 38.521-2 TC 7.5A.1 has been executed. |
| 7.5B.4_1.2 | Void |  |  |  |  |  |  |
| 7.5B.4_1.3 | Void |  |  |  |  |  |  |
| 7.5B.4_1.4 | Void |  |  |  |  |  |  |
| 7.5B.4D | Adjacent Channel Selectivity for inter-band EN-DC including FR2 for UL MIMO | FFS | FFS | FFS | FFS |  | NOTE 1 |
| 7.6B | Blocking characteristics for DC |  |  |  |  |  |  |
| 7.6B.2.1 | Inband blocking for intra-band contiguous EN-DC (2 CCs) | Rel-15 | C009a | UEs supporting Intra-Band Contiguous EN-DC (2DL CCs) | E003a |  |  |
| 7.6B.2.2 | Inband blocking for intra-band non-contiguous EN-DC (2 CCs) | Rel-15 | C010a | UEs supporting Intra-Band Non-Contiguous EN-DC (2DL CCs) | E004a |  | NOTE 5 Skip TC 7.6B.2.2 if UE supports SA and TS 38.521-1 TC 7.6.2 has been executed. |
| 7.6B.2.3 | Inband blocking for inter-band EN-DC within FR1 (1 NR CC) | Rel-15 | C011c | UEs supporting Inter-band EN-DC within FR1 with 1 NR DL CC | E005c |  | NOTE 5 Skip TC 7.6B.2.3 if UE supports SA and TS 38.521-1 TC 7.6.2 has been executed. |
| 7.6B.2.3_1 | Inband blocking for EN-DC within FR1 (>2 CCs) |  |  |  |  |  |  |
| 7.6B.2.3_1.1 | Inband blocking for EN-DC within FR1 (3 CCs) | Rel-15 | C045 | UEs supporting EN-DC within FR1 (3DL CCs) | E006 |  |  |
| 7.6B.2.3_1.2 | Inband blocking for EN-DC within FR1 (4 CCs) | Rel-16 | C046 | UEs supporting EN-DC within FR1 (4DL CCs) | E007 |  |  |
| 7.6B.2.3_1.3 | Inband blocking for EN-DC within FR1 (5 CCs) | Rel-16 | C047 | UEs supporting EN-DC within FR1 (5DL CCs) | E008 |  | Skip TC 7.6B.2.3_1.3 if UE supports SA and TS 38.521-1 TC 7.6A.2.3 has been executed. |
| 7.6B.2.4 | Inband blocking for inter-band EN-DC including FR2 (1 NR CC) | Rel-15 | C012a | UEs supporting Inter-band EN-DC including FR2 with 1 NR DL CC | E010a |  | NOTE 5 Skip TC 7.6B.2.4 if UE supports SA and TS 38.521-2 TC 7.6.2 has been executed. |
| 7.6B.2.4_1 | Inband blocking for inter-band EN-DC including FR2 (>1 NR CC) |  |  |  |  |  |  |
| 7.6B.2.4_1.1 | Inband blocking for inter-band EN-DC including FR2 (2 NR CCs) | Rel-15 | FFS | FFS | FFS |  | NOTE 1 NOTE 5 Skip TC 7.6B.2.4_1.1 if UE supports SA and TS 38.521-2 TC 7.6A.2.1 has been executed. |
| 7.6B.2.4_1.2 | Void |  |  |  |  |  |  |
| 7.6B.2.4_1.3 | Void |  |  |  |  |  |  |
| 7.6B.2.4_1.4 | Void |  |  |  |  |  |  |
| 7.6B.2.4D | Inband blocking for inter-band EN-DC including FR2 for UL MIMO | FFS | FFS | FFS | FFS |  | NOTE 1 |
| 7.6B.3.1 | Out-of-band blocking for intra-band contiguous EN-DC (2 CCs) | Rel-15 | C009a | UEs supporting Intra-Band Contiguous EN-DC (2DL CCs) | E003a |  |  |
| 7.6B.3.2 | Out-of-band blocking for intra-band non-contiguous EN-DC (2 CCs) | Rel-15 | C010a | UEs supporting Intra-Band Non-Contiguous EN-DC (2DL CCs) | E004a |  | NOTE 5 Skip TC 7.6B.3.2 if UE supports SA and TS 38.521-1 TC 7.6.3 has been executed. |
| 7.6B.3.3 | Out-of-band blocking for inter-band EN-DC within FR1 (2 CCs) | Rel-15 | C011a | UEs supporting Inter-band EN-DC within FR1 (2DL CCs) | E005a |  |  |
| 7.6B.3.3_1 | Out-of-band blocking for EN-DC within FR1 (>2 CCs) |  |  |  |  |  |  |
| 7.6B.3.3_1.1 | Out-of-band blocking for EN-DC within FR1 (3 CCs) | Rel-15 | C048 | UEs supporting intra-band contiguous EN-DC within FR1 with 3 DL CCs | E006 |  |  |
| 7.6B.3.3_1.2 | Out-of-band blocking for EN-DC within FR1 (4 CCs) | Rel-16 | C049 | UEs supporting intra-band contiguous EN-DC within FR1 with 4 DL CCs | E007 |  |  |
| 7.6B.4.1 | Narrow band blocking for intra-band contiguous EN-DC (2 CCs) | Rel-15 | C009a | UEs supporting Intra-Band Contiguous EN-DC (2DL CCs) | E003a |  |  |
| 7.6B.4.2 | Narrow band blocking for intra-band non-contiguous EN-DC (2 CCs) | Rel-15 | C010a | UEs supporting Intra-Band Non-Contiguous EN-DC (2DL CCs) | E004a |  | NOTE 5 Skip TC 7.6B.4.2 if UE supports SA and TS 38.521-1 TC 7.6.4 has been executed. |
| 7.6B.4.3 | Narrow band blocking for inter-band EN-DC within FR1 (1 NR CC) | Rel-15 | C011c | UEs supporting Inter-band EN-DC within FR1 with 1 NR DL CC | E005c |  | NOTE 5 Skip TC 7.6B.4.3 if UE supports SA and TS 38.521-1 TC 7.6.4 has been executed. |
| 7.6B.4.3_1 | Narrow band blocking for EN-DC within FR1 (>2 CCs) |  |  |  |  |  |  |
| 7.6B.4.3_1.1 | Narrow band blocking for EN-DC within FR1 (3 CCs) | Rel-15 | C045 | UEs supporting EN-DC within FR1 (3DL CCs) | E006 |  |  |
| 7.6B.4.3_1.2 | Narrow band blocking for EN-DC within FR1 (4 CCs) | Rel-16 | C046 | UEs supporting EN-DC within FR1 (4DL CCs) | E007 |  |  |
| 7.6B.4.3_1.3 | Narrow band blocking for EN-DC within FR1 (5 CCs) | Rel-16 | C047 | UEs supporting EN-DC within FR1 (5DL CCs) | E008 |  | Skip TC 7.6B.4.3_1.3 if UE supports SA and TS 38.521-1 TC 7.6A.4.3 has been executed. |
| 7.7B | Spurious response for DC |  |  |  |  |  |  |
| 7.7B.1 | Spurious Response for intra-band contiguous EN-DC (2 CCs) | Rel-15 | C009a | UEs supporting Intra-Band Contiguous EN-DC (2DL CCs) | E003a |  |  |
| 7.7B.2 | Spurious Response for intra-band non-contiguous EN-DC (2 CCs) | Rel-15 | C010a | UEs supporting Intra-Band Non-Contiguous EN-DC (2DL CCs) | E004a |  | NOTE 5 Skip TC 7.7B.2 if UE supports SA and TS 38.521-1 TC 7.7 has been executed. |
| 7.7B.3 | Spurious Response for inter-band EN-DC within FR1 (2 CCs) | Rel-15 | C011a | UEs supporting Inter-band EN-DC within FR1 (2DL CCs) | E005a |  |  |
| 7.7B.3_1 | Spurious Response for EN-DC within FR1 (>2 CCs) |  |  |  |  |  |  |
| 7.7B.3_1.1 | Spurious Response for EN-DC within FR1 (3 CCs) | Rel-15 | C048 | UEs supporting intra-band contiguous EN-DC within FR1 with 3 DL CCs | E006 |  |  |
| 7.7B.3_1.2 | Spurious Response for EN-DC within FR1 (4 CCs) | Rel-16 | C049 | UEs supporting intra-band contiguous EN-DC within FR1 with 4 DL CCs | E007 |  |  |
| 7.8B | Intermodulation characteristics for DC |  |  |  |  |  |  |
| 7.8B.2.1 | Wideband Intermodulation for intra-band contiguous EN-DC (2 CCs) | Rel-15 | C009a | UEs supporting Intra-Band Contiguous EN-DC (2DL CCs) | E003a |  | NOTE 1 |
| 7.8B.2.2 | Wideband Intermodulation for intra-band non-contiguous EN-DC (2 CCs) | Rel-15 | C010a | UEs supporting Intra-Band non-contiguous EN-DC (2DL CCs) | E004a |  | NOTE 5 Skip TC 7.8B.2.2 if UE supports SA and TS 38.521-1 TC 7.8.2 has been executed. |
| 7.8B.2.3 | Wideband Intermodulation for inter-band EN-DC within FR1 (2 CCs) | Rel-15 | C011a | UEs supporting inter-band EN-DC within FR1 (2DL CCs) | E005c |  | NOTE 5 Skip TC 7.8B.2.3 if UE supports SA and TS 38.521-1 TC 7.8.2 has been executed. |
| 7.8B.2.3_1 | Wideband Intermodulation for EN-DC within FR1 (>2 CCs) |  |  |  |  |  |  |
| 7.8B.2.3_1.1 | Wideband Intermodulation for EN-DC within FR1 (3 CCs) | Rel-15 | C045 | UEs supporting EN-DC within FR1 (3DL CCs) | E006 |  |  |
| 7.8B.2.3_1.2 | Wideband Intermodulation for EN-DC within FR1 (4 CCs) | Rel-15 | C046 | UEs supporting EN-DC within FR1 (4DL CCs) | E007 |  |  |
| 7.8B.2.3_1.3 | Wideband Intermodulation for EN-DC within FR1 (5 CCs) | Rel-15 | FFS | FFS | FFS |  | NOTE 1 |
| 7.9B | Spurious emissions for DC |  |  |  |  |  |  |
| 7.9B.1 | Spurious Emissions for intra-band contiguous EN-DC (2 CCs) | Rel-15 | C009a | UEs supporting Intra-Band Contiguous EN-DC (2DL CCs) | E003a |  | NOTE 5 Skip TC 7.9B.1 if UE supports SA and TS 38.521-1 TC 7.9 has been executed. |
| 7.9B.2 | Spurious Emissions for intra-band non-contiguous EN-DC (2 CCs) | Rel-15 | C010a | UEs supporting Intra-Band non-contiguous EN-DC (2DL CCs) | E004a |  | NOTE 5 Skip TC 7.9B.2 if UE supports SA and TS 38.521-1 TC 7.9 has been executed. |
| 7.9B.3 | Spurious Emissions for inter-band EN-DC within FR1 (1 NR CC) | Rel-15 | C011c | UEs supporting inter-band EN-DC within FR1 with 1 NR DL CC | E005c |  | NOTE 5 Skip TC 7.9B.3 if UE supports SA and TS 38.521-1 TC 7.9 has been executed. |
| 7.9B.3_1 | Spurious Emissions for EN-DC within FR1 (>2 CCs) |  |  |  |  |  |  |
| 7.9B.3_1.1 | Spurious Emissions for EN-DC within FR1 (3 CCs) | Rel-15 | C048 | UEs supporting EN-DC within FR1 with 1 LTE DL CC and 2 inter-band NR DL CCs with DL-only NR band | E006 |  |  |
| 7.9B.4 | Spurious Emissions for inter-band EN-DC including FR2 (1 NR CC) | Rel-15 | C012a | UEs supporting Inter-band including FR2 with 1 NR DL CC | E010a |  | NOTE5 Skip TC 7.9B.4 if UE supports SA and TS 38.521-2 TC 7.9 has been executed. |
| NOTE 1: The test case/branch is incomplete for any Band/CA/DC Configuration but has basic test configurations already. NOTE 1 can be removed only when the test case/branch is complete for at least one Band / CA/DC Configuration for at least one feature included in the test case/branch. Detailed completion status can be found in the corresponding test case section in 38.521-3. NOTE 2: Void. NOTE 3: Void. NOTE 4: Void. NOTE 5: Test only one EN-DC combination per 5G NR band as LTE anchor agnostic approach is applied. |  |  |  |  |  |  |  |

Table 4.1.3-1a: Void

< End of changes >