3GPP TSG-RAN5 Meeting #104	R5-245993

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3315 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to High test channel bandwidth Informative Note 1 for FR1 and FR2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Technologies Ireland, Samsung R&D Institute UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The interpretation of statement “some flexibility in ecosystem” in the informative & normative notes for high channel bandwidth table are not consistent across the test labs. The purpose of this change is to rewrite portions of relevent notes to ensure there is only one clear and consistent interpretation of the notes. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated informative and normative notes for FR1 & FR2 high channel BW to clarify alternative test frequencies to be used. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Interpretation of the notes and test frequency used may be inconsistent across the “ecosystem” (not sure what that means…). |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The changes in this CR are as per the offline agreement captured in May 2024 RAN5 # 103 meeting handling for a corrosponding disucssion paper |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1 updates -> deleted redundunt term “is tested” r2 updates -> reverted changes on informative note for FR1 which is voided in R5-244671r3 and aligned informative note for FR2 as per R5-244671r1 final R5-245993 updates -> FR1 & FR2: removed references to Rel 15 & 16 from normative note. FR2 only: Retained Rel 15 & 16 in informative notes as these legacy UEs may not support 200mhz |  |  |  |  |  |  |  |  |

#### < start of changes >

### 4.3.1	Test frequencies

#### < Unchanged sections skipped >

#### 4.3.1.0C	High test channel bandwidth

The high test channel bandwidth definition for RF is given in Table 4.3.1.0C-1, Table 4.3.1.0C-2 and Table 4.3.1.0C-3 for FR1, FR2 and FR1-NTN respectively.

Table 4.3.1.0C-1: High Test Channel bandwidths for each NR band, FR1

| NR Band | UE High Test Channel bandwidth [MHz]10, 11, 12 | RedCap UE High Test Channel bandwidth [MHz] 10, 11, 12 |
| --- | --- | --- |
| n1 | 50 | 20 |
| n2 | 40 | 20 |
| n3 | 50 | 20 |
| n5 | 201,14, 252 | 20 |
| n7 | 50 | 20 |
| n8 | 201,14, 352 | 20 |
| n12 | 15 | 15 |
| n13 | 10 | 10 |
| n14 | 10 | 10 |
| n20 | 20 | 20 |
| n24 | 10 | 10 |
| n25 | 401, 452 | 20 |
| n26 | 20 | 20 |
| n28 | 30 | 20 |
| n29 | 102 | N/A |
| n30 | 10 | 10 |
| n34 | 15 | 15 |
| n38 | 25, 4013 | 15 |
| n39 | 40 | 20 |
| n40 | 100 | 20 |
| n41 | 100 | 20 |
| n46 | 80 | N/A |
| n48 | 403, 1004 | 20 |
| n50 | 601,14, 802 | 20 |
| n51 | 5 | 5 |
| n53 | 10 | 10 |
| n65 | 20 | 20 |
| n66 | 40 | 20 |
| n70 | 151,14, 252 | 151, 202 |
| n71 | 201, 352 | 20 |
| n74 | 20 | 20 |
| n75 | 202 | N/A |
| n76 | 52 | N/A |
| n77 | 100 | 20 |
| n78 | 100 | 20 |
| n79 | 100 | 20 |
| n80 | 301 | [20] |
| n81 | 201 | [20] |
| n82 | 201 | [20] |
| n83 | 301 | [20] |
| n84 | 501 | [20] |
| n85 | 15 | 15 |
| n86 | 401 | [20] |
| n91 | 101, 52,14 | [10] |
| n92 | 20 | [20] |
| n93 | 101, 52,14 | [10] |
| n94 | 20 | [20] |
| n95 | 151 | [15] |
| n96 | 80 | N/A |
| n97 | 1001 | [20] |
| n99 | 101 | [10] |
| n100 | 5 | 5 |
| n101 | 10 | 10 |
| n106 | 3 | N/A |
| Note 1: This UE channel bandwidth is applicable only to uplink. Note 2: This UE channel bandwidth is applicable only to downlink. Note 3: Applicable for use as single carrier, PCell in CA or PCell in DC configuration. Note 4: Applicable for use as DL SCell in CA or DL SCell in DC configuration. Note 5: Void Note 6: Void Note 7: Void Note 8: Void Note 9: Void Note 10: Maximum channel BW value among all the possible channel BW-SCS combinations per band in Table 5.3.5-1 of TS 38.521-1 [14] are listed. In case such bandwidth is not applicable for a given subcarrier spacing, the maximum bandwidth applicable for such subcarrier spacing shall be tested. In case such bandwidth is not defined in the UE release specification according to 38.101-1 [7] Table 5.3.5-1, the maximum bandwidth defined for that band in the UE release specification shall be tested. In case such bandwidth is optional in the UE release specification according to 38.101-1 [7] Table 5.3.5-1 and not supported by the UE, the maximum non-optional bandwidth for the UE release specification shall be tested. Note 11: Values listed in this table assume that the maximum (non-optional) channel bandwidth specified in Table 5.3.5-1 of TS 38.101-1 [7]is mandatory without capability parameter (i.e., purely mandatory) as defined in [55] TS 38.306 clauses 4.2.7.6/4.2.7.8 for supportedBandwidthDL/ supportedBandwidthUL parameters in a band combination with a single band entry and a single CC entry (i.e., non-CA band combination). Note 12: For CA, DC, SDL and SUL, the High-test channel bandwidth per component carrier is chosen to allow maximum aggregated bandwidth defined for a given bandwidth combination set. In case no set of channel bandwidths per component carrier supported by the UE can achieve maximum aggregated bandwidths in CA, DC, SDL or SUL, testing could be performed with one combination of bandwidth per component carrier within the bandwidth combination set that maximizes the aggregated bandwidth instead of the values described in the Table. Note 13: These UE channel bandwidths are applicable to sidelink operation. Note 14: This UE channel bandwidth is applicable to both uplink and downlink when both uplink and downlink are allocated and testing asymmetric BW is not explicitly intended. |  |  |

NOTE 1 (Informative): In case values listed in table above are higher than those signalled by the UE in supportedBandwidthDL/supportedBandwidthUL, some flexibility could be provided to the ecosystem for Rel-15 and Rel-16 so the value signalled by the UE in supportedBandwidthDL/supportedBandwidthUL is used in single carrier operation instead values described in Table 4.3.1.0C-1.

Table 4.3.1.0C-2: High Test Channel bandwidths for each NR band, FR2

| NR Band | UE High Test Channel bandwidth [MHz]1, 2, 3 | RedCap UE High Test Channel bandwidth [MHz]1, 2 |
| --- | --- | --- |
| n257 | 400 | 100 |
| n258 | 400 | 100 |
| n259 | 400 | N/A |
| n260 | 400 | N/A |
| n261 | 400 | 100 |
| Note 1: Maximum channel BW value among all the possible channel BW-SCS combinations per band in Table 5.3.5-1 of TS 38.521-2 [15] are listed. In case such bandwidth is not applicable for a given subcarrier spacing, the maximum bandwidth applicable for such subcarrier spacing shall be tested. In case such bandwidth is not defined in the UE release specification according to 38.101-2 [8] Table 5.3.5-1, the maximum bandwidth defined for that band in the UE release specification shall be tested. In case such bandwidth is optional in the UE release specification according to 38.101-2 [8] Table 5.3.5-1 and not supported by the UE, the maximum non-optional bandwidth for the UE release specification shall be tested. Note 2: Values listed in this table assume that the maximum (non-optional) channel bandwidth specified in Table 5.3.5-1 of TS 38.101-2 [8] is mandatory without capability parameter (i.e., purely mandatory) as defined in [55] TS 38.306 clauses 4.2.7.6/4.2.7.8 for supportedBandwidthDL/ supportedBandwidthUL parameters in a band combination with a single band entry and a single CC entry (i.e., non-CA band combination). Note 3: For CA and DC, the High-test channel bandwidth per component carrier is chosen to allow maximum aggregated bandwidth defined for a given bandwidth combination set. In case no set of channel bandwidths per component carrier supported by the UE can achieve maximum aggregated bandwidths in CA and DC, testing could be performed using one combination of bandwidth per component carrier within the bandwidth combination set that maximizes the aggregated bandwidth instead of the values described in the Table. |  |  |

NOTE 1 (Informative): In case values listed in table above are higher than those signalled by the UE in supportedBandwidthDL/supportedBandwidthUL parameters in a band combination with a single band entry and a single CC entry (i.e., non-CA band combination), testing for Rel-15 and Rel-16 could be performed usingthe value signalled by the UE in single carrier operation instead of the values described in Table 4.3.1.0C-2.

Table 4.3.1.0C-3: High Test Channel bandwidths for each NTN satellite band, FR1-NTN

| NTN satellite Band | UE High Test Channel bandwidth [MHz]1, 2 |
| --- | --- |
| n256 | 20 |
| n255 | 20 |
| Note 1: Maximum channel BW value among all the possible channel BW-SCS combinations per band in Table 5.3.5-1 of TS 38.521-5 [57] are listed. Note 2: Values listed in this table assume that the maximum (non-optional) channel bandwidth specified in Table 5.3.5-1 of TS 38.101-5 [56] is mandatory without capability parameter (i.e., purely mandatory) as defined in TS 38.306 clauses 4.2.7.6/4.2.7.8 [55] for supportedBandwidthDL/ supportedBandwidthUL parameters in a band combination with a single band entry and a single CC entry (i.e., non-CA band combination). |  |

< end of changes >