3GPP TSG-RAN5 Meeting #104	R5-246031

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 1086 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for the General and UE maximum output power for PC6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_HST_FR2-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Corrrection for the Tx general information for PC6 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding PC6 information in general section. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | WI cannot complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from: R5-245139r1. |  |  |  |  |  |  |  |  |

<Start of change 1>

6.1	General

Editor’s Note: Test configurations/environments that require new spherical scan shall be included in test procedure section and identifying such scenarios is currently FFS and owned by RAN5.

Unless otherwise stated, the transmitter characteristics are specified over the air (OTA) with a single or multiple transmit chains.

Unless otherwise stated, for power class 3 UEs, the beam correspondence side condition for SSB and CSI-RS specified in subclause 6.6 shall apply to the transmission tests.

Unless otherwise stated, the UE min peak EIRP requirements and UE spherical coverage requirements specified in clause 6.2.1 does not apply to initial access and RRC_INACTIVE.

Transmitter requirements for CA operation apply only when the DMRS initialization parameters (including the case when the UE applies cell ID as DMRS scrambling ID) are different across all CCs. The UE may use higher MPR values outside this limitation.

Transmitter requirements for UL MIMO operation apply when the UE transmits on 2 ports on the same CDM group. The UE may use higher MPR values outside this limitation.

For Tx test cases the identified beam peak direction can be stored and reused for a device under test in various configurations/environments for the full duration of device testing as long as beam peak direction is the same.

Unless otherwise stated, Channel Bandwidth shall be prioritized in the selecting of test points. Subcarrier spacing shall be selected after Test Channel Bandwidth is selected.

Uplink RB allocations given in Table 6.1-1 and Table 6.1-2 are used throughout this section, unless otherwise stated by the test case.

The UE under test shall be pre-configured with UL Tx diversity schemes disabled to account for single polarization System Simulator (SS) in the test environment. The UE under test may transmit with dual polarization.

Table 6.1-1: Common Uplink Configuration for PC2, PC3, PC4, PC6 and PC7

| Channel Bandwidth | SCS(kHz) | OFDM | RB allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Outer_Full | Outer_xRB_Left (Note 6) | Outer_xRB_Right (Note 6) | Inner_Full (Note 1) | Inner_xRB_Left (Note 6) | Inner_xRB_Right (Note 6) | Inner_Partial_Left | Inner_Partial_Right | Inner_Partial2_Left | Inner_Partial2_Right |
| 50MHz | 60 | DFT-s | 64@0 | x@0 | x@(66-x) | 20@223 20@204 | x@223 x@14 | x@(44-x)3 x@(65-x)4 | 4@223 8@84 | 4@403 8@504 | 6@64 | 6@544 |
|  |  | CP | 66@0 | x@0 | x@(66-x) | 22@22 | x@223 x@14 | x@(44-x)3 x@(65-x)4 | 4@223 7@74 | 4@403 7@524 | 6@64 | 6@544 |
|  | 120 | DFT-s | 32@0 | x@0 | x@(32-x) | 10@113 10@104 | x@113 x@14 | x@(22-x)3 @(31-x)4 | 4@113 4@44 | 4@183 4@244 | 3@34 | 3@264 |
|  |  | CP | 32@0 | x@0 | x@(32-x) | 11@113 10@104 | x@113 x@14 | x@(22-x)3 x@(31-x)4 | 4@113 4@44 | 4@183 4@244 | 3@34 | 3@264 |
| 100MHz | 60 | DFT-s | 128@0 | x@0 | x@(132-x) | 40@443 40@404 | x@443 x@14 | x@(88-x)3 x@(131-x)4 | 4@443 8@84 | 4@843 8@1164 | 6@64 | 6@1204 |
|  |  | CP | 132@0 | x@0 | x@(132-x) | 44@44 | x@443 x@14 | x@(88-x)3 x@(131-x)4 | 4@443 7@74 | 4@843 7@1184 | 6@64 | 6@1204 |
|  | 120 | DFT-s | 64@0 | x@0 | x@(66-x) | 20@223 20@204 | x@223 x@14 | x@(44-x)3 x@(65-x)4 | 4@223 4@44 | 4@403 4@584 | 3@34 | 3@604 |
|  |  | CP | 66@0 | x@0 | x@(66-x) | 22@22 | x@223 x@14 | x@(44-x)3 x@(65-x)4 | 4@223 4@44 | 4@403 4@584 | 3@34 | 3@604 |
| 200MHz5 | 60 | DFT-s | 256@0 | x@0 | x@(264-x) | 81@883 81@814 | x@883 x@14 | x@(176-x)3 x@(263-x)4 | 4@883 8@84 | 4@1723 8@2484 | 6@64 | 6@2524 |
|  |  | CP | 264@0 | x@0 | x@(264-x) | 88@88 | x@883 x@14 | x@(176-x)3 x@(263-x)4 | 4@883 7@74 | 4@1723 7@2504 | 6@64 | 6@2524 |
|  | 120 | DFT-s | 128@0 | x@0 | x@(132-x) | 40@443 40@404 | x@443 x@14 | x@(88-x)3 x@(131-x)4 | 4@443 4@44 | 4@843 4@1244 | 3@34 | 3@1264 |
|  |  | CP | 132@0 | x@0 | x@(132-x) | 44@44 | x@443 x@14 | x@(88-x)3 x@(131-x)4 | 4@443 4@44 | 4@843 4@1244 | 3@34 | 3@1264 |
| 400MHz5 | 60 | DFT-s | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  | CP | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 120 | DFT-s | 256@0 | x@0 | x@(264-x) | 64@66 | x@66 | x@(198-x) | 4@66 | 4@194 | N/A | N/A |
|  |  | CP | 264@0 | x@0 | x@(264-x) | 66@66 | x@66 | x@(198-x) | 4@66 | 4@194 | N/A | N/A |
| Note 1: RB allocation is left aligned within inner region. Note 2: Inner_Full allocation is selected as the largest RB allocation within Region 1 inner allocation defined in 6.2.2.3.3; Inner_Partial_Left and Inner_Partial_Right are selected as partial allocation within Region 1 inner allocation which are not impacted by MPRnarrow defined in 6.2.2.3.3; Inner_Partial2_Left and Inner_Partial2_Right are selected as partial allocation within Region 1 inner allocation which are impacted by MPRnarrow defined in 6.2.2.3.3 when MPRnarrow=2 dB. Note 3: Applicable to Rel-15 PC3 devices which do not support modifiedMPR-Behaviour bit 0 capability (according to Annex P.1) and to Rel-15 and forward PC2 and PC4 devices.. Note 4: Applicable to Rel-15 PC3 devices which supports modifiedMPR-Behaviour bit 0 capability (according to Annex P.1) and Rel-16 and forward PC3 devices. Note 5: The 200MHz and 400MHz bandwidths are not applicable to PC7 RedCap UEs. Note 6: In case of transform precoding, applicable only if x=2α2.3α3.5α5, where is a set of non- negative integers. |  |  |  |  |  |  |  |  |  |  |  |  |

<End of change 1>