3GPP TSG-RAN5 Meeting #104		R5-246054

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

Agenda item:	5.3.31.9

Source: 	Qualcomm Inc

Title: 	UE beam correspondence for initial access

Document for:	Discussion and Endorsement

## 1.	Introduction

In response to the RAN4 LS [1] to RAN5 for the FR2 UE beam correspondence for initial access and RRC_inactive requirements, there has been several discussions in previous RAN5 meetings. The latest WF is captured in [2]. This paper discusses the alternate method without requiring beam lock test function.

## 2.	Discussion

TS 38.101-2 [3] has defined the FR2 beam correspondence requirements for RACH. Intention is to evaluate UE’s spherical coverage EIRP metric during RACH.

In order to test this requirement, the discussions so far have primarily centered on sending a beamlock function to the UE so that UE can lock its beam before sending RACH for EIRP measurement.

This approach has several complications. Notably, beamlock function has to be sent while UE is in RRC connected mode and then the beamlock settings need to be carried over while the call is released and UE goes to RRC_Idle mode. This beamlock function has no practical purpose in real deployment.

For the alternate method without the need for beamlock function, we considered the RRM random access test case as the reference configuration.

Below are the RACH config parameters for this test case.

As per the RAN4 LS [1] to RAN5,

Enabling multiple PRACH transmissions

Modify the preambleTransMax IE value to a large value to allow multiple PRACH transmissions. Max allowed value as per TS 38.331 is n200.

For each received PRACH transmission, TE can either hold off on sending RAR or send an RAR containing Random access preamble identifier that do not match the transmitted random access preamble.

UE computed power reaches Pcmax before PRACH EIRP measurement

Modify the preambleReceivedTargetPower IE to a large value such that UE computed Tx power reaches Pcmax. Max allowed value as per TS 38.331 is value -60.

Accumulated PRACH power measurement period is atleast 1ms

prach-ConfigurationIndex value 190 corresponds to PRACH format C2. An even longer applicable PRACH format B4 can be configured for the test to reduce the number of PRACH needed to fulfil the 1ms PRACH measurement period.

# of SSB and SSB power level

Below snippets from TS 38.101-2 provide info on the SSB configuration

For PRACH EIRP measurement, only SSB needs to be configured. SSB power level to be configured as per Table 6.6.4.3.1-1. The requirements do not refer to the number of SSB to be configured.

Configuring a single SSB for this test will ensure UE has only one SSB to measure and send its PRACH towards the direction of the SSB.

With the above mentioned changes on top of the RRM random access test case, it is feasible to enable

PRACH EIRP spherical coverage measurement without the use of any beamlock function.

Proposal 1: Configure the FR2 beam correspondence for initial access as per the above mentioned changes using RRM random access test case as baseline configuration.

## 2.1	Summary of noBeamlock vs withBeamlock

| Considerations | with Beamlock function | NO beamlock (using available 38.331 IE parameters to measure FR2 RACH EIRP) |
| --- | --- | --- |
| Complexity | Complicated procedure. -- TE needs to send beamlock during RRC_CONN and then expect UE to go to RRC_Idle mode, refine its beam and then decide to lock after X seconds until it gets into RRC_CONN mode again. -- Allowing initial beam refinement time of 4seconds proposed in Huawei paper (R5-245019) means UE could very well exhaust all its n200 PRACH transmissions by the time 4 seconds elapses and it declares RLF Cons: very complicated procedure with question marks still on actual feasibility. | Straightforward way to configure the test using parameters allowed by 38.331 spec |
| Usability in real network | Since it is a conformance test function, there is no use in real network deployments. The reason RAN4 WI for PRACH EIRP measurement was introduced was to allow infra vendors/operators a meaningful way to measure UE's PRACH EIRP spherical coverage Cons: No use in real network. Defeats the main purpose of introducing the RAN4 WI | Since the entire procedure is based on 38.331 spec allowed parameters, very useful to extrapolate the results to real network deployments |
| RAN4 already provided guidance on using Beamlock function to verify the requirements | RAN4 has provided guidance to RAN5 via LS and also included text on beamlock in the TR (snippet pasted in Apple’s email below) But all these are only guidance to help RAN5 with the repeatability of the test. Note: There is no Beamlock requirement in the TS 38.101-2 requirements clause. | As demonstrated in paper (R5-245297) RAN4 objectives can be met without beamlock. No benefit in going with Beamlock function due to the Cons listed above |
| Concern#1: UE can switch beams without Beamlock | Even the beamlock proposal cannot eliminate switching of UE beams which is why a beam refinement wait time of 4seconds is being proposed by Huawei (R5-245019) which as highlighted above is too high during Idle mode and puts question mark on the feasibility of the test | Cannot eliminate UE switching its beams. But based on the carefully selected config parameters, UE is expected to settle on its optimal beam after ‘X’ PRACH transmissions. Further details on how TE picks the RACH transmissions for EIRP measurement period can be discussed once we converge on the primary approach |
| Concern#2: UE can start Tx with sub-optimal beams without Beamlock | Similar comments as above | Similar comments as above |
| Concern#3: UE can switch beams after receiving invalid RAR or not receiving any RAR for long time |  | 38.321 requires UE to keep performing its RACH procedure when it receives no RAR or invalid RAR. Together with appropriately selected RRC timer T304, it is possible to ensure UE keeps transmitting RACH. With a single SSB configured in the test, a reasonable UE is expected to send its RACH towards the direction of the SSB |

## 3.	Conclusion

Proposal 1: Configure the FR2 beam correspondence for initial access as per the above mentioned changes using RRM random access test case as baseline configuration.

## 4.		References

[1]		R4-2317764, LS on testability issues of beam correspondence requirement for initial access, 	Huawei

[2]		R5-243159, Discussion on FR2 beam correspondence for initial access, Huawei

[3]	TS 38.101-2 v18.6.0