3GPP TSG-RAN5 Meeting #104	 R5-246064

# Maastricht, Netherlands, August 19th - August 23rd 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 3376 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to BFD-LR test cases 4.5.5.1,4.5.5.2,5.5.5.1,5.5.5.2,6.5.5.1 and 6.5.5.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Korea |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - With current PDCCH search space configuration, issue seen while decoding DCI having msg2 post BFD for test cases 4.5.5.1,4.5.5.2,5.5.5.1,5.5.5.2,6.5.5.1 and 6.5.5.2. Similar correction in DCI configuration were made for TC 5.5.5.5 using R5-241836. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - PDCCH Search space configuration is added for USS in test cases 4.5.5.1,4.5.5.2,5.5.5.1,5.5.5.2,6.5.5.1 and 6.5.5.2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TC 4.5.5.1,4.5.5.2,5.5.5.1,5.5.5.2,6.5.5.1 and 6.5.5.2 will remain misconfigured. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5.5.1,4.5.5.2,5.5.5.1,5.5.5.2,6.5.5.1,6.5.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1:Updated reason for change and summary for change in CR coversheet. R5-245038 > R5-245038r1 > R5-246064 |  |  |  |  |  |  |  |  |

<< Start of Changes >>

#### 4.5.5.1	EN-DC FR1 SSB-based beam failure detection and link recovery in non-DRX

4.5.5.1.1	Test purpose

The purpose of this test is to verify that the UE properly detects SSB-based beam failure in the set q0 configured for a serving PSCell and that the UE performs correct SSB-based link recovery based on beam candidate set q1. The purpose is to test the downlink monitoring for beam failure detection within the UEs active DL BWP of the PSCell, during the evaluation period, and link recovery, when no DRX is used. This test will partly verify the SSB based beam failure detection and link recovery for an FR1 serving cell requirements in TS 38.133 [6] clause 8.5.

4.5.5.1.2	Test applicability

This test applies to all types of E-UTRA UE release 15 and forward, supporting EN-DC and link recovery.

4.5.5.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 4.5.5.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.4.5.5.1.

4.5.5.1.4	Test description

The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure 4.5.5.1.4-1 shows the variation of the downlink SNR of the PCell and the SNR of the SSB in set q0 in the active PSCell to emulate SSB based beam failure. Figure 4.5.5.1.4-1 additionally shows the variation of the downlink L1-RSRP of the SSB in set q1 of the candidate beam used for link recovery.

Figure 4.5.5.1.4-1: SNR and L1-RSRP variation for SSB-based beam failure detection and link recovery testing in non-DRX mode

4.5.5.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 4.5.5.1.4.1-1.

Table 4.5.5.1.4.1-1: Supported test configurations for
EN-DC FR1 SSB-based beam failure detection and link recovery in non-DRX

| Configuration | Description |
| --- | --- |
| 4.5.5.1-1 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 4.5.5.1-2 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 4.5.5.1-3 | LTE FDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 4.5.5.1-4 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 4.5.5.1-5 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 4.5.5.1-6 | LTE TDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to pass in one of the supported test configurations in FR1. |  |

Configure the test equipment and the DUT according to the parameters in Table 4.5.5.1.4.1-2.

Table 4.5.5.1.4.1-2: Initial conditions for
EN-DC FR1 SSB-based beam failure detection and link recovery in non-DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.2-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 4.5.5.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part |  |  |

1.	The general test parameter settings are set up according to Table 4.5.5.1.4.1-3.

2.	Message contents are defined in clause 4.5.5.1.4.3.

3.	Cell 1 is the E-UTRA serving cell (PCell) for the EN-DC setup. The power levels and settings for Cell 1 are set according to Annex A.6. Cell 2 is the NR cell (PSCell) with the power level set according to clauses C.1.2 and C.1.3 for this test

Table 4.5.5.1.4.1-3: General test parameters for FR1 PSCell for
SSB-based beam failure detection and link recovery testing in non-DRX mode

| Parameter |  |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Active E-UTRA PCell |  |  |  | Cell 1 |  |
| E-UTRA RF Channel Number |  |  |  | 1 |  |
| Active PSCell |  |  |  | Cell 2 |  |
| RF Channel Number |  |  |  | 2 |  |
| Duplex mode |  | Config 1, 4 |  | FDD |  |
|  |  | Config 2, 3, 5, 6 |  | TDD |  |
| BWchannel |  | Config 1, 4 | MHz | 10: NRB,c = 52 |  |
|  |  | Config 2, 5 |  | 10: NRB,c = 52 |  |
|  |  | Config 3, 6 |  | 40: NRB,c = 106 |  |
| DL initial BWP configuration |  | Config 1, 2, 3, 4, 5, 6 |  | DLBWP.0.1 |  |
| DL dedicated BWP configuration |  | Config 1, 2, 3, 4, 5, 6 |  | DLBWP.1.1 |  |
| UL initial BWP configuration |  | Config 1, 2, 3, 4, 5, 6 |  | ULBWP.0.1 |  |
| UL dedicated BWP configuration |  | Config 1, 2, 3, 4, 5, 6 |  | ULBWP.1.1 |  |
| TDD Configuration |  | Config 1, 4 |  | Not Applicable |  |
|  |  | Config 2, 5 |  | TDDConf.1.1 |  |
|  |  | Config 3, 6 |  | TDDConf.2.1 |  |
| CORESET Reference Channel |  | Config 1, 4 |  | CR.1.1 FDD |  |
|  |  | Config 2, 5 |  | CR.1.1 TDD |  |
|  |  | Config 3, 6 |  | CR.2.1 TDD |  |
| #Dedicated CORESET Reference Channel |  | Config 1, 4 |  | CCR.1.1 FDD |  |
|  |  | Config 2, 5 |  | CCR.1.1 TDD |  |
|  |  | Config 3, 6 |  | CCR.2.1 TDD |  |
| SSB Configuration |  | Config 1, 4 |  | SSB.3 FR1 |  |
|  |  | Config 2, 5 |  | SSB.3 FR1 |  |
|  |  | Config 3, 6 |  | SSB.4 FR1 |  |
| SMTC Configuration |  | Config 1, 2, 4, 5 |  | SMTC.1 |  |
|  |  | Config 3, 6 |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1, 2, 4, 5 |  | 15 KHz |  |
|  |  | Config 3, 6 |  | 30 KHz |  |
| PRACH Configuration |  | Config 1, 2, 4, 5 |  | PRACH.2 FR1 | CFRA for BFR |
|  |  | Config 3, 6 |  | PRACH.2 FR1 | CFRA for BFR |
| SSB Index assigned as BFD RS (q0) |  |  |  | 0 |  |
| SSB Index assigned as CBD RS (q1) |  |  |  | 1 |  |
| OCNG parameters |  |  |  | OP.1 |  |
| CP length |  |  |  | Normal |  |
| Correlation Matrix and Antenna Configuration |  |  |  | 2x2 Low |  |
| Beam failure detection transmission parameters | DCI format |  |  | 1-0 |  |
|  | Number of Control OFDM symbols |  |  | 2 |  |
|  | Aggregation level |  | CCE | 8 |  |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy |  | dB | 0 |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy |  | dB | 0 |  |
|  | DMRS precoder granularity |  |  | REG bundle size |  |
|  | REG bundle size |  |  | 6 |  |
| DRX |  |  |  | OFF |  |
| Gap pattern ID |  |  |  | gp0 |  |
| gapOffset |  |  |  | 0 |  |
| rlmInSyncOutOfSyncThreshold |  |  |  | absent | When the field is absent, the UE applies the value 0. |
| rsrp-ThresholdSSB | Config 1, 2, 4, 5 |  | dBm/SCS kHz | -98 | Threshold used for Qin_LR_SSB |
|  | Config 3, 6 |  | dBm/SCS kHz | -95 | Threshold used for Qin_LR_SSB |
| powerControlOffsetSS |  |  |  | db0 | Used for deriving rsrp-ThresholdCSI-RS |
| beamFailureInstanceMaxCount |  |  |  | n1 | see TS 38.321 [12], clause 5.17 |
| beamFailureDetectionTimer |  |  |  | pbfd4 | see TS 38.321 [12], clause 5.17 |
| CSI-RS configuration for CSI reporting | Config 1, 4 |  |  | CSI-RS.1.1 FDD |  |
|  | Config 2, 5 |  |  | CSI-RS.1.1 TDD |  |
|  | Config 3, 6 |  |  | CSI-RS.2.1 TDD |  |
| CSI-RS for tracking | Config 1, 4 |  |  | TRS.1.1 FDD |  |
|  | Config 2, 5 |  |  | TRS.1.1 TDD |  |
|  | Config 3, 6 |  |  | TRS.1.2 TDD |  |
| SSB Index assigned as RLM RS |  |  |  | 0,1 |  |
| T310 timer |  |  | ms | 1000 |  |
| N310 |  |  |  | 2 |  |
| T1 |  |  | s | 0.2 | During this time the UE shall be fully synchronized to cell 1 |
| T2 |  |  | s | 0.37 |  |
| T3 |  |  | s | 0.24 |  |
| T4 |  |  | s | 0 |  |
| T5 |  |  | s | 0.17 |  |
| D1 |  |  | s | 0.13 |  |
| NOTE 1: All configurations are assigned to the UE prior to the start of time period T1. NOTE 2: UE-specific PDCCH is not transmitted after T1 starts. NOTE 3: E-UTRAN is in non-DRX mode under test. |  |  |  |  |  |

4.5.5.1.4.2	Test procedure

Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1 and cell 2. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5 ms. In the test, DRX configuration is not enabled. The UE is configured to perform inter-frequency measurements using GP ID #0 (40ms).

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	The SS sends an RRCReconfiguration (embeded in RRCConnectionReconfiguration message) message to the UE to configure inter-frequency measurement.

3.	The UE sends an RRCReconfigurationComplete (embeded in RRCConnectionReconfigurationComplete message) message.

4.	Set the parameters of NR Cell according to T1 in Table 4.5.5.1.5-1. Propagation conditions are set according to clause C.2.3. T1 starts.

5.	When T1 expires the SS shall change the SNR value to T2 as specified in Table 4.5.5.1.5-1. T2 starts.

6.	When T2 expires the SS shall change the SNR value to T3 as specified in Table 4.5.5.1.5-1. T3 starts.

7.	When T3 expires the SS shall change the SNR value to T4 as specified in Table 4.5.5.1.5-1. T4 starts.

8.	When T4 expires the SS shall change the SNR value to T5 as specified in Table 4.5.5.1.5-1. T5 starts.

9.	If the SS:

a)	detects uplink power on NR carrier equal to or higher than minimum output power defined in TS 38.521-1 [17] clause 6.3.1.5 in each slot configured for CSI transmission (according CSI reporting on PUCCH) during the period from time point A to time point B; and

b)	does not detect preamble on a beam associated with the candidate beam set q1before time point B; and

c)	detects preamble on a beam associated with the candidate beam set q1 before time point F (D1 after the star of T5),the number of successful tests is increased by one.

Otherwise the number of failed tests is increased by one.

10.	If the iteration or random access procedure for BFD fails, the SS shall first attempt to release and add the PSCell, by ensuring the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On and Test Mode On according to TS 38.508-1 [6] clause 4.5. If that also fails, then the UE is switched OFF/ON to proceed with the next iteration.

11.	Repeat steps 2-10 for all subtests until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

4.5.5.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 4.5.5.1.4.3-1: Common Exception messages for
EN-DC FR1 SSB-based beam failure detection and link recovery in non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Condition INTER-FREQ, L3 FILTERING NEEDED, Table H.3.1-3 with Condition INTER-FREQ MO (where ssbFrequency is set to the ARFCN value of carrier center of High range) Table H.3.1-4 with A3-offset = 0 Table H.3.1-8 with Condition SSB BFD Table H.3.1-10 with Condition SSB Table H.3.1-10A Table H.3.4-4 with Condition gapUE Table H.3.4-5 with Condition BFD Table H.3.5-4 Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 4.5.5.1.4.3-2: PDCCH Search Space for BFR

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-162 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| searchSpaceId | 3 | BFR |  |
| controlResourceSetId | 2 | BFR |  |
| monitoringSlotPeriodicityAndOffset CHOICE { |  |  |  |
| sl1 | NULL |  |  |
| } |  |  |  |
| monitoringSymbolsWithinSlot | 10000000000000 | Symbols 0 and 1 |  |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel1 | n0 |  |  |
| aggregationLevel2 | n0 |  |  |
| aggregationLevel4 | n0 |  |  |
| aggregationLevel8 | n1 | AL8 |  |
| aggregationLevel16 | n0 |  |  |
| } |  |  |  |
| searchSpaceType CHOICE { |  |  |  |
| ue-Specific SEQUENCE { |  |  | USS |
| dci-Formats | formats0-0-And-1-0 | DCI Format 1_0 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 4.5.5.1.4.3-3: RLF-TimersAndConstants

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-150 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLF-TimersAndConstants ::= SEQUENCE { |  |  |  |
| n310 | n2 |  |  |
| } |  |  |  |

Table 4.5.5.1.4.3-4: PDCCH-Config

| Derivation Path: TS 38.501-1 [14],Table 4.6.3-95 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCCH-Config ::= SEQUENCE { |  |  |  |
| controlResourceSetToAddModList SEQUENCE(SIZE (1..3)) OF ControlResourceSet { | 2 entries |  |  |
| ControlResourceSet[2] | ControlResourceSet | entry 2, BFR |  |
| } |  |  |  |
| controlResourceSetToReleaseList | Not present |  |  |
| searchSpacesToAddModList SEQUENCE(SIZE (1..10)) OF SearchSpace { | 2 entries |  |  |
| SearchSpace[2] | SearchSpace | entry 2, BFR |  |
| } |  |  |  |
| searchSpacesToReleaseList | Not present |  |  |
| downlinkPreemption | Not present |  |  |
| tpc-PUSCH | Not present |  |  |
| tpc-PUCCH | Not present |  |  |
| tpc-SRS | Not present |  |  |
| } |  |  |  |

Table 4.5.5.1.4.3-5: ControlResourceSet for BFR

| Derivation Path: TS 38.501-1 [14],Table 7.3.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| controlResourceSetId | 2 |  |  |
| duration | 2 |  |  |
| cce-REG-MappingType CHOICE { |  |  |  |
| interleaved ::= SEQUENCE { |  |  |  |
| reg-BundleSize | n6 |  |  |
| interleaverSize | n2 |  |  |
| shiftIndex | 0 |  |  |
| } |  |  |  |
| tci-StatesPDCCH-ToAddList | Not present |  |  |
| pdcch-DMRS-ScramblingID | 1008 | DMRS scrambling ID different from Physical Cell ID |  |
| } |  |  |  |

Table 4.5.5.1.4.3-6: PDCCH Search Space for USS

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-162 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| searchSpaceId | 2 |  |  |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel1 | n0 |  |  |
| aggregationLevel2 | n1 |  |  |
| aggregationLevel4 | n1 |  |  |
| aggregationLevel8 | n0 |  |  |
| } |  |  |  |
| } |  |  |  |

4.5.5.1.5	Test requirements

Tables 4.5.5.1.4.1-3 and 4.5.5.1.5-1 define the primary level settings including test tolerances for EN-DC FR1 SSB-based beam failure detection and link recovery in non-DRX.

Table 4.5.5.1.5-1: Cell specific test parameters for FR1 PSCell for
SSB-based beam failure detection and link recovery testing in non-DRX mode

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| SNR_SSB of set q0 | Config 1, 4 | dB | 5.8 | -2.2 | -12.8 | -12.8 | -12.8 |
|  | Config 2, 5 |  | 5.8 | -2.2 | -12.8 | -12.8 | -12.8 |
|  | Config 3, 6 |  | 5.8 | -2.2 | -12.8 | -12.8 | -12.8 |
| SNR_SSB of set q1 | Config 1, 4 | dB | -10.2 | -10.2 | 10.2 | 10.2 | 10.2 |
|  | Config 2, 5 |  | -10.2 | -10.2 | 10.2 | 10.2 | 10.2 |
|  | Config 3, 6 |  | -10.2 | -10.2 | 10.2 | 10.2 | 10.2 |
| SSB_RP of set q1 | Config 1, 4 | dBm/SCS kHz | -108.2 | -108.2 | -87.8 | -87.8 | -87.8 |
|  | Config 2, 5 |  | -108.2 | -108.2 | -87.8 | -87.8 | -87.8 |
|  | Config 3, 6 |  | -105.2 | -105.2 | -84.8 | -84.8 | -84.8 |
|  | Config 1, 4 | dBm/15 kHz | -98 |  |  |  |  |
|  | Config 2, 5 |  | -98 |  |  |  |  |
|  | Config 3, 6 |  | -98 |  |  |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz |  |  |  |  |
| NOTE 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. NOTE 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. NOTE 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. NOTE 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. NOTE 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. NOTE 7: SNR levels correspond to the signal to noise ratio over the SSS REs. NOTE 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure 4.5.5.1.4-1. NOTE 9: The SNR values are specified for a UE with 2RX antennas connected under test. For a UE with 4RX antennas connected under test, the SNR for RS in set q0 during T3, T4, and T5 from D.4.1.1, is -15dB-TT = -15.8dB (including test tolerances). |  |  |  |  |  |  |  |

The UE behaviour during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the time duration T1 and T2, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on Cell 1.

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 1 in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 1.

During T3 the UE shall detect beam failure and initiate link recovery. During T4 and T5 the UE measures and evaluate beam candidate from beam candidate set q1.

No later than time point F occurring no later than D1 = 130 ms after the start of T5, the UE shall transmit preamble on a beam associated with the candidate beam set q1. The UE shall not transmit preamble on a beam associated with the candidate beam set q1 earlier than time point B.

Test is concluded once the test equipment has received the initial preamble transmission from the UE. The rate of correct events observed during repeated tests shall be at least 90%.

#### 4.5.5.2	EN-DC FR1 SSB-based beam failure detection and link recovery in DRX

4.5.5.2.1	Test purpose

The purpose of this test is to verify that the UE properly detects SSB-based beam failure in the set q0 configured for a serving PSCell and that the UE performs correct SSB-based link recovery based on beam candidate set q1. The purpose is to test the downlink monitoring for beam failure detection within the UEs active DL BWP of the PSCell, during the evaluation period, and link recovery, when DRX is used. This test will partly verify the SSB based beam failure detection and link recovery for an FR1 serving cell requirements in TS 38.133 [6] clause 8.5.

4.5.5.2.2	Test applicability

This test applies to all types of E-UTRA UE release 15 and forward, supporting EN-DC FR1, link recovery and long DRX cycle.

4.5.5.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 4.5.5.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.4.5.5.2.

4.5.5.2.4	Test description

The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure 4.5.5.2.4-1 shows the variation of the downlink SNR of the PCell and the SNR of the SSB in set q0 in the active PSCell to emulate SSB based beam failure. Figure 4.5.5.2.4-1 additionally shows the variation of the downlink L1-RSRP of the SSB in set q1 of the candidate beam used for link recovery.

Figure 4.5.5.2.4-1: SNR and L1-RSRP variation for SSB-based beam failure detection and
link recovery testing in DRX mode

4.5.5.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 4.5.5.2.4.1-1.

Table 4.5.5.2.4.1-1: Supported test configurations for SSB-based beam failure detection and
link recovery testing in DRX mode

| Configuration | Description |
| --- | --- |
| 4.5.5.2-1 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 4.5.5.2-2 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 4.5.5.2-3 | LTE FDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 4.5.5.2-4 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 4.5.5.2-5 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 4.5.5.2-6 | LTE TDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to pass in one of the supported test configurations in FR1. |  |

Configure the test equipment and the DUT according to the parameters in Table 4.5.5.2.4.1-2.

Table 4.5.5.2.4.1-2: Initial conditions for EN-DC FR1 SSB-based beam failure detection and
link recovery in DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.2-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 4.5.5.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part |  |  |

1.	The general test parameter settings are set up according to Table 4.5.5.2.4.1-3.

2.	Message contents are defined in clause 4.5.5.2.4.3.

3.	Cell 1 is the E-UTRA serving cell (PCell) for the EN-DC setup. The power levels and settings for Cell 1 are set according to Annex A.6. Cell 2 is the NR cell (PSCell) with the power level set according to clauses C.1.2 and C.1.3 for this test

Table 4.5.5.2.4.1-3: General test parameters for FR1 PCell for
SSB-based beam failure detection and link recovery testing in DRX mode

| Parameter |  |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |  |
| Active E-UTRA PCell |  |  |  | Cell 1 |  |
| E-UTRA RF Channel Number |  |  |  | 1 |  |
| Active PSCell |  |  |  | Cell 2 |  |
| RF Channel Number |  |  |  | 2 |  |
| Duplex mode |  | Config 1, 4 |  | FDD |  |
|  |  | Config 2, 3, 5, 6 |  | TDD |  |
| BWchannel |  | Config 1, 4 | MHz | 10: NRB,c = 52 |  |
|  |  | Config 2, 5 |  | 10: NRB,c = 52 |  |
|  |  | Config 3, 6 |  | 40: NRB,c = 106 |  |
| DL initial BWP configuration |  | Config 1, 2, 3, 4, 5, 6 |  | DLBWP.0.1 |  |
| DL dedicated BWP configuration |  | Config 1, 2, 3, 4, 5, 6 |  | DLBWP.1.1 |  |
| UL initial BWP configuration |  | Config 1, 2, 3, 4, 5, 6 |  | ULBWP.0.1 |  |
| UL dedicated BWP configuration |  | Config 1, 2, 3, 4, 5, 6 |  | ULBWP.1.1 |  |
| TDD Configuration |  | Config 1, 4 |  | Not Applicable |  |
|  |  | Config 2, 5 |  | TDDConf.1.1 |  |
|  |  | Config 3, 6 |  | TDDConf.2.1 |  |
| CORESET Reference Channel |  | Config 1, 4 |  | CR. 1.1 FDD |  |
|  |  | Config 2, 5 |  | CR. 1.1 TDD |  |
|  |  | Config 3, 6 |  | CR. 2.1 TDD |  |
| Dedicated CORESET Reference Channel |  | Config 1, 4 |  | CCR.1.1 FDD |  |
|  |  | Config 2, 5 |  | CCR.1.1 TDD |  |
|  |  | Config 3, 6 |  | CCR.2.1 TDD |  |
| SSB Configuration |  | Config 1, 4 |  | SSB.3 FR1 |  |
|  |  | Config 2, 5 |  | SSB.3 FR1 |  |
|  |  | Config 3, 6 |  | SSB.4 FR1 |  |
| SMTC Configuration |  | Config 1, 2, 4, 5 |  | SMTC.1 |  |
|  |  | Config 3, 6 |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1, 2, 4, 5 |  | 15 KHz |  |
|  |  | Config 3, 6 |  | 30 KHz |  |
| PRACH Configuration |  | Config 1, 2, 4, 5 |  | PRACH.2 FR1 | CFRA for BFR |
|  |  | Config 3, 6 |  | PRACH.2 FR1 | CFRA for BFR |
| SSB Index assigned as BFD RS (q0) |  |  |  | 0 |  |
| SSB Index assigned as CBD RS (q1) |  |  |  | 1 |  |
| OCNG parameters |  |  |  | OP.1 |  |
| CP length |  |  |  | Normal |  |
| Correlation Matrix and Antenna Configuration |  |  |  | 2x2 Low |  |
| Beam failure detection transmission parameters | DCI format |  |  | 1-0 |  |
|  | Number of Control OFDM symbols |  |  | 2 |  |
|  | Aggregation level |  | CCE | 8 |  |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy |  | dB | 0 |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy |  | dB | 0 |  |
|  | DMRS precoder granularity |  |  | REG bundle size |  |
|  | REG bundle size |  |  | 6 |  |
| DRX |  |  |  | DRX.7 |  |
| Gap pattern ID |  |  |  | N.A. |  |
| rlmInSyncOutOfSyncThreshold |  |  |  | absent | When the field is absent, the UE applies the value 0. |
| rsrp-ThresholdSSB | Config 1, 2, 4, 5 |  | dBm/SCS kHz | -98 | Threshold used for Qin_LR_SSB |
|  | Config 3, 6 |  | dBm/SCS kHz | -95 | Threshold used for Qin_LR_SSB |
| powerControlOffsetSS |  |  |  | db0 | Used for deriving rsrp-ThresholdCSI-RS |
| beamFailureInstanceMaxCount |  |  |  | n1 | see TS 38.321 [12], clause 5.17 |
| beamFailureDetectionTimer |  |  |  | pbfd4 | see TS 38.321 [12], clause 5.17 |
| CSI-RS configuration for CSI reporting | Config 1, 4 |  |  | CSI-RS.1.1 FDD |  |
|  | Config 2, 5 |  |  | CSI-RS.1.1 TDD |  |
|  | Config 3, 6 |  |  | CSI-RS.2.1 TDD |  |
| CSI-RS for tracking | Config 1, 4 |  |  | TRS.1.1 FDD |  |
|  | Config 2, 5 |  |  | TRS.1.1 TDD |  |
|  | Config 3, 6 |  |  | TRS.1.2 TDD |  |
| SSB Index assigned as RLM RS |  |  |  | 0,1 |  |
| T310 Timer |  |  | ms | 1000 |  |
| N310 |  |  |  | 2 |  |
| T1 |  |  | s | 1 | During this time the UE shall be fully synchronized to cell 1 |
| T2 |  |  | s | 5.17 |  |
| T3 |  |  | s | 3.24 |  |
| T4 |  |  | s | 0 |  |
| T5 |  |  | s | 1.97 |  |
| D1 |  |  | s | 1.93 |  |
| NOTE 1: All configurations are assigned to the UE prior to the start of time period T1. NOTE 2: UE-specific PDCCH is not transmitted after T1 starts. NOTE 3: E-UTRAN is in non-DRX mode under test. |  |  |  |  |  |

4.5.5.2.4.2	Test procedure

Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1 and cell 2. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5 ms. In the test, DRX configuration is enabled in PSCell and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CQI during the period when On-duration timer is running. Time alignment timers shall be set to "infinity" so that UL timing alignment is maintained during the test.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters of NR Cell 1 according to T1 in Table 4.5.5.2.5-1. Propagation conditions are set according to clause C.2.3. T1 starts.

3.	When T1 expires the SS shall change the SNR value to T2 as specified in Table 4.5.5.2.5-1. T2 starts.

4.	When T2 expires the SS shall change the SNR value to T3 as specified in Table 4.5.5.2.5-1. T3 starts.

5.	When T3 expires the SS shall change the SNR value to T4 as specified in Table 4.5.5.2.5-1. T4 starts.

6.	When T4 expires the SS shall change the SNR value to T5 as specified in Table 4.5.5.2.5-1. T5 starts.

7.	If the SS:

a)	detects uplink power on NR carrier equal to or higher than minimum output power defined in TS 38.521-1 [17] clause 6.3.1.5 in each slot configured for CSI transmission (according CSI reporting on PUCCH) during the period from time point A to time point B; and

b)	does not detect preamble on a beam associated with the candidate beam set q1before time point B; and

c)	detects preamble on a beam associated with the candidate beam set q1 before time point F (D1 after the start of T5), the number of successful tests is increased by one.

Otherwise the number of failed tests is increased by one.

8.	If the iteration or random access procedure for BFD fails, the SS shall first attempt to release and add the PSCell, by ensuring the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On and Test Mode On according to TS 38.508-1 [6] clause 4.5. If that also fails, then the UE is switched OFF/ON to proceed with the next iteration.

9.	Repeat steps 2-8 for all subtests until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

4.5.5.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 4.5.5.2.4.3-1: Common Exception messages for
EN-DC FR1 SSB-based beam failure detection and link recovery in DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-8 with Condition SSB BFD Table H.3.1-10 with Condition SSB Table H.3.1-10A Table H.3.5-4 Table H.3.7-1 with Condition DRX.7 Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 4.5.5.2.4.3-2: PDCCH Search Space for BFR

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-162 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| searchSpaceId | 3 | BFR |  |
| controlResourceSetId | 2 | BFR |  |
| monitoringSlotPeriodicityAndOffset CHOICE { |  |  |  |
| sl1 | NULL |  |  |
| } |  |  |  |
| monitoringSymbolsWithinSlot | 10000000000000 | Symbols 0 and 1 |  |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel1 | n0 |  |  |
| aggregationLevel2 | n0 |  |  |
| aggregationLevel4 | n0 |  |  |
| aggregationLevel8 | n1 | AL8 |  |
| aggregationLevel16 | n0 |  |  |
| } |  |  |  |
| searchSpaceType CHOICE { |  |  |  |
| ue-Specific SEQUENCE { |  |  | USS |
| dci-Formats | formats0-0-And-1-0 | DCI Format 1_0 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 4.5.5.2.4.3-3: RLF-TimersAndConstants

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-150 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLF-TimersAndConstants ::= SEQUENCE { |  |  |  |
| n310 | n2 |  |  |
| } |  |  |  |

Table 4.5.5.2.4.3-4: PDCCH-Config

| Derivation Path: TS 38.501-1 [14],Table 4.6.3-95 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCCH-Config ::= SEQUENCE { |  |  |  |
| controlResourceSetToAddModList SEQUENCE(SIZE (1..3)) OF ControlResourceSet { | 2 entries |  |  |
| ControlResourceSet[2] | ControlResourceSet | entry 2, BFR |  |
| } |  |  |  |
| controlResourceSetToReleaseList | Not present |  |  |
| searchSpacesToAddModList SEQUENCE(SIZE (1..10)) OF SearchSpace { | 2 entries |  |  |
| SearchSpace[2] | SearchSpace | entry 2, BFR |  |
| } |  |  |  |
| searchSpacesToReleaseList | Not present |  |  |
| downlinkPreemption | Not present |  |  |
| tpc-PUSCH | Not present |  |  |
| tpc-PUCCH | Not present |  |  |
| tpc-SRS | Not present |  |  |
| } |  |  |  |

Table 4.5.5.2.4.3-5: ControlResourceSet for BFR

| Derivation Path: TS 38.501-1 [14],Table 7.3.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| controlResourceSetId | 2 |  |  |
| duration | 2 |  |  |
| cce-REG-MappingType CHOICE { |  |  |  |
| interleaved ::= SEQUENCE { |  |  |  |
| reg-BundleSize | n6 |  |  |
| interleaverSize | n2 |  |  |
| shiftIndex | 0 |  |  |
| } |  |  |  |
| tci-StatesPDCCH-ToAddList | Not present |  |  |
| pdcch-DMRS-ScramblingID | 1008 | DMRS scrambling ID different from Physical Cell ID |  |
| } |  |  |  |

Table 4.5.5.2.4.3-6: PDCCH Search Space for USS

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-162 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| searchSpaceId | 2 |  |  |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel1 | n0 |  |  |
| aggregationLevel2 | n1 |  |  |
| aggregationLevel4 | n1 |  |  |
| aggregationLevel8 | n0 |  |  |
| } |  |  |  |
| } |  |  |  |

4.5.5.2.5	Test requirements

Tables 4.5.5.2.4.1-3 and 4.5.5.2.5-1 define the primary level settings including test tolerances for EN-DC FR1 SSB-based beam failure detection and link recovery in DRX.

Table 4.5.5.2.5-1: Cell specific test parameters for FR1 PSCell for
SSB-based beam failure detection and link recovery testing in DRX mode

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| SNR_SSB of set q0 | Config 1, 4 | dB | 5.8 | -2.2 | -12.8 | -12.8 | -12.8 |
|  | Config 2, 5 |  | 5.8 | -2.2 | -12.8 | -12.8 | -12.8 |
|  | Config 3, 6 |  | 5.8 | -2.2 | -12.8 | -12.8 | -12.8 |
| SNR_SSB of set q1 | Config 1, 4 | dB | -10.2 | -10.2 | 10.2 | 10.2 | 10.2 |
|  | Config 2, 5 |  | -10.2 | -10.2 | 10.2 | 10.2 | 10.2 |
|  | Config 3, 6 |  | -10.2 | -10.2 | 10.2 | 10.2 | 10.2 |
| SSB_RP of set q1 | Config 1, 4 | dBm/SCS kHz | -108.2 | -108.2 | -87.8 | -87.8 | -87.8 |
|  | Config 2, 5 |  | -108.2 | -108.2 | -87.8 | -87.8 | -87.8 |
|  | Config 3, 6 |  | -105.2 | -105.2 | -84.8 | -84.8 | -84.8 |
|  | Config 1, 4 | dBm/15 kHz | -98 |  |  |  |  |
|  | Config 2, 5 |  | -98 |  |  |  |  |
|  | Config 3, 6 |  | -98 |  |  |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz |  |  |  |  |
| NOTE 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. NOTE 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. NOTE 4: Void NOTE 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. NOTE 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. NOTE 7: SNR levels correspond to the signal to noise ratio over the SSS REs. NOTE 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure 4.5.5.2.4-1. NOTE 9: The SNR values are specified for a UE with 2RX antennas connected under test. For a UE with 4RX antennas connected under test, the SNR for RS in set q0 during T3, T4, and T5 from D.4.1.1, is -15dB-TT = -15.8dB (including test tolerances). |  |  |  |  |  |  |  |

The UE behaviour during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the time duration T1 and T2, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on Cell 1.

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 1 in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 1.

During T3 the UE shall detect beam failure and initiate link recovery. During T4 and T5 the UE measures and evaluate beam candidate from beam candidate set q1.

No later than time point F occurring no later than D1 = 1930 ms after the start of T5, the UE shall transmit preamble on a beam associated with the candidate beam set q1. The UE shall not transmit preamble on a beam associated with the candidate beam set q1 earlier than time point B.

Test is concluded once the test equipment has received the initial preamble transmission from the UE. The rate of correct events observed during repeated tests shall be at least 90%.

#### 5.5.5.1	EN-DC FR2 SSB-based beam failure detection and link recovery in non-DRX

Editor's Note: This test case is complete for the following configurations:

- Test frequency f ≤ 40.8 GHz

- UE PC3

This test case is incomplete for Test frequency f > 40.8 GHz

This test case is incomplete for UE power class other than PC3.

5.5.5.1.1	Test purpose

The purpose of this test is to verify that the UE properly detects SSB-based beam failure in the set q0 configured for a serving PSCell and that the UE performs correct SSB-based link recovery based on beam candidate set q1. The purpose is to test he downlink monitoring for beam failure detection within the UEs active DL BWP of the PSCell, during the evaluation period, and link recovery, when no DRX is used. This test will partly verify the SSB based beam failure detection and link recovery for an FR2 serving cell requirements in TS 38.133 [6] clause 8.5.

5.5.5.1.2	Test applicability

This test applies to all types of NR UE supporting E-UTRA and EN-DC from release 15 onwards.

5.5.5.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 5.5.5.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.5.5.5.1.

5.5.5.1.4	Test description

There are two cells configured in this test: E-UTRAN PCell and NR PSCell. This test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure 5.5.5.1.4-1 shows the five different time durations and the corresponding variation of the downlink SNR of the PCell and the SNR of the SSB in set q0 in the active PSCell to emulate SSB based beam failure. Figure 5.5.5.1.4-2 shows the variation of the downlink L1-RSRP of the SSB in set q1 of the candidate beam used for link recovery.

Figure 5.5.5.1.4-1: SNR for EN-DC FR2 SSB-based beam failure detection and link recovery in non-DRX

Figure 5.5.5.1.4-2: SSB_RP level variation for EN-DC FR2 SSB-based beam failure detection and link recovery in non-DRX

5.5.5.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 5.5.5.1.4.1-1.

Table 5.5.5.1.4.1-1: Supported test configurations for EN-DC FR2 SSB-based beam failure detection and link recovery in non-DRX

| Configuration | Description |
| --- | --- |
| 5.5.5.1-1 | LTE FDD, NR TDD duplex mode, 120 kHz SSB SCS, 100 MHz bandwidth |
| 5.5.5.1-2 | LTE TDD, NR TDD duplex mode, 120 kHz SSB SCS, 100 MHz bandwidth |
| 5.5.5.1-3 | LTE FDD, NR TDD duplex mode, 240 kHz SSB SCS, 100 MHz bandwidth |
| 5.5.5.1-4 | LTE TDD, NR TDD duplex mode, 240 kHz SSB SCS, 100 MHz bandwidth |
| NOTE: The UE is only required to pass in one of the supported test configurations in FR2 |  |

Configure the test equipment and the DUT according to the parameters in Table 5.5.5.1.4.1-2.

Table 5.5.5.1.4.1-2: Initial conditions for EN-DC FR2 SSB-based beam failure detection and link recovery in non-DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.3-1 and TS 38.508-1 [14] clause 4.3.1 for E-UTRA and 7.2.3 for NR. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 5.5.5.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.3.1.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to Table 5.5.5.1.4.1-3.

2.	Message contents are defined in clause 5.5.5.1.4.3.

3.	There are one E-UTRAN cell and one NR cell specified in the test. E-UTRAN Cell 1 is the cell used for connection setup with the power level set according to clause C.1.1 and C.1.2 for this test.

Table 5.5.5.1.4.1-3: General test parameters for EN-DC FR2 SSB-based beam failure detection and link recovery in non-DRX

| Parameter |  | Test Config. | Unit | Value | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |  |
| Active E-UTRA PCell |  | 1-4 |  | Cell 1 |  |
| E-UTRA RF Channel Number |  | 1-4 |  | 1 |  |
| Active PCell |  | 1-4 |  | Cell 2 |  |
| RF Channel Number |  | 1-4 |  | 2 |  |
| Duplex mode |  | 1-4 |  | TDD |  |
| TDD Configuration |  | 1-4 |  | TDDConf.3.1 | Table A.1.5-3 |
| BWchannel |  | 1-4 | MHz | 100: NRB,c = 66 |  |
| Data RBs allocated |  | 1-4 |  | 66 |  |
| PDSCH/PDCCH subcarrier spacing |  | 1-4 | kHz | 120 |  |
| DL initial BWP configuration |  | 1-4 |  | DLBWP.0.1 | Table A.8.1-1 |
| DL dedicated BWP configuration |  | 1-4 |  | DLBWP.1.1 | Table A.8.1-2 |
| UL initial BWP configuration |  | 1-4 |  | ULBWP.0.1 | Table A.8.2-1 |
| UL dedicated BWP configuration |  | 1-4 |  | ULBWP.1.1 | Table A.8.2-2 |
| PDSCH Reference Channel |  | 1-2 |  | SR.3.2 TDD | Table A.1.1.2-3 |
|  |  | 3-4 |  | SR.3.3 TDD |  |
| RMSI CORESET Reference Channel |  | 1-2 |  | CR.3.1 TDD | Table A.1.2.2-3 |
|  |  | 3-4 |  | CR.3.2 TDD |  |
| Dedicated CORESET Reference Channel |  | 1-2 |  | CCR.3.1 TDD | Table A.1.3.2-3 |
|  |  | 3-4 |  | CCR.3.7 TDD |  |
| OCNG parameters |  | 1-4 |  | OP.1 | Table A.2.1-1 |
| CP length |  | 1-4 |  | Normal |  |
| PDSCH/PDCCH TCI state |  | 1-4 |  | TCI.State.0 | Table A.10.2-1 |
| CSI-RS for tracking |  | 1-4 |  | TRS.2.1 TDD | Table A.1.4A.2.1-1 |
| SSB Configuration |  | 1-2 |  | SSB.1 FR2 | Table A.3.2-1 |
|  |  | 3-4 |  | SSB.2 FR2 |  |
| SMTC Configuration |  | 1-4 |  | SMTC.3 | Table A.4-1 |
| PRACH Configuration |  | 1-4 |  | PRACH.2 FR2 | Table A.7.2-1 |
| DRX configuration |  | 1-4 |  | OFF |  |
| SSB index assigned as BFD RS (q0) |  | 1-4 |  | 0 |  |
| SSB index assigned as CBD RS (q1) |  | 1-4 |  | 1 |  |
| SSB index assigned as RLM RS |  | 1-4 |  | 0,1 |  |
| Beam failure detection transmission parameters | DCI format | 1-4 |  | 1-0 |  |
|  | Number of Control OFDM symbols | 1-4 |  | 2 |  |
|  | Aggregation level | 1-4 | CCE | 8 |  |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | 1-4 | dB | 0 |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | 1-4 | dB | 0 |  |
|  | DMRS precoder granularity | 1-4 |  | REG bundle size |  |
|  | REG bundle size | 1-4 |  | 6 |  |
| Gap pattern ID |  | 1-4 |  | gp0 |  |
| gapOffset |  | 1-4 | ms | 0 |  |
| rlmInSyncOutOfSyncThreshold |  | 1-4 |  | absent | Value 0 is applied. (38.133 [6] Table 8.1.1-1). |
| rsrp-ThresholdSSB |  | 1-2 | dBm/SCS | -109Note 3 | Threshold used for Qin_LR_SSB |
|  |  | 3-4 |  | -106Note 3 |  |
| powerControlOffsetSS |  | 1-4 |  | db0 | Used for deriving rsrp-ThresholdCSI-RS |
| beamFailureInstanceMaxCount |  | 1-4 |  | n1 | see TS 38.321 [12], clause 5.17 |
| beamFailureDetectionTimer |  | 1-4 |  | pbfd4 | see TS 38.321 [12], clause 5.17 |
| CSI-RS configuration for CSI reporting |  | 1-4 |  | CSI-RS.3.1 TDD | Table A.1.4.2-3 |
| reportConfigType |  | 1-4 |  | periodic |  |
| reportQuantity |  | 1-4 |  | cri-RI-PMI-CQI |  |
| CSI reporting periodicity |  | 1-4 | slot | 40 |  |
| CSI reporting offset |  | 1-4 | slot | 4 |  |
| T310 |  | 1-4 | ms | 1000 |  |
| N310 |  | 1-4 |  | 2 |  |
| T1 |  | 1-4 | s | 1 | The UE shall be fully synchronized to cell 1 during T1 |
| T2 |  | 1-4 | s | 2.61 |  |
| T3 |  | 1-4 | s | 1.64 |  |
| T4 |  | 1-4 | s | 0 |  |
| T5 |  | 1-4 | s | 1.01 |  |
| D1 |  | 1-4 | s | 0.97 |  |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. Note 3: Including test tolerance given in Annex F.1.3.2. |  |  |  |  |  |

5.5.5.1.4.2	Test procedure

Prior to the start of the time duration T1, the UE shall be fully synchronized to E-UTRA Cell 1 and NR Cell 2. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5  ms. In the test, DRX configuration is not enabled. The UE is configured to perform inter-frequency measurements using GP ID #0 (40ms).

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [14] clause 4.5.4.

2.	Set the parameters of NR Cell 1 according to T1 in Table 5.5.5.1.5-1. Propagation conditions are set according to clause C.2.3. T1 starts.

3.	When T1 expires the SS shall change the SNR value to T2 as specified in Table 5.5.5.1.5-1. T2 starts.

4.	When T2 expires the SS shall change the SNR value to T3 as specified in Table 5.5.5.1.5-1. T3 starts.

5.	When T3 expires the SS shall change the SNR value to T4 as specified in Table 5.5.5.1.5-1. T4 starts.

6.	When T4 expires the SS shall change the SNR value to T5 as specified in Table 5.5.5.1.5-1. T5 starts.

7.	If the SS:

a)	detects uplink power on NR carrier in each slot configured for CQI transmission (according CQI reporting on PUCCH) during the period from time point A to time point B; and

b)	does not detect preamble on a beam associated with candidate beam set q1 before time point B; and

c)	detects preamble on a beam associated with candidate beam set q1 before time point F (D1 after the start of T5),

the number of successful tests is increased by one.Otherwise the number of failed tests is increased by one.

8.	When T5 expires the SS shall change the SNR value to T1 as specified in Table 5.5.5.1.5-1.

9.	Wait 1s for the UE to re-establish the connection or continue directly to step 10. If the UE re-establishes the connection within 1s continue to step 11. Otherwise continue to step 10.

10.	Switch the UE on and off. Ensure the UE is in RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [14] clause 4.5.4.

11.	Repeat steps 2-10 for all subtests until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

5.5.5.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 5.5.5.1.4.3-1: Common Exception messages for EN-DC FR2 SSB-based beam failure detection and link recovery in non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with condition INTER-FREQ Table H.3.1-3 with condition INTER-FREQ MO (where ssbFrequency is set to the ARFCN value of carrier centre of High range) Table H.3.1-4 with A3-offset = 0 Table H.3.1-8 with condition SSB BFD Table H.3.1-10 with condition SSB Table H.3.1-10A Table H.3.4-4 with Condition gapUE Table H.3.4-5 with Condition BFD |

Table 5.5.5.1.4.3-2: PDCCH Search Space for BFR

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-162 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| searchSpaceId | 3 | BFR |  |
| controlResourceSetId | 2 | BFR |  |
| monitoringSlotPeriodicityAndOffset CHOICE { |  |  |  |
| sl1 | NULL |  |  |
| } |  |  |  |
| monitoringSymbolsWithinSlot | 10000000000000 | Symbols 0 and 1 |  |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel1 | n0 |  |  |
| aggregationLevel2 | n0 |  |  |
| aggregationLevel4 | n0 |  |  |
| aggregationLevel8 | n1 | AL8 |  |
| aggregationLevel16 | n0 |  |  |
| } |  |  |  |
| searchSpaceType CHOICE { |  |  |  |
| ue-Specific SEQUENCE { |  |  | USS |
| dci-Formats | formats0-0-And-1-0 | DCI Format 1_0 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.5.5.1.4.3-3: RLF-TimersAndConstants

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-150 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLF-TimersAndConstants ::= SEQUENCE { |  |  |  |
| n310 | n2 |  |  |
| } |  |  |  |

Table 5.5.5.1.4.3-4: PDCCH-Config

| Derivation Path: TS 38.508-1 [14],Table 4.6.3-95 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCCH-Config ::= SEQUENCE { |  |  |  |
| controlResourceSetToAddModList SEQUENCE(SIZE (1..3)) OF ControlResourceSet { | 1 entry |  |  |
| ControlResourceSet[1] | ControlResourceSet | entry 1, BFR |  |
| } |  |  |  |
| controlResourceSetToReleaseList | Not present |  |  |
| searchSpacesToAddModList SEQUENCE(SIZE (1..10)) OF SearchSpace { | 2 entries |  |  |
| SearchSpace[2] | SearchSpace | entry 2, BFR |  |
| } |  |  |  |
| searchSpacesToReleaseList | Not present |  |  |
| downlinkPreemption | Not present |  |  |
| tpc-PUSCH | Not present |  |  |
| tpc-PUCCH | Not present |  |  |
| tpc-SRS | Not present |  |  |
| } |  |  |  |

Table 5.5.5.1.4.3-5: ControlResourceSet for BFR

| Derivation Path: TS 38.508-1 [14],Table 7.3.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| controlResourceSetId | 2 |  |  |
| duration | 2 |  |  |
|  | 1 |  | Test Configuration 3 & 4 |
| cce-REG-MappingType CHOICE { |  |  |  |
| interleaved ::= SEQUENCE { |  |  |  |
| reg-BundleSize | n6 |  |  |
| interleaverSize | n2 |  |  |
| shiftIndex | 0 |  |  |
| } |  |  |  |
| tci-StatesPDCCH-ToAddList | Not present |  |  |
| pdcch-DMRS-ScramblingID | 1008 | DMRS scrambling ID different from Physical Cell ID |  |
| } |  |  |  |

Table 5.5.5.1.4.3-6: PDCCH Search Space for USS

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-162 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| searchSpaceId | 2 |  |  |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel1 | n0 |  |  |
| aggregationLevel2 | n1 |  |  |
| aggregationLevel4 | n2 |  | Test Configuration 1 & 2 |
|  | n1 |  | Test Configuration 3 & 4 |
| aggregationLevel8 | n0 |  |  |
| } |  |  |  |
| } |  |  |  |

5.5.5.1.5	Test requirement

Tables 5.5.5.1.4.1-3 and 5.5.5.1.5-1 define the primary level settings including test tolerances for EN-DC FR2 SSB-based beam failure detection and link recovery in non-DRX.

Table 5.5.5.1.5-1: NR Cell specific test parameters for EN-DC FR2 SSB-based beam failure detection and link recovery in non-DRX

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| AoA setup |  |  | Setup 1 defined in A.9 |  |  |  |  |
| Assumption for UE beamsNote 10 |  |  | Rough |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| SNR_SSB of set q0 | Config 1-4 | dB | 13.7 Note 11,12 | 5.7 Note 11,12 | -12 | -12 | -12 |
| SNR_SSB of set q1 | Config 1-4 | dB | 0.2 | 0.2 | 20 Note 12 | 20 Note 12 | 20 Note 12 |
| SSB_RP of set q1 | Config 1-2 | dBm/ SCS | -104.5 | -104.5 | -84.7 | -84.7 | -84.7 |
|  | Config 3-4 |  | -101.5 | -101.5 | -81.7 | -81.7 | -81.7 |
|  | Config 1-4 | dBm/120 KHz | -104.7 |  |  |  |  |
| Propagation condition |  |  | TDL-A 30ns 75Hz |  |  |  |  |
| NOTE 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. NOTE 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. NOTE 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. NOTE 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. NOTE 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. NOTE 7: SNR levels correspond to the signal to noise ratio over the SSS REs. NOTE 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure 5.5.5.1.4-1. NOTE 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is modified as specified in clause D.4. NOTE 10: Information about types of UE beam is given in TS 38.133 clause B.2.1.3, and does not limit UE implementation or test system implementation Note 11: This value allows up to 1dB degradation from applied SNR to UE baseband Note 12: Including test tolerance given in Annex F.1.3.2 |  |  |  |  |  |  |  |

Table 5.5.5.1.5-2: Void

The UE behaviour during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the time duration T1 and T2, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on Cell 1.

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 1 in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 1.

During T3 the shall detect beam failure and initiate link recovery. During T4 and T5 the UE measures and evaluate beam candidate from beam candidate set q1.

No later than time point F occurring no later than D1 =  960+10 ms after the start of T5, the UE shall transmit preamble on a beam associated with the candidate beam set q1. The UE shall not transmit preamble on a beam associated with the candidate beam set q1 earlier than time point B.

Test is concluded once the test equipment has received the initial preamble transmission from the UE. The rate of correct events observed during repeated tests shall be at least 90%.

#### 5.5.5.2	EN-DC FR2 SSB-based beam failure detection and link recovery in DRX

Editor's Note: This test case is complete for the following configurations:

- Test frequency f ≤ 40.8 GHz

- UE PC3

This test case is incomplete for Test frequency f > 40.8 GHz

This test case is incomplete for UE power class other than PC3.

5.5.5.2.1	Test purpose

The purpose of this test is to verify that the UE properly detects SSB-based beam failure in the set q0 configured for a serving PSCell and that the UE performs correct SSB-based link recovery based on beam candidate set q1. The purpose is to test the downlink monitoring for beam failure detection within the UEs active DL BWP of the PSCell, during the evaluation period, and link recovery, when DRX is used. This test will partly verify the SSB based beam failure detection and link recovery for an FR2 serving cell requirements in TS 38.133 [6] clause 8.5.

5.5.5.2.2	Test applicability

This test applies to all types of NR UE supporting E-UTRA and EN-DC from release 15 onwards and supporting long DRX cycle.

5.5.5.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 5.5.5.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.5.5.5.2.

5.5.5.2.4	Test description

There is one E-UTRAN PCell and one NR PSCell configured in this test. This test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure 5.5.5.2.4-1 shows the variation of the downlink SNR of the PCell and the SNR of the SSB in set q0 in the active PSCell to emulate SSB based beam failure. Figure 5.5.5.2.4-2 shows the variation of the downlink L1-RSRP of the SSB in set q1 of the candidate beam used for link recovery

Figure 5.5.5.2.4-1: SNR variation for EN-DC FR2 SSB-based beam failure detection and link recovery in DRX

Figure 5.5.5.2.4-2: SSB_RP level variation for EN-DC FR2 SSB-based beam failure detection and link recovery in DRX

5.5.5.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 5.5.5.2.4.1-1.

Table 5.5.5.2.4.1-1: Supported test configurations for EN-DC FR2 SSB-based beam failure detection and link recovery in DRX

| Configuration | Description |
| --- | --- |
| 5.5.5.2-1 | LTE FDD, NR TDD duplex mode, 120 kHz SSB SCS, 100 MHz bandwidth |
| 5.5.5.2-2 | LTE TDD, NR TDD duplex mode, 120 kHz SSB SCS, 100 MHz bandwidth |
| 5.5.5.2-3 | LTE FDD, NR TDD duplex mode, 240 kHz SSB SCS, 100 MHz bandwidth |
| 5.5.5.2-4 | LTE TDD, NR TDD duplex mode, 240 kHz SSB SCS, 100 MHz bandwidth |
| NOTE: The UE is only required to pass in one of the supported test configurations in FR2 |  |

Configure the test equipment and the DUT according to the parameters in Table 5.5.5.2.4.1-2.

Table 5.5.5.2.4.1-2: Initial conditions for EN-DC FR2 SSB-based beam failure detection and link recovery in DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.3-1 and TS 38.508-1 [14] clause 4.3.1 for E-UTRA and 7.2.3 for NR. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.5.5.3.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.3.1.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to Table 5.5.5.2.4.1-3. The DRX configuration for is according to Table 5.5.5.2.4.1-3. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

2.	Message contents are defined in clause 5.5.5.2.4.3.

3.	There is one E-UTRAN cell and one NR cell specified in the test. E-UTRAN Cell 1 is the cell used for connection setup with the power level set according to clause C.1.1 and C.1.2 for this test.

Table 5.5.5.2.4.1-3: General test parameters for EN-DC FR2 SSB-based beam failure detection and link recovery in DRX

| Parameter |  | Test Config. | Unit | Value | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |  |
| Active E-UTRA PCell |  | 1-4 |  | Cell 1 |  |
| E-UTRA RF Channel Number |  | 1-4 |  | 1 |  |
| Active PCell |  | 1-4 |  | Cell 2 |  |
| RF Channel Number |  | 1-4 |  | 2 |  |
| Duplex mode |  | 1-4 |  | TDD | Table A.1.5-3 |
| TDD Configuration |  | 1-4 |  | TDDConf.3.1 |  |
| BWchannel |  | 1-4 | MHz | 100: NRB,c = 66 |  |
| Data RBs allocated |  | 1-4 |  | 66 |  |
| PDSCH/PDCCH subcarrier spacing |  | 1-4 | kHz | 120 |  |
| DL initial BWP configuration |  | 1-4 |  | DLBWP.0.1 | Table A.8.1-1 |
| DL dedicated BWP configuration |  | 1-4 |  | DLBWP.1.1 | Table A.8.1-2 |
| UL initial BWP configuration |  | 1-4 |  | ULBWP.0.1 | Table A.8.2-1 |
| UL dedicated BWP configuration |  | 1-4 |  | ULBWP.1.1 | Table A.8.2-2 |
| PDSCH Reference Channel |  | 1-2 |  | SR.3.2 TDD | Table A.1.1.2-3 |
|  |  | 3-4 |  | SR.3.3 TDD |  |
| RMSI CORESET Reference Channel |  | 1-2 |  | CR.3.1 TDD | Table A.1.2.2-3 |
|  |  | 3-4 |  | CR.3.2 TDD |  |
| Dedicated CORESET Reference Channel |  | 1-2 |  | CCR.3.1 TDD | Table A.1.3.2-3 |
|  |  | 3-4 |  | CCR.3.7 TDD |  |
| OCNG parameters |  | 1-4 |  | OP.1 | Table A.2.1-1 |
| CP length |  | 1-4 |  | Normal |  |
| PDSCH/PDCCH TCI state |  | 1-4 |  | TCI.State.0 | Table A.10.2-1 |
| CSI-RS for tracking |  | 1-4 |  | TRS.2.1 TDD | Table A.1.4A.2.1-1 |
| SSB Configuration |  | 1-2 |  | SSB.1 FR2 | Table A.3.2-1 |
|  |  | 3-4 |  | SSB.2 FR2 |  |
| SMTC Configuration |  | 1-4 |  | SMTC.3 | Table A.4-1 |
| PRACH Configuration |  | 1-4 |  | PRACH.2 FR2 | Table A.7.2-1 |
| DRX configuration |  | 1-4 |  | DRX.3 | Table A.5-1 |
| SSB index assigned as BFD RS (q0) |  | 1-4 |  | 0 |  |
| SSB index assigned as CBD RS (q1) |  | 1-4 |  | 1 |  |
| SSB index assigned as RLM RS |  | 1-4 |  | 0,1 |  |
| Beam failure detection transmission parameters | DCI format | 1-4 |  | 1-0 |  |
|  | Number of Control OFDM symbols | 1-4 |  | 2 |  |
|  | Aggregation level | 1-4 | CCE | 8 |  |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | 1-4 | dB | 0 |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | 1-4 | dB | 0 |  |
|  | DMRS precoder granularity | 1-4 |  | REG bundle size |  |
|  | REG bundle size | 1-4 |  | 6 |  |
| Gap pattern ID |  | 1-4 |  | N/A |  |
| rlmInSyncOutOfSyncThreshold |  | 1-4 |  | absent | Value 0 is applied. (TS 38.133 [6] Table 8.1.1-1). |
| rsrp-ThresholdSSB |  | 1-2 | dBm/SCS | -109Note 2 | Threshold used for Qin_LR_SSB |
|  |  | 3-4 |  | -106Note 2 |  |
| powerControlOffsetSS |  | 1-4 |  | db0 | Used for deriving rsrp-ThresholdCSI-RS |
| beamFailureInstanceMaxCount |  | 1-4 |  | n1 | see TS 38.321 [12], clause 5.17 |
| beamFailureDetectionTimer |  | 1-4 |  | pbfd4 | see TS 38.321 [12], clause 5.17 |
| CSI-RS configuration for CSI reporting |  | 1-4 |  | CSI-RS.3.1 TDD | Table A.1.4.2-3 |
| reportConfigType |  | 1-4 |  | periodic |  |
| reportQuantity |  | 1-4 |  | cri-RI-PMI-CQI |  |
| CSI reporting periodicity |  | 1-4 | slot | 40 |  |
| CSI reporting offset |  | 1-4 | slot | 4 |  |
| T310 |  | 1-4 | ms | 1000 |  |
| N310 |  | 1-4 |  | 2 |  |
| T1 |  | 1-4 | s | 1 | The UE shall be fully synchronized to cell 1 during T1 |
| T2 |  | 1-4 | s | 3.37 |  |
| T3 |  | 1-4 | s | 2.8 |  |
| T4 |  | 1-4 | s | 0 |  |
| T5 |  | 1-4 | s | 0.61 |  |
| D1 |  | 1-4 | s | 0.57 |  |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. Note 2: Including test tolerance given in Annex F.1.3.2 |  |  |  |  |  |

5.5.5.2.4.2	Test procedure

Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1 and cell 2. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5 ms. In the test, DRX configuration is enabled in PSCell and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CQI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [14] clause 4.5.4

2.	Set the parameters of NR Cell 1 according to T1 in Table 5.5.5.2.5-1. Propagation conditions are set according to clause C.2.3. T1 starts.

3.	When T1 expires the SS shall change the SNR value to T2 as specified in Table 5.5.5.2.5-1. T2 starts.

4.	When T2 expires the SS shall change the SNR value to T3 as specified in Table 5.5.5.2.5-1. T3 starts.

5.	When T3 expires the SS shall change the SNR value to T4 as specified in Table 5.5.5.2.5-1. T4 starts.

6.	When T4 expires the SS shall change the SNR value to T5 as specified in Table 5.5.5.2.5-1. T5 starts.

7.	If the SS:

a)	detects uplink power on NR carrier in each slot configured for CQI transmission (according CQI reporting on PUCCH) during the period from time point A to time point B; and

b)	does not detect preamble on a beam associated with the candidate beam set q1 before time point B; and

c)	detects preamble on a beam associated with the candidate beam set q1 before time point F (D1 after the start of T5),

the number of successful tests is increased by one. Otherwise the number of failed tests is increased by one.

8.	When T5 expires the SS shall change the SNR value to T1 as specified in Table 5.5.5.2.5-1.

9.	Wait 1s for the UE to re-establish the connection or continue directly to step 10. If the UE re-establishes the connection within 1s continue to step 11. Otherwise continue to step 10.

10.	Switch the UE on and off. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [14] clause 4.5.4.

11.	Repeat steps 2-10 for all subtests until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

5.5.5.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 5.5.5.2.4.3-1: Common Exception messages for EN-DC FR2 SSB-based beam failure detection and link recovery in DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-8 with condition SSB BFD Table H.3.1-10 with condition SSB Table H.3.1-10A Table H.3.7-1 with condition DRX.3 |

Table 5.5.5.2.4.3-2: PDCCH Search Space for BFR

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-162 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| searchSpaceId | 3 | BFR |  |
| controlResourceSetId | 2 | BFR |  |
| monitoringSlotPeriodicityAndOffset CHOICE { |  |  |  |
| sl1 | NULL |  |  |
| } |  |  |  |
| monitoringSymbolsWithinSlot | 10000000000000 | Symbols 0 and 1 |  |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel1 | n0 |  |  |
| aggregationLevel2 | n0 |  |  |
| aggregationLevel4 | n0 |  |  |
| aggregationLevel8 | n1 | AL8 |  |
| aggregationLevel16 | n0 |  |  |
| } |  |  |  |
| searchSpaceType CHOICE { |  |  |  |
| ue-Specific SEQUENCE { |  |  | USS |
| dci-Formats | formats0-0-And-1-0 | DCI Format 1_0 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.5.5.2.4.3-3: RLF-TimersAndConstants

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-150 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLF-TimersAndConstants ::= SEQUENCE { |  |  |  |
| n310 | n2 |  |  |
| } |  |  |  |

Table 5.5.5.2.4.3-4: PDCCH-Config

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-95 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCCH-Config ::= SEQUENCE { |  |  |  |
| controlResourceSetToAddModList SEQUENCE(SIZE (1..3)) OF ControlResourceSet { | 1 entry |  |  |
| ControlResourceSet[1] | ControlResourceSet | entry 1, BFR |  |
| } |  |  |  |
| controlResourceSetToReleaseList | Not present |  |  |
| searchSpacesToAddModList SEQUENCE(SIZE (1..10)) OF SearchSpace { | 2 entries |  |  |
| SearchSpace[2] | SearchSpace | entry 2, BFR |  |
| } |  |  |  |
| searchSpacesToReleaseList | Not present |  |  |
| downlinkPreemption | Not present |  |  |
| tpc-PUSCH | Not present |  |  |
| tpc-PUCCH | Not present |  |  |
| tpc-SRS | Not present |  |  |
| } |  |  |  |

Table 5.5.5.2.4.3-5: ControlResourceSet for BFR

| Derivation Path: TS 38.508-1 [14], Table 7.3.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| controlResourceSetId | 2 |  |  |
| duration | 2 |  |  |
|  | 1 |  | Test Configuration 3 & 4 |
| cce-REG-MappingType CHOICE { |  |  |  |
| interleaved ::= SEQUENCE { |  |  |  |
| reg-BundleSize | n6 |  |  |
| interleaverSize | n2 |  |  |
| shiftIndex | 0 |  |  |
| } |  |  |  |
| tci-StatesPDCCH-ToAddList | Not present |  |  |
| pdcch-DMRS-ScramblingID | 1008 | DMRS scrambling ID different from Physical Cell ID |  |
| } |  |  |  |

Table 5.5.5.2.4.3-6: PDCCH Search Space for USS

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-162 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| searchSpaceId | 2 |  |  |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel1 | n0 |  |  |
| aggregationLevel2 | n1 |  |  |
| aggregationLevel4 | n2 |  | Test Configuration 1 & 2 |
|  | n1 |  | Test Configuration 3 & 4 |
| aggregationLevel8 | n0 |  |  |
| } |  |  |  |
| } |  |  |  |

5.5.5.2.5	Test requirement

Tables 5.5.5.2.4.1-3 and 5.5.5.2.5-1 define the primary level settings including test tolerances for EN-DC FR2 SSB-based beam failure detection and link recovery in DRX.

Table 5.5.5.2.5-1: NR Cell specific test parameters for EN-DC FR2 SSB-based beam failure detection and link recovery in DRX

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| AoA setup |  |  | Setup 1 defined in A.9 |  |  |  |  |
| Assumption for UE beamsNote 10 |  |  | Rough |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| SNR_SSB of set q0 | Config 1-4 | dB | 13.7 Note 11,12 | 5.7 Note 11,12 | -12 | -12 | -12 |
| SNR_SSB of set q1 | Config 1-4 | dB | 0.2 | 0.2 | 20 Note 12 | 20 Note 12 | 20 Note 12 |
| SSB_RP of set q1 | Config 1-2 | dBm/ | -104.5 | -104.5 | -84.7 | -84.7 | -84.7 |
|  | Config 3-4 | SCS kHz | -101.5 | -101.5 | -81.7 | -81.7 | -81.7 |
|  | Config 1-4 | dBm/120 KHz | -104.7 |  |  |  |  |
| Propagation condition |  |  | TDL-A 30ns 75Hz |  |  |  |  |
| NOTE 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. NOTE 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. NOTE 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. NOTE 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. NOTE 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. NOTE 7: SNR levels correspond to the signal to noise ratio over the SSS REs. NOTE 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure 5.5.5.2.4-1. NOTE 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is modified as specified in clause D.4. NOTE 10: Information about types of UE beam is given in TS 38.133 clause B.2.1.3, and does not limit UE implementation or test system implementation Note 11: This value allows up to 1dB degradation from applied SNR to UE baseband Note 12: Including test tolerance given in Annex F.1.3.2 |  |  |  |  |  |  |  |

The UE behaviour during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the time duration T1 and T2, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on Cell 1.

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 1 in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 1.

During T3 the UE shall detect beam failure and initiate link recovery. During T4 and T5 the UE measures and evaluate beam candidate from beam candidate set q1.

No later than time point F occurring no later than D1 = 560+10 ms after the start of T5, the UE shall transmit preamble on a beam associated with the candidate beam set q1. The UE shall not transmit preamble on a beam associated with the candidate beam set q1 earlier than time point B.

Test is concluded once the test equipment has received the initial preamble transmission from the UE. The rate of correct events observed during repeated tests shall be at least 90%.

#### 6.5.5.1	NR SA FR1 SSB-based beam failure detection and link recovery in non-DRX

6.5.5.1.1	Test purpose

The purpose of this test is to verify that the UE properly detects SSB-based beam failure in the set q0 configured for a serving cell and that the UE performs correct SSB-based link recovery based on beam candidate set q1. The purpose is to test the downlink monitoring for beam failure detection within the UEs active DL BWP, during the evaluation period, and link recovery, when no DRX is used. This test will partly verify the SSB based beam failure detection and link recovery for an FR1 serving cell requirements in TS 38.133 [6] clause 8.5.

6.5.5.1.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting 5GS NR SA FR1 and link recovery.

6.5.5.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.5.5.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.5.5.1.

6.5.5.1.4	Test description

The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure 6.5.5.1.4-1 shows the five different time durations and the corresponding variation of the downlink SNR in the active cell to emulate SSB based beam failure. Figure 6.5.5.1.4-1 additionally shows the variation of the downlink L1-RSRP of the SSB in set q1 of the candidate beam used for link recovery

Figure 6.5.5.1.4-1: SNR and L1-RSRP variation for NR SA FR1 SSB-based beam failure detection and link recovery in non-DRX mode

6.5.5.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.5.5.1.4.1-1.

Table 6.5.5.1.4.1-1: Supported test configurations for NR SA FR1 SSB-based beam failure detection and link recovery in non-DRX mode

| Configuration | Description |
| --- | --- |
| 6.5.5.1-1 | FDD duplex mode, 15 kHz SSB SCS, 10MHz bandwidth |
| 6.5.5.1-2 | TDD duplex mode, 15 kHz SSB SCS, 10MHz bandwidth |
| 6.5.5.1-3 | TDD duplex mode, 30 kHz SSB SCS, 40MHz bandwidth |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Configure the test equipment and the DUT according to the parameters in Table 6.5.5.1.4.1-2.

Table 6.5.5.1.4.1-2: Initial conditions for NR SA FR1 SSB-based beam failure detection and link recovery in non-DRX mode

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.5.5.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part |  |  |

1. The general test parameter settings are set up according to Table 6.5.5.1.4.1-3.

2. Message contents are defined in clause 6.5.5.1.4.3.

3. There is one NR carrier and one NR cells specified in the test. Cell 1 is the NR cell used for connection setup with the power level set according to Annex C.1.2 and C.1.3 for this test.

Table 6.5.5.1.4.1-3: General test parameters for NR SA FR1 SSB-based beam failure detection and link recovery in non-DRX mode

| Parameter |  |  |  |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Test 1 |  |
| Active PSCell |  |  |  |  |  | Cell 1 |  |
| RF Channel Number |  |  |  |  |  | 1 |  |
| Duplex mode |  |  |  | Config 1 |  | FDD |  |
|  |  |  |  | Config 2, 3 |  | TDD |  |
| BWchannel |  |  |  | Config 1 | MHz | 10: NRB,c = 52 |  |
|  |  |  |  | Config 2 |  | 10: NRB,c = 52 |  |
|  |  |  |  | Config 3 |  | 40: NRB,c = 106 |  |
| DL initial BWP configuration |  |  |  | Config 1, 2, 3 |  | DLBWP.0.1 |  |
| DL dedicated BWP configuration |  |  |  | Config 1, 2, 3 |  | DLBWP.1.1 |  |
| UL initial BWP configuration |  |  |  | Config 1, 2, 3 |  | ULBWP.0.1 |  |
| UL dedicated BWP configuration |  |  |  | Config 1, 2, 3 |  | ULBWP.1.1 |  |
| TDD Configuration |  |  |  | Config 1 |  | Not Applicable |  |
|  |  |  |  | Config 2 |  | TDDConf.1.1 |  |
|  |  |  |  | Config 3 |  | TDDConf.2.1 |  |
| CORESET Reference Channel |  |  |  | Config 1 |  | CR.1.1 FDD |  |
|  |  |  |  | Config 2 |  | CR.1.1 TDD |  |
|  |  |  |  | Config 3 |  | CR.2.1 TDD |  |
| Dedicated CORESET Reference Channel |  |  |  | Config 1 |  | CCR.1.1. FDD |  |
|  |  |  |  | Config 2 |  | CCR.1.1. TDD |  |
|  |  |  |  | Config 3 |  | CCR.2.1. TDD |  |
| SSB Configuration |  |  |  | Config 1 |  | SSB.3 FR1 |  |
|  |  |  |  | Config 2 |  | SSB.3 FR1 |  |
|  |  |  |  | Config 3 |  | SSB.4 FR1 |  |
| SMTC Configuration |  |  |  | Config 1, 2 |  | SMTC.1 |  |
|  |  |  |  | Config 3 |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing |  |  |  | Config 1, 2 |  | 15 KHz |  |
|  |  |  |  | Config 3 |  | 30 KHz |  |
| PRACH Configuration |  |  |  | Config 1, 2 |  | PRACH.2 FR1 |  |
|  |  |  |  | Config 3 |  | PRACH.2 FR1 |  |
| SSB Index assigned as BFD RS (q0) |  |  |  |  |  | 0 |  |
| SSB Index assigned as CBD RS (q1) |  |  |  |  |  | 1 |  |
| OCNG parameters |  |  |  |  |  | OP.1 |  |
| CP length |  |  |  |  |  | Normal |  |
| Correlation Matrix and Antenna Configuration |  |  |  |  |  | 2x2 Low |  |
| Beam failure detection transmission parameters |  | DCI format |  |  |  | 1-0 |  |
|  |  | Number of Control OFDM symbols |  |  |  | 2 |  |
|  |  | Aggregation level |  |  | CCE | 8 |  |
|  |  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy |  |  | dB | 0 |  |
|  |  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy |  |  | dB | 0 |  |
|  |  | DMRS precoder granularity |  |  |  | REG bundle size |  |
|  |  | REG bundle size |  |  |  | 6 |  |
| DRX |  |  |  |  |  | OFF |  |
| Gap pattern ID |  |  |  |  |  | gp0 |  |
| gapOffset |  |  |  |  |  | 0 |  |
| rlmInSyncOutOfSyncThreshold |  |  |  |  |  | absent | When the field is absent, the UE applies the value 0. |
| rsrp-ThresholdSSB | Config 1, 2 |  |  |  | dBm/SCS kHz | -98 | Threshold used for Qin_LR_SSB |
|  | Config 3 |  |  |  | dBm/SCS kHz | -95 | Threshold used for Qin_LR_SSB |
| powerControlOffsetSS |  |  |  |  |  | db0 | Used for deriving rsrp-ThresholdCSI-RS |
| beamFailureInstanceMaxCount |  |  |  |  |  | n1 | see clause 5.17 of TS 38.321 [12] |
| beamFailureDetectionTimer |  |  |  |  |  | pbfd4 | see clause 5.17 of TS 38.321 [12] |
| CSI-RS configuration for CSI reporting |  |  | Config 1 |  |  | CSI-RS.1.1 FDD |  |
|  |  |  | Config 2 |  |  | CSI-RS.1.1 TDD |  |
|  |  |  | Config 3 |  |  | CSI-RS.2.1 TDD |  |
| CSI-RS for tracking |  |  | Config 1 |  |  | TRS.1.1 FDD |  |
|  |  |  | Config 2 |  |  | TRS.1.1 TDD |  |
|  |  |  | Config 3 |  |  | TRS.1.2 TDD |  |
| SSB Index assigned as RLM RS |  |  |  |  |  | 0, 1 |  |
| T310 Timer |  |  |  |  | ms | 1000 |  |
| N310 |  |  |  |  |  | 2 |  |
| T1 |  |  |  |  | s | 0.2 | During this time the UE shall be fully synchronized to cell 1 |
| T2 |  |  |  |  | s | 0.37 |  |
| T3 |  |  |  |  | s | 0.24 |  |
| T4 |  |  |  |  | s | 0 |  |
| T5 |  |  |  |  | s | 0.17 |  |
| D1 |  |  |  |  | s | 0.13 |  |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |  |  |  |

6.5.5.1.4.2	Test procedure

Prior to the start of the time duration T1, the UE shall be fully synchronized to NR Cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5 ms. In the test, DRX configuration is not enabled. The UE is configured to perform inter-frequency measurements using GP ID #0 (40ms) in test 1

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. The SS sends an RRCReconfiguration message to the UE to configure inter-frequency measurement.

3. The UE sends an RRCReconfigurationComplete message.

4.	Set the parameters of NR Cell 1 according to T1 in Table 6.5.5.1.5-1. Propagation conditions are set according to Annex C.2.3. T1 starts.

5.	When T1 expires the SS shall change the SNR value to T2 as specified in Table 6.5.5.1.5-1. T2 starts.

6.	When T2 expires the SS shall change the SNR value to T3 as specified in Table 6.5.5.1.5-1. T3 starts.

7.	When T3 expires the SS shall change the SNR value to T4 as specified in Table 6.5.5.1.5-1. T4 starts.

8.	When T4 expires the SS shall change the SNR value to T5 as specified in Table 6.5.5.1.5-1. T5 starts.

9.	If the SS:

a) detects uplink power on NR carrier equal to or higher than minimum output power defined in TS 38.521-1 [17] clause 6.3.1.5 in each slot configured for CSI transmission (according CSI reporting on PUCCH) during the period from time point A to time point B

and

b) does not detect preamble on a beam associated with the candidate beam set q1before time point B

and

c) detects preamble on a beam associated with the candidate beam set q1 before time point F (D1 after the start of T5),

the number of successful tests is increased by one.

Otherwise the number of failed tests is increased by one.

10.	Switch the UE off and on. Ensure the UE is in RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

11.	Repeat steps 4-10 for all subtests until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.5.5.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 and clause 7.3 with the following exceptions:

Table 6.5.5.1.4.3-1: Common Exception messages for NR SA FR1 SSB-based beam failure detection and link recovery in non-DRX mode

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-6 with Condition BFD Table H.3.1-1 Table H.3.1-2 with Condition INTER-FREQ, L3 FILTERING NEEDED, GAP_NEEDED Table H.3.1-3 with Condition INTER-FREQ MO (where ssbFrequency is set to the ARFCN value of carrier center of High range) Table H.3.1-4 with A3-offset = 0 Table H.3.1-8 with Condition SSB BFD Table H.3.1-10 with Condition SSB Table H.3.1-10A Table H.3.5-4 Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 6.5.5.1.4.3-2: PDCCH Search Space for BFR

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-162 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| searchSpaceId | 4 | BFR |  |
| controlResourceSetId | 2 | BFR |  |
| monitoringSlotPeriodicityAndOffset CHOICE { |  |  |  |
| sl1 | NULL |  |  |
| } |  |  |  |
| monitoringSymbolsWithinSlot | 10000000000000 | Symbol 0 |  |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel1 | n0 |  |  |
| aggregationLevel2 | n0 |  |  |
| aggregationLevel4 | n0 |  |  |
| aggregationLevel8 | n1 | AL8 |  |
| aggregationLevel16 | n0 |  |  |
| } |  |  |  |
| searchSpaceType CHOICE { |  |  |  |
| ue-Specific SEQUENCE { |  |  | USS |
| dci-Formats | formats0-0-And-1-0 | DCI Format 1_0 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.5.5.1.4.3-3: RLF-TimersAndConstants

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-150 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLF-TimersAndConstants ::= SEQUENCE { |  |  |  |
| n310 | n2 |  |  |
| } |  |  |  |

Table 6.5.5.1.4.3-4: Void

Table 6.5.5.1.4.3-5: PDCCH-Config

| Derivation Path: TS 38.501-1 [14],Table 4.6.3-95 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCCH-Config ::= SEQUENCE { |  |  |  |
| controlResourceSetToAddModList SEQUENCE(SIZE (1..3)) OF ControlResourceSet { | 2 entries |  |  |
| ControlResourceSet[2] | ControlResourceSet | entry 2, BFR |  |
| } |  |  |  |
| controlResourceSetToReleaseList | Not present |  |  |
| searchSpacesToAddModList SEQUENCE(SIZE (1..10)) OF SearchSpace { | 2 entries |  |  |
| SearchSpace[2] | SearchSpace | entry 2, BFR |  |
| } |  |  |  |
| searchSpacesToReleaseList | Not present |  |  |
| downlinkPreemption | Not present |  |  |
| tpc-PUSCH | Not present |  |  |
| tpc-PUCCH | Not present |  |  |
| tpc-SRS | Not present |  |  |
| } |  |  |  |

Table 6.5.5.1.4.3-6: ControlResourceSet for BFR

| Derivation Path: TS 38.501-1 [14],Table 7.3.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| controlResourceSetId | 2 |  |  |
| duration | 2 |  |  |
| cce-REG-MappingType CHOICE { |  |  |  |
| interleaved ::= SEQUENCE { |  |  |  |
| reg-BundleSize | n6 |  |  |
| interleaverSize | n2 |  |  |
| shiftIndex | 0 |  |  |
| } |  |  |  |
| tci-StatesPDCCH-ToAddList | Not present |  |  |
| } |  |  |  |

Table 6.5.5.1.4.3-7: PDCCH Search Space for USS

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-162 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| searchSpaceId | 2 |  |  |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel1 | n0 |  |  |
| aggregationLevel2 | n1 |  |  |
| aggregationLevel4 | n1 |  |  |
| aggregationLevel8 | n0 |  |  |
| } |  |  |  |
| } |  |  |  |

6.5.5.1.5	Test requirement

Tables 6.5.5.1.4.1-3 and 6.5.5.1.5-1 define the primary level settings including test tolerances for NR SA FR1 SSB-based beam failure detection and link recovery in DRX.

Table 6.5.5.1.5-1: NR Cell specific test parameters for NR SA FR1 SSB-based beam failure detection and link recovery in non-DRX mode

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| SNR_ SSB of set q0 | Config 1 | dB | 5.8 | -2.2 | -12.8 | -12.8 | -12.8 |
|  | Config 2 |  | 5.8 | -2.2 | -12.8 | -12.8 | -12.8 |
|  | Config 3 |  | 5.8 | -2.2 | -12.8 | -12.8 | -12.8 |
| SNR_SSB of set q1 | Config 1 | dB | -10.2 | -10.2 | 10.2 | 10.2 | 10.2 |
|  | Config 2 |  | -10.2 | -10.2 | 10.2 | 10.2 | 10.2 |
|  | Config 3 |  | -10.2 | -10.2 | 10.2 | 10.2 | 10.2 |
| SSB_RP of set q1 | Config 1 | dBm/SCS kHz | -108.2 | -108.2 | -87.8 | -87.8 | -87.8 |
|  | Config 2 |  | -108.2 | -108.2 | -87.8 | -87.8 | -87.8 |
|  | Config 3 |  | -105.2 | -105.2 | -84.8 | -84.8 | -84.8 |
|  | Config 1 | dBm/15 kHz | -98 |  |  |  |  |
|  | Config 2 |  | -98 |  |  |  |  |
|  | Config 3 |  | -98 |  |  |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1 Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure 6.5.5.1.4-1. Note 9: The SNR values are specified for a UE with 2RX antennas connected under test. For a UE with 4RX antennas connected under test, the SNR for RS in set q0 during T3, T4, and T5 from D.4.1.1, is -15dB-TT = -15.8dB (including test tolerances). |  |  |  |  |  |  |  |

Table 6.5.5.1.5-2: Void

The UE behaviour during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the time duration T1 and T2, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on Cell 1.

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 1 in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 1.

During T3 the UE shall detect beam failure and initiate link recovery. During T4 and T5 the UE measures and evaluate beam candidate from beam candidate set q1.

No later than time point F occurring no later than D1 = 130 ms after the start of T5, the UE shall transmit preamble on a beam associated with the candidate beam set q1. The UE shall not transmit preamble on a beam associated with the candidate beam set q1 earlier than time point B.

Test is concluded once the test equipment has received the initial preamble transmission from the UE. The rate of correct events observed during repeated tests shall be at least 90%.

#### 6.5.5.2	NR SA FR1 SSB-based beam failure detection and link recovery in DRX

6.5.5.2.1	Test purpose

The purpose of this test is to verify that the UE properly detects SSB-based beam failure in the set q0 configured for a serving cell and that the UE performs correct SSB-based link recovery based on beam candidate set q1. The purpose is to test the downlink monitoring for beam failure detection within the UEs active DL BWP, during the evaluation period, and link recovery, when DRX is used. This test will partly verify the SSB based beam failure detection and link recovery for an FR1 serving cell requirements in TS 38.133 [6] clause 8.5.

6.5.5.2.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting 5GS NR SA FR1, link recovery and long DRX cycle.

6.5.5.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.5.5.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.5.5.2.

6.5.5.2.4	Test description

The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure 6.5.5.2.4-1 shows the five different time durations and the corresponding variation of the downlink SNR in the active cell to emulate SSB based beam failure. Figure 6.5.5.2.4-1 additionally shows the variation of the downlink L1-RSRP of the SSB in set q1 of the candidate beam used for link recovery.

Figure 6.5.5.2.4-1: SNR and L1-RSRP variation for NR SA FR1 SSB-based beam failure detection and link recovery in DRX

6.5.5.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.5.5.2.4.1-1.

Table 6.5.5.2.4.1-1: Supported test configurations for NR SA FR1 SSB-based beam failure detection and link recovery in DRX

| Configuration | Description |
| --- | --- |
| 6.5.5.2-1 | FDD duplex mode, 15 kHz SSB SCS, 10MHz bandwidth |
| 6.5.5.2-2 | TDD duplex mode, 15 kHz SSB SCS, 10MHz bandwidth |
| 6.5.5.2-3 | TDD duplex mode, 30 kHz SSB SCS, 40MHz bandwidth |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Configure the test equipment and the DUT according to the parameters in Table 6.5.5.2.4.1-2.

Table 6.5.5.2.4.1-2: Initial conditions for NR SA FR1 SSB-based beam failure detection and link recovery in DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.5.5.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part |  |  |

1. The general test parameter settings are set up according to Table 6.5.5.2.4.1-3.

2. Message contents are defined in clause 6.5.5.2.4.3.

3. There is one NR carrier and one NR cells specified in the test. Cell 1 is the NR cell used for connection setup with the power level set according to Annex C.1.2 and C.1.3 for this test.

Table 6.5.5.2.4.1-3: General test parameters for NR SA FR1 SSB-based beam failure detection and link recovery in DRX

| Parameter |  |  |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Test 1 |  |
| Active PSCell |  |  |  |  | Cell 1 |  |
| RF Channel Number |  |  |  |  | 1 |  |
| Duplex mode |  |  | Config 1 |  | FDD |  |
|  |  |  | Config 2, 3 |  | TDD |  |
| BWchannel |  |  | Config 1 | MHz | 10: NRB,c = 52 |  |
|  |  |  | Config 2 |  | 10: NRB,c = 52 |  |
|  |  |  | Config 3 |  | 40: NRB,c = 106 |  |
| DL initial BWP configuration |  |  | Config 1, 2, 3 |  | DLBWP.0.1 |  |
| DL dedicated BWP configuration |  |  | Config 1, 2, 3 |  | DLBWP.1.1 |  |
| UL initial BWP configuration |  |  | Config 1, 2, 3 |  | ULBWP.0.1 |  |
| UL dedicated BWP configuration |  |  | Config 1, 2, 3 |  | ULBWP.1.1 |  |
| TDD Configuration |  |  | Config 1 |  | Not Applicable |  |
|  |  |  | Config 2 |  | TDDConf.1.1 |  |
|  |  |  | Config 3 |  | TDDConf.2.1 |  |
| CORESET Reference Channel |  |  | Config 1 |  | CR.1.1 FDD |  |
|  |  |  | Config 2 |  | CR.1.1 TDD |  |
|  |  |  | Config 3 |  | CR.2.1 TDD |  |
| Dedicated CORESET Reference Channel |  |  | Config 1 |  | CCR.1.1. FDD |  |
|  |  |  | Config 2 |  | CCR.1.1. TDD |  |
|  |  |  | Config 3 |  | CCR.2.1. TDD |  |
| SSB Configuration |  |  | Config 1 |  | SSB.3 FR1 |  |
|  |  |  | Config 2 |  | SSB.3 FR1 |  |
|  |  |  | Config 3 |  | SSB.4 FR1 |  |
| SMTC Configuration |  |  | Config 1, 2 |  | SMTC.1 |  |
|  |  |  | Config 3 |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing |  |  | Config 1, 2 |  | 15 KHz |  |
|  |  |  | Config 3 |  | 30 KHz |  |
| PRACH Configuration |  |  | Config 1, 2 |  | PRACH.2 FR1 |  |
|  |  |  | Config 3 |  | PRACH.2 FR1 |  |
| SSB Index assigned as BFD RS (q0) |  |  |  |  | 0 |  |
| SSB Index assigned as CBD RS (q1) |  |  |  |  | 1 |  |
| OCNG parameters |  |  |  |  | OP.1 |  |
| CP length |  |  |  |  | Normal |  |
| Correlation Matrix and Antenna Configuration |  |  |  |  | 2x2 Low |  |
| Beam failure detection transmission parameters | DCI format |  |  |  | 1-0 |  |
|  | Number of Control OFDM symbols |  |  |  | 2 |  |
|  | Aggregation level |  |  | CCE | 8 |  |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy |  |  | dB | 0 |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy |  |  | dB | 0 |  |
|  | DMRS precoder granularity |  |  |  | REG bundle size |  |
|  | REG bundle size |  |  |  | 6 |  |
| DRX |  |  |  |  | DRX.7 | see clause A.3.3.7 of TS 38.133 [6] |
| Gap pattern ID |  |  |  |  | N.A. |  |
| rlmInSyncOutOfSyncThreshold |  |  |  |  | absent | When the field is absent, the UE applies the value 0. |
| rsrp-ThresholdSSB |  | Config 1, 2 |  | dBm/SCS kHz | -98 | Threshold used for Qin_LR_SSB |
|  |  | Config 3 |  | dBm/SCS kHz | -95 | Threshold used for Qin_LR_SSB |
| powerControlOffsetSS |  |  |  |  | db0 | Used for deriving rsrp-ThresholdCSI-RS |
| beamFailureInstanceMaxCount |  |  |  |  | n1 | see clause 5.17 of TS 38.321 [12] |
| beamFailureDetectionTimer |  |  |  |  | pbfd4 | see clause 5.17 of TS 38.321 [12] |
| CSI-RS configuration for CSI reporting |  | Config 1 |  |  | CSI-RS.1.1 FDD |  |
|  |  | Config 2 |  |  | CSI-RS.1.1 TDD |  |
|  |  | Config 3 |  |  | CSI-RS.2.1 TDD |  |
| CSI-RS for tracking |  | Config 1 |  |  | TRS.1.1 FDD |  |
|  |  | Config 2 |  |  | TRS.1.1 TDD |  |
|  |  | Config 3 |  |  | TRS.1.2 TDD |  |
| SSB Index assigned as RLM RS |  |  |  |  | 0, 1 |  |
| T310 Timer |  |  |  | ms | 1000 |  |
| N310 |  |  |  |  | 2 |  |
| T1 |  |  |  | s | 1 | During this time the UE shall be fully synchronized to cell 1 |
| T2 |  |  |  | s | 5.17 |  |
| T3 |  |  |  | s | 3.24 |  |
| T4 |  |  |  | s | 0 |  |
| T5 |  |  |  | s | 1.97 |  |
| D1 |  |  |  | s | 1.93 |  |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |  |  |

6.5.5.2.4.2	Test procedure

Prior to the start of the time duration T1, the UE shall be fully synchronized to NR Cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5 ms. In the test, DRX configuration is enabled.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters of NR Cell 1 according to T1 in Table 6.5.5.2.5-1. Propagation conditions are set according to Annex C.2.3. T1 starts.

3.	When T1 expires the SS shall change the SNR value to T2 as specified in Table 6.5.5.2.5-1. T2 starts.

4.	When T2 expires the SS shall change the SNR value to T3 as specified in Table 6.5.5.2.5-1. T3 starts.

5.	When T3 expires the SS shall change the SNR value to T4 as specified in Table 6.5.5.2.5-1. T4 starts.

6.	When T4 expires the SS shall change the SNR value to T5 as specified in Table 6.5.5.2.5-1. T5 starts.

7.	If the SS:

a) detects uplink power on NR carrier equal to or higher than minimum output power defined in TS 38.521-1 [17] clause 6.3.1.5 in each slot configured for CSI transmission (according CSI reporting on PUCCH) during the period from time point A to time point B

and

b) does not detect preamble on a beam associated with the candidate beam set q1before time point B

and

c) detects preamble on a beam associated with the candidate beam set q1 before time point F (D1 after the start of T5),

the number of successful tests is increased by one.

Otherwise the number of failed tests is increased by one.

8.	Switch the UE off and on. Ensure the UE is in RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

9.	Repeat steps 2-8 for all subtests until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.5.5.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 and clause 7.3 with the following exceptions:

Table 6.5.5.2.4.3-1: Common Exception messages for NR SA FR1 SSB-based beam failure detection and link recovery in DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-8 with Condition SSB BFD Table H.3.1-10 with Condition SSB Table H.3.1-10A Table H.3.5-4 Table H.3.7-1 with Condition DRX.7 Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 6.5.5.2.4.3-2: PDCCH Search Space for BFR

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-162 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| searchSpaceId | 4 | BFR |  |
| controlResourceSetId | 2 | BFR |  |
| monitoringSlotPeriodicityAndOffset CHOICE { |  |  |  |
| sl1 | NULL |  |  |
| } |  |  |  |
| monitoringSymbolsWithinSlot | 10000000000000 | Symbol 0 |  |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel1 | n0 |  |  |
| aggregationLevel2 | n0 |  |  |
| aggregationLevel4 | n0 |  |  |
| aggregationLevel8 | n1 | AL8 |  |
| aggregationLevel16 | n0 |  |  |
| } |  |  |  |
| searchSpaceType CHOICE { |  |  |  |
| ue-Specific SEQUENCE { |  |  | USS |
| dci-Formats | formats0-0-And-1-0 | DCI Format 1_0 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.5.5.2.4.3-3: RLF-TimersAndConstants

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-150 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLF-TimersAndConstants ::= SEQUENCE { |  |  |  |
| n310 | n2 |  |  |
| } |  |  |  |

Table 6.5.5.2.4.3-4: Void

Table 6.5.5.2.4.3-5: PDCCH-Config

| Derivation Path: TS 38.501-1 [14],Table 4.6.3-95 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCCH-Config ::= SEQUENCE { |  |  |  |
| controlResourceSetToAddModList SEQUENCE(SIZE (1..3)) OF ControlResourceSet { | 2 entries |  |  |
| ControlResourceSet[2] | ControlResourceSet | entry 2, BFR |  |
| } |  |  |  |
| controlResourceSetToReleaseList | Not present |  |  |
| searchSpacesToAddModList SEQUENCE(SIZE (1..10)) OF SearchSpace { | 2 entries |  |  |
| SearchSpace[2] | SearchSpace | entry 2, BFR |  |
| } |  |  |  |
| searchSpacesToReleaseList | Not present |  |  |
| downlinkPreemption | Not present |  |  |
| tpc-PUSCH | Not present |  |  |
| tpc-PUCCH | Not present |  |  |
| tpc-SRS | Not present |  |  |
| } |  |  |  |

Table 6.5.5.2.4.3-6: ControlResourceSet for BFR

| Derivation Path: TS 38.501-1 [14],Table 7.3.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| controlResourceSetId | 2 |  |  |
| duration | 2 |  |  |
| cce-REG-MappingType CHOICE { |  |  |  |
| interleaved ::= SEQUENCE { |  |  |  |
| reg-BundleSize | n6 |  |  |
| interleaverSize | n2 |  |  |
| shiftIndex | 0 |  |  |
| } |  |  |  |
| tci-StatesPDCCH-ToAddList | Not present |  |  |
| } |  |  |  |

Table 6.5.5.2.4.3-7: PDCCH Search Space for USS

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-162 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| searchSpaceId | 2 |  |  |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel1 | n0 |  |  |
| aggregationLevel2 | n1 |  |  |
| aggregationLevel4 | n1 |  |  |
| aggregationLevel8 | n0 |  |  |
| } |  |  |  |
| } |  |  |  |

6.5.5.2.5	Test requirement

Tables 6.5.5.2.4.1-3 and 6.5.5.2.5-1 define the primary level settings including test tolerances for NR SA FR1 SSB-based beam failure detection and link recovery in DRX.

Table 6.5.5.2.5-1: NR Cell specific test parameters for NR SA FR1 SSB-based beam failure detection and link recovery in DRX

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| SNR_ SSB of set q0 | Config 1 | dB | 5.8 | -2.2 | -12.8 | -12.8 | -12.8 |
|  | Config 2 |  | 5.8 | -2.2 | -12.8 | -12.8 | -12.8 |
|  | Config 3 |  | 5.8 | -2.2 | -12.8 | -12.8 | -12.8 |
| SNR_SSB of set q1 | Config 1 | dB | -10.2 | -10.2 | 10.2 | 10.2 | 10.2 |
|  | Config 2 |  | -10.2 | -10.2 | 10.2 | 10.2 | 10.2 |
|  | Config 3 |  | -10.2 | -10.2 | 10.2 | 10.2 | 10.2 |
| SSB_RP of set q1 | Config 1 | dBm/SCS kHz | -108.2 | -108.2 | -87.8 | -87.8 | -87.8 |
|  | Config 2 |  | -108.2 | -108.2 | -87.8 | -87.8 | -87.8 |
|  | Config 3 |  | -105.2 | -105.2 | -84.8 | -84.8 | -84.8 |
|  | Config 1 | dBm/15 kHz | -98 |  |  |  |  |
|  | Config 2 |  | -98 |  |  |  |  |
|  | Config 3 |  | -98 |  |  |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Void. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure 6.5.5.2.4-1. Note 9: The SNR values are specified for a UE with 2RX antennas connected under test. For a UE with 4RX antennas connected under test, the SNR for RS in set q0 during T3, T4, and T5 from D.4.1.1, is -15dB-TT = -15.8dB (including test tolerances). |  |  |  |  |  |  |  |

The UE behaviour during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the time duration T1 and T2, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on Cell 1.

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 1 in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 1.

During T3 the UE shall detect beam failure and initiate link recovery. During T4 and T5 the UE measures and evaluate beam candidate from beam candidate set q1.

No later than time point F occurring no later than D1 = 1930 ms after the start of T5, the UE shall transmit preamble on a beam associated with the candidate beam set q1. The UE shall not transmit preamble on a beam associated with the candidate beam set q1 earlier than time point B.

Test is concluded once the test equipment has received the initial preamble transmission from the UE. The rate of correct events observed during repeated tests shall be at least 90%.

<< End of Changes >>