3GPP TSG-RAN5 Meeting #105	R5-246275

# Orlando, US, 18 – 22 Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | 0710 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of bands n13 and n85 into 4 Rx antenna ports capabilities |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 4Rx_NR_bands_R18-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Missing 4 Rx antenna ports capabilities for bands n13 and n85 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding bands n13 and n85 into: - Table A.4.3.9-4a: FDD 4 Rx antenna ports Capabilities |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 4 Rx antenna ports capabilities will not be defined for bands n13 and n85 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.3.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------- Start of changes --------

### A.4.3.9	Additional capabilities for UE declared capability

Table A.4.3.9-1: UE declared capabilities

| Item | UE declared capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Enhanced Type 1 Receiver for NR | 38.101-4, 5 | Rel-15 | pc_nr_enh_type1_receiver | Support for Enhanced Type 1 Receiver (SU-MIMO Interference Mitigation advanced receiver) |
| 2 | Vehicular UE | 38.101-1, 3 | Rel-15 | pc_nr_vehicular_ue |  |
| 3 | MMSE-IRC (Minimum Mean Square Error - Interference Rejection Combining) receiver | 38.101-4, 5 | Rel-15 | pc_nr_mmse_irc_receiver | Support of MMSE-IRC processing for scenarios with inter-cell and intra-cell inter-user interference. |
| 4 | Baseline SU-MIMO 8Rx receiver | 38.306, 5.2 | Rel-17 | pc_baseline_su-mimo_8rx_receiver | Support of 8Rx receivers for SU-MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector in FR1 |
| 5 | Simplified SU-MIMO 8Rx receiver | 38.306, 5.2 | Rel-17 | pc_simplified_su-mimo_8rx_receiver | Support of 8Rx receivers for SU-MIMO transmissions with support of up to 4 layers with two joint 4Rx MIMO detectors in FR1 |

-------- Skipped unchanged clauses --------

Table A.4.3.9-4a: FDD 4 Rx antenna ports Capabilities

| Item | Band | Ref. | Release | Comments |
| --- | --- | --- | --- | --- |
| 1 | FDD Band n1 | 38.101-1, 7.3.2 | Rel-15 |  |
| 2 | FDD Band n2 | 38.101-1, 7.3.2 | Rel-15 |  |
| 3 | FDD Band n3 | 38.101-1, 7.3.2 | Rel-15 |  |
| … |  |  |  |  |
| 5 | FDD Band n5 | 38.101-1, 7.3.2 | Rel-18 | 4 Rx operation is supported by handheld UE |
| … |  |  |  |  |
| 7 | FDD Band n7 | 38.101-1, 7.3.2 | Rel-15 | NOTE 2 |
| 8 | FDD Band n8 | 38.101-1.7.3.2 | Rel-17 | 4 Rx operation is supported by FWA form factor |
|  |  |  | Rel-18 | 4 Rx operation is supported by handheld UE |
| ... |  |  |  |  |
| 13 | FDD Band n13 | 38.101-1, 7.3.2 | Rel-18 |  |
| ... |  |  |  |  |
| 20 | FDD Band n20 | 38.101-1.7.3.2 | Rel-18 | 4 Rx operation is supported by handheld UE |
| … |  |  |  |  |
| 25 | FDD Band n25 | 38.101-1, 7.3.2 | Rel-18 |  |
| 26 | FDD Band n26 | 38.101-1.7.3.2 | Rel-18 | 4 Rx operation is supported by handheld UE |
| … |  |  |  |  |
| 28 | FDD Band n28 | 38.101-1, 7.3.2 | Rel-16 | 4 Rx operation is supported by FWA form factor |
|  |  |  | Rel-18 | 4 Rx operation is supported by handheld UE |
| ... |  |  |  |  |
| 30 | FDD Band n30 | 38.101-1, 7.3.2 | Rel-16 |  |
| ... |  |  |  |  |
| 66 | FDD Band n66 | 38.101-1, 7.3.2 | Rel-15 |  |
| … |  |  |  |  |
| 70 | FDD Band n70 | 38.101-1, 7.3.2 | Rel-15 |  |
| 71 | FDD Band n71 | 38.101-1, 7.3.2 | Rel-16 | 4 Rx operation is supported by FWA form factor |
|  |  |  | Rel-18 | 4 Rx operation is supported by handheld UE |
| ... |  |  |  |  |
| 85 | FDD Band n85 | 38.101-1, 7.3.2 | Rel-18 |  |
| NOTE 1: At least one band from those listed in the present table needs to be supported if UE has indicated support of the capability defined in Table A.4.3.1-7a/2. NOTE 2: Support of 4 Rx for this band is mandatory for non-vehicular UEs i.e. if support has NOT been indicated to the capability specified in Table A.4.3.9-1/2. |  |  |  |  |

-------- End of changes --------