3GPP TSG-RAN5 Meeting #105	R5-246371

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4680 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to CHO TC 8.1.4.4.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_Mob_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The table references were incorrect or incomplete in the specific message tables. To align the specific message content with TS 38.508-1 and TS 38.331. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. The table reference and specific message contents were updated. 2. Editorial updates. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Errors will exist these cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

8.1.4.4.2	Conditional handover / modify conditional handover configuration

8.1.4.4.2.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state and supporting conditional handover and receiving an RRCReconfiguration message including a ConditionalReconfiguration }

ensure that {

when { UE received another RRCReconfiguration message with adding a new entry in the CondReconfigToAddModList before any CHO execution condition is satisfied }
    then { UE add the new entry within the VarConditionalReconfig and apply the latest conditional handover configurations }

}

(2)

with { UE in NR RRC_CONNECTED state and supporting conditional handover and receiving an RRCReconfiguration message including a ConditionalReconfiguration }

ensure that {

when { UE received another RRCReconfiguration message with modifying an existing entry in the CondReconfigToAddModList before any CHO execution condition is satisfied }
    then { UE replace the existing entry with the value received and apply the latest conditional handover configurations }

}

(3)

with { UE in NR RRC_CONNECTED state and supporting conditional handover and receiving an RRCReconfiguration message including a ConditionalReconfiguration }

ensure that {

when { UE received another RRCReconfiguration message with condConfigToRemoveList before any CHO execution condition is satisfied }
    then { UE removes the entry with the matching condConfigId included in the condConfigToRemoveList and apply the latest conditional handover configurations }

}

8.1.4.4.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.300 clause 9.2.3.4.1 and 9.2.3.4.2 and TS 38.331 clause 5.3.5.1, 5.3.5.13.2 and 5.3.5.13.3. Unless otherwise stated these are Rel-16 requirements.

[TS 38.300, clause 9.2.3.4.1]

A Conditional Handover (CHO) is defined as a handover that is executed by the UE when one or more handover execution conditions are met. The UE starts evaluating the execution condition(s) upon receiving the CHO configuration, and stops evaluating the execution condition(s) once a handover is executed (legacy handover or conditional handover execution).

The following principles apply to CHO:

-	The CHO configuration contains the configuration of CHO candidate cell(s) generated by the candidate gNB(s) and execution condition(s) generated by the source gNB.

-	An execution condition may consist of one or two trigger condition(s) (CHO events A3/A5, as defined in [12]). Only single RS type is supported and at most two different trigger quantities (e.g. RSRP and RSRQ, RSRP and SINR, etc.) can be configured simultaneously for the evaluation of CHO execution condition of a single candidate cell.

-	Before any CHO execution condition is satisfied, upon reception of HO command (without CHO configuration), the UE executes the HO procedure as described in clause 9.2.3.2, regardless of any previously received CHO configuration.

-	While executing CHO, i.e. from the time when the UE starts synchronization with target cell, UE does not monitor source cell.

CHO is not supported for NG-C based handover in this release of the specification.

[TS 38.300, clause 9.2.3.4.2]

As in intra-NR RAN handover, in intra-NR RAN CHO, the preparation and execution phase of the conditional handover procedure is performed without involvement of the 5GC; i.e. preparation messages are directly exchanged between gNBs. The release of the resources at the source gNB during the conditional handover completion phase is triggered by the target gNB. The figure below depicts the basic conditional handover scenario where neither the AMF nor the UPF changes:

Figure 9.2.3.4.2-1: Intra-AMF/UPF Conditional Handover

0/1.	Same as step 0, 1 in Figure 9.2.3.2.1-1 of clause 9.2.3.2.1.

2.	The source gNB decides to use CHO.

3.	The source gNB requests CHO for one or more candidate cells belonging to one or more candidate gNBs. A CHO request message is sent for each candidate cell.

4.	Same as step 4 in Figure 9.2.3.2.1-1 of clause 9.2.3.2.1.

5.	The candidate gNB(s) sends CHO response (HO REQUEST ACKNOWLEDGE) including configuration of CHO candidate cell(s) to the source gNB. The CHO response message is sent for each candidate cell.

6.	The source gNB sends an RRCReconfiguration message to the UE, containing the configuration of CHO candidate cell(s) and CHO execution condition(s).

NOTE 1:	CHO configuration of candidate cells can be followed by other reconfiguration from the source gNB.

NOTE 1a:	A configuration of a CHO candidate cell cannot contain a DAPS handover configuration.

7.	The UE sends an RRCReconfigurationComplete message to the source gNB.

7a	If early data forwarding is applied, the source gNB sends the EARLY STATUS TRANSFER message.

8.	The UE maintains connection with the source gNB after receiving CHO configuration, and starts evaluating the CHO execution conditions for the candidate cell(s). If at least one CHO candidate cell satisfies the corresponding CHO execution condition, the UE detaches from the source gNB, applies the stored corresponding configuration for that selected candidate cell, synchronises to that candidate cell and completes the RRC handover procedure by sending RRCReconfigurationComplete message to the target gNB. The UE releases stored CHO configurations after successful completion of RRC handover procedure.

8a/b	The target gNB sends the HANDOVER SUCCESS message to the source gNB to inform that the UE has successfully accessed the target cell. In return, the source gNB sends the SN STATUS TRANSFER message following the principles described in step 7 of Intra-AMF/UPF Handover in clause 9.2.3.2.1.

NOTE 2:	Late data forwarding may be initiated as soon as the source gNB receives the HANDOVER SUCCESS message.

8c.	The source gNB sends the HANDOVER CANCEL message toward the other signalling connections or other candidate target gNBs, if any, to cancel CHO for the UE.

[TS 38.331, clause 5.3.5.1]

Figure 5.3.5.1-1: RRC reconfiguration, successful

Figure 5.3.5.1-2: RRC reconfiguration, failure

The purpose of this procedure is to modify an RRC connection, e.g. to establish/modify/release RBs, to perform reconfiguration with sync, to setup/modify/release measurements, to add/modify/release SCells and cell groups, to add/modify/release conditional handover configuration, to add/modify/release conditional PSCell change configuration. As part of the procedure, NAS dedicated information may be transferred from the Network to the UE.

[TS 38.331, clause 5.3.5.13.2]

The UE shall:

1>	for each condReconfigId value included in the condReconfigToRemoveList that is part of the current UE conditional reconfiguration in VarConditionalReconfig:

2>	remove the entry with the matching condReconfigId from the VarConditionalReconfig;

NOTE:	The UE does not consider the message as erroneous if the condReconfigToRemoveList includes any condReconfigId value that is not part of the current UE configuration.

[TS 38.331, clause 5.3.5.13.3]

For each condReconfigId received in the condReconfigToAddModList IE the UE shall:

1>	if an entry with the matching condReconfigId exists in the condReconfigToAddModList within the VarConditionalReconfig:

2>	if the entry in condReconfigToAddModList includes an condExecutionCond;

3>	replace the entry with the value received for this condReconfigId;

2>	if the entry in condReconfigToAddModList includes an condRRCReconfig;

2>	replace the entry with the value received for this condReconfigId;

1>	else:

2>	add a new entry for this condReconfigId within the VarConditionalReconfig;

1>	perform conditional reconfiguration evaluation as specified in 5.3.5.13.4;

8.1.4.4.2.3	Test description

8.1.4.4.2.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1, NR Cell 2 and NR Cell 4.

-	System information combination NR-2 as defined in TS 38.508-1 [4] clause 4.4.3.1.3 is used for NR cells.

UE:

-	None.

Preamble:

-	If pc_IP_Ping is set to TRUE then, the UE is in 5GS state 3N-A on NR Cell 1 according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3.

-	Else, the UE is in 5GS state 3N-A and Test Loop Function (On) with UE test loop mode B on NR Cell 1 according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3.

8.1.4.4.2.3.2	Test procedure sequence

Tables 8.1.4.4.2.3.2-1 and 8.1.4.4.2.3.2-2 illustrate the downlink power levels to be applied for NR cells at various time instants of the test execution. Row marked "T0" denotes the conditions after the preamble, while the configuration marked "T1", "T2", "T3" and "T4", are applied at the point indicated in the Main behaviour description in Table 8.1.4.4.2.3.2-3.

Table 8.1.4.4.2.3.2-1: Power levels in FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | NR Cell 4 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/ SCS | -82 | -91 | -91 | Power levels are such that UE registered on NR Cell 1 and entry condition for event A3 is not satisfied |
| T1 | SS/PBCH SSS EPRE | dBm/ SCS | -91 | -79 | -91 | Power levels are such that entry condition for event A3 is satisfied for NR Cell 2 |
| T2 | SS/PBCH SSS EPRE | dBm/ SCS | -91 | -91 | -79 | Power levels are such that entry condition for event A3 is satisfied for NR Cell 4 |
| T3 | SS/PBCH SSS EPRE | dBm/ SCS | -91 | -85 | -91 | Power levels are such that leaving condition for NR Cell 4 is satisfied |
| T4 | SS/PBCH SSS EPRE | dBm/ SCS | -85 | -91 | -79 | Power levels are such that entry condition for event A3 is satisfied for NR Cell 1 and NR Cell 4 |

Table 8.1.4.4.2.3.2-2: Power levels in FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | NR Cell 4 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/ SCS | -82 | -91 | -91 | Power levels are such that UE registered on NR Cell 1 and entry condition for event A3 is not satisfied |
| T1 | SS/PBCH SSS EPRE | dBm/ SCS | -91 | -82 | -91 | Power levels are such that entry condition for event A3 is satisfied for NR Cell 2 |
| T2 | SS/PBCH SSS EPRE | dBm/ SCS | -91 | -91 | -82 | Power levels are such that entry condition for event A3 is satisfied for NR Cell 4 |
| T3 | SS/PBCH SSS EPRE | dBm/ SCS | -91 | -82 | -91 | Power levels are such that leaving condition for NR Cell 4 is satisfied |
| T4 | SS/PBCH SSS EPRE | dBm/ SCS | -91 | -100 | -82 | Power levels are such that entry condition for event A3 is satisfied for NR Cell 1 and NR Cell 4 |

Table 8.1.4.4.2.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCReconfiguration message including MeasConfig and ConditionalReconfiguration to set NR Cell 4 as a target candidate cell. | <-- | NR RRC: RRCReconfiguration | - | - |
| 2 | The UE transmits an RRCReconfigurationComplete message in NR Cell 1. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 3 | The SS transmits an RRCReconfiguration message including ConditionalReconfiguration to add NR Cell 2 as another target candidate cell. | <-- | NR RRC: RRCReconfiguration | - | - |
| 4 | The UE transmits an RRCReconfigurationComplete message in NR Cell 1. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 5 | SS adjusts the cell-specific reference signal level according to row "T1". | - | - | - | - |
| 6 | Check: Does the UE initiate a random access procedure by transmitting Preamble in NR Cell 2 within 10s? | --> | (PRACH Preamble) | 1 | P |
| 7 | The SS transmits Random Access Response. | <-- | Random Access Response | - | - |
| 8 | The UE transmits RRCReconfigurationComplete message in NR Cell 2. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 9 | Check: Does the test result of generic test procedure in TS 38.508-1 [4] Table 4.9.1-1 indicate that the UE is capable of exchanging IP data on DRB1 and NR Cell 2? | - | - | 1 | - |
| 10 | The SS transmits an RRCReconfiguration message including MeasConfig and ConditionalReconfiguration to set NR Cell 1 and NR Cell 4 as target candidate cells. | <-- | NR RRC: RRCReconfiguration | - | - |
| 11 | The UE transmits an RRCReconfigurationComplete message in NR Cell 2. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 12 | The SS transmits an RRCReconfiguration message including ConditionalReconfiguration with condConfigToRemoveList to remove NR Cell 4 from the target candidate cell list. | <-- | NR RRC: RRCReconfiguration | - | - |
| 13 | The UE transmits an RRCReconfigurationComplete message in NR Cell 2. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 14 | SS adjusts the cell-specific reference signal level according to row "T2". | - | - | - | - |
| 15 | Check: Does the UE initiate a random access procedure by transmitting Preamble in NR Cell 4 within 10s? | --> | (PRACH Preamble) | 3 | F |
| 15A | SS adjusts the cell-specific reference signal level according to row "T3". | - | - | - | - |
| 16 | The SS transmits an RRCReconfiguration message including ConditionalReconfiguration to replace NR Cell 1 with NR Cell 4 as the target candidate cell. | <-- | NR RRC: RRCReconfiguration | - | - |
| 16A | The UE transmits RRCReconfigurationComplete message in NR Cell 2. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 17 | SS adjusts the cell-specific reference signal level according to row "T4". | - | - | - | - |
| 18 | Check: Does the UE initiate a random access procedure by transmitting Preamble in NR Cell 4 within 10s? | --> | (PRACH Preamble) | 3 | P |
| 19 | The SS transmits Random Access Response. | <-- | Random Access Response | - | - |
| 20 | The UE transmits RRCReconfigurationComplete message in NR Cell 4. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 21 | Check: Does the test result of generic test procedure in TS 38.508-1 [4] Table 4.9.1-1 indicate that the UE is capable of exchanging IP data on DRB1 and NR Cell 4? | - | - | 2 | - |

8.1.4.4.2.3.3	Specific message contents

Table 8.1.4.4.2.3.3-1: RRCReconfiguration (Step 1, step 3, step 10, step 12 and step 16, Table 8.1.4.4.2.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 with condition NR_MEAS and CHO |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| measConfig | Not present |  | Step 3, Step 12, Step 16 |
|  | MeasConfig | Table 8.1.4.4.2.3.3-2 | Step 1, Step 10 |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| conditionalReconfiguration-r16 | ConditionalReconfiguration | Table 8.1.4.4.2.3.3-5 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.4.4.2.3.3-2: MeasConfig (Table 8.1.4.4.2.3.3-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-69 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasObjectToAddMod { | 1 entry |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measObjectId | 1 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR | Table 8.1.4.4.2.3.3-3 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE(SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod { | 1 entry |  |  |
| ReportConfigToAddMod[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | 1 |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigNR | ReportConfigNR-condEventA3 | Table 8.1.4.4.2.3.3-4 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod { | 1 entry |  |  |
| MeasIdToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measId | 1 |  |  |
| measObjectId | 1 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.4.4.2.3.3-3: MeasObjectNR (Table 8.1.4.4.2.3.3-2)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-76 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR ::= SEQUENCE { |  |  |  |
| ssbFrequency | ARFCN-ValueNR for SSB of NR Cell 1 | The SSB of NR Cell 1, NR Cell 2 and NR Cell 4 have the same ARFCN value as specified in TS 38.508-1 [4] clause 6.2.3 |  |
| absThreshSS-BlocksConsolidation | Not present |  |  |
| nrofSS-BlocksToAverage | Not present |  |  |
| } |  |  |  |

Table 8.1.4.4.2.3.3-4: ReportConfigNR-condEventA3 (Table 8.1.4.4.2.3.3-2)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-142 with condition CHO AND EVENT_A3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR ::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| condTriggerConfig-r16 SEQUENCE { |  |  |  |
| condEventId CHOICE { |  |  |  |
| condEventA3 SEQUENCE { |  |  |  |
| a3-Offset CHOICE { |  |  |  |
| rsrp | 2 | 1 dB(2*0.5 dB) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.4.4.2.3.3-5: ConditionalReconfiguration (Table 8.1.4.4.2.3.3-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-25D |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ConditionalReconfiguration-r16 ::= SEQUENCE { |  |  |  |
| condReconfigToRemoveList-r16 | Not present |  |  |
| condReconfigToRemoveList-r16 SEQUENCE (SIZE (1.. maxNrofCondCells-r16)) OF CondReconfigId-r16 { | 1 entry |  | Step 12 |
| CondReconfigId-r16[1] | 2 | entry 1 |  |
| } |  |  |  |
| condReconfigToAddModList-r16 | CondReconfigToAddModList-r16 (Step 1 and step 16) | Table 8.1.4.4.2.3.3-6 | Step 1, Step 16 |
|  | CondReconfigToAddModList-r16 (Step 3) | Table 8.1.4.4.2.3.3-9 | Step 3 |
|  | CondReconfigToAddModList-r16 (Step 10) | Table 8.1.4.4.2.3.3-10 | Step 10 |
| } |  |  |  |

Table 8.1.4.4.2.3.3-6: CondReconfigToAddModList-r16 (Step 1 and step 16, Table 8.1.4.4.2.3.3-6)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-25C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CondReconfigToAddModList-r16 ::= SEQUENCE (SIZE (1.. maxNrofCondCells-r16)) OF CondReconfigToAddMod-r16 { | 1 entry |  |  |
| CondReconfigToAddMod-r16[1] SEQUENCE { |  | entry 1 |  |
| condReconfigId-r16 | 1 |  |  |
| condExecutionCond-r16 SEQUENCE { |  |  |  |
| MeasId [1] | 1 |  |  |
| } |  |  |  |
| condRRCReconfig-r16 | OCTETSTRING (Containing RRCReconfiguration-HO with condition HO_NR Cell 4) | Table 8.1.4.4.2.3.3-7 | Step 1, Step 16 |
| } |  |  |  |
| } |  |  |  |

Table 8.1.4.4.2.3.3-7: RRCReconfiguration-HO (Tables 8.1.4.4.2.3.3-6, 8.1.4.4.2.3.3-9 and 8.1.4.4.2.3.3-10)

| Derivation Path: TS 38.508-1 [4] Table 4.8.1-1A with condition RBConfig_KeyChange |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| masterCellGroup | CellGroupConfig with condition PCI_NR Cell 1 | Table 8.1.4.4.2.3.3-8 | HO_NR Cell 1 |
|  | CellGroupConfig with condition PCI_NR Cell 2 | Table 8.1.4.4.2.3.3-8 | HO_NR Cell 2 |
|  | CellGroupConfig with condition PCI_NR Cell 4 | Table 8.1.4.4.2.3.3-8 | HO_NR Cell 4 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.4.4.2.3.3-8: CellGroupConfig (Table 8.1.4.4.2.3.3-7)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 with Condition PCell_change and CFRA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| reconfigurationWithSync SEQUENCE { |  |  |  |
| spCellConfigCommon SEQUENCE { |  |  |  |
| physCellId | Physical Cell Identity of NR Cell 1 |  | PCI_NR Cell 1 |
|  | Physical Cell Identity of NR Cell 2 |  | PCI_NR Cell 2 |
|  | Physical Cell Identity of NR Cell 4 |  | PCI_NR Cell 4 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.4.4.2.3.3-9: CondReconfigToAddModList-r16 (Step 3, Table 8.1.4.4.2.3.3-6)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-25C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CondReconfigToAddModList-r16 ::= SEQUENCE (SIZE (1.. maxNrofCondCells-r16)) OF CondReconfigToAddMod-r16 { | 1 entry |  |  |
| CondReconfigToAddMod-r16[1] SEQUENCE { |  | entry 1 |  |
| condReconfigId-r16 | 2 |  |  |
| condExecutionCond-r16 SEQUENCE { |  |  |  |
| MeasId [1] | 1 |  |  |
| } |  |  |  |
| condRRCReconfig-r16 | OCTETSTRING (Containing RRCReconfiguration-HO with condition HO_NR Cell 2) | Table 8.1.4.4.2.3.3-7 |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.4.4.2.3.3-10: CondReconfigToAddModList-r16 (Step 10, Table 8.1.4.4.2.3.3-6)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-25C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CondReconfigToAddModList-r16 ::= SEQUENCE (SIZE (1.. maxNrofCondCells-r16)) OF CondReconfigToAddMod-r16 { | 2 entries |  |  |
| CondReconfigToAddMod-r16[1] SEQUENCE { |  | entry 1 |  |
| condReconfigId-r16 | 1 |  |  |
| condExecutionCond-r16 SEQUENCE { |  |  |  |
| MeasId [1] | 1 |  |  |
| } |  |  |  |
| condRRCReconfig-r16 | OCTETSTRING (Containing RRCReconfiguration-HO with condition HO_NR Cell 1) | Table 8.1.4.4.2.3.3-8 |  |
| } |  |  |  |
| CondReconfigToAddMod-r16[2] SEQUENCE { |  | entry 2 |  |
| condReconfigId-r16 | 2 |  |  |
| condExecutionCond-r16 SEQUENCE { |  |  |  |
| MeasId [1] | 1 |  |  |
| } |  |  |  |
| condRRCReconfig-r16 | OCTETSTRING (Containing RRCReconfiguration-HO with condition HO_NR Cell 4) | Table 8.1.4.4.2.3.3-8 |  |
| } |  |  |  |
| } |  |  |  |

[End of change]