3GPP TSG-RAN5 Meeting #105	R5-246388

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5362 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NB-IoT NTN TC 22.5.26 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc., CMCC, China Telecom, SRTC, Sateliot, Inmarsat, ViaSat, EchoStar, Novamint, Thales, ESA |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18_Test |  |  |  |  |  | Date: |  |  | 2024-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To add the new NB-IoT NTN TC 22.5.26 for UE coarse location information reporting. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New TC 22.5.26 NB-IoT / NTN / Normal tracking area update / Coarse location information reporting was added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of test coverage for UE coarse location information reporting. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.5.26 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 36.523-2 CR 1468 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

22.5.26	NB-IoT / NTN / Normal tracking area update / Coarse location information reporting

22.5.26.1	Test Purpose (TP)

(1)

with { UE has valid GNSS position and sets the RCLIN bit to "reporting coarse location information via NAS supported" in the UE network capability IE of the normal TRACKING AREA UPDATE REQUEST message }

ensure that {

when { UE has received a SECURITY MODE COMMAND message with the "coarse location information of the UE requested" }

then { UE shall include the UE coarse location information in the SECURITY MODE COMPLETE message to complete the normal tracking area update procedure }

}

22.5.26.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301 clauses 5.4.3.1, 5.4.3.2, 5.4.3.3 and 5.5.3.2.2. Unless otherwise stated these are Rel-18 requirements.

[TS 24.301, clause 5.4.3.1]

Furthermore, the network may also initiate the security mode control procedure in the following cases:

-	in order to change the NAS security algorithms for a current EPS security context already in use;

-	in order to change the value of uplink NAS COUNT used in the latest SECURITY MODE COMPLETE message as described in 3GPP TS 33.401 [19], clause 7.2.9.2;

-	in order to request the UE radio capability ID from the UE; and

-	in order to request the UE to report the coarse location information.

[TS 24.301, clause 5.4.3.2]

If:

-	the NAS security mode control procedure is initiated during an ongoing attach procedure, tracking area updating procedure, or service request procedure in NB-S1 mode via satellite E-UTRAN access;

-	the network supports coarse location information reporting; and

-	the UE has set the RCLIN bit to "reporting coarse location information via NAS supported" in the UE network capability IE of the ATTACH REQUEST message or TRACKING AREA UPDATE REQUEST message:

then the MME may request the UE to include its coarse location information in the SECURITY MODE COMPLETE message.

NOTE 3:	Based on operator policy, the network can also initiate the NAS security mode control procedure to request the UE to report the coarse location information in NB-S1 mode via satellite E-UTRAN access when secure exchange of NAS messages via a NAS signalling connection is established between the MME and the UE.

[TS 24.301, clause 5.4.3.3]

In NB-S1 mode via satellite E-UTRAN access, if the UE supports reporting coarse location information via NAS and the MME indicated in the SECURITY MODE COMMAND message that the coarse location information of the UE is requested, the UE shall include its current coarse location information in the UE coarse location information IE of the SECURITY MODE COMPLETE message.

If, during an ongoing attach or tracking area updating procedure, the SECURITY MODE COMMAND message includes a HASHMME, the UE shall compare HASHMME with a hash value locally calculated as described in 3GPP TS 33.401 [19] from the entire plain ATTACH REQUEST or TRACKING AREA UPDATE REQUEST message that the UE had sent to initiate the procedure. If HASHMME and the locally calculated hash value are different, the UE shall include the complete ATTACH REQUEST or TRACKING AREA UPDATE REQUEST message which the UE had previously sent in the Replayed NAS message container IE of the SECURITY MODE COMPLETE message.

[TS 24.301, clause 5.5.3.2.2]

In NB-S1 mode via satellite E-UTRAN access, for all cases except case b, if the UE supports reporting coarse location information via NAS, the UE shall set the RCLIN bit to "reporting coarse location information via NAS supported" in the UE network capability IE of the TRACKING AREA UPDATE REQUEST message.

22.5.26.3	Test description

22.5.26.3.1	Pre-test conditions

System Simulator:

-	Ncell 1 and Ncell 11 as specified in TS 36.508 [18] clause 8.1.4.1.2 are configured.

-	System information combination 9 as defined in TS 36.508 [18] clause 8.1.4.3.1.1 is used for both cells.

UE:

-	The pre-configured UE location is defined in TS 36.508 [18] Clause 4.13.

Preamble:

-	The UE is in State 3-NB Registered, Idle Mode on Ncell 1 according to TS 36.508 [18].

22.5.26.3.2	Test procedure sequence

Table 22.5.26.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. The configuration marked "T1" is applied at the point indicated in the Main behaviour description in Table 22.5.26.3.2-2. Cell powers are chosen for a serving cell and a non-suitable “Off” cell as defined in TS 36.508 [18].

Table 22.5.26.3.2-1: Ncell configuration changes over time

|  | Parameter | Unit | Ncell 1 | Ncell 11 | Remarks |
| --- | --- | --- | --- | --- | --- |
| T0 | NRS EPRE | dBm/15kHz | -85 | “Off” | Power level “Off” is defined in TS 36.508 Table 8.3.2.2.1-1 |
| T1 | NRS EPRE | dBm/15kHz | -97 | -85 | Ncell 11 becomes the strongest and highest ranked one |

Table 22.5.26.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS adjusts Ncell levels according to row T1 of table 22.5.26.3.2-1. | - | - | - | - |
| 2 | The UE transmits a TRACKING AREA UPDATE REQUEST message. | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 3 | The SS transmits a NAS SECURITY MODE COMMAND message including UE coarse location information request to activate NAS security. | <-- | NAS: SECURITY MODE COMMAND | - | - |
| 4 | Check: Does the UE transmit a NAS SECURITY MODE COMPLETE message including UE coarse location information? | --> | NAS: SECURITY MODE COMPLETE | 1 | P |
| 5 | The SS responds with TRACKING AREA UPDATE ACCEPT message with new GUTI allocated. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 6 | The UE send a TRACKING AREA UPDATE COMPLETE. | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| 7 | The SS releases the RRC connection. | - | - | - | - |

22.5.26.3.3	Specific message contents

Table 22.5.26.3.3-1: TRACKING AREA UPDATE REQUEST (Step 2, table 22.5.26.3.2-2)

| Derivation Path: TS 36.508 [18], Table 4.7.2-27 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS update type | '000'B | TA updating |  |
| UE network capability |  |  |  |
| Reporting coarse location information via NAS RCLIN (octet 11, bit 4) | '1'B | reporting coarse location information via NAS supported |  |

Table 22.5.26.3.3-2: SECURITY MODE COMMAND (Step 3, table 22.5.26.3.2-2)

| Derivation Path: TS 36.508 [18], Table 4.7.2-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UE coarse location information request |  |  |  |
| UE coarse location information request (UCLIR) (octet 1, bit 1) | '1'B | UE coarse location information requested |  |

Table 22.5.26.3.3-3: SECURITY MODE COMPLETE (Step 4, table 22.5.26.3.2-2)

| Derivation Path: TS 36.508 [18], Table 4.7.2-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UE coarse location information | Present but contents not checked |  |  |

[End of change]