# 3GPP TSG-RAN5 Meeting #105	R5-246706

# Orlando, US, Nov 18th - Nov 22th, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4734 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of test case 8.1.6.1.2.14 for SON_MDT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_ENDC_SON_MDT_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Update SON/MDT TC 8.1.6.1.2.14 Logged MDT / RRC_IDLE / Logging and reporting / IDC mechanism. - As the “Switch on UE’s WLAN” function does not have the AT command in 27.007. The description of “Switch on/off UE’s WLAN”should be removed. By switching on/off the WLAN AP, the UE can also connect to the WLAN AP. - As the logging interval is 2.56s, the waiting time 10s for UE to activate/resume logging is too long, and needs to be updated to a shorter time, e.g. 5s. - Some of the Notes in test procedure is redundant and incorrect, which needs to be removed. - The UEInformationRespons message content needs to be updated to make the description more clear. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The test steps of “Switch on/off UE’s WLAN” are removed. - The Pre-test conditions of UE is changed from “UE’s WLAN is off” to “UE’s WLAN is on”. - As the logging interval is 2.56s, the waiting time to enable UE to activate/resume logging is changed from “10s” to “5s”, which is enough for UE to activate/resume logging. - The Test procedure is updated to remove redundant content. - The UEInformationRespons message content is updated to make the description more clear. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TC is incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.1.2.14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

8.1.6.1.2.14	Logged MDT / RRC_IDLE / Logging and reporting / IDC mechanism

8.1.6.1.2.14.1	Test Purpose (TP)

(1)

with { UE is performing measurement logging and T330 is running }

ensure that {

when { UE detects IDC problem }

then { UE suspend measurement logging and tag MDT report with inDeviceCoexDetected-r17 flag in logMeasInfoList-r16 in VarLogMeasReport }

}

(2)

with { UE has suspended measurement logging and T330 is running }

ensure that {

when { The IDC problems detected by the UE is resolved during the last logging interval }

then { UE resume measurement logging }

}

8.1.6.1.2.14.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.331, clauses 5.5a.3.2. Unless otherwise stated these are Rel-17 requirements.

[TS 38.331 clause 5.5a.3.2]

While T330 is running and SDT procedure is not ongoing, the UE shall:

1>	if measurement logging is suspended:

2>	if during the last logging interval the IDC problems detected by the UE is resolved, resume measurement logging;

1>	if not suspended, perform the logging in accordance with the following:

...

2>	when performing the logging:

3> if InterFreqTargetInfo is configured and if the UE detected IDC problems on at least one of the frequencies included in InterFreqTargetInfo during the last logging interval, or

3>	if InterFreqTargetInfo is not configured and if the UE detected IDC problems during the last logging interval:

4>	if measResultServingCell in the VarLogMeasReport is not empty:

5>	include inDeviceCoexDetected;

5>	suspend measurement logging from the next logging interval;

4>	else:

5>	suspend measurement logging;

...

8.1.6.1.2.14.3	Test description

8.1.6.1.2.14.3.1	Pre-test conditions

System Simulator:

-	NR Cell 14/NRf4 (2390-2400MHz for band n40), WLAN AP 1 (Cell 27) as specified in cl. 4.13 TS 38.508-1[4];

UE:

UE’s WLAN is on

Preamble:

The UE is in state 3N-A on NR Cell 14 according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3.

8.1.6.1.2.14.3.2	Test procedure sequence

Table 8.1.6.1.2.14.3.2-1a for FR1 illustrate the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause. The configuration “C1” for "T0" indicates the initial conditions. Subsequent configurations marked "T1" and "T2" are applied at the points indicated in the Main behaviour description in Table 8.1.6.1.2.14.3.2-1. The configuration “C1” for “T0” and “C2” for “T1” are specified in TS 38.508-1 [4] Table 4.13-1.

Table 8.1.6.1.2.14.3.2-1a: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 14 | WLAN AP 1 (Cell 27) | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Refer to “C1” in TS 38.508-1 [4] |  |  |  |  |
| T1 | Refer to “C2” in TS 38.508-1 [4] |  |  |  |  |
| T2 | Refer to “C1” in TS 38.508-1 [4] |  |  |  |  |

Table 8.1.6.1.2.14.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a LoggedMeasurementConfiguration message on NR Cell 14. | <-- | NR RRC: LoggedMeasurementConfiguration | - | - |
| 2 | The SS transmits a RRCRelease message to release the RRC connection. | <-- | NR RRC: RRCRelease | - | - |
| 3 | Wait 5s for UE to activate logging. | - | - | - | - |
| 3A | The SS re-adjusts the power levels according to row "T1" in Table 8.1.6.1.2.14.3.2-1a. | - | - | - | - |
| 3B | Void | - | - | - | - |
| 3C | Void | - | - | - | - |
| 3D | The UE associates with the WLAN AP and obtains the local IP address. Note: The UL signal to WLAN AP shall generate IDC problem. The UE shall detect IDC problem in RRC_IDLE in configuration “C2” with UL signal to WLAN AP. | - | - | - | - |
| 4 | Void | - | - | - | - |
| 4A | The SS re-adjusts the power levels according to row "T2" in Table 8.1.6.1.2.14.3.2-1a. Note: UE shall not detect IDC problem in RRC_IDLE with configuration “T2” | - | - | - | - |
| 5 | Wait 5s for UE to resume logging. . | - | - | - | - |
| 6-13 | Steps 1 to 8 of the generic procedure for NR RRC_CONNECTED specified in TS 38.508-1 [4] Table 4.5.4.2-3 are performed. | - | - | - | - |
| 14-20 | Void | - | - | - | - |
| 21 | The SS transmits an UEInformationRequest message to get logMeasReport. | <-- | NR RRC: UEInformationRequest | - | - |
| 22 | Check: Does the UE transmit an UEInformationResponse message including logMeasReport with a LogMeasInfoList consisting of at least three LogMeasInfo entries, one of the LogMeasInfo entries, except for the first one and the last one, is tagged with inDeviceCoexDetected-r17? Note 1: The relativeTimeStamp is increased between the subsequent LogMeasInfoList entries. | --> | NR RRC: UEInformationResponse | 1, 2 | P |

8.1.6.1.2.14.3.3		Specific message contents

Table 8.1.6.1.2.14.3.3-1: LoggedMeasurementConfiguration (step 1, Table 8.1.6.1.2.14.3.2-1)

Derivation path: TS 38.508-1 [4], Table 4.6.1-5AA with condition PERIODICAL

Table 8.1.6.1.2.14.3.3-2: UEInformationRequest (steps 21, Table 8.1.6.1.2.14.3.2-1)

Derivation path: TS 38.508-1 [4], Table 4.6.1-32A with condition LOG

Table 8.1.6.1.2.14.3.3-3: UEInformationResponse (step 22, Table 8.1.6.1.2.14.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark |  | Comment |  |  | Condition |  |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |  |  |  |  |
| logMeasReport-r16 SEQUENCE { |  |  |  |  |  |  |  |
| absoluteTimeStamp-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |  |  |  |  |
| traceReference-r16 SEQUENCE { |  |  |  |  |  |  |  |
| plmn-Identity-r16 SEQUENCE { |  |  |  |  |  |  |  |
| mcc SEQUENCE (SIZE (3)) OF MCC-NMC-Digit | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |  |  |  |  |
| mnc SEQUENCE (SIZE (2..3)) OF MCC-NMC-Digit | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| traceId-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| traceRecordingSessionRef-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |  |  |  |  |
| tce-Id-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |  |  |  |  |
| logMeasInfoList-r16 SEQUENCE (SIZE (1..maxLogMeasReport-r16)) OF LogMeasInfo-r16 SEQUENCE { | At least 3 entries where at least 3 entry complies to the entries with index ‘x-1’, ‘x’, ‘x+1’ below. SS records the relativeTimeStamp value for each entry |  |  |  |  |  |  |
| LogMeasInfo-r16[x-1] SEQUENCE { |  |  | entry x-1 |  |  |  |  |
| locationInfo-r16 | Not checked |  |  |  |  |  |  |
| relativeTimeStamp-r16 | SS record the value |  | relativeTimeStamp-r16_x-1 |  |  |  |  |
| servCellIdentity-r16 | Same as NR Cell 14 |  |  |  |  |  |  |
| measResultServCell-16 | Not checked |  |  |  |  |  |  |
| measResultNeighCells-r16 SEQUENCE { |  |  |  |  |  |  |  |
| measResultNeighCellListNR | Not present |  |  |  |  |  |  |
| measResultNeighCellListEUTRA | Not present |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| anyCellSelectionDetected-r16 | Not present |  |  |  |  |  |  |
| inDeviceCoexDetected-r17 | Not present |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| LogMeasInfo-r16[x] SEQUENCE { |  |  | entry x (x can be any but the first and last entry number within logMeasInfoList-r16) |  |  |  |  |
| locationInfo-r16 | Not checked |  |  |  |  |  |  |
| relativeTimeStamp-r16 | SS record the value |  | relativeTimeStamp-r16_x |  |  |  |  |
| servCellIdentity-r16 | Same as NR Cell 14 |  |  |
| measResultServCell-16 | Not checked |  |  |  |  |  |  |
| measResultNeighCells-r16 SEQUENCE { |  |  |  |  |  |  |  |
| measResultNeighCellListNR | Not present |  |  |  |  |  |  |
| measResultNeighCellListEUTRA | Not present |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| anyCellSelectionDetected-r16 | Not present |  |  |  |  |  |  |
| inDeviceCoexDetected-r17 | true |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| LogMeasInfo-r16[x+1] SEQUENCE { |  |  | entry x+1 |  |  |  |  |
| locationInfo-r16 | Not checked |  |  |  |  |  |  |
| relativeTimeStamp-r16 | SS record the value |  | relativeTimeStamp-r16_x+1 |  |  |  |  |
| servCellIdentity-r16 | Same as NR Cell 14 |  |  |  |  |  |  |
| measResultServCell-16 | Not checked |  |  |  |  |  |  |
| measResultNeighCells-r16 SEQUENCE { |  |  |  |  |  |  |  |
| measResultNeighCellListNR | Not present |  |  |  |  |  |  |
| measResultNeighCellListEUTRA | Not present |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| anyCellSelectionDetected-r16 | Not present |  |  |  |  |  |  |
| inDeviceCoexDetected-r17 | Not present |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| logMeasAvailable-r16 | Not present |  |  |  |  |  |  |
| logMeasAvailableBT-r16 | Not present |  |  |  |  |  |  |
| logMeasAvailableWLAN-r16 | Not present |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

<End of modified section>