3GPP TSG RAN5 Meeting #105	 	R5-247337

Orlando, USA, 18th - 22nd November 2024

Source:		Rohde & Schwarz

Title:	On the MU and TT for SISO and MIMO test cases

Agenda Item:		5.4.20

Document for:	Discussion and Endorsement

# Introduction

For several Rel-15 SISO or UL MIMO test cases the MU and TT has not been specified which blocks the progress of these test cases. In this contribution, we propose MU and TT for some of these test cases.

# Discussion

In this paper, we focus our analysis on PC3, CHBW ≤ 400 MHz, IFF with a maximum device size of 40cm.

## UL MIMO test cases

### Occupied bandwidth for UL MIMO

Compared to the OBW SISO TC which applies DFT-s-OFDM QPSK, the MIMO test case uses CP-OFDM QPSK. In contrast to SISO, there is no MBR for MIMO. Considering MBR, MPR and T(MPR), PUmax for MIMO is 2.75 dB (CHBW≤200MHz) and 3.25 dB (CHBW=400MHz) lower than that for SISO for FR2a and FR2b. For FR2c, PUmax for MIMO is 3.0 dB (CHBW≤200MHz) and 3.5 dB (CHBW=400MHz) lower than that for SISO. This results in a lower SNR and, thus, higher OBW uncertainties for the bandwidth and frequency ranges where the impact of SNR is significant.

Based on the characteristics of our test equipment, we propose the uncertainties summarized in Table 1 for PC3, IFF with a max device size ≤ 40 cm.

Proposal 1: We propose the values in Table 1 for MU of OBW with UL MIMO, PC3, IFF, max device size ≤ 40 cm.

Table 1: Proposed MU for OBW for UL MIMO with PC3

| Frequency Range | CHBW (MHz) | MU (%CBW) |
| --- | --- | --- |
| FR2a | 50 | ±0.4 |
|  | 100 | ±0.4 |
|  | 200 | ±1.2 |
|  | 400 | TBD |
| FR2b | 50 | ±0.4 |
|  | 100 | ±0.4 |
|  | 200 | ±1.3 |
|  | 400 | TBD |
| FR2c | 50 | ±0.65 |
|  | 100 | ±0.65 |
|  | 200 | TBD |
|  | 400 | TBD |

For the channel bandwidths and frequency ranges with defined MU, we propose a TT of 0 dB.

Proposal 2: For the channel bandwidths and frequency ranges with defined MU in Table 1, we propose a TT of 0 dB.

## SISO test cases

In the following, we discuss the MU for the EVM SISO test cases.

### EVM for PUCCH

The measurement uncertainty for PUCCH has yet not been specified. The test procedures for both PUSCH and PUCCH test the EVM at the UE's maximum output power level PUMAX. For PUSCH an Outer_Full allocation is applied. For PUCCH Format 1 and Format 3 are applied which use up to 16 RBs. Thus, the power spectral density (PSD) for PUCCH is higher than that for PUSCH. Therefore, the MU for EVM of PUSCH is an upper bound for the MU for EVM of PUCCH and we propose to reuse the MU and TT of EVM PUSCH.

Proposal 3: For EVM PUCCH reuse the MU and TT of EVM PUSCH.

### EVM measurement with dual polarization

Annex E.7 of TS 38.521-2 describes the procedure for EVM measurement for dual transmit polarizations. As defined in clause 6.1 of TS 38.521-2, the UE is allowed to transmit with dual polarization. Therefore, an update of the test procedure is needed to refer to Annex E.7.1 with E.7.3 Maximum Ratio Combining for the EVM SISO test cases.

The currently specified EVM MUs for EVM are based on the measurement with a single polarization and consider 3 dB signal reduction due to polarization mismatch. In the case of maximum ratio combining, the signal from both polarizations are combined so that a signal reduction due to polarization mismatch does not occur. On the other side, the noise from two RF chains needs to be considered for the maximum ratio combining method. Thus, in total the SNR is the same for the EVM measurement with dual polarization with maximum ratio combining. Therefore, we propose to reuse the MU and TT from the single polarization measurement for the dual polarization measurement with maximum ratio combining.

Proposal 4: Leverage the MU and TT for EVM from the single polarization measurement for the dual polarization measurement with maximum ratio combining.

Accompanying CRs in Refs. [1] and [2] document the MU assessment in TR 38.903 [3] and TS 38.521-2 [4], respectively. CR in [5] updates the status of the associated EN-DC TCs in TS 38.521-3 [6]. The applicability in TS 38.522 [7] is updated with CR in Ref. [8].

# Assumptions

| ID | Description | Assumption |
| --- | --- | --- |
| #1 | Frequency ranges under consideration | FR2 |
| #2 | Size of QZ for IFF | 40 cm |
| #3 | UE power class | PC3 |
| #4 | Max EIRP | EIRP MAX= +43 dBm (rms) |
| #5 | Wanted signal PAPR | 13 dB |
| #6 | Temperature range of the test equipment | 20°C – 35°C |
| #7 | Channel bandwidth | 400 MHz |

# Conclusion

In summary, we have assessed the uncertainty and make the following proposals:

Proposal 1: We propose the values in Table 1 for MU of OBW with UL MIMO, PC3, IFF, max device size ≤ 40 cm.

Proposal 2: For the channel bandwidths and frequency ranges with defined MU in Table 1, we propose a TT of 0 dB.

Proposal 3: For EVM PUCCH reuse the MU and TT of EVM PUSCH.

Proposal 4: Leverage the MU and TT for EVM from the single polarization measurement for the dual polarization measurement with maximum ratio combining.

# References

- R5-247338, Documentation of MU for SISO and UL MIMO, Rohde & Schwarz, 3GPP TSG RAN WG 5 Meeting #105, November 2024

- R5-247339, Update of MU and TT for SISO and MIMO, Rohde & Schwarz, 3GPP TSG RAN WG 5 Meeting #105, November 2024

- TR 38.903, V18.4.0, 3GPP TSG RAN WG5, September 2024

- TS 38.521-2, V18.4.0, 3GPP TSG RAN WG5, September 2024

- R5-247340, Update of status of FR2 SISO and UL MIMO test cases, Rohde & Schwarz, 3GPP TSG RAN WG 5 Meeting #105, November 2024

- TS 38.521-3, V18.4.0, 3GPP TSG RAN WG5, September 2024

- TS 38.522, V18.4.0, 3GPP TSG RAN WG5, September 2024

- R5-247341, Update of applicability of FR2 SISO and MIMO test cases, Rohde & Schwarz, 3GPP TSG RAN WG 5 Meeting #105, November 2024